3GPP TSG-RAN WG4 Meeting #82

Athens, GR, 25 Feb - 1 Mar, 2019

Source:        Samsung

Title:           TP analysis on FR1 6.5A.4.1 TX IM for inter-band CA

# Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC TXIM, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for TX IM of 2UL inter-band CA case.

# Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA is defined as table below.

Table 6.7A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2, 3) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 | N/A for this test | QPSK | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@0 | S_12@0 | - | - |
| 50 | 25 |  | QPSK | 20 | P_12@0 | S_8@0 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 75 |  | QPSK | 28 | P_12@0 | S_16@0 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@0 | S_18@0 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 50 |  | QPSK | 28 | P_16@0 | S_12@0 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 100 |  | QPSK | 34 | P_16@0 | S_18@0 | - | - |
| 100 | 50 |  | QPSK | 30 | P_18@0 | S_12@0 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@0 | S_18@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2 Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 3: If in the CA Configuration UE supports CC Combinations with channel BW below 5MHz on at least one CC, these CC Combinations are disregarded for Lowest NRB_agg. Only CC Combinations which have channel BW 5MHz or above on both component carriers are considered for Lowest NRB_agg. |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for TX IM for single carrier mode is defined as table below.

Table 6.5.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for transmit intermodulation test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

## Test Environment

In LTE test case for TX IM, Normal condition is used for test. Based on the baseline agreed in [1], the same test environment as LTE can be reused for FR1.

Proposal 1: Define Test Environment as NC for TX IM of 2UL inter-band CA in FR1.

## Test Frequencies

In LTE test case for TX IM, Mid range is used for test. Based on the baseline agreed in [1], the same frequency range as LTE can be reused for FR1.

Proposal 2: Select Mid range as test frequency for NR TX IM of 2UL inter-band CA for FR1.

## Test Channel Bandwidth

In LTE test case for CA TX IM, both Lowest and highest NRB_agg  are used for testing. Based on the baseline agreed in [1], the same frequency range as LTE can be reused for FR1 CA case.

Proposal 3: Select both Lowest and highest NRB_agg  to measure TX IM for FR1 2UL inter-band CA

## Test Subcarrier Spacing

Same as single carrier case discussed in [3], it is proposed to select lowest and highest SCS case to verify TX IM performance for inter-band CA

Proposal 4: Select the lowest and highest Subcarrier Spacing to test for inter-band 2UL CA.

## DL and UL configuration

Same as single carrier case discussed in [3], DFT-s-OFDM PI/2 BPSK with Inner Full allocation and DFT-s-OFDM QPSK with Inner Full allocation are suggested to be used as UL configuration.

Proposal 5: Select DFT-s-OFDM PI/2 BPSK with Inner Full allocation and DFT-s-OFDM QPSK with Inner Full allocation

## Number of test points

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 2 | 8 |

# Conclusion

For the inter-band UL CA TX IM requirement, the test condition is proposed as below

Proposal 1: Define Test Environment as NC for TX IM of 2UL inter-band CA in FR1.

Proposal 2: Select Mid range as test frequency for NR TX IM of 2UL inter-band CA for FR1.

Proposal 3: Select both Lowest and highest NRB_agg  to measure TX IM for FR1 2UL inter-band CA

Proposal 4: Select the lowest and highest Subcarrier Spacing to test for inter-band 2UL CA.

Proposal 5: Select DFT-s-OFDM PI/2 BPSK with Inner Full allocation and DFT-s-OFDM QPSK with Inner Full allocation

# Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-185423, “Discussion on Uplink configuration for NR Transmit Intermodulation in FR1”

[4] 3GPP TS 36.521-1 v15.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v15.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15).

[6] 3GPP TS 38.101-1 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15).