3GPP TSG-RAN5 Meeting #82	R5-191780

# Athens, Greece, 25th Feb 2019 - 1th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4699 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test requirements in A-MPR for intra-band contiguous UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Table 6.2.4A.1.5-6: Test requirement (network signalled value "CA_NS_06") ID=1,7 : Test requirments are different for test Freq=Low,High due to the difference of Delta Tc. - Table 6.2.4A.1_1.5-1: Test requirement (network signalled value "CA_NS_01") Configration ID12 : Lower tolerane is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table 6.2.4A.1.5-6: Test requirement (network signalled value "CA_NS_06") ID=1,7 : Test requirments are seprately specified for freq=Low and High respectively. - Table 6.2.4A.1_1.5-1: Test requirement (network signalled value "CA_NS_01") Configration ID12 : Lower Tolerance is changed -23.7 -> -24.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test requirements remains incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A.1, 6.2.4A.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[Start of changes]

#### 6.2.4A.1	Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA)

##### 6.2.4A.1.1	Test purpose

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the CA Power Class as specified in Table 6.2.2A.0-2.

##### 6.2.4A.1.2	Test applicability

The requirements of this test apply in test case 6.6.2.2A.1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for network signalled value CA_NS_04 to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA but do not support UL 64QAM or UL 256QAM.

The requirements of this test apply in test case 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for network signalled values CA_NS_01, CA_NS_02 and CA_NS_03, CA_NS_05, CA_NS_06, CA_NS_07 to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA but do not support UL 64QAM or UL 256QAM.

NOTE:	As a result TC 6.2.4A.1 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

##### 6.2.4A.1.3	Minimum conformance requirements

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the CA Power Class as specified in Table 6.2.2A.0-2.

If for intra-band carrier aggregation the UE is configured for transmissions on a single serving cell, then subclauses 6.2.3 and 6.2 4 apply with the Network Signalling value indicated by the field additionalSpectrumEmission.

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2.4A.1.3-1 is allowed for all serving cells of the applicable uplink CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10. Then clause 6.2.3A.1.3 does not apply, i.e. the carrier aggregation MPR = 0 dB, unless the value indicated is CA_NS_31.

For the UE maximum output power modified by A-MPR specified in table 6.2.4A.1.3-1, the power limits specified in subclause 6.2.5A.1.3 apply.

Table 6.2.4A.1.3-1: Additional Maximum Power Reduction (A-MPR) for intra-band contiguous CA

| CA Network Signalling value | Requirements (subclause) | Uplink CA Configuration | A-MPR [dB] (subclause) |
| --- | --- | --- | --- |
| CA_NS_01 | 6.6.3.3A.1.3.1 | CA_1C | 6.2.4A.1.3.1 |
| CA_NS_02 | 6.6.3.3A.1.3.2 | CA_1C | 6.2.4A.1.3.2 |
| CA_NS_03 | 6.6.3.3A.1.3.3 | CA_1C | 6.2.4A.1.3.3 |
| CA_NS_04 | 6.6.2.2A.1.3.1 | CA_41C | 6.2.4A.1.3.4 |
| CA_NS_05 | 6.6.3.3A.1.3.4 | CA_38C | 6.2.4A.1.3.5 |
| CA_NS_06 | 6.6.3.3A.1.3.5 | CA_7C | 6.2.4A.1.3.6 |
| CA_NS_07 | 6.6.3.3A.1.3.6 | CA_39C | 6.2.4A.1.3.7 |
| … |  |  |  |
| CA_NS_31 | NOTE 1 | Table 5.4.2A.1-1 (NOTE 1) | N/A |
| CA_NS_32 | Reserved |  |  |
| NOTE 1: Applicable for uplink CA configurations listed in Table 5.4.2A.1-1 for which none of the additional requirements in subclauses 6.6.3.3A or 6.6.2.2A apply. NOTE 2: The index of the sequence CA_NS corresponds to the value of additionalSpectrumEmissionSCell-r10. |  |  |  |

For PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For intra-band carrier aggregation, the A-MPR is evaluated per slot and given by the maximum value taken over the transmission(s) on all component carriers within the slot; the maximum A-MPR over the two slots is then applied for the entire subframe.

6.2.4A.1.3.1	A-MPR for CA_NS_01 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_01 the allowed maximum output power reduction applied to transmissions on the PCC and the SCC for contiguously aggregated signals is specified in table 6.2.4A.1.3.1-1.

Table 6.2.4A.1.3.1-1: Contiguous allocation A-MPR for CA_NS_01

| CA_1C: CA_NS_01 | RBstart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK and 16-QAM [dB] |
| --- | --- | --- | --- | --- |
| 100 RB / 100 RB | 0 – 23 and 176 - 199 | > 0 | N/A | ≤ 12.0 |
|  | 24 – 105 | > 64 | N/A | ≤ 6.0 |
|  | 106 – 175 | N/A | > 175 | ≤ 5.0 |
| 75 RB / 75 RB | 0 – 6 and 143 – 149 | 0 < LCRB ≤ 10 | N/A | ≤ 11.0 |
|  |  | > 10 | N/A | ≤ 6.0 |
|  | 7 – 90 | > 44 | N/A | ≤ 5.0 |
|  | 91 – 142 | N/A | > 142 | ≤ 2.0 |
| NOTE 1: RB_start indicates the lowest RB index of transmitted resource blocks NOTE 2: L_CRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |

If the UE is configured to CA_1C and it receives IE CA_NS_01 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows:

MA = -22.5 A + 17		; 0 ≤ A < 0.20

-11.0 A + 14.7			; 0.20 ≤ A < 0.70

-1.7 A + 8.2			 ; 0.70 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg

The normative reference for requirement is in TS 36.101 [2] clause 6.2.4A

6.2.4A.1.3.2	A-MPR for CA_NS_02 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_02 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1.3.2-1.

Table 6.2.4A.1.3.2-1: Contiguous allocation A-MPR for CA_NS_02

| CA_1C: CA_NS_02 | RBend | LCRB [RBs] | A-MPR for QPSK and 16 -QAM [dB] |
| --- | --- | --- | --- |
| 100 RB / 100 RB | 0 –20 | > 0 | ≤ 4 dB |
|  | 21 – 46 | > 0 | ≤ 3 dB |
|  | 47 – 99 | > RBend – 20 | ≤ 3 dB |
|  | 100 – 184 | > 75 | ≤ 6 dB |
|  | 185 – 199 | > 0 | ≤ 10 dB |
| 75 RB / 75 RB | 0 – 48 | > 0 | ≤ 2 dB |
|  | 49 – 80 | > RBend – 20 | ≤ 3 dB |
|  | 81 – 129 | > 60 | ≤ 5 dB |
|  | 130 – 149 | > 85 | ≤ 6 dB |
|  | 130 – 149 | 1 – 84 | ≤ 2 dB |

If the UE is configured to CA_1C and it receives IE CA_NS_02 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows:

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

[MA = -22.5 A + 17		; 0 ≤ A < 0.20

-11.0 A + 14.7			; 0.20 ≤ A < 0.70

-1.7 A + 8.2			 ; 0.70 ≤ A ≤ 1]

Where A = NRB_alloc / NRB_agg.

6.2.4A.1.3.3	A-MPR for CA_NS_03 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_03 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1.3.3-1.

Table 6.2.4A.1.3.3-1: Contiguous allocation A-MPR for CA_NS_03

| CA_1C: CA_NS_03 | RBend | LCRB [RBs] | A-MPR for QPSK and 16-QAM [dB] |
| --- | --- | --- | --- |
| 100 RB / 100 RB | 0 – 26 | > 0 | ≤ 10 dB |
|  | 27 – 63 | ≥ RBend – 27 | ≤ 6 dB |
|  | 27 – 63 | < RBend – 27 | ≤ 1 dB |
|  | 64 – 100 | > RBend – 20 | ≤ 4 dB |
|  | 101 – 171 | > 68 | ≤ 7 dB |
|  | 172 – 199 | > 0 | ≤ 10 dB |
| 75 RB / 75 RB | 0 – 20 | > 0 | ≤ 10 dB |
|  | 21 – 45 | > 0 | ≤ 4 dB |
|  | 46 – 75 | > RBend – 13 | ≤ 2 dB |
|  | 76 – 95 | > 45 | ≤ 5 dB |
|  | 96 – 149 | > 43 | ≤ 8 dB |
|  | 120 – 149 | 1 – 43 | ≤ 6 dB |

If the UE is configured to CA_1C and it receives IE CA_NS_03 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows:

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

[MA = -23.33A + 17.5		; 0 ≤ A < 0.15

-7.65A + 15.15			; 0.15 ≤ A ≤ 1]

Where A = NRB_alloc / NRB_agg.

6.2.4A.1.3.4	A-MPR for CA_NS_04 for CA_41C

If the UE is configured to CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1.3.4-1.

Table 6.2.4A.1.3.4-1: Contiguous Allocation A-MPR for CA_NS_04

| CA Bandwidth Class C | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK [dB] | A-MPR for 16QAM [dB] |
| --- | --- | --- | --- | --- | --- |
| 50RB / 100 RB | 0 – 44 and 105 – 149 | >0 | N/A | ≤4dB | ≤4dB |
|  | 45 – 104 | N/A | >105 | ≤3dB | ≤4dB |
| 75 RB / 75 RB | 0 – 44 and 105 – 149 | >0 | N/A | ≤4dB | ≤4dB |
|  | 45 – 104 | N/A | >105 | ≤4dB | ≤4dB |
| 100 RB / 75 RB | 0 – 49 and 125 – 174 | >0 | N/A | ≤4dB | ≤4dB |
|  | 50 - 124 | N/A | >125 | ≤3dB | ≤4dB |
| 100 RB / 100 RB | 0 – 59 and 140 – 199 | >0 | N/A | ≤3dB | ≤4dB |
|  | 60– 139 | N/A | >140 | ≤3dB | ≤4dB |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |  |

If the UE is configured to CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA 	= 	10.5,					0≤ A < 0.05

= -50.0A + 13.00,		0.05≤ A < 0.15

= -4.0A + 6.10,			0.15≤ A < 0.40

= -0.83A + 4.83,				0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

###### 6.2.4A.1.3.5	A-MPR for CA_NS_05 for CA_38C

If the UE is configured to CA_38C and it receives IE CA_NS_05 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1.3.5-1.

Table 6.2.4A.1.3.5-1: Contiguous Allocation A-MPR for CA_NS_05

| CA_38C | RBend | LCRB [RBs] | A-MPR for QPSK and 16-QAM [dB] |
| --- | --- | --- | --- |
| 100RB/100RB | 0 – 12 | >0 | ≤ 5 dB |
|  | 13 – 79 | > RBend – 13 | ≤ 2 dB |
|  | 80 – 180 | >60 | ≤ 6 dB |
|  | 181 – 199 | > 0 | ≤ 11 dB |
| 75RB/75RB | 0 – 70 | > max(0, RBend -10) | ≤ 2 dB |
|  | 71- 108 | > 60 | ≤ 5 dB |
|  | 109 – 140 | >0 | ≤ 5 dB |
|  | 140 – 149 | ≤ 70 | ≤ 2 dB |
|  | 140 – 149 | >70 | ≤ 6 dB |
| NOTE 1: RBend indicates the highest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |

If the UE is configured to CA_38C and it receives IE CA_NS_05 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = -14.17 A + 16.50		; 0 ≤ A < 0.60

-2.50 A + 9.50			; 0.60 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

###### 6.2.4A.1.3.6	A-MPR for CA_NS_06 for CA_7C

If the UE is configured to CA_7C and it receives IE CA_NS_06 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1.3.6-1.

Table 6.2.4A.1.3.6-1: Contiguous Allocation A-MPR for CA_NS_06

| CA Bandwidth Class C | RB_end | LCRB [RBs] | A-MPR for QPSK and 16-QAM [dB] |
| --- | --- | --- | --- |
| 100RB/100RB | [0 –22] | >[0] | ≤ [4] dB |
|  | [23 – 33] | > [RBend – 10] | ≤ [2] dB |
|  | [106 – 142] | > [75] | ≤ [3] dB |
|  | [143 – 178] | >[70] | ≤ [5] dB |
|  | [179 – 199] | > [0] | ≤ [10] dB |
| 75RB/75RB | [0 – 7] | >[0] | ≤ [5] dB |
|  | [20- 75] | > [RBend – 10] | ≤ [2] dB |
|  | [75 – 110] | >[64] | ≤ [2] dB |
|  | [110 – 144] | >[35] | ≤ [6] dB |
|  | [145 – 149] | >[0] | ≤ [10] dB |

If the UE is configured to CA_7C and it receives IE CA_NS_06 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows:

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = [-23.33A + 17.5; 0 ≤ A < 0.15

-7.65A + 15.15; 0.15 ≤ A ≤ 1]

Where A = NRB_alloc / NRB_agg.

###### 6.2.4A.1.3.7	A-MPR for CA_NS_07 for CA_39C

If the UE is configured to CA_39C and it receives IE CA_NS_07 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1.3.7-1.

Table 6.2.4A.1.3.7-1: Contiguous Allocation A-MPR for CA_NS_07

| CA_39C: CA_NS_07 | RBStart | LCRB [RBs] | A-MPR for QPSK and 16-QAM[dB] |
| --- | --- | --- | --- |
| 75 RB / 100 RB and 100 RB / 75 RB | 0 – 13 | > 0 | ≤ 11 |
|  | 14 – 50 | ≤ 60 | ≤ 3 |
|  | 14 – 100 | > 60 | ≤ 7 |
|  | 101 – 155 | > max(155 - RBstart , 0) | ≤ 2 |
|  | 156 – 174 | > 0 | ≤ 5 |
| 50 RB / 100 RB and 100 RB / 50 RB | 0 – 5 | > 0 | ≤ 11 |
|  | 6 – 42 | ≤ 25 | ≤ 3 |
|  |  | > 25 | ≤ 6 |
|  | 43 – 80 | > 50 | ≤ 5 |
|  | 81 – 138 | > 20 | ≤ 2 |
|  | 139 – 149 | > 0 | ≤ 5 |
| 25 RB / 100 RB and 100 RB / 25 RB | 0 – 32 | ≥ 84 | ≤ 6 |
|  |  | < 84 | ≤ 4 |
|  | 33 – 60 | > 50 | ≤ 3 |
|  | 61 – 124 | > 20 | ≤ 3 |

If the UE is configured to CA_39C and it receives IE CA_NS_07 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows:

MA = -16.25A + 21				; 0 ≤ A < 0.80

-2.50 A + 10.00				; 0.80 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg

##### 6.2.4A.1.4	Test description

6.2.4A.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.2.4A.1.4.1-1 to 6.2.4A.1.4.1-7. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4A.1.4.1-1: Test Configuration Table (network signalled value "CA_NS_01")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1.3.1-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | QPSK | 1 |  | P_1@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | QPSK | 150 |  | P_75@0 | S_75@0 |  |  |
| 3 | 75 | 75 |  | QPSK | 45 |  | P_45@7 | S_0@0 |  |  |
| 4 | 75 | 75 |  | QPSK | 8 |  | P_0@0 | S_8@67 |  |  |
| 5 | 75 | 75 |  | QPSK | 129 |  | P_75@0 | S_54@0 |  |  |
| 6 | 75 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| 7 | 100 | 100 |  | QPSK | 200 |  | P_100@0 | S_100@0 |  |  |
| 8 | 100 | 100 |  | QPSK | 1 |  | P_1@0 | S_0@0 |  |  |
| 9 | 100 | 100 |  | QPSK | 175 |  | P_75@25 | S_100@0 |  |  |
| 10 | 100 | 100 |  | QPSK | 25 |  | P_0@0 | S_25@75 |  |  |
| 11 | 100 | 100 |  | QPSK | 64 |  | P_64@24 | S_0@0 |  |  |
| 12 | 100 | 100 |  | QPSK | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1.4.1-2: Test Configuration Table (network signalled value "CA_NS_02")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1.3.2-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | QPSK | 1 |  | P_1@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | QPSK | 75 |  | P_75@0 | S_0@0 |  |  |
| 3 | 75 | 75 |  | QPSK | 129 |  | P_75@0 | S_54@0 |  |  |
| 4 | 75 | 75 |  | QPSK | 129 |  | P_54@21 | S_75@0 |  |  |
| 5 | 75 | 75 |  | QPSK | 1 |  | P_0@0 | S_1@74 |  |  |
| 6 | 75 | 75 |  | QPSK | 1 |  | P_0@0 | S_1@54 |  |  |
| 7 | 75 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| 8 | 100 | 100 |  | QPSK | 1 |  | P_1@0 | S_0@0 |  |  |
| 9 | 100 | 100 |  | QPSK | 1 |  | P_1@21 | S_0@0 |  |  |
| 10 | 100 | 100 |  | QPSK | 90 |  | P_90@0 | S_0@0 |  |  |
| 11 | 100 | 100 |  | QPSK | 180 |  | P_100@0 | S_80@0 |  |  |
| 12 | 100 | 100 |  | QPSK | 1 |  | P_0@0 | S_1@99 |  |  |
| 13 | 100 | 100 |  | QPSK | 1 |  | P_0@0 | S_1@83 |  |  |
| 14 | 100 | 100 |  | QPSK | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1.4.1-3: Test Configuration Table (network signalled value "CA_NS_03")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1.3.3-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | QPSK | 1 |  | P_1@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | QPSK | 1 |  | P_1@21 | S_0@0 |  |  |
| 3 | 75 | 75 |  | QPSK | 75 |  | P_75@0 | S_0@0 |  |  |
| 4 | 75 | 75 |  | QPSK | 90 |  | P_75@0 | S_15@0 |  |  |
| 5 | 75 | 75 |  | QPSK | 150 |  | P_75@0 | S_75@0 |  |  |
| 6 | 75 | 75 |  | QPSK | 1 |  | P_0@0 | S_1@74 |  |  |
| 7 | 75 | 75 |  | QPSK | 1 |  | P_0@0 | S_1@44 |  |  |
| 8 | 75 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| 9 | 100 | 100 |  | QPSK | 1 |  | P_1@0 | S_0@0 |  |  |
| 10 | 100 | 100 |  | QPSK | 60 |  | P_60@0 | S_0@0 |  |  |
| 11 | 100 | 100 |  | QPSK | 1 |  | P_1@63 | S_0@0 |  |  |
| 12 | 100 | 100 |  | QPSK | 90 |  | P_90@0 | S_0@0 |  |  |
| 13 | 100 | 100 |  | QPSK | 164 |  | P_100@0 | S_64@0 |  |  |
| 14 | 100 | 100 |  | QPSK | 1 |  | P_0@0 | S_1@99 |  |  |
| 15 | 100 | 100 |  | QPSK | 1 |  | P_0@0 | S_1@70 |  |  |
| 16 | 100 | 100 |  | QPSK | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1.4.1-4: Test Configuration Table (network signalled value "CA_NS_04")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1.3.4-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 50 | N/A | QPSK | 10 |  | P_10@20 | S_0@0 |  |  |
| 2 | 100 | 50 |  | QPSK | 60 |  | P_50@50 | S_10@0 |  |  |
| 3 | 100 | 50 |  | 16QAM | 15 |  | P_0@0 | S_15@0 |  |  |
| 4 | 100 | 50 |  | QPSK | 2 |  | P_1@0 | S_1@49 |  |  |
| 5 | 75 | 75 |  | QPSK | 10 |  | P_10@20 | S_0@0 |  |  |
| 6 | 75 | 75 |  | QPSK | 75 |  | P_30@45 | S_45@0 |  |  |
| 7 | 75 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| 8 | 100 | 75 |  | QPSK | 10 |  | P_10@20 | S_0@0 |  |  |
| 9 | 100 | 75 |  | QPSK | 80 |  | P_50@50 | S_30@0 |  |  |
| 10 | 100 | 75 |  | 16QAM | 20 |  | P_0@0 | S_20@15 |  |  |
| 11 | 100 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| 12 | 100 | 100 |  | QPSK | 10 |  | P_10@25 | S_0@0 |  |  |
| 13 | 100 | 100 |  | QPSK | 90 |  | P_40@60 | S_50@0 |  |  |
| 14 | 100 | 100 |  | 16QAM | 15 |  | P_0@0 | S_15@40 |  |  |
| 15 | 100 | 100 |  | 16QAM | 20 |  | P_0@0 | S_20@30 |  |  |
| 16 | 100 | 100 |  | QPSK | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1.4.1-5: Test Configuration Table (network signalled value "CA_NS_05")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1.3.5-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | QPSK | 40 |  | P_40@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | QPSK | 80 |  | P_50@25 | S_30@0 |  |  |
| 3 | 75 | 75 |  | QPSK | 60 |  | P_10@65 | S_50@0 |  |  |
| 4 | 75 | 75 |  | QPSK | 65 |  | P_1@74 | S_64@0 |  |  |
| 5 | 75 | 75 |  | QPSK | 92 |  | P_20@55 | S_72@0 |  |  |
| 6 | 75 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| 7 | 100 | 100 |  | QPSK | 8 |  | P_8@0 | S_0@0 |  |  |
| 8 | 100 | 100 |  | QPSK | 40 |  | P_40@0 | S_0@0 |  |  |
| 9 | 100 | 100 |  | QPSK | 80 |  | P_50@50 | S_30@0 |  |  |
| 10 | 100 | 100 |  | QPSK | 150 |  | P_60@40 | S_90@0 |  |  |
| 11 | 100 | 100 |  | QPSK | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1.4.1-6: Test Configuration Table (network signalled value "CA_NS_06")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | As in Table 6.2.4A.1.3.6-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | QPSK | 5 |  | P_5@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | QPSK | 45 |  | P_45@0 | S_0@0 |  |  |
| 3 | 75 | 75 |  | QPSK | 75 |  | P_60@15 | S_15@0 |  |  |
| 4 | 75 | 75 |  | QPSK | 60 |  | P_10@65 | S_50@0 |  |  |
| 5 | 75 | 75 |  | QPSK | 90 |  | P_18@57 | S_72@0 |  |  |
| 6 | 75 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| 7 | 100 | 100 |  | QPSK | 10 |  | P_10@0 | S_0@0 |  |  |
| 8 | 100 | 100 |  | QPSK | 30 |  | P_30@0 | S_0@0 |  |  |
| 9 | 100 | 100 |  | QPSK | 100 |  | P_75@25 | S_25@0 |  |  |
| 10 | 100 | 100 |  | QPSK | 90 |  | P_40@60 | S_50@0 |  |  |
| 11 | 100 | 100 |  | QPSK | 96 |  | P_15@85 | S_81@0 |  |  |
| 12 | 100 | 100 |  | QPSK | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1.4.1-7: Test Configuration Table (network signalled value "CA_NS_07")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | As in Table 6.2.4A.1.3.7-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 25 | N/A | QPSK | 1 |  | P_1@0 | S_0@0 |  |  |
| 2 | 100 | 25 |  | QPSK | 85 |  | P_80@20 | S_5@0 |  |  |
| 3 | 100 | 25 |  | QPSK | 65 |  | P_60@40 | S_5@0 |  |  |
| 4 | 100 | 25 |  | QPSK | 35 |  | P_30@70 | S_5@0 |  |  |
| 5 | 100 | 25 |  | QPSK | 2 |  | P_1@0 | S_1@24 |  |  |
| 6 | 100 | 50 |  | QPSK | 1 |  | P_1@1 | S_0@0 |  |  |
| 7 | 100 | 50 |  | QPSK | 10 |  | P_10@30 | S_0@0 |  |  |
| 8 | 100 | 50 |  | QPSK | 30 |  | P_30@30 | S_0@0 |  |  |
| 9 | 100 | 50 |  | QPSK | 60 |  | P_40@60 | S_20@0 |  |  |
| 10 | 100 | 50 |  | QPSK | 30 |  | P_10@90 | S_20@0 |  |  |
| 11 | 100 | 50 |  | QPSK | 5 |  | P_0@0 | S_5@45 |  |  |
| 12 | 100 | 50 |  | QPSK | 2 |  | P_1@0 | S_1@49 |  |  |
| 13 | 100 | 75 |  | QPSK | 1 |  | P_1@1 | S_0@0 |  |  |
| 14 | 100 | 75 |  | QPSK | 40 |  | P_40@20 | S_0@0 |  |  |
| 15 | 100 | 75 |  | QPSK | 85 |  | P_80@20 | S_5@0 |  |  |
| 16 | 100 | 75 |  | QPSK | 40 |  | P_0@0 | S_40@20 |  |  |
| 17 | 100 | 75 |  | QPSK | 15 |  | P_0@0 | S_15@60 |  |  |
| 18 | 100 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to the applicable table from Table 6.2.4A.1.4.1-1 to 6.2.4A.1.4.1-7.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4A.1.4.3.

6.2.4A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.2.4A.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to the applicable from Table 6.2.4A.1.4.1-1 to Table 6.2.4A.1.4.1-7 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

6.	Measure the mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

6.2.4A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.The following exceptions apply for each network signalled value.

6.2.4A.1.4.3.1	Message contents exceptions (network signalled value "CA_NS_01")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NS_01. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages. This exception indicates that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

Table 6.2.4A.1.4.3.1-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NS_01"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 1 (CA_NS_01) |  |  |

6.2.4A.1.4.3.2	Message contents exceptions (network signalled value "CA_NS_02")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NS_02. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages. This exception indicates that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

Table 6.2.4A.1.4.3.2-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NS_02"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 2 (CA_NS_02) |  |  |

6.2.4A.1.4.3.3	Message contents exceptions (network signalled value "CA_NS_03")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NS_03. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages. This exception indicates that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

Table 6.2.4A.1.4.3.3-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NS_03"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 3 (CA_NS_03) |  |  |

6.2.4A.1.4.3.4	Message contents exceptions (network signalled value "CA_NS_04")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NS_04. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages. This exception indicates that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

Table 6.2.4A.1.4.3.4-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NS_04"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 4 (CA_NS_04) |  |  |

6.2.4A.1.4.3.5	Message contents exceptions (network signalled value "CA_NS_05")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NS_05. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages. This exception indicates that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

Table 6.2.4A.1.4.3.5-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NS_05"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 5 (CA_NS_05) |  |  |

6.2.4A.1.4.3.6	Message contents exceptions (network signalled value "CA_NS_06")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NS_06. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages. This exception indicates that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

Table 6.2.4A.1.4.3.6-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NS_06"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 6 (CA_NS_06) |  |  |

6.2.4A.1.4.3.7	Message contents exceptions (network signalled value "CA_NS_07")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NS_07. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages. This exception indicates that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

Table 6.2.4A.1.4.3.7-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NS_07"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 7 (CA_NS_07) |  |  |

##### 6.2.4A.1.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4A.1.5-1 to Table 6.2.4A.1.5-7. The allowed maximum output power reduction is specified in Table 6.2.4A.1.3-1 and clause 6.2.3A.1 does not apply, i.e. carrier aggregation MPR = 0. For the UE maximum output power modified by A-MPR specified in table 6.2.4A.1.3-1, the power limits specified in Table 6.2.5A.1.3-1 apply.

Table 6.2.4A.1.5-1: Test requirement (network signalled value "CA_NS_01")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -17.7 |
| 2 |  |  |  |  | 23 | +2.7 / -11.7 |
| 3 |  |  |  |  | 23 | +2.7 / -9.7 |
| 4 |  |  |  |  | 23 | +2.7 / -4.7 |
| 5 |  |  |  |  | 23 | +2.7 / -11.7 |
| 6 |  |  |  |  | 23 | +2.7 / -24.7 |
| 7 |  |  |  |  | 23 | +2.7 / -18.7 |
| 8 |  |  |  |  | 23 | +2.7 / -18.7 |
| 9 |  |  |  |  | 23 | +2.7 / -11.7 |
| 10 |  |  |  |  | 23 | +2.7 / -9.7 |
| 11 |  |  |  |  | 23 | +2.7 / -2.7 |
| 12 |  |  |  |  | 23 | +2.7 / -24.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1.5-2: Test requirement (network signalled value "CA_NS_02")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -4.7 |
| 2 |  |  |  |  | 23 | +2.7 / -6.2 |
| 3 |  |  |  |  | 23 | +2.7 / -9.7 |
| 4 |  |  |  |  | 23 | +2.7 / -11.7 |
| 5 |  |  |  |  | 23 | +2.7 / -4.7 |
| 6 |  |  |  |  | 23 | +2.7 / -2.7 |
| 7 |  |  |  |  | 23 | +2.7/-24.7 |
| 8 |  |  |  |  | 23 | +2.7 / -8.2 |
| 9 |  |  |  |  | 23 | +2.7 / -6.2 |
| 10 |  |  |  |  | 23 | +2.7 / -6.2 |
| 11 |  |  |  |  | 23 | +2.7 / -11.7 |
| 12 |  |  |  |  | 23 | +2.7 / -15.7 |
| 13 |  |  |  |  | 23 | +2.7 / -2.7 |
| 14 |  |  |  |  | 23 | +2.7/-24.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1.5-3: Test requirement (network signalled value "CA_NS_03")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -15.7 |
| 2 |  |  |  |  | 23 | +2.7 / -8.2 |
| 3 |  |  |  |  | 23 | +2.7 / -4.7 |
| 4 |  |  |  |  | 23 | +2.7 / -9.7 |
| 5 |  |  |  |  | 23 | +2.7 / -13.7 |
| 6 |  |  |  |  | 23 | +2.7 / -11.7 |
| 7 |  |  |  |  | 23 | +2.7 / -2.7 |
| 8 |  |  |  |  | 23 | +2.7/-25.2 |
| 9 |  |  |  |  | 23 | +2.7 / -15.7 |
| 10 |  |  |  |  | 23 | +2.7 / -11.7 |
| 11 |  |  |  |  | 23 | +2.7 / -3.7 |
| 12 |  |  |  |  | 23 | +2.7 / -8.2 |
| 13 |  |  |  |  | 23 | +2.7 / -12.7 |
| 14 |  |  |  |  | 23 | +2.7 / -15.7 |
| 15 |  |  |  |  | 23 | +2.7 / -2.7 |
| 16 |  |  |  |  | 23 | +2.7/-25.2 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1.5-4: Test requirement (network signalled value "CA_NS_04")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -8.2 |
| 2 |  |  |  |  | 23 | +2.7 / -6.2 |
| 3 |  |  |  |  | 23 | +2.7 / -8.2 |
| 4 |  |  |  |  | 23 | +2.7 / -18.7 |
| 5 |  |  |  |  | 23 | +2.7 / -8.2 |
| 6 |  |  |  |  | 23 | +2.7 / -8.2 |
| 7 |  |  |  |  | 23 | +2.7 / -18.7 |
| 8 |  |  |  |  | 23 | +2.7 / -8.2 |
| 9 |  |  |  |  | 23 | +2.7 / -6.2 |
| 10 |  |  |  |  | 23 | +2.7 / -8.2 |
| 11 |  |  |  |  | 23 | +2.7 / -18.7 |
| 12 |  |  |  |  | 23 | +2.7 / -6.2 |
| 13 |  |  |  |  | 23 | +2.7 / -6.2 |
| 14 |  |  |  |  | 23 | +2.7 / -8.2 |
| 15 |  |  |  |  | 23 | +2.7 / -8.2 |
| 16 |  |  |  |  | 23 | +2.7 / -18.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1.5-5: Test requirement (network signalled value "CA_NS_05")

| Configuration ID | Test Freq. | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Low, High |  |  |  |  | 23 | +2.7 / -4.7 |
| 2 | Low, High |  |  |  |  | 23 | +2.7 / -9.7 |
| 3 | Low, High |  |  |  |  | 23 | +2.7 / -9.7 |
| 4 | Low, High |  |  |  |  | 23 | +2.7 / -9.7 |
| 5 | Low, High |  |  |  |  | 23 | +2.7 / -11.7 |
| 6 | Low, High |  |  |  |  | 23 | +2.7 / -24.2 |
| 7 | Low, High |  |  |  |  | 23 | +2.7 / -9.7 |
| 8 | Low, High |  |  |  |  | 23 | +2.7 / -4.7 |
| 9 | Low, High |  |  |  |  | 23 | +2.7 / -11.7 |
| 10 | Low, High |  |  |  |  | 23 | +2.7 / -17.7 |
| 11 | Low, High |  |  |  |  | 23 | +2.7 / -24.2 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |  |

Table 6.2.4A.1.5-6: Test requirement (network signalled value "CA_NS_06")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 (Low Range) |  |  |  |  | 23 | +2.7 / -12.2 |
| 1 (High Range) |  |  |  |  | 23 | +2.7 / -9.7 |
| 2 |  |  |  |  | 23 | +2.7 / -4.7 |
| 3 |  |  |  |  | 23 | +2.7 / -4.7 |
| 4 |  |  |  |  | 23 | +2.7 / -11.7 |
| 5 |  |  |  |  | 23 | +2.7 / -15.7 |
| 6 |  |  |  |  | 23 | +2.7 / -26.7 |
| 7 (Low Range) |  |  |  |  | 23 | +2.7 / -11.2 |
| 7 (High Range) |  |  |  |  | 23 | +2.7 / -8.2 |
| 8 |  |  |  |  | 23 | +2.7 / -4.7 |
| 9 |  |  |  |  | 23 | +2.7 / -6.2 |
| 10 |  |  |  |  | 23 | +2.7 / -9.7 |
| 11 |  |  |  |  | 23 | +2.7 / -15.7 |
| 12 |  |  |  |  | 23 | +2.7 / -26.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1.5-7: Test requirement (network signalled value "CA_NS_07")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -8.2 |
| 2 |  |  |  |  | 23 | +2.7 / -11.7 |
| 3 |  |  |  |  | 23 | +2.7 / -6.2 |
| 4 |  |  |  |  | 23 | +2.7 / -6.2 |
| 5 |  |  |  |  | 23 | +2.7 / -28.7 |
| 6 |  |  |  |  | 23 | +2.7 / -17.7 |
| 7 |  |  |  |  | 23 | +2.7 / -6.2 |
| 8 |  |  |  |  | 23 | +2.7 / -11.7 |
| 9 |  |  |  |  | 23 | +2.7 / -9.7 |
| 10 |  |  |  |  | 23 | +2.7 / -4.7 |
| 11 |  |  |  |  | 23 | +2.7 / -9.7 |
| 12 |  |  |  |  | 23 | +2.7 / -28.7 |
| 13 |  |  |  |  | 23 | +2.7 / -17.7 |
| 14 |  |  |  |  | 23 | +2.7 / -6.2 |
| 15 |  |  |  |  | 23 | +2.7 / -12.7 |
| 16 |  |  |  |  | 23 | +2.7 / -4.7 |
| 17 |  |  |  |  | 23 | +2.7 / -9.7 |
| 18 |  |  |  |  | 23 | +2.7 / -28.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

#### 6.2.4A.1_1	Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM

Editor's notes:

- Minimum requirement for CA_NS_08 for this test case is undefined in TS 36.101.

- For a transition period until RAN5#83, this test case in version 15.3.1 of 36.521-1 shall be used. This is to ensure no test coverage is lost before the UL 256QAM test case becomes available.

##### 6.2.4A.1_1.1	Test purpose

Same test purpose as in clause 6.2.4A.1.1.

##### 6.2.4A.1_1.2	Test applicability

The requirements of this test apply in test case 6.6.2.2A.1_1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for network signalled value CA_NS_04 to all types of E-UTRA UE from release 13 and forward that support intra-band contiguous DL CA, UL CA and UL 64QAM but do not support UL 256QAM.

The requirements of this test apply in test case 6.6.3.3A.1_1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for network signalled values CA_NS_01, CA_NS_02 and CA_NS_03, CA_NS_05, CA_NS_06, CA_NS_07, CA_NS_08 to all types of E-UTRA UE from release 13 and forward that support intra-band contiguous DL CA, UL CA and UL 64QAM but do not support UL 256QAM.

NOTE:	As a result TC 6.2.4A.1_1 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

##### 6.2.4A.1_1.3	Minimum conformance requirements

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the CA Power Class as specified in Table 6.2.2A.0-2.

If for intra-band carrier aggregation the UE is configured for transmissions on a single serving cell, then subclauses 6.2.3 and 6.2 4 apply with the Network Signalling value indicated by the field additionalSpectrumEmission.

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2.4A.1_1.3-1 is allowed for all serving cells of the applicable uplink CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10. Then clause 6.2.3A.1.3 does not apply, i.e. the carrier aggregation MPR = 0 dB, unless the value indicated is CA_NS_31. For uplink 64QAM, the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in Table 6.2.3A.1_1.3-1 and A-MPR requirements specified in Table 6.2.4A.1_1.3-1.

For the UE maximum output power modified by A-MPR specified in table 6.2.4A.1_1.3-1, the power limits specified in subclause 6.2.5A.1.3 apply.

Table 6.2.4A.1_1.3-1: Additional Maximum Power Reduction (A-MPR) for intra-band contiguous CA

| CA Network Signalling value | Requirements (subclause) | Uplink CA Configuration | A-MPR [dB] (subclause) |
| --- | --- | --- | --- |
| CA_NS_01 | 6.6.3.3A.1.3.1 | CA_1C | 6.2.4A.1_1.3.1 |
| CA_NS_02 | 6.6.3.3A.1.3.2 | CA_1C | 6.2.4A.1_1.3.2 |
| CA_NS_03 | 6.6.3.3A.1.3.3 | CA_1C | 6.2.4A.1_1.3.3 |
| CA_NS_04 | 6.6.2.2A.1.3.1 | CA_41C | 6.2.4A.1_1.3.4 |
| CA_NS_05 | 6.6.3.3A.1.3.4 | CA_38C | 6.2.4A.1_1.3.5 |
| CA_NS_06 | 6.6.3.3A.1.3.5 | CA_7C | 6.2.4A.1_1.3.6 |
| CA_NS_07 | 6.6.3.3A.1.3.6 | CA_39C | 6.2.4A.1_1.3.7 |
| CA_NS_08 | 6.6.3.3A.1.3.7 | CA_42C | 6.2.4A.1_1.3.8 |
| … |  |  |  |
| CA_NS_31 | NOTE 1 | Table 5.4.2A.1-1 (NOTE 1) | N/A |
| CA_NS_32 | Reserved |  |  |
| NOTE 1: Applicable for uplink CA configurations listed in Table 5.4.2A.1-1 for which none of the additional requirements in subclauses 6.6.3.3A or 6.6.2.2A apply. NOTE 2: The index of the sequence CA_NS corresponds to the value of additionalSpectrumEmissionSCell-r10. |  |  |  |

For PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For intra-band carrier aggregation, the A-MPR is evaluated per slot and given by the maximum value taken over the transmission(s) on all component carriers within the slot; the maximum A-MPR over the two slots is then applied for the entire subframe.

6.2.4A.1_1.3.1	A-MPR for CA_NS_01 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_01 the allowed maximum output power reduction applied to transmissions on the PCC and the SCC for contiguously aggregated signals is specified in table 6.2.4A.1_1.3.1-1.

Table 6.2.4A.1_1.3.1-1: Contiguous allocation A-MPR for CA_NS_01

| CA_1C: CA_NS_01 | RBstart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for 64-QAM [dB] |
| --- | --- | --- | --- | --- |
| 100 RB / 100 RB | 0 – 23 and 176 - 199 | > 0 | N/A | ≤ 12.0 |
|  | 24 – 105 | > 64 | N/A | ≤ 6.0 |
|  | 106 – 175 | N/A | > 175 | ≤ 5.0 |
| 75 RB / 75 RB | 0 – 6 and 143 – 149 | 0 < LCRB ≤ 10 | N/A | ≤ 11.0 |
|  |  | > 10 | N/A | ≤ 6.0 |
|  | 7 – 90 | > 44 | N/A | ≤ 5.0 |
|  | 91 – 142 | N/A | > 142 | ≤ 2.0 |
| NOTE 1: RB_start indicates the lowest RB index of transmitted resource blocks NOTE 2: L_CRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |

If the UE is configured to CA_1C and it receives IE CA_NS_01 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows:

MA = -22.5 A + 17		; 0 ≤ A < 0.20

-11.0 A + 14.7			; 0.20 ≤ A < 0.70

-1.7 A + 8.2			 ; 0.70 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg

The normative reference for requirement is in TS 36.101 [2] clause 6.2.4A.

6.2.4A.1_1.3.2	A-MPR for CA_NS_02 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_02 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1_1.3.2-1.

Table 6.2.4A.1_1.3.2-1: Contiguous allocation A-MPR for CA_NS_02

| CA_1C: CA_NS_02 | RBend | LCRB [RBs] | A-MPR for 64-QAM [dB] |
| --- | --- | --- | --- |
| 100 RB / 100 RB | 0 –20 | > 0 | ≤ 4 dB |
|  | 21 – 46 | > 0 | ≤ 3 dB |
|  | 47 – 99 | > RBend – 20 | ≤ 3 dB |
|  | 100 – 184 | > 75 | ≤ 6 dB |
|  | 185 – 199 | > 0 | ≤ 10 dB |
| 75 RB / 75 RB | 0 – 48 | > 0 | ≤ 2 dB |
|  | 49 – 80 | > RBend – 20 | ≤ 3 dB |
|  | 81 – 129 | > 60 | ≤ 5 dB |
|  | 130 – 149 | > 85 | ≤ 6 dB |
|  | 130 – 149 | 1 – 84 | ≤ 2 dB |

If the UE is configured to CA_1C and it receives IE CA_NS_02 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows:

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

[MA = -22.5 A + 17		; 0 ≤ A < 0.20

-11.0 A + 14.7			; 0.20 ≤ A < 0.70

-1.7 A + 8.2			 ; 0.70 ≤ A ≤ 1]

Where A = NRB_alloc / NRB_agg.

6.2.4A.1_1.3.3	A-MPR for CA_NS_03 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_03 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1_1.3.3-1.

Table 6.2.4A.1_1.3.3-1: Contiguous allocation A-MPR for CA_NS_03

| CA_1C: CA_NS_03 | RBend | LCRB [RBs] | A-MPR for 64-QAM [dB] |
| --- | --- | --- | --- |
| 100 RB / 100 RB | 0 – 26 | > 0 | ≤ 10 dB |
|  | 27 – 63 | ≥ RBend – 27 | ≤ 6 dB |
|  | 27 – 63 | < RBend – 27 | ≤ 1 dB |
|  | 64 – 100 | > RBend – 20 | ≤ 4 dB |
|  | 101 – 171 | > 68 | ≤ 7 dB |
|  | 172 – 199 | > 0 | ≤ 10 dB |
| 75 RB / 75 RB | 0 – 20 | > 0 | ≤ 10 dB |
|  | 21 – 45 | > 0 | ≤ 4 dB |
|  | 46 – 75 | > RBend – 13 | ≤ 2 dB |
|  | 76 – 95 | > 45 | ≤ 5 dB |
|  | 96 – 149 | > 43 | ≤ 8 dB |
|  | 120 – 149 | 1 – 43 | ≤ 6 dB |

If the UE is configured to CA_1C and it receives IE CA_NS_03 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows:

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

[MA = -23.33A + 17.5		; 0 ≤ A < 0.15

-7.65A + 15.15			; 0.15 ≤ A ≤ 1]

Where A = NRB_alloc / NRB_agg.

6.2.4A.1_1.3.4	A-MPR for CA_NS_04 for CA_41C

If the UE is configured to CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1_1.3.4-1.

Table 6.2.4A.1.3.4-1: Contiguous Allocation A-MPR for CA_NS_04

| CA Bandwidth Class C | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for 64-QAM [dB] |
| --- | --- | --- | --- | --- |
| 50RB / 100 RB | 0 – 44 and 105 – 149 | >0 | N/A | ≤4dB |
|  | 45 – 104 | N/A | >105 | ≤4dB |
| 75 RB / 75 RB | 0 – 44 and 105 – 149 | >0 | N/A | ≤4dB |
|  | 45 – 104 | N/A | >105 | ≤4dB |
| 100 RB / 75 RB | 0 – 49 and 125 – 174 | >0 | N/A | ≤4dB |
|  | 50 - 124 | N/A | >125 | ≤4dB |
| 100 RB / 100 RB | 0 – 59 and 140 – 199 | >0 | N/A | ≤4dB |
|  | 60– 139 | N/A | >140 | ≤4dB |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |

If the UE is configured to CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA 	= 	10.5,					0≤ A < 0.05

= -50.0A + 13.00,		0.05≤ A < 0.15

= -4.0A + 6.10,			0.15≤ A < 0.40

= -0.83A + 4.83,				0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

###### 6.2.4A.1_1.3.5	A-MPR for CA_NS_05 for CA_38C

If the UE is configured to CA_38C and it receives IE CA_NS_05 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1_1.3.5-1.

Table 6.2.4A.1_1.3.5-1: Contiguous Allocation A-MPR for CA_NS_05

| CA_38C | RBend | LCRB [RBs] | A-MPR for 64-QAM [dB] |
| --- | --- | --- | --- |
| 100RB/100RB | 0 – 12 | >0 | ≤ 5 dB |
|  | 13 – 79 | > RBend – 13 | ≤ 2 dB |
|  | 80 – 180 | >60 | ≤ 6 dB |
|  | 181 – 199 | > 0 | ≤ 11 dB |
| 75RB/75RB | 0 – 70 | > max(0, RBend -10) | ≤ 2 dB |
|  | 71- 108 | > 60 | ≤ 5 dB |
|  | 109 – 140 | >0 | ≤ 5 dB |
|  | 140 – 149 | ≤ 70 | ≤ 2 dB |
|  | 140 – 149 | >70 | ≤ 6 dB |
| NOTE 1: RBend indicates the highest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |

If the UE is configured to CA_38C and it receives IE CA_NS_05 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = -14.17 A + 16.50		; 0 ≤ A < 0.60

-2.50 A + 9.50			; 0.60 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

###### 6.2.4A.1_1.3.6	A-MPR for CA_NS_06 for CA_7C

If the UE is configured to CA_7C and it receives IE CA_NS_06 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1_1.3.6-1.

Table 6.2.4A.1_1.3.6-1: Contiguous Allocation A-MPR for CA_NS_06

| CA Bandwidth Class C | RBend | LCRB [RBs] | A-MPR for 64-QAM [dB] |
| --- | --- | --- | --- |
| 100RB/100RB | 0 –22 | >0 | ≤ 4 dB |
|  | 23 – 99 | > max(0,RBend – 25) | ≤ 2 dB |
|  | 100 – 142 | > 75 | ≤ 3 dB |
|  | 143 – 177 | >70 | ≤ 5 dB |
|  | 178 – 199 | > 0 | ≤ 10 dB |
| 75RB/75RB | 0 – 7 | >0 | ≤ 5 dB |
|  | 8- 74 | > max(0,RBend – 10) | ≤ 2 dB |
|  | 75 – 109 | >64 | ≤ 2 dB |
|  | 110 – 144 | >35 | ≤ 6 dB |
|  | 145 – 149 | >0 | ≤ 10 dB |
| 50RB/100RB and 100RB/50RB | 0 – 10 | > 0 | ≤ 5 dB |
|  | 11 – 75 | > max(0, RB_End – 25) | ≤ 2 dB |
|  | 76 – 103 | > 50 | ≤ 3 dB |
|  | 104 – 144 | > 25 | ≤ 6 dB |
|  | 145 – 149 | > 0 | ≤ 10 dB |
| 75RB/100RB and 100RB/75RB | 0 – 15 | > 0 | ≤ 5 dB |
|  | 16 – 75 | > max(0, RB_End – 15) | ≤ 2 dB |
|  | 76 – 120 | > 50 | ≤ 3 dB |
|  | 121 – 160 | > 50 | ≤ 6 dB |
|  | 161 – 174 | > 0 | ≤ 10 dB |

If the UE is configured to CA_7C and it receives IE CA_NS_06 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows:

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =	-23.33A + 17.5		; 0 ≤ A < 0.15

-7.65A + 15.15			; 0.15 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

###### 6.2.4A.1_1.3.7	A-MPR for CA_NS_07 for CA_39C

If the UE is configured to CA_39C and it receives IE CA_NS_07 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1_1.3.7-1.

Table 6.2.4A.1_1.3.7-1: Contiguous Allocation A-MPR for CA_NS_07

| CA_39C: CA_NS_07 | RBStart | LCRB [RBs] | A-MPR for 64-QAM [dB] |
| --- | --- | --- | --- |
| 75 RB / 100 RB and 100 RB / 75 RB | 0 – 13 | > 0 | ≤ 11 |
|  | 14 – 50 | ≤ 60 | ≤ 3 |
|  | 14 – 100 | > 60 | ≤ 7 |
|  | 101 – 155 | > max(155 - RBstart , 0) | ≤ 2 |
|  | 156 – 174 | > 0 | ≤ 5 |
| 50 RB / 100 RB and 100 RB / 50 RB | 0 – 5 | > 0 | ≤ 11 |
|  | 6 – 42 | ≤ 25 | ≤ 3 |
|  |  | > 25 | ≤ 6 |
|  | 43 – 80 | > 50 | ≤ 5 |
|  | 81 – 138 | > 20 | ≤ 2 |
|  | 139 – 149 | > 0 | ≤ 5 |
| 25 RB / 100 RB and 100 RB / 25 RB | 0 – 32 | ≥ 84 | ≤ 6 |
|  |  | < 84 | ≤ 4 |
|  | 33 – 60 | > 50 | ≤ 3 |
|  | 61 – 124 | > 20 | ≤ 3 |

If the UE is configured to CA_39C and it receives IE CA_NS_07 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = -16.25A + 21				; 0 ≤ A < 0.80

-2.50 A + 10.00				; 0.80 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg

###### 6.2.4A.1_1.3.8	A-MPR for CA_NS_08 for CA_42C

If the UE is configured to CA_42C and it receives IE CA_NS_08 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1_1.3.8-1.

Table 6.2.4A.1_1.3.8-1: Contiguous Allocation A-MPR for CA_NS_08

| CA_42C: CA_NS_08 | RBStart | LCRB [RBs] | A-MPR for 64-QAM[dB] |
| --- | --- | --- | --- |
| 100RB/100RB | - | - | TBD |
| 75 RB / 100 RB and 100 RB / 75 RB | - | - | TBD |
| 50 RB / 100 RB and 100 RB / 50 RB | - | - | TBD |
| 25 RB / 100 RB and 100 RB / 25 RB | - | - | TBD |

##### 6.2.4A.1_1.4	Test description

6.2.4A.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.2.4A.1_1.4.1-1 to 6.2.4A.1_1.4.1-7. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4A.1_1.4.1-1: Test Configuration Table (network signalled value "CA_NS_01")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1_1.3.1-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | 64QAM | 1 |  | P_1@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | 64QAM | 150 |  | P_75@0 | S_75@0 |  |  |
| 3 | 75 | 75 |  | 64QAM | 45 |  | P_45@7 | S_0@0 |  |  |
| 4 | 75 | 75 |  | 64QAM | 8 |  | P_0@0 | S_8@67 |  |  |
| 5 | 75 | 75 |  | 64QAM | 129 |  | P_75@0 | S_54@0 |  |  |
| 6 | 75 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 7 | 100 | 100 |  | 64QAM | 200 |  | P_100@0 | S_100@0 |  |  |
| 8 | 100 | 100 |  | 64QAM | 1 |  | P_1@0 | S_0@0 |  |  |
| 9 | 100 | 100 |  | 64QAM | 175 |  | P_75@25 | S_100@0 |  |  |
| 10 | 100 | 100 |  | 64QAM | 25 |  | P_0@0 | S_25@75 |  |  |
| 11 | 100 | 100 |  | 64QAM | 64 |  | P_64@24 | S_0@0 |  |  |
| 12 | 100 | 100 |  | 64QAM | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1_1.4.1-2: Test Configuration Table (network signalled value "CA_NS_02")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1_1.3.2-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | 64QAM | 1 |  | P_1@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | 64QAM | 75 |  | P_75@0 | S_0@0 |  |  |
| 3 | 75 | 75 |  | 64QAM | 129 |  | P_75@0 | S_54@0 |  |  |
| 4 | 75 | 75 |  | 64QAM | 129 |  | P_54@21 | S_75@0 |  |  |
| 5 | 75 | 75 |  | 64QAM | 1 |  | P_0@0 | S_1@74 |  |  |
| 6 | 75 | 75 |  | 64QAM | 1 |  | P_0@0 | S_1@54 |  |  |
| 7 | 75 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 8 | 100 | 100 |  | 64QAM | 1 |  | P_1@0 | S_0@0 |  |  |
| 9 | 100 | 100 |  | 64QAM | 1 |  | P_1@21 | S_0@0 |  |  |
| 10 | 100 | 100 |  | 64QAM | 90 |  | P_90@0 | S_0@0 |  |  |
| 11 | 100 | 100 |  | 64QAM | 180 |  | P_100@0 | S_80@0 |  |  |
| 12 | 100 | 100 |  | 64QAM | 1 |  | P_0@0 | S_1@99 |  |  |
| 13 | 100 | 100 |  | 64QAM | 1 |  | P_0@0 | S_1@83 |  |  |
| 14 | 100 | 100 |  | 64QAM | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1_1.4.1-3: Test Configuration Table (network signalled value "CA_NS_03")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1_1.3.3-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | 64QAM | 1 |  | P_1@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | 64QAM | 1 |  | P_1@21 | S_0@0 |  |  |
| 3 | 75 | 75 |  | 64QAM | 75 |  | P_75@0 | S_0@0 |  |  |
| 4 | 75 | 75 |  | 64QAM | 90 |  | P_75@0 | S_15@0 |  |  |
| 5 | 75 | 75 |  | 64QAM | 150 |  | P_75@0 | S_75@0 |  |  |
| 6 | 75 | 75 |  | 64QAM | 1 |  | P_0@0 | S_1@74 |  |  |
| 7 | 75 | 75 |  | 64QAM | 1 |  | P_0@0 | S_1@44 |  |  |
| 8 | 75 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 9 | 100 | 100 |  | 64QAM | 1 |  | P_1@0 | S_0@0 |  |  |
| 10 | 100 | 100 |  | 64QAM | 60 |  | P_60@0 | S_0@0 |  |  |
| 11 | 100 | 100 |  | 64QAM | 1 |  | P_1@63 | S_0@0 |  |  |
| 12 | 100 | 100 |  | 64QAM | 90 |  | P_90@0 | S_0@0 |  |  |
| 13 | 100 | 100 |  | 64QAM | 164 |  | P_100@0 | S_64@0 |  |  |
| 14 | 100 | 100 |  | 64QAM | 1 |  | P_0@0 | S_1@99 |  |  |
| 15 | 100 | 100 |  | 64QAM | 1 |  | P_0@0 | S_1@70 |  |  |
| 16 | 100 | 100 |  | 64QAM | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1_1.4.1-4: Test Configuration Table (network signalled value "CA_NS_04")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1_1.3.4-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 50 | N/A | 64QAM | 10 |  | P_10@20 | S_0@0 |  |  |
| 2 | 100 | 50 |  | 64QAM | 60 |  | P_50@50 | S_10@0 |  |  |
| 3 | 100 | 50 |  | 64QAM | 15 |  | P_0@0 | S_15@0 |  |  |
| 4 | 100 | 50 |  | 64QAM | 2 |  | P_1@0 | S_1@49 |  |  |
| 5 | 75 | 75 |  | 64QAM | 10 |  | P_10@20 | S_0@0 |  |  |
| 6 | 75 | 75 |  | 64QAM | 75 |  | P_30@45 | S_45@0 |  |  |
| 7 | 75 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 8 | 100 | 75 |  | 64QAM | 10 |  | P_10@20 | S_0@0 |  |  |
| 9 | 100 | 75 |  | 64QAM | 80 |  | P_50@50 | S_30@0 |  |  |
| 10 | 100 | 75 |  | 64QAM | 20 |  | P_0@0 | S_20@15 |  |  |
| 11 | 100 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 12 | 100 | 100 |  | 64QAM | 10 |  | P_10@25 | S_0@0 |  |  |
| 13 | 100 | 100 |  | 64QAM | 90 |  | P_40@60 | S_50@0 |  |  |
| 14 | 100 | 100 |  | 64QAM | 15 |  | P_0@0 | S_15@40 |  |  |
| 15 | 100 | 100 |  | 64QAM | 20 |  | P_0@0 | S_20@30 |  |  |
| 16 | 100 | 100 |  | 64QAM | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1_1.4.1-5: Test Configuration Table (network signalled value "CA_NS_05")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1_1.3.5-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | 64QAM | 40 |  | P_40@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | 64QAM | 80 |  | P_50@25 | S_30@0 |  |  |
| 3 | 75 | 75 |  | 64QAM | 60 |  | P_10@65 | S_50@0 |  |  |
| 4 | 75 | 75 |  | 64QAM | 65 |  | P_1@74 | S_64@0 |  |  |
| 5 | 75 | 75 |  | 64QAM | 92 |  | P_20@55 | S_72@0 |  |  |
| 6 | 75 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 7 | 100 | 100 |  | 64QAM | 8 |  | P_8@0 | S_0@0 |  |  |
| 8 | 100 | 100 |  | 64QAM | 40 |  | P_40@0 | S_0@0 |  |  |
| 9 | 100 | 100 |  | 64QAM | 80 |  | P_50@50 | S_30@0 |  |  |
| 10 | 100 | 100 |  | 64QAM | 150 |  | P_60@40 | S_90@0 |  |  |
| 11 | 100 | 100 |  | 64QAM | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1_1.4.1-6: Test Configuration Table (network signalled value "CA_NS_06")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | As in Table 6.2.4A.1_1.3.6-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 75 | 75 | N/A | 64QAM | 5 |  | P_5@0 | S_0@0 |  |  |
| 2 | 75 | 75 |  | 64QAM | 45 |  | P_45@0 | S_0@0 |  |  |
| 3 | 75 | 75 |  | 64QAM | 75 |  | P_60@15 | S_15@0 |  |  |
| 4 | 75 | 75 |  | 64QAM | 60 |  | P_10@65 | S_50@0 |  |  |
| 5 | 75 | 75 |  | 64QAM | 90 |  | P_18@57 | S_72@0 |  |  |
| 6 | 75 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 7 | 100 | 100 |  | 64QAM | 10 |  | P_10@0 | S_0@0 |  |  |
| 8 | 100 | 100 |  | 64QAM | 30 |  | P_30@0 | S_0@0 |  |  |
| 9 | 100 | 100 |  | 64QAM | 100 |  | P_75@25 | S_25@0 |  |  |
| 10 | 100 | 100 |  | 64QAM | 90 |  | P_40@60 | S_50@0 |  |  |
| 11 | 100 | 100 |  | 64QAM | 96 |  | P_15@85 | S_81@0 |  |  |
| 12 | 100 | 100 |  | 64QAM | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1_1.4.1-7: Test Configuration Table (network signalled value "CA_NS_07")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | As in Table 6.2.4A.1_1.3.7-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 25 | N/A | 64QAM | 1 |  | P_1@0 | S_0@0 |  |  |
| 2 | 100 | 25 |  | 64QAM | 85 |  | P_80@20 | S_5@0 |  |  |
| 3 | 100 | 25 |  | 64QAM | 65 |  | P_60@40 | S_5@0 |  |  |
| 4 | 100 | 25 |  | 64QAM | 35 |  | P_30@70 | S_5@0 |  |  |
| 5 | 100 | 25 |  | 64QAM | 2 |  | P_1@0 | S_1@24 |  |  |
| 6 | 100 | 50 |  | 64QAM | 1 |  | P_1@1 | S_0@0 |  |  |
| 7 | 100 | 50 |  | 64QAM | 10 |  | P_10@30 | S_0@0 |  |  |
| 8 | 100 | 50 |  | 64QAM | 30 |  | P_30@30 | S_0@0 |  |  |
| 9 | 100 | 50 |  | 64QAM | 60 |  | P_40@60 | S_20@0 |  |  |
| 10 | 100 | 50 |  | 64QAM | 30 |  | P_10@90 | S_20@0 |  |  |
| 11 | 100 | 50 |  | 64QAM | 5 |  | P_0@0 | S_5@45 |  |  |
| 12 | 100 | 50 |  | 64QAM | 2 |  | P_1@0 | S_1@49 |  |  |
| 13 | 100 | 75 |  | 64QAM | 1 |  | P_1@1 | S_0@0 |  |  |
| 14 | 100 | 75 |  | 64QAM | 40 |  | P_40@20 | S_0@0 |  |  |
| 15 | 100 | 75 |  | 64QAM | 85 |  | P_80@20 | S_5@0 |  |  |
| 16 | 100 | 75 |  | 64QAM | 40 |  | P_0@0 | S_40@20 |  |  |
| 17 | 100 | 75 |  | 64QAM | 15 |  | P_0@0 | S_15@60 |  |  |
| 18 | 100 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4A.1_1.4.1-8: Test Configuration Table (network signalled value "CA_NS_08")

[Table 6.2.4A.1_1.4.1-8 TBD]

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to the applicable table from Table 6.2.4A.1_1.4.1-1 to 6.2.4A.1_1.4.1-7.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4A.1_1.4.3.

6.2.4A.1_1.4.2	Test procedure

Same test procedure as in clause 6.2.4A.1.4.2

6.2.4A.1_1.4.3	Message contents

Same message contents as in clause 6.2.4A.1.4.3.

##### 6.2.4A.1_1.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4A.1_1.5-1 to Table 6.2.4A.1_1.5-7. For uplink QPSK or 16QAM, the allowed maximum output power reduction is specified in Table 6.2.4A.1_1.3-1 and clause 6.2.3A.1 does not apply, i.e. carrier aggregation MPR = 0. For uplink 64QAM, the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in Table 6.2.3A.1_1.3-1 and A-MPR requirements specified in Table 6.2.4A.1_1.3-1. For the UE maximum output power modified by MPR specified in Table 6.2.3A.1_1.3-1 or A-MPR specified in Table 6.2.4A.1_1.3-1, the power limits specified in Table 6.2.5A.1.3-1 apply.

Table 6.2.4A.1_1.5-1: Test requirement (network signalled value "CA_NS_01")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -17.7 |
| 2 |  |  |  |  | 23 | +2.7 / -11.7 |
| 3 |  |  |  |  | 23 | +2.7 / -9.7 |
| 4 |  |  |  |  | 23 | +2.7 / -4.7 |
| 5 |  |  |  |  | 23 | +2.7 / -11.7 |
| 6 |  |  |  |  | 23 | +2.7 / -24.7 |
| 7 |  |  |  |  | 23 | +2.7 / -18.7 |
| 8 |  |  |  |  | 23 | +2.7 / -18.7 |
| 9 |  |  |  |  | 23 | +2.7 / -11.7 |
| 10 |  |  |  |  | 23 | +2.7 / -9.7 |
| 11 |  |  |  |  | 23 | +2.7 / -6.2 |
| 12 |  |  |  |  | 23 | +2.7 / -24.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1_1.5-2: Test requirement (network signalled value "CA_NS_02")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -4.7 |
| 2 |  |  |  |  | 23 | +2.7 / -6.2 |
| 3 |  |  |  |  | 23 | +2.7 / -9.7 |
| 4 |  |  |  |  | 23 | +2.7 / -11.7 |
| 5 |  |  |  |  | 23 | +2.7 / -4.7 |
| 6 |  |  |  |  | 23 | +2.7 / -4.7 |
| 7 |  |  |  |  | 23 | +2.7 / -24.7 |
| 8 |  |  |  |  | 23 | +2.7 / -8.2 |
| 9 |  |  |  |  | 23 | +2.7 / -6.2 |
| 10 |  |  |  |  | 23 | +2.7 / -6.2 |
| 11 |  |  |  |  | 23 | +2.7 / -11.7 |
| 12 |  |  |  |  | 23 | +2.7 / -15.7 |
| 13 |  |  |  |  | 23 | +2.7 / -4.7 |
| 14 |  |  |  |  | 23 | +2.7 / -24.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1_1.5-3: Test requirement (network signalled value "CA_NS_03")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -15.7 |
| 2 |  |  |  |  | 23 | +2.7 / -8.2 |
| 3 |  |  |  |  | 23 | +2.7 / -6.2 |
| 4 |  |  |  |  | 23 | +2.7 / -9.7 |
| 5 |  |  |  |  | 23 | +2.7 / -13.7 |
| 6 |  |  |  |  | 23 | +2.7 / -11.7 |
| 7 |  |  |  |  | 23 | +2.7 / -4.7 |
| 8 |  |  |  |  | 23 | +2.7 / -25.2 |
| 9 |  |  |  |  | 23 | +2.7 / -15.7 |
| 10 |  |  |  |  | 23 | +2.7 / -11.7 |
| 11 |  |  |  |  | 23 | +2.7 / -4.7 |
| 12 |  |  |  |  | 23 | +2.7 / -8.2 |
| 13 |  |  |  |  | 23 | +2.7 / -12.7 |
| 14 |  |  |  |  | 23 | +2.7 / -15.7 |
| 15 |  |  |  |  | 23 | +2.7 / -4.7 |
| 16 |  |  |  |  | 23 | +2.7 / -25.2 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1_1.5-4: Test requirement (network signalled value "CA_NS_04")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -8.2 |
| 2 |  |  |  |  | 23 | +2.7 / -8.2 |
| 3 |  |  |  |  | 23 | +2.7 / -8.2 |
| 4 |  |  |  |  | 23 | +2.7 / -18.7 |
| 5 |  |  |  |  | 23 | +2.7 / -8.2 |
| 6 |  |  |  |  | 23 | +2.7 / -8.2 |
| 7 |  |  |  |  | 23 | +2.7 / -18.7 |
| 8 |  |  |  |  | 23 | +2.7 / -8.2 |
| 9 |  |  |  |  | 23 | +2.7 / -8.2 |
| 10 |  |  |  |  | 23 | +2.7 / -8.2 |
| 11 |  |  |  |  | 23 | +2.7 / -18.7 |
| 12 |  |  |  |  | 23 | +2.7 / -8.2 |
| 13 |  |  |  |  | 23 | +2.7 / -8.2 |
| 14 |  |  |  |  | 23 | +2.7 / -8.2 |
| 15 |  |  |  |  | 23 | +2.7 / -8.2 |
| 16 |  |  |  |  | 23 | +2.7 / -18.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1_1.5-5: Test requirement (network signalled value "CA_NS_05")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -6.2 |
| 2 |  |  |  |  | 23 | +2.7 / -9.7 |
| 3 |  |  |  |  | 23 | +2.7 / -9.7 |
| 4 |  |  |  |  | 23 | +2.7 / -9.7 |
| 5 |  |  |  |  | 23 | +2.7 / -11.7 |
| 6 |  |  |  |  | 23 | +2.7 / -24.2 |
| 7 |  |  |  |  | 23 | +2.7 / -9.7 |
| 8 |  |  |  |  | 23 | +2.7 / -6.2 |
| 9 |  |  |  |  | 23 | +2.7 / -11.7 |
| 10 |  |  |  |  | 23 | +2.7 / -17.7 |
| 11 |  |  |  |  | 23 | +2.7 / -24.2 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1_1.5-6: Test requirement (network signalled value "CA_NS_06")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -12.2 |
| 2 |  |  |  |  | 23 | +2.7 / -6.2 |
| 3 |  |  |  |  | 23 | +2.7 / -6.2 |
| 4 |  |  |  |  | 23 | +2.7 / -11.7 |
| 5 |  |  |  |  | 23 | +2.7 / -15.7 |
| 6 |  |  |  |  | 23 | +2.7 / -26.7 |
| 7 |  |  |  |  | 23 | +2.7 / -11.2 |
| 8 |  |  |  |  | 23 | +2.7 / -6.2 |
| 9 |  |  |  |  | 23 | +2.7 / -6.2 |
| 10 |  |  |  |  | 23 | +2.7 / -9.7 |
| 11 |  |  |  |  | 23 | +2.7 / -15.7 |
| 12 |  |  |  |  | 23 | +2.7 / -26.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.4A.1_1.5-7: Test requirement (network signalled value "CA_NS_07")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -8.2 |
| 2 |  |  |  |  | 23 | +2.7 / -11.7 |
| 3 |  |  |  |  | 23 | +2.7 / -6.2 |
| 4 |  |  |  |  | 23 | +2.7 / -6.2 |
| 5 |  |  |  |  | 23 | +2.7 / -28.7 |
| 6 |  |  |  |  | 23 | +2.7 / -17.7 |
| 7 |  |  |  |  | 23 | +2.7 / -6.2 |
| 8 |  |  |  |  | 23 | +2.7 / -11.7 |
| 9 |  |  |  |  | 23 | +2.7 / -9.7 |
| 10 |  |  |  |  | 23 | +2.7 / -6.2 |
| 11 |  |  |  |  | 23 | +2.7 / -9.7 |
| 12 |  |  |  |  | 23 | +2.7 / -28.7 |
| 13 |  |  |  |  | 23 | +2.7 / -17.7 |
| 14 |  |  |  |  | 23 | +2.7 / -6.2 |
| 15 |  |  |  |  | 23 | +2.7 / -12.7 |
| 16 |  |  |  |  | 23 | +2.7 / -6.2 |
| 17 |  |  |  |  | 23 | +2.7 / -9.7 |
| 18 |  |  |  |  | 23 | +2.7 / -28.7 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

[End of changes]