3GPP TSG RAN WG5 #82		R5-192125

Athens, Greece, 25th February – 1st March 2019

Source:		Rohde & Schwarz

Title:	On the SNR for FR2 TRx test cases

Agenda Item:		5.3.8.17

Document for:	Endorsement

# Introduction

At RAN5 #4-5G-NR Adhoc, estimations for the SNR have been discussed [1-4] and a way forward [5] has been created to summarize the assumptions for the SNR analysis. In this document, we present the results of our analysis based on the common assumptions.

# Discussion

The total SNR is derived based on the fact that the two components EIRPθ and EIRPφ are added to obtain the total EIRP:

SNRtotal,lin=Sθ,lin+Sφ,linNθ,lin+Nφ,lin=Slin2Nlin=SNRlin2 , 				(1)

where S and N denote the signal and noise power, respectively. The subscript lin is used for figures in linear units. For an EIRP measurement, the impact of the noise is calculated as

ΔSNR[dB]=10log101+SNRtotal, lin-1=10log101+2SNRlin .	(2)

Thus SNRtotal describes the SNR for the measurement of the total component power of both polarizations. In contrast, SNR is equivalent to measuring the total signal power as a single polarization without adding the power of the other polarization component. As the use of the latter is rather confusing, we focus on SNRtotal in the following. The relation between the two descriptions is

SNRtotal=SNR-3dB.

For spherical coverage the 50%-tile of the CDF is crucial. As a rough estimation of the systematic error due to the impact of noise, the Min EIRP at 50%-tile requirement is used as the relevant signal power level to calculate SNRtotal and ΔSNR. However, further simulations may be required to verify the applicability of this simplified approach as details of CDF statistics may have an effect on the impact of noise.

The impact of noise for TRP has been studied in [1] and [2]. In Ref. [1] the total TRP error due to noise is only analyzed for the case that the noise can be compensated to some extent. To this end, values of the assumed antenna pattern which are below the noise floor are set to 0 W. In this way, the measured TRP value will always be smaller than the real TRP of the DUT. So far, such a method is not yet permitted and needs agreement to be included in the test procedure.

In Ref [2], an analytic approach derives that Eqs. (1) and (2) can be applied for the calculation of the impact of noise when replacing the total signal power by the TRP power. For other grids and quadrature methods the deviation from the ideal integration of the noise results in an higher order correction. However, due to the specified minimum number of grid points the latter is negligible.

In general applying an LNA allows for a reduction of the total noise figure of a measurement setup and thus improves the SNR for low signal powers. However, the improvement is limited by compression effects and one has to take into account the maximum allowed output power of the DUT. The latter is allowed to reach 43 dBm and this does not allow to improve the SNR by much margin. If there is a limitation it is better to limit the accuracy of measurement at 43 dBm instead that of at min peak EIRP level since the 43 dBm case is not likely to occur in the field for PC3 anyways.

In Table 1, the results of our analysis are summarized:

Table 1: SNRtotal

| Test case | Frequency (GHz) | Estimated SNRtotal(dB) | Requirement (dBm) | Impact of Noise without relaxation (dB) | Required Relaxation |
| --- | --- | --- | --- | --- | --- |
| MOP Min Peak EIRP | 23.45 ≤ f ≤ 32.125 | 24.8 | 20.7 (= 22.4 – 1.7) | 0.014 | no |
|  | 32.125 < f ≤ 40.8 | 19.2 | 18.9 (= 20.6 - 1.7) | 0.052 | no |
| MOP Spherical coverage | 23.45 ≤ f ≤ 32.125 | 13.8 | 9.75 (= 11.5 – 1.75) | 0.18 | no |
|  | 32.125 < f ≤ 40.8 | 7.9 | 7.6 (= 8 – 0.4) | 0.65 | no |
| MOP TRP | 23.45 ≤ f ≤ 32.125 | 27.1 | 23 | 0.009 | no |
|  | 32.125 < f ≤ 40.8 | 23.3 | 23 | 0.02 | no |
| Transmit OFF Power (TRP) | 23.45 ≤ f ≤ 32.125 | -30.9 | -35 | 30.9 | do not test |
|  | 32.125 < f ≤ 40.8 | -34.7 | -35 | 34.7 | do not test |
| SEM (TRP) | work in progress |  |  |  |  |
| ACLR (TRP) | work in progress |  |  |  |  |
| REFSENS | work in progress |  |  |  |  |
| Spurious Emissions for UE co-existence | work in progress |  |  |  |  |
| Transmitter Spurious Emissions | work in progress |  |  |  |  |
| Receiver Spurious Emissions | work in progress |  |  |  |  |

Proposal 1: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements 6 GHz – 80 GHz for spurious emissions measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |
| #6 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, SNR estimations for low PSD test cases have been provided and the impact on the systematic uncertainty has been analyzed.

Proposal 1: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise.

# References

- R5-190601, On relaxation of test requirements for Low and High PSD test cases, Keysight, 3GPP TSG RAN WG5 Meeting #4-5G-NR Adhoc, January 2019

- R5-190446, On the power level to derive SNR for FR2 TRx tests, Anritsu, 3GPP TSG RAN WG5 Meeting #4-5G-NR Adhoc, January 2019

- R5-190460, SNR estimation update for FR2 priority 1 and 2 TRx TCs, Anritsu, 3GPP TSG RAN WG5 Meeting #4-5G-NR Adhoc, January 2019

- R5-190059, Impact from noise for FR2 TRx MU, Anritsu, 3GPP TSG RAN WG5 Meeting #4-5G-NR Adhoc, January 2019

- R5-190980, Way Forward on SNR estimation assumptions, Anritsu, Keysight Technologies, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #4-5G-NR Adhoc, January 2019