3GPP TSG RAN WG5 #82	R5-192199

# Athens, Greece, 25 Feb 2019 - 1 Mar, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | xx | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to add UL RMC for 60kHz SCS in Annex A.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2019-02-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UL RMC for 60KHz SCS with 16RB allocation does not exist. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UL RMCs for 60KHz SCS with 16RB allocation were defined. Some minor clarifications and typos in the existing UL RMCs were fixed.. |
| Consequences if not approved: |  | 6.4.2.2 Carrier Leakage and 6.4.2.3 In-Band Emissions tests could not be done without these RMCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

## A.2.3	Reference measurement channels for TDD

For UL RMCs defined below, TDD slot pattern defined in Table A.2.3-1 will be used for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, TDD slot patterns defined for reference sensitivity tests will be used.

.

Table A.2.3-1 Additional reference channels parameters for TDD

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | SCS 60 kHz (µ=2) | SCS 120 kHz (µ=3) |
| UL-DL configuration | referenceSubcarrierSpacing | 60 kHz | 120 kHz |
|  | dl-UL-TransmissionPeriodicity | 2 ms | 2 ms |
|  | nrofDownlinkSlots | 3 | 7 |
|  | nrofDownlinkSymbols | 4 | 12 |
|  | nrofUplinkSlot | 4 | 8 |
|  | nrofUplinkSymbols | 4 | 0 |

### A.2.3.1	DFT-s-OFDM Pi/2-BPSK

Table A.2.3.1-1: Reference Channels for DFT-s-OFDM pi/2-BPSK for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 4) | Total number of bits per slot for UL slots (Note 4) | Total modulated symbols per slot for UL slots (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-200 | 60 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  |  | 50-200 |  | 60 |  | 16 |  | 11 |  | pi/2 BPSK |  | 0 |  | 1/4 |  | 480 |  | 16 |  | 2 |  | 1 |  | 2024 |  | 2024 |  |
|  | 50 | 60 | 32 | 11 | pi/2 BPSK | 0 | 1/4 | 1032 | 16 | 2 | 1 | 4224 | 4224 |
|  | 50 | 60 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 100 | 60 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 100 | 60 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 200 | 60 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 200 | 60 | 256 | 11 | pi/2 BPSK | 0 | 1/4 | 7944 | 24 | 2 | 3 | 33792 | 33792 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 6.1.4.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 4: UL slot numbers are given by the slots satisfying mod(slot index, 8) = {4,5,6,7} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.1-2: Reference Channels for DFT-s-OFDM pi/2-BPSK for 120kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 4) | Total number of bits per slot for UL slots (Note 4) | Total modulated symbols per slot for UL slots (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-400 | 120 | 1 | 11 | pi/2 BPSK | 0 | 1/4 | 32 | 16 | 2 | 1 | 132 | 132 |
|  | 50 | 120 | 16 | 11 | pi/2 BPSK | 0 | 1/4 | 504 | 16 | 2 | 1 | 2112 | 2112 |
|  | 50 | 120 | 32 | 11 | pi/2 BPSK | 0 | 1/4 | 1032 | 16 | 2 | 1 | 4224 | 4224 |
|  | 100 | 120 | 32 | 11 | pi/2 BPSK | 0 | 1/4 | 1032 | 16 | 2 | 1 | 4224 | 4224 |
|  | 100 | 120 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 200 | 120 | 64 | 11 | pi/2 BPSK | 0 | 1/4 | 2024 | 16 | 2 | 1 | 8448 | 8448 |
|  | 200 | 120 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 400 | 120 | 128 | 11 | pi/2 BPSK | 0 | 1/4 | 3976 | 24 | 2 | 2 | 16896 | 16896 |
|  | 400 | 120 | 256 | 11 | pi/2 BPSK | 0 | 1/4 | 7944 | 24 | 2 | 3 | 33792 | 33792 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 6.1.4.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 4: UL slot numbers are given by the slots satisfying mod(slot index, 16) = {8,…,15} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.2	DFT-s-OFDM QPSK

Table A.2.3.2-1: Reference Channels for DFT-s-OFDM QPSK for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 4) | Total number of bits per slot for UL slots (Note 4) | Total modulated symbols per slot for UL slots (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-200 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 50-200 | 60 | 16 | 11 | QPSK | 2 | 1/6 | 768 | 16 | 2 | 1 | 4048 | 2024 |
|  | 50 | 60 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 50 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 100 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 100 | 60 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 200 | 60 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 200 | 60 | 256 | 11 | QPSK | 2 | 1/6 | 12808 | 24 | 2 | 4 | 67584 | 33792 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 6.1.4.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 4: UL slot numbers are given by the slots satisfying mod(slot index, 8) = {4,5,6,7} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.2-2: Reference Channels for DFT-s-OFDM QPSK for 120kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 4) | Total number of bits per slot for UL slots (Note 4) | Total modulated symbols per slot for UL slots (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-400 | 120 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 50 | 120 | 16 | 11 | QPSK | 2 | 1/6 | 808 | 16 | 2 | 1 | 4224 | 2112 |
|  | 50 | 120 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 100 | 120 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 100 | 120 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 200 | 120 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 200 | 120 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 400 | 120 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 400 | 120 | 256 | 11 | QPSK | 2 | 1/6 | 12808 | 24 | 2 | 4 | 67584 | 33792 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 6.1.4.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 4: UL slot numbers are given by the slots satisfying mod(slot index, 16) = {8,…,15} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.3	DFT-s-OFDM 16QAM

Table A.2.3.3-1: Reference Channels for DFT-s-OFDM 16QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 5) | Total number of bits per slot for UL slots (Note 5) | Total modulated symbols per slot for UL slots (Note 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-200 | 60 | 1 | 11 | 16QAM | 10 | 1/3 | 168 | 16 | 2 | 1 | 484 | 121 |
|  | 50 | 60 | 32 | 11 | 16QAM | 10 | 1/3 | 5376 | 24 | 1 | 1 | 16192 | 4048 |
|  | 50 | 60 | 64 | 11 | 16QAM | 10 | 1/3 | 10760 | 24 | 1 | 2 | 32384 | 8096 |
|  | 100 | 60 | 64 | 11 | 16QAM | 10 | 1/3 | 10760 | 24 | 1 | 2 | 32384 | 8096 |
|  | 100 | 60 | 128 | 11 | 16QAM | 10 | 1/3 | 21504 | 24 | 1 | 3 | 64768 | 16192 |
|  | 200 | 60 | 128 | 11 | 16QAM | 10 | 1/3 | 21504 | 24 | 1 | 3 | 64768 | 16192 |
|  | 200 | 60 | 256 | 11 | 16QAM | 10 | 1/3 | 43032 | 24 | 1 | 6 | 129536 | 32384 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 6.1.4.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit. Note 4: PTRS configuration is assumed to be (port 1, per 2PRB in frequency domain, per symbol in time domain). For TBS calculation, N-oh-prb = 6). Note 5: UL slot numbers are given by the slots satisfying mod(slot index, 8) = {4,5,6,7} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.3-2: Reference Channels for DFT-s-OFDM 16QAM for 120kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 5) | Total number of bits per slot for UL slots (Note 5) | Total modulated symbols per slot for UL slots (Note 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-400 | 120 | 1 | 11 | 16QAM | 10 | 1/3 | 168 | 16 | 2 | 1 | 484 | 121 |
|  | 50 | 120 | 16 | 11 | 16QAM | 10 | 1/3 | 2664 | 16 | 2 | 1 | 8096 | 2024 |
|  | 50 | 120 | 32 | 11 | 16QAM | 10 | 1/3 | 5376 | 24 | 1 | 1 | 16192 | 4048 |
|  | 100 | 120 | 32 | 11 | 16QAM | 10 | 1/3 | 5376 | 24 | 1 | 1 | 16192 | 4048 |
|  | 100 | 120 | 64 | 11 | 16QAM | 10 | 1/3 | 10760 | 24 | 1 | 2 | 32384 | 8096 |
|  | 200 | 120 | 64 | 11 | 16QAM | 10 | 1/3 | 10760 | 24 | 1 | 2 | 32384 | 8096 |
|  | 200 | 120 | 128 | 11 | 16QAM | 10 | 1/3 | 21504 | 24 | 1 | 3 | 64768 | 16192 |
|  | 400 | 120 | 128 | 11 | 16QAM | 10 | 1/3 | 21504 | 24 | 1 | 3 | 64768 | 16192 |
|  | 400 | 120 | 256 | 11 | 16QAM | 10 | 1/3 | 43032 | 24 | 1 | 6 | 129536 | 32384 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 6.1.4.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit. Note 4: PTRS configuration is assumed to be (port 1, per 2PRB in frequency domain, per symbol in time domain). For TBS calculation, N-oh-prb = 6). Note 5: UL slot numbers are given by the slots satisfying mod(slot index, 16) = {8,…,15} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.4	DFT-s-OFDM 64QAM

Table A.2.3.4-1: Reference Channels for DFT-s-OFDM 64QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 4) | Total number of bits per slot for UL slots (Note 4) | Total modulated symbols per slot for UL slots (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-200 | 60 | 1 | 11 | 64QAM | 18 | 1/2 | 384 | 16 | 2 | 1 | 726 | 121 |
|  | 50 | 60 | 32 | 11 | 64QAM | 18 | 1/2 | 12296 | 24 | 1 | 2 | 24288 | 4048 |
|  | 50 | 60 | 64 | 11 | 64QAM | 18 | 1/2 | 24576 | 24 | 1 | 3 | 48576 | 8096 |
|  | 100 | 60 | 64 | 11 | 64QAM | 18 | 1/2 | 24576 | 24 | 1 | 3 | 48576 | 8096 |
|  | 100 | 60 | 128 | 11 | 64QAM | 18 | 1/2 | 49176 | 24 | 1 | 6 | 97152 | 16192 |
|  | 200 | 60 | 128 | 11 | 64QAM | 18 | 1/2 | 49176 | 24 | 1 | 6 | 97152 | 16192 |
|  | 200 | 60 | 256 | 11 | 64QAM | 18 | 1/2 | 98376 | 24 | 1 | 12 | 194304 | 32384 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 6.1.4.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit. Note 4: PTRS configuration is assumed to be (port 1, per 2PRB in frequency domain, per symbol in time domain). For TBS calculation, N-oh-prb = 6.) Note 5: UL slot numbers are given by the slots satisfying mod(slot index, 8) = {4,5,6,7} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.4-2: Reference Channels for DFT-s-OFDM 64QAM for 120kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 4) | Total number of bits per slot for UL slots (Note 4) | Total modulated symbols per slot for UL slots (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-400 | 120 | 1 | 11 | 64QAM | 18 | 1/2 | 384 | 16 | 2 | 1 | 726 | 121 |
|  | 50 | 120 | 16 | 11 | 64QAM | 18 | 1/2 | 6144 | 24 | 1 | 1 | 12144 | 2024 |
|  | 50 | 120 | 32 | 11 | 64QAM | 18 | 1/2 | 12296 | 24 | 1 | 2 | 24288 | 4048 |
|  | 100 | 120 | 32 | 11 | 64QAM | 18 | 1/2 | 12296 | 24 | 1 | 2 | 24288 | 4048 |
|  | 100 | 120 | 64 | 11 | 64QAM | 18 | 1/2 | 24576 | 24 | 1 | 3 | 48576 | 8096 |
|  | 200 | 120 | 64 | 11 | 64QAM | 18 | 1/2 | 24576 | 24 | 1 | 3 | 48576 | 8096 |
|  | 200 | 120 | 128 | 11 | 64QAM | 18 | 1/2 | 49176 | 24 | 1 | 6 | 97152 | 16192 |
|  | 400 | 120 | 128 | 11 | 64QAM | 18 | 1/2 | 49176 | 24 | 1 | 6 | 97152 | 16192 |
|  | 400 | 120 | 256 | 11 | 64QAM | 18 | 1/2 | 98376 | 24 | 1 | 12 | 194304 | 32384 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 6.1.4.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit. Note 4: PTRS configuration is assumed to be (port 1, per 2PRB in frequency domain, per symbol in time domain). For TBS calculation, N-oh-prb = 6.) Note 5: UL slot numbers are given by the slots satisfying mod(slot index, 16) = {8,…,15} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.5	CP-OFDM QPSK

Table A.2.3.5-1: Reference Channels for CP-OFDM QPSK for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 4) | Total number of bits per slot for UL slots (Note 4) | Total modulated symbols per slot for UL slots (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-200 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 50-200 | 60 | 16 | 11 | QPSK | 2 | 1/6 | 768 | 16 | 2 | 1 | 4048 | 2024 |
|  | 50 | 60 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 50 | 60 | 66 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17424 | 8712 |
|  | 100 | 60 | 66 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17424 | 8712 |
|  | 100 | 60 | 132 | 11 | QPSK | 2 | 1/6 | 6536 | 24 | 2 | 2 | 34848 | 17424 |
|  | 200 | 60 | 132 | 11 | QPSK | 2 | 1/6 | 6536 | 24 | 2 | 2 | 34848 | 17424 |
|  | 200 | 60 | 264 | 11 | QPSK | 2 | 1/6 | 13064 | 24 | 2 | 4 | 69696 | 34848 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 5.1.3.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: UL slot numbers are given by the slots satisfying mod(slot index, 8) = {4,5,6,7} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.5-2: Reference Channels for CP-OFDM QPSK for 120kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 4) | Total number of bits per slot for UL slots (Note 4) | Total modulated symbols per slot for UL slots (Note 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-400 | 120 | 1 | 11 | QPSK | 2 | 1/6 | 56 | 16 | 2 | 1 | 264 | 132 |
|  | 50 | 120 | 16 | 11 | QPSK | 2 | 1/6 | 808 | 16 | 2 | 1 | 4224 | 2112 |
|  | 50 | 120 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 100 | 120 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 100 | 120 | 66 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17424 | 8712 |
|  | 200 | 120 | 66 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17424 | 8712 |
|  | 200 | 120 | 132 | 11 | QPSK | 2 | 1/6 | 6536 | 24 | 2 | 2 | 34848 | 17424 |
|  | 400 | 120 | 132 | 11 | QPSK | 2 | 1/6 | 6536 | 24 | 2 | 2 | 34848 | 17424 |
|  | 400 | 120 | 264 | 11 | QPSK | 2 | 1/6 | 13064 | 24 | 2 | 4 | 69696 | 34848 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 5.1.3.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: UL slot numbers are given by the slots satisfying mod(slot index, 16) = {8,…,15} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.6	CP-OFDM 16QAM

Table A.2.3.6-1: Reference Channels for CP-OFDM 16QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 5) | Total number of bits per slot for UL slots (Note 5) | Total modulated symbols per slot for UL slots (Note 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-200 | 60 | 1 | 11 | 16QAM | 10 | 1/3 | 168 | 16 | 2 | 1 | 484 | 121 |
|  | 50 | 60 | 33 | 11 | 16QAM | 10 | 1/3 | 5504 | 24 | 1 | 1 | 16698 | 4174.5 |
|  | 50 | 60 | 66 | 11 | 16QAM | 10 | 1/3 | 11016 | 24 | 1 | 2 | 33396 | 8349 |
|  | 100 | 60 | 66 | 11 | 16QAM | 10 | 1/3 | 11016 | 24 | 1 | 2 | 33396 | 8349 |
|  | 100 | 60 | 132 | 11 | 16QAM | 10 | 1/3 | 22032 | 24 | 1 | 3 | 66792 | 16698 |
|  | 200 | 60 | 132 | 11 | 16QAM | 10 | 1/3 | 22032 | 24 | 1 | 3 | 66792 | 16698 |
|  | 200 | 60 | 264 | 11 | 16QAM | 10 | 1/3 | 44040 | 24 | 1 | 6 | 133584 | 33396 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 5.1.3.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit. Note 4: PTRS configuration is assumed to be (port 1, per 2PRB in frequency domain, per symbol in time domain). For TBS calculation, N-oh-prb = 6.) Note 5: UL slot numbers are given by the slots satisfying mod(slot index, 8) = {4,5,6,7} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.6-2: Reference Channels for CP-OFDM 16QAM for 120kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 5) | Total number of bits per slot for UL slots (Note 5) | Total modulated symbols per slot for UL slots (Note 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-400 | 120 | 1 | 11 | 16QAM | 10 | 1/3 | 168 | 16 | 2 | 1 | 484 | 121 |
|  | 50 | 120 | 16 | 11 | 16QAM | 10 | 1/3 | 2664 | 16 | 2 | 1 | 8096 | 2024 |
|  | 50 | 120 | 32 | 11 | 16QAM | 10 | 1/3 | 5376 | 24 | 1 | 1 | 16192 | 4048 |
|  | 100 | 120 | 33 | 11 | 16QAM | 10 | 1/3 | 5504 | 24 | 1 | 1 | 16698 | 4174.5 |
|  | 100 | 120 | 66 | 11 | 16QAM | 10 | 1/3 | 11016 | 24 | 1 | 2 | 33396 | 8349 |
|  | 200 | 120 | 66 | 11 | 16QAM | 10 | 1/3 | 11016 | 24 | 1 | 2 | 33396 | 8349 |
|  | 200 | 120 | 132 | 11 | 16QAM | 10 | 1/3 | 22032 | 24 | 1 | 3 | 66792 | 16698 |
|  | 400 | 120 | 132 | 11 | 16QAM | 10 | 1/3 | 22032 | 24 | 1 | 3 | 66792 | 16698 |
|  | 400 | 120 | 264 | 11 | 16QAM | 10 | 1/3 | 44040 | 24 | 1 | 6 | 133584 | 33396 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 5.1.3.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit. Note 4: PTRS configuration is assumed to be (port 1, per 2PRB in frequency domain, per symbol in time domain). For TBS calculation, N-oh-prb = 6.) Note 5: UL slot numbers are given by the slots satisfying mod(slot index, 16) = {8,…,15} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### A.2.3.7	CP-OFDM 64QAM

Table A.2.3.7-1: Reference Channels for CP-OFDM 64QAM for 60kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for UL slots (Note 4) | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 5) | Total number of bits per slot for UL slots (Note 5) | Total modulated symbols per slot for UL slots (Note 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-200 | 60 | 1 | 11 | 64QAM | 19 | 1/2 | 384 | 16 | 2 | 1 | 726 | 121 |
|  | 50 | 60 | 33 | 11 | 64QAM | 19 | 1/2 | 12552 | 24 | 1 | 2 | 25047 | 4174.5 |
|  | 50 | 60 | 66 | 11 | 64QAM | 19 | 1/2 | 25104 | 24 | 1 | 3 | 50094 | 8349 |
|  | 100 | 60 | 66 | 11 | 64QAM | 19 | 1/2 | 25104 | 24 | 1 | 3 | 50094 | 8349 |
|  | 100 | 60 | 132 | 11 | 64QAM | 19 | 1/2 | 50184 | 24 | 1 | 6 | 100188 | 16698 |
|  | 200 | 60 | 132 | 11 | 64QAM | 19 | 1/2 | 50184 | 24 | 1 | 6 | 100188 | 16698 |
|  | 200 | 60 | 264 | 11 | 64QAM | 19 | 1/2 | 100392 | 24 | 1 | 12 | 200376 | 33396 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 5.1.3.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit. Note 4: PTRS configuration is assumed to be (port 1, per 2PRB in frequency domain, per symbol in time domain). For TBS calculation, N-oh-prb = 6.) Note 5: UL slot numbers are given by the slots satisfying mod(slot index, 8) = {4,5,6,7} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.7-2: Reference Channels for CP-OFDM 64QAM for 120kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots with mod(slot index+1,5) = 0 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for UL slots (Note 3, Note 5) | Total number of bits per slot for UL slots (Note 5) | Total modulated symbols per slot for UL slots (Note 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 50-400 | 120 | 1 | 11 | 64QAM | 19 | 1/2 | 384 | 16 | 2 | 1 | 726 | 121 |
|  | 50 | 120 | 16 | 11 | 64QAM | 19 | 1/2 | 6144 | 24 | 1 | 1 | 12144 | 2024 |
|  | 50 | 120 | 32 | 11 | 64QAM | 19 | 1/2 | 12296 | 24 | 1 | 2 | 24288 | 4048 |
|  | 100 | 120 | 33 | 11 | 64QAM | 19 | 1/2 | 12552 | 24 | 1 | 2 | 25047 | 4174.5 |
|  | 100 | 120 | 66 | 11 | 64QAM | 19 | 1/2 | 25104 | 24 | 1 | 3 | 50094 | 8349 |
|  | 200 | 120 | 66 | 11 | 64QAM | 19 | 1/2 | 25104 | 24 | 1 | 3 | 50094 | 8349 |
|  | 200 | 120 | 132 | 11 | 64QAM | 19 | 1/2 | 50184 | 24 | 1 | 6 | 100188 | 16698 |
|  | 400 | 120 | 132 | 11 | 64QAM | 19 | 1/2 | 50184 | 24 | 1 | 6 | 100188 | 16698 |
|  | 400 | 120 | 264 | 11 | 64QAM | 19 | 1/2 | 100392 | 24 | 1 | 12 | 200376 | 33396 |
| Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS Table 5.1.3.1-1 defined in TS 38.214 [23]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit. Note 4: PTRS configuration is assumed to be (port 1, per 2PRB in frequency domain, per symbol in time domain). For TBS calculation, N-oh-prb = 6.). Note 5: UL slot numbers are given by the slots satisfying mod(slot index, 16) = {8,…,15} with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, UL slot numbers are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>