3GPP TSG-RAN5 Meeting #84	R5-195776

# Ljubljana, Slovenia,  26th Aug – 30th Aug

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4933 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding points 25 25, 15 75 &75 25 on Table 6.5.2A.2.1.4.1-1 for UL-CA_66B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dekra Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing points 25 25, 15 75 &75 25 on Table 6.5.2A.2.1.4.1-1 for UL-CA_66B |  |  |  |  |  |  |  |  |
| Summary of change: |  | adding points 25 25, 15 75 &75 25 on Table 6.5.2A.2.1.4.1-1 for UL-CA_66B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 2DL/2UL: DL-CA_66B / UL-CA_66B would not be available in clause 6.5.2A.2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

##### 6.5.2A.2.1	Carrier leakage for CA (intra-band contiguous DL CA and UL CA)

6.5.2A.2.1.1	Test Purpose

Carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

6.5.2A.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.5.2A.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.2.0

6.5.2A.2.1.4	Test description

6.5.2A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause [4.3.1] for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 2 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 9 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 10 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 11 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 12 | 75 | 25 |  | QPSK | 8 | P_8@67 | S_0@0 |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  | 75 | 75 | NA | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 14 |  |  |  |  |  |  |  |  |  |  | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 15 |  |  |  |  |  |  |  |  |  |  | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 16 |  |  |  |  |  |  |  |  |  |  | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | - | - |
| 17 |  |  |  |  |  |  |  |  |  |  | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 18 |  |  |  |  |  |  |  |  |  |  | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 19 |  |  |  |  |  |  |  |  |  |  | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 20 |  |  |  |  |  |  |  |  |  |  | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | - | - |
| 21 |  |  |  |  |  |  |  |  |  |  | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 22 |  |  |  |  |  |  |  |  |  |  | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 23 |  |  |  |  |  |  |  |  |  |  | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 24 |  |  |  |  |  |  |  |  |  |  | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | - | - |
| 25 |  |  |  |  |  |  |  |  |  |  | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 26 |  |  |  |  |  |  |  |  |  |  | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 27 |  |  |  |  |  |  |  |  |  |  | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 28 |  |  |  |  |  |  |  |  |  |  | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

<< End of changes>>