3GPP TSG-RAN5 Meeting #84	R5-196258

# Ljubljana, Slovenia,  26 – 30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0038 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR FR1 intraband non-contiguous and interband CA tables with combinations CA_66(2A), CA_n66A-n70A, CA_n66A-n71A, CA_n70A-n71a, CA_n66B-n70A, CA_n66(2A)-n70A, CA_n66(2A)-n71A to 38.508-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | WE Certification, DISH Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2019-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR CA combinations CA_66(2A), CA_n66A-n70A, CA_n66A-n71A, CA_n70A-n71a, CA_n66B-n70A, CA_n66(2A)-n70A, CA_n66(2A)-n71A are added to 38.101-1 but 38.508-2 does not contain any intraband non-contiguous or inter-band combinations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of intraband FR1 non-contiguous table with n66(2A) combination and inter-band (two bands) FR1 table with combinations CA_n66A-n70A, CA_n66A-n71A, CA_n70A-n71a, CA_n66B-n70A, CA_n66(2A)-n70A, CA_n66(2A)-n71A to 38.508-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA combinations CA_66(2A), CA_n66A-n70A, CA_n66A-n71A, CA_n70A-n71a, CA_n66B-n70A, CA_n66(2A)-n70A, CA_n66(2A)-n71A applicability cannot be declared |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### A.4.3.2A	NR CA Physical Layer Baseline Implementation Capabilities

#### A.4.3.2A.1	General NR CA capabilities

Table A.4.3.2A.1-1: Downlink NR CA capabilities (for one or more of the supported NR CA configurations in Tables A.4.3.2A.2.1-3, A.4.3.2A.2.2-3, A.4.3.2A.3.2-3 and A.4.3.2A.4-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL NR CA with 2 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A |  |
| 2 | DL NR CA with 3 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A |  |
| 3 | DL NR CA with 4 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A |  |
| 4 | DL NR CA with 5 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A |  |
| 5 | DL NR CA with 6 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A |  |
| 6 | DL NR CA with 7 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A |  |
| 7 | DL NR CA with 8 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A |  |

Table A.4.3.2A.1-2: Uplink CA capabilities (for one or more of the supported NR CA configurations in Tables A.4.3.2A.2.1-3, A.4.3.2A.2.2-3, A.4.3.2A.3.2-3 and A.4.3.2A.4-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL NR CA with 2 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A |  |

#### A.4.3.2A.2	NR CA Intra-band contiguous

##### A.4.3.2A.2.1	NR CA Intra-band contiguous with FR1

Table A.4.3.2A.2.1-1: Downlink Bandwidth Class capabilities for NR Intra-band contiguous CA with FR1 configurations (for one or more of the supported configurations in Table A.4.3.2A.2.1-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL NR FR1 Intra-band contiguous CA BW Class A | 38.101-1, 5.3A.5 |  |
| 2 | DL NR FR1 Intra-band contiguous CA BW Class B | 38.101-1, 5.3A.5 |  |
| 3 | DL NR FR1 Intra-band contiguous CA BW Class C | 38.101-1, 5.3A.5 |  |
| 4 | DL NR FR1 Intra-band contiguous CA BW Class D | 38.101-1, 5.3A.5 |  |
| 5 | DL NR FR1 Intra-band contiguous CA BW Class E | 38.101-1, 5.3A.5 |  |
| 6 | DL NR FR1 Intra-band contiguous CA BW Class F | 38.101-1, 5.3A.5 |  |
| 7 | DL NR FR1 Intra-band contiguous CA BW Class G | 38.101-1, 5.3A.5 |  |
| 8 | DL NR FR1 Intra-band contiguous CA BW Class H | 38.101-1, 5.3A.5 |  |
| 9 | DL NR FR1 Intra-band contiguous CA BW Class I | 38.101-1, 5.3A.5 |  |
| 10 | DL NR FR1 Intra-band contiguous CA BW Class J | 38.101-1, 5.3A.5 |  |
| 11 | DL NR FR1 Intra-band contiguous CA BW Class K | 38.101-1, 5.3A.5 |  |
| 12 | DL NR FR1 Intra-band contiguous CA BW Class L | 38.101-1, 5.3A.5 |  |

Table A.4.3.2A.2.1-2: Uplink Bandwidth Class capabilities for NR Intra-band contiguous CA with FR1 configurations (for one or more of the supported configurations in Table A.4.3.2A.2.1-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL NR FR1 Intra-band contiguous CA BW Class A | 38.101-1, 5.3A.5 |  |
| 2 | UL NR FR1 Intra-band contiguous CA BW Class B | 38.101-1, 5.3A.5 |  |
| 3 | UL NR FR1 Intra-band contiguous CA BW Class C | 38.101-1, 5.3A.5 |  |
| 4 | UL NR FR1 Intra-band contiguous CA BW Class D | 38.101-1, 5.3A.5 |  |
| 5 | UL NR FR1 Intra-band contiguous CA BW Class E | 38.101-1, 5.3A.5 |  |
| 6 | UL NR FR1 Intra-band contiguous CA BW Class F | 38.101-1, 5.3A.5 |  |
| 7 | UL NR FR1 Intra-band contiguous CA BW Class G | 38.101-1, 5.3A.5 |  |
| 8 | UL NR FR1 Intra-band contiguous CA BW Class H | 38.101-1, 5.3A.5 |  |
| 9 | UL NR FR1 Intra-band contiguous CA BW Class I | 38.101-1, 5.3A.5 |  |
| 10 | UL NR FR1 Intra-band contiguous CA BW Class J | 38.101-1, 5.3A.5 |  |
| 11 | UL NR FR1 Intra-band contiguous CA BW Class K | 38.101-1, 5.3A.5 |  |
| 12 | UL NR FR1 Intra-band contiguous CA BW Class L | 38.101-1, 5.3A.5 |  |

Table A.4.3.2A.2.1-3: Supported NR CA configurations for Intra-band contiguous CA with FR1

| NR CA configuration / Item (Note 1) | Release | Supported | Supported CA Bandwidth Class(es) in UL (Note 2,5) | Supported Bandwidth Combination Set(s) (Note 3) |
| --- | --- | --- | --- | --- |
| CA_n41C | Rel-16 |  |  |  |
| CA_n66B | Rel-16 |  |  |  |
| CA_n71B | Rel-16 |  |  |  |
| CA_n77C | Rel-15 |  |  |  |
| CA_n78C | Rel-15 |  |  |  |
| CA_n79C | Rel-15 |  |  |  |
| CA_n77D | Rel-15 |  |  |  |
| CA_n78D | Rel-15 |  |  |  |
| CA_n79D | Rel-15 |  |  |  |
| CA_n77E | Rel-15 |  |  |  |
| CA_n78E | Rel-15 |  |  |  |
| CA_n79E | Rel-15 |  |  |  |
| Note 1: Notation used for intra-band contiguous CA Bands is according to TS 38.101-1 [23] Table 5.5A.1-1, e.g. ‘CA_n77C’ indicates CA operation on NR band n77 with DL CA Bandwidth Class C. Note 2: The UL CA capabilities as per Table A.4.3.2A.2.1-2 can be supported on a single or multiple CA Band(s). The UE supplier shall indicate all supported UL CA Bandwidth Class(es), in uplink of the supported CA Band(s), as per TS 38.101-1 [23] Table 5.5A.1-1. For this release of specification valid choices are ’N’, ‘XB’ and ‘XC’, where X is the band. For example, for CA_1C, N would mean only DL CA, ‘1C’ would mean both DL and UL CA. Note 3: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.1-1. Note 4: Reference to all items is 38.101-1, 5.5A.1 and 38.331, 6.3.4 Note 5: UL(Table A.4.3.2A.2.1-3) shall return all supported CA Configurations where at least one UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_2CC(Table A.4.3.2A.2.1-3) shall return all supported CA Configurations where at least one 2 Carrier UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_3CC(Table A.4.3.2A.2.1-3) shall return all supported CA Configurations where at least one 3 Carrier UL CA Bandwidth Class was declared. |  |  |  |  |

##### A.4.3.2A.2.2	NR CA Intra-band contiguous with FR2

Table A.4.3.2A.2.2-1: Downlink Bandwidth Class capabilities for NR Intra-band contiguous CA with FR2 configurations (for one or more of the supported configurations in Table A.4.3.2A.2.2-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL NR FR2 Intra-band contiguous CA BW Class A | 38.101-2, 5.3A.4 |  |
| 2 | DL NR FR2 Intra-band contiguous CA BW Class B | 38.101-2, 5.3A.4 |  |
| 3 | DL NR FR2 Intra-band contiguous CA BW Class C | 38.101-2, 5.3A.4 |  |
| 4 | DL NR FR2 Intra-band contiguous CA BW Class D | 38.101-2, 5.3A.4 |  |
| 5 | DL NR FR2 Intra-band contiguous CA BW Class E | 38.101-2, 5.3A.4 |  |
| 6 | DL NR FR2 Intra-band contiguous CA BW Class F | 38.101-2, 5.3A.4 |  |
| 7 | DL NR FR2 Intra-band contiguous CA BW Class G | 38.101-2, 5.3A.4 |  |
| 8 | DL NR FR2 Intra-band contiguous CA BW Class H | 38.101-2, 5.3A.4 |  |
| 9 | DL NR FR2 Intra-band contiguous CA BW Class I | 38.101-2, 5.3A.4 |  |
| 10 | DL NR FR2 Intra-band contiguous CA BW Class J | 38.101-2, 5.3A.4 |  |
| 11 | DL NR FR2 Intra-band contiguous CA BW Class K | 38.101-2, 5.3A.4 |  |
| 12 | DL NR FR2 Intra-band contiguous CA BW Class L | 38.101-2, 5.3A.4 |  |
| 13 | DL NR FR2 Intra-band contiguous CA BW Class M | 38.101-2, 5.3A.4 |  |
| 14 | DL NR FR2 Intra-band contiguous CA BW Class O | 38.101-2, 5.3A.4 |  |
| 15 | DL NR FR2 Intra-band contiguous CA BW Class P | 38.101-2, 5.3A.4 |  |
| 16 | DL NR FR2 Intra-band contiguous CA BW Class Q | 38.101-2, 5.3A.4 |  |

Table A.4.3.2A.2.2-2: Uplink Bandwidth Class capabilities for NR Intra-band contiguous CA with FR2 configurations (for one or more of the supported configurations in Table A.4.3.2A.2.2-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL NR FR1 Intra-band contiguous CA BW Class B | 38.101-2, 5.3A.4 |  |
| 2 | UL NR FR1 Intra-band contiguous CA BW Class C | 38.101-2, 5.3A.4 |  |
| 3 | UL NR FR1 Intra-band contiguous CA BW Class D | 38.101-2, 5.3A.4 |  |

Table A.4.3.2A.2.2-3: Supported NR CA configurations for Intra-band contiguous CA with FR2

FFS

#### A.4.3.2A.3	NR CA Intra-band non-contiguous

##### A.4.3.2A.3.1	NR CA Intra-band non-contiguous with FR1

Table A.4.3.2A.3.1-1: Downlink Bandwidth Class capabilities for NR Intra-band non-contiguous configurations CA with FR1 (for one or more of the supported configurations in Table A.4.3.2A.3.1-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL NR FR1 Intra-band non-contiguous CA BW Class Combination A-A | 38.101-2, 5.3A.5 |  |

Table A.4.3.2A.3.1-2: Uplink Bandwidth Class capabilities for NR Intra-band non-contiguous CA with FR1 configurations (for one or more of the supported configurations in Table A.4.3.2A.3.1-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL NR FR1 Intra-band non-contiguous CA BW Class Combination A-A | 38.101-2, 5.3A.5 |  |

Table A.4.3.2A.3.1-3: Supported NR CA configurations for Intra-band non-contiguous CA with FR1

| NR CA configuration / Item | Release | Supported | Supported CA Bandwidth Class(es) in UL (Note 3) | Supported Bandwidth Combination Set(s) (Note 1) |
| --- | --- | --- | --- | --- |
| CA_n66(2A) | Rel-16 |  |  |  |
| Note 1: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.2-1. Note 2: Reference to all items is 38.101-1 [23], 5.5A.2 and 38.331, 6.3.4 Note 3: UL(Table A.4.3.2A.2.1-3) shall return all supported CA Configurations where at least one UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_2CC(Table A.4.3.2A.2.1-3) shall return all supported CA Configurations where at least one 2 Carrier UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_3CC(Table A.4.3.2A.2.1-3) shall return all supported CA Configurations where at least one 3 Carrier UL CA Bandwidth Class was declared. |  |  |  |  |

##### A.4.3.2A.3.2	NR CA Intra-band non-contiguous with FR2

Table A.4.3.2A.3.2-1: Downlink Bandwidth Class capabilities for NR Intra-band non-contiguous configurations CA with FR2 (for one or more of the supported configurations in Table A.4.3.2A.3.2-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL NR FR2 Intra-band non-contiguous CA BW Class Combination A-A | 38.101-2, 5.3A.5 |  |

Table A.4.3.2A.3.2-2: Uplink Bandwidth Class capabilities for NR Intra-band non-contiguous CA with FR2 configurations (for one or more of the supported configurations in Table A.4.3.2A.3.2-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL NR FR1 Intra-band non-contiguous CA BW Class Combination A-A | 38.101-2, 5.3A.5 |  |

Table A.4.3.2A.3.2-3: Supported configurations for NR Intra-band non-contiguous CA with FR2

FFS

#### A.4.3.2A.4	NR Inter-band CA

#### A.4.3.2A.4.1	NR Inter-band CA with FR1 (two bands)

Table A.4.3.2A.4.1-1: Downlink Bandwidth Class Combination capabilities for NR Inter-band CA configuration with FR1 and two bands (for one or more of the supported CA configurations in Table A.4.3.2A.4.1-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL NR FR1 Inter-band CA BW Class Combination A-A (two bands) | 38.101-2, 5.3A.5 |  |

Table A.4.3.2A.4.1-2: Uplink Bandwidth Class Combination capabilities for NR Inter-band CA with FR1 and two bands (for one or more of the supported CA configurations in Table A.4.3.2A.4.1-3)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL NR FR1 Inter-band CA BW Class Combination A-A (two bands) | 38.101-2, 5.3A.5 |  |

Table A.4.3.2A.4.1-3: Supported CA configurations for NR Inter-band CA with FR1 and two bands

| NR CA configuration / Item (Note 1) | Release | Supported | Supported CA Bandwidth Class(es) in UL (Note 2,5) | Supported Bandwidth Combination Set(s) (Note 3) |
| --- | --- | --- | --- | --- |
| CA_n66A-n70A | Rel-16 |  |  |  |
| CA_n66B-n70A | Rel-16 |  |  |  |
| CA_n66(2A)-n70 | Rel-16 |  |  |  |
| CA_n66A-n71A | Rel-16 |  |  |  |
| CA_n66(2A)-n71A | Rel-16 |  |  |  |
| CA_n70A-n71A | Rel-16 |  |  |  |
| Note 1: Notation used for intra-band contiguous CA Bands is according to TS 38.101-1 [23] Table 5.5A.1-1, e.g. ‘CA_n77C’ indicates CA operation on NR band n77 with DL CA Bandwidth Class C. Note 2: The UL CA capabilities as per Table A.4.3.2A.2.1-2 can be supported on a single or multiple CA Band(s). The UE supplier shall indicate all supported UL CA Bandwidth Class(es), in uplink of the supported CA Band(s), as per TS 38.101-1 [23] Table 5.5A.1-1. For this release of specification valid choices are ’N’, ‘XB’ and ‘XC’, where X is the band. For example, for CA_1C, N would mean only DL CA, ‘1C’ would mean both DL and UL CA. Note 3: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.3-1. Note 4: Reference to all items is 38.101-1 [23], 5.5A.3 and 38.331, 6.3.4 Note 5: UL(Table A.4.3.2A.2.1-3) shall return all supported CA Configurations where at least one UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_2CC(Table A.4.3.2A.2.1-3) shall return all supported CA Configurations where at least one 2 Carrier UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_3CC(Table A.4.3.2A.2.1-3) shall return all supported CA Configurations where at least one 3 Carrier UL CA Bandwidth Class was declared. |  |  |  |  |