Title:	Test point selection for Out-of-band blocking for CA in FR1

Source:		CAICT

Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for Out-of-band blocking for CA in FR1 is analyzed.

Discussion

The Out-of-band blocking for CA in FR1 is defined in section 7.6A.3 of TS 38.101-1.

- 7.6A.3	Out-of-band blocking for CA

- 7.6A.3.1	Out-of-band blocking for Intra-band contiguous CA

For intra-band contiguous carrier aggreagation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test.

The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 7.6A.3-1 and Tables 7.6A.3-2 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.6A.3-1: Out-of-band blocking parameters for intra-band contiguous CA

| RX parameter | Units | CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Power in transmission bandwidth configuration | dBm | REFSENS + CA bandwidth class specific value below |  |  |
|  | dB | 9 | [9] | [9] |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |

Table 7.6A.3-2: Out of-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n41 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n77, n78 (NOTE 3) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(200,3CBWchannel CA) or FDL_high+ MAX(200,3CBWchannel CA) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(150,3CBWchannel CA) or FDL_high + MAX(150,3CBWchannel CA) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

- 7.6A.3.2	Void

- 7.6A.3.3	Out-of-band blocking for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the out-of-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.6.3 for each component carrier while all downlink carriers are active.

For inter-band carrier aggregation with uplink assigned to two NR bands, the out-of-band blocking requirements specified in subclause 7.6.3 shall be met with the transmitter power for the uplink set to 7 dB below PCMAX_L,f,c  for each serving cell c.

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.1-1, Pinterferer power defined in Table 7.6.3-2 and 7.6.3-4 is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.1-1.

For inter-band CA combination listed in Table 7.6A.3.3-1, exceptions to the requirement specified in Table 7.6A.3.3-2 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

Table 7.6A.3.3-1: CA band combination with exceptions allowed

CA band combination

CA_n8-n78

CA_n8-n79

CA_n28-n78

Table 7.6A.3.3-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when¦fInterferer- fSUL- fDL¦≤(BWSUL+ BWDL)/2 fInterferer± fULLB- fDLHB≤(BWULLB+ BWDLHB)/2, where fULLB and fDLHB are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. BWULLB and BWDLHB are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

For all interferer frequency ranges specified in subclause 7.6.3 a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of   MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in subclause 7.7 apply.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink and downlink modulations and RB configurations.

Test Environment

In NR single carrier testing, the Out-of-band blocking is tested under Normal condition. The same condition can be selected for NR CA testing.

Proposal 1: Define test environment as Normal for Out-of-band blocking for CA in FR1.

Test Subcarrier Spacing

In NR single carrier testing, the Out-of-band blocking is tested using lowest subcarrier spacing. This can be followed in NR CA testing.

Proposal 2: For Out-of-band blocking for CA, test Lowest subcarrier spacing.

Test Frequencies

In LTE CA, the Test Frequency of Out-of-band blocking for intra-band contiguous CA is Mid range, and the Test Frequency of Out-of-band blocking for inter-band CA is Mid range for all PCC/SCCs. The same approach can be followed in NR testing.

Proposal 3: For Out-of-band blocking for intra-band contiguous CA, the Mid range is tested. For Out-of-band blocking for inter-band CA, the Mid range for all PCC/SCCs is tested.

Test Channel Bandwidth

In LTE CA, the test channel bandwidth of Out-of-band blocking is Highest NRB_agg. The same approach can be followed in NR testing.

Proposal 4: For Out-of-band blocking for CA, the Highest NRB_agg is tested.

Modulation

In NR single carrier testing, the Out-of-band blocking is tested using CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink. This can be followed in NR CA testing.

Proposal 5: For Out-of-band blocking for CA, test CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink.

RB allocation

In NR single carrier testing, the Out-of-band blocking is tested using Full RB allocations for downlink and uplink. This can be followed in NR CA testing.

In NR intra-band contiguous CA testing for Maximum input level and Adjacent channel selectivity, RBs are allocated for all downlink PCC/SCCs and uplink PCC. This can be followed in NR intra-band contiguous CA testing for Out-of-band blocking.

In LTE inter-band CA, RBs are allocated for all downlink PCC/SCCs and uplink PCC. This can be followed in NR CA testing.

Proposal 6: For Out-of-band blocking for CA, test Full RB for all downlink PCC/SCCs and uplink PCC.

Conclusion

To sum up, the following selection of test points is proposed for Out-of-band blocking for CA in FR1:

Proposal 1: Define test environment as Normal for Out-of-band blocking for CA in FR1.

Proposal 2: For Out-of-band blocking for CA, test Lowest subcarrier spacing.

Proposal 3: For Out-of-band blocking for intra-band contiguous CA, the Mid range is tested. For Out-of-band blocking for inter-band CA, the Mid range for all PCC/SCCs is tested.

Proposal 4: For Out-of-band blocking for CA, the Highest NRB_agg is tested.

Proposal 5: For Out-of-band blocking for CA, test CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink.

Proposal 6: For Out-of-band blocking for CA, test Full RB for all downlink PCC/SCCs and uplink PCC.

Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR1.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |