3GPP TSG-RAN5 #85 Meeting	R5-198450

# Reno, USA, 18th – 22nd Nov. 2019

Title:							Proposal for influence of noise

Source:			Anritsu

Agenda Item:			5.3.5.17

Document for:			Endorsement

# 1.	Introduction

We propose the influence of noise for P1 and P2 test cases.

# 2.	Discussion

# 2.1. Estimated SNR

Table 1 summarizes the SNR estimation. Note the SNR estimation for spurious emission test case is provided to align with the QoQZ measurement frequencies.

Table 1 SNR estimations for P1 and P2 test cases

| Test case | Applicable Frequency Range [GHz] | UL signal level [dBm] | Estimated SNR for total component [dB] | ΔSNR=10log10(1+1/SNR), SNR in linear and defined for total component power | Proposed relaxation of test requirement |
| --- | --- | --- | --- | --- | --- |
| MOP (Spherical Coverage, PC3) | 23.45GHz ≤ f ≤ 32.125 GHz | 9.75dBm/ChBW (11.5-1.75) | 8.75 + ChBW | 0.37 | 0 |
|  | 32.125 GHz ≤ f ≤ 40.8 GHz | 7.6dBm/ChBW (8.0-0.4) | 5.9 + ChBW | 1.0 | 0 |
| Off power | 23.45 GHz ≤ f ≤ 32.125 GHz | -35dBm/ChBW | -18.0 + ChBW | 0.41 | 28.0 - ChBW |
|  | 32.125 GHz < f ≤ 40.8 GHz | -35dBm/ChBW | -26.1 + ChBW | 0.41~1.0 | 36.1~32.1 - ChBW |
| ACLR | Treated in the separated paper |  |  |  |  |
| Rx spurious | 6 GHz ≤ f ≤ 23.45 GHz | -47dBm/1MHz | 3.3 | 0.41~1.0 | 6.7~2.7 |
|  | 23.45 GHz < f ≤ 40 GHz |  | -6.2 | 0.41~1.0 | 16.2~12.2 |
|  | 40 GHz < f ≤ 80 GHz |  | -19.6 | 0.41~1.0 | 29.6~25.6 |
| General Tx spurious | 6 GHz ≤ f ≤ 12.75 GHz | -30dBm/1MHz | ~17 | 0.1 | 0 |
|  | 12.75 GHz ≤ f ≤ 23.45 GHz | -13dBm/1MHz | ~ 17 | 0.1 | 0 |
|  | 23.45GHz ≤ f ≤ 80 GHz |  | ~ 10 | 0.41 | 0 |
| NOTE ChBW = 10log10(400MHz/ChBW), e.g. 9.03dB for ChBW=50MHz. NOTE 2 : According to the agreement in [7], the SNR estimation is only valid between 23.45 and 30.3 GHz for other than spurious emission. SNR needs to be revisited once the new band and its core requirement is introduced between 30.3GHz and 32.125GHz. NOTE 4 : 14dB PAPR is assumed in this estimation to judge the LNA availability. |  |  |  |  |  |

Assumptions

| #1 | Assumption | Description |
| --- | --- | --- |
| #2 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurement. 6GHz – 80GHz for spurious measurement. |
| #3 | Size of QZ for IFF | 30 cm |
| #5 | Power range for EIRP measurements considered at the conducted reference plane | +43 dBm – Pathloss tolerable (PC3 max beam peak) |
| #6 | Temperature variation impact | +18 to +28 degrees C |
| #7 | UE power class | PC3 |
| #9 | Characterization for QoQZ for spurious measurements | N/A |

All the SNR is provided based on the “single” test setup.

Considering the above estimation and the discussion until now, followings are proposed.

Proposal 1 : For PC3 Spherical Coverage, no relaxation for the test requirement, ΔSNR = 0.37 dB for FR2a, ΔSNR = 1.0dB forFR2b

Proposal 2 : For PC3 Transmit OFF Power, SNR = 1.0 dB and relaxation 32.1 - ChBW[dB], or ΔSNR = 0.41 dB and relaxation 36.1 - ChBW[dB] for FR2b

For Tx spurious, our estimation is 10dB SNR is achievable for higher frequency and several dB more better SNR for low frequency is possible. As, there are varieties of choice for the implementation method, then to allow the flexible implementation we propose to simply apply the single SNR criteria of SNR>=10dB, and define MTSU based on SNR=10dB for whole frequency range up to 80GHz.

Proposal 3 : For PC3 General Tx Spurious, no relaxation for the test requirement, ΔSNR = 0.41 dB for 6Ghz <= f <= 80GHz.

Proposal 4 : For PC3 Rx Spurious, apply test requirement relaxation of 2.7dB(6 GHz ≤ f ≤ 23.45 GHz), 12.2dB(12.75 GHz ≤ f ≤ 23.45 GHz) or 25.6dB(23.45GHz ≤ f ≤ 80GHz) withSNR=1.0, or apply 6.7dB(6 GHz ≤ f ≤ 23.45 GHz), 16.2dB(12.75 GHz ≤ f ≤ 23.45 GHz) or 29.6dBdB(23.45GHz ≤ f ≤ 80GHz) withSNR=0.41.

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

Proposal 1 : For PC3 Spherical Coverage, no relaxation for the test requirement, ΔSNR = 0.37 dB for FR2a, ΔSNR = 1.0dB forFR2b

Proposal 2 : For PC3 Transmit OFF Power, SNR = 1.0 dB and relaxation 32.1 - ChBW[dB], or ΔSNR = 0.41 dB and relaxation 36.1 - ChBW[dB] for FR2b

Proposal 3 : For PC3 General Tx Spurious, no relaxation for the test requirement, ΔSNR = 0.41 dB for 6Ghz <= f <= 80GHz.

Proposal 4 : For PC3 Rx Spurious, apply relaxation of 2.7dB(6 GHz ≤ f ≤ 23.45 GHz), 12.2dB(12.75 GHz ≤ f ≤ 23.45 GHz) or 25.6dB(23.45GHz ≤ f ≤ 80GHz) with DSNR=1.0, or apply 6.7dB(6 GHz ≤ f ≤ 23.45 GHz), 16.2dB(12.75 GHz ≤ f ≤ 23.45 GHz) or 29.6dBdB(23.45GHz ≤ f ≤ 80GHz) with DSNR=0.41.

# 4.	References

[1] R5-193104	: “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5#NR5 (Apr-2019)

[2] R5-196852	: “On the SNR for FR2 TRx test cases”, ROHDE & SCHWARZ, RAN5#84 (Aug-2019)

[3] R5-196868	: “On noise impact”, Keysight Technologies UK Ltd, RAN5#84 (Aug-2019)

[4] R5-196960	: “Reply LS on 5G-NR FR2 Transmitter & Receiver Testability Issues”, ARIB, RAN5#84 (Aug-2019)