3GPP TSG-RAN5 Meeting #86-e	R5-200020

# Electronic Meeting, 17th - 28th February 2020

Title:							IFF DFF Hybrid setup for FR2 RRM 2AoA Test

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

In [1], benefit of IFF+DFF hybrid setup for FR2 RRM 2 AoA test was discussed.  In this paper we provide the some experiment data to prove feasibility of IFF+DFF hybrid setup, and also clarify some points indicated from the group in RAN5#85.

We re-iterate the benefit of using IFF+DFF hybrid setup for FR2 RRM 2AoA test.

Benefits of IFF+DFF hybrid system

- Benefit 1 : Test of all TRx, Demod and RRM in the single chamber is possible

- Benefit 2 : Chamber size can become smaller

- Benefit 3 : Total test system cost will be reduced

Especially, there is high demand from test industries for testing in the single and smaller chamber, hence meeting these demands is of high benefit for whole mobile industries.

# 2.	Discussion

#### 2.1 Feasibility analysis

Fig 1 shows the example of hybrid setup with one IFF TRxP and DFF TRxPs.

Figure 1 2 AoA RRM Test System utilizing IFF as one TRxP (example)

As a baseline requirement, RRM 2AoA test system needs to realize the relative angular difference of {30, 60, 90, 120, 150} degrees. The concrete way to realize these discrete relative angular differences are up to TE implementations.

##### 2.1.1 Experiments

The main factors that can cause a problem in IFF+DFF hybrid setup is the interference/scattering caused by the IFF reflector, which is the a big, compared to DFF TRxP, object made with metals.

The reflection of the IFF reflector can be calculated geometrically by positional relation of TRxPs and the reflector, hence it is predictable. Therefore, as the major premise, the design of the chamber (e.g. place of TRxPs) should be done so that such predictable reflections do not cause any interference.

The problem is the non-predictable reflections caused by the IFF reflector. Followings are one of the expected problematic interference modes.

- Signal from DFF TRxP@150deg is reflected/scattered at the reflector edge and injected back to DUT

- Signal from DFF TRxP is reflected somewhere inside a chamber and go to IFF reflector and then injected back to DUT

These two cases are depicted in Fig 2 with red-lines.

- Figure 2 Experiment of undesired reflections from IFF reflector

We conducted the experiments to evaluate these undesired interferences for these scenarios. This effect is evaluated by comparing S21 of DUT - DFF TRxP@150deg path with and without IFF reflector as depicted in Fig. 2.

In the experiment, reference antenna is set as transmitter and DFF TRxP@150deg is set as receiver. For reference antenna, HPBW= 60 deg(both side)  is used. Reference antenna is rotated and S21 for each angle is measured.

##### 2.1.2 Measurement result

Fig 3 and 4 show the S21 result for 28GHz for H pol and V-pol signals.

Figure 3 Comparison of S21 between w/ and w/o reflector (H-pol, @28GHz)

Figure 4 Comparison of S21 between w/ and w/o reflector (V-pol, @28GHz)

Same experiment is repeated for 40GHz and the results are shown in Fig.5 and Fig.6.

Figure 5 Comparison of S21 between w/ and w/o reflector (H-pol, @40GHz)

Figure 6 Comparison of S21 between w/ and w/o reflector (V-pol, @40GHz)

# Seeing Fig 3 to 6, difference of the S21 around the main lobe of reference antenna, which contain dominant power and is relevant for the test, coincides well for w/ and w/o reflector cases. This means that the effect from unwanted reflections from IFF reflector can be small enough and not harmful for test purpose.

# Observation 1 :  Effect of unwanted reflections due to reflector in IFF+DFF hybrid setup can be small enough

# Of course, the detailed absolute places, numbers, HPBW, etc of TRxPs are up to TE implementations and choice of them can give different impact for the unwanted reflections inside a chamber. But these experiment results show it is feasible to design IFF + DFF hybrid system so that effect from unwanted reflections due to reflectors becomes small enough. Hence, IFF+DFF hybrid setup can be feasible and can be used for 2AoA RRM tests.

# Observation 2 :  Designing the test system to minimize the effect of unwanted reflections due to IFF reflector is  possible, hence IFF+DFF hybrid setup can be used for 2AoA RRM Test

# Proposal 1 : Permit IFF+DFF hybrid setup for 2AoA RRM Test.

#### 2.2 Clarifications on some aspects

##### 2.2.1 Applicability Criteria

# In 2AoA RRM test case, many test cases require the change of relative angular relations for every iteration. In hybrid setup, it can be considered as a typical case that one of or both of 2 TRxPs is/are DFF TRxP(s). As DFF has less capability in terms of applicable UEs, it is a natural consequence that “applicable UE condition” should follow that for DFF when to test 2AoA RRM Test Cases in IFF+DFF hybrid setup. Hence, quoting the tables in TR 38.810, applicability of the test system for DFF+IFF will be defined as follows.

Table 2.1.1 (Table 5.3-1: DUT Antenna Configuration in TR 38.810)

| DUT Antenna Configuration | Description |
| --- | --- |
| 1 | Maximum one antenna panel with D ≤ 5 cm active at any one time |
| 2 | More than one antenna panel D ≤ 5 cm without phase coherence between panels active at any one time |
| 3 | Any phase coherent antenna panel of any size (e.g. sparse array) |

Table 2.1.2 (Table 5.3-2: Overview of test method applicability for permitted test methods in TR 38.810)

| DUT Antenna Configuration | Direct Far Field (DFF) | Indirect Far Field (IFF) | Near Field to far field transform (NFTF) | IFF+DFF Hybrid setup for RRM 2AoA Test |
| --- | --- | --- | --- | --- |
| 1 | Yes | Yes | Yes | Yes |
| 2 | Yes | Yes | Yes | Yes |
| 3 | No | Yes | No | No |
| NOTE: A positive indication means that applicability exists for at least one RF test cases for the given DUT Antenna Configuration |  |  |  |  |

# Proposal 2 : Define test method applicability as shown in Table 2.1.2 for IFF+DFF hybrid setup for 2AoA RRM test

##### 2.2.2 Quality of Quiet Zone verification

# TR 38.810 says QoQZ evaluation is necessary for only reference probe named P0, where P0 is defined as the TRxP along which the global z-axis is defined.

The absolute position of the probes is left up to implementation. The relative angular relationship between two AoAs can be achieved using any combination of probes. The relative orientation of the coordinate system is defined with respect to one probe, referred to as the reference probe P0 along which the z-axis is defined. The test conditions in Annex C.2 and the positioning guidelines in C.3 apply to the RRM baseline setup. The quality of quiet zone validation defined in Annex D only needs to be performed with the reference probe P0.

# For hybrid setup, our view is at least 1 QoQZ for IFF among whole IFF TRxPs , and at least 1 QoQZ for DFF TRxPs needs to be verified as they can have different QoQZ and are not compatible each other.

# Proposal 3 : Mandate QoQZ verification for at least one of the IFF TRxPs and at least one of the DFF TRxPs.

Note that global z-axis only directs to one of the TRxPs and the other TRxPs will be at different position where global z-axis does not direct to it. In that case, QoQZ needs to be measured along with the coordinate system of which z-axis directs to the TRxP for which QoQZ is measured.

# Proposal 4 : QoQZ verification shall be done along with the coordinate system of which z-axis directs to the target TRxP.

Despite the description in QoQZ verification in 38.810, it is not yet specified in RAN5 test specification. Some clarification on QoQZ verification needs to be added to 38.508-1 not only for IFF+DFF hybrid setup, but also other setups DFF, enhanced IFF.

##### 2.2.3 Quiet Zone size

Quiet zone size of IFF TRxPs and DFF TRxPs follows that of IFF TRx Setup and DFF TRx Setup, respectively. Different quiet zone size for IFF and DFF can be possible (this can be possible even in  DFF or enhanced IFF, though) and in that case the smallest quiet zone size among them is regarded as the quiet zone size of IFF+DF setup for 2AoA RRM Test.

# Proposal 5 : Quiet zone size of IFF TRxPs and DFF TRxPs follows that of IFF TRx setup and DFF TRx setup, respectively.

# Proposal 6 : In case quiet zone size is different between IFF and DFF, the smallest quiet zone size among them is regarded as the quiet zone size of IFF+DFF hybrid setup.

##### 2.2.3 Measurement Uncertainty

# As the measurement uncertainty in RRM test case is defined per cell, i.e. per TRxP, basis, then there is no special thing to be considered for DFF+IFF test setup. MU is just defined for DFF TRxP and IFF TRxP respectively.

# Some more consideration for MU definition for FR2 RRM setup considering different test setup is provided in separated paper[2].

# 3.	Conclusion

# Followings are observed:

# Observation 1 :  Effect of unwanted reflections due to reflector in IFF+DFF hybrid setup can be small enough

# Observation 2 :  Designing the test system to minimize the effect of unwanted reflections due to IFF reflector is  possible, hence IFF+DFF hybrid setup can be used for 2AoA RRM Test

# Observation 1 :  Effect of unwanted reflections due to reflector in IFF+DFF hybrid setup can be small enough

# RAN5 is asked to endorse following proposals.

# Proposal 1 : Permit IFF+DFF hybrid setup for 2AoA RRM Test.

# Proposal 2 : Define test method applicability as shown in Table 2.1.2 for IFF+DFF hybrid setup for 2AoA RRM test

# Proposal 3 : Mandate QoQZ verification for at least one of the IFF TRxPs and at least one of the DFF TRxPs.

# Proposal 4 : QoQZ verification shall be done along with the coordinate system of which z-axis directs to the target TRxP.

# Proposal 5 : Quiet zone size of IFF TRxPs and DFF TRxPs follows that of IFF TRx setup and DFF TRx setup, respectively.

# Proposal 6 : In case quiet zone size is different between IFF and DFF, the smallest quiet zone size among them is regarded as the quiet zone size of IFF+DFF hybrid setup.

# 4.	References

[1] R5-198366, “Benefit of 2 AoA RRM test setup with IFF TRxPs for FR2”, Anritsu, RAN5#85

[2] R5-200741, “Treatment FR2 RRM MU for different test setups”, Anritsu, RAN5#86