3GPP TSG-RAN5 Meeting #86-e		R5-200811
Electronic Meeting, 17th – 28th February 2020

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On RRM test cases list to start FR2 MU analysis

Document for:	Discussion and Endorsement

# Introduction

This contribution is addressing the action point AP#85.24 opened in the last meeting including an MU discussion captured in the WF.

# Discussion

The following action point was opened last meeting:

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline |
| --- | --- | --- | --- | --- | --- |
| AP#85.23 | RF | Choose representative test case (worst MU test case) to start FR2 RRM MU analysis | R&S, KEYS, Anritsu, Samsung, Qualcomm | R5-199500 | RAN5#86 |

as one of the action items for RRM FR2 MU work defined in the WF [1]

As stated in [1], the following MU quantities have been identified:

- DL AWGN absolute power or wanted DL signal absolute power

- DL applied SNR

- DL Fading profile uncertainty

- DL AWGN and signal flatness

- UL absolute power measurement

- UL relative power measurement

- UL signal transmit timing relative to DL

- Relative transmit timing accuracy during UE timing adjustment

- Timing Advance Adjustment accuracy

The list of RRM test cases (worst case) can be selected based on representative scenarios where one or several above MU items apply.

On the other hand, another criteria to identify representative test case, to perform FR2 MU analysis, is the number of AoAs and associated test setup.

In the following table there is a mapping between above MU quantities (new MU quantities are not precluded), test setups and NSA FR2 most representative test case:

| MU quantity | Representative Test Case | FR2 test setup |
| --- | --- | --- |
| DL AWGN absolute power or wanted DL signal absolute power | SS-RSRP, L1-RSRP Event trigger reporting | Setup 1 Setup 3 |
| DL applied SNR | SS-RSRP, L1-RSRP Event trigger reporting | Setup 1 Setup 3 |
| DL Fading profile uncertainty | RLM | Setup 2 |
| UL absolute power measurement | RLM | Setup 2 |
| UL relative power measurement | Contention based random access | Setup 2b |
| UL signal transmit timing relative to DL | Timing accuracy | Setup 1 |
| Relative transmit timing accuracy during UE timing adjustment | Timing accuracy | Setup 1 |
| Timing Advance Adjustment accuracy | Timing advance | Setup 1 |

Table 1 Mapping MU quantity, FR2 setup and Representative TC

Based on Table 1, representative test case list proposal is as follows:

- 5.3.2.2.1 EN-DC FR2 contention based random access

- 5.4.1.1 EN-DC FR2 UE transmit timing accuracy

- 5.4.3.1 EN-DC FR2 timing advance adjustment accuracy

- 5.5.1.1 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode

- 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX

- 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX

- 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX

- 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX

- 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy

- 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy

- 5.7.4.1 EN-DC FR2 SSB-based L1-RSRP measurement accuracy

Intra-freq and inter-freq scenarios have been considered. Other representatives Test cases are not precluded.

# Conclusion

The following proposals were made in this contribution

Proposal 1: Following representative test cases are proposed to perform RRM FR2 MU analysis:

- 5.3.2.2.1 EN-DC FR2 contention based random access

- 5.4.1.1 EN-DC FR2 UE transmit timing accuracy

- 5.4.3.1 EN-DC FR2 timing advance adjustment accuracy

- 5.5.1.1 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode

- 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX

- 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX

- 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX

- 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX

- 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy

- 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy

- 5.7.4.1 EN-DC FR2 SSB-based L1-RSRP measurement accuracy

# References

- R5-199500, Way Forward on RRM FR2 MU, Rohde & Schwarz, Anritsu, Keysight, Qualcomm, Samsung, 3GPP TSG-RAN WG5 #85, November 2019