3GPP TSG RAN Meeting #87-e	 	R5-202082

Electronic Meeting, 18th  – 29th May 2020

Source:		Rohde & Schwarz

Title:	Discussion on size of Quiet Zone above 30cm

Agenda Item:		5.3.2.17

Document for:	Discussion and Endorsement

# Introduction

Following the discussion in previous meetings regarding QZ sizes bigger than 30cm in diameter, we present here our views on the next steps.

This is a resubmission of draft shared before RAN5 #86-e meeting with additional data from a device survey.

# Discussion

In Ref. [1], a brief survey of devices implementing WWAN (LTE) connectivity was presented and two additional QZ sizes (45 and 65cm in diameter) were proposed in order to cover all devices.

From our point of view, no matter the size of the survey, there will be always outliers that will require even larger QZ sizes, and therefore the approach to define the QZ size based on the bigger device is not sustainable.

If discrete QZ sizes are to be defined, it is obvious that a compromise should be pursued, where most of the devices are covered with the minimum number of additional QZ sizes.

Observation 1: The attempt to cover all possible devices sizes with a discrete number of QZ sizes is not a sustainable approach.

Until further feedback is provided from UE vendors and operators, we performed an extensive market survey for 4G and 5G enabled devices larger than 30cm diameter, obtaining a total of 73 devices in addition to the 15 devices listed in Ref. [1]. Figure 2-1 shows the distribution of devices with respect to the required QZ size. The red curve represents a fit of a Gaussian CDF to the empirical data. The null hypothesis that the data follows a normal distribution cannot be rejected by a Kolmogorov-Smirnov test at a 5% significance level.

Figure 2-1: CDF of device sizes. Blue solid line: empirical data. Blue dotted lines: confidence bounds for 95% confidence level. Red curve: Gaussian CDF fit to the data.

The number of devices per type is shown in table 2-1:

| CPE | 15 |
| --- | --- |
| Router | 30 |
| Gateway | 1 |
| Laptop | 29 |
| Tablet | 13 |
| Total | 88 |

Table 2-1: Number of devices per type in the survey

Based on this information, 50cm QZ seems the best compromise since it covers 72% to 89% of devices with one single QZ size with a confidence level of 95%.

Observation 2: ~81% of the devices in the available dataset are covered with a 50cm QZ.

Proposal 1: Select 50cm as the QZ size to test devices larger than current QZ limited to 30cm.

The CRs associated to this discussion paper are Refs. [2], [3] and [4].

It has to be noted that this dataset only refer to currently available devices on the market with WWAN (LTE) capabilities and only a limited amount with NR capabilities.

It is therefore highly recommended that OEMs and operators provide feedback on the foreseen device sizes bigger than 30cm in order to make the analysis more meaningful to actual NR FR2 devices.

Proposal 2: In OEM and operators to provide data on NR FR2 devices size.

# Conclusion

In this contribution, the following observations and proposals were made:

Observation 1: The attempt to cover all possible devices sizes with a discrete number of QZ sizes is not a sustainable approach.

Observation 2: ~81% of the devices in the available dataset are covered with a 50cm QZ.

Proposal 1: Select 50cm as the QZ size to test devices larger than current QZ limited to 30cm.

Proposal 2: OEM and operators to provide data on NR FR2 devices size.

# References

- R5-198262, On Larger Quiet Zone Sizes, Keysight Technologies UK Ltd, 3GPP TSG RAN WG5 Meeting #85-bis, November 2019

- R5-202083, Addition of Quiet Zone size above 30cm to TS 38.521-2, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #87-e, May 2020

- R5-202084, Addition of Quiet Zone size above 30cm to TS 38.508-1, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #87-e, May 2020

- R5-202085, Addition of Quiet Zone size above 30cm to TS 38.903, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #87-e, May 2020