3GPP TSG RAN WG5 Meeting #88-e						R5-204022

Electronic Meeting, 17th – 28th August 2020

Source: 		Samsung

Title:		Discussion on FR2 absolute power control MU

Agenda item:	5.3.2.17

Document for:		Discussion and Endorsement

# 1.	Introduction

At RAN5#83 meetings there have been discussions on how to test the absolute power control requirements defined in core specification TS 38.101-2 [1]. Furthermore, at RAN5#84 agreed on testing the absolute power control requirements using indirectly P0 and alpha parameters method and the parameters.

With that being said, this contribution focus is on TS 38.101-2, clause 6.3A.4.2 [1] power control for CA requirements. The requirements are as such:

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on each active component carriers larger than 20ms. For SRS switching, the absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on component carriers (to which SRS switching occurs) larger than 20ms. The requirement can be tested by time aligning any transmission gaps on the component carriers. For intra-band contiguous CA, the absolute power control tolerance per configured UL CC is given in Tables 6.3.4.2-1 and 6.3.4.2-2.

In this contribution, we briefly discuss our views and recommendation of MU parameters for testing absolute power tolerance.

# 2.	Discussion

The requirements on power control accuracy in CA operation apply under normal conditions and are defined as a directional requirement. The requirements for power control for CA are verified in beam locked mode on beam peak direction. The requirements apply for one single PUCCH, PUSCH or SRS transmission of contiguous PRB allocation per configured UL CC.

The minimum requirements as specified in [1] clause 6.3A.4, for FR2 absolute power control tolerance are provide below for power class 3. The minimum output power requirements specified in TS 38.101-2 [1] Table 6.3A.1.2-1 apply in the power range bounded by the minimum output power (‘Pmin’). The maximum output power as minimum peak EIRP requirements specified in TS 38.101-2 [1] Table 6.2.1.3-1 apply in the power range bounded by the maximum output power (‘Pmax’). The parameter ‘Pmax’ refers to the requirement of minimum max EIRP.

-------------------------------------------Excerpt from TS 38.101-2 start -----------------------------------------

### 6.2A.1	UE maximum output power for CA

For downlink intra-band contiguous and non-contiguous carrier aggregation with a single uplink component carrier configured in the NR band, the maximum output power is specified in section 6.2.1.

For uplink intra-band contiguous carrier aggregation for any CA bandwidth class, the maximum output power is specified in section 6.2.1.

Power class 3 is default power class.

…

### 6.2.1	UE maximum output power

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of total component of EIRP (Link=Beam peak search grids, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.3-1.

Table 6.2.1.3-1: UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Void |  |

…

### 6.3.4	Power control

#### 6.3.4.2	Absolute power tolerance

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error RSRP estimate.

The minimum requirements specified in Table 6.3.4.2.3-1 apply in the power range bounded by the minimum output power as specified in sub-clause 6.3.1.3 (Pmin) and the maximum output power as specified in sub-clause 6.2.1.1 as minimum peak EIRP (‘Pmax’). The intermediate power point ‘Pint’ is defined in table 6.3.4.2.3-2.

Table 6.3.4.2.3-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

Table 6.3.4.2.3-2: Intermediate power point

| Power Parameter | Value |
| --- | --- |
| Pint | Pmax – 12.0 dB |

…

### 6.3A.4	Power control for CA

#### 6.3A.4.2	Absolute power tolerance

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on each active component carriers larger than 20ms. For SRS switching, the absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with transmission gap on component carriers lager than 20ms.

The requirement can be tested by time aligning any transmission gaps on the component carriers. For intra-band contiguous CA, the absolute power control tolerance per configured UL CC is given in Tables 6.3.4.2-1 and 6.3.4.2-2.

The minimum requirements specified in Table 6.3.4.2-1 apply in the power range bounded by the minimum output power as specified in sub-clause 6.3.1 (‘Pmin’) and the maximum output power as specified in sub-clause 6.2.1 as minimum peak EIRP (‘Pmax’). The intermediate power point ‘Pint’ is defined in Table 6.3.4.2-2.

Table 6.3.4.2-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

Table 6.3.4.2-2: Intermediate power point

| Power Parameter | Value |
| --- | --- |
| Pint | Pmax – 12.0 dB |

--------------------------------------------Excerpt from TS 38.101-2 end -----------------------------------------

Based on the minimum requirements, the target power level ‘P’ should be within the power range bound for absolute power tolerance testing. An example of the minimum output power, maximum output power and intermediate power point values for power class 3 with operating bands n257, n258, n261 are provided as such:

- The minimum output power ‘Pmin’ EIRP = -13 dBm,

- The Min peak EIRP = 22.4 dBm, and

- The intermediate power point ‘Pint’ for EIRP = 10.4 dBm

- The Max EIRP ‘Pmax’ = 43 dBm

The absolute power control TT calculations are shown in Table 2-1.

Table 2-1: Absolute power control TT Calculations

| Parameter | Tolerance | MU [dB] | TT = Tolerance + MU [dBm] | Comments |
| --- | --- | --- | --- | --- |
| Pmin | 14 | 5 | ±19 | MU as defined in [3] (±4.89 dB) rounded to ±5 dB |
| Pmax | 12 | 5 | ±17 | MU as defined in [3] (±4.89 dB) rounded to ±5 dB |

We also agreed to have one test point (test point 1) for power range 1 and two test points test (point 2 and test point 3) for power range 2. Based on the information above, the nominal UE power values shown in Table 2-2, Table 3.1-3 and Table 2-3 were selected.

Table 2-2: Test Requirements of Absolute power tolerance (Test Point 1) for power range 1

| Power range | Nominal UE Power [dBm] | Lower limit [dBm] | Upper limit [dBm] |
| --- | --- | --- | --- |
| Pint ≥ P ≥ Pmin | 7 | 7 - [TT] = -12 | 7 + [TT] = 26 |

Table 2-3: Test Requirements of Absolute power tolerance (Test Point 2) for power range 2

| Power range | Nominal UE Power [dBm] | Lower limit [dBm] | Upper limit [dBm] |
| --- | --- | --- | --- |
| Pmax ≥ P > Pint | 11 | 11 - [TT] = -6 | 11 + [TT] = 28 |

Table 2-4: Test Requirements of Absolute power tolerance (Test Point 3) for power range 2

| Power range | Nominal UE Power [dBm] | Lower limit [dBm] | Upper limit [dBm] |
| --- | --- | --- | --- |
| Pmax ≥ P > Pint | 22 | 22 – [TT] = 5 | 22 + [TT] = 39 |

The expected measured power based on the parameters described above are provided in Table 2-5, Table 2-6, and Table 2-7.

Table 2-5: Test point 1 expected measured power value

|  | SCS | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | 60kHz | 8.1 | 7.1 | 8.1 | N/A |
| power | 120kHz | 8.1 | 7.1 | 8.1 | 7.1 |
| Power tolerance |  | ± (14+TT) dB |  |  |  |

Table 2-6: Test point 2 expected measured power value

|  | SCS | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | 60kHz | 12.1 | 11.1 | 12.1 | N/A |
| power | 120kHz | 12.1 | 11.1 | 12.1 | 11.1 |
| Power tolerance |  | ± (12+TT) dB |  |  |  |

Table 2-7: Test point 3 expected measured power value

|  | SCS | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | 60kHz | 22.1 | 22.1 | 22.1 | N/A |
| power | 120kHz | 22.1 | 22.1 | 22.1 | 21.1 |
| Power tolerance |  | ± (12+TT) dB |  |  |  |

Since the UE maximum output power (EIRP) values were used to calculate the Nominal UE Power parameters we propose the following MU values for PC3:

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.3.4.2 Absolute power tolerance | PC3 Minimum peak EIRP, Max EIRP Quiet Zone size ≤ 30 cm ±4.89 dB (FR2a) ±5.09 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-2-2 in TR 38.903) |

Proposal 1: The MU values for absolute power tolerance Quiet Zone size ≤ 30 cm is ±4.89 dB (FR2a) and ±5.09 dB (FR2b).

For the test tolerance we proposed the following values for PC3:

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.3.4.2 Absolute power tolerance | PC3 Minimum peak EIRP IFF (Quiet Zone size ≤ 30 cm) 2.87 dB (FR2a) 2.87 dB (FR2b) | Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) |

Proposal 2: The TT values for absolute power tolerance Quiet Zone size ≤ 30 cm is 2.87 dB (FR2a) and 2.87 dB (FR2b).

Table 2-8: Test Tolerance (Test point 1) for power range 1

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | 2.87 dB | 2.87 dB |

Table 2-9: Test Tolerance (Test point 2 and Test point 3) for power range 2

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | 2.87 dB | 2.87 dB |

Proposal 3: The same MU and TT values for absolute power tolerance Quiet Zone size size ≤ 30 cm are used for absolute power tolerance for CA for each CC.

# 3.	Conclusion

In this contribution, we discussed our views and recommendation of MU parameters for testing absolute power tolerance. Based on the discussion, our views and proposals are summarized as follows:

Proposal 1: The MU values for absolute power tolerance Quiet Zone size ≤ 30 cm is ±4.89 dB (FR2a) and ±5.09 dB (FR2b).

Proposal 2: The TT values for absolute power tolerance Quiet Zone size ≤ 30 cm is 2.87 dB (FR2a) and 2.87 dB (FR2b).

Proposal 3: The same MU and TT values for absolute power tolerance Quiet Zone size size ≤ 30 cm are used for absolute power tolerance for CA for each CC.

# References

[1]		3GPP TS 38.101-2: "NR: User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[2]	R5-199354, "Further discussion on FR2 absolute power control method", Samsung

[3] 		3GPP TR 38.903: "NR: Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases".