3GPP TSG-RAN5 Meeting #88-e		R5-204186
Electronic meeting, 17 August - 28 August 2020

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On Beam Peak Searches for different test configurations

Document for:	Discussion and Endorsement

# Introduction

This contribution addresses the need to perform TX and RX beam peak searches for different test configurations.

# Discussion

In the last meeting, various proposals were presented to allow vendor declarations [1] for different test frequency ranges.  The agreements endorsed in proposals 1 through 3 have been captured in CRs this meeting [2][3]; additional agreements made in this contribution can be captured in those CRs.

The safest approach would be to mandate a separate beam peak search for each test condition. However, due to the long test times of these scans, a re-use of beam peak directions should be allowed if applicable.

At this point, no agreement was reached whether the beam peak searches need to be performed for different channel bandwidths, SCS, modulation, waveforms, etc.

Observation 1: Whether the beam peak searches need to be performed for different channel bandwidths, SCS, modulation, waveforms has not been decided yet.

Since no agreement on a reference CBW could be reached in the last meeting, it is proposed that a beam peak search for every CBW shall be performed. Once more data is readily available, vendors declarations to re-use beam peak search results from one reference CBW for other CBWs can be revisited. The same approach should be applied to different SCS given the lack of data/feedback..

Proposal 1: Beam peak searches shall be performed for every CBW and SCS by default.

A brief investigation on one smartphone UE was performed to determine whether different waveforms and modulations significantly impact the beam peak results. Key parameters chosen for this empirical investigation were as follows:

- EN-DC band combinations: 66A-n260A & 66A-n261A

- NR FR2 Settings:

  - Modulations: QPSK & 16QAM & 64QAM

  - FR2 Test Frequency Ranges: Low, Mid, and High

  - Waveforms: CP-OFDM and DFTs-OFDM

  - CBW: 100MHz

- LTE Settings

  - Frequency Channel: Low

  - BW: 20MHz

  - UL Modulation: QPSK

For all NR FR2 bands, modulations, waveforms, and test frequency ranges, a full 3D TX beam peak search was performed resulting in a Beam Peak direction with a 15o step size granularity. The beam peak direction and EIRP for each DFT QPSK measurement were recorded and considered the reference per channel per band. If the 16QAM or 64QAM beam peak (Peak EIRP) search resulted in a different beam peak (Peak Theta, Peak Phi) direction when compared to DFT QPSK beam peak direction (Ref Theta, Ref Phi), the EIRP (RefPwr) at the corresponding QPSK reference beam peak direction (Ref Theta, Ref Phi) was recorded and compared with the reference EIRP to calculate the difference (RefDelta). The measurement results are tabulated in Table 1. Clearly, the results show that the beam peak results at the reference beam peak direction do not change between different modulations and waveforms.

Table 1: Beam Peak Search Results for different waveforms and modulations

| Band | Test Frequency Range | Wave- form | Mod | Peak Theta | Peak Phi | Peak EIRP | Ref Theta | Ref Phi | RefPwr (dBm) | RefDelta = Peak EIRP - RefPwr (dB) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n260 | L | DFT | QPSK | 90 | 270 | 28.24 | 90 | 270 | 28.24 | 0.00 | Reference |
|  |  |  | 16Q | 90 | 90 | 26.16 | 90 | 270 | 25.96 | 0.20 | NOTE 1 |
|  |  |  | 64Q | 90 | 255 | 23.73 | 90 | 270 | 23.71 | 0.02 | NOTE 1 |
|  |  | CP | QPSK | 90 | 270 | 24.84 | 90 | 270 | 24.84 | 0.00 | NOTE 2 |
|  |  |  | 16Q | 90 | 270 | 23.60 | 90 | 270 | 23.60 | 0.00 | NOTE 2 |
|  |  |  | 64Q | 75 | 255 | 21.54 | 90 | 270 | 21.29 | 0.25 | NOTE 1 |
|  | M | An incorrect channel was measured |  |  |  |  |  |  |  |  |  |  |
|  | H | DFT | QPSK | 15 | 210 | 29.56 | 15 | 210 | 29.56 | 0.00 | Reference |
|  |  |  | 16Q | 165 | 150 | 27.11 | 15 | 210 | 27.08 | 0.03 | NOTE 1 |
|  |  |  | 64Q | 15 | 210 | 24.95 | 15 | 210 | 24.95 | 0.00 | NOTE 2 |
|  |  | CP | QPSK | 15 | 210 | 26.32 | 15 | 210 | 26.28 | 0.04 | NOTE 2 |
|  |  |  | 16Q | 15 | 210 | 25.24 | 15 | 210 | 25.24 | 0.00 | NOTE 2 |
|  |  |  | 64Q | 165 | 150 | 23.16 | 15 | 210 | 23.13 | 0.03 | NOTE 1 |
| n261 | L | DFT | QPSK | 150 | 165 | 29.51 | 150 | 165 | 29.51 | 0.00 | Reference |
|  |  |  | 16Q | 30 | 195 | 27.21 | 150 | 165 | 27.09 | 0.12 | NOTE 1 |
|  |  |  | 64Q | 30 | 195 | 25.16 | 150 | 165 | 25.08 | 0.08 | NOTE 1 |
|  |  | CP | QPSK | 30 | 195 | 26.16 | 150 | 165 | 25.99 | 0.17 | NOTE 1 |
|  |  |  | 16Q | 30 | 195 | 25.00 | 150 | 165 | 24.86 | 0.15 | NOTE 1 |
|  |  |  | 64Q | 30 | 195 | 22.95 | 150 | 165 | 22.86 | 0.09 | NOTE 1 |
|  | M | DFT | QPSK | 165 | 180 | 29.60 | 165 | 180 | 29.60 | 0.00 | Reference |
|  |  |  | 16Q | 165 | 180 | 27.40 | 165 | 180 | 27.39 | 0.01 | NOTE 2 |
|  |  |  | 64Q | 150 | 165 | 25.32 | 165 | 180 | 25.30 | 0.02 | NOTE 1 |
|  |  | CP | QPSK | 105 | 285 | 26.49 | 165 | 180 | 26.36 | 0.13 | NOTE 1 |
|  |  |  | 16Q | 105 | 285 | 25.42 | 165 | 180 | 25.42 | 0.01 | NOTE 1 |
|  |  |  | 64Q | 15 | 180 | 22.98 | 165 | 180 | 22.93 | 0.05 | NOTE 1 |
|  | H | An incorrect channel was measured |  |  |  |  |  |  |  |  |  |  |
| NOTES | NOTE 1: Beam peak direction for this config was different than reference Beam peak direction NOTE 2: Beam peak direction for this config was the same as the reference Beam peak direction |  |  |  |  |  |  |  |  |  |  |  |

Observation 2: The beam peak results at the reference beam peak direction do not change between different modulations and waveforms

Even though the dataset is very limited, but given the trend of the data presented in Table 1, it is proposed that beam peak searches shall be performed for every test frequency range by default unless the device manufacturer explicitly declares that the beam peak at the QPSK modulation is applicable for the remaining 16QAM and 64QAM modulation.

Proposal 2: Beam peak searches shall be performed for every modulation by default unless the device manufacturer explicitly declares that the beam peak at the QPSK modulation is applicable for the remaining 16QAM and 64QAM modulation

The results in Table 1 also show that the beam peak directions for the two waveforms (CP-OFDM and DFTs-OFDM) are the same or similar with some deviations. Similar as above, it is proposed that beam peak searches shall be performed for every waveform by default unless the device manufacturer explicitly declares that the beam peak from one waveform is applicable for the other waveform.

Proposal 3: Beam peak searches shall be performed for every waveform by default unless the device manufacturer explicitly declares that the beam peak from one waveform is applicable for the other waveform.

# Conclusion

The following observations and proposals were made in this contribution

Observation 1: Whether the beam peak searches need to be performed for different channel bandwidths, SCS, modulation, waveforms has not been decided yet.

Observation 2: The beam peak results at the reference beam peak direction do not change between different modulations and waveforms

Proposal 1: Beam peak searches shall be performed for every CBW and SCS by default.

Proposal 2: Beam peak searches shall be performed for every modulation by default unless the device manufacturer explicitly declares that the beam peak at the QPSK modulation is applicable for the remaining 16QAM and 64QAM modulation

Proposal 3: Beam peak searches shall be performed for every waveform by default unless the device manufacturer explicitly declares that the beam peak from one waveform is applicable for the other waveform.

# References

- R5-202939, On Beam Peak Searches for different test configurations, Keysight Technologies, 3GPP TSG-RAN5 Meeting #87-e, May 2020

- R5-204188, CR to 38.521-2 to allow vendor declarations related to beam peak searches, Keysight Technologies, 3GPP TSG-RAN5 Meeting #88-e, August 2020

- R5-204187, CR to 38.508-2 to allow vendor declarations related to beam peak searches, Keysight Technologies, 3GPP TSG-RAN5 Meeting #88-e, August 2020