3GPP TSG-RAN5 Meeting #89-e	R5-205839

# Electronic Meeting, 9th – 20th November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0797 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New EN-DC Rel-16 band combo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2020-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New EN-DC Rel-16 band combo defined by RAN4 with exception are currenty missing in the RAN5 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of New EN-DC_14A-66A-66A_n2A in test configuration table and requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New band requirement could not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

Table 7.3B.2.3_1.2.4.1-2: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB" column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YC_nZA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_1A-3C_n78A Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | 1 | N/A | NOTE 4 | NOTE 5 |  | QPSK | QPSK | NOTE 4 | All RBs |
|  | SCC1(M) | 3 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | 3 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | 78 | 15 kHz | NOTE 4 | NOTE 5 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | NOTE 4 | All RBs |
| 2 | PCC(M) | 3 | N/A | NOTE 5 | NOTE 5 |  | QPSK | QPSK | NOTE 5 | All RBs |
|  | SCC1(M) | 3 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | 1 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | 78 | 15 kHz | NOTE 5 | NOTE 5 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | NOTE 5 | All RBs |
| Test Settings for a DC_14A-66A-66AA_n2A Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3) |  |  |  |  |  |
| 2 |  |  |  |  | No test required, EN-DC configuration affected by 2UL intermodulation exception is tested in 7.3B.2.3_1.1) |  |  |  |  |  |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. NOTE 4: As per DC_1A-n78A configurations in Table 7.3B.2.3.4.2.1-5. NOTE 5: As per DC_3A-n78A configurations in Table 7.3B.2.3.4.2.1-2_7, Table 7.3B.2.3.4.2.1-2_8 and Table 7.3B.2.3.4.2.1-5. NOTE 6: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.2.4.1-3: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (four bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB" column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YA-ZA_nRA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | R | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_1A-3A-5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | 1 | N/A | NOTE 4 | NOTE 4 |  | QPSK | QPSK | NOTE 4 | All RBs |
|  | SCC1(M) | 3 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | 5 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | 78 | 15 kHz | NOTE 4 | NOTE 4 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | NOTE 4 | All RBs |
| 2 | PCC(M) | 3 | N/A | NOTE 5 | NOTE 5 |  | QPSK | QPSK | NOTE 4 | All RBs |
|  | SCC1(M) | 1 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | 5 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | 78 | 15 kHz | NOTE 5 | NOTE 5 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | NOTE 4 | All RBs |
| 3 | PCC(M) | 5 | N/A | NOTE 6 | NOTE 6 |  | QPSK | QPSK | NOTE 5 | All RBs |
|  | SCC1(M) | 3 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | 1 | N/A | 5MHz | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | 78 | 15 kHz | NOTE 6 | NOTE 6 |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | NOTE 5 | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different E-UTRA bands i.e. X != Y != Z, R corresponds to NR band. NOTE 3: For inter-band EN-DC 4CCs configuration with four bands, if there is no additional exceptional test point is defined besides the requirement for default uplink EN-DC configurations defined in Table 5.5B.4.3-1, testing for 4CC with four band can be skipped. NOTE 4: Same as DC_1A-n78A configuration defined in Table 7.3B.2.3.4.2.1-5. NOTE 5: Same as DC_3A-n78A configurations defined in Table 7.3B.2.3.4.2.1-2_7, Table 7.3B.2.3.4.2.1-2_8 and Table 7.3B.2.3.4.2.1-5. NOTE 6: Same as DC_5A-n78A configurations defined in Table 7.3B.2.3.4.2.1-2_9, and Table 7.3B.2.3.4.2.1-5. NOTE 7: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1.2.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2

7.3B.2.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

7.3B.2.3_1.2.5		Test Requirement

For inter-band EN-DC configurations, the throughput of each CG shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with reference receive power level specified in Tables Table 7.3.2.5-1 in TS 38.521-1 [8] and parameters specified Tables 7.3.2.4.1-1, Tables 7.3.2.4.1-2 and Tables 7.3.2.4.1-3 in TS 38.521-1 [8] for NR band.

Each EN-DC combination defined in Table 5.5B.4.3-1 shall be tested in ancher-agnoutic anchor-agnostic mode as described in this clause. If a test point is overlapped with uplink EN-DC combo MSD test points, the requirement with MSD shall apply.

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable for two, three, four and five bands operation.

Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3_1.2.5-1 and Table 7.3B.2.3_1.2.5-2 with uplink configuration specified in Table 7.3B.2.3_1.2.4.1-1 and 7.3B.2.3_1.2.4.1-2.

For inter-band EN-DC 4CCs configuration with four bands, if there is no additional exceptional test point is defined besides the requirement for default uplink EN-DC configurations defined in Table 5.5B.4.3-1,  the test requirement for default fallback uplink EN-DC configurations defined in Table 5.5B.4.3-1 applies.

Table 7.3B.2.3_1.2.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

FFS

Table 7.3B.2.3_1.2.5-2: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | Test ID | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_3C_n78A | 1 | 1 | N/A | -91.3 | - | - | - | FDD | IMD4 |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | 3 | N/A | REFSENS |  | - | - | FDD | N/A |  |
|  |  | n78 | 15 | -71.9 +TT | -71.9 +TT | -71.8 +TT | - | TDD | N/A |  |
|  |  | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | 3 | N/A | REFSENS |  | - | - | FDD | N/A |  |
|  |  | n78 | 15 | -94.7 +TT | -93.2 +TT | -92.4 +TT | - | TDD | N/A |  |
|  |  | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -70.3 |  |  |  | FDD | IMD2 |  |
|  |  | 3 | N/A | -70.3 |  | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -88.3 |  |  |  | FDD | IMD4 |  |
|  |  | 3 | N/A | -88.3 |  | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_14A_66A_66A_n2A | 1 | 14 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -91.2 | - | - | - | FDD | IMD4 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n2 | 15 | REFSENS | - | - | - | TDD | N/A |  |
| NOTE 1: Both of the transmitters shall be set min (+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |  |  |  |

Test tolerance is the same as given in Table 7.3B.2.3.5-2.

## << End of changes >>

7.3B.2.3_1.3	Reference sensitivity for EN-DC within FR1 (5 CCs)