Source:		Rohde & Schwarz

Title:	Discussion on test point selection for Carrier Leakage in FR2

# Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the Carrier Leakage test case is analyzed.

# Discussion

The Carrier Leakage describes to which extend the unmodulated carrier is present in the transmitted signal and is defined in section 6.4.2.2 and Annex F Transmit Modulation of TS 38.101-2 [1].

The minimum requirement is specified as follows:

6.4.2.2	Carrier leakage

6.4.2.2.1	General

Carrier leakage is an additive sinusoid waveform. The carrier leakage requirement is defined for each component carrier. The measurement interval is one slot in the time domain. The relative carrier leakage power is a power ratio of the additive sinusoid waveform to the power in the modulated waveform.

The requirement is verified with the test metric of Carrier Leakage (Link=TX beam peak direction, Meas=Link angle).

6.4.2.2.2	Carrier leakage for power class 1

When carrier leakage is contained inside the spectrum confined within the configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.2-1 for power class 1 UEs.

Table 6.4.2.2.2-1: Minimum requirements for relative carrier leakage power for power class 1

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 17 dBm | -25 |
| 4 dBm ≤ EIRP ≤ 17 dBm | -20 |

6.4.2.2.3	Carrier leakage for power class 2

6.4.2.2.4	Carrier leakage for power class 3

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.4-1 for power class 3 UEs.

Table 6.4.2.2.4-1: Minimum requirements for relative carrier leakage power for power class 3

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 0 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 0 dBm | -20 |

6.4.2.2.5	Carrier leakage for power class 4

## Test Environment

In LTE, Carrier Leakage is tested under extreme conditions. However, since Carrier Leakage is not expected to vary significantly with temperature or supply voltage, it is proposed to apply normal condition as test environment in order to save test time.

Proposal 1: Define test environment as Normal for Carrier Leakage in FR2.

## Test Subcarrier Spacing

As the Carrier Leakage is a sinusoidal unmodulated signal it is not expected that its power level depends on the choice of subcarrier spacing. Smallest SCS is selected since it is the most common one.

Proposal 2: Select smallest SCS for Carrier Leakage in FR2.

## Test Frequencies

In LTE, Carrier Leakage is tested for Low range, Mid range, and High range. For a modulator, the suppression of the carrier typically depends on the operating frequency. The same test coverage as for LTE is needed to ensure that the Carrier Leakage obeys the test requirement for all frequencies.

Proposal 3: Test Carrier Leakage for Low range, Mid range, and High range frequencies in FR2.

## Test Channel Bandwidth

In LTE, Carrier Leakage is tested for Lowest, 5 MHz and Highest channel bandwidths. However it is not expected that the carrier leakage power level depends on the channel bandwidth.

Proposal 4: Test Carrier Leakage for Mid channel bandwidth in FR2.

## Test Modulations and waveforms

In LTE Carrier Leakage is tested for test points without MPR. DFT-s-OFDM QPSK should be selected as modulation and waveform as it has the lowest MPR for power class 3 and is a common modulation.

Proposal 5: Test Carrier Leakage with DFT-s-OFDM QPSK in FR2.

## RB allocation

In LTE, Carrier Leakage is tested with highest allocation without MPR where the partial allocation is located on the lower and upper channel edge. For power class 1 the high MPRnarrow of 10 dB for an allocated bandwidth less or equal to 10.8 MHz shall be avoided which requires an allocation of at least 16 RBs for DFT-s-OFDM. When this is fulfilled the MPRWT applies which is smaller for Inner RB allocations than for outer RB allocations. For a CHBW of 50MHz an Inner_16RB_Left_Region1 allocation does not fulfill the test purpose when assuming that the carrier is at the center of the channel as the RBs at the carrier would be allocated. Therefore, with an Inner-Region1 allocation MPRnarrow cannot be avoided. As a remedy an Inner-Region2 allocation is applied since this results in an MPR of only 3dB for 50 MHz channel bandwidth instead of 10dB for MPRnarrow.

For PC3, to avoid MPR is more crucial than for PC1 in order to minimize the measurement uncertainty due to impact of noise. For PC3, there is no MPRnarrow for an Inner allocation. Therefore, an allocation of Inner_4RB is selected to fulfil the test purpose and optimize the MU.

For 5G the DC carrier is no longer fixed at the center of the channel bandwidth but can be located anywhere inside the channel. However, Carrier Leakage can only be measured when the RB at the position of the DC carrier is not allocated. Therefore, if the signaled carrier position is falling inside the allocation a different allocation needs to be applied.

Proposal 6: For PC1, test Carrier Leakage in FR2 with Inner_16RB_Left_Region2. In case the signaled carrier position is at Inner_16RB_Left_Region2, use Inner_16RB_Right_Region2. 
For PC2, PC3, and PC4, test Carrier Leakage in FR2 with Inner_4RB_Left. In case the signaled carrier position is at Inner_4RB_Left, use Inner_4RB_Right.

# Conclusion

To sum up, the following selection of test points is proposed for Carrier Leakage in FR2:

Proposal 1: Define test environment as Normal for Carrier Leakage in FR2.

Proposal 2: Select smallest SCS for Carrier Leakage in FR2.

Proposal 3: Test Carrier Leakage for Low range, Mid range, and High range frequencies in FR2.

Proposal 4: Test Carrier Leakage for Mid channel bandwidth in FR2.

Proposal 5: Test Carrier Leakage with DFT-s-OFDM QPSK in FR2.

Proposal 6: For PC1, test Carrier Leakage in FR2 with Inner_16RB_Left_Region2. In case the signaled carrier position is at Inner_16RB_Left_Region2, use Inner_16RB_Right_Region2. 
For PC2, PC3, and PC4, test Carrier Leakage in FR2 with Inner_4RB_Left. In case the signaled carrier position is at Inner_4RB_Left, use Inner_4RB_Right.

# Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR2.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 1 | 1 | 1 | 3 |

# References

- R4-1811910, Big CR for 38.101-2, 3GPP TSG RAN4, September 2018