3GPP TSG-RAN5 Meeting #89-e	R5-206174r2

# Online, , 9th Nov 2020 - 20th Nov 2020

Title:						On n259 MU and testability limit

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

In RAN5#88-e, it was endorsed to call for the input on the n259 MU and testability limit [1] for some remaining test cases listed in Table 1.

Table 1 Status of n259 MU and relaxation upper limit [1]

| TC | Upper limit (MU for TT calculation) | Upper limit (relaxation due to testability issue) |
| --- | --- | --- |
| MOP (Max TRP) | 5.5 dB [2] | TBD |
| REFSENS | 5.8 dB [2] | TBD |
| Frequency Error | 0.01 PPM [2] | TBD |
| SEM | 5.8 dB [2] | TBD |
| OFF power | TBD (Treated in [3]) | TBD |
| OBW | Not required because TT = 0 dB [4] | TBD |
| ACLR | TBD | TBD |
| General spurious | Not required because TT = 0 dB [4] | TBD |
| Spurious co-existence | Not required because TT = 0 dB [4] | TBD |
| Inband blocking | Not required because TT = 0 dB [4] | TBD |
| ACS | Not required because TT = 0 dB [4] | TBD |
| Rx spurious | Not required because TT = 0 dB [4] | TBD |

# 2.	Discussion

# Our view on the upper limit assumption for n259 MU and relaxation used for TT determination  is provided in Table 2.

Table 2 Status of n259 MU and relaxation upper limit [1]

| TC | Proposal for max MU assumption for TT determination | Proposal for max relaxation for TT determination | Note, Consideration |
| --- | --- | --- | --- |
| MOP (Max TRP) | - | No relaxation required. |  |
| REFSENS | - | No relaxation required. |  |
| Frequency Error | - | No relaxation required. |  |
| SEM | - | No relaxation required. | (DSNR=1.0) |
| OFF power | Same as FR2b MU | FR2b relaxation(33.1) + 0.73dB +1.0dB = 34.83dB | (0.73+1.0: FSPL and conducted loss increase, keep DSNR=1.0) |
| OBW | << Under calculation now… >> | << Under calculation now… >> |  |
| ACLR | Same as FR2b MU | Values in Table4 |  |
| General spurious | - | No relaxation required | (possible DSNR increase) |
| Spurious co-existence | - | This was already discussed in RAN5#88-e and fixed. |  |
| Inband blocking | - | See Table 7 |  |
| ACS | - | See Table 7 |  |
| Rx spurious | - | f <= 66GHz : Same as existing value. 66GHz <= f <= 87GHz 66GHz relaxation(33.1) + 2.4+1.0 dB = 36.5dB | (2.4+1.0 : FSPL increase and conducted loss against 66GHz) |

# ACLR

# Maximum Testable MPR without relaxation(threshold SNR=6.0dB) for FR2 ACLR test case is listed in Table 3. FR2c values are derived based on 0.73dB+1dB worse floor noise than FR2b. 0.73dB+1dB is corresponding to FSPL and conducted loss difference from FR2b. The estimated relaxation for each MPR is listed in Table 4.

Table 3 FR2c max testable MPR for ACLR test case (single carrier)

|  | FR2c |
| --- | --- |
| ChBW (50MHz) | 4.5 |
| ChBW (100MHz) | 3.0 |
| ChBW (200MHz) | 1.0 |
| ChBW (400MHz) | <0 |

Table 4 Estimation Relaxation for FR2c ACLR

| Test Point MPR | T(MPR) | 400MHz | 200MHz | 100MHz | 50Mhz |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0.28 | 0.00 | 0.00 | 0.00 |
| 0.5 | 1.5 | 2.28 | 0.00 | 0.00 | 0.00 |
| 1 | 1.5 | 2.78 | 0.00 | 0.00 | 0.00 |
| 1.5 | 1.5 | 3.28 | 0.27 | 0.00 | 0.00 |
| 2 | 1.5 | 3.78 | 0.77 | 0.00 | 0.00 |
| 2.5 | 2 | 4.78 | 1.77 | 0.00 | 0.00 |
| 3 | 2 | 5.28 | 2.27 | 0.00 | 0.00 |
| 3.5 | 3 | 6.78 | 3.77 | 0.76 | 0.00 |
| 4 | 3 | 7.28 | 4.27 | 1.26 | 0.00 |
| 4.5 | 4 | 8.78 | 5.77 | 2.76 | 0.00 |
| 5 | 4 | 9.28 | 6.27 | 3.26 | 0.25 |
| 5.5 | 5 | 10.78 | 7.77 | 4.76 | 1.75 |
| 6 | 5 | 11.28 | 8.27 | 5.26 | 2.25 |
| 6.5 | 5 | 11.78 | 8.77 | 5.76 | 2.75 |
| 7 | 5 | 12.28 | 9.27 | 6.26 | 3.25 |
| 7.5 | 5 | 12.78 | 9.77 | 6.76 | 3.75 |
| 8 | 5 | 13.28 | 10.27 | 7.26 | 4.25 |
| 8.5 | 5 | 13.78 | 10.77 | 7.76 | 4.75 |
| 9 | 5 | 14.28 | 11.27 | 8.26 | 5.25 |
| 9.5 | 5 | 14.78 | 11.77 | 8.76 | 5.75 |

# Blocking Test

# As for IBB and ACS Case 1 has the same power level for interfere, we estimate testability limit based on ACS test case.

# For ACS Case 1 and IBB, Interfere power for n259 can be calculated as REFSESN + MBR(0.75) + Offset value against REFSENS as hown in Table 7.5-2 of 38.101-2, and are summarized as below. For ACS Case 2, interfere power is -25dBm.

Table 5 Interferer power for ACS Case 1 and IBB for n259

|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| Pinterferer | -49.45 | -46.45 | -43.45 | -40.45 |

# For the feasible DL power limit, if we reuse already agree value in [2] for FR2a and FR2b, feasible DL power for n259 would be estimated by 0.73+1dB smaller than that for FR2b where 0.73dB+1dB corresponding to the difference of FSPL and conducted loss against FR2b.

Table 6: Feasible Interferer power levels and relaxation values for single carrier

| Band | Feasible Interferer power (dBm) |
| --- | --- |
| n258/n257/n261 | -52.8 |
| n260 | -56 |
| n259 | -56-0.73-1.0=-57.73 |

# Comparing the feasible DL power and interferer power specified by test cases, following are the estimated required relaxation.

Table 7 Estimated relaxation for ACS and IBB for n259

|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- | --- |
| ACS Case1, IBB | Pinterferer [dBm] | -49.45 | -46.45 | -43.45 | -40.45 |
|  | Relaxation [dB] | 8.28 | 11.28 | 14.28 | 17.28 |
| ACS Case 2 | Pinterferer [dBm] | -25 | -25 | -25 | -25 |
|  | Relaxation [dB] | 32.73 | 32.73 | 32.73 | 32.73 |

- Conclusion

Proposal 1 : Use Table 1 for determination of n259 MU and relaxation upper limit assumption for TT determination.

# 4.	References

[1] R5-204851, “Update of WF on Band n259”, NTT Docomo, RAN5#88e

[2] R5-197497, Discussion on testability of ACS and IBB, Rohde & Schwarz, Anritsu, Keysight Technologies, 3GPP TSG RAN WG 5 Meeting #84, August 2019