## 4.6	Default NG-RAN RRC message and information elements contents

### 4.6.0	General

#### 4.6.0.1	Global conditions

Groups of RRC conditions including always one condition set to true.

Table 4.6.0.1-1: Signalling, RF/RRM/Performance

| Condition | Explanation |
| --- | --- |
| SIG | Used for signalling test cases |
| RF | Used for RF/RRM/Performance test cases |

Table 4.6.0.1-2: NR operating bands

| Condition | Explanation |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

Table 4.6.0.1-3: FDD/TDD

| Condition | Explanation |
| --- | --- |
| FDD | Frequency Division Duplex |
| TDD | Time Division Duplex |

Table 4.6.0.1-4: Subcarrier spacing

| Condition | Explanation |
| --- | --- |
| SCS15 | 15kHz |
| SCS30 | 30kHz |
| SCS60 | 60kHz |
| SCS120 | 120kHz |
| SCS240 | 240kHz |

### 4.6.1	Contents of RRC messages

#### –	CounterCheck

Table 4.6.1-1: CounterCheck

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CounterCheck ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| counterCheck SEQUENCE { |  |  |  |  |
| drb-CountMSB-InfoList SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info |  | 1 entry |  |  |
| DRB-CountMSB-Info[1] SEQUENCE { |  |  | entry 1 |  |
| drb-Identity |  | DRB-Identity |  |  |
| countMSB-Uplink |  | 0 |  |  |
| countMSB-Downlink |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	CounterCheckResponse

Table 4.6.1-2: CounterCheckResponse

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CounterCheckResponse ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| counterCheckResponse SEQUENCE { |  |  |  |  |
| drb-CountInfoList SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo { |  | 1 entry |  |  |
| DRB-CountInfo[1] SEQUENCE { |  |  | entry 1 |  |
| drb-Identity |  | DRB-Identity |  |  |
| count-Uplink |  | Not checked |  |  |
| count-Downlink |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	DedicatedSIBRequest

Table 4.6.1-2A: DedicatedSIBRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DedicatedSIBRequest-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	DLDedicatedMessageSegment

Table 4.6.1-2B: DLDedicatedMessageSegment

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	DLInformationTransfer

Table 4.6.1-3: DLInformationTransfer

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message |  | DedicatedNAS-Message |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	DLInformationTransferMRDC

Table 4.6.1-3A: DLInformationTransferMRDC

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransferMRDC-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	FailureInformation

Table 4.6.1-4: FailureInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FailureInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| failureInformation SEQUENCE { |  |  |  |  |
| failureInfoRLC-Bearer SEQUENCE { |  |  |  |  |
| cellGroupId |  | Not checked |  |  |
| logicalChannelIdentity |  | Not checked |  |  |
| failureType |  | Not checked |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	IABOtherInformation

Table 4.6.1-4A: IABOtherInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| IABOtherInformation-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	LocationMeasurementIndication

Table 4.6.1-5: LocationMeasurementIndication

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LocationMeasurementIndication ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| locationMeasurementIndication SEQUENCE { |  |  |  |  |
| measurementIndication CHOICE { |  |  |  |  |
| setup |  | LocationMeasurementInfo |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	LoggedMeasurementConfiguration

Table 4.6.1-5AA: LoggedMeasurementConfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	MCGFailureInformation

Table 4.6.1-5AB: MCGFailureInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MCGFailureInformation-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	MeasurementReport

Table 4.6.1-5A: MeasurementReport

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| measurementReport SEQUENCE { |  |  |  |  |
| measResults |  | MeasResults |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	MIB

Table 4.6.1-6: MIB

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MIB ::= SEQUENCE { |  |  |  |  |
| systemFrameNumber |  | A valid value as defined in TS 38.331 [6] |  |  |
| subCarrierSpacingCommon |  | scs15or60 | For signalling test cases see clause 6.2.3, otherwise see clause 4.3.1. | SCS15or60 |
|  |  | scs30or120 | For signalling test cases see clause 6.2.3, otherwise see clause 4.3.1. | SCS30or120 |
| ssb-subcarrierOffset |  | Set to the integer value of the 4 LSB of kSSB defined for the frequency of the cell | For signalling test cases see clause 6.2.3, otherwise see clause 4.3.1. |  |
| dmrs-TypeA-Position |  | pos2 |  |  |
| pdcch-ConfigSIB1 |  | PDCCH-ConfigSIB1 |  |  |
| cellBarred |  | notBarred |  |  |
| intraFreqReselection |  | allowed |  |  |
| spare |  | 0 |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS15or60 | SCS is 15kHz or 60kHz |
| SCS30or120 | SCS is 30kHz or 120kHz. |

#### –	MobilityFromNRCommand

Table 4.6.1-8: MobilityFromNRCommand

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromNRCommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| mobilityFromNRCommand SEQUENCE { |  |  |  |  |
| targetRAT-Type |  | eutra |  |  |
| targetRAT-MessageContainer |  | OCTET STRING including the RRCConnectionReconfiguration message according TS 36.508 [2], table 4.6.1-8 with condition HO-TO-EUTRA |  |  |
| nas-SecurityParamFromNR |  | The 4 LSB of the downlink NAS COUNT |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	Paging

Table 4.6.1-9: Paging

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |  |
| pagingRecordList SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord { |  | 1 entry |  |  |
| PagingRecord[1] SEQUENCE { |  |  | entry 1 |  |
| ue-Identity CHOICE { |  |  |  |  |
| ng-5G-S-TMSI |  | NG-5G-S-TMSI |  |  |
| fullI-RNTI |  | I-RNTI-Value |  | NR_RRC_RESUME |
| } |  |  |  |  |
| accessType |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR_RRC_RESUME | To page a UE in RRC_INACTIVE state to request RRC connection resumption |

#### –	RRCReestablishment

Table 4.6.1-10: RRCReestablishment

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCReestablishmentComplete

Table 4.6.1-11: RRCReestablishmentComplete

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCReestablishmentRequest

Table 4.6.1-12: RRCReestablishmentRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |  |
| c-RNTI |  | RNTI-Value |  |  |
| physCellId |  | PhysCellId | The physical cell identity of the PCell the UE was connected to prior to the failure |  |
| shortMAC-I |  | ShortMAC-I |  |  |
| } |  |  |  |  |
| reestablishmentCause |  | Not checked |  |  |
| spare |  | Present but contents not checked |  |  |
| } |  |  |  |  |

#### –	RRCReconfiguration

Table 4.6.1-13: RRCReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
|  |  | RadioBearerConfig with conditions SRB2 and DRB1 |  | NR, NR-DC |
| secondaryCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  |  | CellGroupConfig with condition NR-DC_SCG | OCTET STRING (CONTAINING CellGroupConfig) | NR-DC_SCG |
|  |  | CellGroupConfig with conditions EN-DC and PSCell_change | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_HO |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_MEAS |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_SCell_add |
|  |  | Not present |  |  |
| measConfig |  | Not present |  |  |
|  |  | MeasConfig | Measurements configuration | NR_MEAS, EN-DC_MEAS |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR, SCell_add, NR_MEAS, NR-DC |
| masterCellGroup |  | CellGroupConfig with condition SRB2_DRB1 | OCTET STRING (CONTAINING CellGroupConfig) | NR, NR-DC |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | SCell_add |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | NR_MEAS |
| fullConfig |  | Not present |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message { |  | 1 entry |  | NR |
| DedicatedNAS-Message[1] |  | DedicatedNAS-Message | entry 1 A sequence of OCTET STRING (s) containing one or more DedicatedNAS-Message(s) |  |
| } |  |  |  |  |
| masterKeyUpdate |  | Not present |  |  |
| masterKeyUpdate SEQUENCE { |  |  |  | MasterKeyChange |
| keySetChangeIndicator |  | true |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| nas-Container |  | Not present |  |  |
| nas-Container |  | OCTET STRING including the 10 Octets value generated according to TS 24.501 [28] clause 9.11.2.9 |  | Inter_Sys_HO |
| dedicatedSIB1-Delivery |  | Not present |  |  |
| dedicatedSystemInformationDelivery |  | Not present |  |  |
| otherConfig |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR-DC |
| otherConfig |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |
| nr-SCG |  | RRCReconfiguration with condition NR-DC_SCG | OCTET STRING (CONTAINING RRCReconfiguration) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioBearerConfig2 |  | Not present |  |  |
| sk-Counter |  | SK-Counter |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| EN-DC_MEAS | An EN-DC measurement is configured |
| NR_MEAS | A NR measurement is configured |
| NR | NG-RAN NR Radio Access |
| NR-DC | NR-NR Dual Connectivity |
| NR-DC_SCG | Add SCG (NR-DC) |
| EN-DC_HO | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| EN-DC_SCell_add | Add Scell (EN-DC) |
| MasterKeyChange | MasterKeyUpdate when peforming ReconfigurationWithSync and indicating a change of the AS security algorithms associated to the master key. |
| Inter_Sys_HO | Used during inter-system handover to NR procedure. |

#### –	RRCReconfigurationComplete

Table 4.6.1-14: RRCReconfigurationComplete

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCReject

Table 4.6.1-15: RRCReject

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReject ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReject SEQUENCE { |  |  |  |  |
| waitTime |  | 1 |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCRelease

Table 4.6.1-16: RRCRelease

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| redirectedCarrierInfo |  | Not present |  |  |
| cellReselectionPriorities |  | Not present |  |  |
| suspendConfig |  | Not present |  |  |
| suspendConfig SEQUENCE { |  |  |  | NR_RRC_INACTIVE |
| fullI-RNTI |  | I-RNTI-Value |  |  |
| shortI-RNTI |  | ShortI-RNTI-Value |  |  |
| ran-PagingCycle |  | rf32 |  |  |
| ran-NotificationAreaInfo CHOICE { |  |  |  |  |
| cellList SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell { |  | 1 entry |  |  |
| PLMN-RAN-AreaCellList[1] SEQUENCE { |  |  | entry 1 |  |
| plmn-Identity |  | Not present |  |  |
| ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity { |  | 1 entry |  |  |
| CellIdentity[1] |  | CellIdentity | entry 1 Cellidentity for the used cell. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| t380 |  | Not present |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| } |  |  |  |  |
| deprioritisationReq |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR_RRC_INACTIVE | NR RRC state RRC_INACTIVE |

#### –	RRCResume

Table 4.6.1-17: RRCResume

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResume SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition RESUME |  |  |
| masterCellGroup |  | CellGroupConfig with condition RESUME | OCTET STRING (CONTAINING CellGroupConfig) |  |
| measConfig |  | Not present |  |  |
| fullConfig |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCResumeComplete

Table 4.6.1-18: RRCResumeComplete

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message |  | Not checked |  |  |
| selectedPLMN-Identity |  | Not checked |  |  |
| uplinkTxDirectCurrentList |  | Not checked |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCResumeRequest

Table 4.6.1-19: RRCResumeRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |  |
| resumeIdentity |  | ShortI-RNTI-Value |  |  |
| resumeMAC-I |  | Not checked |  |  |
| resumeCause |  | ResumeCause |  |  |
| spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCResumeRequest1

Table 4.6.1-20: RRCResumeRequest1

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeRequest1 ::= SEQUENCE { |  |  |  |  |
| rrcResumeRequest1 SEQUENCE { |  |  |  |  |
| resumeIdentity |  | I-RNTI-Value |  |  |
| resumeMAC-I |  | Not checked |  |  |
| resumeCause |  | ResumeCause |  |  |
| spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCSetup

Table 4.6.1-21: RRCSetup

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition SRB1 |  |  |
| masterCellGroup |  | CellGroupConfig with condition SRB1 | OCTET STRING (CONTAINING CellGroupConfig) |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCSetupComplete

Table 4.6.1-22: RRCSetupComplete

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | Not checked |  |  |
| registeredAMF |  | Not checked |  |  |
| guami-Type |  | Not checked |  |  |
| s-nssai-List |  | Not checked |  |  |
| dedicatedNAS-Message |  | DedicatedNAS-Message |  |  |
| ng-5G-S-TMSI-Value |  | Not checked |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCSetupRequest

Table 4.6.1-23: RRCSetupRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| ue-Identity CHOICE { |  |  |  |  |
| randomValue |  | Not checked |  |  |
| } |  |  |  |  |
| establishmentCause |  | Not checked |  |  |
| spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCSystemInfoRequest

Table 4.6.1-24: RRCSystemInfoRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSystemInfoRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSystemInfoRequest SEQUENCE { |  |  |  |  |
| requested-SI-List |  | Not checked |  |  |
| spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	SCGFailureInformation

Table 4.6.1-24A: SCGFailureInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SCGFailureInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| scgFailureInformation SEQUENCE { |  |  |  |  |
| failureReportSCG |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	SCGFailureInformationEUTRA

Table 4.6.1-24B: SCGFailureInformationEUTRA

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SCGFailureInformationEUTRA ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| scgFailureInformationEUTRA SEQUENCE { |  |  |  |  |
| failureReportSCG-EUTRA |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	SecurityModeCommand

Table 4.6.1-25: SecurityModeCommand

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| securityModeCommand SEQUENCE { |  |  |  |  |
| securityConfigSMC SEQUENCE { |  |  |  |  |
| securityAlgorithmConfig |  | SecurityAlgorithmConfig |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	SecurityModeComplete

Table 4.6.1-26: SecurityModeComplete

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| securityModeComplete SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	SecurityModeFailure

Table 4.6.1-27: SecurityModeFailure

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeFailure ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| securityModeFailure SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	SIB1

Table 4.6.1-28: SIB1

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 dBm | RF |
|  |  | -55 | -110 dBm | SIG AND FR1 |
|  |  | ROUND((-110+Delta(NRfs))/2) | -110+Delta(NRfs) | SIG AND FR2 |
| q-RxLevMinOffset |  | Not present |  |  |
| q-RxLevMinSUL |  | Not Present |  |  |
|  |  | -70 | -140 dBm | SUL AND RF |
|  |  | -55 | -110 dBm | SUL AND SIG |
| q-QualMin |  | -20 | -20dB | QBASED |
|  |  | Not present |  |  |
| q-QualMinOffset |  | Not present |  |  |
| } |  |  |  |  |
| cellAccessRelatedInfo |  | CellAccessRelatedInfo |  |  |
| connEstFailureControl |  | ConnEstFailureControl |  |  |
| si-SchedulingInfo |  | Not present |  | NR_1 |
|  |  | SI-SchedulingInfo |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB |  |  |
| ims-EmergencySupport |  | Not present |  |  |
| eCallOverIMS-Support |  | Not present |  |  |
| ue-TimersAndConstants |  | UE-TimersAndConstants |  |  |
| uac-BarringInfo |  | Not present |  |  |
| useFullResumeID |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| NOTE 1: Delta(NRfs) is derived based on calibration procedure defined in the clause 6.1.3.3. NRfs is NR frequency on which SIB1 is broadcasted. NOTE 2: ROUND is rounded off to the nearest integer. As an example, '1 to 1.49' set to '1' while '1.5 to 2' to '2' and '-2.0 to 1.5' set to '-2' while '-1.49 to -1' set to '-1'. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMinSUL. |
| QBASED | This condition applies to Quality based signalling test cases. |
| NR_1 | System information combination NR-1 according table 4.4.3.1.2-1 is applied. |

#### –	SidelinkUEInformationNR

Table 4.6.1-28A: SidelinkUEInformationNR

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SidelinkUEInformationNR-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	SystemInformation

Table 4.6.1-29: SystemInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| systemInformation SEQUENCE { |  |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE |  | See subclause 4.4.3.1.3 |  |  |
| sibX |  | "X" denotes the SIB number |  |  |
| ... |  |  |  |  |
| sibY |  | "Y" denotes the SIB number |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	UEAssistanceInformation

Table 4.6.1-30: UEAssistanceInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |  |
| delayBudgetReport CHOICE { |  |  |  |  |
| type1 |  | Not checked |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	UECapabilityEnquiry

Table 4.6.1-31: UECapabilityEnquiry

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueCapabilityEnquiry SEQUENCE { |  |  |  |  |
| ue-CapabilityRAT-RequestList |  | UE-CapabilityRAT-RequestList |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| ue-CapabilityEnquiryExt |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	UECapabilityInformation

Table 4.6.1-32: UECapabilityInformation

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueCapabilityInformation SEQUENCE { |  |  |  |  |
| ue-CapabilityRAT-ContainerList |  | UE-CapabilityRAT-ContainerList |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	UEInformationRequest

Table 4.6.1-32A: UEInformationRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	UEInformationResponse

Table 4.6.1-32B: UEInformationResponse

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	ULDedicatedMessageSegment

Table 4.6.1-32C: ULDedicatedMessageSegment

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	ULInformationTransfer

Table 4.6.1-33: ULInformationTransfer

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message |  | DedicatedNAS-Message |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	ULInformationTransferIRAT

Table 4.6.1-33A: ULInformationTransferIRAT

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransferIRAT-r16 ::= SEQUENCE { |  |  |  |  |
| FFS |  |  |  |  |
| } |  |  |  |  |

#### –	ULInformationTransferMRDC

Table 4.6.1-34: ULInformationTransferMRDC

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransferMRDC ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ulInformationTransferMRDC SEQUENCE { |  |  |  |  |
| ul-DCCH-MessageNR |  | Not checked |  |  |
| ul-DCCH-MessageEUTRA |  | Not checked |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

### 4.6.2	System information blocks

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

Table 4.6.2-1: SIB2

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| nrofSS-BlocksToAverage | 2 |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE{ |  |  |  |
| thresholdRSRP | RSRP-Range | Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| rangeToBestCell | dB0 |  |  |
| q-Hyst | dB0 | To reduce interference between intra-frequency multiple cells |  |
| speedStateReselectionPars | Not present |  |  |
| } |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| s-NonIntraSearchQ | Not present |  |  |
| threshServingLowP | 0 | Actual value of threshold = field value * 2 [dB] |  |
| threshServingLowQ | Not present |  |  |
|  | 3 | 3dB | QBASED |
| cellReselectionPriority | 4 | A middle value in the range has been selected |  |
| cellReselectionSubPriority | Not present |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | -140dBm |  |
|  | -55 | -110dBm | SIG |
| q-RxLevMinSUL | Not Present |  |  |
|  | -70 | -140dBm | SUL |
|  | -55 | -110dBm | SUL AND SIG |
| q-QualMin | Not present |  |  |
|  | -20 | -20dB | QBASED |
| s-IntraSearchP | 31 | Actual value of threshold = field value * 2 [dB] |  |
| s-IntraSearchQ | Not present |  |  |
| t-ReselectionNR | 0 |  |  |
| frequencyBandList | Not present |  |  |
| frequencyBandListSUL | Not present |  |  |
| p-Max | Not present |  |  |
| smtc | SSB-MTC | Table 4.6.3-185 |  |
| ss-RSSI-Measurement | Not present |  |  |
| ssb-ToMeasure CHOICE { |  |  |  |
| shortBitmap | '1100'B |  | FR1 AND (2.3GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.3GHz) |
| mediumBitmap | '11000000'B |  | FR1 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | '11000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000'B |  | FR2 |
| } |  |  |  |
| deriveSSB-IndexFromCell | false |  | FDD |
|  | true |  | TDD |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMin-sul. |
| QBASED | This condition applies to Quality based signalling test cases. |
| FREQ<=2.3GHz | Frequency range <= 2.4GHz |
| 2.3GHz<FREQ<=3GHz | Frequency range > 2.3GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |

#### –	SIB3

SIB3 contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters as well as blacklisted cells.

Table 4.6.2-2: SIB3

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList | Not present | Not required unless Qoffset configuration is tested. When Qoffset configuration is tested, see table 6.3.1.1-1 |  |
| intraFreqBlackCellList | Not present | Not required unless Blacklisted cell list configuration is tested. When Blacklisted cell list configuration is tested, see table 6.3.1.1-1 |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

#### –	SIB4

SIB4 contains information relevant only for inter-frequency cell re-selection i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

Table 4.6.2-3: SIB4

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | The same number of entries as the configured inter-freq carriers defined in table 6.3.1.2-1 | n denotes the index of the entry |  |
| InterFreqCarrierFreqInfo[n] SEQUENCE { |  | entry n |  |
| dl-CarrierFreq | Downlink NR SSB ARFCN. See table 6.3.1.2-1 |  |  |
| frequencyBandList | MultiFrequencyBandListNR-SIB |  |  |
| frequencyBandListSUL | Not present |  |  |
| nrofSS-BlocksToAverage | 2 |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | RSRP-Range | Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| smtc | SSB-MTC | Table 4.6.3-185 |  |
| ssbSubcarrierSpacing | SubcarrierSpacing | Table 4.6.3-188 |  |
| ssb-ToMeasure | SSB-ToMeasure |  |  |
| deriveSSB-IndexFromCell | false |  |  |
|  | true |  | FR1_TDD, FR2_TDD |
| ss-RSSI-Measurement | Not present |  |  |
| q-RxLevMin | -70 | -140dBm |  |
|  | -55 | -110dBm | SIG and FR1 |
|  | ROUND((-110+Delta(NRfn))/2) | NOTE1 and NOTE2. | SIG and FR2 |
| q-RxLevMinSUL | -70 | -140dBm, For RF/RRM test cases | SUL |
|  | Not present |  |  |
|  | -55 | -110dBm | SUL and SIG |
| q-QualMin | Not present |  |  |
|  | -20 | -20dB | QBASED |
| p-Max | Not present |  |  |
| t-ReselectionNR | 0 |  |  |
| t-ReselectionNR-SF | Not present | Not required unless speed-dependent cell re-selection is tested. |  |
| threshX-HighP | 2 | 4dB, This value should be higher than threshServingLow of the serving cell to avoid ping-pong with lower priority cells. |  |
| threshX-LowP | 1 | 2dB |  |
| threshX-Q SEQUENCE {} | Not present |  |  |
| threshX-Q SEQUENCE { |  |  | QBASED |
| threshX-HighQ | 5 | 5dB |  |
| threshX-LowQ | 5 | 5dB |  |
| } |  |  |  |
| cellReselectionPriority | 4 | The same priority as the one used for serving cell in SIB 2. |  |
| cellReselectionSubPriority | Not present | The same subpriority as the one used for serving cell in SIB 2. |  |
| q-OffsetFreq | dBXY with XY = (FLOOR((Delta(NRfn) – Delta(NRfs))/2))*2 (NOTE 3) | This value is type of Q-OffsetRange in TS 38.331 [6] which must be even value when its absolute value is larger than dB5. | FR2 AND NOT AbsoluteValue_Within_dB5 |
|  | dBXY with XY = Delta(NRfn) – Delta(NRfs) |  | FR2 AND AbsoluteValue_Within_dB5 |
|  | dB0 |  |  |
| interFreqNeighCellList | Not present | Not required unless Qoffset configuration is tested. |  |
| interFreqBlackCellList | Not present | Not required unless Blacklisted cell list configuration is tested. |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| NOTE 1: Delta(NRfn) and Delta(NRfs) is derived based on calibration procedure defined in the clause 6.1.3.3. NRfn and NRfs are NR frequencies in dl-CarrierFreq[n] and serving cell frequency on which SIB4 is broadcasted. NOTE 2: ROUND is rounded off to the nearest integer. As an example, ‘1 to 1.49’ set to ‘1’ while ‘1.5 to 2’ to ‘2’ and ‘-2.0 to 1.5’ set to ‘-2’ while ‘-1.49 to -1’ set to ‘-1’. NOTE 3: FLOOR is rounded off to the smaller integer. As an example, '1.0 to 1.99' set to 1, '-1.01 to -2.00' set to -2. |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMin-sul. |
| QBASED | This condition applies to Quality based signalling test cases. |
| FREQ<=2.4GHz | Frequency range <= 2.4GHz |
| FREQ>2.4GHz | Frequency range > 2.4GHz |
| FREQ<=3GHz | Frequency range <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| FR1_TDD | TDD frequency range < 6GHz |
| FR2_TDD | TDD frequency range > 6GHz |
| FR1_FDD | FDD frequency range < 6GHz |
| CASE_C | SS Block pattern "Case C" to be applied for the given band and subcarrier spacing according to TS 38.101-1 [7] Table 5.4.3.3-1 |
| AbsoluteValue_Within_dB5 | -5dB <= (Delta(NRfn) – Delta(NRfs)) <= 5dB. |

#### –	SIB5

SIB5 contains information relevant only for inter-RAT cell re-selection i.e. information about E-UTRA frequencies and E-UTRAs neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

Table 4.6.2-4: SIB5

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | The same number of entries as the configured E-UTRA carriers. For Signalling test cases, see table 6.3.1.3-1. | n denotes the index of the entry |  |
| CarrierFreqEUTRA[n] SEQUENCE { |  | entry n |  |
| carrierFreq | Downlink E-UTRA ARFCN under test. For Signalling test cases, see table 6.3.1.3-1. |  |  |
| eutra-multiBandInfoList | Not present |  |  |
| eutra-FreqNeighCellList | Not present | Not required unless EUTRA Qoffset configuration is tested. |  |
| eutra-BlackCellList | Not present | Not required unless Blacklisted cell list configuration is tested. |  |
| allowedMeasBandwidth | EUTRA-AllowedMeasBandwidth | The value of EUTRA-AllowedMeasBandwidth in Table 4.6.5-1. |  |
| presenceAntennaPort1 | FALSE |  |  |
|  | TRUE | At least two cell-specific antenna ports are used in all neighbouring cells. | All neighCells with port1 |
| cellReselectionPriority | 3 |  |  |
| cellReselectionSubPriority | Not Present |  |  |
| threshX-High | 2 (4 dB) |  |  |
| threshX-Low | 1 (2 dB) |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases |  |
|  | -55(-110dBm) | For signalling test cases |  |
| q-QualMin | -20 (-20dB) |  |  |
| p-MaxEUTRA | 23 |  |  |
| threshX-Q SEQUENCE {} | Not present |  |  |
| threshX-Q SEQUENCE { |  |  | QBASED |
| threshX-HighQ | 9 (9dB) |  |  |
| threshX-LowQ | 9 (9dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 0 |  |  |
| t-ReselectionEUTRA-SF | Not present | Not required unless speed-dependent cell re-selection is tested. |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| QBASED | This condition applies to Quality based cell (re)selection signalling test cases. |
| All neighCells with port1 | Used for all neighbouring cells with at least two cell-specific antenna ports |

#### –	SIB6

SIB6 contains an ETWS primary notification.

Table 4.6.2-5: SIB6

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB6 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1. |  |
| warningType | '0000 0101 1000 0000'B | Note 2. |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. Note 2: Warning Type Value (Octet 1 bit 7 ~ 1) set to 'Earthquake and Tsunami', Emergency User Alert (Octet 1 bit 0) set to 'Activate emergency user alert', Popup (Octet 2 bit 7) set to 'Activate Popup', see TS 23.041 [25], Padding (Octet 2 bit 6 ~ 0) set to '000 0000'B. |  |  |  |

#### –	SIB7

SIB7 contains an ETWS secondary notification.

Table 4.6.2-6: SIB7 (1st Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB7 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1. |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246. (see TS 23.041 [25]) |  |
| dataCodingScheme | Bitstring (8) ID of the alphabet/coding and the applied language | see TS 23.041 [25]. | Segment 1 |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB7, otherwise it is not present. |

Table 4.6.2-7: SIB7 (2nd Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB7 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 1 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246. (see TS 23.041 [25]) |  |
| dataCodingScheme | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

Table 4.6.2-8: SIB7 (3rd Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB7 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | LastSegment |  |  |
| warningMessageSegmentNumber | 2 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246. (see TS 23.041 [25]) |  |
| dataCodingScheme | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

#### –	SIB8

SIB8 contains a CMAS notification.

Table 4.6.2-9: SIB8 (1st Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB8 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246. (see TS 23.041 [25]) |  |
| dataCodingScheme | Bitstring (8) ID of the alphabet/coding and the applied language | see TS 23.041 [25] | Segment 1 |
| warningAreaCoordinatesSegment | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

| Condition | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB8, otherwise it is not present. |

Table 4.6.2-10: SIB8 (2nd Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB8 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 1 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246. (see TS 23.041 [25]) |  |
| dataCodingScheme | Not present |  |  |
| warningAreaCoordinatesSegment | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

Table 4.6.2-11: SIB8 (3rd Segment)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB8 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | LastSegment |  |  |
| warningMessageSegmentNumber | 2 |  |  |
| warningMessageSegment | Octetstring of N | Where N ≥ 1 and less than 1246. (see TS 23.041 [25]) |  |
| dataCodingScheme | Not present |  |  |
| warningAreaCoordinatesSegment | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

#### –	SIB9

SIB9 contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time.

NOTE 1:	The UE may use the time information for numerous purposes, possibly involving upper layers e.g. to assist GPS initialisation, to synchronise the UE clock.

NOTE 2:  SIB9 is not defined in the common test environment as test requirements have not been identified.

#### –	SIB10

Table 4.6.2-12: SIB10

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB10-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SIB11

Table 4.6.2-13: SIB11

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB11-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SIB12

Table 4.6.2-14: SIB12

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SIB13

Table 4.6.2-15: SIB13

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB13-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SIB14

Table 4.6.2-16: SIB14

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB14-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

### 4.6.2a	Positioning System information blocks

#### –	PosSystemInformation-r16-IEs

Table 4.6.2a-1: PosSystemInformation-r16-IEs

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PosSystemInformation-r16-IEs ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PosSI-SchedulingInfo

Table 4.6.2a-2: PosSI-SchedulingInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PosSI-SchedulingInfo-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SIBpos

Table 4.6.2a-3: SIBpos

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIBpos-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

### 4.6.3	Radio resource control information elements

#### –	AdditionalSpectrumEmission

Table 4.6.3-1: AdditionalSpectrumEmission

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 0 |  |  |

#### –	Alpha

Table 4.6.3-2: Alpha

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Alpha | alpha0 |  |  |

#### –	AMF-Identifier

Table 4.6.3-3: AMF-Identifier

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AMF-Identifier | FFS |  |  |

#### –	ARFCN-ValueEUTRA

Table 4.6.3-4: ARFCN-ValueEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ARFCN-ValueEUTRA | FFS |  |  |

#### –	ARFCN-ValueNR

Table 4.6.3-5: ARFCN-ValueNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ARFCN-ValueNR | absoluteFrequencySSB as defined for the frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | DL_SSB |
|  | absoluteFrequencyPointA as defined for the DL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | DL_PointA |
|  | absoluteFrequencyPointA as defined for the UL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | UL_PointA |

| Condition | Explanation |
| --- | --- |
| DL_SSB | IE absoluteFrequencySSB for downlink |
| DL_PointA | IE absoluteFrequencyPointA for downlink |
| UL_PointA | IE absoluteFrequencyPointA for uplink |

#### –	ARFCN-ValueUTRA-FDD

Table 4.6.3-5A: ARFCN-ValueUTRA-FDD

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ARFCN-ValueUTRA-FDD-r16 | FFS |  |  |

#### –	AvailabilityCombinationsPerCell

Table 4.6.3-5B: AvailabilityCombinationsPerCell

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AvailabilityCombinationsPerCell-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	AvailabilityIndicator

Table 4.6.3-5C: AvailabilityIndicator

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AvailabilityIndicator-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	BAP-Routing-ID

Table 4.6.3-5D: BAP-Routing-ID

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BAP-Routing-ID-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	BeamFailureRecoveryConfig

Table 4.6.3-6: BeamFailureRecoveryConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryConfig SEQUENCE { |  |  |  |
| rootSequenceIndex-BFR | Set according to the PRACH-rootSequenceIndex given in Table 4.4.2-2 |  |  |
| rach-ConfigBFR | RACH-ConfigGeneric |  |  |
| rsrp-ThresholdSSB | 57 | Actual value = 100 dBm |  |
| candidateBeamRSList SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR { | 1 entry |  |  |
| PRACH-ResourceDedicatedBFR[1] CHOICE { |  | entry 1 |  |
| ssb SEQUENCE { |  |  | SSB |
| ssb | SSB-Index for BFR |  |  |
| ra-PreambleIndex | 56 |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-rs[1] SEQUENCE { |  |  | CSI-RS |
| csi-rs | NZP-CSI-RS-ResourceId for BFR |  |  |
| ra-OccasionList | Not present | The RA occasion associated with the SSB that is QCLed with this CSI-RS is used |  |
| ra-PreambleIndex | Not present | The preamble index associated with the SSB that is QCLed with CSI-RS is used |  |
| } |  |  |  |
| } |  |  |  |
| ssb-perRACH-Occasion | one |  |  |
| ra-ssb-OccasionMaskIndex | 0 |  |  |
| recoverySearchSpaceId | searchSpaceId for BFR search space | The CORESET associated with the BFR search space can not be associated with another search space according to 38.331 [6] |  |
| ra-Prioritization | RA-Prioritization |  |  |
| beamFailureRecoveryTimer | ms200 |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | SSB is used as reference signal for BFR |
| CSI-RS | CSI-RS is used as reference signal for BFR |

#### –	BeamFailureRecoverySCellConfig

Table 4.6.3-6AA: BeamFailureRecoverySCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoverySCellConfig-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	BetaOffsets

Table 4.6.3-6A: BetaOffsets

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BetaOffsets ::= SEQUENCE { |  |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |

#### –	BH-RLC-ChannelConfig

Table 4.6.3-6B: BH-RLC-ChannelConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BH-RLC-ChannelConfig-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	BH-LogicalChannelIdentity

Table 4.6.3-6C: BH-LogicalChannelIdentity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BH-LogicalChannelIdentity-r16 ::= CHOICE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	BH-LogicalChannelIdentity-Ext

Table 4.6.3-6D: BH-LogicalChannelIdentity-Ext

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BH-LogicalChannelIdentity-Ext-r16 | FFS |  |  |

#### –	BH-RLC-ChannelID

Table 4.6.3-6E: BH-RLC-ChannelID

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BH-RLC-ChannelID-r16 | FFS |  |  |

#### –	BSR-Config

Table 4.6.3-7: BSR-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BSR-Config ::= SEQUENCE { |  |  |  |
| periodicBSR-Timer | sf1 |  |  |
| retxBSR-Timer | sf80 |  |  |
| logicalChannelSR-DelayTimer | Not present |  |  |
| } |  |  |  |

#### –	BWP

Table 4.6.3-8: BWP

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | Set to value of locationAndBandwidth in Table 4.3.1.0D-1 for the bandwidth and subcarrier spacing under test. |  | FR1 |
|  | Set to value of locationAndBandwidth in Table 4.3.1.0D-2 for the bandwidth and subcarrier spacing under test. |  | FR2 |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| cyclicPrefix | Not present |  |  |
| } |  |  |  |

#### –	BWP-Downlink

Table 4.6.3-9: BWP-Downlink

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | BWP-Id with condition BWP-Id1 |  |  |
| bwp-Common | BWP-DownlinkCommon |  |  |
| bwp-Dedicated | BWP-DownlinkDedicated |  |  |
| } |  |  |  |

#### –	BWP-DownlinkCommon

Table 4.6.3-10: BWP-DownlinkCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon |  |  |
|  | PDCCH-ConfigCommon with condition SCell_add |  | SCell_Add |
| } |  |  |  |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	BWP-DownlinkDedicated

Table 4.6.3-11: BWP-DownlinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config |  |  |
| } |  |  |  |
| sps-Config | Not present |  |  |
| radioLinkMonitoringConfig | Not present |  |  |
| } |  |  |  |

#### –	BWP-Id

Table 4.6.3-12: BWP-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Id | 0 | Initial BWP |  |
|  | 1 |  | BWP-Id1 |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Additional BWP 1 |

#### –	BWP-Uplink

Table 4.6.3-13: BWP-Uplink

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | BWP-Id with condition BWP-Id1 |  |  |
| bwp-Common | BWP-UplinkCommon |  |  |
| bwp-Dedicated | BWP-UplinkDedicated |  |  |
| } |  |  |  |

#### –	BWP-UplinkCommon

Table 4.6.3-14: BWP-UplinkCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon |  |  |
| } |  |  |  |
| rach-ConfigCommon | Not present |  | SUL_SUL AND RF |
| pusch-ConfigCommon CHOICE { |  |  |  |
| setup | PUSCH-ConfigCommon |  |  |
| } |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon |  |  |
| } |  |  |  |
| pucch-ConfigCommon | Not present |  | SUL_NUL |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL_SUL | On the SUL carrier when supplementary carrier is configured |
| SUL_NUL | On the NUL carrier when supplementary carrier is configured |
| RF | For RF, performance and RRM testing |

#### –	BWP-UplinkDedicated

Table 4.6.3-15: BWP-UplinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config |  |  |
| } |  |  |  |
| pucch-Config | Not present |  | SUL_NUL |
| pusch-Config | Not present |  | RESUME |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| configuredGrantConfig | Not present |  |  |
| srs-Config | Not present |  | Short_DCI |
|  | SRS-Config |  |  |
| beamFailureRecoveryConfig | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SUL_NUL | On the NUL carrier when supplementary carrier is configured |
| RESUME | Used in RRCResume Message |

#### –	CellAccessRelatedInfo

Table 4.6.3-16: CellAccessRelatedInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellAccessRelatedInfo ::= SEQUENCE { |  |  |  |
| plmn-IdentityList | PLMN-IdentityInfoList |  |  |
| cellReservedForOtherUse | Not present |  |  |
| } |  |  |  |

#### –	CellAccessRelatedInfo-EUTRA-5GC

Table 4.6.3-17: CellAccessRelatedInfo-EUTRA-5GC

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellAccessRelatedInfo-EUTRA-5GC ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CellAccessRelatedInfo-EUTRA-EPC

Table 4.6.3-18: CellAccessRelatedInfo-EUTRA-EPC

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellAccessRelatedInfo-EUTRA-EPC ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CellGroupConfig

Table 4.6.3-19: CellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId |  |  |
|  | CellGroupId condition NR-DC_SCG |  | NR-DC_SCG |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | EN-DC |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRB2 | entry 1 |  |
|  | RLC-BearerConfig with conditions AM and DRB2 and Re-establish_RLC |  | PSCell_change |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | SRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 | entry 1 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB1 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB2 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB2 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | DRBn, NR-DC_SCG |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRBn | entry 1 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  | PCell_change |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM, DRB1 and Re-establish_RLC | entry 3 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1+n entries | n is the number of DRBs established before RRC resume | RESUME |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 and RESUME | entry 1 |  |
| RLC-BearerConfig[k+1, k=1..n] | RLC-BearerConfig with condition DRBk and RESUME | entry [k+1, k=1..n] |  |
| } |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| rlc-BearerToReleaseList | Not present |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig |  |  |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add |
| physicalCellGroupConfig | PhysicalCellGroupConfig |  |  |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add |
| spCellConfig | Not present |  | SRB2_DRB1, SRB2_DRB2, SCell_add |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
|  | ServCellIndex |  | EN-DC, EN-DC AND MEAS |
|  | ServCellIndex with condition NR-DC_SCG |  | NR-DC_SCG |
| reconfigurationWithSync | Not present |  |  |
| reconfigurationWithSync SEQUENCE { |  |  | EN-DC, PCell_change, PSCell_change, NR-DC_SCG |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| newUE-Identity | RNTI-Value |  |  |
|  | RNTI-Value with condition NR-DC_SCG |  | NR-DC_SCG |
| t304 | ms1000 |  |  |
| rach-ConfigDedicated | Not present |  |  |
| rach-ConfigDedicated CHOICE { |  |  | CFRA |
| uplink | RACH-ConfigDedicated |  |  |
| supplementaryUplink | RACH-ConfigDedicated |  | SUL AND SIG |
| } |  |  |  |
| } |  |  |  |
| rlf-TimersAndConstants CHOICE { |  |  |  |
| setup | RLF-TimersAndConstants |  |  |
| } |  |  |  |
| rlf-TimersAndConstants | Not present |  | MEAS, RESUME |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig |  | EN-DC, SRB1, PCell_change, PSCell_change, NR-DC_SCG |
|  | Not present |  |  |
|  | ServingCellConfig with condition MEAS |  | MEAS |
|  | ServingCellConfig with condition RESUME |  | RESUME |
| } |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  | SCell_add |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition No_UL |  |  |
| sCellConfigDedicated | ServingCellConfig with condition No_UL |  |  |
| } |  |  |  |
| } |  |  |  |
| sCellToReleaseList | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| CFRA | This condition applies when CFRA is configured |
| SUL | Supplementary Uplink |
| SRB1 | Establishment of SRB1 |
| SRB2_DRB1 | Establishment of SRB2 and DRB1 |
| SRB2_DRB2 | Establishment of SRB2 and DRB2 |
| DRBn | Establishment of DRBn |
| PCell_change | Intra-NR PCell change (standalone NR) |
| PSCell_change | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| MEAS | A NR or EN-DC measurement is configured |
| NR-DC_SCG | Add SCG (NR-DC) |
| RESUME | Used in RRCResume Message |

#### –	CellGroupId

Table 4.6.3-20: CellGroupId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupId | 0 |  |  |
|  | 1 |  | EN-DC, NR-DC_SCG |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| NR-DC_SCG | Add SCG (NR-DC) |

#### –	CellIdentity

Table 4.6.3-21: CellIdentity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellIdentity | Set to NR Cell Identifier defined in table 4.4.2-2 | BIT STRING (SIZE (36)) |  |

#### –	CellReselectionPriority

Table 4.6.3-22: CellReselectionPriority

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellReselectionPriority | FFS |  |  |

#### –	CellReselectionSubPriority

Table 4.6.3-23: CellReselectionSubPriority

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellReselectionSubPriority | FFS |  |  |

#### –	CGI-InfoEUTRA

Table 4.6.3-23A: CGI-InfoEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CGI-InfoEUTRA | FFS |  |  |

#### –	CGI-InfoEUTRALogging

Table 4.6.3-23B: CGI-InfoEUTRALogging

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CGI-InfoEUTRALogging ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CGI-InfoNR

Table 4.6.3-24: CGI-InfNRo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CGI-InfoNR | FFS |  |  |

#### –	CGI-Info-Logging

Table 4.6.3-24A: CGI-Info-Logging

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CGI-Info-Logging-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CLI-RSSI-Range

Table 4.6.3-24B: CLI-RSSI-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CLI-RSSI-Range-r16 | FFS |  |  |

#### –	CodebookConfig

Table 4.6.3-25: CodebookConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| two-one-TypeI-SinglePanel-Restriction | 11111111 |  | FR2 |
| four-one-TypeI-SinglePanel-Restriction | 11111111 11111111 |  | FR1 |
| } |  |  |  |
| typeI-SinglePanel-codebookSubsetRestriction-i2 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 11111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| codebookMode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CommonLocationInfo

Table 4.6.3-25A: CommonLocationInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CommonLocationInfo-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CondRecConfigId

Table 4.6.3-25B: CondRecConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondRecConfigId-r16 | FFS |  |  |

#### –	CondRecConfigToAddModList

Table 4.6.3-25C: CondRecConfigToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondRecConfigToAddModList-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ConditionalReconfiguration

Table 4.6.3-25D: ConditionalReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration -r16::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ConfiguredGrantConfig

Table 4.6.3-26: ConfiguredGrantConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ConfiguredGrantConfigIndex

Table 4.6.3-26A: ConfiguredGrantConfigIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfigIndex-r16 | FFS |  |  |

#### –	ConfiguredGrantConfigIndexMAC

Table 4.6.3-26B: ConfiguredGrantConfigIndexMAC

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfigIndexMAC-r16 | FFS |  |  |

#### –	ConnEstFailureControl

Table 4.6.3-27: ConnEstFailureControl

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConnEstFailureControl ::= SEQUENCE { |  |  |  |
| connEstFailCount | n1 |  |  |
| connEstFailOffsetValidity | s30 |  |  |
| connEstFailOffset | 1 |  |  |
| } |  |  |  |

#### –	ControlResourceSet

Table 4.6.3-28: ControlResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP |  |
| duration | 2 | SearchSpace duration of 2 symbols |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| tci-StatesPDCCH-ToReleaseList | Not present |  |  |
| tci-PresentInDCI | Not present |  |  |
| pdcch-DMRS-ScramblingID | Not present |  |  |
| } |  |  |  |

#### –	ControlResourceSetId

Table 4.6.3-29: ControlResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSetId | 1 |  |  |
|  | 0 |  | Common0 |

| Condition | Explanation |
| --- | --- |
| Common0 | CommonCORESET#0 |

#### –	ControlResourceSetZero

Table 4.6.3-30: ControlResourceSetZero

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSetZero | Set to CORESET#0 Index as defined for the frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. |  |

#### –	CrossCarrierSchedulingConfig

Table 4.6.3-31: CrossCarrierSchedulingConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CrossCarrierSchedulingConfig ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CSI-AperiodicTriggerStateList

Table 4.6.3-32: CSI-AperiodicTriggerStateList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState { | 1 entry |  |  |
| CSI-AperiodicTriggerState[1] SEQUENCE { |  | entry 1 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 8 |  | FR1 |
|  | 16 |  | FR2 |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | TCI-StateId | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesforInteference | 8 |  | FR1 |
|  | 16 |  | FR2 |
| nzp-CSI-RS-ResourcesforInterference | 8 |  | FR1 |
|  | 16 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSI-FrequencyOccupation

Table 4.6.3-33: CSI-FrequencyOccupation

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| startingRB | 0 |  |  |
| nrofRBs | 160 |  | FR1_60MHz |
|  | 216 |  | FR1_80MHz |
|  | 108 |  | FR1_40MHz |
|  | 52 |  | FR1_10MHz |
|  | 272 |  | FR1_100MHz |
|  | 64 |  | FR2_100MHz |
|  | 52 |  | TRS |
|  | 40 |  | TRS AND ((FR1_15MHz AND FR1_SCS30kHz) OR (FR1_30MHz AND FR1_SCS60kHz)) |
|  | 32 |  | TRS AND ((FR1_25MHz AND FR1_SCS60kHz) OR (FR2_50MHz AND FR2_SCS120kHz)) |
|  | 28 |  | TRS AND FR1_5MHz AND FR1_SCS15kHz |
|  | 24 |  | TRS AND ((FR1_5MHz AND FR1_SCS30kHz) OR (FR1_10MHz AND FR1_SCS30kHz) OR (FR1_10MHz AND FR1_SCS60kHz) OR (FR1_15MHz AND FR1_SCS60kHz) OR (FR1_20MHz AND FR1_SCS60kHz)) |
|  | 64 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_40MHz | FR1 is used under the test. CBW is set to 40MHz |
| FR1_60MHz | FR1 is used under the test. CBW is set to 60MHz. |
| FR1_80MHz | FR1 is used under the test. CBW is set to 80MHz. |
| FR1_100MHz | FR1 is used under the test. CBW is set to 100MHz. |
| FR2_100MHz | FR2 is used under the test. CBW is set to 100MHz. |
| TRS | Tracking-Reference Signal |
| FR1_SCS15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_SCS30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| FR1_SCS60kHz | FR1 is used under the test. SCS is set to 60kHz. |
| FR2_SCS120kHz | FR2 is used under the test. SCS is set to 120kHz. |
| FR1_5MHz | FR1 is used under the test. CBW is set to 5MHz. |
| FR1_10MHz | FR1 is used under the test. CBW is set to 10MHz. |
| FR1_15MHz | FR1 is used under the test. CBW is set to 15MHz. |
| FR1_20MHz | FR1 is used under the test. CBW is set to 20MHz. |
| FR1_25MHz | FR1 is used under the test. CBW is set to 25MHz. |
| FR1_30MHz | FR1 is used under the test. CBW is set to 30MHz. |
| FR2_50MHz | FR2 is used under the test. CBW is set to 50MHz. |

#### –	CSI-IM-Resource

Table 4.6.3-34: CSI-IM-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId |  |  |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| pattern1 SEQUENCE { |  |  |  |
| subcarrierLocation-p1 | s4 |  |  |
| symbolLocation-p1 | 3 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| periodicityAndOffset | Not present |  |  |
| } |  |  |  |

#### –	CSI-IM-ResourceId

Table 4.6.3-35: CSI-IM-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceId | 7 |  | FR1 |
|  | 31 |  | FR2 |

#### –	CSI-IM-ResourceSet

Table 4.6.3-36: CSI-IM-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId |  |  |
| csi-IM-Resources SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId { | 1 entry |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSI-IM-ResourceSetId

Table 4.6.3-37: CSI-IM-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | 0 |  |  |

#### –	CSI-MeasConfig

Table 4.6.3-38: CSI-MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource | entry 1 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceToReleaseList | Not present |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet | entry 1 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToReleaseList | Not present |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceToReleaseList | Not present |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceSetToReleaseList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { | 1 entry |  |  |
| CSI-SSB-ResourceSet[1] | CSI-SSB-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetToReleaseList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig | entry 1 |  |
| } |  |  |  |
| csi-ResourceConfigToReleaseList | Not present |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | 1 entry |  |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | entry 1 |  |
| } |  |  |  |
| csi-ReportConfigToReleaseList | Not present |  |  |
| reportTriggerSize | 0 |  |  |
| aperiodicTriggerStateList CHOICE { |  |  |  |
| setup | CSI-AperiodicTriggerStateList |  |  |
| } |  |  |  |
| semiPersistentOnPUSCH-TriggerStateList | Not present |  |  |
| } |  |  |  |

#### –	CSI-ReportConfig

Table 4.6.3-39: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| nzp-CSI-RS-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 14 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL, |  | FR1 |
| cri-RI-LI-PMI-CQI | NULL |  | FR2 |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand | Not present |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table2 |  | FR1 |
|  | table1 |  | FR2 |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

#### –	CSI-ReportConfigId

Table 4.6.3-40: CCSI-ReportConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfigID | 0 |  |  |

#### –	CSI-ResourceConfig

Table 4.6.3-41: CSI-ResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 0 | entry 1 |  |
| NZP-CSI-RS-ResourceSetId[2] | 1 | entry 2 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

#### –	CSI-ResourceConfigId

Table 4.6.3-42: CSI-ResourceConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | 0 |  |  |

#### –	CSI-ResourcePeriodicityAndOffset

Table 4.6.3-43: CSI-ResourcePeriodicityAndOffset

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots80 | 10 |  | FR1 |
| slots320 | 40 |  | FR2 |
| } |  |  |  |

#### –	CSI-RS-ResourceConfigMobility

Table 4.6.3-44: CSI-RS-ResourceConfigMobility

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceConfigMobility ::= SEQUENCE { |  |  |  |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| csi-RS-CellList-Mobility | FFS |  |  |
| } |  |  |  |

#### –	CSI-RS-ResourceMapping

Table 4.6.3-45: CSI-RS-ResourceMapping

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 1000 |  | (FR1 AND TRS) OR (FR2 AND TRS) |
| row4 | 010 |  | FR2 |
| other | 011110 |  | FR1 |
| } |  |  |  |
| nrofPorts | p8 |  | FR1 |
|  | p4 |  | FR2 |
|  | p1 |  | (FR1 AND TRS) OR (FR2 AND TRS) |
| firstOFDMSymbolInTimeDomain | 3 |  | FR1 |
|  | 13 |  | FR2 |
|  | 4 |  | (FR1 AND TRS) OR (FR2 AND TRS) |
| firstOFDMSymbolInTimeDomain2 | Not present |  |  |
| cdm-Type | fd-CDM2 |  |  |
|  | noCDM |  | TRS |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| three | NULL |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal is configured. |

#### –	CSI-SemiPersistentOnPUSCH-TriggerStateList

Table 4.6.3-46: CSI-SemiPersistentOnPUSCH-TriggerStateList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SemiPersistentOnPUSCH-TriggerStateList ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CSI-SSB-ResourceSet

Table 4.6.3-47: CSI-SSB-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SSB-ResourceSet ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CSI-SSB-ResourceSetId

Table 4.6.3-48: CSI-SSB-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SSB-ResourceSetId | FFS |  |  |

#### –	DedicatedNAS-Message

Table 4.6.3-49: DedicatedNAS-Message

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DedicatedNAS-Message | Set according to specific message content |  |  |

#### –	DMRS-DownlinkConfig

Table 4.6.3-50: DMRS-DownlinkConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-Type | Not present | DMRS type 1 |  |
| dmrs-AdditionalPosition | pos1 |  | FR1 |
|  | pos0 |  | FR2 |
| maxLength | Not present | len1 |  |
| scramblingID0 | Not present |  |  |
| scramblingID1 | Not present |  |  |
| phaseTrackingRS | Not present |  | FR1 |
| phaseTrackingRS CHOICE { |  |  | FR2 |
| setup | PTRS-DownlinkConfig |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	DMRS-UplinkConfig

Table 4.6.3-51: DMRS-UplinkConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-Type | Not present | DMRS type 1 |  |
| dmrs-AdditionalPosition | pos1 |  | FR1 |
|  | pos0 |  | FR2 |
| phaseTrackingRS | Not present |  |  |
| phaseTrackingRS CHOICE { |  |  | PTRS_UL_CONFIG |
| setup | PTRS-UplinkConfig |  |  |
| } |  |  |  |
| maxLength | Not present | len1 |  |
| transformPrecodingDisabled SEQUENCE { |  |  |  |
| scramblingID0 | Not present |  |  |
| scramblingID1 | Not present |  |  |
| } |  |  |  |
| transformPrecodingEnabled | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |

#### –	DownlinkConfigCommon

Table 4.6.3-52: DownlinkConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| frequencyInfoDL | FrequencyInfoDL |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon |  |  |
|  | BWP-DownlinkCommon with condition SCell_add |  | SCell_Add |
| } |  |  |  |

#### –	DownlinkConfigCommonSIB

Table 4.6.3-53: DownlinkConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| frequencyInfoDL | FrequencyInfoDL-SIB |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon |  |  |
| bcch-Config SEQUENCE { |  |  |  |
| modificationPeriodCoeff | n4 |  |  |
| } |  |  |  |
| pcch-Config SEQUENCE { |  |  |  |
| defaultPagingCycle | rf128 |  |  |
| nAndPagingFrameOffset CHOICE { |  |  |  |
| halfT | 0 |  |  |
| } |  |  |  |
| ns | one |  |  |
| firstPDCCH-MonitoringOccasionOfPO CHOICE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	DownlinkPreemption

Table 4.6.3-54: DownlinkPreemption

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkPreemption ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	DRB-Identity

Table 4.6.3-55: DRB-Identity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-Identity | n |  | DRBn |

| Condition | Explanation |
| --- | --- |
| DRBn | DRB-Identity n |

#### –	DRX-Config

Table 4.6.3-56: DRX-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms6 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms1280 |  |  |
| drx-HARQ-RTT-TimerDL | 56 |  |  |
| drx-HARQ-RTT-TimerUL | 56 |  |  |
| drx-RetransmissionTimerDL | sl16 |  | FR1 |
|  | sl64 |  | FR2 |
| drx-RetransmissionTimerUL | sl16 |  | FR1 |
|  | sl64 |  | FR2 |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10240 | 0 |  |  |
| } |  |  |  |
| shortDRX | not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |

#### –	DRX-ConfigSecondaryGroup

Table 4.6.3-56A: DRX-ConfigSecondaryGroup

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-ConfigSecondaryGroup ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	FilterCoefficient

Table 4.6.3-57: FilterCoefficient

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FilterCoefficient | fc4 |  |  |

#### –	FreqBandIndicatorNR

Table 4.6.3-58: FreqBandIndicatorNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqBandIndicatorNR | Operating band of the frequency as specified in Table 4.4.2-1 |  |  |
|  | Secondary band under test |  | CA-InterBand |

| Condition | Explanation |
| --- | --- |
| CA-InterBand | Used in CA interBand test cases |

#### –	FrequencyInfoDL

Table 4.6.3-59: FrequencyInfoDL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoDL::= SEQUENCE { |  |  |  |
| absoluteFrequencySSB | ARFCN-ValueNR with condition DL_SSB |  |  |
| frequencyBandList | MultiFrequencyBandListNR |  |  |
| absoluteFrequencyPointA | ARFCN-ValueNR with condition DL_PointA |  |  |
| scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { | 1 entry |  |  |
| SCS-SpecificCarrier[1] | SCS-SpecificCarrier with condition DL_PointA | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	FrequencyInfoDL-SIB

Table 4.6.3-60: FrequencyInfoDL-SIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoDL-SIB ::= SEQUENCE { |  |  |  |
| frequencyBandList | MultiFrequencyBandListNR-SIB |  |  |
| offsetToPointA | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. |  |  |
| scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { | 1 entry |  |  |
| SCS-SpecificCarrier[1] | SCS-SpecificCarrier with condition DL_PointA | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	FrequencyInfoUL

Table 4.6.3-61: FrequencyInfoUL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL::= SEQUENCE { |  |  |  |
| frequencyBandList | MultiFrequencyBandListNR |  | FDD |
|  | Not present |  | TDD |
| absoluteFrequencyPointA | ARFCN-ValueNR with condition UL_PointA |  | FDD |
|  | Not present |  | TDD |
| scs-SpecificCarriers SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { | 1 entry |  |  |
| SCS-SpecificCarrier1 | SCS-SpecificCarrier with condition UL_PointA | entry 1 |  |
| } |  |  |  |
| additionalSpectrumEmission | AdditionalSpectrumEmission |  |  |
| p-Max | P-Max |  |  |
| frequencyShift7p5khz | Not present |  |  |
| } |  |  |  |

#### –	FrequencyInfoUL-SIB

Table 4.6.3-62: FrequencyInfoUL-SIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB ::= SEQUENCE { |  |  |  |
| frequencyBandList | MultiFrequencyBandListNR-SIB |  | FDD |
|  | Not present |  | TDD |
| absoluteFrequencyPointA | ARFCN-ValueNR with condition UL_PointA |  | FDD |
|  | Not present |  | TDD |
| scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { | 1 entry |  |  |
| SCS-SpecificCarrier[1] | SCS-SpecificCarrier with condition UL_PointA | entry 1 |  |
| } |  |  |  |
| p-Max | P-Max |  |  |
| frequencyShift7p5khz | Not present |  |  |
| } |  |  |  |

#### –	HighSpeedConfig

Table 4.6.3-62A: HighSpeedConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HighSpeedConfig-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	Hysteresis

Table 4.6.3-63: Hysteresis

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Hysteresis | 4 |  |  |

#### –	InvalidSymbolPattern

Table 4.6.3-63A: InvalidSymbolPattern

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| InvalidSymbolPattern-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	I-RNTI-Value

Table 4.6.3-64: I-RNTI-Value

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| I-RNTI-Value | SS arbitrarily selects a value between ’00 0000 0001’H and ‘FF FFFF FFFF’H. | BIT STRING (SIZE(40)) |  |

#### –	LBT-FailureRecoveryConfig

Table 4.6.3-64A: LBT-FailureRecoveryConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LBT-FailureRecoveryConfig-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	LocationInfo

Table 4.6.3-64B: LocationInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LocationInfo-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	LocationMeasurementInfo

Table 4.6.3-65: LocationMeasurementInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LocationMeasurementInfo ::= CHOICE { |  |  |  |
| eutra-RSTD SEQUENCE (SIZE (1..maxInterRAT-RSTD-Freq)) OF EUTRA-RSTD-Info { | 1 entry |  |  |
| EUTRA-RSTD-Info[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | ARFCN-ValueEUTRA |  |  |
| measPRS-Offset | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	LogicalChannelConfig

Table 4.6.3-66: LogicalChannelConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LogicalChannelConfig ::= SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| priority | 1 |  |  |
|  | 3 |  | SRB2 |
| prioritisedBitRate | infinity |  |  |
| bucketSizeDuration | ms50 |  |  |
| allowedServingCells | Not present |  |  |
| allowedSCS-List | Not present |  |  |
| maxPUSCH-Duration | Not present |  |  |
| configuredGrantType1Allowed | Not present |  |  |
| logicalChannelGroup | 1 |  | HI |
|  | 2 |  | LO |
|  | 0 |  | SRB1, SRB2, SRB3 |
| schedulingRequestID | SchedulingRequestId |  |  |
| logicalChannelSR-Mask | false |  |  |
| logicalChannelSR-DelayTimerApplied | false |  |  |
| bitRateQueryProhibitTimer | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HI | Used for DRBs with high logical channel priority |
| LO | Used for DRBs with low logical channel priority |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |

#### –	LogicalChannelIdentity

Table 4.6.3-67: LogicalChannelIdentity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LogicalChannelIdentity | 1 |  | SRB1 |
| LogicalChannelIdentity | 2 |  | SRB2 |
| LogicalChannelIdentity | 3 |  | SRB3 |
| LogicalChannelIdentity | n+3 |  | DRBn |

| Condition | Explanation |
| --- | --- |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| DRBn | Establishment of DRBn; n=1..29 |

#### –	MAC-CellGroupConfig

Table 4.6.3-68: MAC-CellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config | Not present |  |  |
| drx-Config CHOICE { |  |  | DRX |
| setup | DRX-Config |  |  |
| } |  |  |  |
| schedulingRequestConfig | SchedulingRequest-Config |  |  |
| bsr-Config | BSR-Config |  |  |
| tag-Config | TAG-Config |  |  |
| phr-Config CHOICE { |  |  |  |
| setup | PHR-Config |  |  |
| } |  |  |  |
| skipUplinkTxDynamic | false |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX | This condition applies when DRX is configured |

#### –	MeasConfig

Table 4.6.3-69: MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | MeasObjectToAddModList |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | ReportConfigToAddModList |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | MeasIdToAddModList |  |  |
| s-MeasureConfig | Not present |  |  |
| quantityConfig | QuantityConfig |  |  |
| measGapConfig | Not present |  |  |
| measGapSharingConfig | Not present |  |  |
| } |  |  |  |

#### –	MeasGapConfig

Table 4.6.3-70: MeasGapConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 | Not present |  |  |
| gapFR2 CHOICE { |  |  | GAP_FR2 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 159 |  |  |
| mgl | ms3dot5 |  |  |
| mgrp | ms160 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapFR1 | Not present |  |  |
| gapFR1 CHOICE { |  |  | GAP_FR1 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  |  |
|  | 9 |  | SIG AND INTER-FREQ_ODD |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| gapUE | Not present |  | GAP_FR1 OR GAP_FR2 |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  |  |
|  | 9 |  | SIG AND INTER-FREQ_ODD |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GAP_FR1 | Configuration for FR1 per-FR gaps |
| GAP_FR2 | Configuration for FR2 per-FR gaps |
| INTER-FREQ_ODD | When the SFNoffset of inter frequency neighbour cell is odd number. SFNoffset is defined in TS 38.523-3 [23]Table 7.1.5.2-1 |

#### –	MeasGapSharingConfig

Table 4.6.3-71: MeasGapSharingConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapSharingConfig ::= SEQUENCE { |  |  |  |
| gapSharingFR2 | Not present |  |  |
| } |  |  |  |

#### –	MeasId

Table 4.6.3-72: MeasId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasId | 1 |  |  |

#### –	MeasIdleConfig

Table 4.6.3-72A: MeasIdleConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigSIB-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasIdToAddModList

Table 4.6.3-73: MeasIdToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | MeasId |  |  |
| measObjectId | MeasObjectId |  |  |
| reportConfigId | ReportConfigId |  |  |
| } |  |  |  |

#### –	MeasObjectCLI

Table 4.6.3-73A: MeasObjectCLI

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCLI-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasObjectEUTRA

Table 4.6.3-74: MeasObjectEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA ::= SEQUENCE { |  |  |  |
| carrierFreq | Downlink EARFCN for Freq |  |  |
| allowedmeasBandwidth | Set according to TS 36.508 [2] Table 4.4.3.4-1 for E-UTRA cell | row ‘measurement Bandwidth’ |  |
| cellsToRemoveListEUTRAN | Not present |  |  |
| cellsToAddModListEUTRAN | Not present |  |  |
| blackCellsToRemoveListEUTRAN | Not present |  |  |
| blackCellsToAddModListEUTRAN | Not present |  |  |
| eutra-PresenceAntennaPort1 | false |  |  |
|  | true | at least two cell-specific antenna ports are used in all neighbouring cells | All neighCells with port1 |
| eutra-Q-OffsetRange | Not present |  |  |
| widebandRSRQ-Meas | false |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| All neighCells with port1 | Used for all neighbouring cells with at least two cell-specific antenna ports |

#### –	MeasObjectId

Table 4.6.3-75: MeasObjectId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectId | 1 |  |  |

#### –	MeasObjectNR

Table 4.6.3-76: MeasObjectNR(Thres)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR with condition DL_SSB |  |  |
| ssbSubcarrierSpacing | SubcarrierSpacing |  |  |
| smtc1 | SSB-MTC |  |  |
| smtc2 | Not present |  |  |
| refFreqCSI-RS | Not present |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure CHOICE { |  |  |  |
| setup | SSB-ToMeasure |  |  |
| } |  |  |  |
| deriveSSB-IndexFromCell | false |  | FDD |
|  | true |  | TDD |
| ss-RSSI-Measurement | Not present |  |  |
| } |  |  |  |
| csi-rs-ResourceConfigMobility | Not present |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | Thres | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| absThreshCSI-RS-Consolidation | Not present |  |  |
| nrofSS-BlocksToAverage | 2 |  |  |
| nrofCSI-RS-ResourcesToAverage | Not present |  |  |
| quantityConfigIndex | 1 |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB0 |  |  |
| rsrqOffsetSSB | dB0 |  |  |
| sinrOffsetSSB | dB0 |  |  |
| rsrpOffsetCSI-RS | dB0 |  |  |
| rsrqOffsetCSI-RS | dB0 |  |  |
| sinrOffsetCSI-RS | dB0 |  |  |
| } |  |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList | Not present |  |  |
| blackCellsToRemoveList | Not present |  |  |
| blackCellsToAddModList | Not present |  |  |
| whiteCellsToRemoveList | Not present |  |  |
| whiteCellsToAddModList | Not present |  |  |
| freqBandIndicatorNR | FreqBandIndicatorNR |  |  |
| } |  |  |  |

#### –	MeasObjectNR-SL

Table 4.6.3-76A: MeasObjectNR-SL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR-SL-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasObjectToAddModList

Table 4.6.3-77: MeasObjectToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | MeasObjectId |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MeasObjectUTRA-FDD

Table 4.6.3-77A: MeasObjectUTRA-FDD

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA-FDD-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultCellListSFTD-NR

Table 4.6.3-78: MeasResultCellListSFTD-NR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultCellListSFTD-NR ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultCellListSFTD-EUTRA

Table 4.6.3-78A: MeasResultCellListSFTD-EUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultCellListSFTD-EUTRA ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResults

Table 4.6.3-79: MeasResults

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measId | MeasId |  |  |
| measResultServingMOList SEQUENCE (SIZE (1.. maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | Not checked |  |  |
| rsrq | Not checked |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| measResultBestNeighCell | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells | Not present |  | A1, A2 |
|  | Set according to specific message content |  | A3, A4, A5, A6 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | If event trigger Id in corresponding Measurement Configuration was Event A1 |
| A2 | If event trigger Id in corresponding Measurement Configuration was Event A2 |
| A3 | If event trigger Id in corresponding Measurement Configuration was Event A3 |
| A4 | If event trigger Id in corresponding Measurement Configuration was Event A4 |
| A5 | If event trigger Id in corresponding Measurement Configuration was Event A5 |
| A6 | If event trigger Id in corresponding Measurement Configuration was Event A6 |

#### –	MeasResult2EUTRA

Table 4.6.3-79A: MeasResult2EUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResult2EUTRA ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResult2NR

Table 4.6.3-79B: MeasResult2NR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResult2NR ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultIdleEUTRA

Table 4.6.3-79C: MeasResultIdleEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultIdleEUTRA-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultIdleNR

Table 4.6.3-79D: MeasResultIdleNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultIdleNR-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasResultSCG-Failure

Table 4.6.3-80: MeasResultSCG-Failure

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultSCG-Failure ::= SEQUENCE { |  | measResultPerMOList for each MeasOjectNR for which a measId is configured (by the NR RRCConfiguration message) and measurement results are available include an entry |  |
| measResultPerMOList SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR { | n entries of MeasResult2NR | MOList [1] n denotes the number of non-serving frequencies being measured |  |
| MeasResult2NR[1] SEQUENCE { |  | entry 1 |  |
| ssbFrequency | ARFCN-ValueNR with condition DL_SSB | the ARFCN if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb |  |
| refFreqCSI-RS | INTEGER (0..3279165) | the ARFCN if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs |  |
| measResultServingCell SEQUENCE { |  | if a serving cell is associated with the MeasObjectNR |  |
| physCellId | INTEGER (0..1007) | the physCellId configured for this serving cell |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| resultsCSI-RS-Cell SEQUENCE { |  |  |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |
| resultsSSB-Indexes SEQUENCE (SIZE (1..maxNrofSSBs)) OF ResultsPerSSB-Index { | n entires of ResultsPerSSB-Index | ResultsPerSSB-IndexList |  |
| ResultsPerSSB-Index[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index | SSB-Index | an SS-Block within an SS-Burst |  |
| ssb-Results SEQUENCE { |  | MeasQuantityResults |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
|  |  | ResultsPerSSB-Index entry [x] if any |  |
| } |  |  |  |
| resultsCSI-RS-Indexes SEQUENCE (SIZE (1..maxNrofCSI-RS)) OF ResultsPerCSI-RS-Index { | n entires of ResultsPerCSI-RS-Index | ResultsPerCSI-RS-IndexList |  |
| ResultsPerCSI-RS-Index[1] SEQUENCE { |  | entry 1 |  |
| csi-RS-Index | INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1) | CSI-RS resource index associated to the measurement information to be reported |  |
| csi-RS-Results SEQUENCE { |  | MeasQuantityResults |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
|  |  | ResultsPerCSI-RS-Index entry [x] if any |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR { | n entires of MeasResultNR | include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | INTEGER (0..1007) | the physCellId configured for the measured cell |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| resultsCSI-RS-Cell SEQUENCE { |  |  |  |
| rsrp | as specified in Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
|  |  | MeasResultNR entry [x] if any |  |
| } |  |  |  |
|  |  | MeasResult2NR entry [x] if any |  |
| } |  |  |  |
|  |  | MOList [x] if any |  |
| } |  |  |  |
| } |  |  |  |

#### –	MeasResultsSL

Table 4.6.3-80A: MeasResultsSL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultsSL-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MeasTriggerQuantityEUTRA

Table 4.6.3-80B: MeasTriggerQuantityEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasTriggerQuantityEUTRA-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MobilityStateParameters

Table 4.6.3-81: MobilityStateParameters

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityStateParameters ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MsgA-ConfigCommon

Table 4.6.3-81A: MsgA-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-ConfigCommonL-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MsgA-PUSCH-Config

Table 4.6.3-81B: MsgA-PUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Config-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	MultiFrequencyBandListNR

Table 4.6.3-82: MultiFrequencyBandListNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MultiFrequencyBandListNR ::= SEQUENCE (SIZE (1..maxNrofMultiBands)) OF FreqBandIndicatorNR { | 1 entry |  |  |
| FreqBandIndicatorNR[1] | FreqBandIndicatorNR | entry 1 |  |
| } |  |  |  |

#### –	MultiFrequencyBandListNR-SIB

Table 4.6.3-82A: MultiFrequencyBandListNR-SIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MultiFrequencyBandListNR-SIB ::= SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo { | 1 entry |  |  |
| NR-MultiBandInfo[1] SEQUENCE { |  | entry 1 |  |
| freqBandIndicatorNR | FreqBandIndicatorNR |  |  |
| nr-NS-PmaxList | NR-NS-PmaxList |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	NeedForGapsConfigNR

Table 4.6.3-82B: NeedForGapsConfigNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForGapsConfigNR-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NeedForGapsInfoNR

Table 4.6.3-82C: NeedForGapsInfoNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NeedForGapsInfoNR-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NextHopChainingCount

Table 4.6.3-83: NextHopChainingCount

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NextHopChainingCount | 0 |  |  |

#### –	NG-5G-S-TMSI

Table 4.6.3-84: NG-5G-S-TMSI

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NG-5G-S-TMSI | Set to the value of the NG-5G-S-TMSI of the UE | BIT STRING (SIZE(40)) |  |

#### –	NPN-Identity

Table 4.6.3-84AA: NPN-Identity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPN-Identity-r16 ::= CHOICE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NPN-IdentityInfoList

Table 4.6.3-84AB: NPN-IdentityInfoList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPN-IdentityInfoList-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	NR-NS-PmaxList

Table 4.6.3-84A: NR-NS-PmaxList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-NS-PmaxList ::= SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-NS-PmaxValue { | 1 entry |  |  |
| NR-NS-PmaxValue[1] SEQUENCE { |  | entry 1 |  |
| additionalPmax | Not present |  |  |
| additionalSpectrumEmission | AdditionalSpectrumEmission |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	NZP-CSI-RS-Resource

Table 4.6.3-85: NZP-CSI-RS-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId |  |  |
| resourceMapping | CSI-RS-ResourceMapping |  |  |
| powerControlOffset | -3 |  |  |
| powerControlOffsetSS | Not present |  |  |
| scramblingID | ScramblingId |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId |  |  |
| } |  |  |  |

#### –	NZP-CSI-RS-ResourceId

Table 4.6.3-86: NZP-CSI-RS-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | 0 |  |  |

#### –	NZP-CSI-RS-ResourceSet

Table 4.6.3-87: NZP-CSI-RS-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId | entry 1 |  |
| } |  |  |  |
| repetition | off |  |  |
|  | Not present |  | TRS |
| aperiodicTriggeringOffset | Not present |  |  |
| trs-Info | Not present |  |  |
|  | true |  | TRS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal |

#### –	NZP-CSI-RS-ResourceSetId

Table 4.6.3-88: NZP-CSI-RS-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | 0 |  |  |

#### –	P-Max

Table 4.6.3-89: P-Max

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | FR1 AND pc_dynamicPowerSharing |
|  | 23 |  | FR1_RF_PC2_Testing_PC3 |
|  | Not present |  | FR1_RF_PC3 |
|  | Not present |  | FR1_RF_PC2 |
|  | 26 |  | FR2 AND pc_dynamicPowerSharing |
|  | 20 | P-Max value when pc_dynamicPowerSharing is set to FALSE | NOT pc_dynamicPowerSharing |

| Condition | Explanation |
| --- | --- |
| FR1_RF_PC3 | Power Class 3 UE testing Power Class 3 requirements |
| FR1_RF_PC2 | Power Class 2 UE testing Power Class 2 requirements |
| FR1_RF_PC2_Testing_PC3 | Power Class 2 UE testing Power Class 3 requirements. |

#### –	PCI-List

Table 4.6.3-90: PCI-List

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-List ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PCI-Range

Table 4.6.3-91: PCI-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-Range ::= SEQUENCE { |  |  |  |
| start | PhysCellId |  |  |
| range | FFS |  |  |
| } |  |  |  |

#### –	PCI-RangeElement

Table 4.6.3-92: PCI-RangeElement

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-RangeElement ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PCI-RangeIndex

Table 4.6.3-93: PCI-RangeIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-RangeIndex | 0 |  |  |

#### –	PCI-RangeIndexList

Table 4.6.3-94: PCI-RangeIndexList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCI-RangeIndexList ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PDCCH-Config

Table 4.6.3-95: PDCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  | EN-DC |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet | entry 1 |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace with condition USS | entry 1 |  |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| } |  |  |  |

#### –	PDCCH-ConfigCommon

Table 4.6.3-96: PDCCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | ControlResourceSetZero |  |  |
|  | Not present |  | SCell_Add |
| commonControlResourceSet | ControlResourceSet |  | EN-DC |
|  | Not present |  |  |
| searchSpaceZero | SearchSpaceZero |  |  |
|  | Not present |  | SCell_Add |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| SearchSpace[2] | SearchSpace with condition SISS | entry 2 |  |
| } |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  | EN-DC |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| commonSearchSpaceList | Not present |  | SCell_Add |
| searchSpaceSIB1 | 0 |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| searchSpaceOtherSystemInformation | SearchSpaceId with condition SISS |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| pagingSearchSpace | 0 |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| ra-SearchSpace | SearchSpaceId with condition CSS |  |  |
|  | Not present |  | SCell_Add |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| SCell_Add | Add SCell |

#### –	PDCCH-ConfigSIB1

Table 4.6.3-97: PDCCH-ConfigSIB1

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigSIB1 ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | ControlResourceSetZero |  |  |
| searchSpaceZero | SearchSpaceZero |  |  |
| } |  |  |  |

#### –	PDCCH-ServingCellConfig

Table 4.6.3-98: PDCCH-ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| slotFormatIndicator | Not present |  |  |
| } |  |  |  |

#### –	PDCP-Config

Table 4.6.3-99: PDCP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| discardTimer | infinity |  |  |
| pdcp-SN-Size-UL | len18bits |  |  |
| pdcp-SN-Size-DL | len18bits |  |  |
| headerCompression CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| integrityProtection | Not present |  |  |
| statusReportRequired | true | AM is default |  |
|  | Not present |  | UM |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| drb | Not present |  | SRB, Split_SRB |
| moreThanOneRLC | Not present |  |  |
| moreThanOneRLC SEQUENCE { |  |  | Split, Split_SRB, NR_split |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | CellGroupId |  |  |
|  | 0 | MCG path | Split_SRB, NR_split |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| ul-DataSplitThreshold | infinity |  |  |
|  | Not present |  | Split_SRB |
| pdcp-Duplication | false |  |  |
|  | Not present | one UL path | Split_SRB |
| } |  |  |  |
| t-Reordering | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Split | More than one RLC |
| SRB | SRB |
| UM | RLC UM DRB |
| Split_SRB | SRB with more than one RLC |
| NR_split | MCG and split for NR-DC. |

#### –	PDSCH-Config

Table 4.6.3-100: PDSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPDSCH | 0 |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeB | Not present |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1 entry |  |  |
| TCI-State[1] | TCI-State | entry 1 |  |
| } |  |  |  |
| tci-StatesToReleaseList | Not present |  |  |
| vrb-ToPRB-Interleaver | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
|  | resourceAllocationType0 |  | Used_for_Type0 |
| pdsch-TimeDomainAllocationList | Not present |  |  |
| pdsch-AggregationFactor | Not present |  |  |
| rateMatchPatternToAddModList | Not present |  |  |
| rateMatchPatternToReleaseList | Not present |  |  |
| rateMatchPatternGroup1 | Not present |  |  |
| rateMatchPatternGroup2 | Not present |  |  |
| rbg-Size | config1 |  |  |
| mcs-Table | Not present | qam64 per default |  |
| maxNrofCodeWordsScheduledByDCI | Not present |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | wideband |  |  |
| } |  |  |  |
| } |  |  |  |
| zp-CSI-RS-ResourceToAddModList | Not present |  |  |
| zp-CSI-RS-ResourceToReleaseList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| p-ZP-CSI-RS-ResourceSet | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Used_for_Type0 | Used for RF performance test cases |

#### –	PDSCH-ConfigCommon

Table 4.6.3-101: PDSCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList |  |  |
| } |  |  |  |

#### –	PDSCH-ServingCellConfig

Table 4.6.3-102: PDSCH-ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| codeBlockGroupTransmission | Not present |  |  |
| xOverhead | Not present |  |  |
| nrofHARQ-ProcessesForPDSCH | n16 |  |  |
|  | Not present | Default value: 8 HARQ processes | SIG |
| pucch-Cell | Not present |  |  |
| } |  |  |  |

#### –	PDSCH-TimeDomainResourceAllocationList

Table 4.6.3-103: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  | FR1 |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | S=2, L=6 |  |
| } |  |  |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocationList SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 1 entry |  | FR2 |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| } |  |  |  |

#### –	PHR-Config

Table 4.6.3-104: PHR-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PHR-Config ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| phr-PeriodicTimer | sf10 |  |  |
| phr-ProhibitTimer | sf0 |  |  |
| phr-Tx-PowerFactorChange | dB1 |  |  |
| multiplePHR | false |  |  |
|  | true |  | MR-DC OR NR-CA |
| dummy | false |  |  |
| Phr-Type2OtherCell | false |  |  |
| phr-ModeOtherCG | real |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MR-DC | EN-DC, NGEN-DC, NE-DC or NR-DC. |
| NR-CA | UL CA for NR |

#### –	PhysCellId

Table 4.6.3-105: PhysCellId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysCellId | Set according to table 4.4.2-2 for the NR Cell. |  |  |

#### –	PhysicalCellGroupConfig

Table 4.6.3-106: PhysicalCellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| harq-ACK-SpatialBundlingPUCCH | Not present |  |  |
| harq-ACK-SpatialBundlingPUSCH | Not present |  |  |
| p-NR-FR1 | P-Max |  |  |
| pdsch-HARQ-ACK-Codebook | dynamic |  |  |
| tpc-SRS-RNTI | Not present |  |  |
| tpc-PUCCH-RNTI | Not present |  |  |
| tpc-PUSCH-RNTI | Not present |  |  |
| sp-CSI-RNTI | Not present |  |  |
| cs-RNTI | Not present |  |  |
| } |  |  |  |

#### –	PLMN-Identity

Table 4.6.3-107: PLMN-Identity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PLMN-Identity ::= SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-MNC-Digit | See table 4.4.2-3 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit | See table 4.4.2-3 |  |  |
| } |  |  |  |

#### –	PLMN-IdentityInfoList

Table 4.6.3-108: PLMN-IdentityInfoList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PLMN-IdentityInfoList ::= SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { | 1 entry |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { | 1 entry |  |  |
| PLMN-Identity[1] | PLMN-Identity | entry 1 |  |
| } |  |  |  |
| trackingAreaCode | TrackingAreaCode |  |  |
| ranac | RAN-AreaCode |  |  |
| cellIdentity | CellIdentity |  |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	PLMN-IdentityList2

Table 4.6.3-108A: PLMN-IdentityList2

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PLMN-IdentityList2-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PRB-Id

Table 4.6.3-109: PRB-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRB-Id | 0 |  |  |
|  | Set to value of the L_RBs - nrofPRBs where L_RBs is found in Table 4.3.1.0D-1 or Table 4.3.1.0D-2 for the bandwidth and SCS configured and nrofPRBs is defined for the corresponding PUCCH-Resource (1 otherwise). |  | secondHopPRB |

| Condition | Explanation |
| --- | --- |
| secondHopPRB | The IE secondHopPRB in PUCCH-Resource is now set. |

#### –	PTRS-DownlinkConfig

Table 4.6.3-110: PTRS-DownlinkConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PTRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| frequencyDensity | Not present |  |  |
| timeDensity | Not present |  |  |
| epre-Ratio | 0 |  |  |
| resourceElementOffset | Not present |  |  |
| } |  |  |  |

#### –	PTRS-UplinkConfig

Table 4.6.3-111: PTRS-UplinkConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PTRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecoderDisabled SEQUENCE { |  |  |  |
| frequencyDensity | Not present |  |  |
| timeDensity | Not present |  |  |
| maxNrofPorts | n1 |  |  |
| resourceElementOffset | Not present |  |  |
| ptrs-Power | p00 |  |  |
| } |  |  |  |
| transformPrecoderEnabled SEQUENCE { |  |  |  |
| sampleDensity SEQUENCE (SIZE (5)) OF INTEGER { | 5 entries |  |  |
| INTEGER[1] | 1 | entry 1 |  |
| INTEGER[2] | 8 | entry 2 |  |
| INTEGER[3] | 32 | entry 3 |  |
| INTEGER[4] | 32 | entry 4 |  |
| INTEGER[5] | 108 | entry 5 |  |
| } |  |  |  |
| timeDensityTransformPrecoding | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	PUCCH-Config

Table 4.6.3-112: PUCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| resourceSetToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet { | 2 entries |  |  |
| PUCCH-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| pucch-ResourceSetId | 0 |  |  |
| resourceList SEQUENCE (SIZE (0..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId { | 8 entries |  |  |
| PUCCH-ResourceId[1] | 0 | entry 1 |  |
| PUCCH-ResourceId[2] | 1 | entry 2 |  |
| PUCCH-ResourceId[3] | 2 | entry 3 |  |
| PUCCH-ResourceId[4] | 3 | entry 4 |  |
| PUCCH-ResourceId[5] | 4 | entry 5 |  |
| PUCCH-ResourceId[6] | 5 | entry 6 |  |
| PUCCH-ResourceId[7] | 6 | entry 7 |  |
| PUCCH-ResourceId[8] | 7 | entry 8 |  |
| } |  |  |  |
| maxPayloadSize | Not present |  |  |
| } |  |  |  |
| PUCCH-ResourceSet[2] SEQUENCE { |  | entry 2 |  |
| pucch-ResourceSetId | 1 |  |  |
| resourceList SEQUENCE (SIZE (8..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId { | 8 entries |  |  |
| PUCCH-ResourceId[1] | 8 | entry 1 |  |
| PUCCH-ResourceId[2] | 9 | entry 2 |  |
| PUCCH-ResourceId[3] | 10 | entry 3 |  |
| PUCCH-ResourceId[4] | 11 | entry 4 |  |
| PUCCH-ResourceId[5] | 12 | entry 5 |  |
| PUCCH-ResourceId[6] | 13 | entry 6 |  |
| PUCCH-ResourceId[7] | 14 | entry 7 |  |
| PUCCH-ResourceId[8] | 15 | entry 8 |  |
| } |  |  |  |
| maxPayloadSize | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceSetToReleaseList | Not present |  |  |
| resourceToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource { | 16 entries |  |  |
| PUCCH-Resource[1] SEQUENCE { |  | entry 1 |  |
| pucch-ResourceId | 0 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[2] SEQUENCE { |  | entry 2 |  |
| pucch-ResourceId | 1 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[3] SEQUENCE { |  | entry 3 |  |
| pucch-ResourceId | 2 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[4] SEQUENCE { |  | entry 4 |  |
| pucch-ResourceId | 3 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[5] SEQUENCE { |  | entry 5 |  |
| pucch-ResourceId | 4 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[6] SEQUENCE { |  | entry 6 |  |
| pucch-ResourceId | 5 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[7] SEQUENCE { |  | entry 7 |  |
| pucch-ResourceId | 6 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[8] SEQUENCE { |  | entry 8 |  |
| pucch-ResourceId | 7 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format1 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 14 |  |  |
| startingSymbolIndex | 0 |  |  |
| timeDomainOCC | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[9] SEQUENCE { |  | entry 9 |  |
| pucch-ResourceId | 8 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[10] SEQUENCE { |  | entry 10 |  |
| pucch-ResourceId | 9 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[11] SEQUENCE { |  | entry 11 |  |
| pucch-ResourceId | 10 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[12] SEQUENCE { |  | entry 12 |  |
| pucch-ResourceId | 11 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBs | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[13] SEQUENCE { |  | entry 13 |  |
| pucch-ResourceId | 12 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRB | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[14] SEQUENCE { |  | entry 14 |  |
| pucch-ResourceId | 13 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRBsinitial | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[15] SEQUENCE { |  | entry 15 |  |
| pucch-ResourceId | 14 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format2 SEQUENCE { |  |  |  |
| nrofPRB | 6 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 12 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| PUCCH-Resource[16] SEQUENCE { |  | entry 16 |  |
| pucch-ResourceId | 15 |  |  |
| startingPRB | PRB-Id |  |  |
| intraSlotFrequencyHopping | enabled |  |  |
| secondHopPRB | PRB-Id with condition secondHopPRB |  |  |
| format CHOICE { |  |  |  |
| format3 SEQUENCE { |  |  |  |
| nrofPRBs | 1 |  |  |
| nrofSymbols | 14 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceToReleaseList | Not present |  |  |
| format1CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| interslotFrequencyHopping | enabled |  |  |
| additionalDMRS | Not Present |  |  |
| maxCodeRate | Not Present |  |  |
| nrofSlots | Not present |  |  |
| pi2BPSK | Not present |  |  |
| simultaneousHARQ-ACK-CSI | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| format2 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| interslotFrequencyHopping | Not Present |  |  |
| additionalDMRS | Not Present |  |  |
| maxCodeRate | zeroDot25 |  |  |
| nrofSlots | Not present |  |  |
| pi2BPSK | Not present |  |  |
| simultaneousHARQ-ACK-CSI | True |  |  |
| } |  |  |  |
| } |  |  |  |
| format3 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| interslotFrequencyHopping | enabled |  |  |
| additionalDMRS | True |  |  |
| maxCodeRate | zeroDot25 |  |  |
| nrofSlots | Not present |  |  |
| pi2BPSK | Not present |  |  |
| simultaneousHARQ-ACK-CSI | true |  |  |
| } |  |  |  |
| } |  |  |  |
| format4 | Not present |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] | SchedulingRequestResourceConfig | entry 1 |  |
| } |  |  |  |
| schedulingRequestResourceToReleaseList | Not present |  |  |
| multi-CSI-PUCCH-ResourceList | Not present |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 8 entries |  |  |
| INTEGER[1] | 2 | entry 1 |  |
| INTEGER[2] | 3 | entry 2 |  |
| INTEGER[3] | 4 | entry 3 |  |
| INTEGER[4] | 5 | entry 4 |  |
| INTEGER[5] | 6 | entry 5 |  |
| INTEGER[6] | 7 | entry 6 |  |
| INTEGER[7] | 8 | entry 7 |  |
| INTEGER[8] | 9 | entry 8 |  |
| } |  |  |  |
| spatialRelationInfoToAddModList | Not present |  |  |
| spatialRelationInfoToReleaseList | Not present |  |  |
| pucch-PowerControl | PUCCH-PowerControl |  |  |
| } |  |  |  |

#### –	PUCCH-ConfigCommon

Table 4.6.3-113: PUCCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | 0 |  |  |
| pucch-GroupHopping | enable |  |  |
| hoppingId | Not present |  |  |
| p0-nominal | -90 |  |  |
| } |  |  |  |

#### –	PUCCH-ConfigurationList

Table 4.6.3-113A: PUCCH-ConfigurationList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigurationList-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PUCCH-PathlossReferenceRS-Id

Table 4.6.3-114: PUCCH-PathlossReferenceRS-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-PathlossReferenceRS-Id | 0 |  |  |

#### –	PUCCH-PowerControl

Table 4.6.3-115: PUCCH-PowerControl

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-PowerControl ::= SEQUENCE { |  |  |  |
| deltaF-PUCCH-f0 | 0 |  |  |
| deltaF-PUCCH-f1 | 0 |  |  |
| deltaF-PUCCH-f2 | 0 |  |  |
| deltaF-PUCCH-f3 | 0 |  |  |
| deltaF-PUCCH-f4 | 0 |  |  |
| p0-Set | Not present |  |  |
| pathlossReferenceRSs SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS { | 1 entry |  |  |
| PUCCH-PathlossReferenceRS[1] SEQUENCE { |  | entry 1 |  |
| pucch-PathlossReferenceRS-Id | PUCCH-PathlossReferenceRS-Id |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| twoPUCCH-PC-AdjustmentStates | Not present |  |  |
| } |  |  |  |

#### –	PUCCH-SpatialRelationInfo

Table 4.6.3-116: PUCCH-SpatialRelationInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-SpatialRelationInfo ::= SEQUENCE { |  |  |  |
| pucch-SpatialRelationInfoId | 1 |  |  |
| servingCellId | ServCellIndex |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| pucch-PathlossReferenceRS-Id | PUCCH-PathlossReferenceRS-Id |  |  |
| p0-PUCCH-Id | 1 |  |  |
| closedLoopIndex | i0 |  |  |
| } |  |  |  |

#### –	PUCCH-SpatialRelationInfo-Id

Table 4.6.3-116A: PUCCH-SpatialRelationInfo-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-SpatialRelationInfoId ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PUCCH-TPC-CommandConfig

Table 4.6.3-117: PUCCH-TPC-CommandConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-TPC-CommandConfig ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	PUSCH-Config

Table 4.6.3-118: PUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPUSCH | Not present |  |  |
| txConfig | Not Present |  | Short_DCI |
|  | codebook |  |  |
| dmrs-UplinkForPUSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-UplinkConfig |  |  |
| } |  |  |  |
| dmrs-UplinkForPUSCH-MappingTypeB | Not present |  |  |
| pusch-PowerControl | PUSCH-PowerControl |  |  |
| frequencyHopping | Not present |  |  |
| frequencyHoppingOffsetLists | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| pusch-TimeDomainAllocationList | Not present |  |  |
| pusch-AggregationFactor | Not present |  |  |
| mcs-Table | Not present |  |  |
| mcs-TableTransformPrecoder | Not present |  |  |
| transformPrecoder | enabled |  | TRANSFORM_PRECODER_ENABLED |
|  | Not present | TRANSFORM_PRECODER_DISABLED |  |
| codebookSubset | Not present |  | Short_DCI |
|  | nonCoherent |  |  |
| maxRank | Not present |  | Short_DCI |
|  | 1 |  |  |
|  | 2 |  | 2TX_UL_MIMO |
| rbg-Size | Not present |  |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| betaOffsets CHOICE { |  |  |  |
| semiStatic | BetaOffsets |  |  |
| } |  |  |  |
| scaling | f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| tp-pi2BPSK | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (DFT-s-OFDM UL waveform is configured) |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |

#### –	PUSCH-ConfigCommon

Table 4.6.3-119: PUSCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| groupHoppingEnabledTransformPrecoding | Not present |  |  |
| pusch-TimeDomainAllocationList | PUSCH-TimeDomainResourceAllocationList |  |  |
| msg3-DeltaPreamble | 1 |  |  |
| p0-NominalWithGrant | -90 |  |  |
| } |  |  |  |

#### –	PUSCH-PowerControl

Table 4.6.3-120: PUSCH-PowerControl

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| tpc-Accumulation | Not present |  |  |
| msg3-Alpha | alpha08 |  |  |
| p0-NominalWithoutGrant | -90 |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  | entry 1 |  |
| p0-PUSCH-AlphaSetId | 0 |  |  |
| p0 | 0 |  |  |
| alpha | alpha08 |  |  |
| } |  |  |  |
| } |  |  |  |
| pathlossReferenceRSToAddModList SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS { | 1 entry |  |  |
| PUSCH-PathlossReferenceRS[1] SEQUENCE { |  | entry 1 |  |
| pusch-PathlossReferenceRS-Id | 0 |  |  |
| referenceSignal CHOICE{ |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pathlossReferenceRSToReleaseList | Not present |  |  |
| twoPUSCH-PC-AdjustmentStates | Not present |  |  |
| deltaMCS | Not present |  |  |
| sri-PUSCH-MappingToAddModList SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl { | 1 entry |  |  |
| SRI-PUSCH-PowerControl[1] SEQUENCE { |  | entry 1 |  |
| sri-PUSCH-PowerControlId | 0 |  |  |
| sri-PUSCH-PathlossReferenceRS-Id | 0 |  |  |
| sri-P0-PUSCH-AlphaSetId | 0 |  |  |
| sri-PUSCH-ClosedLoopIndex | i0 |  |  |
| } |  |  |  |
| } |  |  |  |
| sri-PUSCH-MappingToReleaseList | Not present |  |  |
| } |  |  |  |

#### –	PUSCH-ServingCellConfig

Table 4.6.3-121: PUSCH-ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| codeBlockGroupTransmission | Not present |  |  |
| rateMatching | Not present |  |  |
| xOverhead | Not present |  |  |
| } |  |  |  |

#### –	PUSCH-TimeDomainResourceAllocationList

Table 4.6.3-122: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | 4 |  |  |
|  | 2 |  | RF_FR1_15kHz OR RF_FR1_30kHz OR RF_FR2_DL |
|  | 3 |  | RF_FR2_120kHz_DL |
|  | 6 |  | RF_FR1_60kHz |
|  | 4 |  | RF_FR2_60kHz_UL |
|  | 8 |  | RF_FR2_120kHz_UL |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | Not present |  |  |
|  | 2 | K2+ Δ=4 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_15kHz |
|  | 6 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_30kHz |
|  | 3 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR2 |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Values are chosen so that first slot of a TDD-UL-DL slot configuration period can be used for the Random Access Response and the last slot (of the same or another period) for the corresponding Msg3. |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| RF_FR1_15kHz | RF testing in FR1. SCS is set to 15kHz. |
| RF_FR1_30kHz | RF testing in FR1. SCS is set to 30kHz. |
| RF_FR1_60kHz | RF testing in FR1. SCS is set to 60kHz. |
| RF_FR2_60kHz_DL | RF testing in FR2. SCS is set to 60kHz for Rx measurements. |
| RF_FR2_120kHz_DL | RF testing in FR2. SCS is set to 120kHz for Rx measurements. |
| RF_FR2_60kHz_UL | RF testing in FR2. SCS is set to 60kHz for test cases with 1 ms measurement period for UL measurement. |
| RF_FR2_120kHz_UL | RF testing in FR2. SCS is set to 120kHz for test cases with 1 ms measurement period for UL measurement. |

#### –	PUSCH-TPC-CommandConfig

Table 4.6.3-123: PUSCH-TPC-CommandConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TPC-CommandConfig ::= SEQUENCE { |  |  |  |
| tpc-Index | Not present |  |  |
| tpc-IndexSUL | Not present |  |  |
| targetCell | Not present |  |  |
| } |  |  |  |

#### –	Q-OffsetRange

Table 4.6.3-124: Q-OffsetRange

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Q-OffsetRange | dB0 |  |  |

#### –	Q-QualMin

Table 4.6.3-125: Q-QualMin

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Q-QualMin | FFS |  |  |

#### –	Q-RxLevMin

Table 4.6.3-126: Q-RxLevMin

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Q-RxLevMin | FFS |  |  |

#### –	QuantityConfig

Table 4.6.3-127: QuantityConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig ::= SEQUENCE { |  |  |  |
| quantityConfigNR-List SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR { | 2 entries |  |  |
| QuantityConfigNR[1] SEQUENCE { |  | entry 1 |  |
| quantityConfigCell SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigRS-Index SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| QuantityConfigNR[2] SEQUENCE { |  | entry 2 |  |
| quantityConfigCell SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigRS-Index SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  | INTER-RAT |
| filterCoefficientRSRP | FilterCoefficient |  |  |
| filterCoefficientRSRQ | FilterCoefficient |  |  |
| filterCoefficientRS-SINR | FilterCoefficient |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-RAT | Configuration for EUTRA inter-RAT measurements |

#### –	RACH-ConfigCommon

Table 4.6.3-128: RACH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | Not present |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| one | n8 |  | FR1 |
|  | n4 |  | FR2 |
| } |  |  |  |
| groupBconfigured | Not present |  |  |
| ra-ContentionResolutionTimer | sf64 |  |  |
| rsrp-ThresholdSSB | RSRP-Range |  |  |
| rsrp-ThresholdSSB-SUL | Not present |  |  |
|  | RSRP-Range |  | SUL |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 for the NR Cell. |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| restrictedSetConfig | unrestrictedSet |  |  |
| msg3-transformPrecoder | Not present | transform precoding is disabled for Msg3 PUSCH transmission and any PUSCH transmission scheduled with DCI format 0_0 |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | Supplementary uplink |

#### –	RACH-ConfigCommonTwoStepRA

Table 4.6.3-128A: RACH-ConfigCommonTwoStepRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	RACH-ConfigDedicated

Table 4.6.3-129: RACH-ConfigDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated ::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| ssb-perRACH-Occasion | one |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource { | 1 entry |  |  |
| CFRA-SSB-Resource[1] SEQUENCE { |  | entry 1 |  |
| ssb | SSB-Index |  |  |
| ra-PreambleIndex | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ra-Prioritization | Not present |  |  |
| } |  |  |  |

#### –	RACH-ConfigGeneric

Table 4.6.3-130: RACH-ConfigGeneric

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 160 |  | FR1 |
|  | 149 |  | FR2 |
| msg1-FDM | four |  | FR1 |
|  | one |  | FR2 |
| msg1-FrequencyStart | 0 |  |  |
| zeroCorrelationZoneConfig | 15 |  |  |
| preambleReceivedTargetPower | -118 |  |  |
| preambleTransMax | n7 |  |  |
| powerRampingStep | dB4 |  |  |
| ra-ResponseWindow | sl20 |  |  |
|  | sl10 |  | FR1 AND SCS15 |
| } |  |  |  |

#### –	RACH-ConfigGenericTwoStepRA

Table 4.6.3-130A: RACH-ConfigGenericTwoStepRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGenericTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	RA-Prioritization

Table 4.6.3-131: RA-Prioritization

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RA-Prioritization | 0 |  |  |

#### –	RadioBearerConfig

Table 4.6.3-132: RadioBearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB1 |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB2, RESUME |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | RESUME |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB3 |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| srb-Identity | SRB-Identity with condition SRB3 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 2 entries |  | SRB_NR_PDCP |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | SRB_NR_PDCP AND Re-establish_PDCP |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| SRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| SRB-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | SRB_NR_PDCP AND Re-establish_PDCP |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb3-ToRelease | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | EN-DC_DRB |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | EN-DC_DRB AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | EN-DC_DRB AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | MCG_NR_PDCP |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 12 | EPS Bearer Id of default MCG DRB |  |
| } |  |  |  |
| drb-Identity | 8 | DRB Id of default MCG DRB |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRB1 |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB1 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | DRB1 AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | DRB1 AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRB2 |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | DRB2 AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | DRB2 AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n entries | n is the number of DRBs established before RRC resume | RESUME |
| DRB-ToAddMod[k, k=1..n] SEQUENCE { |  | entry [k, k=1..n] |  |
| cnAssociation | Not present |  |  |
| drb-Identity | DRB-Identity with condition DRBk |  |  |
| reestablishPDCP | true |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| securityConfig | Not present |  | SRB1 |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig | SecurityAlgorithmConfig |  |  |
| keyToUse | master |  |  |
|  | secondary |  | SRB3, EN-DC_DRB |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB3 | Establishment of SRB3 |
| MCG_NR_PDCP | EN-DC MCG DRB configured or reconfigured with NR PDCP |
| SRB_NR_PDCP | EN-DC SRB1 and SRB2 configured with NR PDCP |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| DRB1 | Establishment of DRB1 |
| DRB2 | Establishment of DRB2 |
| EN-DC_DRB | EN-DC DRB configured on SCG |
| Re-establish_PDCP | Re-establishment of PDCP |
| Recover_PDCP | Recovery of PDCP |
| RESUME | Used in RRCResume Message |

#### –	RadioLinkMonitoringConfig

Table 4.6.3-133: RadioLinkMonitoringConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS { | 1 entry |  |  |
| RadioLinkMonitoringRS[1] SEQUENCE { |  | entry 1 |  |
| radioLinkMonitoringRS-Id | RadioLinkMonitoringRS-Id |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| failureDetectionResourcesToReleaseList | Not present |  |  |
| beamFailureInstanceMaxCount | Not present |  |  |
| beamFailureDetectionTimer | Not present |  |  |
| } |  |  |  |

#### –	RadioLinkMonitoringRS-Id

Table 4.6.3-134: RadioLinkMonitoringRS-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringRS-Id | 0 |  |  |

#### –	RAN-AreaCode

Table 4.6.3-135: RAN-AreaCode

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RAN-AreaCode | 1 |  |  |

#### –	RateMatchPattern

Table 4.6.3-136: RateMatchPattern

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPattern ::= SEQUENCE { |  |  |  |
| rateMatchPatternId | RateMatchPatternId |  |  |
| patternType CHOICE { |  |  |  |
| controlResourceSet | ControlResourceSetId |  |  |
| } |  |  |  |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| dummy | semiStatic | Dummy IE value |  |
| } |  |  |  |

#### –	RateMatchPatternId

Table 4.6.3-137: RateMatchPatternId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternId | 0 |  |  |

#### –	RateMatchPatternLTE-CRS

Table 4.6.3-138: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ReferenceTimeInfo

Table 4.6.3-138A: ReferenceTimeInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReferenceTimeInfo-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	RejectWaitTime

Table 4.6.3-139: RejectWaitTime

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RejectWaitTime | 1 |  |  |

#### –	RepetitionSchemeConfig

Table 4.6.3-139A: RepetitionSchemeConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RepetitionSchemeConfig-r16 ::= CHOICE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ReportConfigId

Table 4.6.3-140: ReportConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigId | 1 |  |  |

#### –	ReportConfigInterRAT

Table 4.6.3-141: ReportConfigInterRAT (EUTRA-Thres, NR-Thres)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  | EVENT_B1 |
| b1-ThresholdEUTRA CHOICE { |  |  |  |
| rsrp | EUTRA-Thres | INTEGER (0..97) |  |
| } |  |  |  |
| reportOnLeave | FALSE |  |  |
| Hysteresis | 0 (0 dB) | The actual value is field value * 0.5 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| eventB2 SEQUENCE { |  |  | EVENT_B2 |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | NR-Thres | INTEGER(0..127) |  |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | EUTRA-Thres | INTEGER (0..97) |  |
| } |  |  |  |
| reportOnLeave | FALSE |  |  |
| Hysteresis | 3 (1.5dB) | The actual value is field value * 0.5 dB |  |
| timeToTrigger | ms1024 |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType | ssb |  |  |
| reportInterval | ms120 |  |  |
| reportAmount | r2 |  |  |
| reportQuantity SEQUENCE { |  |  |  |
| Rsrp | TRUE |  |  |
| rsrq | TRUE |  |  |
| sinr | FALSE |  |  |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| } |  |  |  |
| reportCGI SEQUENCE { |  |  | CGI |
| cellForWhichToReportCGI | EUTRA-PhysCellId |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EVENT_B1 | Configuration of Event B1 |
| EVENT_B2 | Configuration of Event B2 |
| CGI | Configuration of CGI measurement |

#### –	ReportConfigNR

Table 4.6.3-142: ReportConfigNR(Thres1, Thres2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| rsType | ssb |  |  |
| reportInterval | ReportInterval |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
|  | true |  | pc_ss_SINR_Meas |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityRS-Indexes | Not present |  |  |
| maxNrofRS-IndexesToReport | Not present |  |  |
| includeBeamMeasurements | false |  |  |
| useWhiteCellList | false |  |  |
| } |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA1 SEQUENCE { |  |  | EVENT_A1 |
| a1-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| eventA2 SEQUENCE { |  |  | EVENT_A2 |
| a2-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| eventA3 SEQUENCE { |  |  | EVENT_A3 |
| a3-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useWhiteCellList | false |  |  |
| } |  |  |  |
| eventA4 SEQUENCE { |  |  | EVENT_A4 |
| a4-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useWhiteCellList | false |  |  |
| } |  |  |  |
| eventA5 SEQUENCE { |  |  | EVENT_A5 |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useWhiteCellList | false |  |  |
| } |  |  |  |
| eventA6 SEQUENCE { |  |  | EVENT_A6 |
| a6-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useWhiteCellList | false |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType | ssb |  |  |
| reportInterval | ReportInterval |  |  |
| reportAmount | r2 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
|  | true |  | pc_ss_SINR_Meas |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityRS-Indexes | Not present |  |  |
| maxNrofRS-IndexesToReport | Not present |  |  |
| includeBeamMeasurements | false |  |  |
| reportAddNeighMeas | Not present |  |  |
| } |  |  |  |
| reportCGI SEQUENCE { |  |  | CGI |
| cellForWhichToReportCGI | PhysCellId |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EVENT_A1 | Configuration of Event A1 |
| EVENT_A2 | Configuration of Event A2 |
| EVENT_A3 | Configuration of Event A3 |
| EVENT_A4 | Configuration of Event A4 |
| EVENT_A5 | Configuration of Event A5 |
| EVENT_A6 | Configuration of Event A6 |
| PERIODICAL | Configuration of periodical reporting |
| CGI | Configuration of CGI measurement |

#### –	ReportConfigNR-SL

Table 4.6.3-142A: ReportConfigNR-SL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR-SL-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ReportConfigToAddModList

Table 4.6.3-143: ReportConfigToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigToAddModList ::= SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | ReportConfigId |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	ReportInterval

Table 4.6.3-144: ReportInterval

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportInterval | ms480 |  |  |

#### –	ReselectionThreshold

Table 4.6.3-145: ReselectionThreshold

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReselectionThreshold | FFS |  |  |

#### –	ReselectionThresholdQ

Table 4.6.3-146: ReselectionThresholdQ

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReselectionThresholdQ | FFS |  |  |

#### –	ResumeCause

Table 4.6.3-147: ResumeCause

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ResumeCause | mt-Access |  |  |

#### –	RLC-BearerConfig

Table 4.6.3-148: RLC-BearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRBn |  | DRBn |
|  | LogicalChannelIdentity with condition SRB1 |  | SRB1 |
|  | LogicalChannelIdentity with condition SRB2 |  | SRB2 |
|  | LogicalChannelIdentity with condition SRB3 |  | SRB3 |
| servedRadioBearer CHOICE { |  |  |  |
| srb-Identity | SRB-Identity with condition SRB1 |  | SRB1 |
|  | SRB-Identity with condition SRB2 |  | SRB2 |
|  | SRB-Identity with condition SRB3 |  | SRB3 |
| drb-Identity | DRB-Identity with condition DRBn |  | DRBn |
| } |  |  |  |
| servedRadioBearer | Not present |  | RESUME |
| reestablishRLC | Not present |  |  |
|  | true |  | Re-establish_RLC, RESUME |
| rlc-Config | RLC-Config using condition AM |  | AM |
|  | RLC-Config using condition UM. |  | UM |
|  | Not present | Use default parameters as per TS 38.331 [6] clause 9.2.1 | SRB1, SRB2, SRB3, RESUME |
| mac-LogicalChannelConfig | LogicalChannelConfig using condition HI |  | AM |
|  | LogicalChannelConfig using condition LO |  | UM |
|  | LogicalChannelConfig using condition SRBn | n= 1, 2, 3 for SRB1, SRB2, SRB3 resp. | SRB1, SRB2, SRB3, RESUME |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| AM | RLC AM DRB |
| UM | RLC UM DRB |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| DRBn | Establishment of DRBn |
| Re-establish_RLC | Re-establishment of RLC |
| RESUME | Used in RRCResume Message |

#### –	RLC-Config

Table 4.6.3-149: RLC-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  | AM |
| ul-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size18 |  |  |
| t-PollRetransmit | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
| pollPDU | p32768 |  |  |
| pollByte | kB750 |  |  |
| maxRetxThreshold | t8 |  |  |
| } |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size18 |  |  |
| t-Reassembly | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
| t-StatusProhibit | ms30 |  |  |
| } |  |  |  |
| } |  |  |  |
| um-Bi-Directional SEQUENCE { |  |  | UM |
| ul-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size12 |  | pc_um_WIthLongSN |
|  | size6 |  | NOT pc_um_WIthLongSN AND pc_um_WithShortSN |
| } |  |  |  |
| dl-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size12 |  | pc_um_WIthLongSN |
|  | size6 |  | NOT pc_um_WIthLongSN AND pc_um_WithShortSN |
| t-Reassembly | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| AM | RLC AM |
| UM | RLC UM |

#### –	RLF-TimersAndConstants

Table 4.6.3-150: RLF-TimersAndConstants

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms1000 |  |  |
| n310 | n1 |  |  |
| n311 | n1 |  |  |
| t311 | ms1000 |  |  |
| } |  |  |  |

#### –	RNTI-Value

Table 4.6.3-151: RNTI-Value

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RNTI-Value | SS arbitrarily selects a value between ‘0001’H and ‘FFEF’H |  |  |
|  | SS arbitrarily selects a value between ‘0001’H and ‘FFEF’H different from the MCG RNTI-Value. |  | NR-DC_SCG |

| Condition | Explanation |
| --- | --- |
| NR-DC_SCG | Add SCG (NR-DC) |

#### –	RSRP-Range

Table 4.6.3-152: RSRP-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RSRP-Range | 0 | For measurements, 0 means L3 SS-RSRP<-156dBm according to Table 10.1.6.1-1 in TS 38.133 [13]. For thresholds, 0 means -156dBm. |  |

#### –	RSRQ-Range

Table 4.6.3-153: RSRQ-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RSRQ-Range | 0 | For measurements, 0 means SS-RSRQ<-43dB according to Table 10.1.11.1-1 in TS 38.133 [14]. For thresholds, 0 means -43.5dB. |  |

#### –	RSSI-Range

Table 4.6.3-153A: RSSI-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RSSI-Range-r16 | FFS |  |  |

#### –	SCellIndex

Table 4.6.3-154: SCellIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellIndex | 1 |  |  |
|  | 2 |  | EN-DC |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |

#### –	SchedulingRequestConfig

Table 4.6.3-155: SchedulingRequestConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestToAddModList SEQUENCE (SIZE(1..maxNrofSR-ConfigPerCellGroup)) OF SSchedulingRequestToAddMod { | 1 entry |  |  |
| SchedulingRequestToAddMod[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestId | SchedulingRequestId |  |  |
| sr-ProhibitTimer | Not present |  |  |
| sr-TransMax | n16 |  |  |
| } |  |  |  |
| } |  |  |  |
| schedulingRequestToReleaseList | Not present |  |  |
| } |  |  |  |

#### –	SchedulingRequestId

Table 4.6.3-156: SchedulingRequestId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestId | 0 |  |  |

#### –	SchedulingRequestResourceConfig

Table 4.6.3-157: SchedulingRequestResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceId | SchedulingRequestResourceId |  |  |
| schedulingRequestID | SchedulingRequestId |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 9 | With SCS = kHz15 results in repetition every 10 ms | SCS15 |
| sl20 | 9 | With SCS = kHz30 results in repetition every 10 ms | SCS30 |
| sl40 | 19 | With SCS = kHz60 results in repetition every 10 ms | FR1 AND SCS60 |
|  | 9 | With SCS = kHz60 results in repetition every 10 ms | FR2 AND SCS60 |
| sl80 | 9 | With SCS = kHz120 results in repetition every 10 ms | SCS120 |
| } |  |  |  |
| resource | 0 | ID of the PUCCH resource as configured by PUCCH-Config (Table 4.6.3-84) |  |
| } |  |  |  |

#### –	SchedulingRequestResourceId

Table 4.6.3-158: SchedulingRequestResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceId | 1 |  |  |

#### –	ScramblingId

Table 4.6.3-159: ScramblingId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ScramblingId | 0 |  |  |

#### –	SCS-SpecificCarrier

Table 4.6.3-160: SCS-SpecificCarrier

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCS-SpecificCarrier ::= SEQUENCE { |  |  |  |
| offsetToCarrier | offsetToCarrier as defined for the DL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | DL_PointA |
|  | offsetToCarrier as defined for the UL frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. | UL_PointA |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| carrierBandwidth | carrierBandwidth as defined for the frequency of the cell | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. |  |
| txDirectCurrentLocation | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DL_PointA | IE absoluteFrequencyPointA for downlink |
| UL_PointA | IE absoluteFrequencyPointA for uplink |

#### –	SDAP-Config

Table 4.6.3-161: SDAP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDAP-Config ::= SEQUENCE { |  |  |  |
| pdu-Session | The same value as the PDU session ID IE of the contained message |  |  |
| sdap-HeaderDL | absent |  |  |
| sdap-HeaderUL | present |  |  |
| defaultDRB | true |  |  |
|  | false |  | NR-DC |
| mappedQoS-FlowsToAdd SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI { | n entries |  |  |
| QFI[n] | The list of QFIs of the Authorized QoS flow descriptions IE of the contained 5GSM message | entry n |  |
| } |  |  |  |
| mappedQoS-FlowsToRelease | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR-DC | NR-NR Dual Connectivity |

#### –	SearchSpace

Table 4.6.3-162: SearchSpace

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition CSS |  | CSS |
|  | SearchSpaceId with condition USS |  | USS |
|  | SearchSpaceId with condition SISS |  | SISS |
| controlResourceSetId | ControlResourceSetId |  |  |
|  | ControlResourceSetId with condition Common0 |  | CSS, SISS |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| sl10 | 5 |  | SISS |
| } |  |  |  |
| duration | Not present | 1 slot per default |  |
|  | 2 |  | SISS |
| monitoringSymbolsWithinSlot | 10000000000000 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n4 |  | FR1 |
|  | n3 |  | FR2 |
| aggregationLevel4 | n2 |  |  |
| aggregationLevel8 | n1 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| dci-Format0-0-AndFormat1-0 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format2-0 | Not present |  |  |
| dci-Format2-1 | Not present |  |  |
| dci-Format2-2 | Not present |  |  |
| dci-Format2-3 | Not present |  |  |
| } |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 |  | Short_DCI |
| dci-Formats | formats0-1-And-1-1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |
| USS | UE-Specific SearchSpace |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SISS | SearchSpace for SI |

#### –	SearchSpaceId

Table 4.6.3-163: SearchSpaceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceID | 1 |  | CSS |
|  | 2 |  | USS |
|  | 3 |  | SISS |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |
| USS | UE-Specific SearchSpace |
| SISS | SearchSpace for SI |

#### –	SearchSpaceZero

Table 4.6.3-164: SearchSpaceZero

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceZero | 0 | Index addressing SearchSpace#0 parameter set in Tables 13.11 .. 13.15 of TS 38.213 [22] |  |

#### –	SecurityAlgorithmConfig

Table 4.6.3-165: SecurityAlgorithmConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityAlgorithmConfig ::= SEQUENCE { |  |  |  |
| cipheringAlgorithm | nea0 |  | RF |
|  | Set according to PIXIT px_NR_CipheringAlgorithm | see TS 38.523-3 [23] | SIG |
| integrityProtAlgorithm | nia2 |  |  |
|  | Set according to PIXIT px_NR_IntegrityProtAlgorithm | see TS 38.523-3 [23] | SIG |
| } |  |  |  |

#### –	SemiStaticChannelAccessConfig

Table 4.6.3-165A: SemiStaticChannelAccessConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SemiStaticChannelAccessConfig ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	Sensor-LocationInfo

Table 4.6.3-165B: Sensor-LocationInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sensor-LocationInfo-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	ServCellIndex

Table 4.6.3-166: ServCellIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServCellIndex | 0 |  |  |
|  | 1 |  | EN-DC, NR-DC_SCG |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| NR-DC_SCG | Add SCG (NR-DC) |

#### –	ServingCellConfig

Table 4.6.3-167: ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
|  | Not present |  | MEAS, RESUME |
| downlinkBWP-ToReleaseList | Not present |  |  |
| downlinkBWP-ToAddModList | Not present |  |  |
| firstActiveDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS |
| bwp-InactivityTimer | Not present |  |  |
| defaultDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS, RESUME |
| uplinkConfig | Not present |  | MEAS, No_UL |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition SUL_NUL |  | PUSCH_PUCCH_ON_SUL |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig | Not present |  | RESUME |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| } |  |  |  |
| supplementaryUplink | Not present |  |  |
| supplementaryUplink SEQUENCE { |  |  | PUSCH_PUCCH_ON_SUL |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDCCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig | Not present |  | MEAS, RESUME |
| pdsch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDSCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdsch-ServingCellConfig | Not present |  | MEAS, RESUME |
| csi-MeasConfig | Not present |  |  |
| sCellDeactivationTimer | Not present |  |  |
| crossCarrierSchedulingConfig | Not present |  |  |
| tag-Id | 0 |  |  |
| dummy | Not present |  |  |
| pathlossReferenceLinking | Not present |  |  |
| servingCellMO | Not present |  |  |
|  | MeasObjectId |  | MEAS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PUSCH_PUCCH_ON_SUL | For the purpose of SUL test under condition that supplementary uplink is configured with both PUSCH and PUCCH on SUL carrier. |
| MEAS | A NR or EN-DC measurement is configured. |
| No_UL | No uplink CA |
| RESUME | Used in RRCResume Message |

#### –	ServingCellConfigCommon

Table 4.6.3-168: ServingCellConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| downlinkConfigCommon | DownlinkConfigCommon |  |  |
|  | DownlinkConfigCommon with condition SCell_add |  | SCell_add |
| uplinkConfigCommon | UplinkConfigCommon |  |  |
|  | Not present |  | No_UL |
| supplementaryUplinkConfig | Not present |  |  |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 0100 |  | FR1 AND SSB#1 AND (2.3GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.3GHZ) |
|  | 1000 |  | FR1 AND SSB#0 AND (2.3GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.3GHz) |
| mediumBitmap | 01000000 |  | FR1 AND SSB#1 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
|  | 10000000 |  | FR1 AND SSB#0 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | 0100000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#1 |
|  | 1000000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#0 |
| } |  |  |  |
| ssb-periodicityServingCell | ms20 |  |  |
| dmrs-TypeA-Position | pos2 |  |  |
| lte-CRS-ToMatchAround | Not present |  |  |
| rateMatchPatternToAddModList | Not present |  |  |
| rateMatchPatternToReleaseList | Not present |  |  |
| ssbSubcarrierSpacing | SubcarrierSpacing | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. Value SS block SCS. |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | TDD |
|  | Not present |  | FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=2.3GHz | Frequency range <= 2.3GHz |
| 2.3GHz<FREQ<=3GHz | Frequency range > 2.3GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| No_UL | No uplink CA |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |

#### –	ServingCellConfigCommonSIB

Table 4.6.3-169: ServingCellConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB |  |  |
|  | UplinkConfigCommonSIB with condition SUL_NUL |  | SUL |
| supplementaryUplink | Not present |  |  |
|  | UplinkConfigCommonSIB with condition SUL_SUL |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid | SSB#1 |
|  | ’1000 0000’B |  | SSB#0 |
| groupPresence | Not present |  | FR1 |
|  | ’1000 0000’B |  | FR2 |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | TDD |
|  | Not present |  | FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | Supplementary uplink |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |

#### –	ShortI-RNTI-Value

Table 4.6.3-170: ShortI-RNTI-Value

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ShortI-RNTI-Value | SS arbitrarily selects a value between ’00 0001’H and ‘FF FFFF’H. | BIT STRING (SIZE(24)) |  |

#### –	ShortMAC-I

Table 4.6.3-171: ShortMAC-I

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ShortMAC-I | The 16 least significant bits of the MAC-I calculated using the security configuration of the source PCell. |  |  |

#### –	SINR-Range

Table 4.6.3-172: SINR-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SINR-Range | 0 | For measurements, 0 means SS-SINR<-23dB according to Table 10.1.16.1-1 in TS 38.133 [14]. For thresholds, 0 means -23dB. |  |

#### –	SI-RequestConfig

Table 4.6.3-172A: SI-RequestConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI-RequestConfig ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SI-SchedulingInfo

Table 4.6.3-173: SI-SchedulingInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI-SchedulingInfo ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo { | n entries See subclause 4.4.3.1 |  |  |
| SchedulingInfo[n] SEQUENCE { |  | entry n |  |
| si-BroadcastStatus | broadcasting |  |  |
| si-Periodicity | See subclause 4.4.3.1 |  |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | n entries |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry n |  |
| type | See subclause 4.4.3.1 |  |  |
| valueTag | 0 |  |  |
| areaScope | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| si-WindowLength | s80 |  | FR1 |
|  | s160 |  | FR2 |
| si-RequestConfig | Not present |  |  |
| si-RequestConfigSUL | Not present |  |  |
| systemInformationAreaID | ’0000 0000 0000 0000 0000 0001’B |  |  |
| } |  |  |  |

#### –	SK-Counter

Table 4.6.3-173A: SK-Counter

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SK-Counter | 0 |  |  |

#### –	SlotFormatCombinationsPerCell

Table 4.6.3-174: SlotFormatCombinationsPerCell

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SlotFormatCombinationsPerCell ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SlotFormatIndicator

Table 4.6.3-175: SlotFormatIndicator

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SlotFormatIndicator ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	S-NSSAI

Table 4.6.3-176: S-NSSAI

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| S-NSSAI ::= CHOICE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SpeedStateScaleFactors

Table 4.6.3-177: SpeedStateScaleFactors

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SpeedStateScaleFactors ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SPS-Config

Table 4.6.3-179: SPS-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SPS-Config ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SPS-ConfigIndex

Table 4.6.3-179A: SPS-ConfigIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SPS-ConfigIndex-r16 | FFS |  |  |

#### –	SPS-PUCCH-AN

Table 4.6.3-179B: SPS-PUCCH-AN

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SPS-PUCCH-AN-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SPS-PUCCH-AN-List

Table 4.6.3-179C: SPS-PUCCH-AN-List

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SPS-PUCCH-AN-List-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SRB-Identity

Table 4.6.3-180: SRB-Identity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRB-Identity | 1 |  | SRB1 |
|  | 2 |  | SRB2 |
|  | 3 |  | SRB3 |

| Condition | Explanation |
| --- | --- |
| SRB1 | SRB1 |
| SRB2 | SRB2 |
| SRB3 | SRB3 |

#### –	SRS-CarrierSwitching

Table 4.6.3-181: SRS-CarrierSwitching

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-CarrierSwitching ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SRS-Config

Table 4.6.3-182: SRS-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet { | 1 entry |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceSetId | 0 |  |  |
| srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId { | 1 entry |  |  |
| SRS-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| aperiodicSRS-ResourceTrigger | 1 |  |  |
| csi-RS | Not present |  |  |
| slotOffset | 7 |  | FR1 |
|  | 4 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| usage | codebook |  |  |
| alpha | Alpha |  |  |
| p0 | 0 |  |  |
| pathlossReferenceRS CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| srs-PowerControlAdjustmentStates | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource { | 1 entry |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| srs-ResourceId | 0 |  |  |
| nrofSRS-Ports | ports2 |  | 2TX_UL_MIMO |
|  | port1 |  |  |
| ptrs-PortIndex | Not present |  |  |
| transmissionComb CHOICE { |  |  |  |
| n2 SEQUENCE { |  |  |  |
| combOffset-n2 | 0 |  |  |
| cyclicShift-n2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqDomainPosition | 0 |  |  |
| freqDomainShift | 0 |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 0 |  |  |
|  | 63 |  | FR1_100MHz |
|  | 17 |  | FR2_100MHz |
| b-SRS | 0 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | groupHopping |  |  |
| resourceType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId | 0 |  |  |
| spatialRelationInfo SEQUENCE { | SRS-SpatialRelationInfo |  |  |
| servingCellId | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| tpc-Accumulation | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| FR1_100MHz | FR1 is used under the test. CBW is set to 100MHz. |
| FR2_100MHz | FR2 is used under the test. CBW is set to 100MHz. |

#### –	SRS-RSRP-Range

Table 4.6.3-182A: SRS-RSRP-Range

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-RSRP-Range-r16 | FFS |  |  |

#### –	SRS-TPC-CommandConfig

Table 4.6.3-183: SRS-TPC-CommandConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-TPC-CommandConfig ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SSB-Index

Table 4.6.3-184: SSB-Index

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-Index | Set according to Table 4.4.2-2 for the NR Cell |  |  |

#### –	SSB-MTC

Table 4.6.3-185: SSB-MTC

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  |  |
|  | 10 |  | SIG AND INTER-FREQ_ODD |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  |  |
|  | 10 |  | SIG AND INTER-FREQ_ODD |
| } |  |  |  |
| duration | sf2 |  | FR1 |
|  | sf3 |  | FR2 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-FREQ_ODD | When the SFNoffset of inter frequency neighbour cell is odd number. SFNoffset is defined in TS 38.523-3 [23]Table 7.1.5.2-1 |

Table 4.6.3-186: SSB-MTC2

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC2 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SSB-PositionQCL-Relation

Table 4.6.3-186A: SSB-PositionQCL-Relation

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-PositionQCL-Relation-r16 | FFS |  |  |

#### –	SSB-ToMeasure

Table 4.6.3-187: SSB-ToMeasure

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-ToMeasure ::= CHOICE { |  |  |  |
| shortBitmap | 1100 |  | (FREQ<=3GHz AND (FR1_FDD OR NOT CASE_C)) OR (FREQ<=2.4GHz AND FR1_TDD) |
| mediumBitmap | 11000000 |  | (FREQ>3GHz AND FR1) OR (FREQ>2.4GHz AND FR1_TDD AND CASE_C) |
| longBitmap | 11000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  | FR2 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=2.4GHz | Frequency range <= 2.4GHz |
| FREQ>2.4GHz | Frequency range > 2.4GHz |
| FREQ<=3GHz | Frequency range <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| FR1_TDD | TDD frequency range < 6GHz |
| FR2_TDD | TDD frequency range > 6GHz |
| FR1_FDD | FDD frequency range < 6GHz |
| CASE_C | SS Block pattern “Case C” to be applied for the given band and subcarrier spacing according to TS 38.101-1 [7] Table 5.4.3.3-1 |

#### –	SS-RSSI-Measurement

Table 4.6.3-187A: SS-RSSI-Measurement

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SS-RSSI-Measurement ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	SubcarrierSpacing

Table 4.6.3-188: SubcarrierSpacing

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SubcarrierSpacing | According to clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise. |  |  |

#### –	TAG-Config

Table 4.6.3-189: TAG-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TAG-Config ::= SEQUENCE { |  |  |  |
| tag-ToReleaseList | Not present |  |  |
| tag-ToAddModList SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG { | 1 entry |  |  |
| TAG[1] SEQUENCE { |  | entry 1 |  |
| tag-Id | 0 |  |  |
| timeAlignmentTimer | infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	TCI-State

Table 4.6.3-190: TCI-State

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-State ::= SEQUENCE { |  |  |  |
| tci-StateId | TCI-StateId |  |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| cell | Not present |  |  |
| bwp-Id | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index |  |  |
| } |  |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| qcl-Type2 | Not present |  |  |
| } |  |  |  |

#### –	TCI-StateId

Table 4.6.3-191: TCI-StateId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-StateId | 0 |  |  |

#### –	TDD-UL-DL-ConfigCommon

Table 4.6.3-192: TDD-UL-DL-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1 |
|  | ms0p625 |  | SIG AND FR2 |
|  | ms2 |  | RF AND FR2 |
| nrofDownlinkSlots | 7 |  | (FR1 AND SCS30) OR (RF AND FR2 AND SCS120) |
|  | 3 |  | (FR1 AND SCS15) OR (FR2 AND SCS60) OR (SIG AND FR2 AND SCS120) |
|  | 14 |  | FR1 AND SCS60 |
| nrofDownlinkSymbols | 6 |  | FR1 AND SCS30 |
|  | 10 |  | (FR1 AND SCS15) OR (SIG AND FR2 AND (SCS60 OR SCS120)) |
|  | 12 |  | (FR1 AND SCS60) OR (RF AND FR2 AND SCS120) |
|  | 4 |  | RF AND FR2 AND SCS60 |
| nrofUplinkSlots | 2 |  | FR1 AND SCS30 |
|  | 1 |  | (FR1 AND SCS15) OR (SIG AND FR2 AND (SCS60 OR SCS120)) |
|  | 4 |  | (FR1 AND SCS60) OR (RF AND FR2 AND SCS60) |
|  | 8 |  | RF AND FR2 AND SCS120 |
| nrofUplinkSymbols | 4 |  | FR1 AND SCS30 |
|  | 2 |  | (FR1 AND SCS15) OR (SIG AND FR2 AND (SCS60 OR SCS120)) |
|  | 8 |  | FR1 AND SCS60 |
|  | 0 |  | RF AND FR2 AND (SCS60 OR SCS120) |
| } |  |  |  |
| pattern2 | Not present |  |  |
| } |  |  |  |

#### –	TDD-UL-DL-ConfigDedicated

Table 4.6.3-192A: TDD-UL-DL-ConfigDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigDedicated ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	TrackingAreaCode

Table 4.6.3-193: TrackingAreaCode

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TrackingAreaCode | See table 4.4.2-3 | BIT STRING (SIZE (24)) |  |

#### –	T-Reselection

Table 4.6.3-194: T-Reselection

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| T-Reselection | FFS |  |  |

#### –	TimeToTrigger

Table 4.6.3-195: TimeToTrigger

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TimeToTrigger | ms320 |  |  |

#### –	UAC-BarringInfoSetIndex

Table 4.6.3-196: UAC-BarringInfoSetIndex

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UAC-BarringInfoSetIndex | FFS |  |  |

#### –	UAC-BarringInfoSetList

Table 4.6.3-197: UAC-BarringInfoSetList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UAC-BarringInfoSetList | FFS |  |  |

#### –	UAC-BarringPerCatList

Table 4.6.3-198: UAC-BarringPerCatList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UAC-BarringPerCatList | FFS |  |  |

#### –	UAC-BarringPerPLMN-List

Table 4.6.3-199: UAC-BarringPerPLMN-List

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UAC-BarringPerPLMN-List | FFS |  |  |

#### –	UE-TimersAndConstants

Table 4.6.3-200: UE-TimersAndConstants

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t300 | ms1000 |  |  |
| t301 | ms1000 |  |  |
| t310 | ms1000 |  |  |
| n310 | n1 |  |  |
| t311 | ms30000 |  |  |
| n311 | n1 |  |  |
| t319 | ms1000 |  |  |
| } |  |  |  |

#### –	UL-DelayValueConfig

Table 4.6.3-200A: UL-DelayValueConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UL-DelayValueConfig-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	UplinkCancellation

Table 4.6.3-200B: UplinkCancellation

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkCancellation-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	UplinkConfigCommon

Table 4.6.3-201: UplinkConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommon ::= SEQUENCE { |  |  |  |
| frequencyInfoUL | FrequencyInfoUL |  |  |
| initialUplinkBWP | BWP-UplinkCommon |  |  |
| timeAlignmentTimerCommon | infinity |  |  |
| } |  |  |  |

#### –	UplinkConfigCommonSIB

Table 4.6.3-202: UplinkConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommonSIB SEQUENCE { |  |  |  |
| frequencyInfoUL | FrequencyInfoUL-SIB |  |  |
| initialUplinkBWP | BWP-UplinkCommon |  |  |
|  | BWP-UplinkCommon with confition SUL_SUL |  | SUL_SUL |
|  | BWP-UplinkCommon with confition SUL_NUL |  | SUL_NUL |
| timeAlignmentTimerCommon | infinity |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL_SUL | On the SUL carrier when supplementary carrier is configured |
| SUL_NUL | On the NUL carrier when supplementary carrier is configured |

#### –	UplinkTxDirectCurrentList

Table 4.6.3-203: UplinkTxDirectCurrentList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkTxDirectCurrentList ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF UplinkTxDirectCurrentCell { | 1 entry |  |  |
| UplinkTxDirectCurrentCell[1] SEQUENCE { |  | entry 1 |  |
| FFS |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	ZP-CSI-RS-Resource

Table 4.6.3-204: ZP-CSI-RS-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| zp-CSI-RS-ResourceId | ZP-CSI-RS-ResourceId |  |  |
| resourceMapping | CSI-RS-ResourceMapping |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset |  |  |
| } |  |  |  |

#### –	ZP-CSI-RS-ResourceSet

Table 4.6.3-205: ZP-CSI-RS-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| zp-CSI-RS-ResourceSetId | ZP-CSI-RS-ResourceSetId |  |  |
| zp-CSI-RS-ResourceIdList SEQUENCE (SIZE(1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId { | 1 entry |  |  |
| ZP-CSI-RS-ResourceId[1] | FFS | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	ZP-CSI-RS-ResourceSetId

Table 4.6.3-206: ZP-CSI-RS-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-ResourceSetId | FFS |  |  |

### 4.6.4	UE capability information elements

#### –	AccessStratumRelease

Table 4.6.4-1: AccessStratumRelease

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AccessStratumRelease | Same as indicated in TC applicability in TS 38.523-2 [19] |  |  |

#### –	BandCombinationList

Table 4.6.4-2: BandCombinationList

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BandCombinationList::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination { | At least 1 entry |  |  |
| BandCombination[1] SEQUENCE { |  | entry 1 |  |
| bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters { | 1 entry |  |  |
| BandParameters[1] CHOICE { |  | entry 1 |  |
| eutra SEQUENCE { |  |  |  |
| bandEUTRA | FreqBandIndicatorEUTRA |  |  |
| ca-BandwidthClassDL-EUTRA | Not checked |  |  |
| ca-BandwidthClassUL-EUTRA | Not checked |  |  |
| } |  |  |  |
| nr SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR |  |  |
| ca-BandwidthClassDL-NR | Not checked |  |  |
| ca-BandwidthClassUL-NR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombination | Not checked |  |  |
| ca-ParametersEUTRA | Not checked |  |  |
| ca-ParametersNR | Not checked |  |  |
| mrdc-Parameters | Not checked |  |  |
| supportedBandwidthCombinationSet | BIT STRING (SIZE (1..32)) |  |  |
| powerClass-v1530 | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CA-BandwidthClassEUTRA

Table 4.6.4-3: CA-BandwidthClassEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CA-BandwidthClassEUTRA | Not checked |  |  |

#### –	CA-BandwidthClassNR

Table 4.6.4-4: CA-BandwidthClassNR

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CA-BandwidthClassNR | Not checked |  |  |

#### –	CA-ParametersEUTRA

Table 4.6.4-5: CA- ParametersEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CA-ParametersEUTRA ::= SEQUENCE { |  |  |  |
| multipleTimingAdvance | Not checked |  |  |
| simultaneousRx-Tx | Not checked |  |  |
| supportedNAICS-2CRS-AP | Not checked |  |  |
| additionalRx-Tx-PerformanceReq | Not checked |  |  |
| ue-CA-PowerClass-N | Not checked |  |  |
| supportedBandwidthCombinationSetEUTRA-v1530 | Not checked |  |  |
| } |  |  |  |

#### –	CA-ParametersNR

Table 4.6.4-6: CA- ParametersNR

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CA-ParametersNR ::= SEQUENCE { |  |  |  |
| dummy | Not checked |  |  |
| parallelTxSRS-PUCCH-PUSCH | Not checked |  |  |
| parallelTxPRACH-SRS-PUCCH-PUSCH | Not checked |  |  |
| simultaneousRxTxInterBandCA | Not checked |  |  |
| simultaneousRxTxSUL | Not checked |  |  |
| diffNumerologyAcrossPUCCH-Group | Not checked |  |  |
| diffNumerologyWithinPUCCH-GroupSmallerSCS | Not checked |  |  |
| supportedNumberTAG | Not checked |  |  |
| } |  |  |  |

#### –	CA-ParametersNRDC

Table 4.6.4-6AA: CA- ParametersNRDC

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CA-ParametersNRDC ::= SEQUENCE { |  |  |  |
| ca-ParametersNR-ForDC | Not checked |  |  |
| ca-ParametersNR-ForDC-v1540 | Not checked |  |  |
| ca-ParametersNR-ForDC-v1550 | Not checked |  |  |
| ca-ParametersNR-ForDC-v1560 | Not checked |  |  |
| featureSetCombinationDC | Not checked |  |  |
| } |  |  |  |

#### –	CodebookParameters

Table 4.6.4-6A: CodebookParameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookParameters ::= SEQUENCE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| singlePanel SEQUENCE { |  |  |  |
| supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource { | 1 entry |  |  |
| SupportedCSI-RS-Resource[1] SEQUENCE { |  | entry 1 |  |
| maxNumberTxPortsPerResource | Not checked |  |  |
| maxNumberResourcesPerBand | Not checked |  |  |
| totalNumberTxPortsPerBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| modes | Not checked |  |  |
| maxNumberCSI-RS-PerResourceSet | Not checked |  |  |
| } |  |  |  |
| multiPanel SEQUENCE { |  |  |  |
| supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource { | 1 entry |  |  |
| SupportedCSI-RS-Resource[1] SEQUENCE { |  | entry 1 |  |
| maxNumberTxPortsPerResource | Not checked |  |  |
| maxNumberResourcesPerBand | Not checked |  |  |
| totalNumberTxPortsPerBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| modes | Not checked |  |  |
| nrofPanels | Not checked |  |  |
| maxNumberCSI-RS-PerResourceSet | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| type2 SEQUENCE { |  |  |  |
| supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource { | 1 entry |  |  |
| SupportedCSI-RS-Resource[1] SEQUENCE { |  | entry 1 |  |
| maxNumberTxPortsPerResource | Not checked |  |  |
| maxNumberResourcesPerBand | Not checked |  |  |
| totalNumberTxPortsPerBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| parameterLx | Not checked |  |  |
| amplitudeScalingType | Not checked |  |  |
| amplitudeSubsetRestriction | Not checked |  |  |
| } |  |  |  |
| type2-PortSelection SEQUENCE { |  |  |  |
| supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource { | 1 entry |  |  |
| SupportedCSI-RS-Resource[1] SEQUENCE { |  | entry 1 |  |
| maxNumberTxPortsPerResource | Not checked |  |  |
| maxNumberResourcesPerBand | Not checked |  |  |
| totalNumberTxPortsPerBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| parameterLx | Not checked |  |  |
| amplitudeScalingType | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	FeatureSetCombination

Table 4.6.4-7: FeatureSetCombination

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetCombination ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand { | 1 entry |  |  |
| FeatureSetsPerBand[1] SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet { | 1 entry | entry 1 |  |
| FeatureSet[1] CHOICE { |  | entry 1 |  |
| nr SEQUENCE { |  |  |  |
| downlinkSetNR | Not checked |  |  |
| uplinkSetNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	FeatureSetCombinationId

Table 4.6.4-8: FeatureSetCombinationId

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetCombinationId | Not checked |  |  |

#### –	FeatureSetDownlink

Table 4.6.4-9: FeatureSetDownlink

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetDownlink ::= SEQUENCE { |  |  |  |
| featureSetListPerDownlinkCC SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id { | 1 entry |  |  |
| FeatureSetDownlinkPerCC-Id[1] | Not checked | entry 1 |  |
| } |  |  |  |
| intraBandFreqSeparationDL | FreqSeparationClass |  |  |
| scalingFactor | Not checked |  |  |
| crossCarrierSchedulingDL-OtherSCS | Not checked |  |  |
| scellWithoutSSB | Not checked |  |  |
| csi-RS-MeasSCellWithoutSSB | Not checked |  |  |
| dummy1 | Not checked |  |  |
| type1-3-CSS | Not checked |  |  |
| pdcchMonitoringAnyOccasions | Not checked |  |  |
| dummy2 | Not checked |  |  |
| ue-SpecificUL-DL-Assignment | Not checked |  |  |
| searchSpaceSharingCA-DL | Not checked |  |  |
| timeDurationForQCL SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scsh-120kHz | Not checked |  |  |
| } |  |  |  |
| pdsch- ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| dummy3 | Not checked |  |  |
| dummy4 | Not checked |  |  |
| dummy5 | Not checked |  |  |
| dummy6 | Not checked |  |  |
| dummy7 | Not checked |  |  |
| } |  |  |  |

#### –	FeatureSetDownlinkId

Table 4.6.4-10: FeatureSetDownlinkId

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetDownlinkId | Not checked |  |  |

#### –	FeatureSetDownlinkPerCC

Table 4.6.4-11: FeatureSetDownlinkPerCC

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetDownlinkPerCC ::= SEQUENCE { |  |  |  |
| supportedSubcarrierSpacingDL | Not checked |  |  |
| supportedBandwidthDL | SupportedBandwidth |  |  |
| channelBW-90mhz | Not checked |  |  |
| maxNumberMIMO-LayersPDSCH | MIMO-LayersDL |  |  |
| supportedModulationOrderDL | ModulationOrder |  |  |
| } |  |  |  |

#### –	FeatureSetDownlinkPerCC-Id

Table 4.6.4-12: FeatureSetDownlinkPerCC-Id

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetDownlinkPerCC-Id | Not checked |  |  |

#### –	FeatureSetEUTRA-DownlinkId

Table 4.6.4-13: FeatureSetEUTRA-DownlinkId

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetEUTRA-DownlinkId | Not checked |  |  |

#### –	FeatureSetEUTRA-UplinkId

Table 4.6.4-14: FeatureSetEUTRA-UplinkId

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetEUTRA-UplinkId | Not checked |  |  |

#### –	FeatureSets

Table 4.6.4-15: FeatureSets

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSets ::= SEQUENCE { |  |  |  |
| featureSetsDownlink SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink { | 1 entry |  |  |
| FeatureSetDownlink[1] | FeatureSetDownlink | entry 1 |  |
| } |  |  |  |
| featureSetsDownlinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC { | 1 entry |  |  |
| FeatureSetDownlinkPerCC[1] | FeatureSetDownlinkPerCC | entry 1 |  |
| } |  |  |  |
| featureSetsUplink SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink { | 1 entry |  |  |
| FeatureSetUplink[1] | FeatureSetUplink | entry 1 |  |
| } |  |  |  |
| featureSetsUplinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC { | 1 entry |  |  |
| FeatureSetUplinkPerCC[1] | FeatureSetUplinkPerCC | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	FeatureSetUplink

Table 4.6.4-16: FeatureSetUplink

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetUplink ::= SEQUENCE { |  |  |  |
| featureSetListPerUplinkCC SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id { | 1 entry |  |  |
| FeatureSetUplinkPerCC-Id[1] | Not checked | entry 1 |  |
| } |  |  |  |
| scalingFactor | Not checked |  |  |
| crossCarrierSchedulingUL-OtherSCS | Not checked |  |  |
| intraBandFreqSeparationUL | FreqSeparationClass |  |  |
| searchSpaceSharingCA-UL | Not checked |  |  |
| dummy1 | Not checked |  |  |
| supportedSRS-Resources SEQUENCE { |  |  |  |
| maxNumberAperiodicSRS-PerBWP | Not Checked |  |  |
| maxNumberAperiodicSRS-PerBWP-PerSlot | Not Checked |  |  |
| maxNumberPeriodicSRS-PerBWP | Not Checked |  |  |
| maxNumberPeriodicSRS-PerBWP-PerSlot | Not Checked |  |  |
| maxNumberSemiPersistentSRS-PerBWP | Not Checked |  |  |
| maxNumberSemiPersistentSRS-PerBWP-PerSlot | Not Checked |  |  |
| maxNumberSRS-Ports-PerResource | Not Checked |  |  |
| } |  |  |  |
| twoPUCCH-Group | Not checked |  |  |
| dynamicSwitchSUL | Not checked |  |  |
| pusch- ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| dummy2 | Not checked |  |  |
| } |  |  |  |

#### –	FeatureSetUplinkId

Table 4.6.4-17: FeatureSetUplinkId

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetUplinkId | Not checked |  |  |

#### –	FeatureSetUplinkPerCC

Table 4.6.4-18: FeatureSetUplinkPerCC

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetUplinkPerCC ::= SEQUENCE { |  |  |  |
| supportedSubcarrierSpacingUL | Not checked |  |  |
| supportedBandwidthUL | SupportedBandwidth |  |  |
| channelBW-90mhz | Not checked |  |  |
| mimo-CB-PUSCH SEQUENCE { |  |  |  |
| maxNumberMIMO-LayersCB-PUSCH | MIMO-LayersUL |  |  |
| maxNumberSRS-ResourcePerSet | Not checke |  |  |
| } |  |  |  |
| maxNumberMIMO-LayersNonCB-PUSCH | MIMO-LayersUL |  |  |
| supportedModulationOrderUL | ModulationOrder |  |  |
| } |  |  |  |

#### –	FeatureSetUplinkPerCC-Id

Table 4.6.4-19: FeatureSetUplinkPerCC-Id

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureSetUplinkPerCC-Id | Not checked |  |  |

#### –	FreqBandIndicatorEUTRA

Table 4.6.4-20: FreqBandIndicatorEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqBandIndicatorEUTRA | EUTRA Operating band under test |  |  |

#### –	FreqBandList

Table 4.6.4-21: FreqBandList

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqBandList::= SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation { | Number of entries depends on the conditions |  |  |
| FreqBandInformation[1] CHOICE { |  | entry 1 | EN-DC |
| bandInformationEUTRA SEQUENCE { |  |  |  |
| bandEUTRA | FreqBandIndicatorEUTRA |  |  |
| ca-BandwidthClassDL-EUTRA | Not checked |  |  |
| ca-BandwidthClassUL-EUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[2] CHOICE { |  | entry 2 |  |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[3] CHOICE { |  | entry 3 | EN-DC AND CA-InterBand |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR with condition CA-InterBand |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[1] CHOICE { |  | entry 1 | NR |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[2] CHOICE { |  | entry 2 | NR AND CA-InterBand |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR with condition CA-InterBand |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| CA-InterBand | Used in NR CA Inter-band test cases |
| NR | NG-RAN NR Radio Access |

#### –	FreqSeparationClass

Table 4.6.4-22: FreqSeparationClass

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqSeparationClass | Not checked |  |  |

#### –	IMS-Parameters

Table 4.6.4-23: IMS-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IMS-Parameters ::= SEQUENCE { |  |  |  |
| ims-ParametersCommon SEQUENCE { |  |  |  |
| voiceOverEUTRA-5GC | Not Checked |  |  |
| } |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	InterRAT-Parameters

Table 4.6.4-24: InterRAT-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| InterRAT-Parameters ::= SEQUENCE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| supportedBandListEUTRA SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA { | 1 entry |  |  |
| FreqBandIndicatorEUTRA[1] | FreqBandIndicatorEUTRA | entry 1 |  |
| } |  |  |  |
| eutra-ParametersCommon SEQUENCE { |  |  |  |
| mfbi-EUTRA | Not Checked |  |  |
| modifiedMPR-BehaviorEUTRA | Not Checked |  |  |
| multiNS-Pmax-EUTRA | Not Checked |  |  |
| rs-SINR-MeasEUTRA | Not Checked |  |  |
| ne-DC | Not Checked |  |  |
| nr-HO-ToEN-DC-r16 | Not Checked |  | >=Rel16 |
| } |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MAC-Parameters

Table 4.6.4-25: MAC-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-Parameters ::= SEQUENCE { |  |  |  |
| mac-ParametersCommon SEQUENCE { |  |  |  |
| lcp-Restriction | Not checked |  |  |
| dummy | Not checked |  |  |
| lch-ToSCellRestriction | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Not checked |  |  |
| logicalChannelSR-DelayTimer | Not checked |  |  |
| longDRX-Cycle | Not checked |  |  |
| shortDRX-Cycle | Not checked |  |  |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MeasAndMobParameters

Table 4.6.4-26: MeasAndMobParameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasAndMobParameters::= SEQUENCE { |  |  |  |
| measAndMobParametersCommon SEQUENCE { |  |  |  |
| supportedGapPattern | Not checked |  |  |
| ssb-RLM | Not checked |  |  |
| ssb-AndCSI-RS-RLM | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Not checked |  |  |
| eventA-MeasAndReport | Not checked |  |  |
| } |  |  |  |
| MeasAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MeasAndMobParametersMRDC

Table 4.6.4-27: MeasAndMobParametersMRDC

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasAndMobParametersMRDC::= SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-Common SEQUENCE { |  |  |  |
| independentGapConfig | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Not checked |  |  |
| sftd-MeasNR-Cell | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MIMO-Layers

Table 4.6.4-28: MIMO-Layers

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MIMO-LayersDL | Not checked |  |  |
| MIMO-LayersUL | Not checked |  |  |

#### –	MIMO-ParametersPerBand

Table 4.6.4-29: MIMO-ParametersPerBand

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MIMO-ParametersPerBand ::= SEQUENCE { |  |  |  |
| tci-StatePDSCH SEQUENCE { |  |  |  |
| maxNumberConfiguredTCIstatesPerCC | Not checked |  |  |
| maxNumberActiveTCI-PerBWP | Not checked |  |  |
| } |  |  |  |
| additionalActiveTCI-StatePDCCH | Not checked |  |  |
| pusch-TransCoherence | Not checked |  |  |
| beamCorrespondenceWithoutUL-BeamSweeping | Not checked |  |  |
| periodicBeamReport | Not checked |  |  |
| aperiodicBeamReport | Not checked |  |  |
| sp-BeamReportPUCCH | Not checked |  |  |
| sp-BeamReportPUSCH | Not checked |  |  |
| dummy1 | Not checked |  |  |
| maxNumberRxBeam | Not checked |  |  |
| maxNumberRxTxBeamSwitchDL SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| scs-240kHz | Not checked |  |  |
| } |  |  |  |
| maxNumberNonGroupBeamReporting | Not checked |  |  |
| groupBeamReporting | Not checked |  |  |
| uplinkBeamManagement SEQUENCE { |  |  |  |
| maxNumberSRS-ResourcePerSet | Not checked |  |  |
| maxNumberSRS-ResourceSet | Not checked |  |  |
| } |  |  |  |
| maxNumberCSI-RS-BFD | Not checked |  |  |
| maxNumbeSSB-BFD | Not checked |  |  |
| maxNumberCSI-RS-SSB-CBD | Not checked |  |  |
| dummy2 | Not checked |  |  |
| twoPortsPTRS-UL | Not checked |  |  |
| dummy5 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| beamReportTiming SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| ptrs-DensityRecommendationSetDL SEQUENCE { |  |  |  |
| scs-15kHz |  |  |  |
| frequencyDensity1 | Not checked |  |  |
| frequencyDensity2 | Not checked |  |  |
| timeDensity1 | Not checked |  |  |
| timeDensity2 | Not checked |  |  |
| timeDensity3 | Not checked |  |  |
| } |  |  |  |
| scs-30kHz |  |  |  |
| frequencyDensity1 | Not checked |  |  |
| frequencyDensity2 | Not checked |  |  |
| timeDensity1 | Not checked |  |  |
| timeDensity2 | Not checked |  |  |
| timeDensity3 | Not checked |  |  |
| } |  |  |  |
| scs-60kHz |  |  |  |
| frequencyDensity1 | Not checked |  |  |
| frequencyDensity2 | Not checked |  |  |
| timeDensity1 | Not checked |  |  |
| timeDensity2 | Not checked |  |  |
| timeDensity3 | Not checked |  |  |
| } |  |  |  |
| scs-120kHz |  |  |  |
| frequencyDensity1 | Not checked |  |  |
| frequencyDensity2 | Not checked |  |  |
| timeDensity1 | Not checked |  |  |
| timeDensity2 | Not checked |  |  |
| timeDensity3 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ptrs-DensityRecommendationSetUL SEQUENCE { |  |  |  |
| scs-15kHz SEQUENCE { |  |  |  |
| frequencyDensity1 | Not checked |  |  |
| frequencyDensity2 | Not checked |  |  |
| timeDensity1 | Not checked |  |  |
| timeDensity2 | Not checked |  |  |
| timeDensity3 | Not checked |  |  |
| sampleDensity1 | Not checked |  |  |
| sampleDensity2 | Not checked |  |  |
| sampleDensity3 | Not checked |  |  |
| sampleDensity4 | Not checked |  |  |
| sampleDensity5 | Not checked |  |  |
| } |  |  |  |
| scs-30kHz SEQUENCE { |  |  |  |
| frequencyDensity1 | Not checked |  |  |
| frequencyDensity2 | Not checked |  |  |
| timeDensity1 | Not checked |  |  |
| timeDensity2 | Not checked |  |  |
| timeDensity3 | Not checked |  |  |
| sampleDensity1 | Not checked |  |  |
| sampleDensity2 | Not checked |  |  |
| sampleDensity3 | Not checked |  |  |
| sampleDensity4 | Not checked |  |  |
| sampleDensity5 | Not checked |  |  |
| scs-60kHz SEQUENCE { |  |  |  |
| frequencyDensity1 | Not checked |  |  |
| frequencyDensity2 | Not checked |  |  |
| timeDensity1 | Not checked |  |  |
| timeDensity2 | Not checked |  |  |
| timeDensity3 | Not checked |  |  |
| sampleDensity1 | Not checked |  |  |
| sampleDensity2 | Not checked |  |  |
| sampleDensity3 | Not checked |  |  |
| sampleDensity4 | Not checked |  |  |
| sampleDensity5 | Not checked |  |  |
| scs-120kHz SEQUENCE { |  |  |  |
| frequencyDensity1 | Not checked |  |  |
| frequencyDensity2 | Not checked |  |  |
| timeDensity1 | Not checked |  |  |
| timeDensity2 | Not checked |  |  |
| timeDensity3 | Not checked |  |  |
| sampleDensity1 | Not checked |  |  |
| sampleDensity2 | Not checked |  |  |
| sampleDensity3 | Not checked |  |  |
| sampleDensity4 | Not checked |  |  |
| sampleDensity5 | Not checked |  |  |
| } |  |  |  |
| dummy4 | Not checked |  |  |
| aperiodicTRS | Not checked |  |  |
| } |  |  |  |

#### –	ModulationOrder

Table 4.6.4-30: ModulationOrder

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ModulationOrder | Not checked |  |  |

#### –	MRDC-Parameters

Table 4.6.4-31: MRDC-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MRDC-Parameters ::= SEQUENCE { |  |  |  |
| singleUL-Transmission | Not checked |  |  |
| dynamicPowerSharingENDC | Not checked |  |  |
| tdm-Pattern | Not checked |  |  |
| ul-SharingEUTRA-NR | Not checked |  |  |
| ul-SwitchingTimeEUTRA-NR | Not checked |  |  |
| simultaneousRxTxInterBandENDC | Not checked |  |  |
| asyncIntraBandENDC | Not checked |  |  |
| } |  |  |  |

#### –	NRDC-Parameters

Table 4.6.4-31A: NRDC-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NRDC-Parameters ::= SEQUENCE { |  |  |  |
| measAndMobParametersNRDC | Not checked |  |  |
| generalParametersNRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Not checked |  |  |
| splitDRB-withUL-Both-MCG-SCG | Not checked |  |  |
| srb3 | Not checked |  |  |
| v2x-EUTRA | Not checked |  |  |
| } |  |  |  |
| fdd-Add-UE-NRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Not checked |  |  |
| sftd-MeasNR-Cell | Not checked |  |  |
| } |  |  |  |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Not checked |  |  |
| splitDRB-withUL-Both-MCG-SCG | Not checked |  |  |
| srb3 | Not checked |  |  |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Not checked |  |  |
| sftd-MeasNR-Cell | Not checked |  |  |
| } |  |  |  |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Not checked |  |  |
| splitDRB-withUL-Both-MCG-SCG | Not checked |  |  |
| srb3 | Not checked |  |  |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-NRDC-Capabilities SEQUENCE { | UE-MRDC-CapabilityAddFRX-Mode |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

#### –	PDCP-Parameters

Table 4.6.4-32: PDCP-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Parameters ::= SEQUENCE { |  |  |  |
| supportedROHC-Profiles SEQUENCE { |  |  |  |
| profile0x0000 | Not checked |  |  |
| profile0x0001 | Not checked |  |  |
| profile0x0002 | Not checked |  |  |
| profile0x0003 | Not checked |  |  |
| profile0x0004 | Not checked |  |  |
| profile0x0006 | Not checked |  |  |
| profile0x0101 | Not checked |  |  |
| profile0x0102 | Not checked |  |  |
| profile0x0103 | Not checked |  |  |
| profile0x0104 | Not checked |  |  |
| } |  |  |  |
| maxNumberROHC-ContextSessions | Not checked |  |  |
| uplinkOnlyROHC-Profiles | Not checked |  |  |
| continueROHC-Context | Not checked |  |  |
| outOfOrderDelivery | Not checked |  |  |
| shortSN | Not checked |  |  |
| pdcp-DuplicationSRB | Not checked |  |  |
| pdcp-DuplicationMCG-OrSCG-DRB | Not checked |  |  |
| } |  |  |  |

#### –	PDCP-ParametersMRDC

Table 4.6.4-33: PDCP-ParametersMRDC

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-ParametersMRDC ::= SEQUENCE { |  |  |  |
| pdcp-DuplicationSplitSRB | Not checked |  |  |
| pdcp-DuplicationSplitDRB | Not checked |  |  |
| } |  |  |  |

#### –	Phy-Parameters

Table 4.6.4-34: Phy-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Phy-Parameters ::= SEQUENCE { |  |  |  |
| phy-ParametersCommon SEQUENCE { |  |  |  |
| csi-RS-CFRA-ForHO | Not checked |  |  |
| dynamicPRB-BundlingDL | Not checked |  |  |
| sp-CSI-ReportPUCCH | Not checked |  |  |
| sp-CSI-ReportPUSCH | Not checked |  |  |
| nzp-CSI-RS-IntefMgmt | Not checked |  |  |
| type2-SP-CSI-Feedback-LongPUCCH | Not checked |  |  |
| precoderGranularityCORESET | Not checked |  |  |
| dynamicHARQ-ACK-Codebook | Not checked |  |  |
| semiStaticHARQ-ACK-Codebook | Not checked |  |  |
| spatialBundlingHARQ-ACK | Not checked |  |  |
| dynamicBetaOffsetInd-HARQ-ACK-CSI | Not checked |  |  |
| pucch-Repetition-F1-3-4 | Not checked |  |  |
| ra-Type0-PUSCH | Not checked |  |  |
| dynamicSwitchRA-Type0-1-PDSCH | Not checked |  |  |
| dynamicSwitchRA-Type0-1-PUSCH | Not checked |  |  |
| pdsch-MappingTypeA | Not checked |  |  |
| pdsch-MappingTypeB | Not checked |  |  |
| interleavingVRB-ToPRB-PDSCH | Not checked |  |  |
| interSlotFreqHopping-PUSCH | Not checked |  |  |
| type1-PUSCH-RepetitionMultiSlots | Not checked |  |  |
| type2-PUSCH-RepetitionMultiSlots | Not checked |  |  |
| pusch-RepetitionMultiSlots | Not checked |  |  |
| pdsch-RepetitionMultiSlots | Not checked |  |  |
| downlinkSPS | Not checked |  |  |
| configuredUL-GrantType1 | Not checked |  |  |
| configuredUL-GrantType2 | Not checked |  |  |
| pre-EmptIndication-DL | Not checked |  |  |
| cbg-TransIndication-DL | Not checked |  |  |
| cbg-TransIndication-UL | Not checked |  |  |
| cbg-FlushIndication-DL | Not checked |  |  |
| dynamicHARQ-ACK-CodeB-CBG-Retx-DL | Not checked |  |  |
| rateMatchingResrcSetSemi-Static | Not checked |  |  |
| rateMatchingResrcSetDynamic | Not checked |  |  |
| bwp-SwitchingDelay | Not checked |  |  |
| } |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH- MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFR1 SEQUENCE { |  |  |  |
| pdcchMonitoringSingleOccasion | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| pdsch-256QAM-FR1 | Not checked |  |  |
| pdsch-RE-MappingFR1- PerSymbol | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFR2 SEQUENCE { |  |  |  |
| dummy | Not checked |  |  |
| pdsch-RE-MappingFR2- PerSymbol | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	Phy-ParametersMRDC

Table 4.6.4-35: Phy-ParametersMRDC

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Phy-ParametersMRDC ::= SEQUENCE { |  |  |  |
| naics-Capability-List SEQUENCE (SIZE (1..maxNrofNAICS-Entries)) OF NAICS-Capability-Entry { | 1 entry |  |  |
| NAICS-Capability-Entry[1] SEQUENCE { |  | entry 1 |  |
| numberOfNAICS-CapableCC | Not checked |  |  |
| numberOfAggregatedPRB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	ProcessingParameters

Table 4.6.4-36: ProcessingParameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProcessingParameters ::= SEQUENCE { |  |  |  |
| fallback | Not checked |  |  |
| differentTB-PerSlot SEQUENCE { |  |  |  |
| upto1 | Not checked |  |  |
| upto2 | Not checked |  |  |
| upto4 | Not checked |  |  |
| upto7 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RAT-Type

Table 4.6.4-37: RAT-Type

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RAT-Type | nr |  |  |
|  | eutra-nr |  | EN-DC |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |

#### –	RF-Parameters

Table 4.6.4-38: RF-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RF-Parameters ::= SEQUENCE { |  |  |  |
| supportedBandListNR SEQUENCE (SIZE (1..maxBands)) OF BandNR { | At least 1 entry |  |  |
| BandNR[1] SEQUENCE { |  | entry 1 |  |
| bandNR | FreqBandIndicatorNR |  |  |
| modifiedMPR-Behaviour | Not checked |  |  |
| mimo-ParametersPerBand | Not checked |  |  |
| extendedCP | Not checked |  |  |
| multipleTCI | Not checked |  |  |
| bwp-WithoutRestriction | Not checked |  |  |
| bwp-SameNumerology | Not checked |  |  |
| bwp-DiffNumerology | Not checked |  |  |
| crossCarrierScheduling-SameSCS | Not checked |  |  |
| pdsch-256QAM-FR2 | Not checked |  |  |
| pusch-256QAM | Not checked |  |  |
| ue-PowerClass | Not checked |  |  |
| rateMatchingLTE-CRS | Not checked |  |  |
| channelBWs-DL CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| channelBWs-UL CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| supportedBandCombinationList | Not checked |  |  |
| appliedFreqBandListFilter | Not present |  |  |
|  | FreqBandList |  | FILTER_REQUESTED |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FILTER_REQUESTED | This condition shall be set to true when UE is requested to filter the information via ‘capabilityRequestFilter’ IE in the NR5GC UECapabilityEnquiry message or via ‘requestedFreqBandsNR-MRDC’ IE in the EN-DC UECapabilityEnquiry message |

#### –	RF-ParametersMRDC

Table 4.6.4-39: RF-ParametersMRDC

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RF-ParametersMRDC ::= SEQUENCE { |  |  |  |
| supportedBandCombinationList | BandCombinationList |  |  |
| appliedFreqBandListFilter | FreqBandList |  |  |
| } |  |  |  |

#### –	RLC-Parameters

Table 4.6.4-40: RLC-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Parameters ::= SEQUENCE { |  |  |  |
| am-WithShortSN | Not checked |  |  |
| um-WithShortSN | Not checked |  |  |
| um-WIthLongSN | Not checked |  |  |
| } |  |  |  |

#### –	SDAP-Parameters

Table 4.6.4-41: SDAP-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDAP-Parameters::= SEQUENCE { |  |  |  |
| as-ReflectiveQoS | Not checked |  |  |
| } |  |  |  |

#### –	SRS-SwitchingTimeNR

Table 4.6.4-42: SRS-SwitchingTimeNR

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-SwitchingTimeNR::= SEQUENCE { |  |  |  |
| switchingTimeDL | Not checked |  |  |
| switchingTimeUL | Not checked |  |  |
| } |  |  |  |

#### –	SRS-SwitchingTimeEUTRA

Table 4.6.4-43: SRS-SwitchingTimeEUTRA

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-SwitchingTimeEUTRA ::= SEQUENCE { |  |  |  |
| switchingTimeDL | Not checked |  |  |
| switchingTimeUL | Not checked |  |  |
| } |  |  |  |

#### –	SupportedBandwidth

Table 4.6.4-44: SupportedBandwidth

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SupportedBandwidth ::= CHOICE { |  |  |  |
| fr1 | Not checked |  | FR1 |
| fr2 | Not checked |  | FR2 |
| } |  |  |  |

#### –	UE-CapabilityRAT-ContainerList

Table 4.6.4-45: UE-CapabilityRAT-ContainerList

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRAT-ContainerList ::= SEQUENCE (SIZE (0.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container { | 1 entry |  |  |
| UE-CapabilityRAT-Container[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | RAT-Type |  |  |
| ue-CapabilityRAT-Container | UE-NR-Capability |  |  |
| ue-CapabilityRAT-Container | UE-MRDC-Capability |  | EN-DC |
| } |  |  |  |
| } |  |  |  |

#### –	UE-CapabilityRAT-RequestList

Table 4.6.4-46: UE-CapabilityRAT-RequestList

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRAT-RequestList ::= SEQUENCE (SIZE (0.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request { | 1 entry |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | RAT-Type |  |  |
| capabilityRequestFilter | UE-CapabilityRequestFilterNR | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) |  |
| } |  |  |  |
| } |  |  |  |

#### –	UE-CapabilityRequestFilterCommon

Table 4.6.4-46A: UE-CapabilityRequestFilterCommon

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRequestFilterCommon ::= SEQUENCE { |  |  |  |
| mrdc-Request SEQUENCE { |  |  |  |
| omitEN-DC | Not checked |  |  |
| includeNR-DC | Not checked |  |  |
| includeNE-DC | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	UE-CapabilityRequestFilterNR

Table 4.6.4-47: UE-CapabilityRequestFilterNR

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRequestFilterNR ::= SEQUENCE { |  |  |  |
| frequencyBandListFilter | FreqBandList |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| srs-SwitchingTimeRequest | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| srs-SwitchingTimeRequest | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	UE-MRDC-Capability

Table 4.6.4-48: UE-MRDC-Capability

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-MRDC-Capability ::= SEQUENCE { |  |  |  |
| measAndMobParametersMRDC | Not checked |  |  |
| phy-ParametersMRDC-v1530 | Not checked |  |  |
| rf-ParametersMRDC | RF-ParametersMRDC |  |  |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Not checked |  |  |
| splitDRB-withUL-Both-MCG-SCG | Not checked |  |  |
| srb3 | Not checked |  |  |
| v2x-EUTRA | Not checked |  |  |
| } |  |  |  |
| fdd-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Not checked |  |  |
| sftd-MeasNR-Cell | Not checked |  |  |
| } |  |  |  |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Not checked |  |  |
| splitDRB-withUL-Both-MCG-SCG | Not checked |  |  |
| srb3 | Not checked |  |  |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Not checked |  |  |
| sftd-MeasNR-Cell | Not checked |  |  |
| } |  |  |  |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Not checked |  |  |
| splitDRB-withUL-Both-MCG-SCG | Not checked |  |  |
| srb3 | Not checked |  |  |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-MRDC-Capabilities |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombinations | Not checked |  |  |
| pdcp-ParametersMRDC-v1530 | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| UE-MRDC-Capability-v1560 SEQUENCE { |  |  |  |
| receivedFilters | Not checked |  |  |
| measAndMobParametersMRDC-v1560 SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff-v1560 SEQUENCE { |  |  |  |
| sftd-MeasPSCell-NEDC | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fdd-Add-UE-MRDC-Capabilities-v1560 SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff-v1560 SEQUENCE { |  |  |  |
| sftd-MeasPSCell-NEDC | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-MRDC-Capabilities-v1560 SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff-v1560 SEQUENCE { |  |  |  |
| sftd-MeasPSCell-NEDC | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	UE-NR-Capability

Table 4.6.4-49: UE-NR-Capability

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-NR-Capability ::= SEQUENCE { |  |  |  |
| accessStratumRelease | AccessStratumRelease |  |  |
| pdcp-Parameters | Not checked |  |  |
| rlc-Parameters | Not checked |  |  |
| mac-Parameters | Not checked |  |  |
| phy-Parameters | Not checked |  |  |
| rf-Parameters | RF-Parameters |  |  |
| measAndMobParameters | Not checked |  |  |
| fdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Not checked |  |  |
| logicalChannelSR-DelayTimer | Not checked |  |  |
| longDRX-Cycle | Not checked |  |  |
| shortDRX-Cycle | Not checked |  |  |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Not checked |  |  |
| eventA-MeasAndReport | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Not checked |  |  |
| logicalChannelSR-DelayTimer | Not checked |  |  |
| longDRX-Cycle | Not checked |  |  |
| shortDRX-Cycle | Not checked |  |  |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Not checked |  |  |
| eventA-MeasAndReport | Not checked |  |  |
| } |  |  |  |
| fr1-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { | Not checked |  |  |
| dynamicSFI | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSets | Not checked |  |  |
| featureSetCombinations | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| fdd-Add-UE-NR-Capabilities-1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA Not Checked |  |  |  |
| } |  |  |  |
| } |  |  |  |
| dummy | Not checked |  |  |
| interRAT-Parameters | Not checked |  |  |
| inactiveState | Not checked |  |  |
| delayBudgetReporting | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdap-Parameters | Not checked |  |  |
| overheatingInd | Not checked |  |  |
| ims-Parameters | Not checked |  |  |
| fr1-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH- MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |
| reducedCP-Latency | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nrdc-Parameters | Not checked |  |  |
| receivedFilters | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 4.6.5	Other information elements

#### –	EUTRA-AllowedMeasBandwidth

Table 4.6.5-1: EUTRA-AllowedMeasBandwidth

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-AllowedMeasBandwidth | Set according to TS 36.508 [2] Table 4.4.3.4-1 for E-UTRA cell | row ‘measurement Bandwidth’ |  |

#### –	EUTRA-MBSFN-SubframeConfigList

Table 4.6.5-2: EUTRA-MBSFN-SubframeConfigList

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-MBSFN-SubframeConfigList ::= SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF EUTRA-MBSFN-SubframeConfig { | 1 entry |  |  |
| MBSFN-SubframeConfig[1] SEQUENCE { |  | entry 1 |  |
| radioframeAllocationPeriod | FFS |  |  |
| radioframeAllocationOffset | FFS |  |  |
| subframeAllocation1 CHOICE { |  |  |  |
| oneFrame | FFS |  |  |
| fourFrames | FFS |  |  |
| } |  |  |  |
| subframeAllocation2 CHOICE { |  |  |  |
| oneFrame | FFS |  |  |
| fourFrames | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	EUTRA-MultiBandInfoList

Table 4.6.5-3: EUTRA-MultiBandInfoList

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-MultiBandInfoList ::= SEQUENCE (SIZE (1..maxMultiBands)) OF EUTRA-MultiBandInfo { | 1 entry |  |  |
| EUTRA-MultiBandInfo[1] SEQUENCE { |  | entry 1 |  |
| eutra-FreqBandIndicator | FreqBandIndicatorEUTRA |  |  |
| eutra-NS-PmaxList | EUTRA-NS-PmaxList |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	EUTRA-NS-PmaxList

Table 4.6.5-4: EUTRA-NS-PmaxList

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-NS-PmaxList ::= SEQUENCE (SIZE (1..maxEUTRA-NS-Pmax)) OF EUTRA-NS-PmaxValue { | 1 entry |  |  |
| EUTRA-NS-PmaxValue[1] SEQUENCE { |  | entry 1 |  |
| additionalPmax | FFS |  |  |
| additionalSpectrumEmission | FFS |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	EUTRA-PhysCellId

Table 4.6.5-5: EUTRA-PhysCellId

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-PhysCellId | Set according to TS 36.508 [2] Table 4.4.2-1A for E-UTRA cell |  |  |

#### –	EUTRA-PhysCellIdRange

Table 4.6.5-6: EUTRA-PhysCellIdRange

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-PhysCellIdRange ::= SEQUENCE { |  |  |  |
| start | EUTRA-PhysCellId |  |  |
| Range | FFS |  |  |
| } |  |  |  |

#### –	EUTRA-PresenceAntennaPort1

Table 4.6.5-7: EUTRA-PresenceAntennaPort1

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-PresenceAntennaPort1 | FFS |  |  |

#### –	EUTRA-Q-OffsetRange

Table 4.6.5-8: EUTRA-Q-OffsetRange

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-Q-OffsetRange | FFS |  |  |

Table 4.6.5-9: Void

Table 4.6.5-10: Void

#### –	OtherConfig

Table 4.6.5-11: OtherConfig

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OtherConfig ::=SEQUENCE { |  |  |  |
| delayBudgetReportingConfig CHOICE{ |  |  |  |
| release | FFS |  |  |
| setup SEQUENCE { |  |  |  |
| delayBudgetReportingProhibitTimer | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRC-TransactionIdentifier

Table 4.6.5-12: RRC-TransactionIdentifier

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRC-TransactionIdentifier | 0 |  |  |

## 4.7	Default 5GC NAS message and information elements contents

### 4.7.0	General

#### 4.7.0.1	Interpretation of IE presence and values

For Uplink NAS messages, the following terms and their meanings shall be used to determine how to test specific IEs:

-	"Not present": test cases fail if IE is present.

-	"Present but contents not checked": test cases fail if IE is not present. No requirements regarding contents of the IE.

-	"If present: contents not checked": IE may or may not be present. No requirements regarding contents of the IE.

-	"If present: <specific values>": IE may or may not be present. If present, its contents shall be as specified.

-	"<specific values>": test cases fail if IE is not present. Its contents shall be as specified.

-	"Present if <condition>: contents not checked: test cases fail if condition is fulfilled and IE is not present. Contents of IE are not checked, even if present.

-	"Present if <condition>: <specific values>": test cases fail if condition is fulfilled and IE is not present. When IE shall be present, its contents shall be as specified.

#### 4.7.0.2	Security protected 5GS NAS messages

In subclause 4.7.1, all 5GS NAS messages are described in the plain 5GS NAS message format.

When a 5GS NAS message is security protected, the message shall be contained by SECURITY PROTECTED 5GS NAS MESSAGE unless contained by another NAS message.

The default contents of SECURITY PROTECTED 5GS NAS MESSAGE message is defined in table 4.7.1-28.

### 4.7.1	Contents of 5GMM messages

#### –	Authentication request

Table 4.7.1-1: AUTHENTICATION REQUEST

| Derivation Path: 24.501 clause 8.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication request message identity |  | ‘0101 0110’B |  |  |
| ngKSI |  |  |  |  |
| NAS key set identifier |  | An arbitrarily selected value between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. |  |  |
| TSC |  | '0'B | native security context (for KSIAMF) |  |
| Spare half octet |  | '0000'B |  |  |
| ABBA |  | ‘0000 0000 0000 0000’B |  |  |
| Authentication parameter RAND (5G authentication challenge) |  | Not Present |  | EAP-AKA |
|  |  | An arbitrarily selected 128 bits value |  | 5G-AKA |
| Authentication parameter AUTN (5G authentication challenge) |  | Not Present |  | EAP-AKA |
|  |  | 128 bits value generated according to TS 24.501 [28] subclause 9.11.3.15 |  | 5G-AKA |
| EAP message |  | Not Present |  | 5G-AKA |
| EAP message |  | EAP-request/AKA'-challenge | See Table 4.7.3.2-01 | EAP-AKA |

| Condition | Explanation |
| --- | --- |
| EAP_AKA | EAP based primary authentication and key agreement procedure |
| 5G-AKA | 5G AKA based primary authentication and key agreement procedure |

NOTE:	Within a test execution this message is sent without integrity protection before NAS security mode control procedure has been successfully completed; and sent integrity protected and ciphered within SECURITY PROTECTED 5GS NAS MESSAGE message after 5GS NAS security mode control procedure has been successfully completed. SS does not maintain information for 5GS NAS security mode control procedure after a TC is completed.

#### –	Authentication response

Table 4.7.1-2: AUTHENTICATION RESPONSE

| Derivation Path: 24.501 clause 8.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication response message identity |  | ‘0101 0111’B |  |  |
| Authentication response parameter |  | 16 octets RES* value calculated according to TS 24.501 [28] subclause 9.11.3.17 |  | 5G-AKA |
|  |  | Not Present |  | EAP-AKA |
| EAP message |  | EAP-response/AKA'-challenge | See Table 4.7.3.2-02 | EAP-AKA |

| Condition | Explanation |
| --- | --- |
| EAP-AKA | EAP based primary authentication and key agreement procedure |
| 5G-AKA | 5G AKA based primary authentication and key agreement procedure |

NOTE:	When sent in response to an AUTHENTICATION REQUEST message which is not integrity protected and not ciphered, the AUTHENTICATION RESPONSE message is sent integrity protected when a valid security context exists and without integrity protection and ciphering otherwise.

#### –	Authentication result

Table 4.7.1-3: AUTHENTICATION RESULT

| Derivation Path: 24.501 clause 8.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication result message identity |  | ‘0101 1010’B |  |  |
| ngKSI |  | The same value as the last AUTHENTICATION REQUEST message |  |  |
| Spare half octet |  | '0000'B |  |  |
| EAP message |  | EAP-Success | See Table 4.7.3.2-03 |  |
| ABBA |  | ‘0000 0000 0000 0000’B |  |  |

NOTE:	The security protection of this message is the same as the previous AUTHENTICATION REQUEST message.

#### –	Authentication failure

Table 4.7.1-4: AUTHENTICATION FAILURE

| Derivation Path: 24.501 clause 8.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication failure message identity |  | ‘0101 1001’B |  |  |
| 5GMM cause |  | Present but contents not checked |  |  |
| Authentication failure parameter |  | If present: contents not checked |  |  |

NOTE:	The security protection of this message is the same as the previous AUTHENTICATION REQUEST message.

#### –	Authentication reject

Table 4.7.1-5: AUTHENTICATION REJECT

| Derivation Path: 24.501 clause 8.2.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication reject message identity |  | ‘0101 1000’B |  |  |
| EAP message |  | Not present |  |  |
| EAP message |  | EAP-Response/AKA-Authentication-Reject | See Table 4.7.3.2-04 | EAP-AKA |

| Condition | Explanation |
| --- | --- |
| EAP-AKA | EAP based primary authentication and key agreement procedure |

NOTE:	This message is sent without integrity protection.

#### –	Registration request

Table 4.7.1-6: REGISTRATION REQUEST

| Derivation Path: 24.501 clause 8.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration request message identity |  | ‘0100 0001’B |  |  |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
|  |  | ‘010’B |  | MOBILITY |
|  |  | ‘011’B |  | PERIODIC |
|  |  | ‘100’B |  | EMERGENCY |
| FOR |  | Present but contents not checked |  |  |
| FOR |  | '1'B | Follow-on request pending | EMERGENCY |
| ngKSI |  | Present but contents not checked |  |  |
| 5GS mobile identity |  | Present but contents not checked |  |  |
| Non-current native NAS key set identifier |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| 5GMM capability |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE security capability |  | If present: contents not checked |  |  |
| Requested NSSAI |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Last visited registered TAI |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| S1 UE network capability |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Uplink data status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| PDU session status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| MICO indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE status |  | If present: contents not checked |  |  |
| Additional GUTI |  | If present: contents not checked |  |  |
| Allowed PDU session status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE's usage setting |  | If present: contents not checked |  | NOT pc_IMS AND NON_CLEARTEXT_IE |
| UE's usage setting |  | Present but contents not checked |  | NON_CLEARTEXT_IE |
| Requested DRX parameters |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| EPS NAS message container |  | If present: contents not checked |  |  |
| LADN indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Payload container type |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Payload container |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Network slicing indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| 5GS update type |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| NAS message container |  | The complete, ciphered, REGISTRATION REQUEST message including all IEs. |  | CIPHERED_MESSAGE |

| Condition | Explanation |
| --- | --- |
| INITIAL | Initial registration |
| MOBILITY | Mobility registration updating |
| PERIODIC | Periodic registration updating |
| EMERGENCY | Emergency registration |
| NON_CLEARTEXT_IE | An information element that is not allowed to be sent in cleartext and shall only be included in the complete REGISTRATION REQUEST message in the NAS message container IE. |
| CIPHERED_MESSAGE | If any of the IEs marked with the condition NON_CLEARTEXT_IE is present, and the UE has a valid 5G NAS security context, this condition applies. |

NOTE:	This message is sent interity protected when a valid security context exists otherwise sent without integrity protection, including only cleartext IEs.

#### –	Registration accept

Table 4.7.1-7: REGISTRATION ACCEPT

| Derivation Path: 24.501 clause 8.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration accept message identity |  | ‘0100 0010’B |  |  |
| 5GS registration result |  |  |  |  |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| SMS allowed |  | ‘0’B | SMS over NAS not allowed |  |
| 5G-GUTI |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |
| Equivalent PLMNs |  | Not Present |  |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents |  | '0000 0111'B | 7 octets |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |
| MNC |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |
| TAC 1 |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |
| SST |  | ‘0000 0001’B | SST value 1 (eMBB) |  |
| Rejected NSSAI |  | Not Present |  |  |
| Configured NSSAI |  | Not Present |  |  |
| 5GS network feature support |  | ‘0000 1101 0000 0000’B | IMS voice over PS session supported over 3GPP access, Emergency services supported in NR connected to 5GCN and E-UTRA connected to 5GCN. All other features set to "not supported" including the 'Interworking without N26 interface not supported'. |  |
| PDU session status |  | The same value as the PDU session status IEof the most recently received REGISTRATION REQUEST message |  |  |
| PDU session reactivation result |  | Not Present |  |  |
| PDU session reactivation result error cause |  | Not Present |  |  |
| LADN information |  | Not Present |  |  |
| MICO indication |  | Not Present |  |  |
| Network slicing indication |  | Not Present |  |  |
| Service area list |  | Not Present |  |  |
| T3512 value |  |  |  | INITIAL |
| Timer value |  | '0 0000'B |  |  |
| Unit |  | '111'B | value indicates that the timer is deactivated |  |
| T3512 value |  | Not Present |  |  |
| Non-3GPP de-registration timer value |  | Not Present |  |  |
| T3502 value |  | Not Present |  |  |
| Emergency number list |  | Not Present |  |  |
| Extended emergency number list |  | Not Present |  |  |
| SOR Transparent container |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |
| NSSAI inclusion mode |  | Not Present |  |  |
| Operator-defined access category definitions |  | Not Present |  |  |
| Negotiated DRX parameters |  | Not Present |  |  |
| Non-3GPP NW policies |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| INITIAL | Initial registration |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Registration complete

Table 4.7.1-8: REGISTRATION COMPLETE

| Derivation Path: 24.501 clause 8.2.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration complete message identity |  | ‘0100 0011’B |  |  |
| SOR transparent container |  | If present: contents not checked |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Registration reject

Table 4.7.1-9: REGISTRATION REJECT

| Derivation Path: 24.501 clause 8.2.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration reject message identity |  | ‘0100 0100’B |  |  |
| 5GMM cause |  | Set according to specific message content |  |  |
| T3346 value |  | Not Present |  |  |
| T3502 value |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |

NOTE:	This message is sent without integrity protection before the secure exchange of NAS messages has been established and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after the secure exchange of NAS messages has been established.

#### –	UL NAS transport

Table 4.7.1-10: UL NAS TRANSPORT

| Derivation Path: 24.501 clause 8.2.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| UL NAS TRANSPORT message identity |  | ‘0110 0111’B |  |  |
| Payload container type |  | Set according to specific message content |  |  |
| Payload container type |  | ‘0001’B | N1 SM information | INITIAL_PDU_REQUEST |
| Spare half octet |  | '0000'B |  |  |
| Payload container |  | Set according to specific message content |  |  |
| Payload container |  | PDU SESSION ESTABLISHMENT REQUEST message |  | INITIAL_PDU_REQUEST |
| PDU session ID |  | If present: contents not checked |  |  |
| PDU session ID |  | Same PDU session ID as defined in the PDU SESSION ESTABLISHMENT REQUEST message in the Payload container |  | INITIAL_PDU_REQUEST |
| Old PDU session ID |  | If present: contents not checked |  |  |
| Request type |  | If present: contents not checked |  |  |
| Request type |  | ‘001’B | initial request | INITIAL_PDU_REQUEST |
| S-NSSAI |  | If present: contents not checked |  |  |
| DNN |  | If present: contents not checked |  |  |
| Additional information |  | If present: contents not checked |  |  |

| Condition | Explanation |
| --- | --- |
| INITIAL_PDU_REQUEST | The UL NAS TRANSPORT message is used to transport a PDU SESSION ESTABLISHMENT REQUEST message to establish a new PDU session. |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	DL NAS transport

Table 4.7.1-11: DL NAS TRANSPORT

| Derivation Path: 24.501 clause 8.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| DL NAS TRANSPORT message identity |  | ‘0110 1000’B |  |  |
| Payload container type |  | Set according to specific message content |  |  |
| Payload container type |  | ‘0001’B | N1 SM information | 5GSM_MESSAGE |
| Spare half octet |  | '0000'B |  |  |
| Payload container |  | Set according to specific message content |  |  |
| Payload container |  | 5GSM message |  | 5GSM_MESSAGE |
| PDU session ID |  | Not Present |  |  |
| PDU session ID |  | Set to the same value as the PDU session ID of the 5GSM message in the Payload container. |  | 5GSM_MESSAGE |
| Additional information |  | Not Present |  |  |
| 5GMM cause |  | Not Present |  |  |
| Back-off timer value |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| 5GSM_MESSAGE | The DL NAS TRANSPORT message is used to transport a 5GSM message |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	De-registration request (UE originating de-registration)

Table 4.7.1-12: DEREGISTRATION REQUEST_1

| Derivation Path: 24.501 clause 8.2.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| De-registration request message identity |  | ‘0100 0101’B |  |  |
| De-registration type |  |  |  |  |
| Switch off |  | ‘0’B |  | NORMAL |
|  |  | ‘1’B |  | SWITCH_OFF |
| Re-registration required |  | ‘0’B |  |  |
| Access type |  | ‘01’B | 3GPP access |  |
| ngKSI |  | Present but contents not checked |  |  |
| 5GS mobile identity |  | Present but contents not checked |  |  |

| Condition | Explanation |
| --- | --- |
| NORMAL | Normal de-registration |
| SWITCH_OFF | Switch off |

NOTE:	If this message is sent as an initial NAS message, it is sent with integrity protection but without ciphering. Otherwise it is sent without integrity protection and ciphering before SS has started the ciphering and integrity and ciphered protected after SS has started the ciphering.

#### –	De-registration accept (UE originating de-registration)

Table 4.7.1-13: DEREGISTRATION ACCEPT_1

| Derivation Path: 24.501 clause 8.2.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| De-registration accept message identity |  | ‘0100 0110’B |  |  |

NOTE:	This message is sent using the same security protection as in the previous DETACH REQUEST message received from the UE.

#### –	De-registration request (UE terminated de-registration)

Table 4.7.1-14: DEREGISTRATION REQUEST_2

| Derivation Path: 24.501 clause 8.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| De-registration request message identity |  | ‘0100 0111’B |  |  |
| De-registration type |  | Set according to specific message content |  |  |
| Spare half octet |  | '0000'B |  |  |
| 5GMM cause |  | Not Present |  |  |
| T3346 value |  | Not Present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	De-registration accept (UE terminated de-registration)

Table 4.7.1-15: DEREGISTRATION ACCEPT_2

| Derivation Path: 24.501 clause 8.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| De-registration accept message identity |  | ‘0100 1000’B |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Service request

Table 4.7.1-16: SERVICE REQUEST

| Derivation Path: 24.501 clause 8.2.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Service request message identity |  | ‘0100 1100’B |  |  |
| ngKSI |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier of the UE |  |  |
| TSC |  | '0'B | native security context (for KSIAMF) |  |
| Service type |  | ‘0010’B | mobile terminated services |  |
| 5G-S-TMSI |  | The valid 5G-S-TMSI of the UE |  |  |
| Uplink data status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| PDU session status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Allowed PDU session status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| NAS message container |  | If present, the complete, ciphered, SERVICE REQUEST message including all IEs. |  | CIPHERED_MESSAGE |

| Condition | Explanation |
| --- | --- |
| NON_CLEARTEXT_IE | An information element that is not allowed to be sent in cleartext and shall only be included in the complete SERIVICE REQUEST message in the NAS message container IE. NOTE: This condition is only applicable if the SERVICE REQUEST message is sent as an initial NAS message. |
| CIPHERED_MESSAGE | If any of the IEs marked with the condition NON_CLEARTEXT_IE is present, this condition applies. NOTE: This condition is only applicable if the SERVICE REQUEST message is sent as an initial NAS message. |

NOTE:	This message is sent without integrity protection, including only cleartext IEs, before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after NAS security mode control procedure has been successfully completed

#### –	Service accept

Table 4.7.1-17: SERVICE ACCEPT

| Derivation Path: 24.501 clause 8.2.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Service accept message identity |  | ‘0100 1110’B |  |  |
| PDU session status |  | Not Present |  |  |
| PDU session reactivation result |  | Not Present |  |  |
| PDU session reactivation result error cause |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Service reject

Table 4.7.1-18: SERVICE REJECT

| Derivation Path: 24.501 clause 8.2.18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Service reject message identity |  | ’0100 1101’B |  |  |
| 5GMM cause |  | The value is set according to specific message content |  |  |
| PDU session status |  | Not Present |  |  |
| T3346 value |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after NAS security mode control procedure has been successfully completed

#### –	Configuration update command

Table 4.7.1-19: CONFIGURATION UPDATE COMMAND

| Derivation Path: 24.501 clause 8.2.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Configuration update command message identity |  | ‘0101 0100’B |  |  |
| Configuration update indication |  | Not Present |  |  |
| 5G-GUTI |  | Not Present |  |  |
| TAI list |  | Not Present |  |  |
| Allowed NSSAI |  | Not Present |  |  |
| Service area list |  | Not Present |  |  |
| Full name for network |  | Not Present |  |  |
| Short name for network |  | Not Present |  |  |
| Local time zone |  | Not Present |  |  |
| Universal time and local time zone |  | Not Present |  |  |
| Network daylight saving time |  | Not Present |  |  |
| LADN information |  | Not Present |  |  |
| MICO indication |  | Not Present |  |  |
| Network slicing indication |  | Not Present |  |  |
| Configured NSSAI |  | Not Present |  |  |
| Rejected NSSAI |  | Not Present |  |  |
| Operator-defined access category definitions |  | Not Present |  |  |
| SMS indication |  | Not Present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Configuration update complete

Table 4.7.1-20: CONFIGURATION UPDATE COMPLETE

| Derivation Path: 24.501 clause 8.2.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Configuration update complete message identity |  | ‘0101 0101’B |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Identity request

Table 4.7.1-21: IDENTITY REQUEST

| Derivation Path: 24.501 clause 8.2.21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Identity request message identity |  | ‘0101 1011’B |  |  |
| Identity type |  | Set according to specific message contents |  |  |
| Spare half octet |  | '0000'B |  |  |

NOTE:	This message is sent without integrity protection before 5GS NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after 5GS NAS security mode control procedure has been successfully completed.

#### –	Identity response

Table 4.7.1-22: IDENTITY RESPONSE

| Derivation Path: 24.501 clause 8.2.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Identity response message identity |  | 0101 1100’B |  |  |
| Mobile identity |  | Present but contents not checked |  |  |

NOTE:	This message is sent without integrity protection before 5GS NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after 5GS NAS security mode control procedure has been successfully completed.

#### –	Notification

Table 4.7.1-23: NOTIFICATION

| Derivation Path: 24.501 clause 8.2.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Notification message identity |  | ‘0110 0101’B |  |  |
| Access type |  | ‘01’B | 3GPP access |  |
| Spare half octet |  | '0000'B |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Notification response

Table 4.7.1-24: NOTIFICATION RESPONSE

| Derivation Path: 24.501 clause 8.2.24 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Notification response message identity |  | ‘0110 0110’B |  |  |
| PDU session status |  | If present: contents not checked |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	Security mode command

Table 4.7.1-25: SECURITY MODE COMMAND

| Derivation Path: 24.501 clause 8.2.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Security mode command message identity |  | ‘0101 1101’B |  |  |
| Selected NAS security algorithms |  |  |  |  |
| Type of ciphering algorithm |  | Set according to PIXIT px_NAS_5GC_CipheringAlgorithm for default ciphering algorithm |  |  |
| Type of ciphering algorithm |  | ‘0000’B | 5G encryption algorithm 5G EA0 (null ciphering algorithm) | For RF |
| Type of integrity protection algorithm |  | Set according to PIXIT px_NAS_5GC_IntegrityProtAlgorithm for default integrity protection algorithm | This value should not be equal to the null integrity algorithm. |  |
| ngKSI |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier |  |  |
| TSC |  | '0'B | native security context (for KSIAMF) |  |
| Spare half octet |  | '0000'B |  |  |
| Replayed UE security capabilities |  | Set according to the received UE security capabilities |  |  |
| IMEISV request |  | Not Present |  |  |
| Selected EPS NAS security algorithms |  | Not Present |  |  |
| Selected EPS NAS security algorithms |  |  |  | UE_S1_SUPPORTED |
| Type of ciphering algorithm |  | Set according to PIXIT px_NAS_CipheringAlgorithm for default ciphering algorithm | The px_NAS_CipheringAlgorithm PIXIT is defined in TS 36.523-3 [x] |  |
| Type of integrity protection algorithm |  | Set according to PIXIT px_NAS_IntegrityProtAlgorithmfor default integrity protection algorithm | The px_NAS_IntegrityProtAlgorithm is defined in TS 36.523-3 [x] |  |
| Additional 5G security information |  | Not Present |  |  |
| Additional 5G security information |  |  |  | NO_VALID_SS_SECURITY_CONTEXT |
| RINMR |  | ‘1’B | Retransmission of the initial NAS message requested |  |
| HDP |  | ‘0’B | KAMF derivation is not required |  |
| EAP message |  | Not Present |  |  |
| EAP message |  | EAP-Success | See Table 4.7.3.2-03 | EAP-AKA |
| ABBA |  | ‘0000 0000 0000 0000’B |  | EAP-AKA |
| Replayed S1 UE security capabilities |  | Not Present |  |  |
| Replayed S1 UE security capabilities |  | Set according to the received UE security capabilities in the last REGISTRATION REQUEST message |  | UE_S1_SUPPORTED |

| Condition | Explanation |
| --- | --- |
| NO_VALID_SS_SECURITY_CONTEXT | If the SS doesn’t have a valid security context |
| EAP_AKA | EAP based primary authentication and key agreement procedure |
| For RF | Used for RF/RRM test cases |
| UE_S1_SUPPORTED | The UE indicated support of S1 in the last REGISTRATION REQUEST message |

NOTE:	This message is always sent integrity protected with new 5GS NAS security context.

#### –	Security mode complete

Table 4.7.1-26: SECURITY MODE COMPLETE

| Derivation Path: 24.501 clause 8.2.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Security mode complete message identity |  | ‘0101 1110’B |  |  |
| IMEISV |  | Not present |  |  |
| NAS message container |  | Not present |  |  |
|  |  | Complete initial NAS message |  | RINMR_INDICATED |

| Condition | Explanation |
| --- | --- |
| RINMR_INDICATED | The SS requested retransmission of the initial NAS message in the last SECURITY MODE COMMAND |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message with new 5GS NAS security context.

#### –	Security mode reject

Table 4.7.1-27: SECURITY MODE REJECT

| Derivation Path: 24.501 clause 8.2.27 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Security mode reject message identity |  | ‘0101 1111’B |  |  |
| 5GMM cause |  | The value is set according to specific message content |  |  |

NOTE:	This message is sent without integrity protection before 5GS NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after 5GS NAS security mode control procedure has been successfully completed.

#### –	Security protected 5GS NAS message

Table 4.7.1-28: SECURITY PROTECTED 5GS NAS MESSAGE

| Derivation Path: 24.501 clause 8.2.28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | '0001'B | Integrity protected | UNCIPHERED |
|  |  | '0010'B | Integrity protected and ciphered | CIPHERED |
|  |  | '0011'B | Integrity protected with new 5G NAS security context | UNCIPHERED-NEW |
|  |  | '0100'B | Integrity protected and ciphered with new 5G NAS security context | CIPHERED-NEW |
| Spare half octet |  | '0000'B |  |  |
| Message authentication code |  | The calculated value of MAC-I for this message. | The value of MAC-I is calculated by SS using Sequence number sent by UE. | SENT-BY-SS |
|  |  | The same value as the XMAC-I value calculated by SS. |  | SENT-BY-UE |
| Sequence number |  | The internal counter of the SS |  | SENT-BY-SS |
|  |  | Any allowed value |  | SENT-BY-UE |
| Plain 5GS NAS message |  | Set according to specific message content |  |  |

| Condition | Explanation |
| --- | --- |
| UNCIPHERED | This condition applies to unciphered NAS message exchange |
| CIPHERED | This condition applies to ciphered NAS message exchange |
| UNCIPHERED-NEW | This condition applies to unciphered NAS message exchange with new 5G NAS security context |
| CIPHERED-NEW | This condition applies to ciphered NAS message exchange with new 5G NAS security context |
| SENT-BY-SS | Use for the message sent from SS to UE |
| SENT-BY-UE | Use for the message sent from UE to SS |

#### –	5GMM status

Table 4.7.1-29: 5GMM STATUS

| Derivation Path: 24.501 clause 8.2.29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| 5GMM STATUS message identity |  | ‘0110 0100’B |  |  |
| 5GMM cause |  | ‘0110 1111’B | Protocol error, unspecified | SENT-BY-SS |
|  |  | Present but contents not checked |  | SENT-BY-UE |

| Condition | Explanation |
| --- | --- |
| SENT-BY-SS | Use for the message sent from SS to UE |
| SENT-BY-UE | Use for the message sent from UE to SS |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

### 4.7.2	Contents of 5GSM messages

#### –	PDU session establishment request

Table 4.7.2-1: PDU SESSION ESTABLISHMENT REQUEST

| Derivation Path: 24.501 clause 8.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Any value according to TS 24.501 [25] subclause 9.4 |  |  |
| PTI |  | Any value from 1 to 254 |  |  |
| PDU SESSION ESTABLISHMENT REQUEST message identity |  | ‘1100 0001’B |  |  |
| Integrity protection maximum data rate |  | Present but contents not checked |  |  |
| PDU session type |  | Any value between '001'B, '010'B and '011'B | The allowed values are respectively IPv4, IPv6 and IPv4v6 |  |
| SSC mode |  | If present: contents not checked |  |  |
| 5GSM capability |  | If present: contents not checked |  |  |
| Maximum number of supported packet filters |  | If present: contents not checked |  |  |
| Always-on PDU session requested |  | If present: contents not checked |  |  |
| SM PDU DN request container |  | If present: contents not checked |  |  |
| Extended protocol configuration options |  | If present: contents not checked | The SS shall remember if this IE is present and its contents because this affects subsequent SS behaviour, e.g. coding of PDU SESSION ESTABLISHMENT ACCEPT. |  |

#### –	PDU session establishment accept

Table 4.7.2-2: PDU SESSION ESTABLISHMENT ACCEPT

| Derivation Path: 24.501 clause 8.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI |  | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PDU SESSION ESTABLISHMENT ACCEPT message identity |  | ‘1100 0010’B |  |  |
| Selected PDU session type |  | '001'B |  | IPv4 |
|  |  | '010'B |  | IPv6 |
|  |  | '011'B |  | IPv4v6 |
| Selected SSC mode |  | ‘001’B | SSC mode 1 |  |
| Authorized QoS rules |  | Reference QoS rule #1 as defined in Table 4.8.2.1-1. |  |  |
| Authorized QoS rules |  | Reference QoS rule #2 as defined in Table 4.8.2.1-2. |  | IMS_DNN |
| Session AMBR |  |  |  |  |
| Unit for Session-AMBR for downlink |  | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for downlink |  | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| Unit for Session-AMBR for uplink |  | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for uplink |  | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| 5GSM cause |  | Not Present |  |  |
| PDU address |  |  |  | IPv4 |
| Length of PDU address contents |  | 5 octets |  |  |
| PDU type value |  | ‘001’B | IPv4 |  |
| PDU address information |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDU address |  |  |  | IPv6 |
| Length of PDU address contents |  | 9 octets |  |  |
| PDU type value |  | ‘010’B | IPv6 |  |
| PDU address information |  | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDU address |  |  |  | IPv4v6 |
| Length of PDU address contents |  | 13 octets |  |  |
| PDU type value |  | ‘011’B | IPv4v6 |  |
| PDU address information (Octets 4 to 11) |  | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDU address information (Octets 12 to 15) |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| RQ timer value |  | Not Present |  |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |
| SST |  | ‘0000 0001’B | SST value 1 (eMBB) |  |
| Always-on PDU session indication |  | Not Present |  |  |
| Always-on PDU session indication |  |  |  |  |
| APSI |  | ‘0’B | Always-on PDU session not allowed | Always_On_Requested |
| Mapped EPS bearer contexts |  | Not Present |  |  |
| Mapped EPS bearer contexts |  |  |  | Interworking_with_EPS |
| Mapped EPS bearer context |  |  |  |  |
| EPS bearer identity |  | The same value as the one specified in the Reference QoS flow referred to from the Reference QoS rule indicated in the IE Authorized QoS rules |  | NOT IMS_DNN |
| EPS bearer identity |  | 5 | EBI 5 | IMS_DNN |
| Operation code |  | ‘001’B | Create new EPS bearer |  |
| E bit |  | '1'B | Parameters list is included |  |
| Number of EPS parameters |  | ’0001’B | 1 parameter |  |
| Mapped EPS QoS parameters |  | See Reference default EPS bearer context #1 in TS 36.508 [7] Table 6.6.1-1 | QCI 9 |  |
| Mapped EPS QoS parameters |  | See Reference default EPS bearer context #2 in TS 36.508 [7] Table 6.6.1-1 | QCI 5 | IMS_DNN_ Requested |
| EAP message |  | Not Present |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #1 as defined in Table 4.8.2.3-1. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #2 as defined in Table 4.8.2.3-2. |  | IMS_DNN_ Requested |
| Extended protocol configuration options |  | Not Present |  |  |
| Extended protocol configuration options |  |  |  | P-CSCF_IPv6 OR P-CSCF_IPv4 |
| Container ID 1 |  | ‘0001’H |  | P-CSCF_IPv6 |
| Length of container ID 1 contents |  |  | Length value determined by testimplementation |  |
| Container ID 1 contents |  | IPv6 address | P-CSCF IPv6 Address |  |
| Container ID 2 |  | ‘000C’H |  | P-CSCF_IPv4 |
| Length of container ID 2 contents |  |  | Length value determined by test implementation |  |
| Container ID 2 contents |  | IPv4 address | P-CSCF IPv4 Address |  |
| DNN |  | The SS defines a Default DNN |  | DefaultDNNand NOT IMS_DNN |
| DNN |  | DNN as provided in the UL NAS TRANSPORT message transporting the last PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| DNN |  | Use DNN Network Identifier as provided in the DNN IE of the UL NAS TRANSPORT message transporting the last PDU SESSION ESTABLISHMENT REQUEST message and the DNN Operator Identifier mnc<MNC>.mcc<MCC>.gprs. The <MNC> and <MCC> are set to the same values as in IMSI. |  | IMS_DNN |

| Condition | Explanation |
| --- | --- |
| IPv4 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '001'B |
| IPv6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '010'B |
| IPv4v6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '011'B |
| IPv4-DHCP | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the IE Extended protocol configuration options contains a configuration protocol option = '000BH' ("IPv4 address allocation via DHCPv4", length of contents = 0). Note: This condition is used in conjunction with IPv4 or IPv4v6 as indicated in the "PDU address information" just above. |
| DefaultDNN | If the UL NAS TRANSPORT message transporting the last PDU SESSION ESTABLISHMENT REQUEST message did not includ the DNN IE |
| Always_On_Requested | If the last PDU SESSION ESTABLISHMENT REQUEST message included the Always-on PDU session requested IE |
| IMS_DNN | NOT DefaultDNN: If the UL NAS TRANSPORT message transporting the last PDU SESSION ESTABLISHMENT REQUEST message included an IMS DNN in the DNN IE DefaultDNN: IF pc_IMS_DNN_default=TRUE [19] |
| P-CSCF_IPv6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message the IE Extended protocol configuration options contains a configuration protocol option = '0001H' (“P-CSCF IPv6 Address Request", length of contents = 0) |
| P-CSCF_IPv4 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message the IE Extended protocol configuration options contains a configuration protocol option = '000CH' (“P-CSCF IPv4 Address Request", length of contents = 0) |
| Interworking_with_EPS | If the UE has indicated support of S1, then the SS shall include this IE to provide details for the interworking with EPS being supported for a PDU session. This requirement is set up for the purpose of facilitating the test description. It is not mandatory for the Network to support Mapped EPS bearer contexts. |

#### –	PDU session establishment reject

Table 4.7.2-3: PDU SESSION ESTABLISHMENT REJECT

| Derivation Path: 24.501 clause 8.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI |  | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PDU SESSION ESTABLISHMENT REJECT message identity |  | ‘1100 0011’B |  |  |
| 5GSM cause |  | The value is set according to specific message content |  |  |
| Back-off timer value |  | Not Present |  |  |
| Allowed SSC mode |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session authentication command

Table 4.7.2-4: PDU SESSION AUTHENTICATION COMMAND

| Derivation Path: 24.501 clause 8.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION AUTHENTICATION COMMAND message identity |  | ‘1100 0101’B |  |  |
| EAP message |  | FFS | See TS 24.501 [25] subclause 9.11.2.2 |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session authentication complete

Table 4.7.2-5: PDU SESSION AUTHENTICATION COMPLETE

| Derivation Path: 24.501 clause 8.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION AUTHENTICATION COMMAND message |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION AUTHENTICATION COMPLETE message identity |  | ‘1100 0110’B |  |  |
| EAP message |  | FFS | See TS 24.501 [25] subclause 9.11.2.2 |  |
| Extended protocol configuration options |  | If present: contents not checked |  |  |

#### –	PDU session authentication result

Table 4.7.2-6: PDU SESSION AUTHENTICATION RESULT

| Derivation Path: 24.501 clause 8.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION AUTHENTICATION COMMAND message |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION AUTHENTICATION RESULT message identity |  | ‘1100 0111’B |  |  |
| EAP message |  | FFS | See TS 24.501 [25] subclause 9.11.2.2 |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session modification request

Table 4.7.2-7: PDU SESSION MODIFICATION REQUEST

| Derivation Path: 24.501 clause 8.3.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI |  | Any value from 1 to 254 |  |  |
| PDU SESSION MODIFICATION REQUEST message identity |  | ‘1100 1001’B |  |  |
| 5GSM capability |  | If present: contents not checked |  |  |
| 5GSM cause |  | If present: contents not checked |  |  |
| Maximum number of supported packet filters |  | If present: contents not checked |  |  |
| Always-on PDU session requested |  | If present: contents not checked |  |  |
| Integrity protection maximum data rate |  | If present: contents not checked |  |  |
| Requested QoS rules |  | If present: contents not checked |  |  |
| Requested QoS flow descriptions |  | If present: contents not checked |  |  |
| Mapped EPS bearer contexts |  | If present: contents not checked |  |  |
| Extended protocol configuration options |  | If present: contents not checked |  |  |

#### –	PDU session modification reject

Table 4.7.2-8: PDU SESSION MODIFICATION REJECT

| Derivation Path: 24.501 clause 8.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  |  |
| PTI |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  |  |
| PDU SESSION MODIFICATION REJECT message identity |  | ‘1100 1010’B |  |  |
| 5GSM cause |  | Set according to specific message content. |  |  |
| Back-off timer value |  | Not Present |  |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session modification command

Table 4.7.2-9: PDU SESSION MODIFICATION COMMAND

| Derivation Path: 24.501 clause 8.3.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PDU session ID |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  | UE_Initiated_Modification |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PTI |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  | UE_Initiated_Modification |
| PDU SESSION MODIFICATION COMMAND message identity |  | ‘1100 1011’B |  |  |
| 5GSM cause |  | Not Present |  |  |
| Session AMBR |  | Not Present |  |  |
| RQ timer value |  | Not Present |  |  |
| Always-on PDU session indication |  | Not Present |  |  |
| Always-on PDU session indication |  |  |  |  |
| APSI |  | ‘0’B | Always-on PDU session not allowed | Always_On_Requested |
| Authorized QoS rules |  | Not Present |  |  |
| Mapped EPS bearer contexts |  | Not Present |  |  |
| Authorized QoS flow descriptions |  | Not Present |  |  |
| Extended protocol configuration options |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Always_On_Requested | If the last PDU SESSION MODIFICATION REQUEST message included the Always-on PDU session requested IE |
| UE_Initiated_Modification | If this message was triggered by a PDU SESSION MODIFICATION REQUEST message sent by the UE |

#### –	PDU session modification complete

Table 4.7.2-10: PDU SESSION MODIFICATION COMPLETE

| Derivation Path: 24.501 clause 8.3.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION MODIFICATION COMMAND message |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PTI |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  | UE_Initiated_Modification |
| PDU SESSION MODIFICATION COMPLETE message identity |  | ‘1100 1100’B |  |  |
| Extended protocol configuration options |  | If present: contents not checked |  |  |

| Condition | Explanation |
| --- | --- |
| UE_Initiated_Modification | If this message was triggered by a PDU SESSION MODIFICATION REQUEST message sent by the UE |

#### –	PDU session modification command reject

Table 4.7.2-11: PDU SESSION MODIFICATION COMMAND REJECT

| Derivation Path: 24.501 clause 8.3.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION MODIFICATION COMMAND message |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION MODIFICATION COMMAND REJECT message identity |  | ‘1100 1101’B |  |  |
| 5GSM cause |  | If present: contents not checked |  |  |
| Extended protocol configuration options |  | If present: contents not checked |  |  |

#### –	PDU session release request

Table 4.7.2-12: PDU SESSION RELEASE REQUEST

| Derivation Path: 24.501 clause 8.3.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI |  | Any value from 1 to 254 |  |  |
| PDU SESSION RELEASE REQUEST message identity |  | ‘1101 0001’B |  |  |
| 5GSM cause |  | If present: contents not checked |  |  |
| Extended protocol configuration options |  | If present: contents not checked |  |  |

#### –	PDU session release reject

Table 4.7.2-13: PDU SESSION RELEASE REJECT

| Derivation Path: 24.501 clause 8.3.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION RELEASE REQUEST message. |  |  |
| PTI |  | The value indicated in PDU SESSION RELEASE REQUEST message. |  |  |
| PDU SESSION RELEASE REJECT message identity |  | ‘1101 0010’B |  |  |
| 5GSM cause |  | Set according to specific message content. |  |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session release command

Table 4.7.2-14: PDU SESSION RELEASE COMMAND

| Derivation Path: 24.501 clause 8.3.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION RELEASE COMMAND message identity |  | ‘1101 0011’B |  |  |
| 5GSM cause |  | ‘0001 1010’B | Insufficient resources |  |
| Back-off timer value |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session release complete

Table 4.7.2-15: PDU SESSION RELEASE COMPLETE

| Derivation Path: 24.501 clause 8.3.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION RELEASE COMMAND message. |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION RELEASE COMPLETE message identity |  | ‘1101 0100’B |  |  |
| 5GSM cause |  | If present: contents not checked |  |  |
| Extended protocol configuration options |  | If present: contents not checked |  |  |

#### –	5GSM status

Table 4.7.2-16: 5GSM STATUS

| Derivation Path: 24.501 clause 8.3.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PTI |  | Set according to specific message content. |  |  |
| 5GSM STATUS message identity |  | ‘1101 0110’B |  |  |
| 5GSM cause |  | Set according to specific message content. |  |  |

### 4.7.3	Contents of EAP-AKA' messages

For all the message definitions below, the acceptable order and syntax of attributes and fields within these attributes must be according to IETF RFCs where those attributes have been defined. Typically the order of attributes is not significant, but there could be well defined exceptions where the order is important.

The contents of the messages described in the present Annex is not complete - only the attributes required to be checked or generated by SS are listed here. The messages sent by the UE may contain additional attributes which are not checked and must thus be ignored by SS.

#### 4.7.3.1	EAP-AKA' message attributes

Table 4.7.3.1-1: AT_RAND_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_RAND |  | '0000 0001'B | 1 |  |
| Length |  | '0000 0101'B | 5 |  |
| Reserved |  | '0000 0000 0000 0000'B |  |  |
| RAND |  | An arbitrarily selected 128 bits value |  |  |

Table 4.7.3.1-2: AT_AUTN_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_AUTN |  | '0000 0010'B | 2 |  |
| Length |  | '0000 0101'B | 5 |  |
| Reserved |  | '0000 0000 0000 0000'B |  |  |
| AUTN |  | 128 bits value generated according to TS 24.501 [28] subclause 9.11.3.15 |  |  |

Table 4.7.3.1-3: AT_KDF_Def

| Derivation Path: IETF RFC 5448 [31] clause 3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_KDF |  | '0001 1000'B | 24 |  |
| Length |  | '0000 0001'B | 1 |  |
| KDF |  | '0000 0000 0000 0001' | 1: EAP_AKA' |  |

Table 4.7.3.1-4: AT_KDF_INPUT_Def

| Derivation Path: IETF RFC 5448 [31] clause 3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_KDF_INPUT |  | '0001 0111'B | 23 |  |
| Length |  | Set to the Length of attribute AT_KDF_INPUT in 4 bytes |  |  |
| Actual Network Name Length |  | Set to the actual length of 'Network Name' in bytes excluding any appended all zero bytes at end |  |  |
| Network Name |  | Value generated according to TS 24.501 [28] clause 9.12.1 and shall be a multiple of 4 bytes (appended with 1,2 or 3 bytes of all zero bits when necessary) |  |  |

Table 4.7.3.1-5: AT_MAC_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_MAC |  | '0000 1011'B | 11 |  |
| Length |  | '0000 0101'B | 5 |  |
| Reserved |  | '0000 0000 0000 0000'B |  |  |
| MAC |  | 128 bits value generated according to RFC 4187 [30] subclause 10.15 |  |  |

Table 4.7.3.1-6: AT_RES_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.08 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_RES |  | '0000 0011'B | 3 |  |
| Length |  | Set to Length of AT_RES attribute in 4 bytes. | 1 byte |  |
| RES_LENGTH |  | Set to the actual length of 'RES' in bytes excluding any appended all zero bytes at end |  |  |
| RES |  | RES* value calculated according to TS 24.501 [28] clause 9.11.3.17, possibly appended with 1,2 or 3 bytes of all zero bits to make lenght multiple of 4 bytes. |  |  |

Table 4.7.3.1-7: AT_AUTS_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.08 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_AUTS |  | '0000 0100'B | 4 |  |
| Length |  | '0000 0100'B | 4 |  |
| AUTS |  | 14 octets RES* value not checked |  |  |

Table 4.7.3.1-8: AT_PERMANENT_ID_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_PERMANENT_ID_REQ |  | '0000 1010'B | 10 |  |
| Length |  | '0000 0001'B | 1 |  |
| Reserved |  | '0000 0000 0000 0000'B |  |  |

Table 4.7.3.1-9: AT_ANY_ID_REQ_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_ANY_ID_REQ |  | '0000 1101'B | 13 |  |
| Length |  | '0000 0001'B | 1 |  |
| Reserved |  | '0000 0000 0000 0000'B |  |  |

Table 4.7.3.1-10: AT_FULLAUTH_ID_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AT_FULLAUTH_ID_REQ |  | '0001 0001'B | 17 |  |
| Length |  | '0000 0001'B | 1 |  |
| Reserved |  | '0000 0000 0000 0000'B |  |  |

Table 4.7.3.1-11: AT_IDENTITY_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| AT_IDENTITY | '0000 1110'B | 14 |  |  |
| Length | Set to the Length of AT_IDENTITY attribute in 4 bytes |  |  |  |
| Actual Identity Length | Set to the actual length of 'identity' in bytes excluding any appended all zero bytes at end |  |  |  |
| Identity | Value generated according to TS 24.501 [28] clause 9.11.3.4 and shall be a multiple of 4 bytes (appended with 1,2 or 3 bytes of all zero bits when necessary) |  |  |  |

Table 4.7.3.1-12: AT_NOTIFICATION_Def

| Derivation Path: IETF RFC 4187 [30] clause 10.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| AT_NOTIFICATION | '0000 1100'B | 12 |  |  |
| Length | '0000 0001'B | 1 |  |  |
| Notification Code | 16 bits value generated according to RFC 4187 [30] subclause 10.19. |  |  |  |

#### 4.7.3.2	EAP-AKA' messages

Table 4.7.3.2-1: EAP-Request/AKA'-Challenge

| Derivation Path: IETF RFC 4187 [30] clause 9.3, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 1 | Request |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  |  |  |  |
| AT_RAND |  | AT_RAND_Def |  |  |
| AT_AUTN |  | AT_AUTN_Def |  |  |
| AT_KDF |  | AT_KDF_Def |  |  |
| AT_KDF_INPUT |  | AT_KDF_INPUT_Def |  |  |
| AT_MAC |  | AT_MAC_Def |  |  |

Table 4.7.3.2-2: EAP-Response/AKA'-Challenge

| Derivation Path: IETF RFC 4187 [30] clause 9.4, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 2 | Response |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  |  |  |  |
| AT_Res |  | AT_Res_Def |  |  |
| AT_MAC |  | AT_MAC_Def |  |  |

Table 4.7.3.2-3: EAP-Succes

| Derivation Path: IETF RFC 4187 [30] clause 6.3.4, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 3 | Success |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  | Not present | Specific attributes not present |  |

Table 4.7.3.2-4: EAP-Response/AKA-Authentication-Reject

| Derivation Path: IETF RFC 4187 [30] clause 9.5, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 4 | Failure |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  | Not checked |  |  |

Table 4.7.3.2-5: EAP-Response/AKA-Synchronization-Failure

| Derivation Path: IETF RFC 4187 [30] clause 9.6, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 4 | Failure |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  |  |  |  |
| AT_AUTS |  | AT_AUTS_Def |  |  |

Table 4.7.3.2-6: EAP-Failure

| Derivation Path: IETF RFC 4187 [30] clause 6.3.3, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 4 | Failure |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  | Not present | Specific attributes not present |  |

Table 4.7.3.2-7: EAP-Request/AKA-Identity

| Derivation Path: IETF RFC 4187 [30] clause 9.1, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 1 | Request |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  |  |  |  |
| AT_PERMANENT_ID_REQ |  | AT_PERMANENT_ID_REQ_Def |  | SS requests that the UE send its permanent identity. |
| AT_ANY_ID_REQ |  | AT_ANY_ID_REQ_Def |  | SS does not specify which kind of an identity the UE should return. |
| AT_FULLAUTH_ID_REQ |  | AT_FULLAUTH_ID_REQ_Def |  | SS requests either the permanent identity or a pseudonym identity. |

Table 4.7.3.2-8: EAP-Response/AKA-Identity

| Derivation Path: IETF RFC 4187 [30] clause 9.2, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 2 | Response |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  |  |  |  |
| AT_IDENTITY |  | AT_IDENTITY_Def |  |  |

Table 4.7.3.2-9: EAP-Request/AKA-Notification

| Derivation Path: IETF RFC 4187 [30] clause 9.10, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 1 | Request |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  |  |  |  |
| AT_NOTIFICATION |  | AT_NOTIFICATION_Def |  |  |

Table 4.7.3.2-10: EAP-Response/AKA-Notification

| Derivation Path: IETF RFC 4187 [30] clause 9.11, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Code |  | 2 | Response |  |
| Length |  | Set to length of EAP packet |  |  |
| Data |  | Not present | Specific attributes not present |  |

## 4.7A	Default TC message and information element contents

This clause contains the default values of common TC (Test Control, see [11]) messages and information elements, which apply to all test cases unless otherwise specified. All the messages and information elements are listed in alphabetical order.

### 4.7A.1	Test mode messages

-	ACTIVATE TEST MODE with the following exception:

-	The supported test modes for 5GS are limited to those specified in subclause 38.509 [11] 5.3.4.

Same as TS 36.508 [2], Table 4.7A-1.

#### -	ACTIVATE TEST MODE COMPLETE

Same as TS 36.508 [2], Table 4.7A-2.

#### -	DEACTIVATE TEST MODE

Same as TS 36.508 [2], Table 4.7A-5.

#### -	DEACTIVATE TEST MODE COMPLETE

Same as TS 36.508 [2], Table 4.7A-6.

### 4.7A.2	Test loop messages

#### -	CLOSE UE TEST LOOP

Same as TS 36.508 [2], Table 4.7A-3 with the following exception:

-	The supported test modes for 5GS are limited to those specified in subclause 38.509 [11] 5.3.4.

#### -	CLOSE UE TEST LOOP COMPLETE

Same as TS 36.508 [2], Table 4.7A-4.

#### -	OPEN UE TEST LOOP

Same as TS 36.508 [2], Table 4.7A-7.

#### -	OPEN UE TEST LOOP COMPLETE

Same as TS 36.508 [2], Table 4.7A-8.

### 4.7A.3	Beamlock messages

#### -	ACTIVATE BEAMLOCK

This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

Table 4.7A.3-1: ACTIVATE BEAMLOCK

| Derivation Path: 38.509 clause 6.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 0 0 0 |  |  |
| UE Beamlock test Function | 0 0 0 0 0 0 0 1 |  | Tx Only |
| UE Beamlock test Function | 0 0 0 0 0 0 1 0 |  | Rx Only |
| UE Beamlock test Function | 0 0 0 0 0 0 1 1 |  | Tx and Rx |

| Condition | Explanation |
| --- | --- |
| Tx Only | Activation UE beamlock function for Tx only |
| Rx Only | Activation UE beamlock function for Rx only |
| Tx and Rx | Activation UE beamlock function for both Tx and Rx |

#### -	ACTIVATE BEAMLOCK COMPLETE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

Table 4.7A.3-2: ACTIVATE BEAMLOCK COMPLETE

| Derivation Path: 38.509 clause 6.4.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 0 0 1 |  |  |

#### -	DEACTIVATE BEAMLOCK

This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

Table 4.7A.3-3: DEACTIVATE BEAMLOCK

| Derivation Path: 38.509 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 0 1 0 |  |  |

#### -	DEACTIVATE BEAMLOCK COMPLETE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

Table 4.7A.3-4: DEACTIVATE BEAMLOCK COMPLETE

| Derivation Path: 38.509 clause 6.4.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 0 1 1 |  |  |

### 4.7A.4	UE SS-RSRP per receiver branch reporting messages

#### -	SS-RSRPB REPORT REQUEST

FFS

#### -	SS-RSRPB REPORT RESPONSE

FFS

### 4.7A.5	UE Positioning messages

#### -	RESET UE POSITIONING STORED INFORMATION

FFS

#### -	UPDATE UE LOCATION INFORMATION

FFS

### 4.7A.6	NSSAI delete messages

#### -	NSSAI DELETE REQUEST

This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

Table 4.7A.6-1: NSSAI DELETE REQUEST

| Derivation Path: 38.509 clause 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 0 0 0 1 1 0 |  |  |
| Delete NSSAI type | Set according to specific message contents |  |  |
| Configured NSSAI | Set according to specific message contents |  |  |
| Allowed NSSAI | Set according to specific message contents |  |  |

#### -	NSSAI DELETE RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

Table 4.7A.6-2: NSSAI DELETE RESPONSE

| Derivation Path: 38.509 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 1 1 1 |  |  |

## 4.8	Reference configurations

### 4.8.1	Radio configurations

#### –	RRCReconfiguration-DRB(n, m)

Table 4.8.1-1: RRCReconfiguration-DRB (n, m)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-DRB(n,m) |  |  |
| secondaryCellGroup | CellGroupConfig-DRB(n.m) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCReconfiguration-HO

Table 4.8.1-1A: RRCReconfiguration-HO

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB1 and SRB2 and DRBn and Re-establish_PDCP |  | RBConfig_KeyChange |
|  | RadioBearerConfig with conditions DRBn and Recover_PDCP |  | RBConfig_NoKeyChange |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with conditions PCell_change | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RBConfig_KeyChange | RadioBearerConfig to perform Intra-NR handover with security key change |
| RBConfig_NoKeyChange | RadioBearerConfig to perform Intra-NR handover without security key change |

#### –	RRCReconfiguration-SRB2-DRB(n, m)

Table 4.8.1-1B: RRCReconfiguration-SRB2-DRB(n, m)

| Derivation Path: Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-SRB2-DRB(n,m) |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-SRB2-DRB(n, m) | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCReconfiguration-NR-DC-SRB2-DRB(n, m)

Table 4.8.1-1CA: RRCReconfiguration-NR-DC-SRB2-DRB(n, m)

| Derivation Path: Table 4.6.1-13 with condition NR-DC. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-SRB2-DRB(n+1, m) | n+1 is chosen as the next available number higher or equal to 2. |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-SRB2-DRB(n, m) | OCTET STRING (CONTAINING CellGroupConfig). n is set to the same value as for the radioBearerConfig IE above. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCReconfiguration-Speech

Table 4.8.1-1C: RRCReconfiguration-Speech

| Derivation Path: Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-DRB(0,1) |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-DRB(0.1) | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CellGroupConfig-DRB(n, m)

Table 4.8.1-2: CellGroupConfig-DRB(n, m)

| Derivation Path: Table 4.6.3-19: CellGroupConfig (the same conditions are applicable as for table 4.6.3-19). |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | n+m entries | BID is the total number of established DRBs in the UE, before applying the contents of this IE |  |
| RLC-BearerConfig[k, k=1..n] | RLC-BearerConfig with conditions AM and DRBj (with j=BID+k) | entry (1..n+1) | n>0 |
| RLC-BearerConfig[k, k=n+1..n+m] | RLC-BearerConfig with conditions UM and DRBj (with j=BID+k) | entry (n+1..n+m) | m>0 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| n>0 | n is greater than zero |
| m>0 | m is greater than zero |

#### –	CellGroupConfig-SRB3

Table 4.8.1-2A: CellGroupConfig-SRB3

| Derivation Path: Table 4.6.3-19: CellGroupConfig with condition EN-DC. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with condition SRB3 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |

#### –	CellGroupConfig-SRB2-DRB(n, m)

Table 4.8.1-2B: CellGroupConfig-SRB2-DRB(n, m)

| Derivation Path: Table 4.6.3-19: CellGroupConfig |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1+n+m entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[k, k=2..n+1] | RLC-BearerConfig with conditions AM and DRBj | entry (2..n+1) j is allocated according to internal TTCN mapping | n>0 |
| ... |  | ... |  |
| RLC-BearerConfig[k, k=n+2..n+m+1] | RLC-BearerConfig with conditions UM and DRBj | entry (n+2..n+m+1) j is allocated according to internal TTCN mapping | m>0 |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

#### –	RadioBearerConfig-DRB (n, m)

Table 4.8.1-3: RadioBearerConfig-DRB (n, m)

| Derivation Path: Table 4.6.3-132 and condition EN-DC. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n+m entries | BID is the total number of established DRBs in the UE, before applying the contents of this IE |  |
| DRB-ToAddMod[k=1..n+m] SEQUENCE { |  | entry (1..n+m) |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | l, l=BID+5..BID+4+n+m |  |  |
| } |  |  |  |
| drb-Identity | l, l=BID+1..BID+n+m |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  | k <= n |
|  | PDCP-Config with condition UM |  | k > n |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RadioBearerConfig-SRB2-DRB (n, m)

Table 4.8.1-4: RadioBearerConfig-SRB2-DRB (n, m)

| Derivation Path: Table 4.6.3-132 and condition SRB2. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n+m entries |  |  |
| DRB-ToAddMod[k=1..n+m] := SEQUENCE { |  | entry (1..n+m) |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config | SDAP-Config is configured according to internal TTCN mapping |  |
| } |  |  |  |
| drb-Identity | j | j is allocated according to internal TTCN mapping |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  | (1<k <= n) OR (k=1 AND NOT NR_split) |
|  | PDCP-Config with condition UM |  | k > n |
|  | PDCP-Config with condition NR_split |  | k=1 AND NR_split |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 4.8.2	5GC configurations

#### 4.8.2.1	Reference QoS rules

Table 4.8.2.1-1: Reference QoS rule #1

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filters |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0001’B | QFI 1 (Table 4.8.2.3-1) |  |

Table 4.8.2.1-2: Reference QoS rule #2

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0010’B | 2 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |

Table 4.8.2.1-3: Reference QoS rule #3

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-2 | Packet filter list #2 |  |
| QoS rule precedence | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0001’B | QFI 1 (Table 4.8.2.3-1) |  |

Table 4.8.2.1-4: Reference QoS rule #4

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0100’B | 4 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-3 | Packet filter list #3 |  |
| QoS rule precedence | ‘0000 00100’B | 4 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |

Table 4.8.2.1-4a: Reference QoS rule #4a

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 1111’B | 15 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-3a | Packet filter list #3a |  |
| QoS rule precedence | ‘0000 1111’B | 15 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0100’B | QFI 4 (Table 4.8.2.3-2a) |  |

Table 4.8.2.1-5: Reference QoS rule #5

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0101’B | 5 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-4 | Packet filter list #4 |  |
| QoS rule precedence | ‘0000 0101’B | 5 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0101’B | QFI 5 (Table 4.8.2.3-3) |  |

Table 4.8.2.1-6: Reference QoS rule #6

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0110’B | 6 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-5 | Packet filter list #5 |  |
| QoS rule precedence | ‘0000 0110’B | 6 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0110’B | QFI 6 (Table 4.8.2.3-4) |  |

Table 4.8.2.1-7: Reference QoS rule #7

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |
| QoS rule |  |  | IMS_VOICE |
| QoS rule identifier | ‘0000 0010’B | 2 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-6 | Packet filter list #6 |  |
| QoS rule precedence | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0101’B | QFI 7 (Table 4.8.2.3-5) |  |
| QoS rule |  |  | IMS_VIDEO |
| QoS rule identifier | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-7 | Packet filter list #7 |  |
| QoS rule precedence | ‘0000 001’B | 2 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0110’B | QFI 8 (Table 4.8.2.3-6) |  |

| Condition | Explanation |
| --- | --- |
| IMS_VOICE | If this QoS rule is used to setup an IMS voice session |
| IMS_VIDEO | If this QoS rule is used to setup an IMS video session |

#### 4.8.2.2	Reference packet filters

Table 4.8.2.2-1: Packet filter list #1

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0000’B | Id 0 |  |
| Component type 1 ID | ‘0000 0001’B | Match-all type |  |

Table 4.8.2.2-2: Packet filter list #2

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0010’B | Id 2 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.2 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C002 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-3: Packet filter list #3

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0011’B | Id 3 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.3 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C003 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-3a: Packet filter list #3a

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘1111’B | Id 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.30 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C030 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-4: Packet filter list #4

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0100’B | Id 4 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.4 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C004 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-5: Packet filter list #5

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0101’B | Id 5 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.5 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C005 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-6: Packet filter list #6

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | 0110’B | Id 6 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS speech media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: According to to TS 26.114 [45] and RFC 4566 [46] a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 4.8.2.2-7: Packet filter list #7

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | 0111’B | Id 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS video media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: According to to TS 26.114 [45] and RFC 4566 [46] a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

#### 4.8.2.3	Reference QoS flow descriptions

Table 4.8.2.3-1: Reference QoS flow #1

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0001’B | QFI 1 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 1001’B | 5QI 9 |  |
| EPS bearer identity | Any not yet assigned value different to '5' |  | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-2: Reference QoS flow #2

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0010’B | QFI 2 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0101’B | 5QI 5 |  |
| EPS bearer identity | 5 | EBI 5 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-2a: Reference QoS flow #2a

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0100’B | QFI 4 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-3: Reference QoS flow #3

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0101’B | QFI 5 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-4: Reference QoS flow #4

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0110’B | QFI 6 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-5: Reference QoS flow #5

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0111’B | QFI 7 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | 5QI 1 | 5QI 1 |  |
| EPS bearer identity | ‘0000 011 ’B | EBI 6 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-6: Reference QoS flow #6

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1000’B | QFI 8 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0010’B | 5QI 2 |  |
| EPS bearer identity | ‘0000 0111 ’B | EBI 7 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message also including the Mapped EPS bearer context IE. |

### 4.8.3	Common test USIM parameters

This clause defines default parameters for programming the elementary files of the test UICC when running conformance test cases defined in 3GPP TS 38.523-1[12].

#### 4.8.3.1	General

See clause 4.9.1 in 3GPP TS 36.508 [2] for the definition of test algorithm for

-	authentication via EPC;

-	authentication via 5GC using 5G AKA based primary authentication and key agreement procedure.

-	authentication via 5GC using EAP-AKA' based primary authentication and key agreement procedure, further the Derivation of MSK, EMSK and other keys shaall be as derived as clause 3.3 of IETF RFC 5448 [31], using Key derivation function HMAC-SHA-256 algorithm.

#### 4.8.3.2	Default parameters for the test USIM and ISIM

Same as clause 4.9.2 in 3GPP TS 36.508 [2] for

-	authentication via EPC;

-	authentication via 5GC using 5G AKA based primary authentication and key agreement procedure.

-	authentication via 5GC using EAP-AKA' based primary authentication and key agreement procedure.

#### 4.8.3.3	Default settings for the Elementary Files (EFs)

Same as clause 4.9.3 in 3GPP TS 36.508 [2] for

-	authentication via EPC;

-	authentication via 5GC using 5G AKA based primary authentication and key agreement procedure

-	authentication via 5GC using EAP-AKA' based primary authentication and key agreement procedure..

##### 4.8.3.3.1	Modified contents of the USIM Elementary Files

Table 4.8.3.3.1-1: EFUST (USIM Service Table)

| Services |  | Activated | Version | Condition |
| --- | --- | --- | --- | --- |
| Service n°122 | 5GS Mobility Management Information | Optional |  | 5GC |
| Service n°123 | 5GS Security Parameters | Optional |  | 5GC |
| Service n°124 | Subscription identifier privacy support | Optional |  | 5GC |
| Service n°125 | SUCI calculation by the USIM | Optional |  | 5GC |
| Service n°126 | UAC Access Identities Configuration | Optional |  | 5GC |
| Service n°127 | Control plane-based steering of UE in VPLMN | Optional |  | 5GC |
| Service n°128 | Call control on PDU Session by USIM | Optional |  |  |
| Service n°129 | 5GS Operator PLMN List | Optional |  |  |
| Note: Only 5GS related services indicated |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| 5GC | Authentication via 5GC |

##### 4.8.3.3.2	Contents of Elementary Files at the DF5GS level

This clause defines the default contents of Elementary Files (EF) that are specific for 5GS and which are grouped in Data File (DF) structure 5GS.

EF5GS3GPPLOCI (5GS 3GPP location information)

File size:			20 Bytes

Default values:	Bytes 1 to 13 (HEX):	FF FF FF FF FF FF FF FF FF FF FF FF FF (5G-GUTI)

Bytes 14 to 19 (HEX):	42 F6 18 FF FF FE (Last visited registered TAI in 5GS for 3GPP access)

Byte 20 (BIN):			00000001 (5GS update status for 3GPP access = "5U2 not updated")

Bytes 14 to 19: TAI-MCC = 246 (bytes 14 to 15) and TAI-MNC = 81 (byte 16) are frequently used. The TAC (bytes 17 to 19) is set to "FF FF FE" since this, in conjunction with byte 20 setting of "01", is used to ensure that the UE performs registration at the beginning of a test.

Bytes in this file (e.g. GUTI in bytes 1 to 13) may be updated as a result of a registration attempt by the UE.

EF5GSN3GPPLOCI (5GS non-3GPP location information)

File size:			20 Bytes

Default values:	Bytes 1 to 13 (HEX):	FF FF FF FF FF FF FF FF FF FF FF FF FF (5G-GUTI)

Bytes 14 to 19 (HEX):	42 F6 18 FF FF FE (Last visited registered TAI in 5GS for non-3GPP access)

Byte 20 (BIN):			00000001 (5GS update status for non-3GPP access = "5U2 not updated")

Bytes 14 to 19: TAI-MCC = 246 (bytes 14 to 15) and TAI-MNC = 81 (byte 16) are frequently used. The TAC (bytes 17 to 19) is set to "FF FF FE" since this, in conjunction with byte 20 setting of "01", is used to ensure that the UE performs registration at the beginning of a test.

Bytes in this file (e.g. GUTI in bytes 1 to 13) may be updated as a result of a registration attempt by the UE.

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

The programming of this EF follows default parameter written in 3GPP TS 31.102 [33], annex E.

EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)

The programming of this EF follows default parameter written in 3GPP TS 31.102 [33], annex E.

EF5GAUTHKEYS (5G authentication keys)

The programming of this EF follows default parameter written in 3GPP TS 31.102 [33], annex E.

EFUAC_AIC (UAC Access Identities Configuration)

The programming of this EF is a test house option.

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

The programming of this EF is a test house option.

EFOPL5G (5GS Operator PLMN List)

The programming of this EF follows default parameter written in 3GPP TS 31.102 [33], annex E.

## 4.9	Test procedures

### 4.9.1	Test procedure to check user plane connectivity on DRB#n

This procedure aims at checking whether the UE User Plane Access Stratum is capable of exchanging data on DRB#n (#n is the DRB Id specified in the test case when the present procedure is called). In case the UE supports IP, it is also checked that the UE IP stack is connected to the UE User Plane Access Stratum.

Table 4.9.1-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Steps 1a1 to 1c2 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place depending on the UE implementation. | - | - | - | - |
| 1a1 | IF (pc_IP_Ping = TRUE AND pc_IPv4 = TRUE) THEN, the SS sends an ICMP Echo request to the IPv4 address assigned to the UE on DRB#n. | <-- | ICMP ECHO REQUEST (NOTE 3) | - | - |
| 1a2 | Check: Does the UE send an ICMP Echo reply on DRB#n? | --> | ICMP ECHO REPLY | - | P |
| 1b1 | ELSE IF (pc_IP_Ping = TRUE AND (pc_IPv4 = FALSE AND pc_IPv6 = TRUE)) THEN, the SS sends an ICMPv6 Echo request to the IPv6 address assigned to the UE on DRB#n. | <-- | ICMPv6 ECHO REQUEST (NOTE 3) | - | - |
| 1b2 | Check: Does the UE send an ICMPv6 Echo reply on DRB#n? | --> | ICMPv6 ECHO REPLY | - | P |
| 1c1 | ELSE, the SS transmits one IP Packet to verify data path on DRB#n. See NOTE 1, 2. | - | - | - | - |
| 1c2 | Check: Does UE send the IP Packet on DRB#n in the uplink? | - | - | - | P |
| NOTE 1: A Test Loop is assumed to already have been closed. NOTE 2: When DRB#n is a dedicated bearer, the IP Packet shall match the packet filters as configured for DRB#n. When DRB#n is a default bearer, the IP Packet shall match none of the dedicated bearers associated to DRB#n (if any). (NOTE 4) NOTE 3: When DRB#n is a dedicated bearer, the source address of the ICMP/ICMPv6 ECHO REQUEST shall be the same as the remote address of the DL/UL packet filters. When DRB#n is a default bearer, the source address of the ICMP/ICMPv6 ECHO REQUEST shall be different than the remote address of the DL/UL packet filters for an associated dedicated bearer (if any). (NOTE 4) NOTE 4: For 5GC QoS rules and the associated packet filters are specified in clause 4.8.2. For EPC the TFTs and associated packet filters are specified in clause 6.6.2 of TS 36.508 [2] and the IP packet shall be as according to clause 7.14.2 of TS 36.523-3 [41]. |  |  |  |  |  |

Table 4.9.1-1A: Test procedure sequence for Data path check for CA tests

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1 is only performed if SCell is not yet activated. | - | - | - | - |
| 1 | The SS transmits an Activation MAC control element to activate Scell. | <-- | MAC PDU (Activation (C1=1)) |  |  |
| - | EXCEPTION: Steps 2a1 to 2c2 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place depending on the UE implementation. | - | - | - | - |
| 2a1 | IF (pc_IP_Ping = TRUE AND pc_IPv4 = TRUE) THEN, the SS sends an ICMP Echo request to the IPv4 address assigned to the UE on DRB#n on the SCell. | <-- | ICMP ECHO REQUEST (NOTE 3) | - | - |
| 2a2 | Check: Does the UE send an ICMP Echo reply on DRB#n on the SpCell? | --> | ICMP ECHO REPLY | - | P |
| 2b1 | ELSE IF (pc_IP_Ping = TRUE AND (pc_IPv4 = FALSE AND pc_IPv6 = TRUE)) THEN, the SS sends an ICMPv6 Echo request to the IPv6 address assigned to the UE on DRB#n on the SCell. | <-- | ICMPv6 ECHO REQUEST (NOTE 3) | - | - |
| 2b2 | Check: Does the UE send an ICMPv6 Echo reply on DRB#n on the SpCell? | --> | ICMPv6 ECHO REPLY | - | P |
| 2c1 | ELSE, the SS transmits one IP Packet to verify data path on DRB#n on the SCell. See NOTE 1, 2. | - | - | - | - |
| 2c2 | Check: Does UE send the IP Packet on DRB#n in the uplink on the SpCell? | - | - | - | P |
| NOTE 1: A Test Loop is assumed to already have been closed. NOTE 2: When DRB#n is a dedicated bearer, the IP Packet shall match the packet filters as configured for DRB#n. When DRB#n is a default bearer, the IP Packet shall match none of the dedicated bearers associated to DRB#n (if any). (NOTE 4) NOTE 3: When DRB#n is a dedicated bearer, the source address of the ICMP/ICMPv6 ECHO REQUEST shall be the same as the remote address of the DL/UL packet filters. When DRB#n is a default bearer, the source address of the ICMP/ICMPv6 ECHO REQUEST shall be different than the remote address of the DL/UL packet filters for an associated dedicated bearer (if any). (NOTE 4) NOTE 4: For 5GC QoS rules and the associated packet filters are specified in clause 4.8.2. For EPC the TFTs and associated packet filters are specified in clause 6.6.2 of TS 36.508 [2] and the IP packet shall be as according to clause 7.14.2 of TS 36.523-3 [41]. |  |  |  |  |  |

### 4.9.2	Test procedure to activate UE Beamlock Test Function (UBF)

4.9.2.1	Initiation

UE is operating in FR2 in RRC_CONNECTED state.

4.9.2.2	Procedure

Table 4.9.2.2-1: Test procedure Sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS request UE to activate UE beamlock function. | <-- | ACTIVATE BEAMLOCK | - | - |
| 2 | UE confirms that UE beamlock function is activated | --> | ACTIVATE BEAMLOCK COMPLETE | - | - |

4.9.2.3	Specific Message contents

Table 4.9.2.3-1: ACTIVATE BEAMLOCK

| Derivation Path: 38.509 clause 6.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 1 0 0 0 0 0 |  |  |
| UE Beamlock test Function |  | 0 0 0 0 0 0 0 1 |  | Tx Only |
| UE Beamlock test Function |  | 0 0 0 0 0 0 1 0 |  | Rx Only |
| UE Beamlock test Function |  | 0 0 0 0 0 0 1 1 |  | Tx and Rx |

| Condition | Explanation |
| --- | --- |
| Tx Only | Activation UE beamlock function for Tx only |
| Rx Only | Activation UE beamlock function for Rx only |
| Tx and Rx | Activation UE beamlock function for both Tx and Rx |

Table 4.9.2.3-2: ACTIVATE BEAMLOCK COMPLETE

| Derivation Path: 38.509 clause 6.4.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 1 0 0 0 0 1 |  |  |

### 4.9.3	Test procedure to deactivate UE Beamlock Test Function (UBF)

4.9.3.1	Initiation

UE is operating in FR2 in RRC_CONNECTED state with UE beamlock test function activated.

4.9.3.2	Procedure

Table 4.9.3.2-1: Test procedure Sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS request UE to deactivate UE beamlock function. | <-- | DEACTIVATE BEAMLOCK | - | - |
| 2 | UE confirms that UE beamlock function is activated | --> | DEACTIVATE BEAMLOCK COMPLETE | - | - |

4.9.3.3	Specific Message contents

Table 4.9.3.3-1: DEACTIVATE BEAMLOCK

| Derivation Path: 38.509 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 1 0 0 0 1 0 |  |  |

Table 4.9.3.3-2: DEACTIVATE BEAMLOCK COMPLETE

| Derivation Path: 38.509 clause 6.4.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 1 0 0 0 1 1 |  |  |

### 4.9.4	Test procedure to check that UE is in state 5GC RRC_IDLE on a certain NR/NGC cell

4.9.4.1	Scope

This procedure aims at checking whether the UE is in state 5GC RRC_IDLE on a certain cell (as specified in the test case).

4.9.4.2	Procedure description

4.9.4.2.1	Initial conditions

As specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.4.2.2	Procedure

Table 4.9.4.2.2-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | Step 1 of Generic procedure for bringing the UE in RRC_CONNECTED state with connectivity NR as specified in Table 4.5.4.2-3 is performed. | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCSetupRequest message on the cell specified in the test case? | --> | NR RRC: RRCSetupRequest | - | P |
| 3-8 | Steps 3-8 of Generic procedure for bringing the UE in RRC_CONNECTED state with connectivity NR as specified in Table 4.5.4.2-3 are performed. | - | - | - | - |
| - | EXCEPTION: Step 9a1 describes a step sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 9a1 | IF 'connected without release' is not present THEN the SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |

4.9.4.2.3	Specific Message content

As specified in the TC which calls the procedure in its entirety or refers to parts of it.

### 4.9.5	Test procedure to check that UE is camped on a new NR/NGC cell belonging to a new TA

4.9.5.1	Scope

This procedure aims at checking whether the UE performs a mobility registration updating (Tracking Area (TA) update) procedure when it camps on a new cell (as specified in the test case) belonging to a new TA.

4.9.5.2	Procedure description

4.9.5.2.1	Initial conditions

As specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.5.2.2	Procedure sequence

Table 4.9.5.2.2-1: Test procedure sequence mobility registration updating (TA update)

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 1 | The UE transmits an RRCConnectionRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 2 | SS transmit an RRCConnectionSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 3 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 4 | SS sends a REGISTRATION ACCEPT message containing a 5G-GUTI. (NOTE 1, NOTE 2) | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 5 | Check: Does the UE send a REGISTRATION COMPLETE? | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | P |
| - | EXCEPTION: Step 6a1 describes a step sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 6a1 | IF 'connected without release' is not present THEN the SS transmits an RRCConnectionRelease message to release RRC connection and move the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| NOTE 1 If a PDU session status IE was included in the REGISTRATION REQUEST message then the SS includes a PDU session status IE in the REGISTRATION ACCEPT message indicating that all the PDU sessions are active. NOTE 2: If the UE has indicated S1 mode supported then the SS shall indicate in the 5GS network feature support IE in the REGISTRATION ACCEPT message the IWK N26 bit set to "interworking without N26 not supported". The setting of the "interworking without N26 not supported" has been chosen to ensure that the UE is operating in the single-registration mode allowing for a clearly pre-determined UE behaviour. |  |  |  |  |  |

4.9.5.2.3	Specific Message content

As specified in the TC which calls the procedure in its entirety or refers to parts of it.

### 4.9.6	Test procedures for Switch off / Power off UE

#### 4.9.6.1	Switch off / Power off procedure in RRC_IDLE

Table 4.9.6.1-1: Switch off procedure in RRC_IDLE

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) | - | - |
| 1a2 | UE transmits an RRCSetupRequest message. | --> | RRC: RRCSetupRequest |
| 1a3 | SS transmit an RRCSetup message. | <-- | RRC: RRCSetup |
| - | EXCEPTION: Steps 1a4Aa1 to 1a4Aa6 specify optional behaviour if the UE has previously performed IMS registration | - | - |
| 1a4Aa1 | The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. | --> | RRC: RRCSetupComplete 5GMM: SERVICE REQUEST |
| 1a4Aa2 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 1a4Aa3 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 1a4Aa4 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT |
| - | EXCEPTION: In parallel to the event described in step 1a4Aa5 below, the generic test procedure in TS 34.229-1 [44] Annex C.30 may be performed. Editor’s note: The reference needs to be changed to TS 34.229-5 when corresponding procedure is added in TS 34.229-5. | - | - |
| 1a4Aa5 | The UE transmits an RRCReconfigurationComplete message. | --> | RRC: RRCReconfigurationComplete |
| 1a4Aa6 | The UE transmits a DEREGISTRATION REQUEST message | - | 5GMM: DEREGISTRATION REQUEST |
| - | EXCEPTION: Step 1a4Ab1 below specifies the behaviour if the UE has not previously performed IMS registration | - | - |
| 1a4Ab1 | The UE transmits an RRCSetupComplete message including the DEREGISTRATION REQUEST message. | --> | RRC: RRCSetupComplete 5GMM: DEREGISTRATION REQUEST |
| 1a4 | Void | - | - |
| 1a5 | The SS transmits an RRCRelease message | <-- | RRC: RRCRelease |
| 1b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |

#### 4.9.6.2	Switch off / Power off procedure in RRC_INACTIVE

4.9.6.2.1	Procedure

Table 4.9.6.2.1-1: Switch off procedure in RRC_INACTIVE

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) | - | - |
| 1a2 | UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest |
| 1a3 | SS transmit an RRCResume message. | <-- | NR RRC: RRCResume |
| - | EXCEPTION: Steps 1a4Aa1 to 1a4Aa6 specify optional behaviour if the UE has previously performed IMS registration | - | - |
| 1a4Aa1 | The UE transmits an RRCResumeComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. | --> | RRC: RRCResumeComplete 5GMM: SERVICE REQUEST |
| 1a4Aa2 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 1a4Aa3 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 1a4Aa4 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT |
| - | EXCEPTION: In parallel to the event described in step 1a4Aa5 below, the generic test procedure in TS 34.229-1 [44] Annex C.30 may be performed. Editor’s note: The reference needs to be changed to TS 34.229-5 when corresponding procedure is added in TS 34.229-5. | - | - |
| 1a4Aa5 | The UE transmits an RRCReconfigurationComplete message. | --> | RRC: RRCReconfigurationComplete |
| 1a4Aa6 | The UE transmits a DEREGISTRATION REQUEST message | - | 5GMM: DEREGISTRATION REQUEST |
| - | EXCEPTION: Step 1a4Ab1 below specifies the behaviour if the UE has not previously performed IMS registration | - | - |
| 1a4 | Void | - | - |
| 1a5 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease |
| 1b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |

4.9.6.2.2	Specific Message contents

Table 4.9.6.2.1-1: RRCResumeRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |  |
| resumeIdentity |  | Not checked |  |  |
| resumeMAC-I |  | Not checked |  |  |
| resumeCause |  | Not checked |  |  |
| spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 4.9.6.3	Switch off / Power off procedure in RRC_CONNECTED

Table 4.9.6.3-1: Switch off procedure in RRC_CONNECTED

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1a1-1a3 | Void | - | - |
| 2-4 | Void | - | - |
| - | EXCEPTION: Steps 5a1 to 5b1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if a particular implementation is under test. | - | - |
| 5a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) |  |  |
| - | EXCEPTION : Step 5a1Aa1 to 5a1Aa2 below specifies optional behaviour if the UE has previously performed IMS registration | - | - |
| 5a1Aa1 | The UE may perform the generic test procedure described in TS 34.229-1 [44] Annex C.30 Editor’s note: The reference needs to be changed to TS 34.229-5 when corresponding procedure is added in TS 34.229-5. | - | - |
| 5a2 | The UE transmits a DEREGISTRATION REQUEST message. | --> | 5GMM: DEREGISTRATION REQUEST |
| 5a3 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease |
| 5b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |

#### 4.9.6.3A	Switch off / Power off procedure in RRC_CONNECTED with T3540 started

Table 4.9.6.3A-1: Switch off procedure in RRC_CONNECTED with T3540 started

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS starts timer1 = T3540 (10 sec). | - | - |
| 2 | The SS locally releases the RRC connection. | - | - |
| 3 | SS waits for Timer1 expires. NOTE: On expiry of T3540 UE shall locally release the established N1 NAS signalling connection | - | - |
| 4 | The generic procedure as described in Table 4.9.6.1-1: Switch off procedure in RRC_IDLE take place. | - | - |

#### 4.9.6.4	Switch off / Power off procedure in State DEREGISTERED

Table 4.9.6.4-1: Switch off procedure in State DEREGISTERED

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE (Note 1) | - | - |
| 1b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal. |  |  |  |

#### 4.9.6.5	Switch off / Power off procedure in WLAN Ipsec_SA_Established

Table 4.9.6.5-1: Switch off procedure in Ipsec_SA_Established

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [30] pc_SwitchOnOff or [31] pc_USIM_Removal is supported | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE (Note 1) | - | - |
| 1a2 | The UE transmits a DEREGISTRATION REQUEST message. | --> | 5GMM: DEREGISTRATION REQUEST |
| 1a3 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5, takes place performing disconnection of security association | - | - |
| 1b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |

### 4.9.7	Test procedure for UE for Tracking area updating / Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode

4.9.7.1	Scope

This procedure aims at verifying that the UE performs a Tracking Area Update (TAU) procedure when it performs inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE.

The procedure provides different security context handling options based on the condition parameters defined in Table 4.9.7.1-1.

Table 4.9.7.1-1: Condition parameters

| Condition | Explanation |
| --- | --- |
| new security context | When this parameter is present the SS will establish and take into account a new security context. |
| existing EPS security context | When this parameter is present the SS will take into account an existing EPS security context. A prerequisite for using this condition is the existence of an EPS security context |
| NOTE 1: If none of the defined condition parameters is present when the procedure is referred to then the SS will apply mapped 5G security context. A prerequisite for using this condition is the existence of 5G security context. |  |

4.9.7.2	Procedure description

4.9.7.2.1	Initial conditions

System Simulator:

-	1 E-UTRA cell connected to EPC, default parameters, system information combination 31 as defined in TS 36.508 [2], subclause 4.4.3.1.1.

NOTE:	Details about the NGC cell from which the UE will move to the E-UTRA cell are to be specified in the test.

User Equipment:

-	The Test UICC shall be inserted. It shall provide relevant details on the EPC and 5GC.

All details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.7.2.2	Procedure sequence

Table 4.9.7.2.2-1: Test procedure sequence UE Tracking area updating / inter-system change from N1 mode to S1 mode in EMM-IDLE mode

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 1 | The UE transmits an RRCConnectionRequest message on the cell specified in the test case. | --> | RRC: RRCConnectionRequest | - | - |
| 2 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 3 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | - | - |
| - | EXCEPTION: Steps 4a1-4b2 describe a step sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 4a1 | IF 'new security context' THEN the SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | RRC: DLInformationTransfer NAS: AUTHENTICATION REQUEST | - | - |
| 4a2 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | RRC: ULInformationTransfer NAS: AUTHENTICATION RESPONSE | - | - |
| 4a3 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND | - | - |
| 4a4 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE | - | - |
| 4b1 | IF 'existing EPS security context' THEN the SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND | - | - |
| 4b2 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE | - | - |
| - | EXCEPTION: If none of the branches 4a or 4b takes place then the SS shall apply mapped 5G security context, otherwise the SS shall apply the security context depending on the branch. | - | - | - | - |
| 5 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 6 | Check: Does the UE transmit TRACKING AREA UPDATE COMPLETE? | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | P |
| 7a1 | Void | - | - | - | - |
| - | EXCEPTION: Steps 8a1 to 8b2a8 describe a step sequence depending on test case scenario; the left-most "lower case letter" identifies a step sequence that take place if the test procedure is called in a particular scenario. (NOTE 1) | - | - | - | - |
| 8a1 | IF Interworking without N26 interface supported THEN The generic procedure for UE-requested PDN connection establishment, specified in subclause 4.5A.2B, takes place performing establishment of UE-requested PDN connection(s) with ExpectedNumberOfNewPDNConnections = pc_noOf_PDNsSameConnection. | - | - | - | - |
| - | EXCEPTION: Steps 8a2a1 to 8a2b1 describe a step sequence depending on test case scenario; the right-most "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 8a2a1 | IF pc_noOf_PDNsNewConnection>0 THEN the SS transmits an RRCConnectionRelease message to release RRC connection and moves the UE to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| 8a2a2 | The procedure E-UTRA RRC_IDLE Unrestricted nr PDN Extension as specified in Table 4.5.2.2-6 takes place. | - | - | - | - |
| 8a2b1 | ELSE IF connected without release is not present THEN, the SS transmits an RRCConnectionRelease message to release RRC connection and move to E-UTRA RRC_IDLE (State 2). | <-- | RRC: RRCConnectionRelease | - | - |
| 8b1 | ELSE (i.e. 'Interworking without N26 interface not supported') The generic procedure for UE-requested PDN connection establishment, specified in subclause 4.5A.2B, takes place performing establishment of UE-requested PDN connection(s) with ExpectedNumberOfNewPDNConnections = pc_noOf_PDNsSameConnection with the exception that IF step 2b1, Table 4.5A.2B.2.2-2 takes place THEN the SS shall not assign Fail but continue with the step below (NOTE 2). | - | - | - | - |
| - | EXCEPTION: Steps 8b2a1 to 8b2b1 describe a step sequence depending on test case scenario; the right-most "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 8b2a1 | IF pc_noOf_PDNsNewConnection>0 THEN the SS transmits an RRCConnectionRelease message to release RRC connection and moves the UE to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| 8b2a2 | The procedure E-UTRA RRC_IDLE Unrestricted nr PDN Extension as specified in table 4.5.2.2-6 takes place. For the referred in step 7, Table 4.5.2.2-6, generic procedure for UE-requested PDN connection establishment, specified in subclause 4.5A.2B, IF step 2b1, Table 4.5A.2B.2.2-2 takes place THEN the SS shall not assign Fail but continue with the next step (NOTE 2). | - | - | - | - |
| 8b2b1 | ELSE IF connected without release is not present THEN, the SS transmits an RRCConnectionRelease message to release RRC connection and move to E-UTRA RRC_IDLE (State 2). | <-- | RRC: RRCConnectionRelease | - | - |
| NOTE 1: The NWK will indicated whether Interworking without N26 interface is supported in the REGISTRATION ACCEPT message, IE '5GS network feature support', IWK N26 bit. Consequently which branch would the procedure sequence go through will depend on the content of the REGISTRATION ACCEPT message applicable to e.g. the test case which calls the present test procedure. NOTE 2: Depending on UE implementation and/or NWK behaviour, the UE may transfer some PDU sessions into PDN connections without re-establishing those utilising the relevant mapped QoS provided in the PDU SESSION ESTABLISHMENT ACCEPT message. This will result in the number of established PDNs, if any, being lower than the pc_noOf_PDNsSameConnection or the pc_noOf_PDNsNewConnection which the UE will establish upon initial attach to the EPS. |  |  |  |  |  |

4.9.7.2.3	Specific Message content

Default message contents as specified in TS 36.508 [2] with the following exceptions.

Table 4.9.7.2.3-1: TRACKING AREA UPDATE REQUEST (Step 3, Table 4.9.7.2.2-1)

| Derivation Path: TS 36.508 [2], Table 4.7.2-27, condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '000'B or '001'B or '010'B or '100'B or '101'B | Any Allowed Value except periodic updating |  |
| "Active" flag | '0'B | No bearer establishment requested |  |
| NAS key set identifier | the eKSI indicating the 5G NAS security context value assigned at the initial registration when the UE entered N1 |  |  |
| Old GUTI | GUTI, mapped from the 5G-GUTI assigned at the initial registration when the UE entered N1 |  |  |
| Additional GUTI | Not present or any allowed value |  |  |
| Last visited registered TAI | The TAI the last visited E-UTRA Cell belonged to, if any. Not included if the UE does not have last stored EPC TAI. |  |  |
| UE radio capability information update needed | '1'B | UE radio capability information update needed | First-N1-to-S1 |
| EPS bearer context status | Present, Content not checked | EBI corresponding to active PDU Sessions need to be set to 1 |  |
| Old GUTI type | "Native GUTI" |  |  |
| UE status | "UE is in 5GMM-REGISTERED state" |  |  |
| NOTE: The message shall be integrity protected using the 5GS security context available in the UE. |  |  |  |

| Condition | Explanation |
| --- | --- |
| First-N1-to-S1 | First N1 to S1 transition following UE registration in N1 mode |

Table 4.9.7.2.3-2: AUTHENTICATION REQUEST (Step 4a1, Table 4.9.7.2.2-1)

| Derivation Path: TS 36.508 [2], Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifierASME |  |  |  |
| NAS key set identifier | An arbitrarily selected value between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. | Value shall be different to the 5G NAS security context value if there is one assigned |  |

Table 4.9.7.2.3-3: SECURITY MODE COMMAND (Step 4a3, Table 4.9.7.2.2-1)

| Derivation Path: TS 36.508 [2], Table 4.7.2-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifierASME |  |  |  |
| NAS key set identifier | The 4G NAS key set identifier assigned in step 4a1. |  |  |

Table 4.9.7.2.3-4: SECURITY MODE COMMAND (Step 4b1, Table 4.9.7.2.2-1)

| Derivation Path: TS 36.508 [2], Table 4.7.2-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifierASME |  |  |  |
| NAS key set identifier | The 4G NAS key set identifier assigned in the latest Authentication procedure. |  |  |

Table 4.9.7.2.3-5: TRACKING AREA UPDATE ACCEPT (Step 5, Table 4.9.7.2.2-1)

| Derivation Path: TS 36.508 [2], Table 4.7.2-24, condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS network feature support | The IWK N26 (octet 4, bit 7) set to '1' |  | Interworking without N26 interface supported |

### 4.9.8	Procedure for Registration Reject

4.9.8.1	Scope

The purpose of this procedure is to reject the registration request, with a specific cause value, which may allow fields to be cleared in the USIM.

4.9.8.2	Procedure description

4.9.8.2.1	Initial conditions

As specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.8.2.2	Procedure sequence

Table 4.9.8.2.2-1: Procedure for Registration Reject

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest |
| 2 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup |
| 3 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST |
| 4 | The SS transmits an AUTHENTICATION REQUEST message including EAP-Request/AKA'-Challenge or 5G AKA Challenge. | <-- | 5GMM: AUTHENTICATION REQUEST |
| 5 | The UE transmits an AUTHENTICATION RESPONSE message including EAP-Response/AKA'-Challenge or 5G AKA Response. | --> | 5GMM: AUTHENTICATION RESPONSE |
| 6 | The SS transmits a SECURITY MODE COMMAND message including EAP-Success if EAP-AKA' used. | <-- | 5GMM: SECURITY MODE COMMAND |
| 7 | The UE transmits a SECURITY MODE COMPLETE message. | --> | 5GMM: SECURITY MODE COMPLETE |
| 8 | The SS transmits a REGISTRATION REJECT message with the cause value set to Reject Cause. | <-- | 5GMM: REGISTRATION REJECT |
| 9 | The SS transmits an RRCRelease message | <-- | RRC: RRCRelease |
| 10 | Test procedure for Switch off / Power off in State DEREGISTERED as specified in subclause 4.9.6.4 |  |  |

4.9.8.2.3	Specific message contents

Table 4.9.8.2.3-1: REGISTRATION REJECT

| Derivation Path: table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GMM cause |  | Set according to Reject Cause | Reject Cause set to #6 Illegal ME as default |  |

### 4.9.9	Test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode

4.9.9.1	Scope

This procedure aims at verifying that the UE performs a Mobility and periodic registration update procedure when it performs inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE.

4.9.9.2	Procedure description

4.9.9.2.1	Initial conditions

System Simulator:

-	1 NGC Cell connected to 5GC, default parameters, system information combination NR-6 as defined in subclause 4.4.3.1.2.

NOTE:	Details about the E-UTRA cell from which the UE will move to the NGC cell are to be specified in the test.

User Equipment:

-	The Test UICC shall be inserted. It shall provide relevant details on the EPC and 5GC.

All details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.9.2.2	Procedure sequence

Table 4.9.9.2.2-1: Test procedure sequence UE Tracking area updating / inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 1-3 | Steps 1-3 from the mobility and periodic registration update procedure as described in Table 4.9.5.2.2-1 are performed. | - | - | - | - |
| 4 | The SS transmits a DLInformationTransfer message and an AUTHENTICATION REQUEST message. | <-- | NR RRC: DLInformationTransfer 5GMM: AUTHENTICATION REQUEST | - | - |
| 5 | The UE transmits an ULInformationTransfer message and an AUTHENTICATION RESPONSE message. | --> | NR RRC: ULInformationTransfer 5GMM: AUTHENTICATION RESPONSE | - | - |
| 6 | The SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 7 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | - |
| 8 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 9 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 10-11 | Steps 4-5 of Table 4.9.5.2.2-1 of the generic procedure are performed. | - | - | - | - |
| 12a1-13a1 | Void. | - | - | - | - |
| - | EXCEPTION: Steps 14a1 to 14b2b1 describe a step sequence depending on test case scenario; the "lower case letter" identifies a step sequence that take place if the test procedure is called in a particular scenario. (NOTE 1) | - | - | - | - |
| 14a1 | IF Interworking without N26 interface supported THEN The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. | - | - | - | - |
| - | EXCEPTION: Steps 14a2a1 to 14a2b1 describe a step sequence depending on test case scenario; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 14a2a1 | IF pc_noOf_PDUsNewConnection > 0 THEN the SS transmits an RRCRelease message to release RRC connection and moves the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| 14a2a2 | The procedure NR RRC_IDLE Extension as specified in table 4.5.2.2-4 takes place. | - | - | - | - |
| 14a2b1 | ELSE IF connected without release is not present THEN, the SS transmits an RRCConnectionRelease message to release RRC connection and move the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| - | EXCEPTION: In parallel to the events described in step 14b1 below, the steps specified in Table 4.9.9.2.2-2 may take place. | - | - | - | - |
| 14b1 | ELSE (i.e. 'Interworking without N26 interface not supported') The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection with the exception that IF step 2b1, Table 4.5A.2.2.2-2 takes place THEN the SS shall not assign Fail but continue with the step below (NOTE 2). | - | - | - | - |
| - | EXCEPTION: Steps 14b2a1 to 14b2b1 describe a step sequence depending on test case scenario; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 14b2a1 | IF pc_noOf_PDUsNewConnection> 0 THEN the SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| 14b23a2 | The procedure NR RRC_IDLE Extension as specified in Table 4.5.2.2-4 takes place. For the referred in step 8, Table 4.5.2.2-4, generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, IF step 2b1, Table 4.5A.2.2.2-2 takes place THEN the SS shall not assign Fail but continue with the next step (NOTE 2). | - | - | - | - |
| 14b2b1 | ELSE IF connected without release is not present THEN, the SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| NOTE 1: The NWK will indicated whether Interworking without N26 interface is supported in the REGISTRATION ACCEPT message, IE '5GS network feature support', IWK N26 bit. Consequently which branch would the procedure sequence go through will depend on the content of the REGISTRATION ACCEPT message applicable to e.g. the test case which calls the present test procedure. NOTE 2: Depending on UE implementation and/or NWK behaviour, the UE may transfer some PDN connections into PDU sessions without re-establishing them with the relevant mapping provided in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. This will result in the number of connection modifications, if any, being lower than the pc_noOf_PDUsSameConnection or the pc_noOf_PDUsNewConnection which the UE will establish upon initial attach to the 5GS. |  |  |  |  |  |

Table 4.9.9.2.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1 describe a step sequence depending on test case scenario; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. NOTE 2, NOTE 3 | - | - | - | - |
| 1 | IF this is the first time in a test case that the UE moves from S1 to N1 THEN the generic procedure for Procedure for UE-requested PDU session modification after the first S1 to N1 mode change / Single-registration mode with N26, specified in subclause 4.5A.2C, takes place with ExpectedNumberOfPDUSessionModifications=(pc_noOf_PDUsSameConnection+pc_noOf_PDUsNewConnection), with the exception that IF step 2b1, Table 4.5A.2C.2.2-2 takes place THEN the SS shall not assign Fail but continue with the step below (NOTE 1). |  |  |  |  |
| NOTE 1: Depending on UE implementation and/or NWK behaviour, the UE may transfer with modification some PDN connections into PDU sessions without re-establishing them with the relevant mapping provided in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. This will result in the number of session modifications, if any, being lower than the pc_noOf_PDUsSameConnection+pc_noOf_PDUsNewConnection. NOTE 2: Whether this is the first time in a test case that the UE moves from S1 to N1 depends on the test scenario (including what happens in the preamble of the test). NOTE 3: It is assumed that the PDU session modification for all transferred PDUs will happen on the same connection with the mobility and periodic registration update procedure. NOTE 4: For PDN connections which will be transferred, tests calling the present procedure shall ensure that: - For each PDN connection established during the UE registration to the EPS, the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message which corresponds to the default EPS bearer of the PDN connectivity being activated, contains the Protocol configuration options IE or the Extended protocol configuration options IE with mapped 5GS PDU a session-AMBR and QoS rule(s). |  |  |  |  |  |

4.9.9.2.3	Specific Message content

Table 4.9.9.2.3-1: REGISTRATION REQUEST (step 1, Table 4.9.9.2.2-1; step 3, Table 4.9.5.2.2-1)

| Derivation Path: Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type | '00xxx010' | mobility registration updating x - not checked |  |
| ngKSI |  |  |  |
| NAS key set identifier | KSIAMF that was created when the UE last registered to 5GCN |  | Registered_Previously_on_5GCN |
|  | ‘111’B | No key | Not_Registered_Previously_on_5GCN |
| TSC | '0'B | native security context (for KSIAMF) | Registered_Previously_on_5GCN |
|  | Not applicable | TSC does not apply for NAS key set identifier value "111" | Not_Registered_Previously_on_5GCN |
| 5GS mobile identity | 5G-GUTI mapped from the 4G-GUTI assigned when the UE last registered to EPC E-UTRA |  |  |
| Non-current native NAS key set identifier | Not present |  |  |
| 5GMM capability | '0000 0xx1' | S1 mode supported x - not checked |  |
| Last visited registered TAI | The TAI the last visited NGC Cell belonged to, if any. Not included if the UE does not have last stored 5GC TAI. |  |  |
| S1 UE network capability |  |  |  |
| All octets with the exception of octet 8, bit 8 and octet 9, bit 6 | Not checked |  |  |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) | '1' | Extended protocol configuration options IE supported |  |
| N1 mode supported (N1mode) (octet 9, bit 6) | '1' | N1 mode supported |  |
| PDU session status | Any allowed value |  |  |
| UE status | "UE is in EMM-REGISTERED state" |  |  |
| Additional GUTI | 5G-GUTI assigned when the UE last registered to 5GC N |  | Registered_Previously_on_5GCN |
|  | Not present |  | Not_Registered_Previously_on_5GCN |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 4.9.9.2.3-2 |  |

| Condition | Explanation |
| --- | --- |
| Not_Registered_Previously_on_5GCN | UE has not_registered_previously_on_5GCN. UE does not have valid 5G NAS security context and 5G-GUTI. |
| Registered_Previously_on_5GCN | UE has registered_previously_on_5GCN. UE have valid 5G NAS security context and 5G-GUTI |

Table 4.9.9.2.3-2: TRACKING AREA UPDATE REQUEST (Table 4.9.9.2.3-1)

| Derivation Path: TS 36.508 [2], Table 4.7.2-27. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '000'B | TA updating |  |
| "Active" flag | '0'B | No Bearer Establishment requested |  |
| NAS key set identifier | the eKSI for the current EPS security context |  |  |
| TSC | '0'B |  |  |
|  | '1'B |  | Mapped EPS security context |
| Old GUTI | 4G-GUTI assigned when the UE last registered to EPC E-UTRA |  |  |
| UE network capability | Not present |  |  |
| Last visited registered TAI | Not present |  |  |
| DRX parameter | Not present |  |  |
| UE radio capability information update needed | Not present |  |  |
| EPS bearer context status | Not present |  |  |
| MS network capability | Not present |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | Not present |  |  |
| Mobile station classmark 2 | Not present |  |  |
| Mobile station classmark 3 | Not present |  |  |
| Supported Codecs | Not present |  |  |
| Additional update type | Not present |  |  |
| Voice domain preference and UE's usage setting | Not present |  |  |
| Old GUTI type | Not present |  |  |
| Device properties | Not present |  |  |
| MS network feature support | Not present |  |  |
| TMSI based NRI container | Not present |  |  |
| T3324 value | Not present |  |  |
| T3412 extended value | Not present |  |  |
| Extended DRX parameters | Not present |  |  |
| UE additional security capability | Not present |  |  |
| UE status | Not present |  |  |
| Additional information requested | Not present |  |  |
| NOTE: The message shall be integrity protected using the current EPS security context. |  |  |  |

| Condition | Explanation |
| --- | --- |
| Mapped EPS security context | When explicitly specified by the test case in which the message is used. |

Editor's note: Step12a2 has been deleted, so this table is obsolete?

Table 4.9.9.2.3-4: SERVICE REQUEST (step 12a2, Table 4.9.9.2.2-1)

| Derivation Path: Table 4.7.1-16. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  | ‘0000’B | signalling |  |

Table 4.9.9.2.3-3: REGISTRATION ACCEPT (step 10, Table 4.9.9.2.2-1; step 4, Table 4.9.5.2.2-1)

| Derivation Path: Table 4.7.1-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS network feature support | The IWK N26 (octet 4, bit 7) set to '1' |  | Interworking without N26 interface supported |

### 4.9.10	Test procedure to check that the UE is in RRC_CONNECTED state

4.9.10.1	Scope

This procedure aims at checking whether the UE is in the RRC_CONNECTED state.

4.9.10.2	Procedure description

4.9.10.2.1	Initial conditions

As specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.10.2.2	Procedure

Table 4.9.10.2.2-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The SS sends UECapabilityEnquiry message to the UE. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 2 | Check: Does the UE send a UECapabilityInformation message? | --> | NR RRC: UECapabilityInformation |  | P |

4.9.10.2.3	Specific Message content

None.

### 4.9.11	Test Procedure for IMS Emergency call establishment in 5GC with IMS emergency registration

4.9.11.1	Scope

This procedure aims at verifying the UE establishment of IMS Emergency call in 5GC when the UE is in 5GMM-IDLE and when IMS emergency registration is required e.g. under Normal Service conditions.

4.9.11.2	Procedure description

4.9.11.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The Test UICC shall be inserted. It shall provide Emergency Numbers.

The procedure assumes that the UE is in test state 1N-A, subclause 4.4A.2 on the NR Cell. All necessary details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.11.2.2	Procedure sequence

Table 4.9.11.2.2-1: Test procedure sequence UE IMS Emergency call establishment in 5GC with IMS emergency registration

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 1 | Check: Does the UE transmit an RRCSetupRequest message with 'establishmentCause' set to 'emergency'? | --> | NR RRC: RRCSetupRequest | - | P |
| 2 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 3 | Check: Does the UE transmit an RRCSetupComplete message and a SERVICE REQUEST message with 'Service type' set to 'emergency services'? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | P |
| 4 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 5 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 6 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 7 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 8 | Check: Does the UE transmit an UL NAS TRANSPORT message with 'Request type' set to 'initial emergency request', and, a PDU SESSION ESTABLISHMENT REQUEST? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | P |
| - | EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in steps 1a1 to 2 of Table 4.9.11.2.2-2 take place. | - | - | - | - |
| 9 | The SS transmits an RRCReconfiguration message and an PDU SESSION ESTABLISHMENT ACCEPT | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to the events described in steps 11-13 below the events specified in steps 3 of Table 4.9.11.2.2-2 take place. | - | - | - | - |
| 11 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 12 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 13 | Check: Does the UE transmits a ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |

Table 4.9.11.2.2-2: IMS signalling and Emergency call establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 1a1 | The generic procedure for IP address allocation in the user plane specified in subclause 4.5A.3 takes place. | - | - | - | - |
| 2 | Generic Test Procedure for IMS Emergency registration / 5GS as defined in TS 34.229-5 [47], annex A.3 is performed. | - | - | - | - |
| 3 | Generic test procedure for setting up IMS Emergency Voice Call - 5G as defined in TS 34.229-51 [47], annex A.6 is performed. | - | - | - | - |

4.9.11.2.3	Specific Message content

All specific message contents shall be according subclause 4.6 and 4.7 with the exceptions below.

Table 4.9.12.2.3-1: SIB1 (at any time prior and during the procedure, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.6.1-28. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| ims-EmergencySupport |  | Present | true |  |
| } |  |  |  |  |

Table 4.9.11.2.3-2: RRCSetupRequest (step 1, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.6.1-23. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| establishmentCause |  | emergency |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.11.2.3-3: SERVICE REQUEST (step 3, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.1-16. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  | ‘0011’B | emergency services |  |

Table 4.9.11.2.3-4: UL NAS TRANSPORT (step 8, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.1-10, condition INITIAL_PDU_REQUEST. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | ‘011’B | initial emergency request |  |
| S-NSSAI |  | Not Present |  |  |
| DNN |  | Not Present |  |  |

Table 4.9.11.2.3-5: PDU SESSION ESTABLISHMENT REQUEST (step 8, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | A value that is not currently being used by another PDU session |  |  |
| PTI |  | A value currently not used |  |  |
| SSC mode |  | ‘001’B | SSC mode 1 |  |

Table 4.9.11.2.3-6: DL NAS TRANSPORT (step 9, Table 4.9.11.2.2-1)

Derivation Path: Table 4.7.1-11, condition 5GSM_MESSAGE.

Table 4.9.11.2.3-7: PDU SESSION ESTABLISHMENT ACCEPT (step 9, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Selected SSC mode |  | ‘001’B | SSC mode 1 |  |
| Authorized QoS rules |  | Reference QoS rule #2 as defined in Table 4.8.2.1-2. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #2 as defined in Table 4.8.2.3-2. |  |  |

Table 4.9.11.2.3-8: RRCReconfiguration (step 9, Table 4.9.11.2.2-1)

Derivation Path: Table 4.8.1-1: RRCReconfiguration-DRB (1, 0)

Table 4.9.11.2.3-9: PDU SESSION MODIFICATION COMMAND (step 11, Table 4.9.11.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  | Reference QoS rule #7 as defined in Table 4.8.2.1-7 using condition IMS_VOICE. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #5 as defined in Table 4.8.2.3-5. |  |  |

Table 4.9.11.2.3-10: RRCReconfiguration (step 11, Table 4.9.11.2.2-1)

Derivation Path: 4.8.1-1C RRCReconfiguration-Speech

### 4.9.12	Generic Test Procedure for IMS Emergency call establishment in 5GC without IMS emergency registration

4.9.12.1	Scope

This procedure aims at verifying the UE establishment of IMS Emergency call in 5GC without the need for IMS emergency registration to take place beforehand e.g. under Limited Service or SIM/USIM not available, the SIM/USIM is considered invalid by the UE conditions.

4.9.12.2	Procedure description

4.9.12.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters unless specified otherwise. PLMN/TAI which the cell belongs to, shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it

User Equipment:

-	Whether the Test UICC shall be inserted or not, and, its settings e.g. in regard to the provision of Emergency Numbers, shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.12.2.2	Procedure sequence

Table 4.9.12.2.2-1: Test procedure sequence UE IMS Emergency call establishment in 5GC without IMS emergency registration

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 1 | Check: Does the UE transmits an RRCSetupRequest message with 'establishmentCause' set to 'emergency'? | --> | NR RRC: RRCSetupRequest | - | P |
| 2 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 3 | Check: Does the UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message with 'Service type' set to 'emergency services'? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | P |
| 4 | The SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message with 'Selected NAS security algorithms' set to "null integrity protection algorithm" (5G-IA0), "null ciphering algorithm" (5G-EA0). | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 5 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | P |
| 6 | The SS transmits a SecurityModeCommand message with cipheringAlgorithm set to 'NULL' ciphering algorithm (nea0) and integrityProtAlgorithm set to 'NULL' integrity protection algorithm (nia0). | <-- | NR RRC: SecurityModeCommand | - | - |
| 7 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | P |
| 8 | The SS transmits a UECapabilityEnquiry message. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 9 | The UE transmits a UECapabilityInformation message. | --> | NR RRC: UECapabilityInformation | - | - |
| 10 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 11 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| 12 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 13 | Check: Does the UE transmits an UL NAS TRANSPORT message with 'Request type' set to 'initial emergency request', and, a PDU SESSION ESTABLISHMENT REQUEST? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | P |
| - | EXCEPTION: In parallel to the events described in steps 14-15 below the events specified in steps 1a1 to 2 of Table 4.9.12.2.2-2 take place. | - | - | - | - |
| 14 | The SS transmits an RRCReconfiguration message and an PDU SESSION ESTABLISHMENT ACCEPT | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to the events described in steps 16-18 below the events specified in steps 3 of Table 4.9.12.2.2-2 take place. | - | - | - | - |
| 16 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 17 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 18 | Check: Does the UE transmits a ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |

Table 4.9.12.2.2-2: IMS signalling and Emergency call establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 1a1 | The generic procedure for IP address allocation in the user plane specified in subclause 4.5A.3 takes place. | - | - | - | - |
| 2 | Generic test procedure for setting up IMS Emergency Voice Call as defined in TS 34.229-5 [47] annex A.6 is performed. | - | - | - | - |

4.9.12.2.3	Specific Message content

All specific message contents shall be according subclause 4.6 and 4.7 with the exceptions below.

NOTE:	Some of the specific message contents provided below assume that the UE is in the state 5GMM-DEREGISTERED.LIMITED-SERVICE or 5GMM-REGISTERED.LIMITED-SERVICE (e.g. the selected cell over 3GPP access is in a forbidden PLMN or is in a forbidden tracking area respectively), or, in 5GMM-DEREGISTERED.NO-SUPI as defined in TS 24.501 [28], subclauses 5.1.3.2.1.3.3 and 5.1.3.2.1.3.6 respectively. All necessary details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

Table 4.9.12.2.3-1: SIB1 (at any time prior and during the procedure, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.6.1-28. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| ims-EmergencySupport |  | Present | true |  |
| } |  |  |  |  |

Table 4.9.12.2.3-2: RRCSetupRequest (step 1, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.6.1-23. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| establishmentCause |  | emergency |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.12.2.3-3: REGISTRATION REQUEST (step 3, Table 4.9.12.2.2-1)

Derivation Path: Table 4.7.1-6, condition EMERGENCY.

Table 4.9.12.2.3-4: SECURITY MODE COMMAND (step 4, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.1-25. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |  |
| Type of ciphering algorithm |  | 5G-EA0 | null ciphering algorithm |  |
| Type of integrity protection algorithm |  | 5G-IA0 | null integrity protection algorithm |  |
| ngKSI |  |  |  |  |
| NAS key set identifier |  | '000'B |  |  |

Table 4.9.12.2.3-5: SecurityModeCommand (step 6, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.6.1-25. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| securityModeCommand SEQUENCE { |  |  |  |  |
| securityConfigSMC SEQUENCE { |  |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |  |
| cipheringAlgorithm |  | nea0 | 'NULL' ciphering algorithm |  |
| integrityProtAlgorithm |  | nia0 | 'NULL' integrity protection algorithm |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.12.2.3-6: REGISTRATION ACCEPT (step 10, Table 4.9.12.2.2-1)

Derivation Path: Table 4.7.1-7, condition EMERGENCY.

Table 4.9.12.2.3-7: UL NAS TRANSPORT (step 13, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.1-10, condition INITIAL_PDU_REQUEST. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | ‘011’B | initial emergency request |  |
| S-NSSAI |  | Not Present |  |  |
| DNN |  | Not Present |  |  |

Table 4.9.12.2.3-8: PDU SESSION ESTABLISHMENT REQUEST (step 13, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | A value that is not currently being used by another PDU session |  |  |
| PTI |  | A value currently not used |  |  |
| SSC mode |  | ‘001’B | SSC mode 1 |  |

Table 4.9.12.2.3-9: DL NAS TRANSPORT (step 14, Table 4.9.12.2.2-1)

Derivation Path: Table 4.7.1-11, condition 5GSM_MESSAGE.

Table 4.9.12.2.3-10: PDU SESSION ESTABLISHMENT ACCEPT (step 14, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Selected SSC mode |  | ‘001’B | SSC mode 1 |  |
| Authorized QoS rules |  | Reference QoS rule #2 as defined in Table 4.8.2.1-2. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #2 as defined in Table 4.8.2.3-2. |  |  |

Table 4.9.12.2.3-11: RRCReconfiguration (step 14, Table 4.9.12.2.2-1)

Derivation Path: Table 4.8.1-1: RRCReconfiguration-DRB (1, 0)

Table 4.9.12.2.3-12: PDU SESSION MODIFICATION COMMAND (step 16, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  | Reference QoS rule #7 as defined in Table 4.8.2.1-7 using condition IMS_VOICE. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #5 as defined in Table 4.8.2.3-5. |  |  |

Table 4.9.12.2.3-13: RRCReconfiguration (step 16, Table 4.9.12.2.2-1)

Derivation Path: 4.8.1-1C RRCReconfiguration-Speech

### 4.9.12A	Test Procedure for IMS MO Emergency call release

4.9.12A.1	Scope

This procedure aims at verifying the UE initiated release of an ongoing IMS Emergency call in 5GC

4.9.12A.2	Procedure description

4.9.12A.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The Test UICC shall be inserted. It shall provide Emergency Numbers.

The procedure assumes that the UE is in test state 3N-A, subclause 4.4A.2 on the NR Cell with an active IMS emergency call. All necessary details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.12A.2.2	Procedure sequence

Table 4.9.12A.2.2-1: Test procedure sequence IMS MO Emergency call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1-3 | Generic test procedure for MO Release of Voice Call / 5GS, as defined in Annex A.7 of TS 34.229-5 [47], is performed. | - | - | - | - |
| 4 | The SS transmits a RRCReconfiguration message and a PDU SESSION RELEASE COMMAND message. | <-- | NR RRC: RRCReconfiguration NR NAS: PDU SESSION RELEASE COMMAND | - | - |
| 5 | Check: Does the UE transmit a RRCReconfigurationComplete message and a PDU SESSION RELEASE COMPLETE message? | --> | NR RRC: RRCReconfigurationComplete NR NAS: PDU SESSION RELEASE COMPLETE | - | P |

4.9.12A.2.3	Specific Message content

All specific message contents shall be according subclause 4.6 and 4.7 with the exceptions below.

Table 4.9.12A.2.3-1: RRCReconfiguration (step 4, Table 4.9.12A.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 2 entries |  |  |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS emergency signalling bearer | entry 1 |  |
| DRB-Identity[2] |  | DRB-Identity linked to the IMS emergency speech bearer | entry 2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 2 entries |  |  |
| logicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| logicalChannelIdentity[2] |  | Same value as drb-Identity[2] above | entry 2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.12A.2.3-2: PDU SESSION RELEASE COMMAND (step 4, Table 4.9.12A.2.2-1)

| Derivation Path: Table 4.7.1-14. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Set according to the Emergency session ID. |  |  |
| 5GSM cause |  | ‘0010 0100’B | Regular deactivation |  |

### 4.9.12B	Test Procedure for IMS MT Emergency call release

4.9.12B.1	Scope

This procedure aims at verifying the network initiated release of an ongoing IMS Emergency call in 5GC

4.9.12B.2	Procedure description

4.9.12B.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The Test UICC shall be inserted. It shall provide Emergency Numbers.

The procedure assumes that the UE is in test state 3N-A, subclause 4.4A.2 on the NR Cell with an active IMS emergency call. All necessary details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.12B.2.2	Procedure sequence

Table 4.9.12B.2.2-1: Test procedure sequence IMS MT Emergency call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1-2 | Generic test procedure for MT release of speech call, steps 1-2, as defined in Annex A.8 of TS 34.229-5 [47] is performed. | - | - | - | - |
| 3 | The SS transmits a RRCReconfiguration message and a PDU SESSION RELEASE COMMAND message. | <-- | NR RRC: RRCReconfiguration NR NAS: PDU SESSION RELEASE COMMAND | - | - |
| 4 | Check: Does the UE transmit a RRCReconfigurationComplete message and a PDU SESSION RELEASE COMPLETE message? | --> | NR RRC: RRCReconfigurationComplete NR NAS: PDU SESSION RELEASE COMPLETE | - | P |

4.9.12B.2.3	Specific Message content

All specific message contents shall be according subclause 4.6 and 4.7 with the exceptions below.

Table 4.9.12B.2.3-1: RRCReconfiguration (step 3, Table 4.9.12B.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 2 entries |  |  |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS emergency signalling bearer | entry 1 |  |
| DRB-Identity[2] |  | DRB-Identity linked to the IMS emergency speech bearer | entry 2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 2 entries |  |  |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| LogicalChannelIdentity[2] |  | Same value as drb-Identity[2] above | entry 2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.12B.2.3-2: PDU SESSION RELEASE COMMAND (step 3, Table 4.9.12B.2.2-1)

| Derivation Path: Table 4.7.1-14. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Set according to the Emergency session ID. |  |  |
| 5GSM cause |  | ‘0010 0100’B | Regular deactivation |  |

### 4.9.13	Test procedure for no response to paging

4.9.13.1	Scope

This procedure aims at checking that the UE ignores paging messages with a specified identity.

4.9.13.2	Procedure description

4.9.13.2.1	Initial conditions

As specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.13.2.2	Procedure sequence

Table 4.9.13.2.2-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a paging message using the NG-5G-S-TMSI which is specified in the referring test step, and on the cell which is specified in the referring test step. | <-- | Paging | - | - |
| 2 | Check: Does the UE send an RRCSetupRequest message on the cell where the paging was transmitted within the next 3s? | --> | RRCSetupRequest | - | F |

4.9.13.3	Specific Message content

None.

### 4.9.14	Void

### 4.9.15	Generic Test Procedure for IMS MO speech call establishment in 5GC

4.9.15.1	Scope

The purpose of this procedure is to establish an IMS MO speech call.

4.9.15.2	Procedure description

4.9.15.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 1N-A and registered to the IMS.

4.9.15.2.2	Procedure sequence

Table 4.9.15.2.2-1: IMS MO speech call establishment in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Make the UE attempt an IMS speech call | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCSetupRequest message with 'establishmentCause' set to 'mo-Data'? | --> | NR RRC: RRCSetupRequest | - | P |
| 3 | SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | P |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | Check: Does the UE transmit a SecurityModeComplete message and establish the initial security configuration? | --> | NR RRC: SecurityModeComplete | - | P |
| 7 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB(s). | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| - | EXCEPTION: In parallel to step 8 below, the steps specified in Table 4.9.15.2.2-2 take place. | - | - | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 9a1 to 9b4 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 9a1-9a4 | IF the UE was configured to not use preconditions THEN steps 2-5 of Annex A.4.2 of TS 34.229-5 [47] take place. | - | - | - | - |
| 9b1-9b4 | ELSE steps 2-5 of Annex A.4.1 of TS 34.229-5 [47] take place. | - | - | - | - |
| 10 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 12 | Check: Does the UE transmit a ULInformationTransfer message, an UL NAS TRANSPORT message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |
| - | EXCEPTION: Steps 13a1 to 13b8 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 13a1-13a3 | IF the UE was configured to not use preconditions THEN steps 6-8 of Annex A.4.2 of TS 34.229-5 [47] take place. | - | - | - | - |
| 13b1-13b7 | ELSE steps 6-12 of Annex A.4.1 of TS 34.229-5 [47] take place. | - | - | - | - |

Table 4.9.15.2.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 1a1 | IF the UE was configured to not use preconditions THEN step 1 of Annex A.4.2 of TS 34.229-5 [47] takes place. | - | - | - | - |
| 1b1 | ELSE step 1 of Annex A.4.1 of TS 34.229-5 [47] takes place | - | - | - | - |

4.9.15.3	Specific message contents

Table 4.9.15.3-1: RRCSetupRequest (step 2, Table 4.9.15.2.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-23. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| establishmentCause |  | mo-Data |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.15.3-2: SERVICE REQUEST (step 4, Table 4.9.15.2.2-1)

| Derivation Path: 38.508-1 [4], Table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  |  |  |  |
| Service type value |  | ‘0001’B | data |  |

Table 4.9.15.3-3: PDU SESSION MODIFICATION COMMAND (step 10, Table 4.9.15.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  | Reference QoS rule #7 as defined in Table 4.8.2.1-7 using condition IMS_VOICE. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #5 as defined in Table 4.8.2.3-5. |  |  |

Table 4.9.15.3-4: RRCReconfiguration (step 10, Table 4.9.15.2.2-1)

Derivation Path: 4.8.1-1C RRCReconfiguration-Speech

### 4.9.16	Generic Test Procedure for IMS MT speech call establishment in 5GC

4.9.16.1	Scope

The purpose of this procedure is to establish an IMS MT speech call.

4.9.16.2	Procedure description

4.9.16.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 1N-A and registered to the IMS.

4.9.16.2.2	Procedure sequence

Table 4.9.16.2.2-1: IMS MT speech call establishment in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 2 | Check: Does the UE transmit an RRCSetupRequest message with 'establishmentCause' set to 'mt-Access'? | --> | NR RRC: RRCSetupRequest | - | P |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | Check: Does the UE transmit an RRCSetupComplete message and a SERVICE REQUEST message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | P |
| 5 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 7 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB(s). The RRCReconfiguration message is configured using RRCReconfiguration-SRB2-DRB(n, m) where n and m are the number of DRB(s) configured with RLC-AM and RLC-UM respectively. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 8 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | - | P |
| - | EXCEPTION: Steps 9a1 to 9b5 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 9a1-9a5 | IF the UE was configured to not use preconditions THEN steps 1-5 of Annex A.5.2 of TS 34.229-5 [47] take place. | - | - | - | - |
| 9b1-9b5 | ELSE steps 1-5 of Annex A.4.1 of TS 34.229-5 [47] take place. | - | - | - | - |
| 10 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 12 | Check: Does the UE transmit a ULInformationTransfer message, an UL NAS TRANSPORT message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |
| - | EXCEPTION: Steps 13a1 to 13b7 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 13a1-13a5 | IF the UE was configured to not use preconditions THEN steps 6-10 of Annex A.5.2 of TS 34.229-5 [47] take place. | - | - | - | - |
| 13b1-13b7 | ELSE steps 6-12 of Annex A.4.1 of TS 34.229-5 [47] take place. | - | - | - | - |

4.9.16.3	Specific message contents

Table 4.9.16.3-1: RRCSetupRequest (step 2, Table 4.9.16.2.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-23. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| establishmentCause |  | mt-Access |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.16.3-2: SERVICE REQUEST (step 4, Table 4.9.16.2.2-1)

| Derivation Path: 38.508-1 [4], Table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  |  |  |  |
| Service type value |  | ‘0001’B | data |  |

Table 4.9.16.3-3: PDU SESSION MODIFICATION COMMAND (step 10, Table 4.9.16.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  | Reference QoS rule #7 as defined in Table 4.8.2.1-7 using condition IMS_VOICE. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #5 as defined in Table 4.8.2.3-5. |  |  |

Table 4.9.16.3-4: RRCReconfiguration (step 10, Table 4.9.16.2.2-1)

Derivation Path: 4.8.1-1C RRCReconfiguration-Speech

### 4.9.17	Generic Test Procedure for IMS MO call release in 5GC

4.9.17.1	Scope

The purpose of this procedure is to make a UE initiated release of an ongoing IMS speech call.

4.9.17.2	Procedure description

4.9.17.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 3N-A on the NR Cell with an active IMS call.

4.9.17.2.2	Procedure sequence

Table 4.9.17.2.2-1: IMS MO speech call establishment in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1-2 | Generic test procedure for MO release of speech call, steps 1-2, as defined in annex A.7 of TS 34.229-5 [47] are performed. | - | - | - | - |
| 3 | The SS transmits a RRCReconfiguration message and a PDU SESSION RELEASE COMMAND message. | <-- | NR RRC: RRCReconfiguration NR NAS: PDU SESSION RELEASE COMMAND | - | - |
| 4 | Check: Does the UE transmit a RRCReconfigurationComplete message and a PDU SESSION RELEASE COMPLETE message? | --> | NR RRC: RRCReconfigurationComplete NR NAS: PDU SESSION RELEASE COMPLETE | - | P |

4.9.17.2.3	Specific message contents

Table 4.9.17.2.3-1: RRCReconfiguration (step 3, Table 4.9.17.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 1 entry |  |  |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 1 entry |  |  |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.17.2.3-2: PDU SESSION RELEASE COMMAND (step 3, Table 4.9.17.2.2-1)

| Derivation Path: Table 4.7.1-14. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| 5GSM cause |  | ‘0010 0100’B | Regular deactivation |  |

### 4.9.18	Generic Test Procedure for IMS MT call release in 5GC

4.9.18.1	Scope

The purpose of this procedure is to make the network release an ongoing IMS speech call.

4.9.18.2	Procedure description

4.9.18.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 3N-A on the NR Cell with an active IMS call.

4.9.18.2.2	Procedure sequence

Table 4.9.18.2.2-1: IMS MO speech call establishment in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1-2 | Generic test procedure for MT release of speech call, steps FFS, as defined in clause FFS of TS FFS are performed. | - | - | - | - |
| 3 | The SS transmits a RRCReconfiguration message and a PDU SESSION RELEASE COMMAND message. | <-- | NR RRC: RRCReconfiguration NR NAS: PDU SESSION RELEASE COMMAND | - | - |
| 4 | The UE transmit a RRCReconfigurationComplete message and a PDU SESSION RELEASE COMPLETE message? | --> | NR RRC: RRCReconfigurationComplete NR NAS: PDU SESSION RELEASE COMPLETE | - | P |

4.9.18.2.3	Specific message contents

Table 4.9.18.2.3-1: RRCReconfiguration (step 3, Table 4.9.18.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 1 entry |  |  |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 1 entry |  |  |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.18.2.3-2: PDU SESSION RELEASE COMMAND (step 3, Table 4.9.18.2.2-1)

| Derivation Path: Table 4.7.1-14. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| 5GSM cause |  | ‘0010 0100’B | Regular deactivation |  |

### 4.9.19	Generic Test Procedure for IMS MO SMS in 5GC

4.9.19.1	Scope

The purpose of this procedure is to transmit an IMS MO SMS.

4.9.19.2	Procedure description

4.9.19.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 1N-A and registered to the IMS.

4.9.19.2.2	Procedure sequence

Table 4.9.19.2.2-1: IMS MO speech call establishment in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Make the UE attempt an IMS MO SMS | - | - | - | - |
| 2 | Check: Does the UE transmit an RRCSetupRequest message with 'establishmentCause' set to 'mo-Data'? | --> | NR RRC: RRCSetupRequest | - | P |
| 3 | SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | P |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | Check: Does the UE transmit a SecurityModeComplete message and establish the initial security configuration? | --> | NR RRC: SecurityModeComplete | - | P |
| 7 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB(s). | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 8-15 | The steps 2-9 from the expected sequence defined in X.X.X of TS 34.229-5 [47] IMS MO SMS take place. | - | - | - | - |

4.9.19.3	Specific message contents

Table 4.9.19.3-1: RRCSetupRequest (step 2, Table 4.9.19.2.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-23. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| establishmentCause |  | mo-Data |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.19.3-2: SERVICE REQUEST (step 4, Table 4.9.19.2.2-1)

| Derivation Path: 38.508-1 [4], Table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  |  |  |  |
| Service type value |  | ‘0001’B | data |  |

### 4.9.20	Generic Test Procedure for IMS MT SMS in 5GC

4.9.20.1	Scope

The purpose of this procedure is to receive an IMS MT SMS.

4.9.20.2	Procedure description

4.9.20.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 1N-A and registered to the IMS.

4.9.20.2.2	Procedure sequence

Table 4.9.20.2.2-1: IMS MT speech call establishment in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 2 | Check: Does the UE transmit an RRCSetupRequest message with 'establishmentCause' set to 'mt-Access'? | --> | NR RRC: RRCSetupRequest | - | P |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | Check: Does the UE transmit an RRCSetupComplete message and a SERVICE REQUEST message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | P |
| 5 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 7 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB(s). The RRCReconfiguration message is configured using RRCReconfiguration-SRB2-DRB(n, m) where n and m are the number of DRB(s) configured with RLC-AM and RLC-UM respectively. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 8 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | - | P |
| 9-12 | The steps 1-4 from the expected sequence defined in X.X.X of TS 34.229-5 [47] IMS MT SMS take place. | - | - | - | - |

4.9.20.3	Specific message contents

Table 4.9.20.3-1: RRCSetupRequest (step 2, Table 4.9.20.2.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-23. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| establishmentCause |  | mt-Access |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.20.3-2: SERVICE REQUEST (step 4, Table 4.9.20.2.2-1)

| Derivation Path: 38.508-1 [4], Table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  |  |  |  |
| Service type value |  | ‘0001’B | data |  |

### 4.9.21	Procedure for PDU Session Release

4.9.21.1	Scope

The purpose of this procedure is to release both the PDU session and the user plane resources.

4.9.21.2	Procedure description

4.9.21.2.1	Initial conditions

The UE is in state 3N-A with PDU SESSION ACTIVE as per Table 4.4A.2-3. If this is a UE triggered PDU Session Release, this will be specified in the test case together with the sending of a PDU SESSION RELEASE REQUEST by the UE.

4.9.21.2.2	Procedure sequence

Table 4.9.21.2.2-1: Procedure for PDU Session Release

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The SS transmits an RRCReconfiguration message and a PDU SESSION RELEASE COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMMAND |
| 2 | The UE transmit a RRCReconfigurationComplete message and a PDU SESSION RELEASE COMPLETE message? | --> | NR RRC: RRCReconfigurationComplete 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMPLETE |

4.9.21.3	Specific message contents

The NAS message contents will be as specified in the test case.

Table 4.9.21.3-1: RRCReconfiguration (step 1, Table 4.9.21.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 1 entry |  |  |
| DRB-Identity[1] |  | DRB-Identity linked to the PDU Session to be released |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 1 entry |  |  |
| logicalChannelIdentity[1] |  | Same value as drb-Identity[1] above |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# 5	Test environments for RF test

## 5.0	General

### 5.0.1	Single PDU configuration for RF testing

For RF and performance test case execution on 5G SA UE's defined in TS 38.521-1 [14], TS 38.521-2 [15],  TS 38.521-4 [17], IMS shall not be considered and UE's shall be able use RRC (IDLE, CONNECTED) preambles defined in TS 38.508-1 clause 4.5. Before entering RRC_CONNECTED or RRC_IDLE state during initial conditions or test procedure, it is recommended that UE is pre-configured with only 1 PDU (non-IMS) along with appropriate settings to ensure UE operates and stays on NR cell.

## 5.1	Requirements of test equipment

### 5.1.1	Requirements for transmission and reception tests

#### 5.1.1.1	Requirements common for conducted and OTA tests

No common RF test environment requirements are specified in addition to the common requirements described in clause 4.2.

#### 5.1.1.2	Requirements for conducted tests

No common RF test environment requirements are specified in addition to the common requirements described in clause 4.2.

#### 5.1.1.3	Requirements for OTA tests

Editor’s Note:

-	The UE pre-configuration mentioned below to disable UL Tx diversity schemes shall be voided once a test methodology solution to minimize spectral flatness artefacts between TE and UE over all test points is defined.

-	The permitted test methods for transmission and reception test are DFF, DFF with simplification for centre of beam measurements, IFF and NFTF and are described in TR 38.810[24]. The minimum requirements for each test setup are described in the following clauses.

-	For conformance testing using the OTA test environment, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

##### 5.1.1.3.1	DFF and DFF with simplification for centre of beam measurements

-	Far-field measurement system in an anechoic chamber.

-	The minimum far-field distance R for a traditional far field anechoic chamber can be calculated based on the following equation: , where D is the diameter of the smallest sphere that encloses the radiating parts of the DUT.

-	A positioning system such that the angle between the dual-polarized measurement antenna and the DUT has at least two axes of freedom and maintains a polarization reference.

-	For DFF(without simplification), a positioning system such that the angle between the link antenna and the DUT has at least two axes of freedom and maintains a polarization reference; this positioning system for the link antenna is in addition to the positioning system for the measurement antenna and provides for an angular relationship independently controllable from the measurement antenna.

-	For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1 UL configuration, an LTE link antenna is used to provide the LTE link to the DUT. The LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

-	For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

-	Maximum permitted test system uncertainty is specified in Annex F in 38.521-2[15].

##### 5.1.1.3.2	IFF

-	Indirect Far field of Compact Antenna Test Range(CATR) with quiet zone diameter at least D.

-	The CATR system does not require a measurement distance of  to achieve a plane wave as in a standard far field range.

-	A positioning system such that the angle between the dual-polarized measurement antenna and the DUT has at least two axes of freedom and maintains a polarization reference.

-	For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1UL configuration, an LTE link antenna is used to provide the LTE link to the DUT. The LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

-	For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

-	Maximum permitted test system uncertainty is specified in Annex F in 38.521-2[15].

##### 5.1.1.3.3	NFTF

-	Radiated Near Field UE beam pattern are measured and based on the NFTF mathematical transform, the final metric such as EIRP is the same as the metric for the DFF setup

-	A positioning system such as the angle between the dual-polarized measurement/link antenna and the DUT has at least two axes of freedom and maintains a polarization reference

-	For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1UL configuration, an LTE link antenna is used to provide the LTE link to the DUT. The LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

-	For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

-	Maximum permitted test system uncertainty is specified in Annex F in 38.521-2[15].

### 5.1.2	Requirements for performance tests

#### 5.1.2.1	Requirements common for conducted and OTA tests

Editor’s Note: This subclause is intended to describe the test equipment requirements which are specific to performance tests and common for conducted and OTA tests.

#### 5.1.2.2	Requirements for conducted test method

Editor’s Note: This subclause is intended to describe the test equipment requirements which are specific to conducted test environment for performance tests.

#### 5.1.2.3	Requirements for OTA test method

Editor’s Note: This subclause is intended to describe the test equipment requirements which are specific to OTA test environment for performance tests.

-	The UE pre-configuration mentioned below to disable UL Tx diversity schemes shall be voided once a test methodology solution to minimize spectral flatness artefacts between TE and UE over all test points is defined.

-	For conformance testing using the OTA test environment, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

## 5.2	Reference test conditions

### 5.2.1	Signal levels

#### 5.2.1.1	Signal Levels for conducted testing

For NR FR1 cell, the downlink power settings are specified in TS 38.521-1[14] and TS 38.521-3[16].

The uncertainty value is specified in TS 38.521-1 [14] Annex F or in TS 38.521-2 [15] Annex F.

#### 5.2.1.2	Signal Levels for OTA testing

#### 5.2.1.2.1	Downlink Signal Levels

For E-UTRA cell in EN-DC with FR2 NR, the downlink power settings are specified in clause 4.7 of TS 38.521-3[16].

For FR2 NR cell, the downlink power settings are specified in Annex C.0 of TS 38.521-2[15] and Annex C.0 of TS 38.521-3[16].

## 5.3	Void

Editor‘s Note: Reserved for future use.

## 5.4	Default NG-RAN RRC message and information elements contents

### 5.4.1	Radio resource control information elements

As defined in clause 4.6.3 with the following exceptions:

For Tx test cases in which Power Class 3 UEs verifying Power Class 3 requirements, refer to Table 5.4.1-1; For Tx test cases in which Power Class 2 UEs verifying Power Class 2 requirements, refer to Table 5.4.1-2; And for Tx test cases in which Power Class 2 UEs verifying Power Class 3 requirements, refer to Table 5.4.1-3.

Table 5.4.1-1: P-Max-PC3

Derivation Path: Table 4.6.3-89 with condition FR1_RF_PC3

Table 5.4.1-2: P-Max-PC2

Derivation Path: Table 4.6.3-89 with condition FR1_RF_PC2

Table 5.4.1-3: P-Max-PC2_Testing_PC3

Derivation Path: Table 4.6.3-89 with condition FR1_RF_PC2_Testing_PC3

#### CSI-RS for Tracking

Table 5.4.1-4: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 1000 |  | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 6 | l0 = 6 for CSI-RS resource 1 and 3 | TRS AND FR1 |
|  | 10 | l0 = 10 for CSI-RS resource 2 and 4 | TRS AND FR1 |
|  | 8 | l0 = 8 for CSI-RS resource 1 | TRS AND FR2 |
|  | 12 | l0 = 12 for CSI-RS resource 2 | TRS AND FR2 |
| nrofPorts | p1 |  | TRS |
| Cdm-Type | noCDM |  | TRS |
| Density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  | TRS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal |

Table 5.4.1-5: CSI-ResourcePeriodicityAndOffset for TRS

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 0 | SCS 15kHz, CSI-RS resource 1 and 2 | TRS AND FR1_15kHz |
|  | 1 | SCS 15kHz, CSI-RS resource 3 and 4 | TRS AND FR1_15kHz |
| Slots20 | 1 | SCS 30kHz, CSI-RS resource 1 and 2 | TRS AND FR1_30kHz |
|  | 2 | SCS 30kHz, CSI-RS resource 3 and 4 | TRS AND FR1_30kHz |
| Slots40 | 2 | SCS 60kHz, FR1, CSI-RS resource 1 and 2 | TRS AND FR1_60kHz |
|  | 3 | SCS 60kHz, FR1, CSI-RS resource 3 and 4 | TRS AND FR1_60kHz |
| Slots80 | 40 | SCS 60kHz, FR2, CSI-RS resource 1 and 2 | TRS AND FR2_60kHz |
| Slots160 | 80 | SCS 120kHz, CSI-RS resource 1 and 2 | TRS AND FR2_120kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| FR1_60kHz | FR1 is used under the test. SCS is set to 60kHz. |
| FR2_60kHz | FR2 is used under the test. SCS is set to 60kHz. |
| FR2_120kHz | FR2 is used under the test. SCS is set to 120kHz. |
| TRS | Tracking-Reference Signal |

Table 5.4.1-6: CSI-MeasConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 2 entries in case of FR2 4 entries in case of FR1 | 2 entries in case of FR2 4 entries in case of FR1 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 | FR1 |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 | FR1 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| } |  |  |  |
| reportTriggerSize | Not present |  |  |
| aperiodicTriggerStateList SetupRelease | Not present |  |  |
| } |  |  |  |

Table 5.4.1-7: NZP-CSI-RS-Resource for TRS

| Derivation Path: 38.508-1 Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| NZP-CSI-RS-ResourceId | 0 | CSI-RS resource 1 |  |
|  | 1 | CSI-RS resource 2 |  |
|  | 2 | CSI-RS resource 3 |  |
|  | 3 | CSI-RS resource 4 |  |
| CSI-RS-ResourceMapping | CSI-RS-ResourceMapping for TRS |  | Content conditioned by the CSI-RS resource # |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | ScramblingId |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for TRS |  | Content conditioned by the CSI-RS resource # |
| qcl-InfoPeriodicCSI-RS | TCI-StateId |  |  |
| } |  |  |  |

Table 5.4.1-8: NZP-CSI-RS-ResourceSet for TRS

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 0 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries in case of FR2 4 entries in case of FR1 |  |  |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-ResourceId[3] | 2 | entry 3 CSI-RS resource 3 | FR1 |
| NZP-CSI-RS-ResourceId[4] | 3 | entry 4 CSI-RS resource 4 | FR1 |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | true |  |  |
| } |  |  |  |

Table 5.4.1-9: CSI-ResourceConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 0 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

#### PDCCH Configuration

##### PDCCH-config

Table 5.4.1-16: PDCCH ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP | RFACS AND AL8 |
|  | 11111111 11111111 00000000 00000000 00000000 00000 | CORESET to use the least significant 96 RBs of the BWP | RFACS AND AL16 |

| Condition | Explanation |
| --- | --- |
| RFACS | Used in RF Adjacent Channel Selectivity testing |
| AL8 | PDCCH aggregationLevel 8 is configured |
| AL16 | PDCCH aggregationLevel 16 is configured |

Table 5.4.1-17: PDCCH Search Space

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n2 |  | RFACS |
| aggregationLevel8 | n2 |  | RFACS |
| aggregationLevel16 | n2 |  | RFACS |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RFACS | Used in RF Adjacent Channel Selectivity testing |

### 5.4.2	Radio resource control information elements for Demodulation Performance and CSI reporting tests

As defined in clause 4.6.3 with the following exceptions:

Table 5.4.2-1: Void

Table 5.4.2-2: Void

Table 5.4.2-3a: Void

Table 5.4.2-3: Void

Table 5.4.2-4: Void

Table 5.4.2-4A: Void

Table 5.4.2-5 to -20: Void

#### 5.4.2.0	Parameters common to all Demod and CSI tests

##### Physical layer parameters

Table 5.4.2.0-1: Phy sical layer parameters for DCI format 1_1

| Derivation Path: Table 4.3.6.1.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PUCCH resource indicator | PUCCH-ResourceId[1] = 0 in pucch-ResourceSetID[1] or PUCCH-ResourceId[1] = 8 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘000’B | FR1 |
|  | PUCCH-ResourceId[5] = 12 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘100’B | FR2_SCS60kHz |
|  | PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘110’B | FR2_SCS120kHz |
| PDSCH-to-HARQ_feedback timing indicator | K1 slots as specified in 9.2.3 in TS 38.213 Specific to each TDD UL-DL pattern and as defined in Annex A.1.2, A.1.3 of TS 38.521-4 |  |  |

##### Common Serving Parameters

###### ServingCellConfigCommon

Table 5.4.2.0-2: ServingCellConfigCommon

| Derivation Path: Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| downlinkConfigCommon | DownlinkConfigCommon |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 1000 |  | FR1 AND SSB#0 AND (2.4GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.4GHz) |
| mediumBitmap | 10000000 |  | FR1 AND SSB#0 AND (2.4GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| } |  |  |  |
| ssb-periodicityServingCell | ms20 |  |  |
| dmrs-TypeA-Position | pos2 |  |  |
| subcarrierSpacing | SubcarrierSpacing according to test case id |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  |  |
|  | Not present |  | FR1.30-1A FR2.120-1A FR1.FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=2.4GHz | Frequency range <= 2.4GHz |
| 2.4GHz<FREQ<=3GHz | Frequency range > 2.4GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| FR1.30-1A | TDD UL-DL pattern FR1.30-1A is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR2.120-1A | TDD UL-DL pattern FR2.120-1A is used. Ref Annex A.1.3 of TS 38.521-4 |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |

###### TDD-UL-DL-Config

Table 5.4.2.0-3: TDD-UL-DL-Config

| Derivation Path: Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1.15-1, FR1.30-1 |
|  | ms2p5 |  | FR1.30-2, FR1.30-3 |
|  | ms3 |  | FR1.30-4 |
|  | ms2 |  | FR1.30-5 |
|  | ms1 |  | FR1.30-6 FR2.60-1 |
|  | ms0p625 |  | FR2.120.1 |
|  | ms0p5 |  | FR2.120.2 |
| nrofDownlinkSlots | 7 |  | FR1.30-1 |
|  | 3 |  | FR1.15-1 FR1.30-2 FR1.30-3 FR1.30-4 FR2.120.1 |
|  | 1 |  | FR1.30-5 FR1.30-6 |
|  | 2 |  | FR2.60-1 FR2.120.2 |
| nrofDownlinkSymbols | 6 |  | FR1.30-1 FR1.30-4 |
|  | 10 |  | FR1.15-1 FR1.30-2 FR1.30-3 FR1.30-6 FR2.120.1 |
|  | 12 |  | FR1.30-5 |
|  | 11 |  | FR2.60-1 FR2.120.2 |
| nrofUplinkSlots | 2 |  | FR1.30-1 FR1.30-4 FR1.30-5 |
|  | 1 |  | FR1.15-1 FR1.30-2 FR1.30-3 FR2.60-1 FR2.120.1 FR2.120.2 |
|  | 0 |  | FR1.30-6 |
| nrofUplinkSymbols | 4 |  | FR1.30-1 FR1.30-4 FR1.30-5 |
|  | 0 |  | FR1.30-6 FR2.60-1 FR2.120.2 |
|  | 2 |  | FR1.15-1 FR1.30-2 FR1.30-3 FR2.60-1 FR2.120.1 FR2.120.2 |
| } |  |  |  |
| pattern2 | Not present |  | FR1.15-1, FR1.30-1 FR1.30-2 FR1.30-5 FR2.60-1 FR2.120.1 FR2.120.2 |
| pattern2 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms2p5 |  | FR1.30-3 |
|  | ms2 |  | FR1.30-4 |
|  | ms1 |  | FR1.30-6 |
| nrofDownlinkSlots | 2 |  | FR1.30-3 |
|  | 4 |  | FR1.30-4 |
|  | 0 |  | FR1.30-6 |
| nrofDownlinkSymbols | 10 |  | FR1.30-3 |
|  | 0 |  | FR1.30-4 |
|  | 12 |  | FR1.30-6 |
| nrofUplinkSlots | 2 |  | FR1.30-3 |
|  | 0 |  | FR1.30-4 |
|  | 1 |  | FR1.30-6 |
| nrofUplinkSymbols | 2 |  | FR1.30-3 |
|  | 0 |  | FR1.30-4 FR1.30-6 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1.15-1 | TDD UL-DL pattern FR1.15-1 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-1 | TDD UL-DL pattern FR1.30-1 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-2 | TDD UL-DL pattern FR1.30-2 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-3 | TDD UL-DL pattern FR1.30-3 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-4 | TDD UL-DL pattern FR1.30-4 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-5 | TDD UL-DL pattern FR1.30-5 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-6 | TDD UL-DL pattern FR1.30-6 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR2.60-1 | TDD UL-DL pattern FR2.60-1 is used. Ref Annex A.1.3 of TS 38.521-4 |
| FR2.120-1 | TDD UL-DL pattern FR2.120-1 is used. Ref Annex A.1.3 of TS 38.521-4 |
| FR2.120-2 | TDD UL-DL pattern FR2.120-2 is used. Ref Annex A.1.3 of TS 38.521-4 |

##### PDCCH Configuration

###### PDCCH-ConfigCommon

Table 5.4.2.0-4: PDCCH-ConfigCommon

| Derivation Path: Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon::= SEQUENCE { |  |  |  |
| searchSpaceZero | 4 |  | FDD FR1.30-1 FR1.30-2 FR1.30-3 FR1.30-4 |
|  | 2 |  | FR1.30-5 FR1.30-6 |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry | 1 search space for both SA and NSA |  |
| SearchSpace [1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| searchSpaceOtherSystemInformation | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |

##### ServingCellConfig

Table 5.4.2.0-5: ServingCellConfig

| Derivation Path: Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig CHOICE { |  |  |  |
| setup | csi-MeasConfig |  |  |
| } |  |  |  |

##### PDCCH-config

Table 5.4.2.0-6: PDCCH-ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | Table 5.2-2 for tested channel bandwidth and subcarrier spacing |  |  |
| Duration | 2 | SearchSpace duration of 2 symbols | DEMOD_FR1, CSI_FR1, CSI_FR2 |
|  | 1 | SearchSpace duration of 1 symbols | DEMOD_FR2, PDCCH_FR1, PDCCH_FR2 |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | Null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 0 | entry 1 TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DEMOD_FR1 | DEMOD testing in FR1. |
| DEMOD_FR2 | DEMOD testing in FR2. |
| PDCCH_FR1 | PDCCH testing in FR1 |
| PDCCH_FR2 | PDCCH testing in FR2 |
| CSI_FR1 | CSI testing in FR1 |
| CSI_FR2 | CSI testing in FR2 |

Table 5.4.2.0-7: PDCCH Search Space

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Starting symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  |  |
| ue-Specific SEQUENCE { |  |  |  |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### NZP-CSI-RS for Tracking

###### NZP-CSI-RS-Resource

Table 5.4.2.0-8: NZP-CSI-RS-Resource for TRS

| Derivation Path: Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 0 | CSI-RS resource 1 |  |
|  | 1 | CSI-RS resource 2 |  |
|  | 2 | CSI-RS resource 3 |  |
|  | 3 | CSI-RS resource 4 |  |
| resourceMapping | CSI-RS-ResourceMapping for TRS |  |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for TRS |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId 0 |  |  |
| } |  |  |  |

###### CSI-RS-ResourceMapping

Table 5.4.2.0-9: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 6 | l0 = 6 for CSI-RS resource 1 and 3 | TRS |
|  | 10 | l0 = 10 for CSI-RS resource 2 and 4 | TRS |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 | TRS |
| Cdm-Type | noCDM |  | TRS |
| Density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  | TRS |
| } |  |  |  |

###### CSI-ResourcePeriodicityAndOffset

Table 5.4.2.0-10: CSI-ResourcePeriodicityAndOffset for TRS

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots20 | 10 | Periodicity 20 slots and offset 10 for CSI-RS resource 1 and 2 | SCS 15KHz |
| slots20 | 11 | Periodicity 20 slots and offset 11 for CSI-RS resource 3 and 4 | SCS 15 kHz |
| Slots40 | 20 | Periodicity 40 slots and offset 20 for CSI-RS resource 1 and 2 | SCS 30 kHz |
| Slots40 | 21 | Periodicity 40 slots and offset 21 for CSI-RS resource 3 and 4 | SCS 30 kHz |
| Slots80 | 40 | Periodicity 80 slots and offset 40 for CSI-RS resource 1 and 2 | SCS 60 kHz |
| Slots80 | 41 | Periodicity 80 slots and offset 41 for CSI-RS resource 3 and 4 | SCS 60 kHz |
| Slots160 | 80 | Periodicity 160 slots and offset 80 for CSI-RS resource 1 and 2 | SCS 120 kHz |
| Slots160 | 81 | Periodicity 160 slots and offset 81 for CSI-RS resource 3 and 4 | SCS 120 kHz |
| } |  |  |  |

###### CSI-FrequencyOccupation

Table 5.4.2.0-11: CSI-FrequencyOccupation for TRS

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| nrofRBs | 52 | BW 10 MHz SCS 15kHz | TRS |
|  | 52 | BW 20 MHz SCS 30kHz | TRS |
|  | 108 | BW 40 MHz SCS 30kHz | TRS |
|  | 68 | BW 100 MHz SCS 120 kHz | TRS |
| } |  |  |  |

###### NZP-CSI-RS-ResourceSet

Table 5.4.2.0-12: NZP-CSI-RS-ResourceSet for TRS

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 0 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries |  | FR1, FR2 |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-ResourceId[3] | 2 | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-ResourceId[4] | 3 | entry 4 CSI-RS resource 4 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | true |  |  |
| } |  |  |  |

###### CSI-ResourceConfig

Table 5.4.2.0-13: CSI-ResourceConfig for TRS

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 0 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

##### NZP CSI-RS for CSI Acquisition

###### NZP-CSI-RS-Resource

Table 5.4.2.0-14: NZP-CSI-RS-Resource

| Derivation Path: Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  | DEMOD_FR1 DEMOD_FR2 |
| nzp-CSI-RS-ResourceId | 4 | CSI-RS resource 5 |  |
| resourceMapping | CSI-RS-ResourceMapping |  |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-State #1 |  |  |
| } |  |  |  |

###### CSI-RS-ResourceMapping

Table 5.4.2.0-15: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000001 | K0 = 0, row3, 2Tx test cases |  |
| row4 | 001 | K0 = 0, 4Tx test cases |  |
| } |  |  |  |
| nrofPorts | P2 | 2Tx test cases |  |
|  | P4 | 4Tx test cases |  |
| firstOFDMSymbolInTimeDomain | 12 | I0 = 12 |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

###### CSI-ResourcePeriodicityAndOffset

Table 5.4.2.0-16: CSI-ResourcePeriodicityAndOffset

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots20 | 0 |  | SCS 15kHz |
| Slots40 | 0 |  | SCS 30kHz |
| Slots80 | 0 |  | SCS 60kHz |
| Slots160 | 0 |  | SCS 120kHz |
| } |  |  |  |

###### CSI-FrequencyOccupation

Table 5.4.2.0-17: CSI-FrequencyOccupation for CSI Acquisition

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| nrofRBs | 52 |  | BW 10 MHz SCS 15kHz |
|  | 52 |  | BW 20 MHz SCS 30kHz |
|  | 108 |  | BW 40 MHz SCS 30kHz |
|  | 68 |  | BW 100MHz SCS 120kHz |
| } |  |  |  |

###### NZP-CSI-RS-ResourceSet

Table 5.4.2.0-18: NZP-CSI-RS-ResourceSet for CSI Acquisition

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 1 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  | FR1, FR2 |
| NZP-CSI-RS-ResourceId[1] | 4 | entry 1 CSI-RS resource 5 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | Not present |  |  |
| } |  |  |  |

###### CSI-ResourceConfig

Table 5.4.2.0-19: CSI-ResourceConfig for CSI Acquisition

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 1 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 1 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

##### ZP CSI-RS for CSI Acquisition

###### ZP-CSI-RS-Resource

Table 5.4.2.0-20: ZP-CSI-RS-Resource

| Derivation Path: Table 4.6.3-204 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-Resource ::= SEQUENCE { |  |  | DEMOD_FR1 DEMOD_FR2 |
| zp-CSI-RS-ResourceId | ZP-CSI-RS-ResourceId |  |  |
| resourceMapping | ZP CSI-RS-ResourceMapping |  |  |
| periodicityAndOffset | ZP CSI-ResourcePeriodicityAndOffset |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-State #1 |  |  |
| } |  |  |  |

###### CSI-RS-ResourceMapping

Table 5.4.2.0-21: ZP CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 | K0 = 4 |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 12 | I0 = 12 |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | ZP CSI-FrequencyOccupation |  |  |
| } |  |  |  |

###### CSI-ResourcePeriodicityAndOffset

Table 5.4.2.0-22: ZP CSI-ResourcePeriodicityAndOffset

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots20 | 0 |  | SCS 15kHz |
| Slots40 | 0 |  | SCS 30kHz |
| Slots80 | 0 |  | SCS 60kHz |
| Slots160 | 0 |  | SCS 120kHz |
| } |  |  |  |

###### CSI-FrequencyOccupation

Table 5.4.2.0-23: ZP CSI-FrequencyOccupation

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| nrofRBs | 52 |  | BW 10 MHz SCS 15kHz |
|  | 52 |  | BW 20 MHz SCS 30kHz |
|  | 108 |  | BW 40 MHz SCS 30KHz |
|  | 68 |  | BW 100 MHz SCS 120KHz |
| } |  |  |  |

##### PDSCH DMRS Configuration

###### DMRS-DownlinkConfig

Table 5.4.2.0-24: DMRS-DownlinkConfig

| Derivation Path: Table 4.6.3-50 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-Type | Type 1 |  |  |
| dmrs-AdditionalPosition | pos1 |  |  |
| maxLength | len1 |  |  |
| phaseTrackingRS | Not present |  | DEMOD_FR1 |
| phaseTrackingRS SEQUENCE { |  |  | DEMOD_FR2 |
| epre-Ratio | 0 |  |  |
| resourceElementOffset | Offset10 |  |  |
| } |  |  |  |
| } |  |  |  |

##### PDSCH Configuration

###### PDSCH-ServingCellConfig

Table 5.4.2.0-25: PDSCH-ServingCellConfig

| Derivation Path: Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| codeBlockGroupTransmission | Not present |  |  |
| xOverhead | Not present |  |  |
| nrofHARQ-ProcessesForPDSCH | Set according to the test id | Typically n4 for FDD, n8 for TDD |  |
| pucch-Cell | Not present |  |  |
| } |  |  |  |

###### PDSCH-Config

Table 5.4.2.0-26: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPDSCH | 0 |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| Setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 2 entries |  |  |
| TCI-State[1] SEQUENCE { |  | entry 1 |  |
| tci-StateId | TCI-StateId 0 |  |  |
| qcl-type1 SEQUENCE { | QCL Type is Type1 |  |  |
| Cell | not present |  |  |
| Bwp-id | Not present | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 SEQUENCE { | QCL Type is Type2 |  | DEMOD_FR2 |
| Cell | not present |  |  |
| Bwp-id | Not present | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| TCI-State[2] SEQUENCE { |  | entry 2 |  |
| tci-StateId | TCI-StateId 1 |  |  |
| qcl-type1 { | QCL Type is Type1 |  |  |
| Cell | Not present |  |  |
| Bwp-id | 0 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 SEQUENCE { | QCL Type is Type2 |  | DEMOD_FR2 |
| Cell | Not present |  |  |
| Bwp-id | 0 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| vrb-ToPRB-Interleaver | Not present |  |  |
| resourceAllocation | resourceAllocationType0 |  |  |
| pdsch-AggregationFactor | Not present |  |  |
| rbg-Size | config2 | The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [21], clause 5.1.2.2.1). |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | Not present | PRB Bundling size of 2 |  |
| } |  |  |  |
| } |  |  |  |
| ZP-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| ZP-CSI-RS-Resource[1] | ZP-CSI-RS-Resource | entry 1 |  |
| } |  |  |  |
| p-ZP-CSI-RS-ResourceSet | p-ZP-CSI-RS-ResourceSet |  |  |
| } |  |  |  |

###### PDSCH-TimeDomainResourceAllocationList

Table 5.4.2.0-27: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 44 | Start symbol(S)=2, Length(L)=4 | For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |
|  | 96 | Start symbol(S)=1, Length(L)=9 | DEMOD_FR2 |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
|  | 53 | Start symbol(S)=2, Length(L)=12 | For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |
|  | 40 | Start symbol(S)=1, Length(L)=13 | DEMOD_FR2 |
| } |  |  |  |
| } |  |  |  |

##### CRS for Rate Matching

###### RateMatchPatternLTE-CRS

Table 5.4.2.0-28: RateMatchPatternLTE-CRS

| Derivation Path: Table 4.6.3-138 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  | TC 5.2.2.1.4-2 and TC 5.2.3.1.4-2 of TS 38.521-4 |
| carrierFreqDL | LTE EARFCN |  |  |
| carrierBandwidthDL | n50 | 10MHz |  |
| nrofCRS-Ports | n4 |  |  |
| v-Shift | n0 |  |  |
| } |  |  |  |

##### CSI-RS for beam refinement

###### NZP-CSI-RS-Resource

Table 5.4.2.0-29: NZP-CSI-RS-Resource

| Derivation Path: Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  | DEMOD_FR2 |
| nzp-CSI-RS-ResourceId | 5 | CSI-RS resource 6 | Beam refinement |
|  | 6 | CSI-RS resource 7 | Beam refinement |
| resourceMapping | CSI-RS-ResourceMapping |  |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-State #1 |  |  |
| } |  |  |  |

###### CSI-RS-ResourceMapping

Table 5.4.2.0-30: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  | DEMOD_FR2 |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | K0 = 0, row1 for resource 1 and 2 |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 8 | I0 = 8 for resource 1 |  |
|  | 9 | I0 = 9 for resource 2 |  |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

###### CSI-ResourcePeriodicityAndOffset

Table 5.4.2.0-31: CSI-ResourcePeriodicityAndOffset

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  | DEMOD_FR2 |
| Slots80 | 0 |  | SCS 60kHz |
| Slots160 | 0 |  | SCS 120kHz |
| } |  |  |  |

###### CSI-FrequencyOccupation

Table 5.4.2.0-32: CSI-FrequencyOccupation for beam refinement

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  | DEMOD_FR2 |
| nrofRBs | 68 |  | BW 100MHz SCS 120kHz |
| } |  |  |  |

###### NZP-CSI-RS-ResourceSet

Table 5.4.2.0-33: NZP-CSI-RS-ResourceSet for beam refinement

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  | DEMOD_FR2 |
| nzp_CSI_ResourceSetId | 2 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 5 | entry 1 CSI-RS resource 6 |  |
| NZP-CSI-RS-ResourceId[2] | 6 | entry 1 CSI-RS resource 7 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | Not present |  |  |
| } |  |  |  |

###### CSI-ResourceConfig

Table 5.4.2.0-34: CSI-ResourceConfig for beam refinement

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 2 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 2 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

##### CSI-RS for beam management

###### NZP-CSI-RS-Resource

Table 5.4.2.0-29: NZP-CSI-RS-Resource for beam management

| Derivation Path: Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  | PDCCH_FR2 |
| nzp-CSI-RS-ResourceId | 4 | CSI-RS resource 5 |  |
|  | 5 | CSI-RS resource 6 |  |
| resourceMapping | CSI-RS-ResourceMapping for beam management |  |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for beam management |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-State #1 |  |  |
| } |  |  |  |

###### CSI-RS-ResourceMapping

Table 5.4.2.0-30: CSI-RS-ResourceMapping for beam management

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  | PDCCH_FR2 |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | K0 = 0, row1 for resource 1 and 2 |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 8 | I0 = 8 for resource 1 |  |
|  | 9 | I0 = 9 for resource 2 |  |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation for beam management |  |  |
| } |  |  |  |

###### CSI-ResourcePeriodicityAndOffset

Table 5.4.2.0-31: CSI-ResourcePeriodicityAndOffset for beam management

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  | PDCCH_FR2 |
| Slots80 | 0 |  | SCS 60kHz |
| Slots160 | 0 |  | SCS 120kHz |
| } |  |  |  |

###### CSI-FrequencyOccupation

Table 5.4.2.0-32: CSI-FrequencyOccupation for beam management

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  | PDCCH_FR2 |
| nrofRBs | 68 |  | BW 100MHz SCS 120kHz |
| } |  |  |  |

###### NZP-CSI-RS-ResourceSet

Table 5.4.2.0-33: NZP-CSI-RS-ResourceSet for beam management

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  | PDCCH_FR2 |
| nzp_CSI_ResourceSetId | 1 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 4 | entry 1 CSI-RS resource 5 |  |
| NZP-CSI-RS-ResourceId[2] | 5 | entry 1 CSI-RS resource 6 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | Not present |  |  |
| } |  |  |  |

###### CSI-ResourceConfig

Table 5.4.2.0-34: CSI-ResourceConfig for beam management

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 1 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 1 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

#### 5.4.2.1	Message contents for PDSCH Demodulation requirements

##### ZP CSI-RS for CSI Acquisition

###### p-ZP-CSI-RS-ResourceSet

Table 5.4.2.1-0: p-ZP-CSI-RS-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-ZP-CSI-RS-ResourceSet CHOICE { |  |  |  |
| setup | ZP-CSI-RS-ResourceSet |  |  |
| } |  |  |  |

Table 5.4.2.1-1: ZP-CSI-RS-ResourceSet

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-ZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| zp_CSI-RS_ResourceSetId | 0 |  |  |
| zp-CSI-RS-ResourceIdList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId { |  | 1 entry |  |
| ZP-CSI-RS-ResourceId[1] | 0 | entry 1 ZP CSI-RS resource 1 |  |
| } |  |  |  |
| } |  |  |  |

##### CSI-MeasConfig

Table 5.4.2.1-2: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | dependent on test condition |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| NZP-CSI-RS-Resource[6] | CSI-RS-Resource for beam refinement | entry 5 CSI-RS resource 6 | DEMOD_FR2 |
| NZP-CSI-RS-Resource[7] | CSI-RS-Resource for beam refinement | entry 5 CSI-RS resource 7 | DEMOD_FR2 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | n entries | dependent on test condition |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2 |  |
| NZP-CSI-RS-ResourceSet[3] | CSI-RS-ResourceSet for beam refinement | entry 3 | DEMOD_FR2 |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | n entries | dependent on test condition |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for CSI Acquisition | entry 2 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for beam refinement | entry 3 | DEMOD_FR2 |
| } |  |  |  |
| } |  |  |  |

#### 5.4.2.2	Message contents for PDCCH Demodulation requirements

##### NZP-CSI-RS for Tracking

###### CSI-RS-ResourceMapping

Table 5.4.2.2-1: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 | l0 = 4 for CSI-RS resource 1 and 3 | TRS |
|  | 8 | l0 = 8 for CSI-RS resource 2 and 4 | TRS |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 | TRS |
| Cdm-Type | noCDM |  | TRS |
| Density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  | TRS |
| } |  |  |  |

##### CSI-MeasConfig

Table 5.4.2.2-2: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | Dependent on test condition |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for beam management (5) | entry 4 CSI-RS resource 5 | PDCCH_FR2 |
| NZP-CSI-RS-Resource[6] | NZP-CSI-RS-Resource for beam management (6) | entry 4 CSI-RS resource 6 | PDCCH_FR2 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for beam management | entry 1 | PDCCH_FR2 |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for beam management | entry 1 | PDCCH_FR2 |
| } |  |  |  |
| } |  |  |  |

##### PDSCH Configuration

###### PDSCH-Config

Table 5.4.2.2-3: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPDSCH | 0 |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| Setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 2 entries |  |  |
| TCI-State[1] SEQUENCE { |  | entry 1 |  |
| tci-StateId | TCI-StateId 0 |  |  |
| qcl-type1 SEQUENCE { | QCL Type is Type1 |  |  |
| Cell | Not Present |  |  |
| Bwp-id | Not present | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 SEQUENCE { | QCL Type is Type2 |  |  |
| Cell | Not Present |  |  |
| Bwp-id | Not present | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| TCI-State[2] SEQUENCE { |  | entry 2 |  |
| tci-StateId | TCI-StateId 1 |  |  |
| qcl-type1 SEQUENCE { | QCL Type is Type1 |  |  |
| Cell | Not Present |  |  |
| Bwp-id | 0 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 SEQUENCE { | QCL Type is Type2 |  |  |
| Cell | Not Present |  |  |
| Bwp-id | 0 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| vrb-ToPRB-Interleaver | Not present |  |  |
| resourceAllocation | resourceAllocationType0 |  |  |
| pdsch-AggregationFactor | Not present |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | Not present | PRB Bundling size of 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 5.4.2.3	Message contents for Sustained downlink data rate requirements

FFS

#### 5.4.2.4	Message contents for CQI reporting requirements

##### NZP-CSI-RS for Tracking

###### CSI-RS-ResourceMapping

Table 5.4.2.4-1: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 | l0 = 4 for CSI-RS resource 1 and 3 | TRS |
|  | 8 | l0 = 8 for CSI-RS resource 2 and 4 | TRS |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 | TRS |
| Cdm-Type | noCDM |  | TRS |
| Density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  | TRS |
| } |  |  |  |

##### NZP CSI-RS for CSI Acquisition

###### CSI-RS-ResourceMapping

Table 5.4.2.4-2: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001000 | K0 = 6, row3, |  |
| } |  |  |  |
| nrofPorts | P2 |  |  |
| firstOFDMSymbolInTimeDomain | 13 | I0 = 13 |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

##### ZP CSI-RS for CSI Acquisition

###### CSI-RS-ResourceMapping

Table 5.4.2.4-3: ZP CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 | K0 = 4 |  |
| } |  |  |  |
| nrofPorts | P4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 | I0 = 9 |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | ZP CSI-FrequencyOccupation |  |  |
| } |  |  |  |

###### CSI-ResourcePeriodicityAndOffset

Table 5.4.2.4-4: ZP CSI-ResourcePeriodicityAndOffset

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots5 | 1 |  |  |
| } |  |  |  |

###### p-ZP-CSI-RS-ResourceSet

Table 5.4.2.4-5: p-ZP-CSI-RS-ResourceSet

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-ZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| zp_CSI-RS_ResourceSetId | 0 |  |  |
| zp-CSI-RS-ResourceIdList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId{ | 1 entry |  |  |
| ZP-CSI-RS-ResourceId[1] | 0 | entry 1 ZP CSI-RS resource 1 |  |
| } |  |  |  |
| } |  |  |  |

##### CSI-IM Configuration

###### CSI-IM-Resource

Table 5.4.2.4-6: CSI-IM-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId |  |  |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| Pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  | FR1 |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| periodicityAndOffset | Not present |  |  |
| } |  |  |  |

###### CSI-IM-ResourceId

Table 5.4.2.4-7: CSI-IM-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceId | 0 |  |  |

###### CSI-IM-ResourceSet

Table 5.4.2.4-8: CSI-IM-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId |  |  |
| csi-IM-Resources SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) { | 1 entry |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId |  |  |
| } |  |  |  |
| } |  |  |  |

###### CSI-IM-ResourceSetId

Table 5.4.2.4-9: CSI-IM-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | 0 |  |  |

###### CSI-IM-ResourceConfig

Table 5.4.2.4-10: CSI-IM-ResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId { | 1 entry |  |  |
| csi-IM-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

##### CSI-MeasConfig

Table 5.4.2.4-11: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 5 entries |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | 2 entries |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2 |  |
| } |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource |  |  |
| } |  |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 3 entries |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for CSI Acquisition | entry 2 |  |
| CSI-ResourceConfig[3] | CSI-IM-ResourceConfig | entry 3 |  |
| } |  |  |  |
| } |  |  |  |

##### CSI-ReportConfig

Table 5.4.2.4-12: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId for CSI Acquisition |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId for CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig | CSI-ReportPeriodicityAndOffset |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { |  |  |  |
| PUCCH-CSI-Resource [1] | PUCCH-CSI-Resource |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL, |  | FR1 |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | '1111111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table2 |  | FR1 |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

###### CSI-ReportPeriodicityAndOffset

Table 5.4.2.4-13: CSI-ReportPeriodicityAndOffset

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportPeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 9 |  | FR1_TDD |
| slots5 | 0 |  | FR1_FDD |
| } |  |  |  |

###### PUCCH-CSI-Resource

Table 5.4.2.4-14: PUCCH-CSI-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-CSI-Resource ::= SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-id |  | FR1 |
| pucch-Resource | 8 |  | FR1 |
| } |  |  |  |

###### CodebookConfig

Table 5.4.2.4-15: CodebookConfig

| Derivation Path: Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| two CHOICE { |  |  |  |
| twoTX-CodebookSubsetRestriction | '010000'B |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | ‘11111111’B |  |  |
| } |  |  |  |
| } |  |  |  |
| codebookMode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 5.4.2.5	Message contents for PMI reporting requirements

###### NZP-CSI-RS for Tracking

FFS

###### CSI-RS-ResourceMapping

Table 5.4.2.5-1: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 | l0 = 4 for CSI-RS resource 1 and 3 | TRS |
|  | 8 | l0 = 8 for CSI-RS resource 2 and 4 | TRS |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 | TRS |
| cdm-Type | noCDM |  | TRS |
| density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  | TRS |
| } |  |  |  |

###### NZP CSI-RS for CSI Acquisition

FFS

###### CSI-RS-ResourceMapping

Table 5.4.2.5-2: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row4 | 001 | K0 = 0, row4 | 4Tx |
| other | 001100 | K0 = 4, K1 = 6, row8 | 8Tx |
| } |  |  |  |
| nrofPorts | p4 |  | 4Tx |
|  | p8 |  | 8Tx |
| firstOFDMSymbolInTimeDomain | 13 | I0 = 13 | 4Tx |
|  | 5 | I0 = 5 | 8Tx |
| cdm-Type | fd-CDM2 |  | 4Tx |
|  | cdm4-FD2-TD2 |  | 8Tx |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 4Tx | For test cases using 4 CSI-RS ports for NZP-CSI-RS for CSI acquisition. |
| 8Tx | For test cases using 8 CSI-RS ports for NZP-CSI-RS for CSI acquisition. |

###### ZP CSI-RS for CSI Acquisition

FFS

###### CSI-RS-ResourceMapping

Table 5.4.2.5-3: ZP CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 | K0 = 4 |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 | I0 = 9 |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | ZP CSI-FrequencyOccupation |  |  |
| } |  |  |  |

###### PDSCH-Config

Table 5.4.2.5-4: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList | Ap-ZP-CSI-RS-ResourceSet |  |  |
| p-ZP-CSI-RS-ResourceSet | Not present |  |  |
| } |  |  |  |

###### Ap-ZP-CSI-RS-ResourceSet

Table 5.4.2.5-5: Ap-ZP-CSI-RS-ResourceSet

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList ::= SEQUENCE { |  |  |  |
| zp_CSI-RS_ResourceSetId | 0 |  |  |
| zp-CSI-RS-ResourceIdList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId{ | 1 entry |  |  |
| ZP-CSI-RS-ResourceId[1] | 0 | entry 1 ZP CSI-RS resource 1 |  |
| } |  |  |  |
| } |  |  |  |

###### CSI-ResourceConfig

Table 5.4.2.5-6: CSI-ResourceConfig for CSI Acquisition

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 1 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 1 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | aperiodic |  |  |
| } |  |  |  |

###### CSI-IM Configuration

FFS

###### CSI-IM-Resource

Table 5.4.2.5-7: CSI-IM-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId |  |  |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| Pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  | FR1 |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| periodicityAndOffset | Not present |  |  |
| } |  |  |  |

###### CSI-IM-ResourceId

Table 5.4.2.5-8: CSI-IM-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceId | 0 |  |  |

###### CSI-IM-ResourceSet

Table 5.4.2.5-9: CSI-IM-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId |  |  |
| csi-IM-Resources SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) { | 1 entry |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

###### CSI-IM-ResourceSetId

Table 5.4.2.5-10: CSI-IM-ResourceSetId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | 0 |  |  |

###### CSI-IM-ResourceConfig

Table 5.4.2.5-11: CSI-IM-ResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId { | 1 entry |  |  |
| csi-IM-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | aperiodic |  |  |
| } |  |  |  |

##### CSI-MeasConfig

Table 5.4.2.5-12: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 5 entries |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | 2 entries |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2 |  |
| } |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource |  |  |
| } |  |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 3 entries |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for CSI Acquisition | entry 2 |  |
| CSI-ResourceConfig[3] | CSI-IM-ResourceConfig | entry 3 |  |
| } |  |  |  |
| } |  |  |  |

##### CSI-ReportConfig

Table 5.4.2.5-13: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId for CSI Acquisition |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId for CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE { | 2 entry |  |  |
| reportSlotOffsetList[1] | 4 |  | FDD_4Tx |
|  | 5 |  | FDD_8Tx |
|  | 8 |  | TDD |
| reportSlotOffsetList[2] | 4 |  | FDD_4Tx |
|  | 5 |  | FDD_8Tx |
|  | 8 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | '1111111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table1 |  |  |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

###### CodebookConfig

Table 5.4.2.5-14: CodebookConfig

| Derivation Path: Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| two-one-TypeI-SinglePanel-Restriction | '11111111'B |  |  |
| } |  |  |  |
| typeI-SinglePanel-codebookSubsetRestriction-i2 | Not present |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | '00000001'B |  | 4Tx |
|  | '00000010'B |  | 8Tx |
| } |  |  |  |
| codebookMode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### CSI-AperiodicTriggerStateList

Table 5.4.2.5-15: CSI-AperiodicTriggerStateList

| Derivation Path: Table 4.6.3-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState { | 1 entry |  |  |
| CSI-AperiodicTriggerState[1] SEQUENCE { |  | entry 1 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 0 |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 1 |  |  |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesforInteference | 1 |  |  |
| nzp-CSI-RS-ResourcesforInterference | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## 5.5	Common procedures for RF testing

### 5.5.1	Procedure to configure SCC for NR RF CA testing

5.5.1.1	Scope

The purpose of this procedure is to establish one or more SCC for NR CA testing.

5.5.1.2	Procedure description

5.5.1.2.1	Initial conditions

UE is operating in NR RRC_CONNECTED state on NR Cell 1 without any SCell configured.

System Simulator:

-	SS configures the number of SCells used by the test case using NR parameters for NR Cell 2 for SCC1, NR Cell 3 for SCC2, NR Cell 4 for SCC3 etc. as specified in Table 4.4.2-2.

-	System information combination NR-2 as defined in clause 4.4.3.1.2 is used in all NR cells.

5.5.1.2.2	Procedure sequence

Table 5.5.1.2.2-1: Procedure to configure SCC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including sCellToAddModList with SCell addition for the SCC under test. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

5.5.1.2.3	Specific message contents

Table 5.5.1.2.3-1: RRCReconfiguration-SCell(n)

| Derivation Path: Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-SCell(n) | n is number of SCC to be added. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 5.5.1.2.3-2: CellGroupConfig-SCell(n) (Table 5.5.1.2.3-1)

| Derivation Path: Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | n entries | n the number of SCC to be added |  |
| SCellConfig[k, k=1..n] SEQUENCE { |  | entry (1..n) |  |
| sCellIndex |  |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add |  |  |
| } |  |  |  |
| } |  |  |  |

### 5.5.2	Procedure to configure SCC for EN-DC RF CA testing

5.5.2.1	Scope

The purpose of this procedure is to establish one or more SCC for EN-DC CA testing.

5.5.2.2	Procedure description

5.5.2.2.1	Initial conditions

The UE is in RRC_CONNECTED state.

5.5.2.2.2	Procedure sequence

Table 5.5.2.2.2-1: Procedure to configure SCC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message using n. | <-- | RRC: RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 3a1 to 3a3 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 3a1-3a3 | IF E-UTRA SCC > 0, same as TS 36.508 [2] table 5.2A.4-1, steps 1-3. | - | - | - | - |
| Note 1: n > 0 in step 1. |  |  |  |  |  |

5.5.2.2.3	Specific message contents

Table 5.5.2.2.2.3-1: RRCReconfiguration (step 1, Table 5.5.2.2.2-1)

| Derivation Path: Table 4.6.1-13 with condition EN-DC_SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig (n) | n is number of SCC to be added. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 5.5.2.2.2.3-2: CellGroupConfig (n) (Table 5.5.2.2.2.3-1)

| Derivation Path: Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | n entries | n is equal to the number of SCCs to be added |  |
| SCellConfig[ k, k=1..n] |  | entry (1..n) |  |
| sCellIndex |  |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add |  |  |
| sCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# 6	Test environments for Signalling test

## 6.1	Requirements of test equipment

### 6.1.1	Requirements common for conducted and OTA tests

The requirements of test equipment specified in this subclause apply to Signalling test cases defined in TS 38.523-1 [12], in addition to the common requirements of test equipment specified in clause 4.2 of this specification.

Test equipment shall be able to simulate cells of Radio Access Technologies NR and E-UTRA, The number of cells to be simulated simultaneously by the test equipment shall not exceed the resources specified in Table 6.1-1

Table 6.1-1: Maximum resources in terms of number / configuration of cells to be simulated simultaneously in a test setup

| Simulation of | Max. number of cells (NR) |  | Max. number of cells (E-UTRA) |
| --- | --- | --- | --- |
|  | Conducted | OTA |  |
| NR single-mode networks (FDD or TDD) | 4 cells | FFS | n/a |
| NR dual-mode networks (FDD and TDD) | 4 cells | FFS | n/a |
| NR networks involving Carrier Aggregation | 4 cells | FFS | n/a |
| NR dual connectivity (NR-DC) | 4 cells | FFS | n/a |
| NR dual connectivity (EN-DC) | 4 cells | FFS | 2 cells |
| NR dual connectivity (EN-DC) involving Carrier Aggregation | 4 cells | FFS | 2 cells |
| Mixed E-UTRA / NR networks | 4 cells | FFS | 2 cells |
| Note 1: No differentiation between cell configuration types (as defined in clause 6.3.1) here, because these types are only relevant to specific test cases and their TTCN-3 implementation. Note 2: Only network scenarios specified in clauses 4.4.1 and 6.3.2.1 have been covered. Note 3: In case of Carrier Aggregation, each cell can act as a SpCell, an SCell, or a standalone cell (not used as a CA component carrier). Note 4: In order to support test case requirements for conducted and OTA test methods, the number of active cells at any given time should be minimised in order to ensure maximum re use of SS Tx/Rx resources. |  |  |  |

Exceptions to the requirements outlined above are possible but need special evidence to be provided explicitly in the test case prose and should be allowed only if the test case purpose cannot be met otherwise.

Due to limited power level range for FR2 OTA test methods, when defining test cases requirements, care shall be taken to ensure that the number of active cells is minimised as this has an impact to have distinguishable power level difference. Cells that are used in initial parts of test cases and are no longer required for the rest of the procedure shall be clearly defined as Non-suitable "Off" cell to facilitate re use of SS Tx/Rx resources

### 6.1.2	Requirements for conducted test method

Editor’s Note: This subclause is intended to describe the test equipment requirements which are specific to conducted test environment for signalling tests.

### 6.1.3	Requirements for OTA test method

#### 6.1.3.1	General

Editor’s Note: The UE pre-configuration mentioned below to disable UL Tx diversity schemes shall be voided once a test methodology solution to minimize spectral flatness artefacts between TE and UE over all test points is defined.

The DFF or IFF based OTA test methodologies, defined in Annex B.1 should be used for Signalling test.

NOTE:	For single cell test cases, usage of NF test methodology is not precluded.

Clause 6.1.3.2 describes a sample OTA measurement test setup and clause 6.1.3.3 describes approaches to select a UE orientation.

For conformance testing using the OTA test environment, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization. This approach also applies to calibration stage..

#### 6.1.3.2	Sample OTA Measurement Test Setup

Please refer to Figure 6.1.3.2-1 for a sample OTA measurement test setup.

Figure 6.1.3.2-1: Sample OTA measurement setup

NOTE:	Figure 6.1.3.2-1 is for illustrative purposes only.

For 5G NR signalling test cases, depending on the dynamic range of measurements the system complexity can be reduced. In the switch unit, as shown in Figure 6.1.3.2-1, the switches K7, K8, K9, K10 can be removed. The amplifier (PA/LNA) is optional. For the "single cell" and "multiple cell" test cases, the gNB emulator can be directly connected to the feed horn.

#### 6.1.3.3	Procedure for selecting UE Orientation and for calibration

Set calibrated power level at the centre of the quiet zone for each polarization individually [FFS].

Before starting the test, the UE orientation with which the test system can provide a wide enough dynamic range to perform the test scenarios needs to be identified in order to obtain sufficient link budget.

The UE orientation can be determined by either of the approaches below:

-	Approach 1: UE vendor declares the direction in which the measurement has to be made. In this case, the declaration confirms that the Rx Beam peak conditions in FR2 specified in TS 38.133 [13] Annex B are met

-	Approach 2: Perform an Rx-beam peak search

For Approach 2 an Rx beam peak search needs to be performed as per the procedure in TS 38.521-2 [15] Annex K, which finds the direction in which Rx Beam peak conditions in FR2 specified in TS 38.133 [13] Annex B are met.

RSRP measurements can be configured by SS in X2NR meas configurations using FFS preambles in NSA (Ex - RRC_Connected with connectivity parameter E-UTRA with MCG Only bearer established and meas config enabled for event B1 (Ex-per TC 8.2.3.1.1 of TS 38.523-1)) and FFS preambles in SA modes.

When signal level calibrated with a reference antenna (only applicable to single-cell test cases without thresholds):

-	The RSRP reported from the DUT is within [±FFSdB] of expected RSRP mentioned in Table 6.2.2.2-1.

When signal level calibrated with the RSRP-based calibration:

-	Before starting the tests, Rx-beam peak directions need to be determined using Approach 1 or Approach 2 above. Rx beam peak direction may depend on the operating band under test. If Rx-beam peak directions for all the operating bands required for test scenarios are identical, three different levels in Table 6.2.2.2-2 can be used in the test scenarios.

-	Rx-beam peak directions are decided to be ‘identical’, if the detected beam peak positions are direct neighbours on the measurement grid.

If Rx-beam peak directions are identical:

1.	Position the UE so that the Rx beam peak direction is aligned towards the measurement antenna.

2.	Make the UE report SS-RSRP at each frequency used in the test scenarios, while setting the downlink SS power at the centre of the quiet zone to -82dBm/SCS. Here, the SS-RSRP reported levels are denoted as PRSRP(f).

3.	Calculate ‘Delta(NRf)’ for each carrier frequency used in the test case, using the equation: Delta(NRf) = PRSRP(f) + 82.

#### 6.1.3.4	Handling of Thresholds

Where a threshold value is specified in the test case (value identified as THtest case) it is signalled to the UE with a value THsig according to table 6.1.3.4-1.

Table 6.1.3.4-1: Handling of signalled threshold values

| Type of Threshold | Signalled value | Comment |
| --- | --- | --- |
| Absolute | THsig(f) = THtest case + Delta(NRf) | Delta(NRf) value according to the frequency of the cell being compared to the threshold |
| Relative, intra-frequency | THsig(f) = THtest case |  |
| Relative, inter-frequency | a3-Offset sig = a3-Offset test case + Delta(NRfn) - Delta(NRfp) | Delta(NRfp) is the Delta value for the SpCell frequency Delta(NRfn) is the Delta value for the neighbour cell frequency |
| Relative, inter-frequency, Event A6 | a6-Offset sig = a6-Offset test case + Delta(NRfn) - Delta(NRfs) | Delta(NRfs) is the Delta value for the SCell frequency Delta(NRfn) is the Delta value for the neighbour cell frequency |

### 6.1.4	Requirements for timer tolerances

The timer tolerances specified for the test environment in this subclause apply to all Signalling test cases defined in TS 38.523-1 [12] unless otherwise specified.

All the timers used during testing are within a tolerance margin given by the equation below. If for a specific test a different tolerance value is required, then this should be specified in the relevant test document (i.e. the document where the test is described).

Timer tolerance = 10%.

## 6.2	Reference test conditions

### 6.2.1	Physical Channel Allocations

#### 6.2.1.1	Antennas

If the UE has two or more Rx antennas, the same downlink signal is applied to each one, except if MIMO is tested. All UE Rx antennas shall be connected.

If the UE has one Rx antenna, the downlink signal is applied to it.

#### 6.2.1.2	Downlink physical channels and physical signals

In general for signalling test cases the power allocation for downlink physical channels and signals is specified in relation to a reference cell power.

Unless specifically specified otherwise in a signalling test case prose, all cells use only one beam.

In case of only one beam per cell this reference cell power is the EPRE of the secondary synchronization signal (SSS) and referred to as “SS/PBCH SSS EPRE”.

In case of more than one beam per cell the power levels of the different SS/PBCH blocks may be different what makes it difficult to specify the EPREs of other physical channels and signals relative to the EPRE of any SSS. Therefore for multiple beams test cases the power levels are specified relative to the reference cell power.

For single beam per cell test cases the power allocation of downlink physical channels for signalling test cases is specified in table 6.2.1.2-1, for multiple beams per cell test cases the power allocation is specified in table 6.2.1.2-2.

Table 6.2.1.2-1: Power allocation for OFDM symbols and reference signals for signalling test cases (single beam)

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SSS transmit power | dBm/SCS | Test specific (Note 1) | referred to as “SS/PBCH SSS EPRE” |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |  |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |  |
| EPRE ratio of PDCCH DMRS to SSS | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS | dB | 0 |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB | -3 | To reduce interference from PDSCH of intra-frequency neighbour cells. |
| EPRE ratio of PTRS to PDSCH | dB | 3 | i.e. the EPRE ratio of PTRS to SSS is 0dB |
| Note 1: Power level chosen to align with cell power level as specified in clause 6.2.2. |  |  |  |

Table 6.2.1.2-2: Power allocation for OFDM symbols and reference signals for signalling test cases (multiple beam)

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell power EPRECellRef | dBm/SCS | Test specific (Note 1) |  |
| EPRE ratio of SSSSSB#N to EPRECellRef | dB | Test specific (Note 2) | power of SSS within SSB with index N |
| EPRE ratio of PSSSSB#N to SSSSSB#N | dB | 0 | power of PSS within SSB with index N |
| EPRE ratio of PBCH DMRSSSB#N to SSSSSB#N | dB | 0 | power of PBCH DMRS within SSB with index N |
| EPRE ratio of PBCHSSB#N to PBCH DMRSSSB#N | dB | 0 | power of PBCH within SSB with index N |
| EPRE ratio of PDCCH DMRS to EPRECellRef | dB | 0 | (Note 3) |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to EPRECellRef | dB | 0 | (Note 3) |
| EPRE ratio of PDSCH to PDSCH DMRS | dB | -3 | To reduce interference from PDSCH of intra-frequency neighbour cells. |
| EPRE ratio of PTRS to PDSCH | dB | 3 | i.e. the EPRE ratio of PTRS to EPRECellRef is 0dB |
| EPRE ratio of CSI-RSN to EPRECellRef | dB | Test specific (Note 2) | power of CSI-RS with index N; CSI-RS configured if required by a test case in TS 38.523-1 [12] |
| Note 1: Power level chosen to align with cell power level as specified in clause 6.2.2. Note 2: Test cases may specify “OFF” in which case the attenuation shall result in an absolute EPRE value being equal or less than the power level specified for a non-suitable "Off" cell in clause 6.2.2. Note 3: In general the UE cannot distinguish from which beam DL data is sent PDCCH and PDSCH are considered as cell specific rather than beam specific. |  |  |  |

### 6.2.2	Signal levels

#### 6.2.2.1	Signal Levels for conducted testing

For NR FR1 cell, the downlink power settings in Table 6.2.2.1-1 and 6.2.2.1-2 are used unless otherwise specified in a test case.

Table 6.2.2.1-1: Default Downlink power levels for FR1 NR cell (5MHz – 25MHz)

|  | SCS(kHz) | Unit | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz |
| Channel BW Power | 15 | dBm | -63 | -60 | -58 | -57 | -56 |
|  | 30 | dBm | -67 | -63 | -61 | -60 | -59 |
|  | 60 | dBm | N/A | -67 | -65 | -63 | -62 |
| SS/PBCH SSS EPRE | All | dBm/SCS (Note 3) | -88 | -88 | -88 | -88 | -88 |
| Note 1: The channel bandwidth powers are informative, based on -88 dBm/ SCS(SubCarrier Spacing) SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at each UE Rx antenna. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of -88 dBm/SCS(SubCarrier Spacing). |  |  |  |  |  |  |  |

Table 6.2.2.1-2: Default Downlink power levels for FR1 NR cell (30MHz – 100MHz)

|  |  | SCS(kHz) |  | Unit |  | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 30MHz | 40MHz | 50MHz | 60MHz | 80MHz | 90MHz | 100MHz |
| Channel BW Power |  | 15 |  | dBm |  | -55 |  | -54 |  |  | -53 |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 30 |  | dBm |  | -58 |  | -57 |  |  | -56 |  | -55 |  | -54 |  | -53 |  | -53 |  |
|  |  | 60 |  | dBm |  | -61 |  | -60 |  |  | -59 |  | -58 |  | -57 |  | -56 |  | -56 |  |
| SS/PBCH SSS EPRE |  | All |  | dBm/SCS (Note 3) |  | -88 |  | -88 |  |  | -88 |  | -88 |  | -88 |  | -88 |  | -88 |  |
| Note 1: The channel bandwidth powers are informative, based on -88dBm/SCS(SubCarrier Spacing) SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at each UE Rx antenna. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with a power spectrum density of -88dBm/SCS(SubCarrier Spacing). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

With simultaneous transmission of 24 RBs, a maximum of -78dBm/SCS SS/PBCH SSS EPRE can be allocated as cell power level.

The default settings of suitable cells and non-suitable cells for NR are specified in table 6.2.2.1-3.

Cells which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.1-3.

Table 6.2.2.1-3: Default settings of suitable / non-suitable cells

| Power level type | NR (Note 1-3) |  | E-UTRAN |
| --- | --- | --- | --- |
|  | Unit | Power level |  |
| Serving cell | dBm/SCS | -88 | Table 6.2.2.1-1 [2] |
| Suitable neighbour intra-frequency cell | dBm/SCS | -94 | Table 6.2.2.1-1 [2] |
| Suitable neighbour inter-frequency cell | dBm/SCS | -99 | Table 6.2.2.1-1 [2] |
| Non-suitable cell | dBm/SCS | -115 | Table 6.2.2.1-1 [2] |
| Non-suitable "Off" cell | dBm/SCS | ≤ -145 | Table 6.2.2.1-1 [2] |
| Note 1: The power level is specified in terms of SS/PBCH SSS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the Full RE allocation with no boost or deboost is assumed. SS. Note 2: The power level is specified at each UE Rx antenna. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of -88dBm/SCS. Note 4: The default settings assume that the UE is making relative measurements of neighbour cells compared to the serving cell. |  |  |  |

The default signal level uncertainty is specified in table 6.2.2.1-4 for any level specified, unless a tighter uncertainty is specified by a test case in TS 38.523-1 [12].

Table 6.2.2.1-4: SS signal level uncertainty

|  | Absolute signal level uncertainty for each cell | Relative signal level uncertainty between multiple cells |
| --- | --- | --- |
| Intra-frequency | +/-3 dB at each test port | +/-3 dB |
| Inter-frequency | +/-3 dB at each test port | See Note 1 |
| Note 1: For Inter-frequency cells the relative signal level uncertainty between multiple cells is determined by the absolute uncertainty of each cell, and does not have any additional constraint. |  |  |

SS/PBCH SSS EPRE setting should be equal to or higher than -115 dBm except for Non-suitable "Off" cell. The figure is chosen to ensure that for all bands the DL signal is within the RSRP measurement range specified in TS 38.133 [13], taking into account the SS default absolute signal level uncertainty.

NOTE:	(The power spectral density of a white noise source; specified in TS 38.133 [13]) can be assumed to be -Infinity [dBm/SCS] for all intra and inter frequency test cases. It is applicable to both idle mode and connected mode in TS 38.523-1 [12], unless otherwise specified in specific test cases.

##### 6.2.2.1.1	Measurement accuracy and side conditions

RSRP measurement accuracy in RRC_CONNECTED state is specified in table 6.2.2.1.1-1, derived from TS 38.133 [13] clauses 10.1.2 and 10.1.4 selecting Normal condition with maximum Io less than -50 dBm/BWChannel. The ranges and side conditions in TS 38.133 [13] clauses 10.1.2 and 10.1.4 apply. This measurement accuracy is applicable to connected mode test cases specified in TS 38.523-1 [12]. For the serving cell and suitable neighbour cells, the following side conditions shall be satisfied including the effect of signal level uncertainty.

- RSRP  -124 dBm

- RSRP Ês/Iot > -6 dB

- Io: 117.5 dBm/SCS for 15kHz SCS and -114.5 dBm/SCS for 15kHz SCS dBm/SCS … -50 dBm/BWChannel (for absolute and relative RSRP measurement accuracy).

RSRP measurement accuracy in RRC_CONNECTED state is specified in table 6.2.2.1.1-1, derived from TS 38.133 [13] clauses 10.1.2 and 10.1.4 selecting Normal condition.

Table 6.2.2.1.1-1: RSRP measurement accuracy in RRC_CONNECTED state

|  | Absolute RSRP measurement accuracy | Relative RSRP measurement accuracy |
| --- | --- | --- |
| Intra-frequency | +/-8 dB | +/-3 dB |
| Inter-frequency | +/-8 dB | +/-4.5 dB |

#### 6.2.2.2	Signal Levels for OTA testing

Editor’s Note:	It is FFS whether some or all of the restrictions listed in the working assumptions below will be removed.

The power levels defined in this clause are based on the following assumptions:

-	No more than one E-UTRA cell is configured in the test case and this E-UTRA cell uses an uncalibrated OTA link with no power level change.

-	No more than one NR FR1 cell is configured in the test case and this NR FR1 cell uses an uncalibrated OTA link with no power level change.

-	AWGN is not configured in the test case

For NR FR2 cell, the downlink power settings in Table 6.2.2.2-1 are used unless otherwise specified in a test case.

Table 6.2.2.2-1: Default Downlink power levels for FR2 NR cell (50MHz - 400MHz)

|  | SCS(kHz) | Unit | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |
| Channel BW Power | 60 | dBm | FFS | FFS | FFS | FFS |
|  | 120 | dBm | -57 | -57 | -57 | -57 |
| SS/PBCH SSS EPRE | All | dBm/SCS | -82 | -82 | -82 | -82 |
| Note 1: The channel bandwidth powers are informative, based on -82 dBm/SCS SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. A maximumRE allocation of 24 simultaneously transmitted RBs with no boost or deboost is assumed. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |  |  |  |

The default settings of suitable cells and non-suitable cells for NR FR2 are specified in table 6.2.2.2-2.

NR FR2 cells which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.2-2.

Table 6.2.2.2-2: Default settings of suitable / non-suitable FR2 NR cells

| Power level type | NR (Note 1-3) |  |
| --- | --- | --- |
|  | Unit | Power level |
| Serving cell | dBm/SCS | -82 |
| Suitable neighbour intra-frequency cell | dBm/SCS | -91 |
| Suitable neighbour inter-frequency cell | dBm/SCS | -91 |
| Non-suitable cell | dBm/SCS | -100 |
| Non-suitable "Off" cell | dBm/SCS | ≤-139 |
| Note 1: The power level is specified in terms of SS/PBCH SSS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: The power level is specified at the centre of quiet zone. Note 3: DL level is applied for any of the Subcarrier Spacing configuration (µ) with the same power spectrum density in dBm/SCS (SubCarrier Spacing). |  |  |  |

For E-UTRA cell with FR2 NR, since the LTE OTA link is uncalibrated in the signalling test setup, the table 6.2.2.2-3 provides only suggestive value. It is left to the TE vendor to ensure that LTE cell power level fulfils the cell selection criteria.

Table 6.2.2.2-3: Default Downlink power levels for E-UTRA cells with NR FR2

|  | Unit | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Number of RBs |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Channel BW Power | dBm | -77 | -73 | -71 | -68 | -66 | -65 |
| RS EPRE | dBm/15kHz | -96 | -96 | -96 | -96 | -96 | -96 |
| Note 1: The channel bandwidth powers are informative, based on -96 dBm/15kHz RS_EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |  |  |  |  |

The default settings of suitable cells and non-suitable cells for E-UTRA with FR2 NR are specified in table 6.2.2.2-4.

E-UTRA Cells with FR2 NR which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.2-4.

Table 6.2.2.2-4: Default settings of suitable / non-suitable E-UTRA cells with NR FR2

| Power level type | E-UTRAN (Note 1-2) |  |
| --- | --- | --- |
|  | Unit | Power level |
| Serving cell | dBm/15KHz | -96 |
| Suitable neighbour intra-frequency cell | dBm/15KHz | TBD |
| Suitable neighbour inter-frequency cell | dBm/15KHz | TBD |
| Non-suitable cell | dBm/15KHz | TBD |
| Non-suitable "Off" cell | dBm/15KHz | TBD |
| Note 1: The power level is specified in terms ofcell-specific RS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |

The Test system default signal level uncertainty is specified in tables 6.2.2.2-5 and 6.2.2.2-6 for any level specified, unless a tighter uncertainty is specified by a test case in TS 38.523-1 [12].

Table 6.2.2.2-5: SS Absolute FR2 NR signal level uncertainty

|  | Absolute signal level uncertainty |
| --- | --- |
| At each frequency | +/-6 dB at centre of the quiet zone |

Table 6.2.2.2-6: SS Relative FR2 NR signal level uncertainty

|  | Relative signal level uncertainty between any two SS EPRE levels at the same frequency |
| --- | --- |
| At each frequency | +/-2.0 dB |

For NR cell in FR1 with FR2 NR, since the NR FR1 OTA link is uncalibrated in the signalling test setup, -88dBm/SCS should be applied as defined in the table 6.2.2.1-1 and table 6.2.2.1-2. -88dBm/SCS is suggested value and it is left to the TE vendor to ensure that NR cell power level fulfils the cell selection criteria.

The default settings of suitable cells and non-suitable cells for NR in FR1 with FR2 NR are specified in table 6.2.2.2-7.

NR Cells in FR1 with FR2 NR which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.2-7.

Table 6.2.2.2-7: Default settings of suitable / non-suitable NR cells in FR1 with NR FR2

| Power level type | NR (Note 1-2) |  |
| --- | --- | --- |
|  | Unit | Power level |
| Serving cell | dBm/SCS | -88 |
| Suitable neighbour intra-frequency cell | dBm/SCS | TBD |
| Suitable neighbour inter-frequency cell | dBm/SCS | TBD |
| Non-suitable cell | dBm/SCS | TBD |
| Non-suitable "Off" cell | dBm/SCS | TBD |
| Note 1: The power level is specified in terms of SS/PBCH SSS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |

### 6.2.3	Default test frequencies

Editor’s note:	For FR2 test frequencies using 100 MHz default channel bandwidth it is FFS if 100MHz channel bandwidth can be used for FR2 multicell protocol testing.

#### 6.2.3.1	Test frequencies for NR standalone signalling testing

The default channel bandwidth for signalling test is specified per NR band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For signalling test cases, the mapping of frequency ranges to NR test frequencies are as follows:

-	for band with only one test frequency (e.g. n51): Low Range (NRf1);

-	for band with up to two test frequencies: Low Range (NRf1) and High Range (NRf2);

-	for band with up to three test frequencies: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3);

-	for band with up to four test frequencies: Low Range (NRf1), Mid Low Range (NRf2), Mid High Range (NRf3) and High Range (NRf4);

The signalling test frequencies NRf5, NRf6, NRf7 are mapped respectively as NRf1, NRf2, NRf3 on the operating band for inter-band.

The test frequencies, subcarrier spacing, default channel bandwidth, SS/PBCH block and CORESET#0 parameters for signalling is specified in Table 6.2.3.1-1 (FDD FR1 BW 5MHz), Table 6.2.3.1-2 (FDD FR1 BW 10MHz), Table 6.2.3.1-3 (TDD FR1 BW 5MHz), Table 6.2.3.1-4 (TDD FR1 BW 10MHz), Table 6.2.3.1-4A (TDD FR1 BW 60MHz), Table 6.2.3.1-5 (TDD FR1 BW 100MHz), Table 6.2.3.1-6 (TDD FR2 BW 100MHz) and Table 6.2.3.1-7 (NR FDD FR1 SUL bands).

Table 6.2.3.1-1: Test frequencies for NR FDD FR1 bands using 5 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 |  | 15 |  | 5 |  | 25 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.15 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 878.2 | 175640 | 873.79 | 174758 | 12 | 15 | 2197 | 175730 | 0 | 6 | 2 (4) | 17 |
|  |  |  |  |  | Mid-High | 884.8 | 176960 | 878.23 | 175646 | 24 |  | 2212 | 176930 | 8 | 6 | 0 (0) | 25 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.5 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 833.2 | 166640 | 824.47 | 164894 | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 839.8 | 167960 | 817.03 | 163406 | 114 | - | - | - | - | - | - | - |
| n8 |  | 15 |  | 5 |  | 25 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.8 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 937.5 | 187500 | 933.09 | 186618 | 12 | 15 | 2343 | 187470 | 8 | 6 | 0 (0) | 13 |
|  |  |  |  |  | Mid-High | 947.5 | 189500 | 940.93 | 188186 | 24 |  | 2368 | 189410 | 0 | 0 | 0 (0) | 24 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.8 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 892.5 | 178500 | 883.77 | 176754 | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 902.5 | 180500 | 879.73 | 175946 | 114 | - | - | - | - | - | - | - |
| n12 |  | 15 |  | 5 |  | 25 |  | Downlink |  | Low, Mid, High |  | Same values as for Low, Mid and High range in clause 4.3.1.1.1.12 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | Low, Mid, High |  | Same values as for Low, Mid and High range in clause 4.3.1.1.1.12 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 |  | 15 |  | 5 |  | 25 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.20 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 801.8 | 160360 | 797.39 | 159478 | 12 | 15 | 2003 | 160330 | 8 | 6 | 0 (0) | 13 |
|  |  |  |  |  | Mid-High | 810.2 | 162040 | 803.63 | 160726 | 24 |  | 2024 | 162010 | 8 | 6 | 0 (0) | 25 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.20 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 842.8 | 168560 | 834.07 | 166814 | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 851.2 | 170240 | 828.43 | 165686 | 114 | - | - | - | - | - | - | - |
| n26 |  | 15 |  | 5 |  | 25 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.26 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 871.5 | 174300 | 867.09 | 173418 | 12 | 15 | 2178 | 174270 | 8 | 1 | 0 (0) | 13 |
|  |  |  |  |  | Mid-High | 881.5 | 176300 | 874.93 | 174986 | 24 |  | 2203 | 176210 | 0 | 0 | 0 (0) | 24 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.26 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 826.5 | 165300 | 817.77 | 163554 | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 836.5 | 167300 | 813.73 | 162746 | 114 |  | - | - | - | - | - | - |
| n29 |  | 15 |  | 5 |  | 25 |  | Downlink (SDL) |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.29 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 |  | 15 |  | 5 |  | 25 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.30 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | Low, Mid, High |  | Same values as for Low and High range in clause 4.3.1.1.1.30 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
| n70 |  | 15 |  | 5 |  | 25 |  | Downlink |  | Low, Mid, High |  | Same values as for Low, Mid and High range in clause 4.3.1.1.1.70 for DL bandwidth=5 MHz, UL bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | Low, Mid, High |  | Same values as for Low, Mid and High range in clause 4.3.1.1.1.70 for DL bandwidth=5 MHz, UL bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 |  | 15 |  | 5 |  | 25 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.71 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 629.5 | 125900 | 625.09 | 125018 | 12 | 15 | 1573 | 125810 | 0 | 0 | 0 (0) | 12 |
|  |  |  |  |  | Mid-High | 639.5 | 127900 | 632.93 | 126586 | 24 |  | 1598 | 127930 | 4 | 6 | 1 (2) | 27 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.71 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 675.5 | 135100 | 666.77 | 133354 | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 685.5 | 137100 | 662.73 | 132546 | 114 | - | - | - | - | - | - | - |
| n76 |  | 15 |  | 5 |  | 25 |  | Downlink (SDL) |  | Low |  | Same values as for Low range in clause 4.3.1.1.1.76 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-2: Test frequencies for NR FDD FR1 bands using 10 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] |  | point A [MHz] |  | absoluteFrequencyPointA[ARFCN] |  | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 2131.7 | 426340 |  | 2124.86 |  | 424972 |  | 12 | 15 | 5321 | 425770 | 2 | 0 | 0 (0) | 12 |
|  |  |  |  |  | Mid-High | 2148.3 | 429660 |  | 2139.3 |  | 427860 |  | 24 |  | 5364 | 429150 | 10 | 6 | 0 (0) | 25 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1941.7 | 388340 |  | 1930.54 |  | 386108 |  | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 1958.3 | 391660 |  | 1933.1 |  | 386620 |  | 114 | - | - | - | - | - | - | - |
| n2 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1951.7 | 390340 |  | 1944.86 |  | 388972 |  | 12 | 15 | 4871 | 389770 | 2 | 0 | 0 (0) | 12 |
|  |  |  |  |  | Mid-High | 1968.3 | 393660 |  | 1959.3 |  | 391860 |  | 24 |  | 4914 | 393150 | 10 | 6 | 0 (0) | 25 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1871.7 | 374340 |  | 1860.54 |  | 372108 |  | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 1888.3 | 377660 |  | 1863.1 |  | 372620 |  | 114 | - | - | - | - | - | - | - |
| n3 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.3 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1831.7 | 366340 |  | 1824.86 |  | 364972 |  | 12 | 15 | 4571 | 365770 | 2 | 0 | 0 (0) | 12 |
|  |  |  |  |  | Mid-High | 1853.3 | 370660 |  | 1844.3 |  | 368860 |  | 24 |  | 4625 | 370090 | 2 | 0 | 0 (0) | 24 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.3 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1736.7 | 347340 |  | 1725.54 |  | 345108 |  | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 1758.3 | 351660 |  | 1733.1 |  | 346620 |  | 114 | - | - | - | - | - | - | - |
| n7 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.7 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 2645 | 529000 |  | 2638.16 |  | 527632 |  | 12 | 15 | 6605 | 528490 | 10 | 6 | 0 (0) | 13 |
|  |  |  |  |  | Mid-High | 2665 | 533000 |  | 2656 |  | 531200 |  | 24 |  | 6658 | 532610 | 2 | 6 | 2 (4) | 29 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.7 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 2525 | 505000 |  | 2513.84 |  | 502768 |  | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 2545 | 509000 |  | 2519.8 |  | 503960 |  | 114 | - | - | - | - | - | - | - |
| n25 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.25 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1953.3 | 390660 |  | 1946.46 |  | 389292 |  | 12 | 15 | 4878 | 390270 | 2 | 6 | 2 (4) | 17 |
|  |  |  |  |  | Mid-High | 1971.7 | 394340 |  | 1962.7 |  | 392540 |  | 24 |  | 4924 | 393890 | 6 | 6 | 1 (2) | 27 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.25 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1873.3 | 374660 |  | 1862.14 |  | 372428 |  | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 1891.7 | 378340 |  | 1866.5 |  | 373300 |  | 114 | - | - | - | - | - | - | - |
| n28 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.28 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 774.7 | 154940 |  | 767.86 |  | 153572 |  | 12 | 15 | 1930 | 154370 | 2 | 0 | 0 (0) | 12 |
|  |  |  |  |  | Mid-High | 786.3 | 157260 |  | 777.3 |  | 155460 |  | 24 |  | 1959 | 156750 | 10 | 6 | 0 (0) | 25 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.28 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 719.7 | 143940 |  | 708.54 |  | 141708 |  | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 731.3 | 146260 |  | 706.1 |  | 141220 |  | 114 | - | - | - | - | - | - | - |
| n65 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.65 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 2141.7 | 428340 |  | 2134.86 |  | 426972 |  | 12 | 15 | 5349 | 427950 | 2 | 6 | 2 (4) | 17 |
|  |  |  |  |  | Mid-High | 2168.3 | 433660 |  | 2159.3 |  | 431860 |  | 24 |  | 5414 | 433210 | 6 | 6 | 1 (2) | 27 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.65 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1951.7 | 390340 |  | 1940.54 |  | 388108 |  | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 1978.3 | 395660 |  | 1953.1 |  | 390620 |  | 114 | - | - | - | - | - | - | - |
| n66 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.66 for DL bandwidth=10 MHz, UL bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 2141.7 | 428340 |  | 2134.86 |  | 426972 |  | 12 | 15 | 5349 | 427950 | 2 | 6 | 2 (4) | 17 |
|  |  |  |  |  | Mid-High | 2168.3 | 433660 |  | 2159.3 |  | 431860 |  | 24 |  | 5414 | 433210 | 6 | 6 | 1 (2) | 27 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.66 for DL bandwidth=10 MHz, UL bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1741.7 | 348340 |  | 1730.54 |  | 346108 |  | 36 | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 1768.3 | 353660 |  | 1743.1 |  | 348620 |  | 114 | - | - | - | - | - | - | - |
| n74 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.74 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1491 | 298200 | 1484.16 |  | 296832 |  | 12 |  | 15 | 3720 | 297630 | 2 | 0 | 0 (0) | 12 |
|  |  |  |  |  | Mid-High | 1502 | 300400 | 1493 |  | 298600 |  | 24 |  |  | 3749 | 300010 | 2 | 6 | 2 (4) | 29 |
|  |  |  |  |  |  |  |  | Uplink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.74 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Mid-Low | 1443 | 288600 | 1431.84 |  | 286368 |  | 36 |  | - | - | - | - | - | - | - |
|  |  |  |  |  | Mid-High | 1454 | 290800 | 1428.8 |  | 285760 |  | 114 |  | - | - | - | - | - | - | - |
| n75 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.75 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | (SDL) | Mid-Low | 1462 | 292400 |  | 1455.16 |  | 291032 |  | 12 | 15 | - | 291824 | 31 | - | 0 | - |
|  |  |  |  | Note 2 | Mid-High | 1487 | 297400 |  | 1478 |  | 295600 |  | 24 |  | - | 296824 | 31 | - | 0 | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: FR1 carrier without any coreset is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-3: Test frequencies for NR TDD FR1 bands using 5 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n34 |  | 15 |  | 5 |  | 25 |  | Downlink & Uplink |  | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.12 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
| n51 |  | 15 |  | 5 |  | 25 |  | Downlink & Uplink |  | Low | Same values as for Low range in clause 4.3.1.1.1.51 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-4: Test frequencies for NR TDD FR1 bands using 10 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n38 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.38 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 2588.3 | 517660 | 2581.46 | 516292 | 12 | 15 | 6464 | 517210 | 6 | 6 | 1 (2) | 15 |
|  |  |  |  | Uplink | Mid-High | 2601.7 | 520340 | 2592.7 | 518540 | 24 |  | 6499 | 519890 | 6 | 6 | 1 (2) | 27 |
| n39 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.39 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 1895 | 379000 | 1888.16 | 377632 | 12 | 15 | 4730 | 378490 | 10 | 6 | 0 (0) | 13 |
|  |  |  |  | Uplink | Mid-High | 1905 | 381000 | 1896 | 379200 | 24 |  | 4755 | 380430 | 2 | 0 | 0 (0) | 24 |
| n40 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.40 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 2335 | 467000 | 2328.16 | 465632 | 12 | 15 | 5833 | 466610 | 2 | 6 | 2 (4) | 17 |
|  |  |  |  | Uplink | Mid-High | 2365 | 473000 | 2356 | 471200 | 24 |  | 5908 | 472610 | 2 | 6 | 2 (4) | 29 |
| n48 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.48 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3601.665 | 640111 | 3594.825 | 639655 | 12 | 30 | 7917 | 640128 | 5 | 6 | 1 (6) | 19 |
|  |  |  |  | Uplink | Mid-High | 3648.33 | 643222 | 3639.33 | 642622 | 24 |  | 7949 | 643200 | 2 | 6 | 0 (2) | 28 |
| n50 |  | 15 |  | 10 |  | 52 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.50 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 1462 | 292400 | 1455.16 | 291032 | 12 | 15 | 3649 | 291890 | 10 | 6 | 0 (0) | 13 |
|  |  |  |  | Uplink | Mid-High | 1487 | 297400 | 1478 | 295600 | 24 |  | 3710 | 296890 | 10 | 6 | 0 (0) | 25 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22] for all bands in the table. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-4A: Test frequencies for NR TDD FR1 bands using 60 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n41 |  | 30 |  | 60 |  | 162 |  | Downlink & Uplink |  | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.41 for bandwidth=60 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-5: Test frequencies for NR TDD FR1 bands using 100 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 |  | 30 |  | 100 |  | 273 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.77 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3616.68 | 641112 | 3563.22 | 637548 | 12 | 30 | 7896 | 638112 | 12 | 0 | 1 (1) | 26 |
|  |  |  |  | Uplink | Mid-High | 3883.32 | 658888 | 3825.54 | 655036 | 24 |  | 8081 | 655872 | 20 | 0 | 0 (0) | 48 |
| n78 |  | 30 |  | 100 |  | 273 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 3483.33 | 632222 | 3429.87 | 628658 | 12 | 30 | 7804 | 629280 | 22 | 0 | 3 (3) | 30 |
|  |  |  |  | Uplink | Mid-High | 3616.68 | 641112 | 3558.9 | 637260 | 24 |  | 7896 | 638112 | 12 | 0 | 1 (1) | 50 |
| n79 |  | 30 |  | 100 |  | 273 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.1.1.79 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 4616.67 | 707778 | 4563.21 | 704214 | 12 | 30 | 8592 | 704928 | 18 | 6 | 1 (4) | 38 |
|  |  |  |  | Uplink | Mid-High | 4783.35 | 718890 | 4725.57 | 715038 | 24 |  | 8720 | 717216 | 18 | 54 | 1 (4) | 160 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22] for all bands in the table except for band n79 where Table 13-6 apply. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-6: Test frequencies for NR TDD FR2 bands using 100 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n257 |  | 120 |  | 100 |  | 66 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.2.1.1 for bandwidth=100 MHz and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 27516.6 | 2071109 | 27451.8 | 2070029 | 12 | 120 | 22444 | 2070811 | 7 | 6 | 1 (4) | 44 |
|  |  |  |  | Uplink | Mid-High | 28483.32 | 2087221 | 28394.04 | 2085733 | 24 |  | 22500 | 2086939 | 3 | 4 | 1 | 80 |
| n258 |  | 120 |  | 100 |  | 66 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.2.1.2 for bandwidth=100 MHz and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 25350 | 2034999 | 25285.2 | 2033919 | 12 | 120 | 22318 | 2034523 | 2 | 6 | 0 (0) | 30 |
|  |  |  |  | Uplink | Mid-High | 26400 | 2052499 | 26317.92 | 2051131 | 24 |  | 22379 | 2052091 | 0 | 6 | 1 (4) | 60 |
| n259 |  | 120 |  | 100 |  | 66 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.2.1.3 for bandwidth=100 MHz and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 40850.04 | 2293333 | 40785.24 | 2292253 | 12 | 120 | 23215 | 2292859 | 3 | 3 | 0 (0) | 30 |
|  |  |  |  | Uplink | Mid-High | 42150 | 2314999 | 42067.92 | 2313631 | 24 |  | 23290 | 2314459 | 6 | 0 | 0 (0) | 48 |
| n260 |  | 120 |  | 100 |  | 66 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.2.1.4 for bandwidth=100 MHz and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 38016.6 | 2246109 | 37951.8 | 2245029 | 12 | 120 | 23051 | 2245627 | 11 | 6 | 0 (0) | 28 |
|  |  |  |  | Uplink | Mid-High | 38983.32 | 2262221 | 38901.24 | 2260853 | 24 |  | 23107 | 2261755 | 7 | 6 | 0 (0) | 54 |
| n261 |  | 120 |  | 100 |  | 66 |  | Downlink |  | Low, High |  | Same values as for Low and High range in clause 4.3.1.2.1.5 for bandwidth=100 MHz and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | & | Mid-Low | 27800.04 | 2075833 | 27735.24 | 2074753 | 12 | 120 | 22460 | 2075419 | 9 | 6 | 1 (4) | 34 |
|  |  |  |  | Uplink | Mid-High | 28050 | 2079999 | 27967.92 | 2078631 | 24 |  | 22474 | 2079451 | 2 | 0 | 0 (0) | 48 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-7: Test frequencies for NR FDD FR1 SUL bands

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | AbsoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.80-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n81 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.81-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n82 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.82-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n83 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.83-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n84 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.84-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |
| n86 | 15 | 10 | 52 | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.86-1 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |

#### 6.2.3.2	Test frequencies for EN-DC band combinations for signalling testing

##### 6.2.3.2.1	General

The default channel bandwidths for EN-DC signalling test are specified per NR and E-UTRA band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

##### 6.2.3.2.2	E-UTRA 1CC and NR 1CC

For EN-DC Inter-band case with E-UTRA 1CC and NR 1CC (one E-UTRA band and one NR band) the EN-DC configurations are specified in clause 4.3.1.4.1.2 for EN-DC with NR FR1 and 4.3.1.5.1.2 for EN-DC with NR FR2.

The E-UTRA and NR test frequencies are specified in TS 36.508 [2], clause 6.2.3.1 for the E-UTRA band (E-UTRA f1, f2, f3 and f4); and in clause 6.2.3.1 for the NR band (NRf1, NRf2, NRf3, NRf4) and for the secondary NR band (NRf5, NRf6, NRf7) of the secondary EN-DC inter-band configuration.

For EN-DC Intra-band Contiguous case with E-UTRA 1CC and NR 1CC the EN-DC configurations and the test frequencies are specified in Table 6.2.3.2-1.

For EN-DC Intra-band Non-Contiguous with E-UTRA 1CC and NR 1CC case the EN-DC configurations and test frequencies are specified in Table 6.2.3.2-2.

For EN-DC Intra-Band Contiguous and EN-DC Intra-Band Non-Contiguous cases with E-UTRA 1CC and NR 1CC the mapping of frequency ranges to NR test frequencies NRf1, NRf2, NRf3, and NRf4 to PSCell; and to E-UTRA test frequencies f1, f2, f3, and f4 for PCell are as follows:

-	for band combinations with only one test frequency: Low Range (NRf1, f1);

-	for band combinations with up to two frequencies: Low Range (NRf1, f1), High Range (NRf2, f2);

-	for band combinations with up to three frequencies: Mid Range (NRf3, f3), Low Range (NRf1, f1) and High Range (NRf2, f2);

-	for band combinations with up to four frequencies: Mid-Low Range (NRf3, f3), High Range (NRf2, f2), Low Range (NRf1, f1) and Mid-High Range (NRf4, f4).

Table 6.2.3.2.2-1: Test frequencies for EN-DC Intra-band Contiguous configurations with E-UTRA 1CC and NR 1CC

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)41AA |  | E-UTRA CC1 |  | 20 |  | 100 |  | Downlink & Uplink |  | Low, High |  | Same values as for Low and High range values in Table 4.3.1.4.2.41.1-2 (SCS 30 kHz, 30 kHz NR raster and NR CC at the band edges) and EN-DC channel bandwidth combination “E-UTRA: 20MHz + NR: 60MHz”. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR CC1 |  | 60 |  | 162 |  | Downlink & Uplink |  | Low, High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_(n)71AA |  | E-UTRA |  | 5 |  | 25 |  | Downlink |  | Low, Mid, High |  | Same values as for Low, Mid and High range values in Table 4.3.1.4.2.71.1-1 (SCS 15 kHz, 100 kHz NR raster and NR CC at the band edges) and EN-DC channel bandwidth combination “E-UTRA: 5MHz + NR: 5MHz”. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC1 |  |  |  |  |  | Uplink |  | Low, Mid, High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 5 | 25 | Downlink | Low, Mid, High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC1 |  |  | Uplink | Low, Mid, High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.2.2-2: Test frequencies for EN-DC Intra-Band Non-Contiguous configurations with E-UTRA 1CC and NR 1CC

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n41A | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | Low | Same values as for Test Frequency ID = ”Low with maxWgap (NR – E-UTRA)” in Table 4.3.1.4.41.1-1 with NR SCS=30 kHz and CBW=60MHz; and E-UTRA CBW=20 MHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | NR CC1 | 60 | 162 | Downlink & Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |

##### 6.2.3.2.3	E-UTRA 1CC and NR CA 2CC

For EN-DC Inter-band case with E-UTRA 1CC and NR CA 2CC the EN-DC configurations are specified in clauses 4.3.1.4.1.2 (two bands) and 4.3.1.4.1.3 (three bands) for EN-DC and NR CA 2CC with FR1 bands, 4.3.1.5.1.2 (two bands) and 4.3.1.5.1.3 (three bands) for EN-DC with NR CA 2CC with FR2 bands and 4.3.1.6.1.2 for EN-DC and NR CA 2CC´with FR1 and FR2 bands.

For EN-DC Inter-band case (3 bands), the E-UTRA test frequencies are specified in TS 36.508 [2], clause 6.2.3.1 for the E-UTRA band (E-UTRA f1) and the NR test frequencies are specified in clause 6.2.3.1 for the NR band used as PSCell (NRf1, NRf2, NRf3, NRf4) and for the NR band used as SCell (NRf5, NRf6, NRf7).

For EN-DC Inter-band case (2 bands) with NR Intra-band contiguous CA 2CC and NR Intra-band non-contiguous CA 2CC, the E-UTRA test frequencies are specified in TS 36.508 [2], clause 6.2.3.1 for the E-UTRA band (E-UTRA f1) and the NR test frequencies are specified in clause 6.2.3.4 for the NR CA CC1 used as PSCell (NRf1, NRf3) and for the NR CA CC2 used as SCell (NRf2, NRf4,).

For EN-DC Intra-band Contiguous case with E-UTRA 1CC and NR CA 2CC the EN-DC configurations and the test frequencies are specified in Table 6.2.3.2.3-1.

For EN-DC Intra-band Non-Contiguous case with E-UTRA 1CC and NR 2CC the EN-DC configurations and test frequencies are specified in Table 6.2.3.2.3-2.

For EN-DC Intra-Band Contiguous and EN-DC Intra-Band Non-Contiguous cases with E-UTRA 1CC and NR CA 2CC the mapping of frequency ranges to NR test frequencies NRf1 for PSCell (CC1) and NRf2 for SCell (CC2); and to E-UTRA test frequency f1 for PCell is:

-	for band combinations with only one test frequency: Low Range (NRf1=CC1, NRf2=CC2, f1); and

-	for band combinations with up to two frequencies: Low Range (NRf1=CC1, NRf2=CC2, f1), High Range (NRf3=CC1, NRf4=CC2, f2).

Editor’s note:	No EN-DC Intra-band Contiguous configurations with NR CA 2CC have yet been introduced in TS 38.101-3.

Table 6.2.3.2.3-1: Test frequencies for EN-DC Intra-band Contiguous configurations with E-UTRA 1CC and NR CA 2CC

FFS

Editor’s note:	No EN-DC Intra-band Non-Contiguous configurations with NR CA 2CC have yet been introduced in TS 38.101-3.

Table 6.2.3.3-2: Test frequencies for EN-DC Intra-Band Non-Contiguous configurations with E-UTRA 1CC and NR CA 2CC

FFS

#### 6.2.3.3	Test frequencies for NR and E-UTRA Inter-RAT signalling testing

For NR and E-UTRA Inter-RAT testing, it is assumed that the NR and E-UTRA bands under test are different in order to avoid unnecessary interferences:

-	for NR bands, the frequencies NRf1, NRf2, NRf3 and NRf4 are mapped as per clause 6.2.3.1

-	for E-UTRA bands, the signalling test frequencies E-UTRA f1, E-UTRA f2, E-UTRA f3 and E-UTRA f4 are mapped respectively on f1, f2, f3 and f4 as per TS 36.508 [2] clause 6.2.3.1.

#### 6.2.3.4	Test frequencies for NR CA configurations for signalling testing

The default channel bandwidths for NR CA signalling test are specified per NR band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For NR CA Inter-band case (2 bands) the NR CA configurations are specified in clause 4.3.1.1.2 and NR test frequencies are specified in clause 6.2.3.1 for the NR band used as PCell (NRf1, NRf2, NRf3, NRf4) and for the NR band used as Scell (NRf5, NRf6, NRf7).

For NR CA Intra-band Contiguous case (2 CCs) the NR CA configurations and the test frequencies are specified in Table 6.2.3.4-1 for FR1 and in Table 6.2.3.4-2 for FR2. For NR CA Intra-band Non-Contiguous (2 CCs) case the NR CA configurations and test frequencies are specified in Table 6.2.3.4-3 for FR1 and in Table 6.2.3.4-4 for FR2.

For NR CA Intra-Band Contiguous case (2 CCs) and NR CA Intra-Band Non-Contiguous case (2 CCs) the mapping of frequency ranges to NR test frequencies NRf1, NRf2, NRf3, and NRf4 and PCell (CC1) amd SCell (CC2) are as follows:

-	for Intra-band configurations with only one test frequency: Low Range (NRf1=CC1 and NRf2=CC2); and

-	for Intra-band configurations with up to two frequencies: Low Range (NRf1=CC1 and NRf2=CC2), High Range (NRf3=CC1 and NRf4=CC2).

Table 6.2.3.4-1: Test frequencies for NR CA Intra-band Contiguous configurations with FR1

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41C | CC1, CC2 | 60+60 | 162+ 162 | Downlink & Uplink | Low | Same values as for Low range in Table 4.3.1.1.3.41.1-1 for CBW combination 60+60. |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66B | CC1, | 10+ | 52 | Downlink | Low | Same values as for Low range in Table 4.3.1.1.3.66.1-1 for CBW combination 10+15. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | +79 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78C | CC1, | 100+100 | 273+ | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.3.78.1-1 for CBW combination 100+100. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  | 273 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-2: Test frequencies for NR CA Intra-band Contiguous configurations with FR2

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n257G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.1.6-1 for CBW combination 100+100. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.4.6-1 for CBW combination 100+100. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.5.6-1 for CBW combination 100+100. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-3: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR1

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A) | CC1, | 10+10 | 52+52 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.66-1 for CBW combination 10+10. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-4: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR2

FFS

#### 6.2.3.5	Test frequencies for MFBI signalling testing

For signalling test cases, the mapping of MFBI frequency ranges to NR test frequencies are as follows: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3).

The test frequencies, subcarrier spacing, default channel bandwidth, SS/PBCH block and CORESET#0 parameters for signalling are specified in Table 6.2.3.5-1.

Table 6.2.3.5-1: Test frequencies for MFBI NR bands in FR1

| NR Band | MFBI overlapping NR Band | SCS [kHz] | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n2 | n25 | 15 | 10 | 52 | Downlink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n25 | n2 | 15 | 10 | 52 | Downlink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n38 | n41 | 15 | 10 | 52 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.38 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n41 | n38 | 15 | 10 | 52 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.38 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n77 | n78 | 30 | 100 | 273 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n78 | n77 | 30 | 100 | 273 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-2: Test frequencies for MFBI NR bands in FR2

| NR Band | MFBI overlapping NR Band | SCS [kHz] | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] |  | absoluteFrequencyPointA[ARFCN] |  | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | n258 | 120 | 100 | 66 | Downlink & Uplink | Low | 26557.08 | 2055117 |  | 26509.56 |  | 2054325 |  | 0 | 120 |  | 22388 |  | 2054683 |  | 0 | 1 (4) | 1 | 8 |
|  |  |  |  |  |  | Mid | 27006.36 | 2062605 |  | 26811.96 |  | 2059365 |  | 102 |  |  | 22414 |  | 2062171 |  | 0 | 1 (4) | 1 | 212 |
|  |  |  |  |  |  | High | 27438.36 | 2069805 |  | 26665.08 |  | 2056917 |  | 504 |  |  | 22439 |  | 2069371 |  | 0 | 1 (4) | 1 | 1016 |
| n257 | n261 | 120 | 100 | 66 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.2.1.5 for bandwidth=100 MHz and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n258 | n257 | 120 | 100 | 66 | Downlink & Uplink | Low | 26557.08 | 2055117 |  | 26509.56 |  | 2054325 |  | 0 | 120 |  | 22388 |  | 2054683 |  | 11 | 0 | 1 (4) | 8 |
|  |  |  |  |  |  | Mid | 27006.36 | 2062605 |  | 26811.96 |  | 2059365 |  | 102 |  |  | 22414 |  | 2062171 |  | 11 | 0 | 1 (4) | 212 |
|  |  |  |  |  |  | High | 27438.36 | 2069805 |  | 26665.08 |  | 2056917 |  | 504 |  |  | 22439 |  | 2069371 |  | 11 | 0 | 1 (4) | 1016 |
| n261 | n257 | 120 | 100 | 66 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.2.1.5 for bandwidth=100 MHz and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.2.3.6	Test frequencies for NR DC configurations for signalling testing

The default channel bandwidths for NR DC signalling test are specified per NR band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For NR DC (2 bands, 2CC) the NR DC configurations are specified in clause 4.3.1.3.2.1 for 1CC FR1 and 1CC FR2. NR test frequencies are specified in clause 6.2.3.1 for the NR band used as PCell (NRf1, NRf2, NRf3, NRf4) and for the NR band used as PSCell (NRf5, NRf6, NRf7).

## 6.3	Reference system configurations

### 6.3.1	Cell configurations

Editor’s Note: To define different types of SS cell configurations. It may be similar as defined in 3GPP TS 36.508 [2], clause 6.3.3 and 6.3.4 i.e. full, minimum uplink, broadcast only and virtual cell configuration. But details are FFS and depending on different connectivity options (MR-DC and SA).

#### 6.3.1.1	Intra-frequency neighbouring cell list in SIB3 for NR cells

Intra-frequency neighbouring cell list for signalling test cases is defined in table 6.3.1.1-1. This table is referred to in the default contents of IE intraFreqNeighCellList in SIB3 defined in table 4.6.2-2.

Table 6.3.1.1-1: Intra-frequency neighbouring cell lists for NR cells

| cell ID | Test Frequency | intra-frequency neighbouring cell list |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | number of entries | physCellId[n] |  |  |
|  |  |  | 1 | 1 | 3 |
| NR Cell 1 | NRf1 | 3 | NR Cell 2 | NR Cell 4 | NR Cell 11 |
| NR Cell 2 | NRf1 | 3 | NR Cell 1 | NR Cell 4 | NR Cell 11 |
| NR Cell 4 | NRf1 | 3 | NR Cell 1 | NR Cell 2 | NR Cell 11 |
| NR Cell 11 | NRf1 | 3 | NR Cell 1 | NR Cell 2 | NR Cell 4 |
| NR Cell 3 | NRf2 | 1 | NR Cell 23 | - | - |
| NR Cell 23 | NRf2 | 1 | NR Cell 3 | - | - |

Editor’s Note: The intra-frequency NR neighbouring cell list for signalling NAS test cases when cells are on same PLMN is FFS.

#### 6.3.1.2	Inter-frequency carrier frequency list in SIB4 for NR cells

Inter-frequency NR carrier frequency list for signalling test cases is defined in table 6.3.1.2-1. This table is referred to in the default contents of IE interFreqCarrierFreqList in SIB4 defined in table 4.6.2-3.

Table 6.3.1.2-1: Inter-frequency carrier frequency lists for NR cells

| cell ID | Test Frequency | interFreqCarrierFreqList |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | number of entries | dl-CarrierFreq[n] |  |  |
|  |  |  |  | 1 | 2 | 3 |
| NR Cell 1 NR Cell 2 NR Cell 4 NR Cell 11 | NRf1 (Note 2) | 3 | NRf2 | NRf3 | NRf5 |
| NR Cell 3 NR Cell 23 | NRf2 (Note 2) | 3 | NRf1 | NRf3 | NRf5 |
| NR Cell 6 | NRf3 (Note 2) | 3 | NRf1 | NRf2 | NRf5 |
| NR Cell 10 | NRf5 (Note 3) | 3 | NRf1 | NRf2 | NRf3 |
| Note 1: Depending on the Band under test, NRf3 may not be applicable. Note 2: In case of Test frequency NRf1, NRf2 and NRf3, dl-CarrierFreq NRf5 as part of inter-frequency list is applicable only in case of multi-band scenarios. Note 3: Test frequency NRf5 is applicable only in case of multi-band scenarios. |  |  |  |  |  |

Editor’s Note: The inter-frequency NR carrier frequency list for signalling NAS test cases when cells are on same PLMN is FFS.

#### 6.3.1.3	E-UTRA carrier frequency list in SIB5 for NR cells

The frequency mapping of E-UTRA cells are defined as per TS 36.508 [2] clause 4.4.2 and TS 36.508 [2] clause 6.3.2 for NGC NAS test cases, E-UTRA frequency mapping is according to clause 6.2.3.3. E-UTRA carrier frequency list for signalling test cases is defined in table 6.3.1.3-1. This table is referred to in the default contents of IE carrierFreqListEUTRA in SIB5 defined in table 4.6.2-4.

Table 6.3.1.3-1: E-UTRA carrier frequency lists for NR cells

| interFreqCarrierFreqList |  |
| --- | --- |
| number of entries | carrierFreq[n] |
|  | 1 |
| 1 | E-UTRA f1 |
| 2 | E-UTRA f2 |
| 3 | E-UTRA f3 |
| 4 | E-UTRA f4 |
| Note 1: E-UTRAf1, E-UTRAf2, E-UTRAf3, E-UTRAf4 are according to clause 6.2.3.3 Note 2: Depending on the Band under test, E-UTRA f2 or E-UTRA f3 or E-UTRA f4 may not be applicable. |  |

Table 6.3.1.3-2: Void

### 6.3.2	Default configurations for NAS test cases

The default configurations specified in this subclause apply only to NAS test cases. They apply to all NAS test cases unless otherwise specified.

#### 6.3.2.1	Simulated network scenarios for NAS test cases

Simulated network scenarios for NAS test cases to be tested are specified in the pre-test conditions of each individual test case.

Any combination is allowed with the following restrictions:

-	NGC Cell B shall not be used if NGC Cell D is used

-	a maximum 3 cells on the same frequency can be used, i.e. only 3 cells out of NGC Cell A, NGC Cell B, NGC Cell C and NGC Cell D may be used simultaneously in each individual test case when cells in the test case are in different PLMNs (refer to Table 6.3.2.2-3).

#### 6.3.2.2	Simulated NAS cells

Simulated NAS cells and default NAS parameters are specified in Table 6.3.2.2-1. Unless otherwise specified in a test case, default radio parameters of the NAS cells are specified as per Table 6.3.2.2-2.

Unless otherwise specified, the default parameters specified in clause 4.4.2 will also apply to all NAS cells.

Table 6.3.2.2-1: Default NAS parameters for simulated NAS cells

| NAS cell ID | Tracking Area |  |  |  |  | TA# list (Note 1) | 5G-GUTI (Note 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  |  | TAC |  | AMF Identifier |  |  | 5G-TMSI |
|  |  | MCC |  | MNC |  |  | AMF Region ID | AMF Set ID | AMF Pointer |  |
| NGC Cell A | TAI-1 | (Note 3) |  |  | 1 | TAI-1 | 254 | 1 | 1 | Arbitrarily selected according to TS 23.003 subclause 2.10.1 [26]. |
| NGC Cell B | TAI-2 | (Note 3) |  |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NGC Cell C | TAI-3 | (Note 3) |  |  | 3 | TAI-3 | 252 | 1 | 1 |  |
| NGC Cell D | TAI-4 | (Note 3) |  |  | 4 | TAI-4 | 252 | 1 | 1 |  |
| NGC Cell E | TAI-12 | 002 |  | 101 | 3 | TAI-12 | 244 | 1 | 1 |  |
| NGC Cell F | TAI-11 | 003 | 101 |  | 2 | TAI-11 | 239 | 1 | 1 |  |
| NGC Cell G | TAI-7 | (Note 4) |  | 02 | 1 | TAI-7 | 238 | 1 | 1 |  |
| NGC Cell H | TAI-8 | (Note 4) |  | 02 | 2 | TAI-8 | 237 | 1 | 1 |  |
| NGC Cell I | TAI-9 | 002 |  | 101 | 1 | TAI-9 | 244 | 1 | 1 |  |
| NGC Cell J | TAI-10 | 003 |  | 101 | 1 | TAI-10 | 236 | 1 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure for initial access or mobility (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column 5G-GUTI indicates GUTI included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.8.3). Note 4: Set to the same Mobile Country Code stored in EFIMSI on the test USIM card (subclause 4.8.3). |  |  |  |  |  |  |  |  |  |  |

Table 6.3.2.2-2: Default radio parameters for simulated NAS cells when cells are in same PLMN and access stratum is NR

| NAS cell ID | Frequency | NR Cell ID (Note 1) |
| --- | --- | --- |
| NGC Cell A | NRf1 | NR Cell 1 |
| NGC Cell B | NRf1 | NR Cell 2 |
| NGC Cell C | NRf1 | NR Cell 4 |
| NGC Cell D | NRf1 | NR Cell 11 |
| NGC Cell E | NA | NA |
| NGC Cell F | NRf2 | NR Cell 3 |
| NGC Cell G | NA | NA |
| NGC Cell H | NA | NA |
| NGC Cell I | NA | NA |
| NGC Cell J | NRf2 | NR Cell 12 |
| Note 1: Default NR parameters for simulated NR cells are as specified in Table 4.4.2-2. Note 2: For signalling tests, simultaneous co-existence of NGC Cells B and D is not allowed (in line with Table 4.4.2-1) |  |  |

Table 6.3.2.2-3: Default PLMN and radio parameters for simulated NAS cells when cells are in different PLMNs and access stratum is NR

| NAS cell ID | PLMN | Frequency | NR Cell ID (Note 1) |
| --- | --- | --- | --- |
| NGC Cell A | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 1 |
| NGC Cell B | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 2 |
| NGC Cell C | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 4 |
| NGC Cell D | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 11 |
| NGC Cell E | MCC=002 MNC=101 | NRf2 | NR Cell 3 |
| NGC Cell F | MCC=003 MNC=101 | NRf4 | NR Cell 14 |
| NGC Cell G | MCC = MCC in USIM MNC=02 | NRf2 | NR Cell 12 |
| NGC Cell H | MCC = MCC in USIM MNC=02 | NRf2 | NR Cell 23 |
| NGC Cell I | MCC=002 MNC=101 | NRf3 | NR Cell 6 |
| NGC Cell J | MCC=002 MNC=101 | NRf3 | NR Cell 13 |
| Note 1: Default NR parameters for simulated NR cells are as specified in Table 4.4.2-2 Note 2: for signalling tests, simultaneous co-existence of NGC Cells B and D is not allowed (In line with Table 4.4.2-1) |  |  |  |

## 6.4	Signalling Test Case specific USIM Configurations

### 6.4.1	General

The default USIM fields are specified in section 4.8.3. Specific USIM fields are set according to the USIM configuration specified in the tables below. PLMN settings are defined in TS 36.523-1 [42] Table 6.0.1-1.

Note:	Changes to any existing USIM configuration can be done only if the change WILL NOT HAVE IMPACT on any of the tests which are referring to the configuration! To establish whether this might be the case, the test case author needs to review all tests in all RAN5 test specifications, which refer to the particular USIM configuration e.g. all test cases in TS 38.523-1 [12].

Table 6.4.1-1: USIM Configuration 1

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | Default | Default |
|  | 2 | PLMN17 | All specified |
|  | 3 | PLMN16 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN15 | All specified |
|  |  | Remaining defined entries use default values |  |
| EFHPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Services 20, 42, 43 and 74 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) |  |

Table 6.4.1-2: USIM Configuration 2

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°71 and n°74 are "available" |  |
| EFEHPLMN | 1 | PLMN15 |  |
|  | 2 | PLMN1 |  |
| EFLRPLMNSI |  | 01 |  |

Table 6.4.1-3: USIM Configuration 3

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°74 is "available" |  |
| EFEHPLMN |  | Empty |  |
| EFLRPLMNSI |  | 01 |  |

Table 6.4.1-4: USIM configuration 4

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEHPLMN | 1 | PLMN1 |  |
|  |  | Remaining mandatory entries use default values |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN3 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFUST |  | Services 20, 42 and 71 are supported. |  |

Table 6.4.1-5: USIM configuration 5

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 (See preamble) |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service 71 is not supported |  |
|  |  | Service 74 is supported. |  |
| EFLRPLMNSI |  | 00 |  |
| EFEHPLMN |  | 0xFF..FF |  |

Table 6.4.1-6: USIM configuration 6

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN3. |  |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN2 | NG-RAN |
|  | 2 | PLMN4 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service 71 is not supported |  |

Table 6.4.1-7: USIM configuration 7

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| E.FPLMNwAcT | 1 | PLMN13 | NG-RAN |  |
|  | 2 | PLMN13 | E-UTRAN |  |
| EFOPLMNwAcT | 1 | PLMN2 | All |  |
|  | 2 | PLMN14 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |

Table 6.4.1-8: USIM configuration 8

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN15 | NG-RAN |  |
|  | 2 | PLMN15 | E-UTRAN |  |
|  | 3 | PLMN17 | E-UTRAN |  |
|  | 4 | PLMN16 | NG-RAN |  |

Table 6.4.1-9: USIM configuration 9

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |  |
|  | 2 | PLMN15 | E-UTRAN |  |
| EFHPPLMN |  | 1(=6 min) |  | The HPLMN Search Period on the USIM shall be set to 6 minutes. |

Table 6.4.1-10: USIM configuration 10

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFOPLMNwACT | 1 | PLMN14 | NG-RAN |
|  | 2 | PLMN13 | NG-RAN |
|  | 3 | PLMN2 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service n°127 is "available" |  |
| EFHPPLMN |  | 1(=6 min) |  |

Table 6.4.1-11: USIM configuration 11

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN16. |  |
| EFPLMNwAcT | 1 | Default | NG-RAN |
|  | 2 | PLMN15 | NG-RAN |
| EFOPLMNwACT | 1 | PLMN1 |  |
|  |  | Remaining defined entries use default values |  |
| EFHPLMNwAcT | 1 | PLMN16 | NG-RAN |
| EFUST |  | Services 20, 42, 43, 74 and 96 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) |  |
| EFNASCONFIG |  | MinimumPeriodicSearchTimer set to 7 minutes |  |

Table 6.4.1-12: USIM configuration 12

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT |  | 3GPP TS 31.102, Annex E |  | The EF is empty. |
| EFOPLMNwAcT | 1 | PLMN2 | NG-RAN |  |
|  | 2 | PLMN13 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |

Table 6.4.1-13: USIM configuration 13

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN2 | NG-RAN |  |
|  | 2 | PLMN2 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |

Table 6.4.1-14: Void

Table 6.4.1-15: USIM Configuration 15

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFFPLMN |  | PLMN2 |  |

Table 6.4.1-16: Void

Table 6.4.1-17: USIM Configuration 17

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Service n°126 is "available". |  |
| EFEHPLMN |  | PLMN1 |  |
| EFUAC_AIC and EFACC |  | For Bits b4 and b8 in byte 1 of EFACC (defined in TS 31.102 clause 4.2.15), only single bit is set to 1. Bits b1 and b2 in byte 1 of EFUAC_AIC (defined in TS 31.102 clause 4.4.11.7), and all remaining bits of EFACC and EFUAC_AIC are set to 0. |  |

Table 6.4.1-18: USIM Configuration 18

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
| EFHPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Service n°126 (for UAC Access Identities Configuration) defined in TS 31.102 clause 4.2.8 is declared "available" |  |
| EFUAC_AIC |  | Bit b1 in byte 1 defined in TS 31.102 clause 4.4.11.7 is set to 1 and bit b2 in byte 1 is set to 0. |  |
| EFACC |  | For Bit b1 in byte 2 defined in TS 31.102 clause 4.2.15, only single bit set to 1. All remaining bits of byte 1 and byte 2 are set to 0. |  |

Table 6.4.1-19: USIM Configuration 19

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
| EFHPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Service n°126 (for UAC Access Identities Configuration) defined in TS 31.102 clause 4.2.8 is declared "available" |  |
| EFUAC_AIC |  | Bit b2 in byte 1 defined in TS 31.102 clause 4.4.11.7 is set to 1 and bit b1 in byte 1 is set to 0. |  |
| EFACC |  | For Bit b2 in byte 2 defined in TS 31.102 clause 4.2.15, only single bit set to 1. All remaining bits of byte 1 and byte 2 are set to 0. |  |

Table 6.4.1-20: USIM Configuration 20

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFECC |  | 144, 117 |  |

Table 6.4.1-21: USIM configuration 21

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFOPLMNwACT | 1 | PLMN14 | NG-RAN |
|  | 2 | PLMN13 | NG-RAN |
|  | 3 | PLMN2 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service n°127 is not "available" |  |
| EFHPPLMN |  | 1(=6 min) |  |

Table 6.4.1-22: USIM Configuration 22

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°19 and n°51 defined in TS 31.102 clause 4.2.8 is declared "service not available" |  |

# 7	Test environments for RRM tests

## 7.0	General

### 7.0.1	Single PDU configuration for RRM testing

For RRM test case execution on 5G SA UEs defined in TS 38.533 [18]7.1	Requirements, IMS shall not be considered and UE's shall be able use RRC (IDLE, CONNECTED) preambles defined in TS 38.508-1 Section 4.5. Before entering RRC_CONNECTED or RRC_IDLE state during initial conditions or test procedure, it is recommended that UE is pre-configured with only 1 PDU (non-IMS) along with appropriate settings to ensure UE operates and stays on NR cell.

## 7.1	Test equipment requirements

### 7.1.1	Requirements common for conducted and OTA tests

Editor’s Note: This subclause is intended to describe the test equipment requirements which are specific to RRM tests and common for conducted and OTA tests.

### 7.1.2	Requirements for conducted test method

Editor’s Note: This subclause is intended to describe the test equipment requirements which are specific to conducted test environment for RRM tests.

### 7.1.3	Requirements for OTA test method

#### 7.1.3.1	General

Editor’s Note:

-	The UE pre-configuration mentioned below to disable UL Tx diversity schemes shall be voided once a test methodology solution to minimize spectral flatness artefacts between TE and UE over all test points is defined.

For conformance testing using the OTA test environment, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

#### 7.1.3.2	RRM baseline setup

The RRM baseline setup shall fulfil the capabilities detailed in this section.

The following permitted test setups are considered for OTA RRM testing:

-	DFF test setup as described in Clause B.2.2.

-	Simplified DFF test setup as described in Clause B.2.3.

-	IFF test setup as described in Clause B.2.4.

-	Enhanced IFF test setup based in the IFF test setup described in Clause B.2.4, with the enhancements described in this clause.

-	IFF+DFF Hybrid teset setup as described in Clause B.2.2 for DFF TRxP(s) and B2.4 for IFF TRxP(s), with the enhancements described in this clause.

##### 7.1.3.2.1	General description

TRxPs and Cells:

-	Up to 2 NR transmission reception points TRxPs are emulated.

Support of interworking scenarios

-	For test scenarios involving both, LTE and NR FR2 carriers, the test setup shall be capable to provide LTE link to the DUT. The emulated LTE cell provides a stable LTE signal without precise propagation modelling or path loss control between it and the DUT. No performance verification for and relative to LTE carriers is supported.

-	For test scenarios involving both, NR FR1 and NR FR2 carriers, the test setup shall be capable to provide NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control. No performance verification for and relative to NR FR1 carriers is supported.

Antennas, polarization, simultaneously active AoAs:

-	N dual-polarized antennas transmitting the signals from the emulated gNB sources to the DUT.

-	The antennas transmit into the test zone in such a way that signal polarization does not prevent the DUT receiving a consistent, predictable power level.

-	N ≥ NMAX_AoAs, where NMAX_AoAs is the maximum number of simultaneously active (emulating signal) angles of arrival AoAs. The NMAX_AoAs for the different permitted test methods is:

-	For UE RRM baseline measurement setup based on DFF, the supported NMAX_AoAs = 2.

-	For UE RRM baseline measurement setup based on simplified DFF, the supported NMAX_AoAs = 1.

-	For UE RRM baseline measurement setup based on IFF, the supported NMAX_AoAs = 1.

-	For UE RRM baseline measurement setup based on enhanced IFF, the supported NMAX_AoAs = 2.

-	For UE RRM baseline measurement setup based on IFF+DFF, the supported NMAX_AoAs = 2.

Angular Relationship:

-	A positioning system such that an angular relationship with two axes of freedom is provided between the DUT and the test system antennas (or the setup should provide equivalent functionality).

-	For NMAX_AoAs = 2 the setup shall enable following relative angular relationships between the NMAX_AoAs simultaneously active AoAs: 30°, 60°, 90°, 120° and 150°.

-	For single active probe scenarios, in case that step change of AoA is required, the setup shall enable following relative angular change between initial and target AoA: 30°, 60°, 90°, 120° and 150°.

Wanted and noise (AWGN) signals can be transmitted from one or both active probes. Test description will define the exact signal/noise/SNR/SINR level per TRxP at the reference point.

Multiple DL transmission antenna ports:

-	In case of multiple DL transmission antenna ports are required for RRM testing, the different antenna ports are mapped to different polarizations.

Measurement Uncertainty:

-	The threshold MU for the equivalence framework for RRM will be based on direct far field (DFF) test method for D ≤ 5 cm and on indirect far field (IFF) test method for D > 5 cm. If the MTSU for the IFF test method for D ≤ 5 cm is finalized before DFF, the IFF MTSU shall be used as provisional threshold MU until DFF is completed.

##### 7.1.3.2.2	Applicability criteria

The applicability criteria for the RRM measurement setup based on DFF is described in B.2.2.1.

The applicability criteria for the RRM measurement setup based on simplified DFF is described in B.2.3.1.

The applicability criteria for the RRM measurement setup based on IFF is described in B.2.4.1.

The applicability criteria for the RRM measurement setup based on enhanced IFF is described in B.2.4.1:

The applicability criteria for the RRM measurement setup based on IFF+DFF follows DFF as described in B.2.2.1:

##### 7.1.3.2.3	Measurement distance and quiet zone

For RRM baseline measurement setup based on DFF:

-	The measurement distance defined for the DFF UE RF test method described in B.2.2.4 applies.

-	A DFF measurement setup has the centre of the Quiet Zone (QZ) located at the centre of the rotational axes (of DUT and measurement antenna). For the RRM measurement baseline setup based on DFF, the vertices of the N probes have to be aligned to the resulting centre of the QZ. The centre of the QZ is taken as the reference point for MU definition for each probe. The same QZ size as for DFF UE RF test method described in B.2.2.2 applies.

For RRM baseline measurement setup based on simplified DFF:

-	The measurement distance defined for the simplified DFF UE RF test method described in B.2.3.4 applies.

-	The same QZ size and definition as for simplified DFF UE RF test method described in B.2.3.2 applies.

For RRM baseline measurement setup based on IFF:

-	The measurement distance defined for the IFF UE RF test method described in B.2.4.4 applies.

-	The Quiet Zone definition for the IFF UE RF test method described in B.2.4.2 applies.

For RRM baseline measurement setup based on enhanced IFF:

-	The measurement distance defined for the IFF UE RF test method described in B.2.4.4 applies.

-	An IFF measurement setup has the centre of the Quiet Zone (QZ) located at the centre of the rotational axes (of DUT). For the RRM measurement baseline setup based on IFF, the reflectors have to be aligned to transmit a plane wave to the resulting centre of the QZ. The centre of the QZ is taken as the reference point for MU definition for each reflector. The QZ is a sphere of radius R. The size of the QZ defined in B.2.4.2 applies.

For RRM baseline measurement setup based on IFF+DFF:

-	For IFF TRxPs, the measurement distance defined for the IFF UE RF test method described in B.2.4.4 applies.

-	For DFF TRxPs, the measurement distance defined for the DFF UE RF test method described in B.2.2.4 applies.

-	An IFF+DFF measurement setup has the centre of the Quiet Zone (QZ) located at the centre of the rotational axes (of DUT). For the RRM measurement baseline setup based on IFF+DFF, IFF reflectors have to be aligned to transmit a plane wave to the resulting centre of the QZ, and the vertices of the DFF probes have to be aligned to the resulting centre of the QZ. The centre of the QZ is taken as the reference point for MU definition for each reflector or probe. The QZ is a sphere of radius R. The size of the QZ defined in B.2.4.2 applies for IFF TRxPs and B.2.2.2 for DFF TRxPs.

##### 7.1.3.2.4	Quality of the quiet zone

For RRM, the quality of the quiet zone validation defined in Annex O of TS 38.521-2 [15] needs to assess only the single-directional EIRP and EIS metrics. For measurement setups with multiple probes, the QoQZ procedure needs to be performed with all probes present and in the conditions used for RRM testing.

The quality of the quiet zone for the RRM measurement setup based on DFF is described in B.2.2.3. The QoQZ validation needs to be performed only with the reference probe P0.

The quality of the quiet zone for the RRM measurement setup based on simplified DFF is described in B.2.3.3.

The quality of the quiet zone for the RRM measurement setup based on IFF is described in B.2.4.3.

The quality of the quiet zone for the RRM measurement setup based on enhanced IFF is described in B.2.4.3. The QoQZ validation needs to be performed only with the reference reflector, P0, if same sized IFF reflectors are used..

The quality of the quiet zone for the RRM measurement setup based on IFF+DFF is FFS.

## 7.2	Reference test conditions

### 7.2.1	Signal levels

#### 7.2.1.1	Signal Levels for conducted testing

TBD

#### 7.2.1.2	Signal Levels for OTA testing

TBD

### 7.2.2	Physical layer parameters

#### 7.2.2.1	Downlink physical layer parameters

As defined in clause 4.3.6 with the following exceptions:

Table 7.2.2.1-1: Physical layer parameters for DCI format 1_1

| Derivation Path: Table 4.3.6.1.2.2-1 |  |  |
| --- | --- | --- |
| Parameter | Value | Value in binary |
| PUCCH resource indicator | PUCCH-ResourceId[1] = 0 in pucch-ResourceSetID[1] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213 [22]) | “000” |

## 7.3	Default NG-RAN RRC message and information elements contents for RRM

### 7.3.1	Radio resource control information elements for RRM

As defined in clause 4.6.3 with the following exceptions:

#### –	TDD-UL-DL-Config

Table 7.3.1-1: TDD-UL-DL-Config

| Derivation Path: Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms0p625 |  | TDDConf.3.1 |
|  | ms5 |  |  |
| nrofDownlinkSlots | 3 |  | TDDConf.2.1, TDDConf.3.1 |
|  | 1 |  | TDDConf.1.1 |
| nrofDownlinkSymbols | 10 |  | TDDConf.1.1, TDDConf.3.1 |
|  | 6 |  | TDDConf.2.1 |
| nrofUplinkSlots | 2 |  | TDDConf.1.1 |
|  | 1 |  | TDDConf.3.1 |
|  | 4 |  | TDDConf.2.1 |
| nrofUplinkSymbols | 4 |  | TDDConf.2.1 |
|  | 2 |  | TDDConf.1.1, TDDConf.3.1 |
| dl-UL-TransmissionPeriodicity-v1530 | Not present |  |  |
|  | ms4 |  | TDDConf.1.1, TDDConf.2.1 |
| } |  |  |  |
| pattern2 | Not present |  |  |
| pattern2 SEQUENCE { |  |  | TDDConf.1.1, TDDConf.2.1 |
| dl-UL-TransmissionPeriodicity | ms1 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofDownlinkSlots | 1 |  | TDDConf.1.1 |
|  | 2 |  | TDDConf.2.1 |
| nrofDownlinkSymbols | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofUplinkSlots | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofUplinkSymbols | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TDDConf.1.1 | TDD UL/DL configuration for SCS=15kHz |
| TDDConf.2.1 | TDD UL/DL configuration for SCS=30kHz |
| TDDConf.3.1 | TDD UL/DL configuration for SCS=120kHz |

#### –	FilterCoefficient

Table 7.3.1-2: FilterCoefficient

| Derivation Path: Table 4.6.3-57 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FilterCoefficient | fc0 | L3 filtering is not used |  |

#### –	SSB-MTC

Table 7.3.1-3: SSB-MTC

| Derivation Path: Table 4.6.3-185 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  | SMTC.1, SMTC.2 |
| sf20 | 10 |  | SMTC.4, SMTC.5 |
| sf160 | 0 |  | SMTC.3 |
| } |  |  |  |
| duration | sf1 |  | SMTC.1, SMTC.3, SMTC.4 |
|  | sf5 |  | SMTC.2, SMTC.5 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n as defined in 38.533 Annex A.4 |

#### –	SubcarrierSpacing

Table 7.3.1-3a: SubcarrierSpacing

| Derivation Path: Table 4.6.3-188 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ssbSubcarrierSpacing | kHz15 |  | SSB.1 FR1, SSB.3 FR1 or SSB.5 FR1 |
|  | kHz30 |  | SSB.2 FR1, SSB.4 FR1 or SSB.6 FR1 |
|  | kHz120 |  | SSB.1 FR2, SSB.3 FR2, SSB.5 FR2 or SSB.7 FR2 |
|  | kHz240 |  | SSB.2 FR2, SSB.4 FR2, SSB.6 FR2 or SSB.8 FR2 |

| Condition | Explanation |
| --- | --- |
| SSB.n FR1 | SSB RMC n for FR1 as defined in 38.533 Annex A.3.1 |
| SSB.n FR2 | SSB RMC n for FR2 as defined in 38.533 Annex A.3.2 |

#### –	ServingCellConfigCommon

Table 7.3.1-4: ServingCellConfigCommon

| Derivation Path: Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 1000 | 1 SS Block in low FR1 frequencies | LOW_FREQ |
|  | 1100 | 2 SS Blocks in low FR1 frequencies | 2SSB AND LOW_FREQ |
| mediumBitmap | 10000000 | 1 SS Block in high FR1 frequencies | HIGH_FREQ |
|  | 11000000 | 2 SS Blocks in high FR1 frequencies | 2SSB AND HIGH_FREQ |
| longBitmap | 1000000000000000000000000000000000000000000000000000000000000000 | 1 SS Block in FR2 | FR2 |
|  | 1100000000000000000000000000000000000000000000000000000000000000 | 2 SS Blocks in FR2 | 2SSB AND FR2 |
| } |  |  |  |
| ssb-periodicityServingCell | ms20 |  |  |
| ssbSubcarrierSpacing | kHz15 |  |  |
|  | kHz30 |  | SCS30kHz |
|  | kHz120 |  | FR2 |
| } | kHz240 |  | FR2 AND SCS240kHz |

| Condition | Explanation |
| --- | --- |
| LOW_FREQ | Frequency <= 2.4 GHz for TDD or Frequency <= 3 GHz for FDD |
| HIGH_FREQ | FR1 and (Frequency > 2.4 GHz for TDD or Frequency > 3 GHz for FDD or CASE_C) |
| SSB.n FRm | As defined in TS 38.533 [18] Annex A.3.1, where n=1, 2, 3 or 4 and m = 1 or 2 |
| 2SSB | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 contain 2 SSBs within a burst |
| SCS30kHz | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 is for 30 kHz SCS |
| SCS240kHz | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 is for 240 kHz SCS |

#### –	ServingCellConfigCommonSIB

Table 7.3.1-5: ServingCellConfigCommonSIB-RRM

| Derivation Path: Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’1000 0000’B | When carrier frequency <= 3 GHz for FDD or <= 2.4 GHz for TDD, only the 4 leftmost bits are valid; |  |
|  | ’1100 0000’B |  | 2SSB |
| groupPresence | Not present |  |  |
|  | ’1000 0000’B |  | FR2 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR2 | Frequency range 2 |
| 2SSB | For configuration with 2 SS Blocks |

#### -	CSI-RS for Tracking, CSI reporting and beam management

Table 7.3.1-6: CSI-MeasConfig for RRM

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n1+n2+n3+n4 entries | n1=1 if CSI-RS for CSI is configured in test case, n1=0 otherwise; n2=2 if CSI-RS for BM is configured in test case, n2=0 otherwise; n3=4 if TRS is configured in test case, n3=0 otherwise; n4=4 if second resource set of TRS is configured in test case, n4=0 otherwise; |  |
| NZP-CSI-RS-Resource[k, k=1..n1] | NZP-CSI-RS-Resource for CSI | entry 1 | n1>0 |
| NZP-CSI-RS-Resource[k, k=n1+1.. n1+n2] | NZP-CSI-RS-Resource for BM (k-n1-1) | entry ... | n2>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+1.. n1+n2+n3] | NZP-CSI-RS-Resource for TRS (k-n1-n2) | entry ... | n3>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+1.. n1+n2+n3+n4] | NZP-CSI-RS-Resource for TRS (k-n1-n2-n3) with condition SECOND_SET | entry ... | n4>0 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | m1+m2+m3+m4 entries | mi=1 if ni>0, mi=0 otherwise |  |
| NZP-CSI-RS-ResourceSet[k, k=1..m1] | NZP-CSI-RS-ResourceSet for CSI | entry 1 | n1>0 |
| NZP-CSI-RS-ResourceSet[k, k=m1+1.. m1+m2] | NZP-CSI-RS-ResourceSet for BM | entry ... | n2>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+1.. m1+m2+m3] | NZP-CSI-RS-ResourceSet for TRS | entry ... | n3>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+1.. m1+m2+m3+m4] | NZP-CSI-RS-ResourceSet for TRS with condition SECOND_SET | entry ... | n4>0 |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | m1+m2+m3 entries |  |  |
| CSI-ResourceConfig[k, k=1..m1] | CSI-ResourceConfig for CSI | entry 1 | n1>0 |
| CSI-ResourceConfig[k, k=m1+1.. m1+m2] | CSI-ResourceConfig for BM | entry ... | n2>0 |
| CSI-ResourceConfig[k, k= m1+m2+1.. m1+m2+m3] | CSI-ResourceConfig for TRS | entry ... | n3>0 and n4=0 |
|  | CSI-ResourceConfig for TRS with condition SECOND_SET |  | n4>0 |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | m1+m2 entries |  |  |
| CSI-ReportConfig[k, k=1..m1] | CSI-ReportConfig for CSI | entry 1 | n1>0 |
| CSI-ReportConfig[k, k=m1+1.. m1+m2] | CSI-ResourceConfig for BM | entry ... | n2>0 |
| } |  |  |  |
| reportTriggerSize | Not present |  |  |
| aperiodicTriggerStateList | Not present |  |  |
| } |  |  |  |

Table 7.3.1-7: NZP-CSI-RS-Resource for TRS(Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| NZP-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for TRS(Id) |  |  |
|  | NZP-CSI-RS-ResourceId for TRS(Id) with Condition SECOND_SET |  | SECOND_SET |
| CSI-RS-ResourceMapping | CSI-RS-ResourceMapping for TRS(Id) |  |  |
|  | CSI-RS-ResourceMapping for TRS(Id) with condition SECOND_SET |  | SECOND_SET |
| powerControlOffsetSS | db-3 |  |  |
| scramblingID | PhysCellId | PCI of the cell sending the TRS |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for TRS(Id) |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId-RRM(0) |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For resource belong to the second resource set for TRS, only applies to FR2 test |

Table 7.3.1-7A: NZP-CSI-RS-Resource for CSI

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for CSI |  |  |
| resourceMapping | CSI-RS-ResourceMapping for CSI |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for CSI |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId-RRM(0) |  |  |
| } |  |  |  |

Table 7.3.1-7B: NZP-CSI-RS-Resource for BM(Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for BM (Id) |  |  |
| resourceMapping | CSI-RS-ResourceMapping for BM (Id) |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for BM |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId-RRM(Id) |  |  |
| } |  |  |  |

Table 7.3.1-7C: NZP-CSI-RS-ResourceId for TRS(Id)

| Derivation Path: Table 4.6.3-86 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | n+Id-1 | n is the first NZP-CSI-RS-ResourceId allocated for TRS resource set. Value of n is left to internal implementation Id = 1,2,3,4 |  |
|  | n+Id+3 |  | SECOND_SET |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second TRS resource set configured in test, only applies to FR2 test |

Table 7.3.1-7D: NZP-CSI-RS-ResourceId for CSI

| Derivation Path: Table 4.6.3-86 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | n | n is the NZP-CSI-RS-ResourceId allocated for CSI-RS for CSI report. Value of n is left to internal implementation |  |

Table 7.3.1-7E: NZP-CSI-RS-ResourceId for BM(Id)

| Derivation Path: Table 4.6.3-86 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | n+Id | n is the first NZP-CSI-RS-ResourceId allocated for CSI-RS for BM. Value of n is left to internal implementation Id = 0,1 |  |

Table 7.3.1-8: CSI-RS-ResourceMapping for TRS(Id)

| Derivation Path: Table 4.6.3-45 with condition TRS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 5 | l0 = 5 for CSI-RS resource 1 and 3 | (Id = 1 or 3) AND FR1 |
|  | 9 | l0 = 9 for CSI-RS resource 2 and 4 | (Id = 2 or 4) AND FR1 |
|  | 1 |  | (Id = 1 or 3) AND FR2 |
|  | 2 |  | (Id = 1 or 3) AND FR2 AND SECOND_SET |
|  | 5 |  | (Id = 2 or 4) AND FR2 |
|  | 6 |  | (Id = 1 or 3) AND FR2 AND SECOND_SET |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second resource set for TRS configured in test, only applies to FR2 test |

Table 7.3.1-8A: CSI-RS-ResourceMapping for CSI

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000001 |  |  |
| } |  |  |  |
| nrofPorts | p2 |  | FR1 |
|  | p1 |  | FR2 |
| firstOFDMSymbolInTimeDomain | 5 |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

Table 7.3.1-8B: CSI-RS-ResourceMapping for BM (Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 |  |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 6 |  | Id = 0 |
|  | 10 |  | Id = 1 |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

Table 7.3.1-9: CSI-ResourcePeriodicityAndOffset for TRS(Id)

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots80 | 40 |  | (Id = 1 or 3) AND SCS120 |
|  | 41 |  | (Id = 2 or 4) AND SCS120 |
| slots40 | 20 | Periodicity 40 slots and offset 20 for CSI-RS resource 1 and 2 | (Id = 1 or 3) AND SCS30 |
|  | 21 | Periodicity 40 slots and offset 21 for CSI-RS resource 3 and 4 | (Id = 2 or 3) AND SCS30 |
| slots20 | 10 | Periodicity 20 slots and offset 10 for CSI-RS resource 1 and 2 | (Id = 1 or 3) AND SCS15 |
|  | 11 | Periodicity 20 slots and offset 11 for CSI-RS resource 3 and 4 | (Id = 2 or 4) AND SCS15 |
| } |  |  |  |

Table 7.3.1-9A: CSI-ResourcePeriodicityAndOffset for CSI

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 1 |  | SCS15 |
| slots10 | 2 |  | SCS30 |
| slots40 | 8 |  | SCS120 |
| } |  |  |  |

Table 7.3.1-9B: CSI-ResourcePeriodicityAndOffset for BM

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 1 |  | SCS15 |
| slots20 | 2 |  | SCS30 |
| slots80 | 8 |  | SCS120 |
| } |  |  |  |

Table 7.3.1-10: CSI-FrequencyOccupation-RRM

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| startingRB | 4*floor(n/4) | n is the start RB of active BWP floor() means rounding down to the nearest integer startingRB can only be multiple of 4 |  |
| nrofRBs | max(4*ceil(n/4-floor(n/4)+m/4),24) | m is the bandwidth of active BWP ceil() means rounding up to the nearest integer Bandwidth of CSI-RS used in RRM test is required to be same as active BWP according to 38.133. nrofRBs shall be no less than 24 and shall be multiple of 4 |  |
| } |  |  |  |

Table 7.3.1-11: NZP-CSI-RS-ResourceSet for RRM

| Derivation Path: Table 4.6.3-87 with Condition TRS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | NZP-CSI-RS-ResourceSetId-TRS |  |  |
|  | NZP-CSI-RS-ResourceSetId-TRS with condition SECOND_SET |  | SECOND_SET |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for TRS(1) | entry 1 |  |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for TRS(2) | entry 2 |  |
| NZP-CSI-RS-ResourceId[3] | NZP-CSI-RS-ResourceId for TRS(3) | entry 3 |  |
| NZP-CSI-RS-ResourceId[4] | NZP-CSI-RS-ResourceId for TRS(4) | entry 4 |  |
| } |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries |  | SECOND_SET |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for TRS(1) with condition SECOND_SET | entry 1 |  |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for TRS(2) with condition SECOND_SET | entry 2 |  |
| NZP-CSI-RS-ResourceId[3] | NZP-CSI-RS-ResourceId for TRS(3) with condition SECOND_SET | entry 3 |  |
| NZP-CSI-RS-ResourceId[4] | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | entry 4 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second TRS resource set configured in test, only applies to FR2 test |

Table 7.3.1-11A: NZP-CSI-RS-ResourceSet for CSI

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | NZP-CSI-RS-ResourceSetId-CSI |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for CSI | entry 1 |  |
| } |  |  |  |
| repetition | Not present |  |  |
| } |  |  |  |

Table 7.3.1-11B: NZP-CSI-RS-ResourceSet for BM

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId-BM |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for BM (0) | entry 1 |  |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for BM (0) | entry 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.1-11C: NZP-CSI-RS-ResourceSetId-TRS

| Derivation Path: Table 4.6.3-88 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | n | n is the first NZP-CSI-RS-ResourceSetId allocated for TRS resource set. Value of n is left to internal implementation |  |
|  | n+1 |  | SECOND_SET |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second TRS resource set configured in test, only applies to FR2 test |

Table 7.3.1-11D: NZP-CSI-RS-ResourceSetId-CSI

| Derivation Path: Table 4.6.3-88 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | n | n is the NZP-CSI-RS-ResourceSetId allocated for resource set of CSI-RS for CSI reporting. Value of n is left to internal implementation |  |

Table 7.3.1-11E: NZP-CSI-RS-ResourceSetId-BM

| Derivation Path: Table 4.6.3-88 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | n | n is the NZP-CSI-RS-ResourceSetId allocated for resource set of CSI-RS for BM. Value of n is left to internal implementation |  |

Table 7.3.1-12: CSI-ResourceConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-TRS |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-TRS 1 | entry 1 Editor's note: '1' was probably to be deleted. |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 2 entries |  | SECOND_SET |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSetId[2] | NZP-CSI-RS-ResourceSetId-TRS with condition SECOND_SET | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second resource set for TRS configured in test, only applies to FR2 test |

Table 7.3.1-12A: CSI-ResourceConfig for CSI

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-CSI |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-CSI | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| } |  |  |  |

Table 7.3.1-12B: CSI-ResourceConfig for BM

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-BM |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-BM |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| } |  |  |  |

Table 7.3.1-12C: CSI-ResourceConfigId-TRS

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of TRS. Value of n is left to internal implementation |  |

Table 7.3.1-12D: CSI-ResourceConfigId-CSI

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of CSI-RS for CSI reporting. Value of n is left to internal implementation |  |

Table 7.3.1-12E: CSI-ResourceConfigId-BM

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of CSI-RS for BM. Value of n is left to internal implementation |  |

Table 7.3.1-12F: CSI-ReportConfig for CSI

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-CSI |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-CSI |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic | Set according to parameters given in test cases |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.1-12G: CSI-ReportConfig for BM

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-BM |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-BM |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RSRP | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.1-12H: CSI-ReportConfigId-CSI

| Derivation Path: Table 4.6.3-40 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfigId | n | n is the CSI-ReportConfigId allocated for report config of CSI-RS for CSI. Value of n is left to internal implementation |  |

Table 7.3.1-12I: CSI-ReportConfigId-BM

| Derivation Path: Table 4.6.3-40 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfigId | n | n is the CSI-ReportConfigId allocated for report config of CSI-RS for BM. Value of n is left to internal implementation |  |

#### –	RACH-ConfigCommon

Table 7.3.1-13: RACH-ConfigCommon

| Derivation Path: TS 38.508-1 Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n48 |  |  |
| } |  |  |  |
| groupBconfigured | Not present |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| rsrp-ThresholdSSB | 51 |  |  |
| rsrp-ThresholdSSB-SUL | Not present |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 for the NR Cell. |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| restrictedSetConfig | unrestrictedSet |  |  |
| msg3-transformPrecoder | Not present | transform precoding is disabled for Msg3 PUSCH transmission and any PUSCH transmission scheduled with DCI format 0_0 |  |
| } |  |  |  |

#### –	RACH-ConfigGeneric

Table 7.3.1-14: RACH-ConfigGeneric

| Derivation Path: TS 38.508-1 Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 102 |  | FR1 |
|  | 190 |  | FR2 |
| msg1-FDM | one |  |  |
| msg1-FrequencyStart | 0 |  |  |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

#### –	PDCCH-Config

Table 7.3.1-15: PDCCH-ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| duration | 2 |  | FR1 |
|  | 1 |  | FR2 |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | Null |  |  |
| interleaved ::= SEQUENCE { |  |  | CCR |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | TCI-StateId-RRM(0) | entry 1 TCI State #0, QCLed to SSB index #0 |  |
|  | TCI-StateId-RRM(2) | TCI State #2, QCLed to TRS resource #4 in the first resource set | TRS |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CCR | When Control Channel RMC is configured |
| TRS | When at least one TRS resource set is configured. |

#### -	SchedulingRequestResourceConfig

Table 7.3.1-16: SchedulingRequestResourceConfig

| Derivation Path: Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 7 | With SCS = kHz15 results in repetition every 10 ms | SCS_15kHz |
| sl20 | 7 | With SCS = kHz30 results in repetition every 10 ms | SCS_30kHz |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for frequency of the cell according to clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| SCS_30kHz | SCS=30kHz for frequency of the cell according to clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |

#### -	SearchSpace

Table 7.3.1-17: SearchSpace

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl10 | 1 |  | SISS |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SISS | SearchSpace for SI |

#### –	PDSCH-Config

Table 7.3.1-18: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1+n1+n2+n3 entries | n1 = 1 if SSB configuration used in test case contains two SSBs in a burst, n1 = 0 otherwise n2 = 1 if TRS is configured in test case, n1 = 0 otherwise n3 = 1 if two resource sets of TRS are configured in test case, n3 = 0 otherwise |  |
| TCI-State[1] | TCI-State(0) | entry 1 QCLed to SSB index #0 |  |
| TCI-State[k, k=2..1+n1] | TCI-State(1) | entry ... QCLed to SSB index #1 | SECOND_SSB |
| TCI-State[k, k=2+n1..1+n1+n2] | TCI-State(2) | entry ... QCLed to TRS resource #4 in the first resource set | TRS |
| TCI-State[k, k=2+n1+n2..1+n1+n2+n3] | TCI-State(3) | entry ... QCLed to TRS resource #4 in the second resource set | SECOND_SET |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SSB | SSB configuration used in test case contain two SSBs in a burst |
| TRS | One resource set for TRS is configured in test case |
| SECOND_SET | Two resource sets for TRS are configured in test case, only applies to FR2. |

#### –	TCI-State

Table 7.3.1-19: TCI-State(Id)

| Derivation Path: Table 4.6.3-190 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-State ::= SEQUENCE { |  |  |  |
| tci-StateId | TCI-StateId-RRM(Id) |  |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| bwp-Id | BWP-Id of the active BWP |  | Id = 2 or 3 |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #0 |  | Id = 0 |
|  | SSB-Index of SSB #1 |  | Id = 1 |
| csi-rs | NZP-CSI-RS-ResourceId for TRS (4) |  | Id = 2 |
|  | NZP-CSI-RS-ResourceId for TRS (4) with condition SECOND SET |  | Id = 3 |
| } |  |  |  |
| qcl-Type | typeC |  | Id = 0 or 1 |
|  | typeA |  | Id = 2 or 3 |
| } |  |  |  |
| qcl-Type2 | Not present |  |  |
| qcl-Type2 SEQUENCE { |  |  | FR2 |
| cell | Not present |  |  |
| bwp-Id | Not present |  | Id = 0 or 1 |
|  | BWP-Id of the active BWP |  | Id = 2 or 3 |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #0 |  | Id = 0 |
|  | SSB-Index of SSB #1 |  | Id = 1 |
| csi-rs | NZP-CSI-RS-ResourceId for TRS (4) |  | Id = 2 |
|  | NZP-CSI-RS-ResourceId for TRS (4) with condition SECOND SET |  | Id = 3 |
| } |  |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.1-20: TCI-StateId-RRM(Id)

| Derivation Path: Table 4.6.3-191 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-StateId | n+Id | n is the first TCI-StateId allocated for TCI-State configured in RRM test. Value of n is left to internal implementation Id = 0,1,2,3 |  |

## 7.4	FFS

Void.

## 7.5	Common procedures for RRM testing

### 7.5.1	Procedure to configure SCC(s) for NR RRM CA testing

Same procedure as described in clause 5.5.1.

### 7.5.2	Procedure to configure SCC(s) for EN-DC RRM CA testing

Same procedure as described in clause 5.5.1.

Annex A (informative): Connection Diagrams

# A.1	Definition of Terms

System Simulator or SS – A device or system, that is capable of generating simulated Node B signalling and analysing UE signalling responses on one or more RF channels, in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Measurement and control of the UE Tx output power through TPC commands

2.	Measurement of Throughput

3.	Measurement of signalling timing and delays

4.	Ability to simulate UTRAN and/or E-UTRAN and/or GERAN signalling

Test System – A combination of devices brought together into a system for the purpose of making one or more measurements on a UE in accordance with the test case requirements. A test system may include one or more System Simulators if additional signalling is required for the test case. The following diagrams are all examples of Test Systems.

NOTE 1:	The above terms are logical definitions to be used to describe the test methods used in the documents TS 38.521-1, TS 38.521-2, TS 38.521-3, TS 38.523-1 and TS 38.533 in practice, real devices called 'System Simulators' may also include additional measurement capabilities or may only support those features required for the test cases they are designed to perform.

NOTE 2:	Components in the connection diagrams:
The components in the connection diagrams represent ideal components. They are intended to display the wanted signal flow. They don’t mandate real implementations.

Connection: Each connection is displayed as a one or two sided arrow, showing the intended signal flow. In some cases, for some tests, some connections shown may not be necessary (for example UL RX connection for a second cell).

Circulator: The signal, entering one port, is conducted to the adjacent port, indicated by the arrow. The attenuation among the above mentioned ports is ideally 0 and the isolation among the other ports is ideally ∞.

Splitter: a splitter has one input and 2 or more outputs. The signal at the input is equally divided to the outputs. The attenuation from input to the outputs is ideally 0 and the isolation between the outputs is ideally ∞.

Combiner: a combiner has one output and 2 or more inputs. The signals at the inputs are conducted to the output, all with the same, ideally 0 attenuation. The isolation between the inputs is ideally ∞.

Switch: contacts a sink (or source) alternatively to two or more sources (or sinks).

Fader: The fader has one input and one output. The MIMO fading channel is represented by several single faders (e.g. 8 in case of a MIMO antenna configuration 4x2) The correlation among the faders is described in TS 36.521-1 clause B.2.2. In some cases, for some tests, diagrams with fader(s) are referenced when no fading is required; in this case the fader(s) is omitted.

Attenuator: TBD

Test Equipment Part (TE): is the section of the connection diagram focused including a combination of devices to perform one or several measurements on a UE depending on the test requirements specified in 3GPP TS 38.101-1 [7], 3GPP TS 38.101-2 [8] and 3GPP TS 38.101-3 [9]. The basic TE is the system simulator to enable the connection between the gNB (and the eNB, if NSA mode) and the DUT. The number of cells, the number of streams per cell and how to combine them, channel and propagations conditions, etc. are also part of the TE. Other instruments as external spectrum analyser, interferer generators, external faders or external AWGN generators can be also considered part of the TE, as these instruments allow to measure a test requirement or to set the UE under certain conditions.

DUT Part (UE): for conducted measurement this section is focused on the number of physical antenna connectors and how to combine in the DUT. For radiated measurement this section shows the connections needed to translate the UL/DL streams to the radiated part.

# A.2	General considerations on Connections Diagram

In order to improve the maintainability and the readability of this section and to make easy to identify the whole connection diagram to use per each test case, several considerations have been used for this section:

-	The whole connection diagram to use for a specific test has been split in Test Equipment (TE) and User Equipment (UE) parts.

-	The same connection diagram will be used for SA and NSA, where the LTE link is specified in each connection diagram (TE and UE) with a dashed line (and this part will be only used for NSA).

-	To obtain the whole connection diagram required per each test case is necessary to specify the TE part required for each measurement and the UE part will depend on the UE antenna implementation.

# A.3	Setup Diagrams

## A.3.1	Test Equipment Parts for Conducted Measurements

### A.3.1.1	Basic Transmitter/Receiver tests

Figure A.3.1.1.1: Test Equipment connection for basic single cell, RX and TX tests

Figure A.3.1.1.2: Test Equipment connection for single cell, RX and TX tests for NR UL MIMO

Figure A.3.1.1.3: Test Equipment connection for NR CA, basic RX and TX tests

Figure A.3.1.1.4: Test Equipment connection for NR SUL, basic RX and TX tests

### A.3.1.2	Transmitter tests using Spectrum Analyser

Figure A.3.1.2.1: Test Equipment connection for TX-tests with additional Spectrum Analyzer

Figure A.3.1.2.2: Test Equipment connection for TX-tests for UL MIMO with additional Spectrum Analyser

Figure A.3.1.2.3: Test Equipment connection for NR CA TX-tests with additional Spectrum Analyser

Figure A.3.1.2.4: Test Equipment connection for NR SUL TX-tests with additional Spectrum Analyzer

### A.3.1.3	Transmitter tests using Spectrum Analyser and Signal Generator

Figure A.3.1.3.1: Test Equipment connection for Transmitter tests with CW Interference and spectrum analyser

Figure A.3.1.3.2: Test Equipment connection for Transmitter tests for UL MIMO with CW Interference and spectrum analyser

Figure A.3.1.3.3: Test Equipment connection for NR CA Transmitter tests with CW Interference and spectrum analyser

Figure A.3.1.3.4: Test Equipment connection for Transmitter tests for SUL with CW Interference and spectrum analyzer

### A.3.1.4	Receiver tests using Signal Generator

Figure A.3.1.4.1: Test Equipment connection for Receiver tests with Modulated Interference

Figure A.3.1.4.2: Test Equipment connection for Receiver tests with CW Interference

Figure A.3.1.4.3: Test Equipment connection for Receiver tests both Modulated and additional CW Interference signal

Figure A.3.1.4.4: Test Equipment connection for Receiver tests for UL MIMO with Modulated Interference

Figure A.3.1.4.5: Test Equipment connection for Receiver tests for UL MIMO with CW Interference

Figure A.3.1.4.6: Test Equipment connection for Receiver tests for UL MIMO with both Modulated and additional CW Interference signal

Figure A.3.1.4.7: Test Equipment connection for NR CA Receiver tests with additional Modulated Interference signal and/or CW Interference signal

Figure A.3.1.4.8: Test Equipment connection for NR SUL Receiver tests with Modulated Interference

Figure A.3.1.4.9: Test Equipment connection for NR SUL Receiver tests with CW Interference

### A.3.1.5	Receiver tests using Spectrum Analyser

Figure A.3.1.5.1: Test Equipment connection for RX-tests with additional Spectrum Analyzer

Figure A.3.1.5.2: Test Equipment connection for NR CA RX-tests with additional Spectrum Analyzer

### A.3.1.6	Receiver Performance tests

Figure A.3.1.6.1: Test Equipment connection for Receiver Performance tests with antenna configuration 1x2

### A.3.1.7	Demodulation Performance and CSI reporting tests

Figure A.3.1.7.1: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 2x2

Figure A.3.1.7.2: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 1x2

Figure A.3.1.7.3: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 1x4
(Note: LTE can be 2Rx or 4Rx and not dependent on NR #Rx)

Figure A.3.1.7.4: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 2x4 
(Note: LTE can be 2Rx or 4Rx and not dependent on NR #Rx)

Figure A.3.1.7.5: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 4x4
(Note: LTE can be 2Rx or 4Rx and not dependent on NR #Rx)

Figure A.3.1.7.6: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 4x2

Note: LTE can be 2Rx or 4Rx and not dependent on NR #Rx.

### A.3.1.8	RRM tests with more than one NR cell

The figures in this section represent connection diagrams for test cases with more than one NR cell. The parameters in the connection diagram, e.g. the number of cells n or the value of the phase rotator φi shall be defined by the test cases.

Figure A.3.1.8.1: Test Equipment connection for tests with more than one NR cell and antenna configuration 1x1

Figure A.3.1.8.2: Test Equipment connection for tests with more than one NR cell and antenna configuration 1x2

Figure A.3.1.8.3: Test Equipment connection for tests with more than one NR cell and antenna configuration 1x2 and fading

Figure A.3.1.8.4: Test Equipment connection for tests with more than one NR cell for 4Rx capable UEs with fading

Figure A.3.1.8.5: Test Equipment connection for tests with more than one NR cell and antenna configuration 1x4

## A.3.2	User Equipment Parts for Conducted Measurements

### A.3.2.1	General

The User Equipment part is focused on the number of physical antenna connectors and how to combine in the DUT. Depending on the DUT implementation only one of the following connection diagrams applies. These connection diagrams are examples of User equipment parts.

### A.3.2.2	One Antenna Connector

Figure A.3.2.2.1: User Equipment connection for single basic cell

### A.3.2.3	Two Antenna Connectors

Figure A.3.2.3.1: User Equipment connection for single basic cell with NR and LTE cells at different separated connectors

Figure A.3.2.3.2: User Equipment connection for single basic cell with NR and LTE cells at the same connectors for both cells

Figure A.3.2.3.3: 2 Tx User Equipment connection for single basic cell with NR and LTE cells at the same connectors for both cells and 2TX UL MIMO supported

Figure A.3.2.3.4: User Equipment connection for UEs with NR and LTE RxTx and Rx antenna at same connectors

Figure A.3.2.3.5: User Equipment connection for single basic cell with NR and LTE cells at different separated connectors with NR SUL and NR NUL transmitted on separate antenna connectors

Figure A.3.2.3.6: User Equipment connection for single basic cell with NR and LTE cells at different separated connectors with NR SUL and NR NUL transmitted on the same antenna connector

Figure A.3.2.3.7: User Equipment connection for single basic cell with NR and LTE cells at the same connectors for both cells with NR SUL and NR NUL transmitted on separate antenna connectors

Figure A.3.2.3.8: User Equipment connection for single basic cell with NR and LTE cells at the same connectors for both cells with NR SUL and NR NUL transmitted on the same antenna connector

### A.3.2.4	Three Antenna Connectors

Figure A.3.2.4.1: 2Tx User Equipment connection for single basic cell with NR and LTE cells at different separated connectors and 2TX UL MIMO supported

### A.3.2.5	Four Antenna Connectors

Figure A.3.2.5.1: User Equipment connection for UEs with NR and LTE RxTx and Rx antenna at different separated connectors

Figure A.3.2.5.2: User Equipment connection for 4Rx capable UEs without any 2Rx RF bands (NR and LTE at same connectors)

Figure A.3.2.5.3: User Equipment connection for UEs with NR CA (component carriers on common connector) and LTE

### A.3.2.6	Over Four Antenna Connectors

Figure A.3.2.6.1: User Equipment connection for UEs with NR CA (component carriers on separated connectors) and LTE

Figure A.3.2.6.2: User Equipment connection for UEs with NR CA and NR 4Rx (component carriers on separated connectors) and LTE

(Note: NR may be 2Rx on some of the CCs, in that case RX3 and RX4 are not used).

## A.3.3	Test Equipment Parts for Radiated Measurements

### A.3.3.1	Basic Transmitter/Receiver tests

The Test Equipment part is focused on logical representation of TE measurement and link antenna(s) and positioner controller. The Test Equipment connection diagram below is applicable for NR radiated RX and TX tests, including CA and UL MIMO tests.

Figure A.3.3.1.1: TE diagram for radiated RX and TX tests

### A.3.3.2	Demodulation and CSI tests

Figure A.3.3.2.1: Demodulation and CSI tests

## A.3.4	User Equipment Parts for Radiated Measurements

### A.3.4.1	Basic Transmitter/Receiver tests

The User Equipment part is focused on logical representation of UE antenna(s), DUT positioner and positioner controller. The UE connection diagram below is applicable for NR radiated RX and TX tests, including CA and UL MIMO tests.

Figure A.3.4.1.1: UE diagram for radiated RX and TX tests

### A.3.4.2	Demodulation and CSI tests

Same as Figure A.3.4.1.1

Annex B (normative): Permitted test methods For OTA Testing

# B.1	General

Editor's Note: The working assumption is that the DFF or IFF: CATR based OTA test methodologies defined in Annexes B.2.2 and B.2.4 respectively should be used for Signalling test.

The applicability of the permitted test methods herein is defined by the appropriate references within clauses 5, 6, and 7. A summary of the applicability is shown in Table B.1-1.

Table B.1-1: Permitted Test Methods Applicability Summary

FFS

# B.2	Permitted Test Methods

## B.2.1	General

The main objective of this annex is to specify basic parameters of permitted OTA test methods suitable for RF Tx and Rx, Performance, and RRM measurements and Signalling Conformance tests performed at high frequency in the FR2 operating bands defined in clause 4.3.1.2. The applicability of each OTA test method is summarized in Table B.1-1.

## B.2.2	Direct far field (DFF)

### B.2.2.1	Description

The DFF measurement setup for FR2 is capable of centre and off-centre of beam measurements and is shown in Figure B.2.2.1-1 below.

Figure B.2.2.1-1: DFF measurement setup

The key aspects of the DFF setup are:

-	Far-field measurement system in an anechoic chamber

-	The criterion for determining the far-field distance is described in B.2.2.4.

-	A positioning system such that the angle between the dual-polarized measurement antenna and the DUT has at least two axes of freedom and maintains a polarization reference.

-	A positioning system such that the angle between the link antenna and the DUT has at least two axes of freedom and maintains a polarization reference; this positioning system for the link antenna is in addition to the positioning system for the measurement antenna and provides for an angular relationship independently controllable from the measurement antenna.

-	For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1 UL configuration, an LTE link antenna is used to provide the LTE link to the DUT. The LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

-	For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

The applicability criteria of the DFF setup are:

-	The DUT radiating aperture is D ≤ 5 cm

-	Either a single radiating aperture, multiple non-coherent apertures, or multiple coherent apertures DUTs can be tested

-	If multiple antenna panels that are phase coherent are defined as a single array, the criterion on DUT radiating aperture applies to this single array

-	D is based on the MU assessment in Annex B.1.1.3 of TR 38.810 [24]

-	A measurement distance larger than the far-field criteria defined in B.2.2.4 is not precluded

-	If the uncertainties can be further optimized, the MU may be reduced or D may be increased

-	A manufacturer declaration on the following elements is needed unless the entire DUT size is contained in a sphere of diameter of ≤ 5 cm:

-	Manufacturer declares antenna array size

### B.2.2.2	Quiet zone dimension

The quiet zone shall be large enough to fully contain the DUT. In order to allow testing of DUTs of various size and to allow for flexibility in test chamber implementations, there will be two defined quiet zone dimensions. The smaller quiet zone shall have a radius of 100mm to accommodate DUTs such as smartphones. The larger quiet zone shall have a radius of 150mm to accommodate larger DUTs such as tablets. The device types are listed as examples and other device types are not precluded. In either case, the DUT shall be fully contained in one of the quiet zone sizes defined herein.

### B.2.2.3	Quality of the quiet zone

The quality of the quiet zone shall be measured for the frequencies defined in FFS. The measured quality of the quiet zone performance is used in uncertainty calculations for the appropriate quality of the quiet zone dimension utilized for the DUT.

### B.2.2.4	Measurement Distance

For far-field measurements, the distance R between the DUT and the measurement antenna shall be calculated by the following equation.

where  is the largest wavelength within the frequency band of interest and D is the diameter of the smallest sphere that encloses the radiating parts of the DUT.

For DFF, free space path loss is calculated by applying the Free Space Loss formula with R equal to the far field distance: .

The minimum range length of a DFF system, i.e., the minimum distance between the centre of the quiet zone and the measurement antenna, needs to take into account the unknown offset of the antenna aperture from the centre of quiet zone in order to guarantee far-field conditions for any antenna array integrated inside the DUT. The distance between the centre of the quiet zone to the measurement antenna is referenced as RDFF, while the radius of the quiet zone is RQZ as illustrated in Figure B.2.2.4-1. The minimum distance between the antenna array integrated anywhere within the DUT and the measurement antenna needs to meet the far-field distance, RFF = 2D2/.

Figure B.2.2.4-1: Illustration of DFF System for range length definition

The setup in Figure B.2.2.4-2 is used to derive the minimum range length for NR FR2 DFF systems where the sphere enclosing the DUT matches the QZ and the DUT antenna with radiating aperture diameter D located in the corner of the DUT. With this setup, the minimum range length, RDFF, can be determined as

RDFF = RQZ – D/2 + RFF = RQZ – D/2 + 2D2/

which is tabulated in Table B.2.2.4-1 for two different QZ sizes assuming D=5cm.

Figure B.2.2.4-2: Illustration of DFF System for minimum range length definition

Table B.2.2.4-1: Minimum Range Length of DFF System for D = 5cm

| f [GHz] QZ [cm] | 24.25 | 30 | 40 | 50 | 52.6 |
| --- | --- | --- | --- | --- | --- |
| 20 | 0.48 | 0.58 | 0.74 | 0.91 | 0.95 |
| 30 | 0.53 | 0.63 | 0.79 | 0.96 | 1.00 |

The influence of measurement distance on measurement uncertainty is discussed in Annex B.2.1 of TR 38.903 [XX].

## B.2.3	Direct far field (DFF) setup simplification for centre of beam measurements

### B.2.3.1	Description

The DFF setup in Annex B.2.2 can be simplified in the following way to perform centre of the beam measurements:

-	The measurement and the link antenna can be combined so that the single antenna is used to steer the beam and to perform UE measurements.

The measurement setup for FR2 capable of centre of beam measurements is shown in Figure B.2.3.1-1 below.

Figure B.2.3.1-1: DFF simplification for centre of beam measurement setup

The applicability criteria of the simplified DFF setup for centre of beam measurements are defined in B.2.2.1.

### B.2.3.2	Quiet zone dimension

Same as Annex B.2.2.2.

### B.2.3.3	Quality of the quiet zone

Same as Annex B.2.2.3.

### B.2.3.4	Measurement Distance

Same as Annex B.2.2.4.

## B.2.4	Indirect far field (IFF): Compact Antenna Test Range (CATR)

### B.2.4.1	Description

The IFF method utilizing a compact antenna test range (CATR) creates the far field environment using a transformation with a parabolic reflector.

The IFF CATR measurement setup for FR2 is capable of centre and off-centre of beam measurements and an example setup is shown in Figure B.2.4.1-1 below. The relative orientation of the coordinate system with respect to the reflector and the axes of rotation apply to any CATR measurement setup.

Figure B.2.4.1-1: Example of IFF: CATR measurement setup

The key aspects of this test method setup are:

-	Indirect Far Field using Compact Antenna Test Range as described in TR 38.810 [24] with quiet zone diameter that meets the requirements of B.2.4.2.

-	A positioning system such that the angle between the dual-polarized measurement antenna and the DUT has at least two axes of freedom and maintains a polarization reference.

-	Before performing the UE Beamlock Test Function as defined in clause 4.9.2, the measurement probe acts as a link antenna maintaining polarization reference with respect to the DUT. Once the beam is locked then the link is to be passed to the link antenna which maintains reliable signal level with respect to the DUT.

-	For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1UL configuration, an LTE link antenna is used to provide the LTE link to the DUT. The LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

-	For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

The applicability criteria of this test method are:

-	The total test volume, i.e., the quiet zone is defined as a sphere with radius R.

-	DUT must fit within the quiet zone for the entire duration of the test.

-	Either a single radiating aperture, multiple non-coherent apertures or multiple coherent apertures DUTs can be tested.

-	No manufacturer declaration of the antenna array size is needed.

### B.2.4.2	Quiet zone dimension

Same as Annex B.2.2.2.

B.2.4.3	Quality of the quiet zone

Same as Annex B.2.2.3.

### B.2.4.4	Measurement Distance

The CATR system does not require a measurement distance of  to achieve a plane wave as in a standard far field range.

For the CATR system, the far-field distance is seen as the focal length. The focal length is the distance between the feed and the reflector of the CATR. Further information on the focal length of a CATR system can be found in clause 5.2.3.2 of TR 38.810 [24].

The measurement distance for any CATR system implementation shall be adequate to meet the quiet zone dimensions defined in B.2.4.2.

In a CATR, from the reflector to the quiet zone, there is a plane wave with no free space path loss.

For CATR, free space path loss is calculated by applying the Free Space Loss formula with R equal to the far field distance based on the focal length: .

A summary of the comparison of path losses which can be expected for the CATR compared to a Fraunhofer limit distance () for different antenna sizes and frequencies can be found in clause 5.2.3.2 of TR 38.810 [24].

The influence of measurement distance on measurement uncertainty can be considered as zero as defined in Annex B.2.2 of TR 38.903 [XX].

## B.2.5	Near field to far field transform (NFTF)

### B.2.5.1	Description

The NFTF method computes the metrics defined in Far Field by using the Near Field to Far Field transformation.

The NFTF measurement setup of UE RF characteristics for FR2 is capable of centre and off centre of beam measurements and an example setup is shown in Figure B.2.5.1-1:

Figure B.2.5.1-1: Example of NFTF measurement setup

The key aspects of the Near Field test range are:

-	Radiated Near Field UE beam pattern is measured and based on the NFTF mathematical transform, the final metric such as EIRP is the same as the metric for the baseline setup

-	A positioning system such as the angle between the dual-polarized measurement/link antenna and the DUT has at least two axes of freedom and maintains a polarization reference

-	For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1UL configuration, an LTE link antenna is used to provide the LTE link to the DUT. The LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

-	For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

The applicability criteria of the NFTF setup are:

-	The DUT radiating aperture is D ≤ 5 cm

-	Either a single radiating aperture, multiple non-coherent apertures or multiple coherent apertures DUTs can be tested

-	If multiple antenna panels that are phase coherent are defined as a single array, the criterion on DUT radiating aperture applies to this single array

-	D is based on the MU assessment in Annex B.1.4.3 of TR 38.810 [24]

-	If the uncertainties can be further optimized, the MU may be reduced or D may be increased

-	A manufacturer declaration on the following elements is needed unless the entire DUT size is contained in a sphere of diameter of ≤ 5 cm:

-	Manufacturer declares antenna array size

-	EIRP, TRP, and spurious emissions metrics can be tested.

### B.2.5.2	Quiet zone dimension

Same as Annex B.2.2.2.

### B.2.5.3	Quality of the quiet zone

Same as Annex B.2.2.3.

### B.2.5.4	Measurement Distance

The NFTF system does not require a measurement distance of  as in a standard far field range due to the use of the Near Field to Far Field transformation.

The measurement distance for any NFTF system implementation shall ensure that the DUT is not measured in the reactive near-field region and is adequate to meet the quiet zone dimensions defined in B.2.5.2.

Annex C (informative): Calculation of test frequencies

# C.0	General

Test frequencies are defined in clause 4.3.1 with extensions for signalling test cases in clause 6.2.3. This annex gives a guideline to determine these test frequencies and the associated signalling parameters for a given NR band, NR CA or NR DC band combination.

Clause C.1 describes definitions and parameters used by the procedures to determine test frequencies, SS/PBCH Block (SSB) and CORESET#0 configuration parameters.

Clause C.2 describes how to calculate test frequencies for symmetric NR bands, asymmetric NR bands, NR CA and NR DC configurations.

Clause C.3.2 describes how to determine the SSB, CORESET#0 and signalling parameters for a PCell.

Clause C.3.3 describes how to determine the SSB and signalling parameters for a Cell without CORESET#0.

# C.1	Definitions and Parameters

Figure C.1-1 shows SSB and CORESET#0 and related parameters.

Figure C.1-1: location of SSB and CORESET#0 within a channel

The parameters referenced in figure C.1-1 are defined in Table C.1-1.

Table C.1-1: Definition of parameters in Figure C.1-1 used in Annex C.

| Parameter | Description |
| --- | --- |
| FPointA | Reference Point A frequency. |
| Fcarrier | Fcarrier is the centre frequency of a carrier corresponding to its NR-ARFCN value. |
| FcarrierLow | FcarrierLow is the centre frequency of lowest subcarrier of the carrier. FcarrierLow = Fcarrier - 12 * SCSCarrier * (NRB / 2) with NRB according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8] for the channel bandwidth of the carrier. |
| ΔFcarrierBandwidth | ΔFcarrierBandwidth is the carrier’s channel bandwidth as provided in carrierBandwidth to the UE (SCS-SpecificCarrier). |
| ΔFoffsetToCarrier | ΔFoffsetToCarrier is the frequency offset between Point A and the lower edge of the carrier. FoffsetToCarrier = offsetToCarrier * PRB size, where PRB size according to the subcarrier spacing of the carrier. offsetToCarrier is signalled to the UE (SCS-SpecificCarrier). |
| FSSref | Centre frequency of SSB. For a cell selectable as PCell the FSSref corresponds to a valid GSCN value according to clause 5.4.3.1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFOffset(RBs) | ΔFOffset(RBs) = 12* Offset(RBs) * subCarrierSpacingCommon, where Offset(RBs) is given in tables 13-1 to 13-10 of TS 38.213 [22]. |
| ΔFkSSB | ΔFKSSB = kSSB * subcarrier spacing of SSB (SCSSSB). |
| ΔFOffsetSSB-CORESET0 | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of CORESET#0. ΔFOffsetSSB-CORESET0 = ΔFOffset(RBs) + ΔFkSSB. |
| ΔFOffsetCORESET-0-Carrier | Frequency offset, FOffsetCORESET#0-Carrier, between the lowest subcarrier of CORESET#0 and the lowest subcarrier of the carrier. |
| ΔFOffsetCORESET-1-Carrier | Frequency offset, FOffsetCORESET1-Carrier, between the lowest subcarrier of CORESET#1 and the lowest subcarrier of the carrier. |
| ΔFOffsetSSB-Carrier | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of the carrier. |
| FCORESET0Low | Centre frequency of subcarrier 0 of CORESET#0. |
| FOffsetToPointA | Frequency of the lowest subcarrier of the lowest resource block, which has the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and overlaps with the SS/PBCH block used by the UE for initial cell selection, expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2 (TS 38.211 [29] clause 4.4.4.2). |
| ΔFOffsetToPointA | Frequency offset between FOffsetToPointA and point A. ΔFOffsetToPointA = offsetToPointA * {15 kHz for FR1; 60 kHz for FR2} (TS 38.211 [29] clause 4.4.4.2). |

Additional parameters used in this annex are defined in Table C.1-2.

Table C.1-2: Definition of additional parameters used in Annex C.

| kSSB | as defined in TS 38.211 [29] clause 7.4.3.1 |
| --- | --- |
| SCSCarrier | subcarrier spacing for the carrier (SCS-SpecificCarrier): FR1: 15kHz, 30kHz or 60kHz according to TS 38.101-1 [7] Table 5.3.5-1 FR2: 60kHz or 120kHz according to TS 38.101-2 [8] Table 5.3.5-1 |
| SCSSSB | SS/PBCH block subcarrier spacing FR1: 15kHz or 30kHz according to TS 38.101-1 [7] Table 5.4.3.3-1 FR2: 120kHz or 240kHz according to TS 38.101-2 [8] Table 5.4.3.3-1 NOTE: According to the tables in clause 13 of TS 38.213 [22] not all combinations of SCSSSB and SCSCarrier are applicable. |
| SCScommon | Subcarrier spacing for SIB1, Msg.2/4 for initial access, paging and broadcast SI-messages. Provided to the UE in the MIB in IE subCarrierSpacingCommon. |
| PRBsize | Physical Resource Block size of the carrier = 12 * SCSCarrier. |
| CRBsize | Common Resource Block size = 12 * SCScommon. |
| FDL_Low, FUL_Low | Lowest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FDL_High, FUL_High | Highest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFRaster | Frequency raster of the band as defined in clause 5.4.2.3 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| BWDL | Bandwidth of downlink frequency range of the band. |
| BWUL | Bandwidth of uplink frequency range of the band. |
| CBWDL | Downlink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| CBWUL | Uplink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FTx-Rx_separation | Default Tx – Rx carrier centre frequency separation of the band as defined in clause 5.4.4 of TS 38.101-1 [7]. For TDD bands FTx-Rx_separation = 0. |
| ΔFTx-Rx_separation | ΔFTx-Rx = ¦ (BWDL – BWUL)/2 ¦ is the deviation to the default Tx-Rx carrier centre frequency separation (FTx-Rx_separation) for FDD FR1 bands supporting asymmetric channel bandwidths as defined in clause 5.3.6 of TS 38.101-1 [7]. |
| BWSSB | Bandwidth of the SSB. BWSSB = 12 * SCSSSB * 20 |
| ΔGSCN, GSCNMIN, GSCNMAX | GSCN step size, GSCN minimum and GSCN maximum values for the NR band according to table 5.4.3.3-1 of TS 38.101-1 [7] and TS 38.101-2 [8] |
| OffsetRBs | Offset (RBs) according to the applicable table 13-1 to 13-10 in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier. |
| OffsetRBs,max | Maximum value for Offset (RBs) for the carrier. |
| OffsetRBs,min | Minimum value for Offset (RBs) for the carrier. |

# C.2	Determination of test frequencies

## C.2.0	General

Test frequencies are determined as:

For symmetric NR bands (supporting same bandwidth in UL and DL):

-	test frequencies for the supported symmetric channel bandwidth combinations are determined as described in clause C.2.1; and

-	the test frequencies for the supported asymmetric channel bandwidth combinations are determined as described in clause C.2.3.

For asymmetric NR bands (supporting different bandwidth in UL and DL):

-	the test frequencies for the supported symmetric channel bandwidth combinations are determined as described in clause C.2.2; and

-	the test frequencies for the supported asymmetric channel bandwidth combinations are determined as described in clause C.2.3.

For NR CA and NR DC:

-	the test frequencies are determined as described in the relevant subclause in C.2.4 depending to the type of configuration.

The carrier test frequencies are determined considering the channel raster according to clause 5.4.2.3 in TS 38.101-1 [7] for FR1 and in TS 38.101-2 [8] for FR2.

## C.2.1	Determination of test frequencies for symmetric NR bands and symmetric uplink and downlink channel bandwidth combinations

### C.2.1.1	Determination of test frequencies for Low-, Mid- and High-Range

Downlink:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq1 |
| --- | --- |
| FDL_MidRange = Round((FDL_Low + BWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq2 |
| FDL_HighRange = Floor((FDL_High - CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq3 |

FDL_LowRange is rounded up and FDL_HighRange is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

Uplink:

| FUL_LowRange = FDL_LowRange - FTx-Rx_separation | C.2.1.1-Eq4 |
| --- | --- |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation | C.2.1.1-Eq5 |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation | C.2.1.1-Eq6 |

### C.2.1.2	Determination test frequencies for of Mid-Low and Mid-High-Range for signalling tests

| FMid-LowRange = Round((FLowRange+ (FHighRange - FLowRange )/3) / ΔFRaster) * ΔFRaster | C.2.1.2-Eq1 |
| --- | --- |
| FMid-HighRange = Round((FLowRange + 2*(FHighRange - FLowRange )/3) / ΔFRaster) * ΔFRaster | C.2.1.2-Eq2 |

## C.2.2	Determination of test frequencies for asymmetric NR bands and symmetric uplink and downlink channel bandwidth combinations

Determination of test frequencies for asymmetric NR bands, and symmetric uplink and downlink channel bandwidth combinations are determined using the procedure in clause C.2.3 with ΔFTx-Rx = 0.

## C.2.3	Determination of test frequencies for bands supporting asymmetric channel bandwidth combinations

### C.2.3.1	General

The following procedure is used to calculate test frequencies for NR bands supporting asymmetric UL and DL channel bandwidths as described below, where CBWUL and CBWDL refer to the carrier’s UL and DL channel bandwidths; and BWUL and BWDL refer to the band’s total UL and DL bandwidths.

The procedure is also used to calculate test frequencies for symmetric UL and DL bandwidth combinations for asymmetric NR bands.

For FDD bands supporting asymmetric uplink and downlink bandwidth combinations a deviation of ΔFTX-RX (C.2.3.1-Eq1) is to be added to the default Tx-Rx carrier centre frequency separation, FTx-Rx_separation (TS 38.101-1 [7] clause 5.3.6).

| ΔFTX-RX = ¦(CBWDL – CBWUL)/2¦ | C.2.3.1-Eq1 |
| --- | --- |

For the case of asymmetric NR bands and symmetric UL and DL bandwidth combinations ΔFTX-RX = 0.To meet the Tx-Rx frequency separation requirement for asymmetric NR bands were the supported overall UL bandwidth is smaller than the supported overall DL bandwidth it may not be possible to cover the full DL frequency range for all UL and DL channel bandwidth combinations. For CA when such band is only used for DL CC the full range can be used for all DL channel bandwidths.

To maximize the tested frequency range for the non-CA case the UL frequency range, as being smaller than the DL frequency range, need to be used as the starting point to calculate the uplink and downlink test frequencies.

### C.2.3.2	Determination of Low-, Mid- and High-Range for bands supporting asymmetric uplink and downlink bandwidth combinations

The following procedure is used to determine the test frequencies for Low-, Mid- and High-Range for bands supporting asymmetric UL and DL bandwidth combinations.

1.	Calculate uplink carrier centre frequencies:

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq1 |
| --- | --- |
| FUL_MidRange = Round((FUL_Low + BWUL_Band/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq2 |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq3 |

2.	Calculate the downlink frequencies:

Calculate the DL carrier centre frequencies from the UL frequencies in step 1.

| FDL_LowRange = FUL_LowRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq4 |
| --- | --- |
| FDL_MidRange = FUL_MidRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq5 |
| FDL_HighRange = FUL_HighRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq6 |

3.	Check that the calculated centre test frequencies in step 2 for the BWDL fits within the bands DL frequency range:

If FDL_LowRange is smaller than the lowest frequency of the band then recalculate the minimum FDL_LowRange and modify the associated FUL_LowRange as:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq7 |
| --- | --- |
| FUL_LowRange = FDL_LowRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8 |

If FDL_HighRange is larger than the higher frequency of the band then recalculate the maximum FDL_HighRange and modify the associated FUL_HighRange as:

| FDL_HighRange = Floor((FDL_Low + BWDL_Band – CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq9 |
| --- | --- |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq10 |

## C.2.4	Frequency determination for NR CA and NR DC configurations

### C.2.4.1	Determination of test frequencies for NR Inter-band CA and NR DC

Test frequencies for NR Inter-band CA configurations and NR DC use the single carrier test frequencies for each of the included NR bands in the configuration as specified in clause 4.3.1.1.1 for FR1 bands and in clause 4.3.1.2.1 for FR2 bands.

### C.2.4.2	Determination of test frequencies for NR Intra-band Contiguous CA

#### C.2.4.2.1	General

By default, test frequencies for NR Intra-band Contiguous CA in clause 4.3.1 are specified using the nominal channel spacing between the carrier components as specified in TS 38.101-1 [7] clause 5.4A.1 for FR1 and TS 38.101-2 [8] clause 5.4A.1 for FR2. In addition, some NR bands may have test frequencies specified based on an adjusted channel spacing as specified in in TS 38.101-1 [7] clause 5.4A.1 for FR1 and TS 38.101-2 [8] clause 5.4A.1 for FR2.

The test frequencies for NR Intra-band Contiguous CA is calculated for each CC and specific test cases can decide which CC is used as PCell. This means that all CC test frequencies is calculated with a CORESET#0.

Note:	For NR Intra-band Contiguous CA configurations for bands where Tx frequency range is lower than Rx frequency range the RAN4 requirements for reference sensitivity testing is specified having the PCC frequency lower than the SCC frequencies such that UL PRB maximise the Tx-Rx separation. This means that CC1 shall be used as PCell in the reference test case for bands where Tx frequency range is lower than Rx frequency range; and highest CC shall be used as PCell in the reference test case for bands where Tx frequency range is higher than Rx frequency range.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate carrier components (CC) test frequencies for NR Intra-band Contiguous and Non-contiguous CA configurations:

| Parameter | Description |
| --- | --- |
| NCC | Number of CCs in the for NR Intra-band configuration |
| CCBWDL(i) | Channel bandwidth (MHz) of downlink CC(i), where i = 1 to NCC |
| FChannel_Spacing(i) | Channel spacing between CC(i) and CC(i+1), where i = 1 to (NCC-1) |

#### C.2.4.2.2	Determination of test frequencies for Low-, Mid- and High-Range

Downlink CC(1), lowest frequency CC:

FDL_LowRange_CC(1) is rounded up and FDL_HighRange_CC(1) is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

| FDL_LowRange_CC(1) = Ceil((FDL_Low + CCBWDL(1) / 2) / ΔFRaster) * ΔFRaster | C.2.4.2.2-Eq1 |
| --- | --- |
| FDL_MidRange_CC(1) = Round((FDL_Low + BWDL/2 – ∑k=1 to (Ncc) CCBWDL (k)/2 + CCBWDL(1)/2) / ΔFRaster) * ΔFRaster | C.2.4.2.2-Eq2 |
| FDL_HighRange_CC(1) = Floor((FDL_High - CCBWDL(NCC)/2– ∑k=1 to (Ncc-1) FChannel_Spacing(k)) / ΔFRaster) * ΔFRaster | C.2.4.2.2-Eq3 |

Downlink CC(2) to CC(NCC), in increasing frequency order:

| FDL_LowRange_CC(i) = FDL_LowRange_CC(i-1) + FChannel_Spacing(i)), i=2 to NCC | C.2.4.2.2-Eq4 |
| --- | --- |
| FDL_MidRange_CC(i) = FDL_MidRange_CC(i-1) + FChannel_Spacing(i)), i=2 to NCC | C.2.4.2.2-Eq5 |
| FDL_HighRange_CC(i) = FDL_HighRange_CC(i-1) + FChannel_Spacing(i)), i=2 to NCC | C.2.4.2.2-Eq6 |

Uplink CC(i), i=1 to NCC:

| FUL_LowRange_CC(i) = FDL_LowRange_CC(i) - FTx-Rx_separation | C.2.4.2.2-Eq7 |
| --- | --- |
| FUL_MidRange_CC(i) = FDL_MidRange_CC(i) - FTx-Rx_separation | C.2.4.2.2-Eq8 |
| FUL_HighRange_CC(i) = FDL_HighRange_CC(i) - FTx-Rx_separation | C.2.4.2.2-Eq9 |

### C.2.4.3	Determination of test frequencies for NR Intra-band Non-Contiguous CA

#### C.2.4.3.1	General

The default test frequencies in clause 4.3.1 for NR Intra-band Non-Contiguous CA are based on maximum Wgap between the carrier components of the different bands.

Test frequencies with Wgap different from maximum Wgap are specified in the specific test cases using them.

#### C.2.4.3.2	Maximum Wgap, two non-contiguous components

For the case a NR Intra-band Non-Contiguous CA configuration is limited to two non-contiguous components, where each non-contiguous components may be a single CC component or a multi CC contiguous CA component, the test frequencies for each non-contiguous component is selected such that the first component use the test frequency of the Low range and the second component use the test frequency for High range.

If a non-contiguous component is single CC component, then the test frequencies as specified in clause 4.3.1.1.1 for FR1 bands and in clause 4.3.1.2.1 for FR2 bands are used. If a non-contiguous component is a multi CC contiguous CA component, then the test frequencies as specified in clause 4.3.1.1.3 for FR1 bands and in clause 4.3.1.2.3 for FR2 bands are used.

#### C.2.4.3.3	Maximum Wgap, more than two non-contiguous components

For the case a NR Intra-band Non-Contiguous CA configuration includes more than two non-contiguous components, where each non-contiguous components may be a single CC component or a multi CC contiguous CA component the maximum Wgap is calculated as:

| Maximum Wgap = (BWDL - ∑k=1 to (Ncc-1) CCBWDL(i)) / (NCC-1) | C.2.4.3.3-Eq1 |
| --- | --- |

For each non-contiguous components the test frequencies are calculated based on the principles in clause C.2.1 for symmetric NR bands with symmetric uplink and downlink channel bandwidth combinations or; in clause C.2.2 for asymmetric NR bands with symmetric uplink and downlink channel bandwidth combinations; in clause C.2.3 for symmetric or asymmetric NR bands with asymmetric uplink and downlink channel bandwidth combinations.

# C.3	Determination of SSB and CORESET#0

## C.3.1	General

The requirements to be met and the principles used for determining the SSB and CORESET#0 for a PCell are:

1.	The complete SSB and CORESET#0 shall be within the carrier’s channel bandwidth.

2.	The SSB centre frequency (SSref) shall be on the synchronisation raster.

3.	The SSB shall be kept as close as possible to the carrier’s lower edge centre frequency.

4.	CORESET#0 configuration is selected using lowest number of RBs and symbols in applicable table in TS 38.213 clause 13.

5.	The first SSB subcarrier shall be aligned with the defined resource grid given by SCS indicated by subCarrierSpacingCommon in the MIB.

## C.3.2	Determination of SSB, CORESET#0 and signalling parameters for a PCell

The following procedure is used to determine an SSB on the synchronisation raster (GSCN) and a CORESET#0 configuration (kSSB, OffsetRBs and OffsetToPointA) as close as possible to the carrier’s lower edge. See figure C1-1 and clause C.1 for definition of parameters referenced in the procedure.

1.	Determine SSB and CORESET#0:

1a.	Calculate the lower of FSSref, FSSref_Min, correspondent to SSB lowest subcarrier being at the same frequency as the carrier’s lowest subcarrier; and the higher limit of FSSref, FSSref_Max, correspondent to SSB highest subcarrier being at the same frequency as the carrier’s highest subcarrier FSSref_Min

FcarrierLow = see formula for FcarrierLow in Table C.1-1

FSSref_Min = FcarrierLow + CRBsize * OffsetRBs,min + BWSSB / 2

FSSref_Max = FcarrierLow + ΔFcarrierBandwidth - BWSSB / 2

1b.	Calculate GSCNMIN correspondent to FSSref_Min in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2 and select the closest valid GSCN value with GSCN >= GSCNMIN for the carrier in according to the carrier’s synchronisation raster as specified in clause 5.4.3.3 in TS 38.101-1 [7] and TS 38.101-2 [8].

1c.	Calculate the FSSref for the selected GSCN value in step 1b in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2.

1d.	Calculate the frequency FoffsetToPointA, which is the lowest subcarrier of the lowest resource block with the subcarrier spacing being a multiple of resource blocks expressed in terms of common PRB size and overlaps with the SS/PBCH block subcarrier 0 of the first resource block of the SS/PBCH block, FSSBlow (TS 38.211 [3], clause 7.4.3.1):

FSSBlow = FSSref - BWSSB / 2

FoffsetToPointA = CRBsize * Floor((FSSBlow - FcarrierLow) / CRBsize) + FcarrierLow

1e.	Calculate the maximum OffsetRBs value with FCORESET0Low >= FcarrierLow:

Max_OffsetRBs = (FoffsetToPointA - FcarrierLow) / CRBsize

1f.	Select the largest valid OffsetRBs value equal or smaller than the calculated max value, Max_OffsetRBs in step 1e within the applicable values for the carrier in TS 38.213 [4], table 13-1 to 13-10 limited to the table indexes with number of RBs and number of symbols  equal to the minimum value of in the table and minimum value of  for the selected .If a valid OffsetRBs value is found, then continue from step 1g..

If a valid OffsetRBs value is found, then continue from step 1g.

If no valid OffsetRBs value is found, then select the next valid GSCN with FSSref <= FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1f.

If no valid OffsetRBs value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.2 and the procedure is completed.

1g.	Calculate kSSB

kSSB = (FSSBlow - FoffsetToPointA) / {15 kHz for FR1, subCarrierSpacingCommon (MIB) for FR2} (TS 38.211 [3], clause 7.4.3.1).

N = SCSSSB / {15 kHz for FR1; subCarrierSpacingCommon (MIB) for FR2}. kSSB MOD N <> 0 indicates that the SSB subcarriers are not aligned with the resource grid given by the SCS indicated by subCarrierSpacingCommon in the MIB.

If kSSB is an integer and kSSB MOD N = 0, then continue from step 2.

If kSSB is not an integer value or kSSB MOD N <> 0, then select the next valid GSCN with FSSref < FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1g. 

If N > 1 and no valid kSSB value found within the valid GSCN range for the currently selected carrier frequency Fcarrier then shift Fcarrier up by ΔFRaster for Low range; or down by ΔFRaster for Mid, Mid-Low, Mid-High and High ranges and repeat steps 1a to 1g for a maximum shift of 3 * ΔFRaster (see clause C.3.1, Note 1).

If no valid kSSB value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.2 and the procedure is completed.

2.	Determine OffsetToCarrier

Select offsetToCarrier value for the carrier in accordance to Table C.3.2-1.

Table C.3.2-1: Downlink and uplink offsetToCarrier default values for different frequency ranges

| Frequency range | Downlink offsetToCarrier | Uplink offsetToCarrier |
| --- | --- | --- |
| Low range | 0 | 0 |
| Mid range | 102 | 504 |
| High range | 504 | 6 |
| Mid-Low range | 12 | 36 |
| Mid-High range | 24 | 114 |
| Note: Different values of offsetToCarrier have been selected for Low, Mid-Low, Mid, Mid-High and High ranges to achieve enhanced test coverage of the offsetToCarrier range of values. |  |  |

2b.	Determine FpointA:

FPointA = FcarrierLow - offsetToCarrier * PRBsize

3.	Determination of CORESET#1 start position in BWP:

ΔFOffsetCORESET-0-Carrier = FoffsetToPointA - OffsetRB * CRBsize - FcarrierLow

ΔFOffsetCORESET-1-Carrier = 6 * CRBsize * ceiling(ΔFOffsetCORESET-0-Carrier / (6 * CRBsize))

CORESET#1 Start CRB = ΔFOffsetCORESET-1-Carrier / CRBsize

4.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. For the case kSSB > 15 the extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [22]. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | Set to the index associated with the selected OffsetRBs value in the applicable table, 13-1 to 13-10, in TS 38.213 [22]. The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToPointA | (FOffsetToPointA - FPointA) / (12*{15 kHz for FR1; 60 kHz for FR2}). The IE field offsetToPointA is signalled in IE FrequencyInfoDL-SIB. |
| offsetToCarrier | Set to value calculated in step 2a. The IE field offsetToCarrier is signalled in IE SCS-SpecificCarrier. |

## C.3.3	Determination of SSB and signalling parameters for a carrier without CORESET#0

The following procedure is used for calculation of SSB and signalling parameters for a carrier without a CORESET#0.

1. 	Calculate FSSref, kSSB and FPointA with the SSB lowest subcarrier at the carrier’s lowest subcarrier (ΔFOffsetSSB-Carrier in Figure C.1-1 = 0):

FSSref = FcarrierLow + BWSSB / 2

kSSB = {31 for FR1; 15 for FR2} indicating that no CORESET#0 is present for the carrier (TS 38.213 [4], clause 13).

offsetToCarrier = target value for offsetToCarrier dependent on frequency range as specified in Table C.3.1-1.

FPointA = FcarrierLow - offsetToCarrier * PRBsize

2.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. For the case kSSB > 15 the extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [22]. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | Set to 0 indicating that no CORESET#0 exist (TS 38.213 [22], clause 13). The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| searchSpaceZero | Set to 0 indicating that no CORESET#0 exist (TS 38.213 [22], clause 13). The IE field searchSpaceZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToCarrier | Set to offsetToCarrier target value selected in step 1. |

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-12 | RAN5#77 | R5-176995 | - | - | - | TP on clauses of test equipment requirement in 38.508-1 | 0.1.0 |
| 2017-12 | RAN5#77 | R5-176779 | - | - | - | Add references | 0.1.0 |
| 2017-12 | RAN5#77 | R5-176917 | - | - | - | Introduce general chapter for generic procedures | 0.1.0 |
| 2017-12 | RAN5#77 | R5-176918 | - | - | - | Add generic procedures RRC_IDLE and RRC_CONNECTED | 0.1.0 |
| 2017-12 | RAN5#77 | R5-176920 | - | - | - | Introduce RRC chapters | 0.1.0 |
| 2018-01 | RAN5#1-5G-NR Adhoc | R5-180066 | - | - | - | Definition of downlink physical layer parameters for NR | 0.2.0 |
| 2018-03 | RAN5#78 | R5-181697 | - | - | - | Addition of the environmental information into TS 38.508-1 | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180265 | - | - | - | Introduce chapter for reference configurations | 0.3.0 |
| 2018-03 | RAN5#78 | R5-181311 | - | - | - | Update the general chapter | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180382 | - | - | - | Update RRCReconfiguration | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180383 | - | - | - | Add draft RRC messages | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180577 | - | - | - | Update chapter for test frequencies | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180709 | - | - | - | Add CellGroupConfig | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180773 | - | - | - | Add radioBearerConfig | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180775 | - | - | - | Add draft Radio resource control information elements | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180966 | - | - | - | Update RRC Connected state | 0.3.0 |
| 2018-03 | RAN5#78 | R5-181035 | - | - | - | Update RRC IDLE state | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180253 | - | - | - | Revised WID on: UE Conformance Test Aspects - 5G system with NR and LTE | 0.3.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181812 | - | - | - | Update Radio resource control information elements | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182109 | - | - | - | Update CellGroupConfig | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182064 | - | - | - | Update radioBearerConfig | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182062 | - | - | - | Update MIB | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182063 | - | - | - | Introduce radio conditions | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181786 | - | - | - | Update RRCReconfiguration | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181971 | - | - | - | Add Other information elements | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182065 | - | - | - | Update chapter 4.5.1 General | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181813 | - | - | - | Update RRC IDLE state | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182066 | - | - | - | Update RRC CONNECTED state | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182110 | - | - | - | Text proposal to add clause 4.4 reference system configurations to TS 38.508-1 | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182067 | - | - | - | TP for definition of physical channel allocations in 38.508-1 | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182091 | - | - | - | TP for clauses of signal level | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181972 | - | - | - | TP for updating of Downlink physical layer parameters | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181893 | - | - | - | Addition of UE capability information elements | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181973 | - | - | - | TP for adding Mid channel BW definition in TS 38.508-1 | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181974 | - | - | - | Addition of SRB3 | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182068 | - | - | - | Update MeasConfig information elements | 0.4.0 |
| 2018-05 | RAN5#79 | R5-183082 | - | - | - | Update radio resource control information elements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182288 | - | - | - | TP for updating of downlink physical layer parameters in 38.508-1 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182349 | - | - | - | Corrections to clause 4.4 reference system configurations | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182792 | - | - | - | TP for clauses of Supported Channels for a NR cell | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183218 | - | - | - | pCR update chapter for test frequencies - EN-DC | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183234 | - | - | - | TP for updating of physical channel allocation part in 38.508-1 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183256 | - | - | - | pCR update chapter for test frequencies - FR1 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183916 | - | - | - | TP for Annex A in TS 38.508-1 and adding a set of Connection Diagrams | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183920 | - | - | - | Introduction of Environmental conditions for FR1 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182249 | - | - | - | Add reference to NR cell table | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183210 | - | - | - | Update PDCCH | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182312 | - | - | - | Update chapter 4.5.1 General | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182313 | - | - | - | Update RRC CONNECTED state | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183087 | - | - | - | Addition of new RRCReconfiguration definition for AM/UM bearers | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183088 | - | - | - | Updates to UE capability information elements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183250 | - | - | - | Updates to UE capability information elements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183083 | - | - | - | Update RACH | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183084 | - | - | - | Update ARFCN | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183211 | - | - | - | Update BWP-UplinkDedicated | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183212 | - | - | - | Update serving cell | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183214 | - | - | - | Update RadioBearerConfig | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183215 | - | - | - | Update RRCReconfiguration | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182381 | - | - | - | Update MIB | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183090 | - | - | - | Update RRCReconfiguration for measurements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183264 | - | - | - | Corrections to clause 4.5 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183249 | - | - | - | Correction to the Table CellGroupConfig | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183255 | - | - | - | Update of FR1 signal levels | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183216 | - | - | - | Update CellGroupConfig and some related information elements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183086 | - | - | - | Update CSI-MeasConfig | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183260 | - | - | - | Update some information elements related to MeasConfig | 1.0.0 |
| 2018-06 | RAN#80 | RP-181207 | - | - | - | put under revision control as v15.0.0 with small editorial changes | 15.0.0 |
| 2018-09 | RAN#81 | R5-184087 | 0004 | - | F | Update chapter 3 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184297 | 0012 | - | F | Addition of Mid channel bandwidth definition for several missing bands | 15.1.0 |
| 2018-09 | RAN#81 | R5-184327 | 0014 | - | F | Adding condition for CP-OFDM waveform | 15.1.0 |
| 2018-09 | RAN#81 | R5-184347 | 0019 | - | F | Modified RRC_IDLE procedure to allow multi PDN configuration throughout the test case | 15.1.0 |
| 2018-09 | RAN#81 | R5-184471 | 0044 | - | F | Introduction of test frequencies for NR band n77 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184472 | 0045 | - | F | Introduction of test frequencies for NR band n78 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184473 | 0046 | - | F | Introduction of test frequencies for NR band n79 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184474 | 0047 | - | F | Introduction of test frequencies for NR band n257 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184475 | 0048 | - | F | Introduction of test frequencies for NR band n258 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184476 | 0049 | - | F | Introduction of test frequencies for NR band n260 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184477 | 0050 | - | F | Introduction of test frequencies for NR band n261 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184599 | 0056 | - | F | Add IE SS-RSSI-Measurement | 15.1.0 |
| 2018-09 | RAN#81 | R5-184617 | 0059 | - | F | Update MIB | 15.1.0 |
| 2018-09 | RAN#81 | R5-184630 | 0072 | - | F | Editorial Update in clause 4.6.3 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184783 | 0079 | - | F | Introduce 5GMM messages | 15.1.0 |
| 2018-09 | RAN#81 | R5-184785 | 0080 | - | F | Introduce 5GSM messages | 15.1.0 |
| 2018-09 | RAN#81 | R5-184806 | 0081 | - | F | Mid test CH BW for n71 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185028 | 0002 | 1 | F | Add SRB1 and SRB2 with NR PDCP | 15.1.0 |
| 2018-09 | RAN#81 | R5-185029 | 0003 | 1 | F | Update serving cell | 15.1.0 |
| 2018-09 | RAN#81 | R5-185030 | 0005 | 1 | F | Introduce SA RRC messages | 15.1.0 |
| 2018-09 | RAN#81 | R5-185031 | 0006 | 1 | F | Correct IE FrequencyInfoDL | 15.1.0 |
| 2018-09 | RAN#81 | R5-185032 | 0007 | 1 | F | Introduce SA system information blocks | 15.1.0 |
| 2018-09 | RAN#81 | R5-185033 | 0008 | 1 | F | Introduce SA other information elements | 15.1.0 |
| 2018-09 | RAN#81 | R5-185035 | 0013 | 1 | F | Correct IE GSCN-ValueNR | 15.1.0 |
| 2018-09 | RAN#81 | R5-185036 | 0017 | 1 | F | Update of FR1 signal levels | 15.1.0 |
| 2018-09 | RAN#81 | R5-185037 | 0022 | 1 | F | Addition of IP Connectivity check procedure | 15.1.0 |
| 2018-09 | RAN#81 | R5-185038 | 0053 | 1 | F | Introduce SA radio resource control information elements | 15.1.0 |
| 2018-09 | RAN#81 | R5-185039 | 0054 | 1 | F | Update IE PhysicalCellGroupConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185040 | 0055 | 1 | F | Introduce cell configurations and timer tolerances chapter headers | 15.1.0 |
| 2018-09 | RAN#81 | R5-185041 | 0057 | 1 | F | Add IE SSB-MTC | 15.1.0 |
| 2018-09 | RAN#81 | R5-185042 | 0058 | 1 | F | Update BWP | 15.1.0 |
| 2018-09 | RAN#81 | R5-185043 | 0060 | 1 | F | Update PDSCH-Config | 15.1.0 |
| 2018-09 | RAN#81 | R5-185044 | 0062 | 1 | F | Update PUCCH and PUSCH configuration | 15.1.0 |
| 2018-09 | RAN#81 | R5-185045 | 0063 | 1 | F | Update RACH configuration | 15.1.0 |
| 2018-09 | RAN#81 | R5-185046 | 0065 | 1 | F | Update CellGroupConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185047 | 0066 | 1 | F | Update CSI-MeasConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185048 | 0067 | 1 | F | Update MeasConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185049 | 0068 | 1 | F | Update other information elements | 15.1.0 |
| 2018-09 | RAN#81 | R5-185050 | 0070 | 1 | F | Update RadioBearerConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185051 | 0073 | 1 | F | Specifying content for MeasResultSCG-Failure | 15.1.0 |
| 2018-09 | RAN#81 | R5-185052 | 0075 | 1 | F | Editorial correction to band representation of non-contiguous EN-DC band combination | 15.1.0 |
| 2018-09 | RAN#81 | R5-185053 | 0076 | 1 | F | Correction to RLC-Config IE | 15.1.0 |
| 2018-09 | RAN#81 | R5-185054 | 0077 | 1 | F | Correction to RadioBearerConfig-DRB | 15.1.0 |
| 2018-09 | RAN#81 | R5-185055 | 0078 | 1 | F | Corrections and updates to BandCombinationList and Feature Set IEs | 15.1.0 |
| 2018-09 | RAN#81 | R5-185056 | 0084 | 1 | F | Corrections and updates to UE Capability IEs | 15.1.0 |
| 2018-09 | RAN#81 | R5-185085 | 0087 | - | F | Addition of UM condition to RLC-Bearer-Config IE | 15.1.0 |
| 2018-09 | RAN#81 | R5-185133 | 0086 | 1 | F | Correction of clause 4.3.3.2.3 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185163 | 0018 | 1 | F | Modified RRC_Connected procedure for Multi PDN throughout the test case. | 15.1.0 |
| 2018-09 | RAN#81 | R5-185165 | 0020 | 1 | F | Update EN-DC Generic Procedure Parameter for Multi-PDN addition throughout Test Case | 15.1.0 |
| 2018-09 | RAN#81 | R5-185168 | 0082 | 1 | F | Introduction of OTA signalling test environment | 15.1.0 |
| 2018-09 | RAN#81 | R5-185171 | 0009 | 2 | F | Updates to PDCCH and SearchSpace configurations | 15.1.0 |
| 2018-09 | RAN#81 | R5-185173 | 0016 | 1 | F | Test Frequencies | 15.1.0 |
| 2018-09 | RAN#81 | R5-185177 | 0051 | 1 | F | Introduction of test frequencies for signalling testing in clause 6 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185250 | 0023 | 1 | F | Introduction of test frequencies for NR band n1 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185251 | 0024 | 1 | F | Introduction of test frequencies for NR band n2 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185252 | 0025 | 1 | F | Introduction of test frequencies for NR band n3 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185253 | 0026 | 1 | F | Introduction of test frequencies for NR band n5 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185254 | 0027 | 1 | F | Introduction of test frequencies for NR band n7 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185255 | 0028 | 1 | F | Introduction of test frequencies for NR band n8 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185256 | 0029 | 1 | F | Introduction of test frequencies for NR band n12 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185257 | 0030 | 1 | F | Introduction of test frequencies for NR band n20 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185258 | 0031 | 1 | F | Introduction of test frequencies for NR band n25 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185259 | 0032 | 1 | F | Introduction of test frequencies for NR band n28 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185260 | 0033 | 1 | F | Introduction of test frequencies for NR band n34 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185261 | 0034 | 1 | F | Introduction of test frequencies for NR band n38 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185262 | 0035 | 1 | F | Introduction of test frequencies for NR band n39 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185263 | 0036 | 1 | F | Introduction of test frequencies for NR band n40 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185264 | 0037 | 1 | F | Update of test frequencies for NR band n41 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185265 | 0038 | 1 | F | Introduction of test frequencies for NR band n51 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185266 | 0039 | 1 | F | Introduction of test frequencies for NR band n66 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185267 | 0040 | 1 | F | Introduction of test frequencies for NR band n70 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185268 | 0041 | 1 | F | Update of test frequencies for NR band n71 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185269 | 0042 | 1 | F | Introduction of test frequencies for NR band n75 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185270 | 0043 | 1 | F | Introduction of test frequencies for NR band n76 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185443 | 0052 | 1 | F | Correction to power level for FR1 RF tests | 15.1.0 |
| 2018-09 | RAN#81 | R5-185557 | 0085 | 1 | F | FR2_UE_BeamlockProcedure_38.508-1 | 15.1.0 |
| 2018-12 | RAN#82 | R5-186453 | 0239 | - | F | Updates to clause 4.3.3, physical channel allocations | 15.2.0 |
| 2018-12 | RAN#82 | R5-186457 | 0240 | - | F | Correction to E-UTRA test frequency for intra-band contiguous configuration for band 41 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186468 | 0241 | - | F | E-UTRA test frequencies for EN-DC intra-band contiguous configurations for band 71 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186491 | 0245 | - | F | Update chapter 4.5 for RF connected procedure | 15.2.0 |
| 2018-12 | RAN#82 | R5-186508 | 0249 | - | F | FR2 UE and TE radiated connection diagram | 15.2.0 |
| 2018-12 | RAN#82 | R5-186575 | 0251 | - | F | Update IE ServingCellConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186612 | 0252 | - | F | Add CounterCheck | 15.2.0 |
| 2018-12 | RAN#82 | R5-186613 | 0253 | - | F | Update DLInformationTransfer | 15.2.0 |
| 2018-12 | RAN#82 | R5-186641 | 0255 | - | F | Update IE SchedulingRequestResourceConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186665 | 0258 | - | F | Update LocationMeasurementIndication | 15.2.0 |
| 2018-12 | RAN#82 | R5-186666 | 0259 | - | F | Update MeasurementReport | 15.2.0 |
| 2018-12 | RAN#82 | R5-186677 | 0261 | - | F | Resubmission of update to 38.508 for mid channel bandwidth | 15.2.0 |
| 2018-12 | RAN#82 | R5-186682 | 0262 | - | F | Update MobilityFromNRCommand | 15.2.0 |
| 2018-12 | RAN#82 | R5-186691 | 0264 | - | F | Update Paging | 15.2.0 |
| 2018-12 | RAN#82 | R5-186692 | 0265 | - | F | Update RRCReestablishment | 15.2.0 |
| 2018-12 | RAN#82 | R5-186714 | 0267 | - | F | Update RRCReject | 15.2.0 |
| 2018-12 | RAN#82 | R5-186719 | 0268 | - | F | Updates related to introduction of test frequencies | 15.2.0 |
| 2018-12 | RAN#82 | R5-186722 | 0271 | - | F | Update SecurityAlgorithmConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186723 | 0272 | - | F | Updates to MeasResults | 15.2.0 |
| 2018-12 | RAN#82 | R5-186734 | 0273 | - | F | Update RRCRelease | 15.2.0 |
| 2018-12 | RAN#82 | R5-186744 | 0274 | - | F | Update RRCResume | 15.2.0 |
| 2018-12 | RAN#82 | R5-186825 | 0279 | - | F | Correction of test frequencies for NR band n1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186826 | 0280 | - | F | Correction of test frequencies for NR band n2 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186827 | 0281 | - | F | Correction of test frequencies for NR band n3 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186828 | 0282 | - | F | Correction of test frequencies for NR band n5 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186829 | 0283 | - | F | Correction of test frequencies for NR band n7 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186830 | 0284 | - | F | Correction of test frequencies for NR band n8 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186831 | 0285 | - | F | Correction of test frequencies for NR band n12 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186832 | 0286 | - | F | Correction of test frequencies for NR band n20 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186833 | 0287 | - | F | Correction of test frequencies for NR band n25 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186834 | 0288 | - | F | Correction of test frequencies for NR band n28 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186835 | 0289 | - | F | Correction of test frequencies for NR band n34 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186836 | 0290 | - | F | Correction of test frequencies for NR band n38 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186837 | 0291 | - | F | Correction of test frequencies for NR band n39 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186838 | 0292 | - | F | Correction of test frequencies for NR band n40 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186839 | 0293 | - | F | Correction of test frequencies for NR band n41 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186840 | 0294 | - | F | Correction of test frequencies for NR band n51 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186841 | 0295 | - | F | Introduction of test frequencies for NR band n66 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186842 | 0296 | - | F | Introduction of test frequencies for NR band n70 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186844 | 0298 | - | F | Correction of test frequencies for NR band n75 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186845 | 0299 | - | F | Correction of test frequencies for NR band n76 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186846 | 0300 | - | F | Correction of test frequencies for NR band n77 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186847 | 0301 | - | F | Correction of test frequencies for NR band n78 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186848 | 0302 | - | F | Correction of test frequencies for NR band n79 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186850 | 0304 | - | F | Correction of test frequencies for NR band n258 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186851 | 0305 | - | F | Correction of test frequencies for NR band n260 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186852 | 0306 | - | F | Correction of test frequencies for NR band n261 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186855 | 0309 | - | F | Introduction of preamble test states | 15.2.0 |
| 2018-12 | RAN#82 | R5-186857 | 0311 | - | F | Introduction DCI format 1_0 for paging, SI and random access | 15.2.0 |
| 2018-12 | RAN#82 | R5-186858 | 0312 | - | F | Correction to DCI format 1_1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186859 | 0313 | - | F | Update IE RateMatchPattern | 15.2.0 |
| 2018-12 | RAN#82 | R5-186861 | 0315 | - | F | Correction of generic procedure parameter naming for test loop function | 15.2.0 |
| 2018-12 | RAN#82 | R5-186862 | 0316 | - | F | Correction of test procedures to activate and deactivate UE Beamlock Function | 15.2.0 |
| 2018-12 | RAN#82 | R5-186893 | 0318 | - | F | Corrections to the notes in the OTA signal level tables | 15.2.0 |
| 2018-12 | RAN#82 | R5-186911 | 0320 | - | F | Add RRCSetupComplete | 15.2.0 |
| 2018-12 | RAN#82 | R5-186912 | 0321 | - | F | Add RRCSetupRequest | 15.2.0 |
| 2018-12 | RAN#82 | R5-186913 | 0322 | - | F | Add RRCSystemInfoRequest | 15.2.0 |
| 2018-12 | RAN#82 | R5-186916 | 0323 | - | F | Add SecurityModeCommand | 15.2.0 |
| 2018-12 | RAN#82 | R5-186918 | 0324 | - | F | Update SystemInformation | 15.2.0 |
| 2018-12 | RAN#82 | R5-186920 | 0325 | - | F | Add UEAssistanceInformation | 15.2.0 |
| 2018-12 | RAN#82 | R5-186921 | 0326 | - | F | Update UECapabilityEnquiry | 15.2.0 |
| 2018-12 | RAN#82 | R5-186922 | 0327 | - | F | Update ULInformationTransfer | 15.2.0 |
| 2018-12 | RAN#82 | R5-186923 | 0328 | - | F | Update IE PTRS-UplinkConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186925 | 0330 | - | F | Update RRCResumeRequest | 15.2.0 |
| 2018-12 | RAN#82 | R5-186929 | 0331 | - | F | Update PTRS-DownlinkConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186936 | 0335 | - | F | Update PUCCH-SpatialRelationInfo | 15.2.0 |
| 2018-12 | RAN#82 | R5-186987 | 0342 | - | F | Addition of SIB3 message_Resubmission of 185792 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186988 | 0343 | - | F | Addition of SIB5 message_Resubmission of 186054 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186989 | 0344 | - | F | Addition of SIB6 - SIB8 message_Resubmission of 186055 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186990 | 0345 | - | F | Addition of SIB9 message_Resubmission of 186056 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187026 | 0348 | - | F | Addition of P-Max in Test environment for RF test | 15.2.0 |
| 2018-12 | RAN#82 | R5-187028 | 0350 | - | F | Addition of test frequencies for SUL band n80 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187030 | 0352 | - | F | Addition of test frequencies for SUL band n82 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187031 | 0353 | - | F | Addition of test frequencies for SUL band n83 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187032 | 0354 | - | F | Addition of test frequencies for SUL band n84 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187033 | 0355 | - | F | Addition of test frequencies for SUL band n86 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187110 | 0358 | - | F | Correction to default message contents for SRB3 configuration | 15.2.0 |
| 2018-12 | RAN#82 | R5-187159 | 0361 | - | F | Updates to Configuration Update 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187160 | 0362 | - | F | Updates to De-registration 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187161 | 0363 | - | F | Updates to Identity 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187162 | 0364 | - | F | Updates to NAS Transport 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187163 | 0365 | - | F | Updates to Notification 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187164 | 0366 | - | F | Updates to PDU session authentication 5GSM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187166 | 0368 | - | F | Updates to PDU session modification 5GSM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187172 | 0374 | - | F | Removal of Editor’s Notes in section 4.6.3 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187175 | 0377 | - | F | Addition and updates to Information Elements in section 4.6.5 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187270 | 0381 | - | F | Updating 4.2.1 General functional requirements | 15.2.0 |
| 2018-12 | RAN#82 | R5-187271 | 0382 | - | F | Update the section for test equipment requirements for TRx | 15.2.0 |
| 2018-12 | RAN#82 | R5-187272 | 0383 | - | F | FR2 downlink signal level(38.508-1) | 15.2.0 |
| 2018-12 | RAN#82 | R5-187413 | 0389 | - | F | Uplink RNTI to valid value in TS 38.508-1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187415 | 0390 | - | F | Update maxPayloadMinus1 in PUCCH config in TS 38.508-1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187420 | 0393 | - | F | Addition of connection diagram for 2 TX UL MIMO | 15.2.0 |
| 2018-12 | RAN#82 | R5-187557 | 0396 | - | F | Addition of low and high test channel bandwidth in 38.508 | 15.2.0 |
| 2018-12 | RAN#82 | R5-188205 | 0397 | 1 | F | Updates to Annex B to add Permitted OTA Test Methods | 15.2.0 |
| 2018-12 | RAN#82 | R5-187610 | 0398 | - | F | Corrections to IEs part of PDSCH-ServingCellConfig, ServingCellConfig and ServingCellConfigCommon | 15.2.0 |
| 2018-12 | RAN#82 | R5-187659 | 0243 | 1 | F | Wordings for Uplink NAS messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187660 | 0247 | 1 | F | Default cell configurations for NAS | 15.2.0 |
| 2018-12 | RAN#82 | R5-187661 | 0248 | 1 | F | Update IE SI-SchedulingInfo | 15.2.0 |
| 2018-12 | RAN#82 | R5-187662 | 0349 | 1 | F | Addition of Combinations of system information blocks in 4.4.3.1.2 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187664 | 0263 | 1 | F | Correction to various Radio resource control IEs | 15.2.0 |
| 2018-12 | RAN#82 | R5-187665 | 0308 | 1 | F | Correction to DCI formats 0_0 and 0_1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187666 | 0310 | 1 | F | Introduction of SDL and SUL cells in simulated cells in clause 4.4.2 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187667 | 0314 | 1 | F | Correction to RRC_IDLE procedure | 15.2.0 |
| 2018-12 | RAN#82 | R5-187668 | 0332 | 1 | F | Update CSI related information elements | 15.2.0 |
| 2018-12 | RAN#82 | R5-187669 | 0333 | 1 | F | Update ServingCellConfigCommon and TDD-UL-DL-Config | 15.2.0 |
| 2018-12 | RAN#82 | R5-187670 | 0334 | 1 | F | Update SRS-Config | 15.2.0 |
| 2018-12 | RAN#82 | R5-187671 | 0336 | 1 | F | Update some information elements for measurements | 15.2.0 |
| 2018-12 | RAN#82 | R5-187672 | 0337 | 1 | F | Update CellGroupConfig and related information elements | 15.2.0 |
| 2018-12 | RAN#82 | R5-187673 | 0338 | 1 | F | CR of NR 508-1 clause 4.6.2_SIB2, SIB4 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187674 | 0339 | 1 | F | CR of NR 508-1 Table 4.4.2-2_Default NR Cells parameters | 15.2.0 |
| 2018-12 | RAN#82 | R5-187675 | 0341 | 1 | F | Update RLC-Config | 15.2.0 |
| 2018-12 | RAN#82 | R5-187676 | 0357 | 1 | F | Specifying Test procedure to check that UE is camped on a new NR cell belonging to a new TA | 15.2.0 |
| 2018-12 | RAN#82 | R5-187677 | 0360 | 1 | F | Updates to Authentication 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187678 | 0369 | 1 | F | Updates to PDU session release 5GSM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187679 | 0371 | 1 | F | Updates to Security mode 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187680 | 0375 | 1 | F | Addition of new Information Elements in section 4.6.3 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187681 | 0379 | 1 | F | Updates to SIG OTA Calibration for FR2 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187682 | 0394 | 1 | F | Addition of default QoS configurations | 15.2.0 |
| 2018-12 | RAN#82 | R5-187720 | 0319 | 2 | F | Uplink PTRS disable for RF testing | 15.2.0 |
| 2018-12 | RAN#82 | R5-188238 | 0242 | 2 | F | Addition to E-UTRA test frequencies for intra-band contiguous configuration for band 41 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187723 | 0303 | 1 | F | Correction of test frequencies for NR band n257 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187724 | 0269 | 1 | F | New annex for NR test frequency calculations | 15.2.0 |
| 2018-12 | RAN#82 | R5-187725 | 0297 | 1 | F | Correction of test frequencies for NR band n71 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187745 | 0238 | 1 | F | Update SIB1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187747 | 0257 | 1 | F | Correction to Signal levels for conducted testing | 15.2.0 |
| 2018-12 | RAN#82 | R5-187748 | 0270 | 1 | F | Updates to E-UTRA RRC_CONNECTED generic procedure | 15.2.0 |
| 2018-12 | RAN#82 | R5-187750 | 0275 | 1 | F | Add RRCResumeComplete | 15.2.0 |
| 2018-12 | RAN#82 | R5-187751 | 0278 | 1 | F | Update chapter 4.5.3 RRC_INACTIVE | 15.2.0 |
| 2018-12 | RAN#82 | R5-187752 | 0307 | 1 | F | Correction of test frequencies for signalling testing in clause 6 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187753 | 0317 | 1 | F | Specifying Test procedure to check that UE is in RRC_IDLE state on a certain NR cell | 15.2.0 |
| 2018-12 | RAN#82 | R5-187754 | 0329 | 1 | F | Update IE RLF-TimersAndConstants | 15.2.0 |
| 2018-12 | RAN#82 | R5-187755 | 0346 | 1 | F | Add RRCSetup | 15.2.0 |
| 2018-12 | RAN#82 | R5-187756 | 0347 | 1 | F | Update RRCReconfiguration | 15.2.0 |
| 2018-12 | RAN#82 | R5-187757 | 0356 | 1 | F | Update IE RadioBearerConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-187759 | 0370 | 1 | F | Updates to Registration 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187760 | 0372 | 1 | F | Updates to Security protected 5GS NAS and 5GMM status messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187761 | 0373 | 1 | F | Updates to Service Request 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187762 | 0376 | 1 | F | Addition and updates to Information Elements in section 4.6.4 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187763 | 0388 | 1 | F | Addition of 5GS related new EFs to Test UICC definition | 15.2.0 |
| 2018-12 | RAN#82 | R5-187764 | 0395 | 1 | F | Update IE CellGroupConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-187802 | 0384 | 1 | F | Updating power levels for LTE Anchor Link | 15.2.0 |
| 2018-12 | RAN#82 | R5-187887 | 0351 | 1 | F | Addition of test frequencies for SUL band n81 | 15.2.0 |
| 2018-12 | RAN#82 | R5-188031 | 0391 | 1 | F | Addition of 2TX_UL_MIMO condition | 15.2.0 |
| 2018-12 | RAN#82 | R5-188107 | 0367 | 2 | F | Updates to PDU session establishment 5GSM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-188122 | 0260 | 2 | F | Update chapter 4.5.2 RRC_IDLE | 15.2.0 |
| 2018-12 | RAN#82 | R5-188123 | 0250 | 1 | F | Update chapter 4.5.4 RRC_CONNECTED | 15.2.0 |
| 2019-03 | RAN#83 | R5-191047 | 0526 | - | F | Update IE PDCCH-ConfigCommon | 15.3.0 |
| 2019-03 | RAN#83 | R5-191048 | 0527 | - | F | Update IE RadioBearerConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-191094 | 0529 | - | F | Updates of test channel bandwidth in TS 38.508-1 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191129 | 0530 | - | F | Update IE SDAP-Config | 15.3.0 |
| 2019-03 | RAN#83 | R5-191145 | 0531 | - | F | Update IE CellGroupId | 15.3.0 |
| 2019-03 | RAN#83 | R5-191155 | 0532 | - | F | Correction to temperature and voltage of Common test environments | 15.3.0 |
| 2019-03 | RAN#83 | R5-191187 | 0534 | - | F | Updates for Other SI support | 15.3.0 |
| 2019-03 | RAN#83 | R5-191189 | 0536 | - | F | Correction to RadioBearerConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-191191 | 0538 | - | F | Correction to SystemInformation | 15.3.0 |
| 2019-03 | RAN#83 | R5-191192 | 0539 | - | F | Correction to PUCCH-Config | 15.3.0 |
| 2019-03 | RAN#83 | R5-191193 | 0540 | - | F | Correction to SIB3 and SIB4 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191194 | 0541 | - | F | Correction of PUSCH-TimeDomainResourceAllocationList | 15.3.0 |
| 2019-03 | RAN#83 | R5-191195 | 0542 | - | F | Corrections and clarifications regarding DCI formats 0_1 and 1_1 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191219 | 0545 | - | F | Updates to Authentication 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191220 | 0546 | - | F | Updates to Configuration Update 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191221 | 0547 | - | F | Updates to De-registration 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191222 | 0548 | - | F | Updates to NAS transport 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191223 | 0549 | - | F | Updates to PDU session establishment 5GSM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191224 | 0550 | - | F | Updates to PDU session modification 5GSM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191225 | 0551 | - | F | Updates to PDU session release 5GSM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191226 | 0552 | - | F | Updates to Registration 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191227 | 0553 | - | F | Updates to Security Mode 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191228 | 0554 | - | F | Updates to Security Protected 5GS NAS message | 15.3.0 |
| 2019-03 | RAN#83 | R5-191229 | 0555 | - | F | Updates to Service Request 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191233 | 0556 | - | F | Update IE BWP-Id | 15.3.0 |
| 2019-03 | RAN#83 | R5-191234 | 0557 | - | F | Add IE RejectWaitTime | 15.3.0 |
| 2019-03 | RAN#83 | R5-191235 | 0558 | - | F | Update IE ShortMAC-I | 15.3.0 |
| 2019-03 | RAN#83 | R5-191236 | 0559 | - | F | Update IE UE-TimersAndConstants | 15.3.0 |
| 2019-03 | RAN#83 | R5-191237 | 0560 | - | F | Update IE PUCCH-ConfigCommon | 15.3.0 |
| 2019-03 | RAN#83 | R5-191242 | 0561 | - | F | Addition of Positioning specifications | 15.3.0 |
| 2019-03 | RAN#83 | R5-191243 | 0562 | - | F | Update AS security Algorithm for RF testing | 15.3.0 |
| 2019-03 | RAN#83 | R5-191274 | 0563 | - | F | Update of structure of test frequency clauses | 15.3.0 |
| 2019-03 | RAN#83 | R5-191280 | 0564 | - | F | Correction to UL configuration | 15.3.0 |
| 2019-03 | RAN#83 | R5-191281 | 0565 | - | F | Correction to default value of IE’s in PDSCH-Config in Table 4.6.3-75 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191301 | 0568 | - | F | Correction of test frequencies for signalling testing in clause 6 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191302 | 0569 | - | F | Correction of test frequencies for EN-DC configuration DC_(n)41 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191304 | 0571 | - | F | Correction of test frequencies for NR band n1 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191305 | 0572 | - | F | Correction of test frequencies for NR band n2 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191306 | 0573 | - | F | Correction of test frequencies for NR band n3 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191307 | 0574 | - | F | Correction of test frequencies for NR band n5 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191308 | 0575 | - | F | Correction of test frequencies for NR band n7 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191309 | 0576 | - | F | Correction of test frequencies for NR band n8 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191310 | 0577 | - | F | Correction of test frequencies for NR band n12 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191311 | 0578 | - | F | Correction of test frequencies for NR band n20 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191312 | 0579 | - | F | Correction of test frequencies for NR band n25 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191313 | 0580 | - | F | Correction of test frequencies for NR band n28 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191314 | 0581 | - | F | Correction of test frequencies for NR band n34 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191315 | 0582 | - | F | Correction of test frequencies for NR band n38 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191316 | 0583 | - | F | Correction of test frequencies for NR band n39 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191317 | 0584 | - | F | Correction of test frequencies for NR band n40 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191318 | 0585 | - | F | Correction of test frequencies for NR band n41 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191319 | 0586 | - | F | Introduction of test frequencies for NR band n50 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191320 | 0587 | - | F | Correction of test frequencies for NR band n51 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191321 | 0588 | - | F | Correction of test frequencies for NR band n66 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191322 | 0589 | - | F | Correction of test frequencies for NR band n70 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191323 | 0590 | - | F | Correction of test frequencies for NR band n71 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191324 | 0591 | - | F | Introduction of test frequencies for NR band n74 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191325 | 0592 | - | F | Correction of test frequencies for NR band n75 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191326 | 0593 | - | F | Correction of test frequencies for NR band n76 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191327 | 0594 | - | F | Correction of test frequencies for NR band n77 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191328 | 0595 | - | F | Correction of test frequencies for NR band n78 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191329 | 0596 | - | F | Correction of test frequencies for NR band n79 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191330 | 0597 | - | F | Correction of test frequencies for NR band n257 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191331 | 0598 | - | F | Correction of test frequencies for NR band n258 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191332 | 0599 | - | F | Correction of test frequencies for NR band n260 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191333 | 0600 | - | F | Correction of test frequencies for NR band n261 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191334 | 0601 | - | F | Correction of DCI format 1_0 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191352 | 0603 | - | F | Update CounterCheckResponse | 15.3.0 |
| 2019-03 | RAN#83 | R5-191354 | 0604 | - | F | Add FailureInformation | 15.3.0 |
| 2019-03 | RAN#83 | R5-191355 | 0605 | - | F | Update LocationMeasurementIndication | 15.3.0 |
| 2019-03 | RAN#83 | R5-191356 | 0606 | - | F | Updates to section 4.8.3 (test USIM parameters) | 15.3.0 |
| 2019-03 | RAN#83 | R5-191360 | 0607 | - | F | Update MeasurementReport | 15.3.0 |
| 2019-03 | RAN#83 | R5-191361 | 0608 | - | F | Update MobilityFromNRCommand | 15.3.0 |
| 2019-03 | RAN#83 | R5-191364 | 0609 | - | F | Update Paging | 15.3.0 |
| 2019-03 | RAN#83 | R5-191366 | 0610 | - | F | Update RRCSetupComplete | 15.3.0 |
| 2019-03 | RAN#83 | R5-191368 | 0611 | - | F | Update SecurityModeComplete | 15.3.0 |
| 2019-03 | RAN#83 | R5-191370 | 0612 | - | F | Update SecurityModeFailure | 15.3.0 |
| 2019-03 | RAN#83 | R5-191371 | 0613 | - | F | Update UEAssistanceInformation | 15.3.0 |
| 2019-03 | RAN#83 | R5-191372 | 0614 | - | F | Update UECapabilityInformation | 15.3.0 |
| 2019-03 | RAN#83 | R5-191384 | 0616 | - | F | Correction to SecurityConfig of RadioBearerConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-191385 | 0617 | - | F | Correction to SIB9 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191386 | 0618 | - | F | Correction to SRS-Config of BWP-UplinkDedicated | 15.3.0 |
| 2019-03 | RAN#83 | R5-191446 | 0620 | - | F | Correction of default configuration of RRC IEs in 38.508-1 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191450 | 0621 | - | F | Addition of NR system information combination SIB6, SIB7 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191538 | 0624 | - | F | Update ULInformationTransfer | 15.3.0 |
| 2019-03 | RAN#83 | R5-191539 | 0625 | - | F | Update IE QuantityConfig and CSI-ReportConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-191620 | 0629 | - | F | Clarification for NR inter-band measurement test case configuration | 15.3.0 |
| 2019-03 | RAN#83 | R5-191762 | 0637 | - | F | Editorial update in MeasObjectNR and ReportConfigNR | 15.3.0 |
| 2019-03 | RAN#83 | R5-191763 | 0638 | - | F | Update ReportConfigNR and TimeToTrigger | 15.3.0 |
| 2019-03 | RAN#83 | R5-192271 | 0570 | 1 | F | Correction of test frequencies for EN-DC configuration DC_(n)71 | 15.3.0 |
| 2019-03 | RAN#83 | R5-192272 | 0602 | 1 | F | Update chapter 4.5 RRC Connected initiation | 15.3.0 |
| 2019-03 | RAN#83 | R5-192273 | 0626 | 1 | F | Update RRCRelease | 15.3.0 |
| 2019-03 | RAN#83 | R5-192274 | 0615 | 1 | F | Correction to NR SchedulingRequestResourceConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-192275 | 0627 | 1 | F | Update IE I-RNTI-Value | 15.3.0 |
| 2019-03 | RAN#83 | R5-192276 | 0628 | 1 | F | Update IE ShortI-RNTI-Value | 15.3.0 |
| 2019-03 | RAN#83 | R5-192277 | 0630 | 1 | F | Updates to test environments for Signalling test | 15.3.0 |
| 2019-03 | RAN#83 | R5-192278 | 0633 | 1 | F | Addition of USIM Profiles for Signaling TC | 15.3.0 |
| 2019-03 | RAN#83 | R5-192279 | 0636 | 1 | F | Update QoS Configuration | 15.3.0 |
| 2019-03 | RAN#83 | R5-192280 | 0643 | 1 | F | Update to of Generic procedure E-UTRA RRC_IDLE | 15.3.0 |
| 2019-03 | RAN#83 | R5-192281 | 0644 | 1 | F | Introduction of EAP AKA | 15.3.0 |
| 2019-03 | RAN#83 | R5-192290 | 0655 | - | F | Update chapter 4.5 RRC_INACTIVE | 15.3.0 |
| 2019-03 | RAN#83 | R5-192363 | 0631 | 1 | F | Updating P-Max IE | 15.3.0 |
| 2019-03 | RAN#83 | R5-192364 | 0632 | 2 | F | Updating IEs part of SearchSpace | 15.3.0 |
| 2019-03 | RAN#83 | R5-192400 | 0528 | 1 | F | Setup diagram for receiver test using spectrum analyzer | 15.3.0 |
| 2019-03 | RAN#83 | R5-192541 | 0622 | 1 | F | Connection diagrams for RRM tests | 15.3.0 |
| 2019-03 | RAN#83 | R5-192542 | 0646 | 1 | F | Antenna Connection diagram for UE part for RRM | 15.3.0 |
| 2019-03 | RAN#83 | R5-192543 | 0649 | 1 | F | Connection diagram for FR1 demod test cases | 15.3.0 |
| 2019-03 | RAN#83 | R5-192705 | 0645 | 1 | F | Introduction of Non 3GPP Access over WLAN | 15.3.0 |
| 2019-03 | RAN#83 | R5-192735 | 0533 | 1 | F | Correction to PUSCH-Config | 15.3.0 |
| 2019-03 | RAN#83 | R5-192736 | 0535 | 1 | F | Addition of details on Test State 0 | 15.3.0 |
| 2019-03 | RAN#83 | R5-192737 | 0537 | 1 | F | Correction of CellGroupConfig tables and logical channel identities | 15.3.0 |
| 2019-03 | RAN#83 | R5-192738 | 0543 | 1 | F | Additions and updates to UE capability Information Elements | 15.3.0 |
| 2019-03 | RAN#83 | R5-192739 | 0544 | 1 | F | Updates and additions of default QoS configurations | 15.3.0 |
| 2019-03 | RAN#83 | R5-192740 | 0566 | 1 | F | Update chapter 4.5 General for PDUs | 15.3.0 |
| 2019-03 | RAN#83 | R5-192741 | 0567 | 1 | F | Update of Annex C on calculation of test frequencies | 15.3.0 |
| 2019-03 | RAN#83 | R5-192742 | 0619 | 1 | F | Correction to schedulingRequestID Configuration | 15.3.0 |
| 2019-03 | RAN#83 | R5-192743 | 0639 | 1 | F | Addition of Switch/Power UE procedures | 15.3.0 |
| 2019-03 | RAN#83 | R5-192744 | 0640 | 1 | F | Update to Test procedure to check that UE is camped on a new cell belonging to a new TA | 15.3.0 |
| 2019-03 | RAN#83 | R5-192745 | 0641 | 1 | F | Update to Test procedure to check that UE is in state 5GC RRC_IDLE on a certain cell | 15.3.0 |
| 2019-03 | RAN#83 | R5-192846 | 0648 | 1 | F | Updates to Annex B to add Permitted OTA Test Methods | 15.3.0 |
| 2019-03 | RAN#83 | - | - | - | - | Editorial updates of table numbering | 15.3.0 |
| 2019-06 | RAN#84 | R5-193537 | 0680 | - | F | Remove unused DCI formats from 38.508-1 | 15.4.0 |
| 2019-06 | RAN#84 | R5-193540 | 0681 | - | F | Adding setup diagram for Receiver performance tests 2x2 | 15.4.0 |
| 2019-06 | RAN#84 | R5-193542 | 0682 | - | F | Remove brackets from parameters for DCI formats for scheduling | 15.4.0 |
| 2019-06 | RAN#84 | R5-193613 | 0691 | - | F | Update default configuration of QuantityConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-193681 | 0693 | - | F | Update chapter 4.5.3 RRC_INACTIVE procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-193682 | 0694 | - | F | Update chapter 4.5.4 RRC_CONNECTED procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-193683 | 0695 | - | F | Update chapter 4.5.5 SWITCHED_OFF procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-193690 | 0696 | - | F | Resubmission: Connection diagram for 1x2 Demod test cases | 15.4.0 |
| 2019-06 | RAN#84 | R5-193734 | 0701 | - | F | Update IE I-RNTI-Value | 15.4.0 |
| 2019-06 | RAN#84 | R5-193735 | 0702 | - | F | Update IE ShortI-RNTI-Value | 15.4.0 |
| 2019-06 | RAN#84 | R5-193746 | 0710 | - | F | Update IE SubcarrierSpacing | 15.4.0 |
| 2019-06 | RAN#84 | R5-193813 | 0711 | - | F | Update of USIM EF5GS3GPPLOCI & EF5GSN3GPPLOCI | 15.4.0 |
| 2019-06 | RAN#84 | R5-193828 | 0713 | - | F | Add IE MultiFrequencyBandListNR-SIB | 15.4.0 |
| 2019-06 | RAN#84 | R5-193829 | 0714 | - | F | Add IE NR-NS-PmaxList | 15.4.0 |
| 2019-06 | RAN#84 | R5-193843 | 0716 | - | F | Update IE ServingCellConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-193862 | 0717 | - | F | Corrections to References | 15.4.0 |
| 2019-06 | RAN#84 | R5-193980 | 0725 | - | F | New test procedure for Registration Reject | 15.4.0 |
| 2019-06 | RAN#84 | R5-193981 | 0726 | - | F | Updates to test procedure 4.9.1 | 15.4.0 |
| 2019-06 | RAN#84 | R5-194038 | 0728 | - | F | Editorial Correction - USIM Profiles for Signaling TC | 15.4.0 |
| 2019-06 | RAN#84 | R5-194040 | 0729 | - | F | Correction to QoS Configuration | 15.4.0 |
| 2019-06 | RAN#84 | R5-194086 | 0733 | - | F | Update K2 value to align with RF DL RMC | 15.4.0 |
| 2019-06 | RAN#84 | R5-194087 | 0734 | - | F | Update aggregationlevel2 in SearchSpace IE | 15.4.0 |
| 2019-06 | RAN#84 | R5-194303 | 0740 | - | F | TDD-UL-DL-Config for FR1 SCS 60kHz | 15.4.0 |
| 2019-06 | RAN#84 | R5-194359 | 0742 | - | F | Removal of column for Number of PDU sessions established from tables for Test States | 15.4.0 |
| 2019-06 | RAN#84 | R5-194362 | 0743 | - | F | Editorial correction to test frequency clauses | 15.4.0 |
| 2019-06 | RAN#84 | R5-194364 | 0744 | - | F | Update of test frequencies for EN-DC combination DC_41A_n41A | 15.4.0 |
| 2019-06 | RAN#84 | R5-194367 | 0745 | - | F | Common procedure to configure SCC for CA RF testing | 15.4.0 |
| 2019-06 | RAN#84 | R5-194369 | 0746 | - | F | Introduction of test frequencies for inter-band Rel-15 EN-DC two bands configurations | 15.4.0 |
| 2019-06 | RAN#84 | R5-194420 | 0751 | - | F | Update IE BWP-Downlink | 15.4.0 |
| 2019-06 | RAN#84 | R5-194435 | 0752 | - | F | Update IE BWP-Id | 15.4.0 |
| 2019-06 | RAN#84 | R5-194438 | 0755 | - | F | Updates to UE 4.6.5 Other Information Elements | 15.4.0 |
| 2019-06 | RAN#84 | R5-194441 | 0757 | - | F | Update IE BWP-Uplink | 15.4.0 |
| 2019-06 | RAN#84 | R5-194479 | 0758 | - | F | Editorial updates to 4.7.1 Contents of 5GMM messages | 15.4.0 |
| 2019-06 | RAN#84 | R5-194480 | 0759 | - | F | Editorial updates to 4.7.2 Contents of 5GSM messages | 15.4.0 |
| 2019-06 | RAN#84 | R5-194510 | 0762 | - | F | Update of Switch off - Power off procedure in RRC_CONNECTED | 15.4.0 |
| 2019-06 | RAN#84 | R5-194539 | 0767 | - | F | Introduction of test frequencies for EN-DC CA configuration DC_30A_n260(A-I) | 15.4.0 |
| 2019-06 | RAN#84 | R5-194541 | 0768 | - | F | Antenna Connection diagram for TE part for RRM | 15.4.0 |
| 2019-06 | RAN#84 | R5-194709 | 0785 | - | F | Update 38.508 RF and RRM clauses with agreed recommendation to configure UE as non-IMS | 15.4.0 |
| 2019-06 | RAN#84 | R5-194783 | 0774 | - | F | Introduction of test frequencies for NR band n50 and signalling testing | 15.4.0 |
| 2019-06 | RAN#84 | R5-194784 | 0775 | - | F | Introduction of test frequencies for NR band n74 and signalling testing | 15.4.0 |
| 2019-06 | RAN#84 | R5-194790 | 0778 | - | F | Updates to power allocations | 15.4.0 |
| 2019-06 | RAN#84 | R5-194791 | 0779 | - | F | Update of DownlinkConfigCommonSIB | 15.4.0 |
| 2019-06 | RAN#84 | R5-194794 | 0684 | 1 | F | Update IE PDSCH-Config | 15.4.0 |
| 2019-06 | RAN#84 | R5-194795 | 0687 | 1 | F | Update NR MeasObjectNR | 15.4.0 |
| 2019-06 | RAN#84 | R5-194796 | 0690 | 1 | F | Update default configuration of ReportConfigNR | 15.4.0 |
| 2019-06 | RAN#84 | R5-194797 | 0692 | 1 | F | Update chapter 4.5.2 RRC_IDLE procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-194798 | 0704 | 1 | F | Correction to the note associated to the Table 4.7.1-2 | 15.4.0 |
| 2019-06 | RAN#84 | R5-194800 | 0708 | 1 | F | Update IE MIB | 15.4.0 |
| 2019-06 | RAN#84 | R5-194801 | 0709 | 1 | F | Update IE SchedulingRequestResourceConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-194802 | 0712 | 1 | F | Correct clause numbers in 4.5A | 15.4.0 |
| 2019-06 | RAN#84 | R5-194803 | 0718 | 1 | F | Update IE ServingCellConfigCommon | 15.4.0 |
| 2019-06 | RAN#84 | R5-194804 | 0721 | 1 | F | Update IE FrequencyInfoUL | 15.4.0 |
| 2019-06 | RAN#84 | R5-194805 | 0722 | 1 | F | Update IE FrequencyInfoUL-SIB | 15.4.0 |
| 2019-06 | RAN#84 | R5-194806 | 0723 | 1 | F | Update generic procedures chapter general | 15.4.0 |
| 2019-06 | RAN#84 | R5-194807 | 0724 | 1 | F | Update chapter 4.5.2 RRC_IDLE Initiation | 15.4.0 |
| 2019-06 | RAN#84 | R5-194808 | 0730 | 1 | F | Updates to RadioBearerConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-194809 | 0732 | 1 | F | Updates to PhysicalCellGroupConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-194810 | 0739 | 1 | F | New test procedure for RRC_CONNECTED | 15.4.0 |
| 2019-06 | RAN#84 | R5-194811 | 0741 | 1 | F | Updated IE MeasObjectEUTRA and ReportConfigInterRAT | 15.4.0 |
| 2019-06 | RAN#84 | R5-194812 | 0753 | 1 | F | Updates to Registration 5GMM messages | 15.4.0 |
| 2019-06 | RAN#84 | R5-194813 | 0754 | 1 | F | Updates to UE 4.6.4 UE Capability Information Elements | 15.4.0 |
| 2019-06 | RAN#84 | R5-194814 | 0760 | 1 | F | New Test procedure for UE for Tracking area updating / inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode | 15.4.0 |
| 2019-06 | RAN#84 | R5-194817 | 0777 | 1 | F | New Test procedure for UE for Tracking area updating / inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 15.4.0 |
| 2019-06 | RAN#84 | R5-194821 | 0780 | - | F | Introducing conditions for Handover in RRCReconfiguration and RadioBearerConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-194824 | 0781 | - | F | Updates to Service Request 5GMM message | 15.4.0 |
| 2019-06 | RAN#84 | R5-194879 | 0735 | 1 | F | Updates to Multi-Cell SIG OTA testing for FR2 | 15.4.0 |
| 2019-06 | RAN#84 | R5-194881 | 0763 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n257B | 15.4.0 |
| 2019-06 | RAN#84 | R5-194882 | 0764 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260B | 15.4.0 |
| 2019-06 | RAN#84 | R5-194883 | 0765 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260I | 15.4.0 |
| 2019-06 | RAN#84 | R5-194884 | 0766 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n261B | 15.4.0 |
| 2019-06 | RAN#84 | R5-194885 | 0782 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260(A-I) | 15.4.0 |
| 2019-06 | RAN#84 | R5-194889 | 0737 | 1 | F | corrections to Non 3GPP Access over WLAN procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-194894 | 0783 | - | F | Update FFS in ResumeCause | 15.4.0 |
| 2019-06 | RAN#84 | R5-194896 | 0784 | - | F | Updates to reference QoS configurations for EPS interworking | 15.4.0 |
| 2019-06 | RAN#84 | R5-194902 | 0685 | 1 | F | Correction of Setup Diagrams for Receiver tests using Signal Generator in 38.508-1 | 15.4.0 |
| 2019-06 | RAN#84 | R5-195095 | 0750 | 1 | F | Introduction of Connection diagram for 2x4 and 4x4 Demod test cases | 15.4.0 |
| 2019-06 | RAN#84 | R5-195322 | 0686 | 1 | F | Update NR SIB1 | 15.4.0 |
| 2019-06 | RAN#84 | R5-195323 | 0703 | 1 | F | Update IE CommonCellGroupConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-195324 | 0715 | 1 | F | Update default configuration of MeasGapConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-195325 | 0719 | 1 | F | Addition of Switch off / Power off procedure in RRC_INACTIVE | 15.4.0 |
| 2019-06 | RAN#84 | R5-195326 | 0720 | 1 | F | Update of SIB5 | 15.4.0 |
| 2019-06 | RAN#84 | R5-195327 | 0731 | 1 | F | Updates to RLC-BearerConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-195328 | 0756 | 1 | F | Updates to PDU session establishment 5GSM messages | 15.4.0 |
| 2019-06 | RAN#84 | R5-195329 | 0773 | 1 | F | Introduction of test frequencies for inter-RAT signalling testing | 15.4.0 |
| 2019-06 | RAN#84 | R5-195330 | 0776 | 1 | F | Correction to PUSCH-Config | 15.4.0 |
| 2019-06 | RAN#84 | R5-195426 | 0727 | 2 | F | 38.508-1 implementation of FR2 UL demod OTA tests using single pol Rx TE | 15.4.0 |
| 2019-06 | RAN#84 | R5-195427 | 0772 | 2 | F | Addition of message contents needed for DEMOD test cases | 15.4.0 |
| 2019-06 | RAN#84 | R5-194370 | 0747 | - | F | Introduction of test frequencies for inter-band Rel-16 EN-DC two bands configurations | 16.0.0 |
| 2019-06 | RAN#84 | R5-194371 | 0748 | - | F | Introduction of test frequencies for inter-band Rel-16 EN-DC five bands configurations | 16.0.0 |
| 2019-06 | RAN#84 | R5-194373 | 0749 | - | F | Introduction of test frequencies for NR CA configuration CA_n41C | 16.0.0 |
| 2019-09 | RAN#85 | R5-195696 | 0795 | - | F | Update IE PDCP-Config | 16.1.0 |
| 2019-09 | RAN#85 | R5-195711 | 0797 | - | F | Add IE CGI-InfoEUTRA | 16.1.0 |
| 2019-09 | RAN#85 | R5-195729 | 0798 | - | F | Update IE CGI-Info | 16.1.0 |
| 2019-09 | RAN#85 | R5-195730 | 0799 | - | F | Update IE MeasResults | 16.1.0 |
| 2019-09 | RAN#85 | R5-195731 | 0800 | - | F | Update of 4.3.1.0A mid test CBW in 38.508-1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-195747 | 0803 | - | F | Update IE MeasResultCellListSFTD | 16.1.0 |
| 2019-09 | RAN#85 | R5-195748 | 0804 | - | F | Add IE MeasResultCellListSFTD-EUTRA | 16.1.0 |
| 2019-09 | RAN#85 | R5-195749 | 0805 | - | F | Add IE MeasResult2EUTRA | 16.1.0 |
| 2019-09 | RAN#85 | R5-195750 | 0806 | - | F | Add IE MeasResult2NR | 16.1.0 |
| 2019-09 | RAN#85 | R5-195751 | 0807 | - | F | Add IE SK-Counter | 16.1.0 |
| 2019-09 | RAN#85 | R5-195752 | 0808 | - | F | Update IE SS-RSSI-Measurement | 16.1.0 |
| 2019-09 | RAN#85 | R5-195792 | 0811 | - | F | Update MeasurementReport | 16.1.0 |
| 2019-09 | RAN#85 | R5-195885 | 0814 | - | F | Update RRCResume | 16.1.0 |
| 2019-09 | RAN#85 | R5-195886 | 0815 | - | F | Editorial update RRCReconfigurationComplete | 16.1.0 |
| 2019-09 | RAN#85 | R5-195887 | 0816 | - | F | Editorial update RRCReject | 16.1.0 |
| 2019-09 | RAN#85 | R5-195888 | 0817 | - | F | Editorial update RRCRelease | 16.1.0 |
| 2019-09 | RAN#85 | R5-195889 | 0818 | - | F | Add SCGFailureInformation | 16.1.0 |
| 2019-09 | RAN#85 | R5-195890 | 0819 | - | F | Add SCGFailureInformationEUTRA | 16.1.0 |
| 2019-09 | RAN#85 | R5-195895 | 0820 | - | F | Update UECapabilityEnquiry | 16.1.0 |
| 2019-09 | RAN#85 | R5-195909 | 0821 | - | F | Editorial update UECapabilityInformation | 16.1.0 |
| 2019-09 | RAN#85 | R5-195910 | 0822 | - | F | Add ULInformationTransferMRDC | 16.1.0 |
| 2019-09 | RAN#85 | R5-195926 | 0823 | - | F | Editorial update RRC IEs | 16.1.0 |
| 2019-09 | RAN#85 | R5-195927 | 0824 | - | F | Editorial update S-NSSAI | 16.1.0 |
| 2019-09 | RAN#85 | R5-195944 | 0826 | - | F | Correction to ReportConfigNR | 16.1.0 |
| 2019-09 | RAN#85 | R5-195945 | 0827 | - | F | Updates to default configurations for 5GC NAS test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196030 | 0829 | - | F | Handling of thresholds in FR2 when Events A3 and A6 are inter-frequency | 16.1.0 |
| 2019-09 | RAN#85 | R5-196031 | 0830 | - | F | Adding references to TS 38.508-1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196136 | 0836 | - | F | Addition new NR cell for SS-RSRP RRM tests | 16.1.0 |
| 2019-09 | RAN#85 | R5-196148 | 0837 | - | F | Update of Annex C for selecting SSB location for cells not selectable as PCell | 16.1.0 |
| 2019-09 | RAN#85 | R5-196158 | 0838 | - | F | Correction of references to test frequency tables | 16.1.0 |
| 2019-09 | RAN#85 | R5-196159 | 0839 | - | F | Correction of clause numbers for test frequencies for Non-3GPP Access | 16.1.0 |
| 2019-09 | RAN#85 | R5-196168 | 0840 | - | F | Correction of test frequency parameters for SSB location for NR band n1 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196169 | 0841 | - | F | Correction of test frequency parameters for SSB location for NR band n2 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196170 | 0842 | - | F | Correction of test frequency parameters for SSB location for NR band n3 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196171 | 0843 | - | F | Correction of test frequency parameters for SSB location for NR band n7 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196172 | 0844 | - | F | Correction of test frequency parameters for SSB location for NR band n25 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196174 | 0846 | - | F | Correction of test frequency parameters for SSB location for NR band n38 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196175 | 0847 | - | F | Correction of test frequency parameters for SSB location for NR band n39 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196176 | 0848 | - | F | Correction of test frequency parameters for SSB location for NR band n40 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196177 | 0849 | - | F | Correction of test frequency parameters for SSB location for NR band n41 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196178 | 0850 | - | F | Correction of test frequency parameters for SSB location for NR band n50 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196179 | 0851 | - | F | Correction of test frequency parameters for SSB location for NR band n66 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196180 | 0852 | - | F | Correction of test frequency parameters for SSB location for NR band n70 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196181 | 0853 | - | F | Correction of test frequency parameters for SSB location for NR band n74 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196182 | 0854 | - | F | Correction of test frequency parameters for SSB location for NR band n75 and SCS 15kHz and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196183 | 0855 | - | F | Correction of test frequency parameters for SSB location for NR band n77 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196184 | 0856 | - | F | Correction of test frequency parameters for SSB location for NR band n78 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196185 | 0857 | - | F | Correction of test frequency parameters for SSB location for NR band n79 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196197 | 0860 | - | F | Update IE ServingCellConfigCommon | 16.1.0 |
| 2019-09 | RAN#85 | R5-196198 | 0861 | - | F | Update IE SubcarrierSpacing | 16.1.0 |
| 2019-09 | RAN#85 | R5-196262 | 0863 | - | F | Editorial update IE RLC-BearerConfig | 16.1.0 |
| 2019-09 | RAN#85 | R5-196289 | 0864 | - | F | Update chapter 4.5A.2 UE-requested PDU session establishment procedure | 16.1.0 |
| 2019-09 | RAN#85 | R5-196310 | 0867 | - | F | Addition of SUL bands for protocol testing in clause 6.2.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196311 | 0868 | - | F | Update of test frequency parameters for NR CA configuration CA_n41C | 16.1.0 |
| 2019-09 | RAN#85 | R5-196315 | 0872 | - | F | Update of test frequency parameters for NR CA configuration CA_n260(A-I) | 16.1.0 |
| 2019-09 | RAN#85 | R5-196316 | 0873 | - | F | Introduction of test frequencies for NR CA configuration CA_n261B and CA_n261C | 16.1.0 |
| 2019-09 | RAN#85 | R5-196317 | 0874 | - | F | Update of test frequency table for EN-DC configuration DC_41A_n41A for BCS1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196318 | 0875 | - | F | Correction of test frequency parameters for EN-DC configuration DC_(n)41AA | 16.1.0 |
| 2019-09 | RAN#85 | R5-196319 | 0876 | - | F | Correction of test frequency parameters for EN-DC configuration DC_(n)71AA | 16.1.0 |
| 2019-09 | RAN#85 | R5-196468 | 0885 | - | F | Introduction of test frequencies for NR CA configuration CA_n258B and CA_n258C | 16.1.0 |
| 2019-09 | RAN#85 | R5-196469 | 0886 | - | F | Introduction of test frequencies for NR CA configuration CA_n258G to CA_n258M | 16.1.0 |
| 2019-09 | RAN#85 | R5-196470 | 0887 | - | F | Introduction of test frequencies for NR CA configuration CA_n260G to CA_n260I | 16.1.0 |
| 2019-09 | RAN#85 | R5-196472 | 0889 | - | F | Introduction of test frequencies for NR CA configuration CA_n261G to CA_n261I | 16.1.0 |
| 2019-09 | RAN#85 | R5-196473 | 0890 | - | F | Introduction of test frequencies for NR CA configuration CA_n261O to CA_n261Q | 16.1.0 |
| 2019-09 | RAN#85 | R5-196490 | 0894 | - | F | Introduction of test frequencies for NR CA configuration CA_n78C | 16.1.0 |
| 2019-09 | RAN#85 | R5-196539 | 0895 | - | F | Update to 38.508-1 for Demod specific message contents | 16.1.0 |
| 2019-09 | RAN#85 | R5-196581 | 0897 | - | F | Removing brackets from values for DCI formats | 16.1.0 |
| 2019-09 | RAN#85 | R5-196597 | 0899 | - | F | Cleanup of editor note of EFOPL5G | 16.1.0 |
| 2019-09 | RAN#85 | R5-196637 | 0900 | - | F | Update of default messages for EMERGENCY services test scenarios | 16.1.0 |
| 2019-09 | RAN#85 | R5-196641 | 0904 | - | F | Update of Test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 16.1.0 |
| 2019-09 | RAN#85 | R5-196654 | 0905 | - | F | Editorial correction of reference test conditions | 16.1.0 |
| 2019-09 | RAN#85 | R5-196751 | 0911 | - | F | AP#82.01: Update default DCI format to 0_1 / 1_1 in TS 38.508-1 for SIG test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196824 | 0917 | - | F | Updates to UE 4.6.4 UE Capability Information Elements | 16.1.0 |
| 2019-09 | RAN#85 | R5-196825 | 0918 | - | F | Addition of default test control messages | 16.1.0 |
| 2019-09 | RAN#85 | R5-196873 | 0922 | - | F | Introduction of test frequencies for NR CA configuration CA_n258D to CA_n258F | 16.1.0 |
| 2019-09 | RAN#85 | R5-196874 | 0923 | - | F | Introduction of test frequencies for NR CA configuration CA_n260D to CA_n260F | 16.1.0 |
| 2019-09 | RAN#85 | R5-196875 | 0924 | - | F | Introduction of test frequencies for NR CA configuration CA_n260O to CA_n260Q | 16.1.0 |
| 2019-09 | RAN#85 | R5-196942 | 0927 | - | F | Correction of clause 2 and 4.3 in 38.508-1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196980 | 0786 | 1 | F | Using generic procedure for IMS registration to 5GS | 16.1.0 |
| 2019-09 | RAN#85 | R5-196981 | 0788 | 1 | F | Update of SIB2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196982 | 0790 | 1 | F | Update of SIB5 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196984 | 0792 | 1 | F | Update of frequency definition for Inter-RAT test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196985 | 0793 | 1 | F | Update IE CellGroupConfig | 16.1.0 |
| 2019-09 | RAN#85 | R5-196986 | 0858 | 1 | F | Update MIB | 16.1.0 |
| 2019-09 | RAN#85 | R5-196987 | 0865 | 1 | F | Update chapter 4.5.4 RRC_CONNECTED procedures | 16.1.0 |
| 2019-09 | RAN#85 | R5-196988 | 0831 | 1 | F | Addition of IE MasterKeyUpdate | 16.1.0 |
| 2019-09 | RAN#85 | R5-196990 | 0833 | 1 | F | USIM Configuration for Signalling Test Cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196991 | 0835 | 1 | F | Correction to SIG OTA UE Orientation procedure | 16.1.0 |
| 2019-09 | RAN#85 | R5-196992 | 0878 | 1 | F | Addition of New Test Procedure - Response\No response to Paging for 5GC NAS testing | 16.1.0 |
| 2019-09 | RAN#85 | R5-196994 | 0913 | 1 | F | Update IE ServingCellConfig | 16.1.0 |
| 2019-09 | RAN#85 | R5-196995 | 0910 | 1 | F | Corrections to DCI_1_0 configuration | 16.1.0 |
| 2019-09 | RAN#85 | R5-196996 | 0919 | 1 | F | Updates to generic procedure using SERVICE REQUEST procedure | 16.1.0 |
| 2019-09 | RAN#85 | R5-196997 | 0901 | 1 | F | Introduction of Test Procedure for IMS Emergency call establishment in 5GC NORMAL-SERVICE | 16.1.0 |
| 2019-09 | RAN#85 | R5-196998 | 0903 | 1 | F | Update of Test procedure for UE for Tracking area updating / Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode | 16.1.0 |
| 2019-09 | RAN#85 | R5-197014 | 0928 | - | F | Addition of NR CA test frequencies for protocol testing in clause 6.2.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197099 | 0929 | - | F | Correction to Switch off-Power off procedure in RRC_CONNECTED | 16.1.0 |
| 2019-09 | RAN#85 | R5-197104 | 0884 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n257G to CA_n257M | 16.1.0 |
| 2019-09 | RAN#85 | R5-197106 | 0892 | 1 | F | Update of EN-DC inter-band configurations in clause 4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197139 | 0891 | 2 | F | Update of NR CA inter-band configurations in clause 4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197226 | 0915 | 1 | F | changes for Non 3GPP Access over WLAN | 16.1.0 |
| 2019-09 | RAN#85 | R5-197230 | 0883 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n257D to CA_n257F | 16.1.0 |
| 2019-09 | RAN#85 | R5-197231 | 0869 | 1 | F | Update of test frequency parameters for NR CA configuration CA_n257B and CA_n257C | 16.1.0 |
| 2019-09 | RAN#85 | R5-197232 | 0870 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260B and CA_n260C | 16.1.0 |
| 2019-09 | RAN#85 | R5-197233 | 0888 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260J to CA_n260M | 16.1.0 |
| 2019-09 | RAN#85 | R5-197234 | 0813 | 1 | F | Update RRCReconfiguration | 16.1.0 |
| 2019-09 | RAN#85 | R5-197235 | 0796 | 1 | F | Update RadioBearerConfig-DRB | 16.1.0 |
| 2019-09 | RAN#85 | R5-197236 | 0825 | 1 | F | Update RRCReconfiguration-HO | 16.1.0 |
| 2019-09 | RAN#85 | R5-197241 | 0791 | 1 | F | Update of EUTRA-AllowedMeasBandwidth | 16.1.0 |
| 2019-09 | RAN#85 | R5-197243 | 0809 | 1 | F | Addition of Delta to signalling threshold in System Information in FR2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197244 | 0866 | 1 | F | Correction to REGISTRATION REJECT message | 16.1.0 |
| 2019-09 | RAN#85 | R5-197246 | 0902 | 1 | F | Introduction of Test Procedure for IMS Emergency call establishment in 5GC LIMITED-SERVICE or NO-SUPI | 16.1.0 |
| 2019-09 | RAN#85 | R5-197296 | 0898 | 2 | F | Update of PHR-Config | 16.1.0 |
| 2019-09 | RAN#85 | R5-197300 | 0787 | 1 | F | 4x2 Connection Diagram for demodulation tests | 16.1.0 |
| 2019-09 | RAN#85 | R5-197301 | 0862 | 1 | F | Correction to Section 5.4.2 Message definition for Performance Test | 16.1.0 |
| 2019-09 | RAN#85 | R5-197302 | 0882 | 1 | F | Addition of FR2 CA connection diagram | 16.1.0 |
| 2019-09 | RAN#85 | R5-197303 | 0920 | 1 | F | Corrections to test frequencies and formulas | 16.1.0 |
| 2019-09 | RAN#85 | R5-197304 | 0896 | 1 | F | Removing IOT bit information from test channel bandwidth tables | 16.1.0 |
| 2019-09 | RAN#85 | R5-197305 | 0908 | 1 | F | Addition of SMTC Configuration for RRM test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197507 | 0906 | 1 | F | Addition of TDD UL DL Config for RRM test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197508 | 0907 | 1 | F | Addition of FilterCoefficient configuration for RRM test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197638 | 0881 | 2 | F | Addition of FR1 CA connection diagram | 16.1.0 |
| 2019-10 | RAN#85 | - | - | - | - | Implementation fixes | 16.1.1 |
| 2019-12 | RAN#86 | R5-197727 | 0932 | - | F | Editorial update IE BWP-Id | 16.2.0 |
| 2019-12 | RAN#86 | R5-197751 | 0933 | - | F | Editorial update IE PDSCH-TimeDomainResourceAllocationList | 16.2.0 |
| 2019-12 | RAN#86 | R5-197835 | 0937 | - | F | Correction to IE ReportConfigNR | 16.2.0 |
| 2019-12 | RAN#86 | R5-197897 | 0940 | - | F | Editorial update IE CodebookConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-197932 | 0946 | - | F | Editorial update IE PDSCH-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-197967 | 0948 | - | F | Update of Annex C on calculation of test frequencies and parameters | 16.2.0 |
| 2019-12 | RAN#86 | R5-197968 | 0949 | - | F | Correction of test frequency parameters for NR band n1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197969 | 0950 | - | F | Correction of test frequency parameters for NR band n2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197970 | 0951 | - | F | Correction of test frequency parameters for NR band n3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197971 | 0952 | - | F | Correction of test frequency parameters for NR band n5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197972 | 0953 | - | F | Correction of test frequency parameters for NR band n7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197973 | 0954 | - | F | Correction of test frequency parameters for NR band n8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197974 | 0955 | - | F | Correction of test frequency parameters for NR band n12 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197975 | 0956 | - | F | Correction of test frequency parameters for NR band n20 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197976 | 0957 | - | F | Correction of test frequency parameters for NR band n25 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197977 | 0958 | - | F | Correction of test frequency parameters for NR band n28 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197978 | 0959 | - | F | Correction of test frequency parameters for NR band n34 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197979 | 0960 | - | F | Correction of test frequency parameters for NR band n38 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197980 | 0961 | - | F | Correction of test frequency parameters for NR band n39 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197981 | 0962 | - | F | Correction of test frequency parameters for NR band n40 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197982 | 0963 | - | F | Correction of test frequency parameters for NR band n41 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197983 | 0964 | - | F | Correction of test frequency parameters for NR band n50 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197984 | 0965 | - | F | Correction of test frequency parameters for NR band n51 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197985 | 0966 | - | F | Correction of test frequency parameters for NR band n66 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197986 | 0967 | - | F | Correction of test frequency parameters for NR band n70 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197987 | 0968 | - | F | Correction of test frequency parameters for NR band n71 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197988 | 0969 | - | F | Correction of test frequency parameters for NR band n74 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197989 | 0970 | - | F | Correction of test frequency parameters for NR band n75 (SDL) | 16.2.0 |
| 2019-12 | RAN#86 | R5-197990 | 0971 | - | F | Correction of test frequency parameters for NR band n76 (SDL) | 16.2.0 |
| 2019-12 | RAN#86 | R5-197991 | 0972 | - | F | Correction of test frequency parameters for NR band n77 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197992 | 0973 | - | F | Correction of test frequency parameters for NR band n78 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197993 | 0974 | - | F | Correction of test frequency parameters for NR band n79 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197994 | 0975 | - | F | Editorial correction to note 1 in frequency tables for NR bands n257, n258, n260 and n261 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197997 | 0978 | - | F | Introduction of test frequencies for NR CA configuration CA_n261D to CA_n261F | 16.2.0 |
| 2019-12 | RAN#86 | R5-197998 | 0979 | - | F | Introduction of test frequencies for NR CA configuration CA_n261J to CA_n261M | 16.2.0 |
| 2019-12 | RAN#86 | R5-198016 | 0983 | - | F | Introduction of test frequencies parameters for Rel-16 NR CA configuration CA_n66B | 16.2.0 |
| 2019-12 | RAN#86 | R5-198017 | 0984 | - | F | Introduction of test frequencies parameters for Rel-16 NR CA configuration CA_n66(2A) | 16.2.0 |
| 2019-12 | RAN#86 | R5-198018 | 0985 | - | F | Introduction of test frequencies and parameters for NR band n29 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198019 | 0986 | - | F | Introduction of test frequencies and parameters for NR band n65 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198028 | 0988 | - | F | Add 4Rx connection diagram for RRM measurement tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198057 | 0993 | - | F | Editorial update IE RateMatchPattern | 16.2.0 |
| 2019-12 | RAN#86 | R5-198058 | 0994 | - | F | Editorial update IE SchedulingRequestResourceConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-198082 | 0999 | - | F | Introduce general chapter in 4.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198120 | 1004 | - | F | Correction of test frequencies parameters for Rel-15 EN-DC configuration DC_(n)41AA | 16.2.0 |
| 2019-12 | RAN#86 | R5-198121 | 1005 | - | F | Correction of test frequencies parameters for Rel-15 EN-DC configuration DC_(n)71AA | 16.2.0 |
| 2019-12 | RAN#86 | R5-198125 | 1009 | - | F | Introduction of test frequencies and parameters for NR band n48 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198126 | 1010 | - | F | Introduction of test frequencies for NR band b41 and CBW 30MHz | 16.2.0 |
| 2019-12 | RAN#86 | R5-198131 | 1012 | - | F | Update of USIM Configuration 15 for forbidden PLMN | 16.2.0 |
| 2019-12 | RAN#86 | R5-198133 | 1014 | - | F | Update IE ServingCellConfigCommonSIB | 16.2.0 |
| 2019-12 | RAN#86 | R5-198141 | 1015 | - | F | Clarification on default radio configuration of NAS cells | 16.2.0 |
| 2019-12 | RAN#86 | R5-198217 | 1019 | - | F | Correction of test frequency parameters for protocol testing and NR bands with scs=15kHz | 16.2.0 |
| 2019-12 | RAN#86 | R5-198223 | 1021 | - | F | Add IE BetaOffsets | 16.2.0 |
| 2019-12 | RAN#86 | R5-198224 | 1022 | - | F | Update IE PUSCH-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198250 | 1023 | - | F | Update IE CSI-FrequencyOccupation | 16.2.0 |
| 2019-12 | RAN#86 | R5-198251 | 1024 | - | F | Update IE PHR-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198258 | 1026 | - | F | Editorial update IE DRX-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198282 | 1027 | - | F | Update to Connection diagram for 2x4 and 4x4 Demod test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-198286 | 1028 | - | F | Correction of mapping of frequency ranges to NR test frequencies for NR SA | 16.2.0 |
| 2019-12 | RAN#86 | R5-198304 | 1029 | - | F | Editorial update IE LogicalChannelConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-198370 | 1035 | - | F | Connection diagram for FR2 Demod and CSI test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-198480 | 1039 | - | F | Editorial update IE PDCCH-ConfigCommon | 16.2.0 |
| 2019-12 | RAN#86 | R5-198485 | 1041 | - | F | Editorial update IE PDCP-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198506 | 1044 | - | F | Addition of RRCReconfiguration for Speech call setup | 16.2.0 |
| 2019-12 | RAN#86 | R5-198507 | 1045 | - | F | Editorial updates to section 4.7.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198508 | 1046 | - | F | New reference QoS configurations for IMS voice and video | 16.2.0 |
| 2019-12 | RAN#86 | R5-198509 | 1047 | - | F | Updates to REGISTRATION ACCEPT 5GMM message | 16.2.0 |
| 2019-12 | RAN#86 | R5-198510 | 1048 | - | F | Updates to test control messages | 16.2.0 |
| 2019-12 | RAN#86 | R5-198540 | 1051 | - | F | Update of REGISTRATION ACCEPT for IMS emergency support | 16.2.0 |
| 2019-12 | RAN#86 | R5-198544 | 1053 | - | F | Update of Table 4.6.3-162 SearchSpace in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198638 | 1058 | - | F | Corrections on test frequencies for NR CA band n260 in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198649 | 1059 | - | F | Corrections on test frequencies for NR CA band n261 in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198659 | 1062 | - | F | Update TCI State Cell parameter in Demod section | 16.2.0 |
| 2019-12 | RAN#86 | R5-198718 | 1067 | - | F | Updates to RSRP-Range, RSRQ-Range and SINR-Range | 16.2.0 |
| 2019-12 | RAN#86 | R5-198847 | 0941 | 1 | F | Corrections to DCI_1_1 configuration | 16.2.0 |
| 2019-12 | RAN#86 | R5-198848 | 0931 | 1 | F | Update IE PUSCH-TimeDomainResourceAllocationList | 16.2.0 |
| 2019-12 | RAN#86 | R5-198850 | 0934 | 1 | F | Correction to IE MasterKeyUpdate | 16.2.0 |
| 2019-12 | RAN#86 | R5-198851 | 0935 | 1 | F | Update of NR SIBs | 16.2.0 |
| 2019-12 | RAN#86 | R5-198852 | 0936 | 1 | F | Correction to USIM configuration | 16.2.0 |
| 2019-12 | RAN#86 | R5-198853 | 0938 | 1 | F | Correction to IE ReportConfigInterRAT | 16.2.0 |
| 2019-12 | RAN#86 | R5-198854 | 0942 | 1 | F | Correction to Table 4.9.9.2.3-1 for Inter-system change from S1 mode to N1 mode in 5GMM-IDLE mode | 16.2.0 |
| 2019-12 | RAN#86 | R5-198856 | 0982 | 1 | F | Addition of frequency configurations for NR MFBI testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-198857 | 0992 | 1 | F | Editorial update IE CSI-AperiodicTriggerStateList | 16.2.0 |
| 2019-12 | RAN#86 | R5-198858 | 0998 | 1 | F | Editorial update IE ServingCellConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-198859 | 1000 | 1 | F | Editorial update IE SecurityAlgorithmConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-198860 | 1002 | 1 | F | Update IE SRS-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198861 | 1001 | 1 | F | Update of Generic Test Procedures for IMS Emergency call establishment 4.9.11 and 4.9.12 to reflect the fact that they can be used in multiple states and scenarios | 16.2.0 |
| 2019-12 | RAN#86 | R5-198862 | 1013 | 1 | F | Update IE ServingCellConfigCommon | 16.2.0 |
| 2019-12 | RAN#86 | R5-198863 | 1016 | 1 | F | Update IE CSI-RS-ResourceMapping | 16.2.0 |
| 2019-12 | RAN#86 | R5-198865 | 1032 | 1 | F | Update RRCReconfiguration | 16.2.0 |
| 2019-12 | RAN#86 | R5-198866 | 1033 | 1 | F | Update chapter 4.5.1 General | 16.2.0 |
| 2019-12 | RAN#86 | R5-198867 | 1034 | 1 | F | Update to PDU SESSION ESTABLISHMENT ACCEPT and Reference QoS flow descriptions to align EPS bearer id format | 16.2.0 |
| 2019-12 | RAN#86 | R5-198869 | 1061 | 1 | F | New Test Procedures for IMS Emergency call release | 16.2.0 |
| 2019-12 | RAN#86 | R5-198870 | 1063 | 1 | F | Update chapter 4.5.2 RRC_IDLE | 16.2.0 |
| 2019-12 | RAN#86 | R5-198871 | 1072 | 1 | F | Update of RRCReconfiguration for measurement configuration | 16.2.0 |
| 2019-12 | RAN#86 | R5-198955 | 0995 | 1 | F | Update procedure for NR RF CA testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-198956 | 0996 | 1 | F | Update procedure for EN-DC RF CA testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-198957 | 1036 | 1 | F | Update to 38.508-1 for DEMOD message contents | 16.2.0 |
| 2019-12 | RAN#86 | R5-198958 | 0991 | 1 | F | Update IE PUCCH-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198959 | 0976 | 1 | F | Introduction of test frequencies for Rel-15 EN-DC inter-band configurations | 16.2.0 |
| 2019-12 | RAN#86 | R5-198960 | 0980 | 1 | F | Introduction of test frequencies for Rel-16 NR inter-band CA configurations | 16.2.0 |
| 2019-12 | RAN#86 | R5-198961 | 0987 | 1 | F | Introduction of test frequencies for NR configuration CA_n29A-n66A | 16.2.0 |
| 2019-12 | RAN#86 | R5-198962 | 1003 | 1 | F | Introduction of test frequencies for Rel-15 NR DC configurations | 16.2.0 |
| 2019-12 | RAN#86 | R5-198997 | 1068 | 1 | F | Introduction of test frequencies and parameters for NR bands n29, n48 and n65 for protocol testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-199008 | 0997 | 1 | F | Editorial update WLAN table 4.5.2.2-3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199013 | 0943 | 1 | F | Correction to SMTC and GAP for inter frequency cell | 16.2.0 |
| 2019-12 | RAN#86 | R5-199015 | 0981 | 1 | F | Correction of test frequencies for NR CA and EN-DC protocol testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-199016 | 1049 | 1 | F | Updates to Test Procedure 4.9.11 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199017 | 1050 | 1 | F | Updates to Test Procedure 4.9.12 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199020 | 1079 | - | F | Update default setting of deriveSSB-IndexFromCell | 16.2.0 |
| 2019-12 | RAN#86 | R5-199021 | 1011 | 1 | F | Update IE TDD-UL-DL-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-199022 | 1070 | 1 | F | Updates to Signalling Reference test conditions | 16.2.0 |
| 2019-12 | RAN#86 | R5-199026 | 1025 | 2 | F | Update IE CellGroupConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-199071 | 0944 | 2 | F | Update of SUL related messages | 16.2.0 |
| 2019-12 | RAN#86 | R5-199075 | 1080 | 1 | F | Correction to NR RRC_IDLE mode procedure | 16.2.0 |
| 2019-12 | RAN#86 | R5-199093 | 1065 | 2 | F | Update chapter 4.5.4 RRC_CONNECTED | 16.2.0 |
| 2019-12 | RAN#86 | R5-199094 | 1069 | 1 | F | Updates for handling of Multiple PDU sessions / Multiple DRBs | 16.2.0 |
| 2019-12 | RAN#86 | R5-199103 | 1076 | 2 | F | Adding new generic procedure for UE-requested PDU session modification after the first S1 to N1 mode change | 16.2.0 |
| 2019-12 | RAN#86 | R5-199300 | 1042 | 1 | F | Corrections on category of EN-DC configurations for test frequencies in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199301 | 1054 | 1 | F | Addition of ServingCellConfigCommon for RRM tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-199302 | 1057 | 1 | F | Corrections on test frequencies for NR CA band n257 in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199303 | 1060 | 1 | F | Corrections on test frequencies for NR CA band n258 in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199304 | 1066 | 1 | F | Update Radio resource control information elements for RRM to add CSI-RS for Tracking | 16.2.0 |
| 2019-12 | RAN#86 | R5-199423 | 1077 | - | F | Update ra-responseWindow in TS 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199481 | 0989 | 1 | F | Addition of FR1 NR CA and NR 4Rx connection diagrams | 16.2.0 |
| 2019-12 | RAN#86 | R5-199511 | 1078 | - | F | Update of quiet zone size | 16.2.0 |
| 2019-12 | RAN#86 | R5-199545 | 1020 | 1 | F | Addition of multi-AoA capabilities for IFF | 16.2.0 |
| 2020-03 | RAN#87 | R5-200135 | 1120 |  | F | Removal of Correction to SIG OTA UE Orientation procedure | 16.3.0 |
| 2020-03 | RAN#87 | R5-200147 | 1130 |  | F | Update to USIM config 6.4.1-11 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200244 | 1133 |  | F | Correction to nAndPagingFrameOffset | 16.3.0 |
| 2020-03 | RAN#87 | R5-200296 | 1136 |  | F | Addition of generic procedure for IMS MO speech setup | 16.3.0 |
| 2020-03 | RAN#87 | R5-200297 | 1137 |  | F | Addition of generic procedure for IMS MT speech setup | 16.3.0 |
| 2020-03 | RAN#87 | R5-200298 | 1138 |  | F | Addition of generic procedure for IMS MO call release | 16.3.0 |
| 2020-03 | RAN#87 | R5-200299 | 1139 |  | F | Addition of generic procedure for IMS MT call release | 16.3.0 |
| 2020-03 | RAN#87 | R5-200349 | 1142 |  | F | Correction to frequencyBandList in SIB4 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200431 | 1146 |  | F | Correction to CSI-FrequencyOccupation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200432 | 1147 |  | F | Correction to default setting of additionalPmax | 16.3.0 |
| 2020-03 | RAN#87 | R5-200433 | 1148 |  | F | Correction to powerControlOffset for performance tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200434 | 1149 |  | F | Correction to RACH configuration for RRM tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200435 | 1150 |  | F | Correction to TDD UL-DL Config for performance test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200477 | 1154 |  | F | Update to Registration REQ and Authentication Response message | 16.3.0 |
| 2020-03 | RAN#87 | R5-200499 | 1157 |  | F | Correction of test frequency tables for NR band n1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200500 | 1158 |  | F | Correction of test frequency tables for NR band n2 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200501 | 1159 |  | F | Correction of test frequency tables for NR band n3 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200502 | 1160 |  | F | Correction of test frequency tables for NR band n7 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200503 | 1161 |  | F | Correction of test frequency tables for NR band n25 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200504 | 1162 |  | F | Correction of test frequency tables for NR band n28 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200505 | 1163 |  | F | Correction of test frequency tables for NR band n34 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200506 | 1164 |  | F | Correction of test frequency tables for NR band n38 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200507 | 1165 |  | F | Correction of test frequency tables for NR band n39 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200508 | 1166 |  | F | Correction of test frequency tables for NR band n40 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200510 | 1168 |  | F | Correction of test frequency tables for NR band n50 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200511 | 1169 |  | F | Correction of test frequency tables for NR band n66 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200512 | 1170 |  | F | Correction of test frequency tables for NR band n70 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200513 | 1171 |  | F | Correction of test frequency tables for NR band n71 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200514 | 1172 |  | F | Correction of test frequency tables for NR band n74 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200515 | 1173 |  | F | Correction of test frequency tables for NR band n75 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200531 | 1189 |  | F | Correction of test frequency tables for NR band n29 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200532 | 1190 |  | F | Correction of test frequency tables for NR band n48 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200533 | 1191 |  | F | Correction of test frequency tables for NR band n65 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200597 | 1200 |  | F | Introduction of test frequencies for inter-band Rel-16 EN-DC configurations in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200605 | 1202 |  | F | Addition of test frequencies for n95 SUL band | 16.3.0 |
| 2020-03 | RAN#87 | R5-200645 | 1206 |  | F | Updates to 4.6.4 UE Capability Information Elements | 16.3.0 |
| 2020-03 | RAN#87 | R5-200646 | 1207 |  | F | Correction to QoS rule number 7 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200647 | 1208 |  | F | Correction to IMS emergency call release procedures | 16.3.0 |
| 2020-03 | RAN#87 | R5-201246 | 1209 | 1 | F | TRS configuration messages definition for RF in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200678 | 1210 |  | F | Update of IE ControlResourceSet to introduce band and channel bandwidth specific values for frequencyDomainResources | 16.3.0 |
| 2020-03 | RAN#87 | R5-200703 | 1213 |  | F | Correction to IE BeamFailureRecoveryConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200774 | 1215 |  | F | Editorial update IE MeasConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200775 | 1216 |  | F | Editorial update IE radioLinkMonitoringRS-Id | 16.3.0 |
| 2020-03 | RAN#87 | R5-200804 | 1218 |  | F | Correction of test frequency tables for NR band n5 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200805 | 1219 |  | F | Correction of test frequency tables for NR band n8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200806 | 1220 |  | F | Correction of test frequency tables for NR band n12 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200807 | 1221 |  | F | Correction of test frequency tables for NR band n20 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200808 | 1222 |  | F | Correction of test frequency tables for NR band n51 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200900 | 1197 | 1 | F | Corrections on test frequencies for EN-DC band combinations including FR1 and FR2 in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200901 | 1198 | 1 | F | Corrections on test frequencies for EN-DC band combinations including FR2 in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200902 | 1199 | 1 | F | Corrections on uplink EN-DC configurations for test frequencies in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200921 | 1132 | 1 | F | Addition of Rel-16 inter-band CA and EN-DC FR1 two bands test configurations | 16.3.0 |
| 2020-03 | RAN#87 | R5-200930 | 1081 | 1 | F | Update SIB1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200931 | 1082 | 1 | F | Update CounterCheck | 16.3.0 |
| 2020-03 | RAN#87 | R5-200932 | 1083 | 1 | F | Editorial update DLInformationTransfer | 16.3.0 |
| 2020-03 | RAN#87 | R5-200933 | 1084 | 1 | F | Editorial update FailureInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200934 | 1085 | 1 | F | Editorial update MeasurementReport | 16.3.0 |
| 2020-03 | RAN#87 | R5-200935 | 1086 | 1 | F | Editorial update MobilityFromNRCommand | 16.3.0 |
| 2020-03 | RAN#87 | R5-200936 | 1087 | 1 | F | Editorial update Paging | 16.3.0 |
| 2020-03 | RAN#87 | R5-200937 | 1088 | 1 | F | Editorial update RRCReestablishment | 16.3.0 |
| 2020-03 | RAN#87 | R5-200938 | 1090 | 1 | F | Editorial update RRCReconfigurationComplete | 16.3.0 |
| 2020-03 | RAN#87 | R5-200939 | 1091 | 1 | F | Editorial update RRCReject | 16.3.0 |
| 2020-03 | RAN#87 | R5-200940 | 1092 | 1 | F | Editorial update RRCRelease | 16.3.0 |
| 2020-03 | RAN#87 | R5-200941 | 1093 | 1 | F | Editorial update RRCResumeComplete | 16.3.0 |
| 2020-03 | RAN#87 | R5-200942 | 1094 | 1 | F | Editorial update RRCSetup | 16.3.0 |
| 2020-03 | RAN#87 | R5-200943 | 1096 | 1 | F | Editorial update SCGFailureInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200944 | 1097 | 1 | F | Editorial update SecurityMode | 16.3.0 |
| 2020-03 | RAN#87 | R5-200945 | 1098 | 1 | F | Update SystemInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200946 | 1099 | 1 | F | Editorial update UEAssistanceInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200947 | 1100 | 1 | F | Editorial update UECapability | 16.3.0 |
| 2020-03 | RAN#87 | R5-200948 | 1101 | 1 | F | Editorial update ULInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200949 | 1103 | 1 | F | Editorial update IE RLC-BearerConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200951 | 1108 | 1 | F | Add IE TDD-UL-DL-ConfigDedicated | 16.3.0 |
| 2020-03 | RAN#87 | R5-200952 | 1111 | 1 | F | Update IE ServingCellConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200953 | 1112 | 1 | F | Update IE ServingCellConfigCommonSIB | 16.3.0 |
| 2020-03 | RAN#87 | R5-200954 | 1113 | 1 | F | Update IE DMRS-DownlinkConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200955 | 1114 | 1 | F | Update IE FrequencyInfoUL | 16.3.0 |
| 2020-03 | RAN#87 | R5-200956 | 1118 | 1 | F | Update chapter 4.5.1 General | 16.3.0 |
| 2020-03 | RAN#87 | R5-200957 | 1122 | 1 | F | Update chapter 4.5.4 RRC_CONNECTED | 16.3.0 |
| 2020-03 | RAN#87 | R5-200958 | 1125 | 1 | F | Update IE CellGroupId | 16.3.0 |
| 2020-03 | RAN#87 | R5-200959 | 1126 | 1 | F | Update IE ServCellIndex | 16.3.0 |
| 2020-03 | RAN#87 | R5-200960 | 1127 | 1 | F | Update IE SK-Counter | 16.3.0 |
| 2020-03 | RAN#87 | R5-200961 | 1128 | 1 | F | Update IE SDAP-Config | 16.3.0 |
| 2020-03 | RAN#87 | R5-200965 | 1145 | 1 | F | Correction to CORESET and search space configuration | 16.3.0 |
| 2020-03 | RAN#87 | R5-200966 | 1193 | 1 | F | Addition of NR SUL connection diagrams | 16.3.0 |
| 2020-03 | RAN#87 | R5-200967 | 1201 | 1 | F | Clarification to high test channel bandwidth table | 16.3.0 |
| 2020-03 | RAN#87 | R5-200968 | 1203 | 1 | F | Addition of missing EN-DC test frequencies | 16.3.0 |
| 2020-03 | RAN#87 | R5-200996 | 1124 | 1 | F | Correction to PUCCH-Config for Format1 and Format2 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201005 | 1131 | 1 | F | Update of Annex C on calculation of test frequencies to achieve full bandwidth testing of NR bands | 16.3.0 |
| 2020-03 | RAN#87 | R5-201020 | 1155 | 1 | F | Update SIG test frequencies in clause 6.2.3.x | 16.3.0 |
| 2020-03 | RAN#87 | R5-201021 | 1167 | 1 | F | Correction of test frequency tables for NR band n41 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201022 | 1174 | 1 | F | Correction of test frequency tables for NR band n77 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201023 | 1175 | 1 | F | Correction of test frequency tables for NR band n78 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201024 | 1176 | 1 | F | Correction of test frequency tables for NR band n79 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201025 | 1177 | 1 | F | Correction of test frequency tables for NR band n257 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201026 | 1178 | 1 | F | Correction of test frequency tables for NR band n258 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201027 | 1179 | 1 | F | Correction of test frequency tables for NR band n260 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201028 | 1180 | 1 | F | Correction of test frequency tables for NR band n261 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201029 | 1192 | 1 | F | Update of clause 4.4.2 on simulated cells | 16.3.0 |
| 2020-03 | RAN#87 | R5-201061 | 1153 | 1 | F | Addition of a few R16s inter-band EN-DC FR1 test configurations | 16.3.0 |
| 2020-03 | RAN#87 | R5-201065 | 1194 | 1 | F | Addition of test channel bandwidth for NR bands in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201092 | 1123 | 1 | F | Updates to NR FR1 and LTE Power levels in OTA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201093 | 1224 | 1 | F | Message content Updates for Carrier Aggregation | 16.3.0 |
| 2020-03 | RAN#87 | R5-201108 | 1143 | 1 | F | Correction to EUTRA-AllowedMeasBandwidth | 16.3.0 |
| 2020-03 | RAN#87 | R5-201116 | 1204 | 1 | F | Updates to 4.7.3 Contents of EAP-AKA messages in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201148 | 1134 | 1 | F | Updates to default SSB index of intra-frequency NR cells | 16.3.0 |
| 2020-03 | RAN#87 | R5-201159 | 1151 | 1 | F | Correction to test frequencies for n257 intra-band contiguous CA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201173 | 1117 | 1 | F | Update IE TDD-UL-DL-Config | 16.3.0 |
| 2020-03 | RAN#87 | R5-201174 | 1095 | 1 | F | Update RRCSystemInfoRequest | 16.3.0 |
| 2020-03 | RAN#87 | R5-201175 | 1106 | 1 | F | Update IE RLF-TimersAndConstants | 16.3.0 |
| 2020-03 | RAN#87 | R5-201176 | 1107 | 1 | F | Update IE SCS-SpecificCarrier | 16.3.0 |
| 2020-03 | RAN#87 | R5-201177 | 1109 | 1 | F | Update chapter 4.6.0 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201179 | 1116 | 1 | F | Update IE MeasObjectNR | 16.3.0 |
| 2020-03 | RAN#87 | R5-201189 | 1214 | 1 | F | Addition of IFF DFF Hybrid Setup for FR2 2AoA RRM test | 16.3.0 |
| 2020-03 | RAN#87 | R5-201194 | 1141 | 1 | F | Update to Common Coreset RB IE and section 5-6 Demod message contents | 16.3.0 |
| 2020-03 | RAN#87 | R5-201195 | 1144 | 1 | F | Update of DCI 1_0 and DCI_1_1 configuration | 16.3.0 |
| 2020-03 | RAN#87 | R5-201197 | 1152 | 1 | F | Correction to TRS configuration for RRM tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-201202 | 1205 | 1 | F | Update to Switch Off/ Power off procedure in RRC_CONNECTED mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201203 | 1129 | 1 | F | Update to PDCP-Config | 16.3.0 |
| 2020-03 | RAN#87 | R5-201217 | 1217 | 1 | F | Updates to PsDU session modification procedures | 16.3.0 |
| 2020-03 | RAN#87 | R5-201221 | 1089 | 1 | F | Update RRCReconfiguration | 16.3.0 |
| 2020-03 | RAN#87 | R5-201222 | 1121 | 1 | F | Update IE CellGroupConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-201232 | 1140 | 2 | F | CR to 38.508-1 to introduce DFF Range Length | 16.3.0 |
| 2020-03 | RAN#87 | R5-201234 | 1110 | 2 | F | Update IE ServingCellConfigCommon | 16.3.0 |
| 2020-03 | RAN#87 | R5-201148 | 1134 | 1 | F | Add new missing column of Table 4.4.2-2 | 16.3.1 |
| 2020-06 | RAN#88 | R5-201320 | 1225 | - | F | Update IE CellGroupConfig | 16.4.0 |
| 2020-06 | RAN#88 | R5-201322 | 1227 | - | F | Update of default value of frequencyDomainResources in ControlResourceSet IE | 16.4.0 |
| 2020-06 | RAN#88 | R5-201331 | 1228 | - | F | Correction to Table 4.9.6.1-1-Switch off in Idle | 16.4.0 |
| 2020-06 | RAN#88 | R5-201333 | 1230 | - | F | Addition of 4.9.6.3A Switch off Power off procedure in RRC_CONNECTED with T3540 started | 16.4.0 |
| 2020-06 | RAN#88 | R5-201335 | 1232 | - | F | Update to USIM config 6.4.1-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201336 | 1233 | - | F | Update to USIM Table 6.4.1-10 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201337 | 1234 | - | F | Correction to Table 7.3.1-7 NZP-CSI-RS-Resource for TRS | 16.4.0 |
| 2020-06 | RAN#88 | R5-201446 | 1235 | - | F | Fixing wrong reference for RRC_CONNECTED state on WLAN access | 16.4.0 |
| 2020-06 | RAN#88 | R5-201572 | 1253 | - | F | Corrections to default content of DCI messages | 16.4.0 |
| 2020-06 | RAN#88 | RP-201138 | 1258 | 1 | F | Correction to IE SearchSpace | 16.4.0 |
| 2020-06 | RAN#88 | R5-201731 | 1265 | - | F | Addition of NR SUL connection diagram in A.3.1.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201800 | 1267 | - | F | Addition of USIM configuration for TC 6.3.1.8 and TC 6.3.1.9 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201837 | 1268 | - | F | Update of test channel bandwidths for band n48 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201932 | 1271 | - | F | Removing brackets from mid test channel BWs for FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201956 | 1273 | - | F | Correction of clause 4.4.2 on simulated cells | 16.4.0 |
| 2020-06 | RAN#88 | R5-201958 | 1275 | - | F | Correction to Annex C on calculation of kSSB to align SSB and RMSI subcarriers | 16.4.0 |
| 2020-06 | RAN#88 | R5-201959 | 1276 | - | F | Removal of definition of frequencyDomainResources value dependent on CORESET#0 configuration in Annex C. | 16.4.0 |
| 2020-06 | RAN#88 | R5-201961 | 1278 | - | F | Correction of test frequency tables for NR band n1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201962 | 1279 | - | F | Correction of test frequency tables for NR band n2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201963 | 1280 | - | F | Correction of test frequency tables for NR band n3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201964 | 1281 | - | F | Correction of test frequency tables for NR band n5 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201965 | 1282 | - | F | Correction of test frequency tables for NR band n7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201966 | 1283 | - | F | Correction of test frequency tables for NR band n8 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201967 | 1284 | - | F | Correction of test frequency tables for NR band n12 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201968 | 1285 | - | F | Correction of test frequency tables for NR band n20 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201969 | 1286 | - | F | Correction of test frequency tables for NR band n25 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201970 | 1287 | - | F | Correction of test frequency tables for NR band n28 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201971 | 1288 | - | F | Correction of test frequency tables for NR band n34 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201972 | 1289 | - | F | Correction of test frequency tables for NR band n38 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201973 | 1290 | - | F | Correction of test frequency tables for NR band n39 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201974 | 1291 | - | F | Correction of test frequency tables for NR band n40 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201975 | 1292 | - | F | Correction of test frequency tables for NR band n41 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201976 | 1293 | - | F | Correction of test frequency tables for NR band n50 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201977 | 1294 | - | F | Correction of test frequency tables for NR band n51 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201978 | 1295 | - | F | Correction of test frequency tables for NR band n66 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201979 | 1296 | - | F | Correction of test frequency tables for NR band n70 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201980 | 1297 | - | F | Correction of test frequency tables for NR band n71 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201981 | 1298 | - | F | Correction of test frequency tables for NR band n74 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201982 | 1299 | - | F | Correction of test frequency tables for NR band n77 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201983 | 1300 | - | F | Correction of test frequency tables for NR band n78 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201984 | 1301 | - | F | Correction of test frequency tables for NR band n79 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201989 | 1306 | - | F | Corrections of test frequency tables for CA_n41C | 16.4.0 |
| 2020-06 | RAN#88 | R5-201990 | 1307 | - | F | Corrections of test frequency tables for CA_n78C | 16.4.0 |
| 2020-06 | RAN#88 | R5-201991 | 1308 | - | F | Editorial correction to test frequency clause numbering | 16.4.0 |
| 2020-06 | RAN#88 | R5-201995 | 1312 | - | F | Correction of test frequency tables for CA_n66B | 16.4.0 |
| 2020-06 | RAN#88 | R5-201997 | 1314 | - | F | Introduction of test frequencies for NR band n26 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201998 | 1315 | - | F | Introduction of test frequencies for NR band 26 for protocol testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-201999 | 1316 | - | F | Correction of test frequency tables for NR band n29 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202000 | 1317 | - | F | Correction of test frequency tables for NR band n48 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202001 | 1318 | - | F | Correction of test frequency tables for NR band n65 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202024 | 1327 | - | F | Update IE PDCP-Config | 16.4.0 |
| 2020-06 | RAN#88 | R5-202123 | 1330 | - | F | CR to 38.508-1 to clarify the test zone/quiet zone | 16.4.0 |
| 2020-06 | RAN#88 | R5-202186 | 1333 | - | F | Addition of locationAndBandwidth in BWP for FR1 in 38.508-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202210 | 1335 | - | F | Corrections on test frequencies for inter-band EN-DC configurations within FR1 for five bands in 38.508-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202212 | 1336 | - | F | Corrections of test frequency tables for CA_n257x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202213 | 1337 | - | F | Corrections of test frequency tables for CA_n258x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202214 | 1338 | - | F | Corrections of test frequency tables for CA_n260x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202215 | 1339 | - | F | Corrections of test frequency tables for CA_n261x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202244 | 1343 | - | F | Correction to nrofRBs under TRS CSI-FrequencyOccupation for Demod test cases with 10 MHz CBW | 16.4.0 |
| 2020-06 | RAN#88 | R5-202284 | 1351 | - | F | Correction to configuration bwp-id parameter in TCI-State IE | 16.4.0 |
| 2020-06 | RAN#88 | R5-202409 | 1354 | - | F | Update PDCCH-ControlResourceSet for RRM testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202410 | 1355 | - | F | Addition Physical Layer Parameter section for RRM testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202449 | 1356 | - | F | Correction of test frequencies for DC n71AA | 16.4.0 |
| 2020-06 | RAN#88 | R5-202486 | 1358 | - | F | Correction to content of EF5GSN3GPPLOCI | 16.4.0 |
| 2020-06 | RAN#88 | R5-202525 | 1360 | - | F | Correction to System Information Combination for PWS | 16.4.0 |
| 2020-06 | RAN#88 | R5-202531 | 1353 | 1 | F | Addition of R16 new channel bandwidths for n1 in 38.508-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202534 | 1266 | 1 | F | Removal of USIM configuration 14 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202549 | 1350 | 1 | F | Aligning the tabular representation of ASN.1 with PRD13 sections 4.8 and 5 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202554 | 1252 | 1 | F | Updates to PDCCH-ConfigCommon | 16.4.0 |
| 2020-06 | RAN#88 | R5-202561 | 1226 | 1 | F | Update IE default content for control resource set establishment and common search space mapping | 16.4.0 |
| 2020-06 | RAN#88 | R5-202562 | 1229 | 1 | F | Update to 4.9.6.3 Switch off Power off procedure in RRC_CONNECTED | 16.4.0 |
| 2020-06 | RAN#88 | R5-202563 | 1231 | 1 | F | Correction to Table 4.9.7.2.3-1-Tracking Area Update Request | 16.4.0 |
| 2020-06 | RAN#88 | R5-202564 | 1236 | 1 | F | Correction to Table 4.5.2.2-2-Adding second SMC procedure for Selected EPS NAS security algorithms IE | 16.4.0 |
| 2020-06 | RAN#88 | R5-202565 | 1237 | 1 | F | Clarification to ROUND for negative Threshold values in SIB1 and SIB4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202573 | 1251 | 1 | F | Updates to test frequency definitions for SDL NR bands | 16.4.0 |
| 2020-06 | RAN#88 | R5-202574 | 1257 | 1 | F | Correction to condition SRB_NR_PDCP in RadioBearerConfig | 16.4.0 |
| 2020-06 | RAN#88 | R5-202575 | 1260 | 1 | F | Correction to UECapabilityEnquiry in case of EN-DC interband CA | 16.4.0 |
| 2020-06 | RAN#88 | R5-202576 | 1262 | 1 | F | Updates to PDCP-Config | 16.4.0 |
| 2020-06 | RAN#88 | R5-202577 | 1274 | 1 | F | Clarifications of Annex C on calculation of test frequencies | 16.4.0 |
| 2020-06 | RAN#88 | R5-202578 | 1277 | 1 | F | Update SIG test frequencies in clause 6.2.3.x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202579 | 1302 | 1 | F | Correction of test frequency tables for NR band n257 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202580 | 1303 | 1 | F | Correction of test frequency tables for NR band n258 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202581 | 1304 | 1 | F | Correction of test frequency tables for NR band n260 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202582 | 1305 | 1 | F | Correction of test frequency tables for NR band n261 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202583 | 1322 | 1 | F | Introduction of protocol testing applicability for EN-DC inter-band, NR-CA inter-band and NR DC test frequency tables | 16.4.0 |
| 2020-06 | RAN#88 | R5-202585 | 1331 | 1 | F | Correction to Reference QoS rules | 16.4.0 |
| 2020-06 | RAN#88 | R5-202586 | 1346 | 1 | F | Updates to Generic Test Procedure for IMS MT speech call establishment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202587 | 1347 | 1 | F | Updates to Generic Test Procedure for IMS MO call release | 16.4.0 |
| 2020-06 | RAN#88 | R5-202588 | 1349 | 1 | F | Aligning the tabular representation of ASN.1 with PRD13 section 4.6 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202589 | 1359 | 1 | F | Update the default USIM configurations | 16.4.0 |
| 2020-06 | RAN#88 | R5-202590 | 1361 | 1 | F | Addition of Generic procedure to check user plane connectivity for CA tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202703 | 1255 | 1 | F | Clarifications on the QoQZ validation procedure for RRM | 16.4.0 |
| 2020-06 | RAN#88 | R5-202708 | 1254 | 1 | F | TRS - PowerControlOffset correction for UE RF testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202820 | 1352 | 1 | F | Correction to PRB-Id for secondHopPRB | 16.4.0 |
| 2020-06 | RAN#88 | R5-202859 | 1329 | 1 | F | Updates on FR2 inter-band EN-DC configurations for test frequencies in 38.508-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202879 | 1332 | 1 | F | Addition of BW to Table 4.6.3-33 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202880 | 1334 | 1 | F | Clarification of disabling Tx diversity for FR2 UE | 16.4.0 |
| 2020-06 | RAN#88 | R5-202881 | 1341 | 1 | F | Restructuring 38.508-1 message contents for Demod and CSI reporting test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202882 | 1342 | 1 | F | Update of PUCCH-ResourceId for Demod test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202883 | 1348 | 1 | F | Configuration of p-ZP-CSI-RS-ResourceSet for PDSCH Demod test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202956 | 1270 | 1 | F | Update of default test channel BW | 16.4.0 |
| 2020-06 | RAN#88 | R5-202962 | 1269 | 2 | F | Updating DCI related messages | 16.4.0 |
| 2020-06 | RAN#88 | R5-202967 | 1344 | 1 | F | Update to default value of PDSCH-to-HARQ_feedback timing indicator (k1) | 16.4.0 |
| 2020-06 | RAN#88 | R5-203056 | 1313 | 1 | F | Introduction of test frequencies for Rel-16 NR CA configuration CA_n66B and CA_n66(2A) in cl 6.2.3.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-203057 | 1319 | 1 | F | Addition of test frequencies for additional channel bandwidths for NR band n66 | 16.4.0 |
| 2020-06 | RAN#88 | R5-203078 | 1325 | 1 | F | Updates to Generic Test Procedure for IMS MO speech call establishment | 16.4.0 |
| 2020-06 | RAN#88 | R5-203079 | 1326 | 2 | F | Update NR-DC in chapter 4 | 16.4.0 |
| 2020-09 | RAN#89 | R5-203275 | 1364 | - | F | Editorial update IE ARFCN-ValueNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-203277 | 1366 | - | F | Add IEs ARFCN-ValueUTRA-FDD, AvailabilityCombinationsPerCell, AvailabilityIndicator and BAP-Routing-ID | 16.5.0 |
| 2020-09 | RAN#89 | R5-203278 | 1367 | - | F | n26 Default CH BW in 38.508-1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203287 | 1368 | - | F | Correction PRB-Id for PUCCH secondHopPRB | 16.5.0 |
| 2020-09 | RAN#89 | R5-203320 | 1369 | - | F | Add IE BeamFailureRecoverySCellConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203339 | 1372 | - | F | Add IE CGI-InfoEUTRALogging | 16.5.0 |
| 2020-09 | RAN#89 | R5-203340 | 1373 | - | F | Add IEs CGI-Info-Logging and CLI-RSSI-Range | 16.5.0 |
| 2020-09 | RAN#89 | R5-203341 | 1374 | - | F | Add IEs CommonLocationInfo, CondReconfigId, CondReconfigToAddModList and ConditionalReconfiguration | 16.5.0 |
| 2020-09 | RAN#89 | R5-203342 | 1375 | - | F | Add IEs ConfiguredGrantConfigIndex and ConfiguredGrantConfigIndexMAC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203343 | 1376 | - | F | Add IE DRX-ConfigSecondaryGroup | 16.5.0 |
| 2020-09 | RAN#89 | R5-203344 | 1377 | - | F | Add IE HighSpeedConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203345 | 1378 | - | F | Add IE InvalidSymbolPattern | 16.5.0 |
| 2020-09 | RAN#89 | R5-203346 | 1379 | - | F | Add IEs LBT-FailureRecoveryConfig and LocationInfo | 16.5.0 |
| 2020-09 | RAN#89 | R5-203347 | 1380 | - | F | Add IE MeasIdleConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203348 | 1381 | - | F | Add IE MeasObjectCLI | 16.5.0 |
| 2020-09 | RAN#89 | R5-203349 | 1382 | - | F | Add IE MeasObjectNR-SL | 16.5.0 |
| 2020-09 | RAN#89 | R5-203350 | 1383 | - | F | Add IE MeasObjectUTRA-FDD | 16.5.0 |
| 2020-09 | RAN#89 | R5-203351 | 1384 | - | F | Add IEs MeasResultIdleEUTRA and MeasResultIdleNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-203355 | 1387 | - | F | Add IEs MsgA-ConfigCommon and MsgA-PUSCH-Config | 16.5.0 |
| 2020-09 | RAN#89 | R5-203356 | 1388 | - | F | Add IEs NeedForGapsConfigNR and NeedForGapsInfoNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-203357 | 1389 | - | F | Correction to Table 4.5.2.2-2-second SMC procedure for Selected EPS NAS security algorithms IE | 16.5.0 |
| 2020-09 | RAN#89 | R5-203359 | 1391 | - | F | Correction to Table 4.6.3-141 ReportConfigInterRAT | 16.5.0 |
| 2020-09 | RAN#89 | R5-203446 | 1395 | - | F | Add IEs NPN-Identity and NPN-IdentityInfoList | 16.5.0 |
| 2020-09 | RAN#89 | R5-203449 | 1396 | - | F | Add IE PLMN-IdentityList2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203450 | 1397 | - | F | Add IE PUCCH-ConfigurationList | 16.5.0 |
| 2020-09 | RAN#89 | R5-203451 | 1398 | - | F | Add IE PUCCH-SpatialRelationInfo-Id | 16.5.0 |
| 2020-09 | RAN#89 | R5-203455 | 1399 | - | F | Corrections to 4.5.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203456 | 1400 | - | F | Updating usages of TS 34.229-1 to TS 34.229-5 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203467 | 1401 | - | F | Add IE RACH-ConfigCommonTwoStepRA | 16.5.0 |
| 2020-09 | RAN#89 | R5-203470 | 1402 | - | F | Add IE RACH-ConfigGenericTwoStepRA | 16.5.0 |
| 2020-09 | RAN#89 | R5-203471 | 1403 | - | F | Add IE ReferenceTimeInfo | 16.5.0 |
| 2020-09 | RAN#89 | R5-203472 | 1404 | - | F | Add IE RepetitionSchemeConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203476 | 1405 | - | F | Add IE ReportConfigNR-SL | 16.5.0 |
| 2020-09 | RAN#89 | R5-203500 | 1410 | - | F | Update to Table 4.6.3-74: MeasObjectEUTRA | 16.5.0 |
| 2020-09 | RAN#89 | R5-203506 | 1411 | - | F | Add IE RSSI-Range | 16.5.0 |
| 2020-09 | RAN#89 | R5-203507 | 1412 | - | F | Add IEs SemiStaticChannelAccessConfig and Sensor-LocationInfo | 16.5.0 |
| 2020-09 | RAN#89 | R5-203508 | 1413 | - | F | Add IE SI-RequestConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203509 | 1414 | - | F | Add IEs SPS-ConfigIndex, SPS-PUCCH-AN and SPS-PUCCH-AN-List | 16.5.0 |
| 2020-09 | RAN#89 | R5-203510 | 1415 | - | F | Add IE SRS-RSRP-Range | 16.5.0 |
| 2020-09 | RAN#89 | R5-203533 | 1417 | - | F | Update to PDSCH-ServingCellConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203534 | 1418 | - | F | Updates to CellGroupConfig and RNTI-Value for NR-DC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203557 | 1420 | - | F | Add IEs UL-DelayValueConfig and UplinkCancellation | 16.5.0 |
| 2020-09 | RAN#89 | R5-203575 | 1422 | - | F | Add chapter Positioning System information blocks | 16.5.0 |
| 2020-09 | RAN#89 | R5-203577 | 1423 | - | F | Add IEs SIB10, SIB11, SIB12, SIB13 and SIB14 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203582 | 1424 | - | F | Add messages DedicatedSIBRequest, DLDedicatedMessageSegment, DLInformationTransferMRDC and IABOtherInformation | 16.5.0 |
| 2020-09 | RAN#89 | R5-203634 | 1429 | - | F | Introduction of test frequencies for additional Rel-16 EN-DC inter-band configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-203662 | 1432 | - | F | Removal of USIM profile #16 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203671 | 1433 | - | F | Update of PDSCH-to-HARQ_feedback timing indicator (k1) value | 16.5.0 |
| 2020-09 | RAN#89 | R5-203681 | 1436 | - | F | Editorial correction typos in annex C.2.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203704 | 1438 | - | F | Correction to Table 4.6.3-87 NZP-CSI-RS-ResourceSet | 16.5.0 |
| 2020-09 | RAN#89 | R5-203719 | 1441 | - | F | Add messages LoggedMeasurementConfiguration, MCGFailureInformation and SidelinkUEInformationNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-203725 | 1442 | - | F | Add messages UEInformationRequest, UEInformationResponse, ULDedicatedMessageSegment and ULInformationTransferIRAT | 16.5.0 |
| 2020-09 | RAN#89 | R5-203729 | 1443 | - | F | Update IE RACH-ConfigGeneric | 16.5.0 |
| 2020-09 | RAN#89 | R5-203730 | 1444 | - | F | Scheduling Request Resource config for RRM test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203731 | 1445 | - | F | OSI scheduling config for RRM test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203755 | 1447 | - | F | Update IE SchedulingRequestResourceConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203767 | 1449 | - | F | Addition of test frequencies for new Rel-16 CBW for NR band n77 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203768 | 1450 | - | F | Addition of test frequencies for new Rel-16 CBW for NR band n78 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203769 | 1451 | - | F | Introduction of test frequencies for Rel-16 NR band n30 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203793 | 1454 | - | F | Correction of n29 test frequencies for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-203794 | 1455 | - | F | Introduction of n30 test frequencies for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-203796 | 1456 | - | F | Correction of EN-DC test frequency information for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-203813 | 1457 | - | F | Correction to PUCCH-Config | 16.5.0 |
| 2020-09 | RAN#89 | R5-203815 | 1458 | - | F | FR2 PUSCH K2 values alignment to TS 38.214 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203908 | 1463 | - | F | Corrections on test frequencies for NR FR2 CA band n261 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203998 | 1467 | - | F | Addition of test frequencies for new Rel-16 CBW 25 and 50 MHz for NR band n1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204021 | 1469 | - | F | Correction of n51 and n76 test frequencies for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-204032 | 1470 | - | F | Introduction of n259 test frequencies for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-204038 | 1471 | - | F | Corrections of test frequency tables for EN-DC configuration DC_(n)41AA | 16.5.0 |
| 2020-09 | RAN#89 | R5-204039 | 1472 | - | F | Corrections of test frequency tables for EN-DC configuration DC_(n)71AA | 16.5.0 |
| 2020-09 | RAN#89 | R5-204041 | 1473 | - | F | Addition of test channel bandwidths for n1 new CBW in 38.508-1 R16 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204049 | 1475 | - | F | Correction to the procedure for determination of SSB and CORESET0 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204052 | 1478 | - | F | PUCCH Resource ID for CSI TCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204053 | 1479 | - | F | Correction to PDCCH-ConfigCommon for performance tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204150 | 1484 | - | F | Update Table 5.4.2.0-2: ServingCellConfigCommon | 16.5.0 |
| 2020-09 | RAN#89 | R5-204167 | 1488 | - | F | Corrections of test frequency tables for CA_n260(A-I) | 16.5.0 |
| 2020-09 | RAN#89 | R5-204168 | 1489 | - | F | Update Table 7.3.1-4: ServingCellConfigCommon | 16.5.0 |
| 2020-09 | RAN#89 | R5-204223 | 1491 | - | F | Update missing SMTC configurations in RRM message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204238 | 1493 | - | F | Correction to default contents of RRCReestablishmentRequest message | 16.5.0 |
| 2020-09 | RAN#89 | R5-204325 | 1495 | - | F | Adding procedure for establishment of multiple additional PDN connections in EPS (S1 mode) | 16.5.0 |
| 2020-09 | RAN#89 | R5-204327 | 1497 | - | F | Updates to Test procedure for UE for Tracking area updating / Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode | 16.5.0 |
| 2020-09 | RAN#89 | R5-204329 | 1499 | - | F | Update of 4.5A.2 UE-requested PDU session establishment procedure | 16.5.0 |
| 2020-09 | RAN#89 | R5-204330 | 1500 | - | F | Updates Procedure to UE-requested PDU session modification after the first S1 to N1 mode change | 16.5.0 |
| 2020-09 | RAN#89 | R5-204331 | 1501 | - | F | Void 4.9.14 Procedure for UE-requested PDU session modification after the first S1 to N1 mode change | 16.5.0 |
| 2020-09 | RAN#89 | R5-204346 | 1502 | - | F | Introduction of test frequencies for Rel-16 NR band n259 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204378 | 1427 | 1 | F | Correction to NR inter-band CA configurations in FR1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204384 | 1409 | 1 | F | Correction to test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 16.5.0 |
| 2020-09 | RAN#89 | R5-204386 | 1390 | 1 | F | Correction to Table 4.6.1-17 RRCResume | 16.5.0 |
| 2020-09 | RAN#89 | R5-204387 | 1406 | 1 | F | Corrections to generic procedures for MO and MT speech call establishment | 16.5.0 |
| 2020-09 | RAN#89 | R5-204388 | 1407 | 1 | F | Correction to USIM configurations 7 and 13 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204389 | 1408 | 1 | F | Correction to switch off / power off procedures for IMS | 16.5.0 |
| 2020-09 | RAN#89 | R5-204390 | 1421 | 1 | F | Correction of description of NGEN-DC in table 4.5.1-1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204391 | 1425 | 1 | F | Correction to UE-CapabilityRAT-RequestList and UE-CapabilityRequestFilterNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-204392 | 1439 | 1 | F | Addition of Generic Test Procedure for IMS MO SMS in 5GC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204393 | 1440 | 1 | F | Addition of Generic Test Procedure for IMS MT SMS in 5GC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204394 | 1446 | 1 | F | Update IE RLC-BearerConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-204395 | 1481 | 1 | F | Update IE SIB2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204396 | 1482 | 1 | F | New procedure for PDU Session Release | 16.5.0 |
| 2020-09 | RAN#89 | R5-204397 | 1483 | 1 | F | Update to FreqBandList | 16.5.0 |
| 2020-09 | RAN#89 | R5-204398 | 1490 | 1 | F | Update IE ServingCellConfigCommon | 16.5.0 |
| 2020-09 | RAN#89 | R5-204399 | 1494 | 1 | F | Adding generic procedure E-UTRA RRC_IDLE with unrestricted number of PDN connections | 16.5.0 |
| 2020-09 | RAN#89 | R5-204400 | 1496 | 1 | F | Update of PDU SESSION ESTABLISHMENT ACCEPT for multi PDU-PDN handling | 16.5.0 |
| 2020-09 | RAN#89 | R5-204401 | 1498 | 1 | F | Updates of Test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 16.5.0 |
| 2020-09 | RAN#89 | R5-204507 | 1371 | 1 | F | Add IEs BH-RLC-ChannelConfig, BH-LogicalChannelIdentity, BH-LogicalChannelIdentity-Ext and BH-RLC-ChannelID | 16.5.0 |
| 2020-09 | RAN#89 | R5-204508 | 1386 | 1 | F | Add IEs MeasResultsSL and MeasTriggerQuantityEUTRA | 16.5.0 |
| 2020-09 | RAN#89 | R5-204509 | 1416 | 1 | F | Modification to InterRAT-Parameters to add the UE capability nr-HO-ToEN-DC-r16 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204510 | 1419 | 1 | F | Add IE SSB-PositionQCL-Relation | 16.5.0 |
| 2020-09 | RAN#89 | R5-204704 | 1503 | - | F | Adding the test frequency for DC_3A-7A_n78A | 16.5.0 |
| 2020-09 | RAN#89 | R5-204706 | 1504 | - | F | Adding the test frequency for DC_28A_n3A | 16.5.0 |
| 2020-09 | RAN#89 | R5-204708 | 1435 | 1 | F | Updating indicator for SUL FR1 test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204709 | 1437 | 1 | F | Update frequencyDomainResources and nrofcandidates | 16.5.0 |
| 2020-09 | RAN#89 | R5-204751 | 1428 | 1 | F | Introduction of test frequencies for additional Rel-15 band EN-DC inter-band configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-204752 | 1460 | 1 | F | Correction to TCI-state related configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-204753 | 1461 | 1 | F | Correction to CSI-RS related configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-204754 | 1468 | 1 | F | Update on test frequencies for EN-DC configurations including FR2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204755 | 1485 | 1 | F | Corrections of test frequency tables for CA_n258x | 16.5.0 |
| 2020-09 | RAN#89 | R5-204756 | 1486 | 1 | F | Corrections of test frequency tables for CA_n260x | 16.5.0 |
| 2020-09 | RAN#89 | R5-204757 | 1487 | 1 | F | Corrections of test frequency tables for CA_n261x | 16.5.0 |
| 2020-09 | RAN#89 | R5-204758 | 1492 | 1 | F | Add SSB subcarrier spacing configurations in RRM message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204800 | 1362 | 1 | F | Introduction of test frequencies for Rel-16 inter-band EN-DC combinations within FR1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204852 | 1434 | 1 | F | Jumbo CR for update to Demod message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204896 | 1370 | 1 | F | Addition of test frequencies for n28 with CBW of 30MHz | 16.5.0 |
| 2020-09 | RAN#89 | R5-204899 | 1448 | 1 | F | Adding default value for IE rbg-Size for demodulation and CSI reporting tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204900 | 1462 | 1 | F | Introduction of definition of Mid, Low, High test channel bandwidth and removal of NOTEs that incorrectly permit UE not to support mandatory BWs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204901 | 1476 | 1 | F | Correction to message configuration for performance tests | 16.5.0 |
| 2020-10 | RAN#89 | R5-204325 | 1495 | - | F | Addition of missing Table 4.5A.2B.2.2-2 and specific message contents of R5-204325 | 16.5.1 |