3GPP TSG-RAN WG5 Meeting #90-e	R5-210223

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0109 | rev | - | Current version: | 14.9.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MCPTT Test Case 5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Structure and content of specific message content incorrect - PIDF structure and content of mcptt extension - TS 24.379 clause 9.2.2: p-id in NOTIFY only present if induced by PUBLISH and reflected - TS 24.379 cl. 9.2.2.2.8 calling user id in mcptt info of SIP MESSAGE in step 39 missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Structure and content of specific message content - PIDF structure and content corrected – split of tables into UL and DL - P-id correctly set - Calling user id in mcptt info of SIP MESSAGE added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 5.3 will remain incorrectly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0107 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

## 5.3	Configuration / Group Affiliation / Remote change / De-affiliation / Home MCPTT system

5.3.1	Test Purpose (TP)

(1)

with { MCPTT Client already provisioned with the group information or a pointer to the group information, that the MCPTT Client is allowed to be affiliated }

ensure that {
  when { MCPTT User requests for current affiliation status and to subscribe to affiliation status changes for the MCPTT User }

then { UE (MCPTT Client) requests to subscribe to affiliation status changes for the MCPTT User by sending the SS a SIP SUBSCRIBE message and starts informing the MCPTT User of any affiliation status changes for the MCPTT User after the subscription is accepted }

}

(2)

with { MCPTT Client already provisioned with the group information or a pointer to the group information, that the MCPTT Client is allowed to be affiliated }

ensure that {
  when { MCPTT User requests to affiliate to an MCPTT group }

then { UE (MCPTT Client) requests to affiliate to an MCPTT group by sending the SS a SIP PUBLISH message }

}

(3)

with { MCPTT Client already provisioned with the group information or a pointer to the group information, that the MCPTT Client is allowed to be affiliated }

ensure that {
  when { MCPTT User requests for current affiliation status and to subscribe to affiliation status changes for a target user }

then { UE (MCPTT Client) requests to subscribe to affiliation status changes for the target user by sending the SS a SIP SUBSCRIBE message and starts informing the MCPTT User of any affiliation status changes for the target user after the subscription is accepted }

}

(4)

with { MCPTT client already provisioned with the group information or a pointer to the group information that the MCPTT client is allowed to make affiliation changes for another user }

ensure that {
  when { MCPTT User requests that a target user be affiliated to an MCPTT group via mandatory mode }

then { UE (MCPTT Client) requests that a target user be affiliated to an MCPTT group via mandatory mode by sending the SS a SIP PUBLISH message }

}

(5)

with { MCPTT client already provisioned with the group information or a pointer to the group information that the MCPTT client is allowed to make affiliation changes for another user }

ensure that {
  when { MCPTT User requests that a target user be de-affiliated to an MCPTT group via mandatory mode }

then { UE (MCPTT Client) requests that a target user be de-affiliated to an MCPTT group via mandatory mode by sending the SS a SIP PUBLISH message }

}

(6)

with { MCPTT client already provisioned with the group information or a pointer to the group information that the MCPTT client is allowed to make affiliation changes for another user }

ensure that {
  when { MCPTT User requests that a target user be affiliated to an MCPTT group via negotiated mode }

then { UE (MCPTT Client) requests that a target user be affiliated to an MCPTT group via negotiated mode by sending the SS a SIP MESSAGE message }

}

(7)

with { MCPTT Client already provisioned with the group information or a pointer to the group information, that the MCPTT Client is allowed to be affiliated }

ensure that {
  when { MCPTT User requests to de-subscribe to affiliation status changes for a target user }

then { UE (MCPTT Client) requests to de-subscribe to affiliation status changes for a target user by sending the SS a SIP SUBSCRIBE message }

}

(8)

with { MCPTT Client already affiliated with an MCPTT group }

ensure that {
  when { MCPTT User requests to de-affiliate from an MCPTT group }

then { UE (MCPTT Client) requests to de-affiliate from an MCPTT group by sending the SS a SIP PUBLISH message }

}

(9)

with { MCPTT Client already provisioned with the group information or a pointer to the group information, that the MCPTT Client is allowed to be affiliated }

ensure that {
  when { MCPTT Server requests that the MCPTT User choose to affiliate to an MCPTT group via negotiated mode by sending a SIP MESSAGE message }

then { UE (MCPTT Client) accepts to affiliate to an MCPTT group by sending the SS a SIP PUBLISH message }

}

5.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.379, clauses 9.2.1.2, 9.2.1.3, 9.2.1.4, and 9.2.1.5. Unless otherwise stated, these are Rel-13 requirements.

[TS 24.379, clause 9.2.1.2]

In order:

-	to indicate that an MCPTT user is interested in one or more MCPTT group(s) at an MCPTT client;

-	to indicate that the MCPTT user is no longer interested in one or more MCPTT group(s) at the MCPTT client;

-	to refresh indication of an MCPTT user interest in one or more MCPTT group(s) at an MCPTT client due to near expiration of the expiration time of an MCPTT group with the affiliation status set to the "affiliated" state received in a SIP NOTIFY request in subclause 9.2.1.3;

-	to send an affiliation status change request in mandatory mode to another MCPTT user; or

-	any combination of the above;

the MCPTT client shall generate a SIP PUBLISH request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37], and IETF RFC 3856 [51].

In the SIP PUBLISH request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT ID of the MCPTT user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

4)	if the targeted MCPTT user is interested in at least one MCPTT group at the targeted MCPTT client, shall set the Expires header field according to IETF RFC 3903 [37], to 4294967295;

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	if the targeted MCPTT user is no longer interested in any MCPTT group at the targeted MCPTT client, shall set the Expires header field according to IETF RFC 3903 [37], to zero; and

6)	shall include an application/pidf+xml MIME body indicating per-user affiliation information according to subclause 9.3.1. In the MIME body, the MCPTT client:

a)	shall include all MCPTT groups where the targeted MCPTT user indicates its interest at the targeted MCPTT client;

b)	shall include the MCPTT client ID of the targeted MCPTT client;

c)	shall not include the "status" attribute and the "expires" attribute in the <affiliation> element; and

d)	shall set the <p-id> child element of the <presence> root element to a globally unique value.

The MCPTT client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [4].

[TS 24.379, clause 9.2.1.3]

NOTE 1:	The MCPTT UE also uses this procedure to determine which MCPTT groups the MCPTT user successfully affiliated to.

In order to discover MCPTT groups:

1)	which the MCPTT user at an MCPTT client is affiliated to; or

2)	which another MCPTT user is affiliated to;

the MCPTT client shall generate an initial SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [27].

In the SIP SUBSCRIBE request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT ID of the targeted MCPTT user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

4)	if the MCPTT client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	if the MCPTT client wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [26], to zero; and

6)	shall include an Accept header field containing the application/pidf+xml MIME type; and

7)	if requesting MCPTT groups where the MCPTT user is affiliated to at the MCPTT client, shall include an application/simple-filter+xml MIME body indicating per client restrictions of presence event package notification information according to subclause 9.3.2.

In order to re-subscribe or de-subscribe, the MCPTT client shall generate an in-dialog SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26]. In the SIP SUBSCRIBE request, the MCPTT client:

1)	if the MCPTT client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295;

NOTE 3:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

2)	if the MCPTT client wants to de-subscribe, shall set the Expires header field according to IETF RFC 6665 [26], to zero; and

3)	shall include an Accept header field containing the application/pidf+xml MIME type.

Upon receiving a SIP NOTIFY request according to 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26], if SIP NOTIFY request contains an application/pidf+xml MIME body indicating per-user affiliation information constructed according to subclause 9.3.1, then the MCPTT client shall determine affiliation status of the MCPTT user for each MCPTT group at the MCPTT client(s) in the MIME body. If the <p-id> child element of the <presence> root element of the application/pidf+xml MIME body of the SIP NOTIFY request is included, the <p-id> element value indicates the SIP PUBLISH request which triggered sending of the SIP NOTIFY request.

[TS 24.379, clause 9.2.1.4]

NOTE:	Procedure for sending affiliation status change request in negotiated mode to several target MCPTT users is not supported in this version of the specification.

Upon receiving a request from the MCPTT user to send an affiliation status change request in negotiated mode to a target MCPTT user, the MCPTT client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]. In the SIP MESSAGE request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT ID of the target MCPTT user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP MESSAGE request;

4)	shall include an application/vnd.3gpp.mcptt-affiliation-command+xml MIME body as specified in Annex F.4; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCPTT client shall indicate to the user that the request has been delivered to an MCPTT client of the target MCPTT user.

[TS 24.379, clause 9.2.1.5]

Upon receiving a SIP MESSAGE request containing:

1)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9]; and

2)	an application/vnd.3gpp.mcptt-affiliation-command+xml MIME body with a list of MCPTT groups for affiliation under the <affiliate> element and a list of MCPTT groups for de-affiliation under the <de-affiliate> element;

then the MCPTT client:

1)	shall send a 200 (OK) response to the SIP MESSAGE request;

2)	shall seek confirmation of the list of MCPTT groups for affiliation and the list of MCPTT groups for de-affiliation, resulting in an accepted list of MCPTT groups for affiliation and an accepted list of MCPTT groups for de-affiliation; and

3)	if the user accepts the request:

a)	shall perform affiliation for each entry in the accepted list of MCPTT groups for affiliation for which the MCPTT client is not affiliated, as specified in subclause 9.2.1.2; and

b)	shall perform de-affiliation for each entry in the accepted list of MCPTT groups for de-affiliation for which the MCPTT client is affiliated, as specified in subclause 9.2.1.2.

5.3.3	Test description

5.3.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	UE States at the end of the preamble:

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

5.3.3.2	Test procedure sequence

Table 5.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCPTT User send a request to discover which groups the MCPTT User is affiliated to and to subscribe to affiliation status changes for the MCPTT User. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP SUBSCRIBE message to subscribe to its own affiliation status changes? | --> | SIP SUBSCRIBE | 1 | P |
| 3 | The SS responds to the SIP SUBSCRIBE message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 4 | The SS sends a SIP NOTIFY informing that the affiliation status of the MCPTT USER with GROUP A is “affiliated”, but no other group is "affiliated" | <-- | SIP NOTIFY | - | - |
| 4A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 4B | Check: Does the UE (MCPTT client) notify the user that the MCPTT User is affiliated with GROUP A? (NOTE 1) | - | - | 1 | P |
| 4C | Make the MCPTT User send a request to de-affiliate from an MCPTT group, GROUP A. (NOTE 1) | - | - | - | - |
| 4D | Check: Does the UE (MCPTT Client) send a SIP PUBLISH message to de-affiliate with GROUP A? | --> | SIP PUBLISH | 8 | P |
| 4E | The SS responds to the SIP PUBLISH message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 4F | The SS sends a SIP NOTIFY informing that the affiliation status of the user with GROUP A is “de-affiliating” | <-- | SIP NOTIFY | - | - |
| 4G | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 4H | The SS sends a SIP NOTIFY informing that the affiliation status of the user is not affiliated to any group | <-- | SIP NOTIFY | - | - |
| 4I | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 4J | Check: Does the UE (MCPTT client) notify the user that the MCPTT User is no longer affiliated with GROUP A? (NOTE 1) | - | - | 1 | P |
| 5 | Make the MCPTT User send a request to affiliate to an MCPTT group, GROUP A. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCPTT Client) send a SIP PUBLISH message to affiliate with GROUP A? | --> | SIP PUBLISH | 2 | P |
| 7 | The SS responds to the SIP PUBLISH message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 8 | The SS sends a SIP NOTIFY informing that the affiliation status of the MCPTT USER with GROUP A is “affiliating” | <-- | SIP NOTIFY | - | - |
| 8A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 9 | The SS sends a SIP NOTIFY informing that the affiliation status of the MCPTT USER with GROUP A is “affiliated” | <-- | SIP NOTIFY | - | - |
| 9A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 10 | Check: Does the UE (MCPTT client) notify the user that the MCPTT User is now affiliated with GROUP A? (NOTE 1) | - | - | 1 | P |
| 11 | Make the MCPTT User send a request to discover which groups a target user is affiliated to and to subscribe to affiliation status changes for that target user. (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCPTT Client) send a SIP SUBSCRIBE message to subscribe to affiliation status changes of a target user? | --> | SIP SUBSCRIBE | 3 | P |
| 13 | The SS responds to the SIP SUBSCRIBE message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 13A | The SS sends a SIP NOTIFY informing that the affiliation status of the user is not affiliated to any group | <-- | SIP NOTIFY | - | - |
| 13B | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 14 | Make the MCPTT User send a request to have a target user affiliate to an MCPTT group, GROUP A (mandatory mode). (NOTE 1) | - | - | - | - |
| 15 | Check: Does the UE (MCPTT Client) send a SIP PUBLISH message to have the target user affiliate with GROUP A? | --> | SIP PUBLISH | 4 | P |
| 16 | The SS responds to the SIP PUBLISH message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 17 | The SS sends a SIP NOTIFY informing that the affiliation status of the target user with GROUP A is “affiliating” | <-- | SIP NOTIFY | - | - |
| 17A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 18 | The SS sends a SIP NOTIFY informing that the affiliation status of the target user with GROUP A is “affiliated” | <-- | SIP NOTIFY | - | - |
| 18A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 19 | Check: Does the UE (MCPTT client) notify the user that the target user is now affiliated with GROUP A? (NOTE 1) | - | - | 3 | P |
| 20 | Make the MCPTT User send a request to have a target user de-affiliate to an MCPTT group, GROUP A (mandatory mode). (NOTE 1) | - | - | - | - |
| 21 | Check: Does the UE (MCPTT Client) send a SIP PUBLISH message to have the target user de-affiliate with GROUP A? | --> | SIP PUBLISH | 5 | P |
| 22 | The SS responds to the SIP PUBLISH message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 23 | The SS sends a SIP NOTIFY informing that the affiliation status of the target user with GROUP A is “de-affiliating” | <-- | SIP NOTIFY | - | - |
| 23A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 23B | The SS sends a SIP NOTIFY informing that the affiliation status of the target user is not affiliated to any group | <-- | SIP NOTIFY | - | - |
| 23C | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 24 | Check: Does the UE (MCPTT client) notify the user that the target user is now de-affiliated with GROUP A? (NOTE 1) | - | - | 3 | P |
| 25 | Make the MCPTT User send a request to have a target user affiliate to an MCPTT group, GROUP A (negotiated mode). (NOTE 1) | - | - | - | - |
| 26 | Check: Does the UE (MCPTT Client) send a SIP MESSAGE message to have a target user affiliate with GROUP A via negotiated mode? | --> | SIP MESSAGE | 6 | P |
| 27 | The SS responds to the SIP MESSAGE message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 28 | The SS sends a SIP NOTIFY informing that the affiliation status of the target user with GROUP A is “affiliating” | <-- | SIP NOTIFY | - | - |
| 28A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 29 | The SS sends a SIP NOTIFY informing that the affiliation status of the target user with GROUP A is “affiliated” | <-- | SIP NOTIFY | - | - |
| 29A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 30 | Check: Does the UE (MCPTT client) notify the user that the target user is now affiliated with GROUP A? (NOTE 1) | - | - | 3 | P |
| 31 | Make the MCPTT User send a request to de-subscribe from affiliation status changes of a target user. (NOTE 1) | - | - | - | - |
| 32 | Check: Does the UE (MCPTT Client) send a SIP SUBSCRIBE message with the Expires header field set to zero to de-subscribe from affiliation status changes for a target user? | --> | SIP SUBSCRIBE | 7 | P |
| 33 | The SS responds to the SIP SUBSCRIBE message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 34 | Make the MCPTT User send a request to de-affiliate from an MCPTT group, GROUP A. (NOTE 1) | - | - | - | - |
| 35 | Check: Does the UE (MCPTT Client) send a SIP PUBLISH message to de-affiliate with GROUP A? | --> | SIP PUBLISH | 8 | P |
| 36 | The SS responds to the SIP PUBLISH message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 37 | The SS sends a SIP NOTIFY informing that the affiliation status of the user with GROUP A is “de-affiliating” | <-- | SIP NOTIFY | - | - |
| 37A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 37B | The SS sends a SIP NOTIFY informing that the affiliation status of the user is not affiliated to any group” | <-- | SIP NOTIFY | - | - |
| 37C | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 38 | Check: Does the UE (MCPTT client) notify the user that the MCPTT User is no longer affiliated with GROUP A? (NOTE 1) | - | - | 1 | P |
| 39 | The SS sends a SIP MESSAGE to the MCPTT Client to affiliate the MCPTT User to GROUP A via negotiated mode | <-- | SIP MESSAGE | - | - |
| 40 | Check: Does the UE (MCPTT Client) send a SIP 200 (OK) message in response to the SIP MESSAGE message? | --> | SIP 200 (OK | 9 | P |
| 41 | Check: Does the UE (MCPTT client) notify the user that another user is requesting the MCPTT User affiliate to GROUP A? (NOTE 1) | - | - | 9 | P |
| 42 | Make the MCPTT User accept to affiliate to GROUP A. (NOTE 1) | - | - | - | - |
| 43 | Check: Does the UE (MCPTT Client) send a SIP PUBLISH message to affiliate with GROUP A? | --> | SIP PUBLISH | 9 | P |
| 44 | The SS responds to the SIP PUBLISH message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 45 | The SS sends a SIP NOTIFY informing that the affiliation status of the MCPTT USER with GROUP A is “affiliating” | <-- | SIP NOTIFY | - | - |
| 45A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 46 | The SS sends a SIP NOTIFY informing that the affiliation status of the MCPTT USER with GROUP A is “affiliated” | <-- | SIP NOTIFY | - | - |
| 46A | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 47 | Check: Does the UE (MCPTT client) notify the user that the MCPTT User is now affiliated with GROUP A? (NOTE 1) | - | - | 1 | P |
| - | EXCEPTION: SS releases the E-UTRA connection. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3.3.3	Specific message contents

Table 5.3.3.3-1: SIP SUBSCRIBE (step 2, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "4294967295" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.3.3-2 |  |  |  |

Table 5.3.3.3-2: MCPTT-INFO in SIP SUBSCRIBE (Table 5.3.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  |  |  |

Table 5.3.3.3-3: SIP SUBSCRIBE (step 12, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "4294967295" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.3.3-4 |  |  |  |

Table 5.3.3.3-4: MCPTT-INFO in SIP SUBSCRIBE (Table 5.3.3.3-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_B |  |  |  |

Table 5.3.3.3-5: Void

Table 5.3.3.3-6: SIP SUBSCRIBE (step 32, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.14-1 condition re-SUBSCRIBE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "0" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.3.3-7 |  |  |  |

Table 5.3.3.3-7: MCPTT-INFO in SIP SUBSCRIBE (Table 5.3.3.3-6)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_B |  |  |  |

Table 5.3.3.3-8: Void

Table 5.3.3.3-9: SIP 200 (OK) (steps 3, 4G, 4I, 7, 13, 16, 27, 44, 23C Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "4294967295" |  |  |  |

Table 5.3.3.3-10: SIP 200 (OK) (steps 4E, 22, 33, 36, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "0" |  |  |  |

Table 5.3.3.3-11: SIP 200 (OK) (step 40, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "4294967295" |  |  |  |

Table 5.3.3.3-12: SIP PUBLISH (steps 6, 43, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.11-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.3.3-13 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-14 |  |  |  |

Table 5.3.3.3-13: MCPTT-INFO in SIP PUBLISH (Table 5.3.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  |  |  |

Table 5.3.3.3-14: PIDF in SIP PUBLISH (Table 5.3.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| ..p-id | any allowed value | p-id shall be present acc. to TS 24.379 [9] clause 9.2.2.2.5 and clause 9.2.2.3.5 |  |  |

Table 5.3.3.3-15: SIP PUBLISH (step 15, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.11-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.3.3-16 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-17 |  |  |  |

Table 5.3.3.3-16: MCPTT-INFO in SIP PUBLISH (Table 5.3.3.3-15)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_B |  |  |  |

Table 5.3.3.3-17: PIDF in SIP PUBLISH (Table 5.3.3.3-15)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| Entity attribute | px_MCPTT_ID_User_B |  |  |  |
| tuple |  |  |  |  |
| Id attribute | px_MCPTT_Client_B_ID |  |  |  |
| ..p-id | any allowed value | p-id shall be present acc. to TS 24.379 [9] clause 9.2.2.2.5 and clause 9.2.2.3.5 |  |  |

Table 5.3.3.3-18: SIP PUBLISH (step 21, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.11-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "0" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.3.3-19 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-20 |  |  |  |

Table 5.3.3.3-19: MCPTT-INFO in SIP PUBLISH (Table 5.3.3.3-18)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_B |  |  |  |

Table 5.3.3.3-20: PIDF in SIP PUBLISH (Table 5.3.3.3-18)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| entity attribute | px_MCPTT_ID_User_B |  |  |  |
| tuple |  |  |  |  |
| Id attribute | px_MCPTT_Client_B_ID |  |  |  |
| status | not present |  |  |  |
| ..p-id | any allowed value | p-id shall be present acc. to TS 24.379 [9] clause 9.2.2.2.5 and clause 9.2.2.3.5 |  |  |

Table 5.3.3.3-21: SIP PUBLISH (step 4D, 35, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.11-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "0" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.3.3-22 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-23 |  |  |  |

Table 5.3.3.3-22: MCPTT-INFO in SIP PUBLISH (Table 5.3.3.3-21)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  |  |  |

Table 5.3.3.3-23: PIDF in SIP PUBLISH (Table 5.3.3.3-21)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status | not present |  |  |  |
| ..p-id | any allowed value | p-id shall be present acc. to TS 24.379 [9] clause 9.2.2.2.5 and clause 9.2.2.3.5 |  |  |

Table 5.3.3.3-24: SIP NOTIFY (step 4, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-25 |  |  |  |

Table 5.3.3.3-25: PIDF in SIP NOTIFY (Table 5.3.3.3-24)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “affiliated” |  |  |  |

Table 5.3.3.3-26: SIP NOTIFY (steps 8, 45, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-27 |  |  |  |

Table 5.3.3.3-27: PIDF in SIP NOTIFY (Table 5.3.3.3-26)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “affiliating” |  |  |  |
| ..p-id | same value as received in SIP PUBLISH in step 4D |  |  |  |

Table 5.3.3.3-28: SIP NOTIFY (steps 9, 46, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-29 |  |  |  |

Table 5.3.3.3-29: PIDF in SIP NOTIFY (Table 5.3.3.3-28)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “affiliated” |  |  |  |
| ..p-id | same value as received in SIP PUBLISH in step 4D |  |  |  |

Table 5.3.3.3-30: SIP NOTIFY (step 17, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-31 |  |  |  |

Table 5.3.3.3-31: PIDF in SIP NOTIFY (Table 5.3.3.3-30)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| entity attribute | px_MCPTT_ID_User_B |  |  |  |
| tuple |  |  |  |  |
| Id attribute | px_MCPTT_Client_B_ID |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “affiliating” |  |  |  |
| ..p-id | same value as received in SIP PUBLISH in step 15 |  |  |  |

Table 5.3.3.3-31A: SIP NOTIFY (step 28, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-31B |  |  |  |

Table 5.3.3.3-31B: PIDF in SIP NOTIFY (Table 5.3.3.3-31A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| entity attribute | px_MCPTT_ID_User_B |  |  |  |
| tuple |  |  |  |  |
| Id attribute | px_MCPTT_Client_B_ID |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “affiliating” |  |  |  |

Table 5.3.3.3-32: SIP NOTIFY (step 18, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-33 |  |  |  |

Table 5.3.3.3-33: PIDF in SIP NOTIFY (Table 5.3.3.3-32)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence entity |  |  |  |  |
| entity attribute | px_MCPTT_ID_User_B |  |  |  |
| tuple id |  |  |  |  |
| Id attribute | px_MCPTT_Client_B_ID |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “affiliated” |  |  |  |
| ..p-id | same value as received in SIP PUBLISH in step 15 |  |  |  |

Table 5.3.3.3-33A: SIP NOTIFY (step 29, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-33B |  |  |  |

Table 5.3.3.3-33B: PIDF in SIP NOTIFY (Table 5.3.3.3-33A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| entity attribute | px_MCPTT_ID_User_B |  |  |  |
| tuple |  |  |  |  |
| Id attribute | px_MCPTT_Client_B_ID |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “affiliated” |  |  |  |

Table 5.3.3.3-34: SIP NOTIFY (step 23, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-35 |  |  |  |

Table 5.3.3.3-35: PIDF in SIP NOTIFY (Table 5.3.3.3-34)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| entity attribute | px_MCPTT_ID_User_B |  |  |  |
| tuple |  |  |  |  |
| Id attribute | px_MCPTT_Client_B_ID |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “deaffiliating” |  |  |  |
| ..p-id | same value as received in SIP PUBLISH in step 21 |  |  |  |

Table 5.3.3.3-36: SIP NOTIFY (step 37, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-37 |  |  |  |

Table 5.3.3.3-37: PIDF in SIP NOTIFY (Table 5.3.3.3-36)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “deaffiliating” |  |  |  |
| ..p-id | same value as received in SIP PUBLISH in step 35 |  |  |  |

Table 5.3.3.3-38: SIP MESSAGE (step 26, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 with condition AFFILIATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.3.3-39 |  |  |  |

Table 5.3.3.3-39: MCPTT-INFO in SIP MESSAGE (Table 5.3.3.3-38)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_B |  |  |  |

Table 5.3.3.3-40: Void

Table 5.3.3.3-41: SIP MESSAGE (step 39, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 with condition AFFILIATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.3.3-42 |  |  |  |

Table 5.3.3.3-42: MCPTT-INFO in SIP MESSAGE (Table 5.3.3.3-41)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |

Table 5.3.3.3-43: Void

Table 5.3.3.3-44: SIP NOTIFY (step 13A, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-45 |  |  |  |

Table 5.3.3.3-45: PIDF in SIP NOTIFY (Table 5.3.3.3-44)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status | not present |  |  |  |

Table 5.3.3.3-46: SIP NOTIFY (step 4H, 37B, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-46A |  |  |  |

Table 5.3.3.3-46A: PIDF in SIP NOTIFY (Table 5.3.3.3-46)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple id |  |  |  |  |
| status | not present |  |  |  |
| ..p-id | same value as received in the corresponding SIP PUBLISH |  |  |  |

Table 5.3.3.3-46B: SIP NOTIFY (step 23B, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-46C |  |  |  |

Table 5.3.3.3-46C: PIDF in SIP NOTIFY (Table 5.3.3.3-46B)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| entity attribute | px_MCPTT_ID_User_B |  |  |  |
| tuple id |  |  |  |  |
| Id attribute | px_MCPTT_Client_B_ID |  |  |  |
| status | not present |  |  |  |
| ..p-id | same value as received in the SIP PUBLISH in step 21 |  |  |  |

Table 5.3.3.3-47: SIP NOTIFY (step 4F, Table 5.3.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 5.3.3.3-48 |  |  |  |

Table 5.3.3.3-48: PIDF in SIP NOTIFY (Table 5.3.3.3-47)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status |  |  |  |  |
| affiliation |  |  |  |  |
| status | “deaffiliating” |  |  |  |
| ..p-id | same value as received in SIP PUBLISH in step 4D |  |  |  |

##### <End of modification>