3GPP TSG-RAN5 Meeting #90-e	R5-210293

# Electronic Meeting, 22nd Feb – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0452 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New FR2 Common Uplink Configurations definition in section 6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 6.4.2.3, the following RB allocacations are required in table 6.4.2.3.4.1-1: - Inner_4RB_Left for PC2, PC3, PC4 - Inner_16RB_Left_Region2 for PC1 - Inner_4RB_Right for PC2, PC3, PC4 - Inner_16RB_Right_Region2 for PC1 According to Note 1 in that table:The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. However, the definition of the required UL RB allocations is missing in those tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Update Table 6.1-1 with Inner_4RB_Left and Inner_4RB_Right configurations. -Update Table 6.1-2 with Inner_16RB_Left_Region2 and Inner_16RB_Right_Region2 configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

# 6	Transmitter characteristics

## 6.1	General

Editor’s Note: Test configurations/environments that require new spherical scan shall be included in test procedure section and identifying such scenarios is currently FFS and owned by RAN5.

Unless otherwise stated, the transmitter characteristics are specified over the air (OTA) with a single or multiple transmit chains.

Unless otherwise stated, for power class 3 UEs, the beam correspondence side condition for SSB and CSI-RS specified in subclause 6.6.4 shall apply to the transmission tests.

Transmitter requirements for CA operation apply only when the DMRS initialization parameters (including the case when the UE applies cell ID as DMRS scrambling ID) are different across all CCs. The UE may use higher MPR values outside this limitation.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

For Tx test cases the identified beam peak direction can be stored and reused for a device under test in various configurations/environments for the full duration of device testing as long as beam peak direction is the same.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

Uplink RB allocations given in Table 6.1-1 are used throughout this section, unless otherwise stated by the test case.

The UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

Table 6.1-1: Common Uplink Configuration PC2, for PC3 and PC4

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Outer_Full | Outer_1RB_Left | Outer_1RB_Right | Inner_Full (Note 1) | Inner_1RB_Left | Inner_1RB_Right | Inner_4RB_Left | Inner_4RB_Right |
| 50MHz |  | 60 |  | DFT-s |  | 64@0 | 1@0 | 1@65 | 20@22 | 1@22 | 1@43 | 4@22 | 4@40 |
|  |  |  |  | CP |  | 66@0 | 1@0 | 1@65 | 22@22 | 1@22 | 1@43 | 4@22 | 4@40 |
|  |  | 120 |  | DFT-s |  | 32@0 | 1@0 | 1@31 | 10@11 | 1@11 | 1@21 | 4@11 | 4@18 |
|  |  |  |  | CP |  | 32@0 | 1@0 | 1@31 | 11@11 | 1@11 | 1@21 | 4@11 | 4@18 |
| 100MHz |  | 60 |  | DFT-s |  | 128@0 | 1@0 | 1@131 | 40@44 | 1@44 | 1@87 | 4@44 | 4@84 |
|  |  |  |  | CP |  | 132@0 | 1@0 | 1@131 | 44@44 | 1@44 | 1@87 | 4@44 | 4@84 |
|  |  | 120 |  | DFT-s |  | 64@0 | 1@0 | 1@65 | 20@22 | 1@22 | 1@43 | 4@22 | 4@40 |
|  |  |  |  | CP |  | 66@0 | 1@0 | 1@65 | 22@22 | 1@22 | 1@43 | 4@22 | 4@40 |
| 200MHz |  | 60 |  | DFT-s |  | 256@0 | 1@0 | 1@263 | 81@88 | 1@88 | 1@175 | 4@88 | 4@172 |
|  |  |  |  | CP |  | 264@0 | 1@0 | 1@263 | 88@88 | 1@88 | 1@175 | 4@88 | 4@172 |
|  |  | 120 |  | DFT-s |  | 128@0 | 1@0 | 1@131 | 40@44 | 1@44 | 1@87 | 4@44 | 4@84 |
|  |  |  |  | CP |  | 132@0 | 1@0 | 1@131 | 44@44 | 1@44 | 1@87 | 4@44 | 4@84 |
| 400MHz |  | 60 |  | DFT-s |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  |  | CP |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 120 |  | DFT-s |  | 256@0 | 1@0 | 1@263 | 64@66 | 1@66 | 1@197 | 4@66 | 4@194 |
|  |  |  |  | CP |  | 264@0 | 1@0 | 1@263 | 64@66 | 1@66 | 1@197 | 4@66 | 4@194 |
| Note 1: RB allocation is left aligned within inner region |  |  |  |  |  |  |  |  |  |  |

Table 6.1-2: Common Uplink Configuration for PC1

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Outer_Full | Outer_1RB_Left | Outer_1RB_Right | Inner_Full_Region1 | Innner_1RB_Left_Region1 | Inner_1RB_Right_Region1 | Inner_Full_Region2 | Innner_1RB_Left_Region2 | Inner_1RB_Right_Region2 | Innner_16RB_Left_Region2 | Inner_16RB_Right_Region2 |
| 50MHz |  | 60 |  | DFT-s |  | 64@0 | 1@0 | 1@65 | 20@22 | 1@22 | 1@43 | 32@16 | 1@1 | 1@64 | 16@1 | 16@49 |
|  |  |  |  | CP |  | 66@0 | 1@0 | 1@65 | 23@22 | 1@22 | 1@43 | 33@16 | 1@1 | 1@64 | 16@1 | 16@49 |
|  |  | 120 |  | DFT-s |  | 32@0 | 1@0 | 1@31 | 12@11 | 1@11 | 1@21 | 16@8 | 1@1 | 1@30 | 16@1 | 16@15 |
|  |  |  |  | CP |  | 32@0 | 1@0 | 1@31 | 12@11 | 1@11 | 1@21 | 16@8 | 1@1 | 1@30 | 16@1 | 16@15 |
| 100MHz |  | 60 |  | DFT-s |  | 128@0 | 1@0 | 1@131 | 45@44 | 1@44 | 1@87 | 64@32 | 1@1 | 1@130 | 16@1 | 16@115 |
|  |  |  |  | CP |  | 132@0 | 1@0 | 1@131 | 45@44 | 1@44 | 1@87 | 66@33 | 1@1 | 1@130 | 16@1 | 16@115 |
|  |  | 120 |  | DFT-s |  | 64@0 | 1@0 | 1@65 | 20@22 | 1@22 | 1@43 | 32@16 | 1@1 | 1@64 | 16@1 | 16@49 |
|  |  |  |  | CP |  | 66@0 | 1@0 | 1@65 | 23@22 | 1@22 | 1@43 | 33@16 | 1@1 | 1@64 | 16@1 | 16@49 |
| 200MHz |  | 60 |  | DFT-s |  | 256@0 | 1@0 | 1@263 | 81@88 | 1@88 | 1@175 | 128@64 | 1@1 | 1@262 | 16@1 | 16@247 |
|  |  |  |  | CP |  | 264@0 | 1@0 | 1@263 | 89@88 | 1@88 | 1@175 | 132@66 | 1@1 | 1@262 | 16@1 | 16@247 |
|  |  | 120 |  | DFT-s |  | 128@0 | 1@0 | 1@131 | 45@44 | 1@44 | 1@87 | 64@32 | 1@1 | 1@130 | 16@1 | 16@115 |
|  |  |  |  | CP |  | 132@0 | 1@0 | 1@131 | 45@44 | 1@44 | 1@87 | 66@33 | 1@1 | 1@130 | 16@1 | 16@115 |
| 400MHz |  | 60 |  | DFT-s |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  |  | CP |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 120 |  | DFT-s |  | 256@0 | 1@0 | 1@263 | 64@66 | 1@66 | 1@197 | 128@64 | 1@1 | 1@262 | 16@1 | 16@247 |
|  |  |  |  | CP |  | 264@0 | 1@0 | 1@263 | 66@66 | 1@66 | 1@197 | 132@66 | 1@1 | 1@262 | 16@1 | 16@247 |
| Note 1: RB allocation is left aligned within inner region 1 or inner region 2 as defined in clause 6.2.2.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>