3GPP TSG-RAN5 Meeting #90-e	R5-210318

# Online,  22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0273 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to E-UTRA link setup for NSA testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The E-UTRA link setup configuration for transmission mode and antenna configuration is not aligned with the assumptions in TS 38.101 chapter 9.1.2. Transmission mode 1 shall be used instead of TM3 and antenna config 1x2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct transmission mode to TM1 Update antenna config to 1x2 Update DCI format to 1A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | E-UTRA link setup remains incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex D.1, Annex D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

Annex D (normative):
E-UTRA link setup config for NSA testing

# D.0	General

Below clauses define the E-UTRA link setup config for NSA Demodulation and CSI tests cases unless otherwise specified within the main test case.

# D.1	E-UTRA test parameters

Below are the common test parameters to be configured for E-UTRA link.

Table D.1-1: Common Test Parameters (FDD)

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| Inter-TTI Distance |  | 1 |  |
| Number of HARQ processes | Processes | 8 | For FDD, 8 HARQ processes in the DL, as specified in TS 36.213 [10] clause 7. All 8 HARQ processes are used. |
| Scheduling of retransmissions |  |  | 1. Retransmissions use the same Transport Block Size (TBS) as the initial transmission. 2. HARQ processes are scheduled consecutively, independent of the fact, whether retransmissions (for negatively acknowledged HARQ processes) or new transmissions (for positively acknowledged HARQ processes) occur. |
| Maximum number of HARQ transmission |  | 4 | It is always 4 for FDD, as specified in TS 36.213 [10] clause 8 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK |  |
| Number of OFDM symbols for PDCCH | OFDM symbols | 3 for 5 MHz bandwidths, 2 for 10 MHz, 20MHz | The PCFICH carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe, as specified in TS 36.211 [8] clause 6.7 |
| Cyclic Prefix |  | Normal | CP consist of the following physical resource blocks (RBs) parameters: 12 consecutive subcarriers at a 15 kHz spacing and 7 OFDM symbols, as specified in TS 36.211 [8] clause 6.2.3 |
| Cell ID |  | 0 (Note 1) | The Cell ID is uniquely defined by a number in the range of 0 to 503, representing the physical-layer cell identity, as specified in TS 36.211 [8] clause 6.11. |
| DCI format for PDSCH | Format 1A |  |  |  |
| DCI format for PUSCH | Format 0 |  |  |  |

Table D.1-2: Common Test Parameters (TDD)

| Parameter | Unit | Value | Comments |  |
| --- | --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 1 |  |  |
| Special subframe configuration (Note 2) |  | 4 |  |  |
| Inter-TTI Distance |  | 1 |  |  |
| Number of HARQ processes | Processes | 7 | For TDD, 7 HARQ processes in the DL, as specified in TS 36.213 [10] clause 7. All 7 HARQ processes are used. |  |
| Scheduling of retransmissions |  |  | 1. Retransmissions use the same Transport Block Size (TBS) as the initial transmission. 2. HARQ processes are scheduled consecutively, independent of the fact, whether retransmissions (for negatively acknowledged HARQ processes) or new transmissions (for positively acknowledged HARQ processes) occur. 3. In case when the initial transmission and the retransmissions are scheduled in subframes with a different NPRB (in terms of TS 36.213 [10] subclause 7.1.7) according to TS 36.213 [10] subclause 7.1.7.2 and the appropriate modulation is used. |  |
| Maximum number of HARQ transmission |  | 4 | It is always 4 for TDD, as specified in TS 36.213 [10] clause 8 |  |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK |  |  |
| Number of OFDM symbols for PDCCH | OFDM symbols | 3 for 5 MHz bandwidths, 2 for 10 MHz | The PCFICH carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe, as specified in TS 36.211 [8] clause 6.7 |  |
| yclic Prefix |  | Normal | CP consist of the following physical resource blocks (RBs) parameters: 12 consecutive subcarriers at a 15 kHz spacing and 7 OFDM symbols, as specified in TS 36.211 [8] clause 6.2.3 |  |
| Cell ID |  | 0 (Note 3) | The Cell ID is uniquely defined by a number in the range of 0 to 503, representing the physical-layer cell identity, as specified in TS 36.211 [8] clause 6.11. |  |
| DCI format for PDSCH | Format 1A |  |  |
| DCI format for PUSCH | Format 0 |  |  |
| NOTE 1: as specified in Table 4.2-2 in TS 36.211 [8]. NOTE 2: as specified in Table 4.2-1 in TS 36.211 [8]. NOTE 3: For CA tests, Cell ID = 0 applies only to P-Cell. For (n)th S-Cell, Cell ID = n is used. |  |  |  |  |

# D.2	E-UTRA configuration

This clause defines the E-UTRA link settings for the test cases defined in clauses 5 and 6. The LTE link is supposed to be a functional link. The configuration defined in this clause ensures establishment of LTE link. Unless otherwise stated, ensure the UE is in state 3A-RF on the E-UTRA cell as defined in TS 36.508 [19].

Table D.2-1: E-UTRA configuration for EN-DC tests

| Parameter | Value | Comments |
| --- | --- | --- |
| Test Frequency during and after connection setup | Mid | As defined in TS 36.508 [19] for inter band test cases and as defined in TS 38.508-1 [6] clause 4.3.1 for intra band test cases, with NR SCS as per the test case for the LTE band under test |
| Bandwidth during and after connection setup | 5 MHz (Note 1) | Supported by all LTE bands. |
| PDSCH transmission mode and antenna config | TM1 1x2 |  |
| OCNG pattern | OP.1 for FDD OP.1 for TDD | These physical resource blocks are assigned to an arbitrary number of virtual UE' s with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |
| DL RMC | R.10-2 FDD for FDD R.10 TDD for TDD | Reuse the 10MHz one and change channel BW to 5 MHz or 20 MHz as applicable. |
| DL RB allocation | 25 | Full RB allocation assuming 5 MHz ChBW. 100 RB for 20 MHz ChBW as applicable |
| UL Signal levels during connection setup | PUSCH Power | Attained by enabling open loop power control and setting up UL signal levels according to Annexes H.0, H.2 and H.3 of TS 36.521-1 [16] |
| TA adjustments | TimeAlignmentTimerDedicated IE to be set to infinity | TimeAlignmentTimerDedicated IE to be set to infinity to ensure UE doesn't look for TA adjustments (See Table D.2-4) |
| CQI reports and SRS after connection setup | Disabled (See Table D.2-2 and D.2-3) | Disable periodic and aperiodic CQI reports to ensure none of these transmissions occur on the LTE uplink. |
| NOTE: If none of the UE supported EN-DC band combos support 5MHz E-UTRA carrier, configure 20 MHz channel BW. |  |  |

Table D.2-2 -CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table D.2-3: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated |  | Not present |  | RBC |
| } |  |  |  |  |

Table D.2-4: MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

< End of changes >