3GPP TSG-RAN WG5 Meeting #90-e	R5-210368

# Electronic Meeting,  22nd February 2021 - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0093 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS over 5GS TC 7.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. UE shall send an ACK to acknowledge the reception of 421 (Extension Required) from NW per RFC 3261. [RFC 3261, Cl. 13.1] After possibly receiving one or more provisional responses, the UAC will get one or more 2xx responses or one non-2xx final response. Because of the protracted amount of time it can take to receive final responses to INVITE, the reliability mechanisms for INVITE transactions differ from those of other requests (like OPTIONS). Once it receives a final response, the UAC needs to send an ACK for every final response it receives. 2. Step no. mentioned in Table 7.3.3.3-1 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added step 5A to handle reception of ACK - Corrected the Step no. in ‘Table 7.3.3.3-1: 421 Extension Required’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

## 7.3	MTSI MO Voice Call / 421 Extension Required / 5GS

### 7.3.1	Test Purpose (TP)

(1)

with { UE supporting preconditions and the precondition mechanism being enabled and UE having sent INVITE without preconditions }

ensure that {

when { UE receives 421 Extension Required response with precondition option-tag in the Require header }

then { UE sends a new INVITE request using preconditions }

}

(2)

with { UE having received 200 OK for first PRACK request }

ensure that {

when { Resources becoming available }

then { UE sends UPDATE request }

}

### 7.3.2	Conformance Requirements

[TS 24.229, clause 5.1.3.1]:

…

Upon receiving a 421 (Extension Required) response to an initial INVITE request in which the precondition mechanism was not used, including the "precondition" option-tag in the Require header field, if the UE supports the precondition mechanism and the precondition mechanism is enabled as specified in subclause 5.1.5A, the originating UE shall:

-	send a new INVITE request using the precondition mechanism; and

-	send an UPDATE request as soon as the necessary resources are available and a 200 (OK) response for the first PRACK request has been received.

…

Reference(s)

3GPP TS 24.229 [7], clauses 5.1.3.1.

### 7.3.3	Test description

#### 7.3.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to not use the precondition mechanism.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1[21]) and registered to IMS.

#### 7.3.3.2	Test procedure sequence

Table 7.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call. | - | - | - | - |
| 2 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 3 | Step 1 of Annex A.4.2 happens | --> | INVITE | - | - |
| 4 | Step 2 of Annex A.4.2 happens | <-- | 100 Trying | - | - |
| 5 | The SS sends a 421 response including the "precondition" option-tag in the Require header field | <-- | 421 Extension Required | - | - |
| 5A | UE acknowledges the reception of 421 (Extension Required) message. | --> | ACK | - | - |
| 6 | Step 1 of Annex A.4.1 happens (Check: does the UE sends new INVITE request using preconditions?) | --> | INVITE | 1 | P |
| 7 | Step 2 of Annex A.4.1 happens | <-- | 100 Trying | - | - |
| 8 | Step 3 of Annex A.4.1 happens | <-- | 183 Session Progress | - | - |
| 9 | Step 4 of Annex A.4.1 happens | --> | PRACK | - | - |
| 10 | Step 5 of Annex A.4.1 happens | <-- | 200 OK | - | - |
| 11 | Check: Does the UE send UPDATE request? (Step 6 of Annex A.4.1 happens) | --> | UPDATE | 2 | P |
| 12 | Step 7 of Annex A.4.1 happens | <-- | 200 OK | - | - |
| 13 | Step 8 of Annex A.4.1 happens | <-- | 180 Ringing | - | - |
| 14 | Step 9 of Annex A.4.1 happens | --> | PRACK | - | - |
| 15 | Step 10 of Annex A.4.1 happens | <-- | 200 OK | - | - |
| 16 | Step 11 of Annex A.4.1 happens | <-- | 200 OK | - | - |
| 17 | Step 12 of Annex A.4.1 happens | --> | ACK | - | - |

#### 7.3.3.3	Specific message contents

Table 7.3.3.3-1: 421 Extension Required (step 5, table 7.3.3.2-1)

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [6] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 421 |  |  |
| Reason-Phrase |  | Extension Required |  |  |
| Via |  |  |  | RFC 3261 [6] |
| via-parm |  | same value as received in request |  |  |
| From |  |  |  | RFC 3261 [6] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | same value as received in request |  |  |
| To |  |  |  | RFC 3261 [6] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | same value as received in request |  |  |
| Call-ID |  |  |  | RFC 3261 [6] |
| callid |  | same value as received in request |  |  |
| CSeq |  |  |  | RFC 3261 [6] |
| value |  | same value as received in request |  |  |
| Require |  |  |  | RFC 3261 [6] |
| option-tag |  | precondition |  | RFC 3312 [29] |

<End of modification>