3GPP TSG-RAN WG5 #90-e		R5-210452
Electronic Meeting, 22nd February – 5th March 2021

Agenda item:	5.3.2.17

Source:	Rohde & Schwarz

Title:	On the DL AWGN MTSU for RRM

Document for:	Endorsement

# Introduction

The DL AWGN (or wanted signal) absolute level MTSU for RRM is not yet finalized. The following table captures the status for the different test setups:

|  | 1 AoA setups |  | Multi-AoA setups |  |  |
| --- | --- | --- | --- | --- | --- |
|  | IFF | Simplified DFF | Enhanced IFF | DFF | DFF-IFF |
| DL AWGN MU | 5.19 dB | FFS | 5.25 dB | [5.53] dB | FFS |

This paper provides a way forward to define the DL AWGN MTSU for RRM.

# Discussion

There are 134 RRM test cases for FR2. Out of those 134, 106 are defined with AoA Setups 1, 2a or 2b, which require only 1 AoA.

Observation 1: ~80% of the RRM FR2 test cases require only 1AoA.

Therefore, it seems reasonable to select a 1 AoA test setup as the baseline for these test cases, since it is the most optimized approach.

Proposal 1: Use a 1AoA test setup as baseline to define the DL AWGN MTSU for RRM 1 AoA test cases.

The 1AoA test setups for RRM are the same as for RF. Even for RF, simplified DFF has not been pursued. Thus, our proposal is to use IFF and the baseline for 1AoA RRM test cases.

Proposal 2: Set the DL AWGN MTSU for RRM 1 AoA test cases to 5.19 dB.

For 2 AoA test cases, we have the following statement in TS 38.508-1:

Measurement Uncertainty:

-	The threshold MU for the equivalence framework for RRM will be based on direct far field (DFF) test method for D ≤ 5 cm and on indirect far field (IFF) test method for D > 5 cm. If the MTSU for the IFF test method for D ≤ 5 cm is finalized before DFF, the IFF MTSU shall be used as provisional threshold MU until DFF is completed.

Observation 2: Enhanced IFF MTSU has been finalized and the DFF MTSU is still under discussion.

Observation 3: DFF-IFF was not considered at the time of writing the above statement in TS 38.508-1

Given Observations 2 and 3, our proposal is to use the enhanced IFF MTSU as working assumption to progress the RRM FR2 test case analysis until the DFF discussion is finalized.

Proposal 3: Use 5.25 dB as working assumption MTSU for RRM FR2 test cases.

# Conclusion

Proposal 1: Use a 1AoA test setup as baseline to define the DL AWGN MTSU for RRM 1 AoA test cases.

Proposal 2: Set the DL AWGN MTSU for RRM 1 AoA test cases to 5.19 dB.

Proposal 3: Use 5.25 dB as working assumption MTSU for RRM FR2 test cases.

# References

- TS 38.508-1, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”