3GPP TSG-RAN5 Meeting #90-e	R5-210469

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1705 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changes to RRM default message contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some of the tables in section 7.3.1 do not have proper headers, making it very difficult to navigate between them Several tables in section 7.3.1 refer to CSI, TRS and BM, without giving any explanation of when to use which of them. The Title of Table 7.3.1-15 is incorrect, it is not PDCCH-, the table is Control Resource Set. Also, the condition Non-TRS is not needed, there is already a condition TRS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Headers have been added to all tables A new section 7.3.0 with general definitions have been introduced, adding a clarification of the meaning of CSI, TRS and BM within clause 7.3 Corrected title of Table 7.3.1-15. Removed superfluous condition Non-TRS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The navigation of the message contents will not be intuitive It would be unclear when to apply the message contents for CSI, for TRS or for BM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

## 7.3	Default NG-RAN RRC message and information elements contents for RRM

### 7.3.0	General definitions

This section defines general concepts and conditions used in the RRM message contents in clause 7.3:

#### -	CSI-RS for Tracking, CSI reporting and beam management

Table 7.3.0-1: Definitions of CSI-RS for Tracking, CSI reporting and beam management

| Type | Short version | Explanation |
| --- | --- | --- |
| CSI-RS for tracking | TRS | Corresponds to TRS.X.Y FDD/TDD RMCs in TS 38.533 [18] |
| CSI-RS for BM | BM | Corresponds to CSI-RS.X.2 FDD/TDD RMCs in TS 38.533 [18] |
| CSI-RS for CSI reporting | CSI | Corresponds to CSI-RS.X.1 FDD/TDD RMCs in TS 38.533 [18] |

Editor's Note: The message contents for CSI-RS.X.3/4 FDD/TDD RMCs in TS 38.533 [18] are not yet defined.

### 7.3.1	Radio resource control information elements for RRM

As defined in clause 4.6.3 with the following exceptions:

#### –	TDD-UL-DL-Config

Table 7.3.1-1: TDD-UL-DL-Config

| Derivation Path: Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms0p625 |  | TDDConf.3.1 |
|  | ms5 |  |  |
| nrofDownlinkSlots | 3 |  | TDDConf.2.1, TDDConf.3.1 |
|  | 1 |  | TDDConf.1.1 |
| nrofDownlinkSymbols | 10 |  | TDDConf.1.1, TDDConf.3.1 |
|  | 6 |  | TDDConf.2.1 |
| nrofUplinkSlots | 2 |  | TDDConf.1.1 |
|  | 1 |  | TDDConf.3.1 |
|  | 4 |  | TDDConf.2.1 |
| nrofUplinkSymbols | 4 |  | TDDConf.2.1 |
|  | 2 |  | TDDConf.1.1, TDDConf.3.1 |
| dl-UL-TransmissionPeriodicity-v1530 | Not present |  |  |
|  | ms4 |  | TDDConf.1.1, TDDConf.2.1 |
| } |  |  |  |
| pattern2 | Not present |  |  |
| pattern2 SEQUENCE { |  |  | TDDConf.1.1, TDDConf.2.1 |
| dl-UL-TransmissionPeriodicity | ms1 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofDownlinkSlots | 1 |  | TDDConf.1.1 |
|  | 2 |  | TDDConf.2.1 |
| nrofDownlinkSymbols | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofUplinkSlots | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| nrofUplinkSymbols | 0 |  | TDDConf.1.1, TDDConf.2.1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TDDConf.1.1 | TDD UL/DL configuration for SCS=15kHz |
| TDDConf.2.1 | TDD UL/DL configuration for SCS=30kHz |
| TDDConf.3.1 | TDD UL/DL configuration for SCS=120kHz |

#### –	FilterCoefficient

Table 7.3.1-2: FilterCoefficient

| Derivation Path: Table 4.6.3-57 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FilterCoefficient | fc0 | L3 filtering is not used |  |

#### –	SSB-MTC

Table 7.3.1-3: SSB-MTC

| Derivation Path: Table 4.6.3-185 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  | SMTC.1, SMTC.2 |
| sf20 | 10 |  | SMTC.4, SMTC.5 |
| sf160 | 0 |  | SMTC.3 |
| } |  |  |  |
| duration | sf1 |  | SMTC.1, SMTC.3, SMTC.4 |
|  | sf5 |  | SMTC.2, SMTC.5 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n as defined in 38.533 Annex A.4 |

#### –	SubcarrierSpacing

Table 7.3.1-3a: SubcarrierSpacing

| Derivation Path: Table 4.6.3-188 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ssbSubcarrierSpacing | kHz15 |  | SSB.1 FR1, SSB.3 FR1 or SSB.5 FR1 |
|  | kHz30 |  | SSB.2 FR1, SSB.4 FR1 or SSB.6 FR1 |
|  | kHz120 |  | SSB.1 FR2, SSB.3 FR2, SSB.5 FR2 or SSB.7 FR2 |
|  | kHz240 |  | SSB.2 FR2, SSB.4 FR2, SSB.6 FR2 or SSB.8 FR2 |

| Condition | Explanation |
| --- | --- |
| SSB.n FR1 | SSB RMC n for FR1 as defined in 38.533 Annex A.3.1 |
| SSB.n FR2 | SSB RMC n for FR2 as defined in 38.533 Annex A.3.2 |

#### –	ServingCellConfigCommon

Table 7.3.1-4: ServingCellConfigCommon

| Derivation Path: Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 1000 | 1 SS Block in low FR1 frequencies | LOW_FREQ |
|  | 1100 | 2 SS Blocks in low FR1 frequencies | 2SSB AND LOW_FREQ |
| mediumBitmap | 10000000 | 1 SS Block in high FR1 frequencies | HIGH_FREQ |
|  | 11000000 | 2 SS Blocks in high FR1 frequencies | 2SSB AND HIGH_FREQ |
| longBitmap | 1000000000000000000000000000000000000000000000000000000000000000 | 1 SS Block in FR2 | FR2 |
|  | 1100000000000000000000000000000000000000000000000000000000000000 | 2 SS Blocks in FR2 | 2SSB AND FR2 |
| } |  |  |  |
| ssb-periodicityServingCell | ms20 |  |  |
| ssbSubcarrierSpacing | kHz15 |  |  |
|  | kHz30 |  | SCS30kHz |
|  | kHz120 |  | FR2 |
| } | kHz240 |  | FR2 AND SCS240kHz |

| Condition | Explanation |
| --- | --- |
| LOW_FREQ | Frequency <= 2.4 GHz for TDD or Frequency <= 3 GHz for FDD |
| HIGH_FREQ | FR1 and (Frequency > 2.4 GHz for TDD or Frequency > 3 GHz for FDD or CASE_C) |
| 2SSB | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 contain 2 SSBs within a burst |
| SCS30kHz | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 is for 30 kHz SCS |
| SCS240kHz | The SSB pattern as defined in TS 38.533 [18] Annex A.3.1 is for 240 kHz SCS |

#### –	ServingCellConfigCommonSIB

Table 7.3.1-5: ServingCellConfigCommonSIB-RRM

| Derivation Path: Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’1000 0000’B | When carrier frequency <= 3 GHz for FDD or <= 2.4 GHz for TDD, only the 4 leftmost bits are valid; |  |
|  | ’1100 0000’B |  | 2SSB |
| groupPresence | Not present |  |  |
|  | ’1000 0000’B |  | FR2 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR2 | Frequency range 2 |
| 2SSB | For configuration with 2 SS Blocks |

#### -	CSI-MeasConfig

Table 7.3.1-6: CSI-MeasConfig for RRM

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n1+n2+n3+n4+n5 entries | n1=1 if CSI-RS for CSI is configured in test case, n1=0 otherwise; n2=2 if CSI-RS for BM is configured in test case, n2=0 otherwise; n3=4 if TRS is configured in test case, n3=0 otherwise; n4=4 if second resource set of TRS is configured in test case, n4=0 otherwise; n5=2 if aperiodic CSI-RS for BM is configured in test case, n5=0 otherwise; |  |
| NZP-CSI-RS-Resource[k, k=1..n1] | NZP-CSI-RS-Resource for CSI | entry 1 | n1>0 |
| NZP-CSI-RS-Resource[k, k=n1+1.. n1+n2] | NZP-CSI-RS-Resource for BM (k-n1-1) | entry ... | n2>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+1.. n1+n2+n3] | NZP-CSI-RS-Resource for TRS (k-n1-n2) | entry ... | n3>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+1.. n1+n2+n3+n4] | NZP-CSI-RS-Resource for TRS (k-n1-n2-n3) with condition SECOND_SET | entry ... | n4>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+n4+1.. n1+n2+n3+n4+n5] | NZP-CSI-RS-Resource for BM (k-n1-n2-n3-n4) with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | m1+m2+m3+m4+m5 entries | mi=1 if ni>0, mi=0 otherwise |  |
| NZP-CSI-RS-ResourceSet[k, k=1..m1] | NZP-CSI-RS-ResourceSet for CSI | entry 1 | n1>0 |
| NZP-CSI-RS-ResourceSet[k, k=m1+1.. m1+m2] | NZP-CSI-RS-ResourceSet for BM | entry ... | n2>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+1.. m1+m2+m3] | NZP-CSI-RS-ResourceSet for TRS | entry ... | n3>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+1.. m1+m2+m3+m4] | NZP-CSI-RS-ResourceSet for TRS with condition SECOND_SET | entry ... | n4>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+m4+1.. m1+m2+m3+m4+m5] | NZP-CSI-RS-ResourceSet for BM with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | m1+m2+m3+m5 entries |  |  |
| CSI-ResourceConfig[k, k=1..m1] | CSI-ResourceConfig for CSI | entry 1 | n1>0 |
| CSI-ResourceConfig[k, k=m1+1.. m1+m2] | CSI-ResourceConfig for BM | entry ... | n2>0 |
| CSI-ResourceConfig[k, k= m1+m2+1.. m1+m2+m3] | CSI-ResourceConfig for TRS | entry ... | n3>0 and n4=0 |
|  | CSI-ResourceConfig for TRS with condition SECOND_SET |  | n4>0 |
| CSI-ResourceConfig[k, k= m1+m2+m3+1.. m1+m2+m3+m5] | CSI-ResourceConfig for BM with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | m1+m2+m5 entries |  |  |
| CSI-ReportConfig[k, k=1..m1] | CSI-ReportConfig for CSI | entry 1 | n1>0 |
| CSI-ReportConfig[k, k=m1+1.. m1+m2] | CSI-ResourceConfig for BM | entry ... | n2>0 |
| CSI-ReportConfig[k, k=m1+m2+1.. m1+m2+m5] | CSI-ResourceConfig for BM with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| reportTriggerSize | Not present |  |  |
|  | 1 |  | n5>0 |
| aperiodicTriggerStateList CHOICE { | Not present |  |  |
| setup | CSI-AperiodicTriggerStateList |  | n5>0 |
| } |  |  |  |
| } |  |  |  |

#### –	NZP-CSI-RS-Resource for TRS

Table 7.3.1-7: NZP-CSI-RS-Resource for TRS(Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| NZP-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for TRS(Id) |  |  |
|  | NZP-CSI-RS-ResourceId for TRS(Id) with Condition SECOND_SET |  | SECOND_SET |
| CSI-RS-ResourceMapping | CSI-RS-ResourceMapping for TRS(Id) |  |  |
|  | CSI-RS-ResourceMapping for TRS(Id) with condition SECOND_SET |  | SECOND_SET |
| powerControlOffsetSS | db-3 |  |  |
| scramblingID | PhysCellId | PCI of the cell sending the TRS |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for TRS(Id) |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId-RRM(2) |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For resource belong to the second resource set for TRS, only applies to FR2 test |

#### –	NZP-CSI-RS-Resource for CSI

Table 7.3.1-7A: NZP-CSI-RS-Resource for CSI

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for CSI |  |  |
| resourceMapping | CSI-RS-ResourceMapping for CSI |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for CSI |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId-RRM(0) |  |  |
| } |  |  |  |

#### –	NZP-CSI-RS-Resource for BM

Table 7.3.1-7B: NZP-CSI-RS-Resource for BM(Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for BM (Id) |  |  |
|  | NZP-CSI-RS-ResourceId for BM (Id) with condition APERIODIC |  | APERIODIC |
| resourceMapping | CSI-RS-ResourceMapping for BM (Id) |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for BM |  |  |
|  | Not present |  | APERIODIC |
| qcl-InfoPeriodicCSI-RS | TCI-StateId-RRM(Id) |  |  |
|  | Not present |  | APERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	NZP-CSI-RS-ResourceId for TRS

Table 7.3.1-7C: NZP-CSI-RS-ResourceId for TRS(Id)

| Derivation Path: Table 4.6.3-86 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | n+Id-1 | n is the first NZP-CSI-RS-ResourceId allocated for TRS resource set. Value of n is left to internal implementation Id = 1,2,3,4 |  |
|  | n+Id+3 |  | SECOND_SET |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second TRS resource set configured in test, only applies to FR2 test |

#### –	NZP-CSI-RS-ResourceId for CSI

Table 7.3.1-7D: NZP-CSI-RS-ResourceId for CSI

| Derivation Path: Table 4.6.3-86 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | n | n is the NZP-CSI-RS-ResourceId allocated for CSI-RS for CSI report. Value of n is left to internal implementation |  |

#### –	NZP-CSI-RS-ResourceId for BM

Table 7.3.1-7E: NZP-CSI-RS-ResourceId for BM(Id)

| Derivation Path: Table 4.6.3-86 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | n+Id | n is the first NZP-CSI-RS-ResourceId allocated for CSI-RS for BM. Value of n is left to internal implementation Id = 0,1 |  |
|  | m+Id | m is the first NZP-CSI-RS-ResourceId allocated for aperiodic CSI-RS for BM. Value of m is left to internal implementation Id = 0,1 | APERIODIC |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-RS-ResourceMapping for TRS

Table 7.3.1-8: CSI-RS-ResourceMapping for TRS(Id)

| Derivation Path: Table 4.6.3-45 with condition TRS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 5 | l0 = 5 for CSI-RS resource 1 and 3 | (Id = 1 or 3) AND FR1 |
|  | 9 | l0 = 9 for CSI-RS resource 2 and 4 | (Id = 2 or 4) AND FR1 |
|  | 1 |  | (Id = 1 or 3) AND FR2 |
|  | 2 |  | (Id = 1 or 3) AND FR2 AND SECOND_SET |
|  | 5 |  | (Id = 2 or 4) AND FR2 |
|  | 6 |  | (Id = 1 or 3) AND FR2 AND SECOND_SET |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second resource set for TRS configured in test, only applies to FR2 test |

#### –	CSI-RS-ResourceMapping for CSI

Table 7.3.1-8A: CSI-RS-ResourceMapping for CSI

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000001 |  |  |
| } |  |  |  |
| nrofPorts | p2 |  | FR1 |
|  | p1 |  | FR2 |
| firstOFDMSymbolInTimeDomain | 5 |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

#### –	CSI-RS-ResourceMapping for BM

Table 7.3.1-8B: CSI-RS-ResourceMapping for BM (Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 |  |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 6 |  | Id = 0 |
|  | 10 |  | Id = 1 |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

#### –	CSI-ResourcePeriodicityAndOffset for TRS

Table 7.3.1-9: CSI-ResourcePeriodicityAndOffset for TRS(Id)

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots80 | 40 |  | (Id = 1 or 3) AND SCS120 |
|  | 41 |  | (Id = 2 or 4) AND SCS120 |
| slots40 | 20 | Periodicity 40 slots and offset 20 for CSI-RS resource 1 and 2 | (Id = 1 or 3) AND SCS30 |
|  | 21 | Periodicity 40 slots and offset 21 for CSI-RS resource 3 and 4 | (Id = 2 or 3) AND SCS30 |
| slots20 | 10 | Periodicity 20 slots and offset 10 for CSI-RS resource 1 and 2 | (Id = 1 or 3) AND SCS15 |
|  | 11 | Periodicity 20 slots and offset 11 for CSI-RS resource 3 and 4 | (Id = 2 or 4) AND SCS15 |
| } |  |  |  |

#### –	CSI-ResourcePeriodicityAndOffset for CSI

Table 7.3.1-9A: CSI-ResourcePeriodicityAndOffset for CSI

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 1 |  | SCS15 |
| slots10 | 2 |  | SCS30 |
| slots40 | 8 |  | SCS120 |
| } |  |  |  |

#### –	CSI-ResourcePeriodicityAndOffset for BM

Table 7.3.1-9B: CSI-ResourcePeriodicityAndOffset for BM

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 1 |  | SCS15 |
| slots20 | 2 |  | SCS30 |
| slots80 | 8 |  | SCS120 |
| } |  |  |  |

#### –	CSI-FrequencyOccupation

Table 7.3.1-10: CSI-FrequencyOccupation-RRM

| Derivation Path: Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| startingRB | 4*floor(n/4) | n is the start RB of active BWP floor() means rounding down to the nearest integer startingRB can only be multiple of 4 |  |
| nrofRBs | max(4*ceil(n/4-floor(n/4)+m/4),24) | m is the bandwidth of active BWP ceil() means rounding up to the nearest integer Bandwidth of CSI-RS used in RRM test is required to be same as active BWP according to 38.133. nrofRBs shall be no less than 24 and shall be multiple of 4 |  |
| } |  |  |  |

#### –	NZP-CSI-RS-ResourceSet for TRS

Table 7.3.1-11: NZP-CSI-RS-ResourceSet for TRS

| Derivation Path: Table 4.6.3-87 with Condition TRS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | NZP-CSI-RS-ResourceSetId-TRS |  |  |
|  | NZP-CSI-RS-ResourceSetId-TRS with condition SECOND_SET |  | SECOND_SET |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for TRS(1) | entry 1 |  |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for TRS(2) | entry 2 |  |
| NZP-CSI-RS-ResourceId[3] | NZP-CSI-RS-ResourceId for TRS(3) | entry 3 |  |
| NZP-CSI-RS-ResourceId[4] | NZP-CSI-RS-ResourceId for TRS(4) | entry 4 |  |
| } |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries |  | SECOND_SET |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for TRS(1) with condition SECOND_SET | entry 1 |  |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for TRS(2) with condition SECOND_SET | entry 2 |  |
| NZP-CSI-RS-ResourceId[3] | NZP-CSI-RS-ResourceId for TRS(3) with condition SECOND_SET | entry 3 |  |
| NZP-CSI-RS-ResourceId[4] | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | entry 4 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second TRS resource set configured in test, only applies to FR2 test |

#### –	NZP-CSI-RS-ResourceSet for CSI

Table 7.3.1-11A: NZP-CSI-RS-ResourceSet for CSI

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | NZP-CSI-RS-ResourceSetId-CSI |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for CSI | entry 1 |  |
| } |  |  |  |
| repetition | Not present |  |  |
| } |  |  |  |

#### –	NZP-CSI-RS-ResourceSet for BM

Table 7.3.1-11B: NZP-CSI-RS-ResourceSet for BM

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId-BM |  |  |
|  | NZP-CSI-RS-ResourceSetId-BM with condition APERIODIC |  | APERIODIC |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for BM (0) | entry 1 |  |
|  | NZP-CSI-RS-ResourceId for BM (0) with condition APERIODIC |  | APERIODIC |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for BM (1) | entry 2 |  |
|  | NZP-CSI-RS-ResourceId for BM (1) with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| aperiodicTriggeringOffset | 6 |  | APERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	NZP-CSI-RS-ResourceSetId for TRS

Table 7.3.1-11C: NZP-CSI-RS-ResourceSetId-TRS

| Derivation Path: Table 4.6.3-88 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | n | n is the first NZP-CSI-RS-ResourceSetId allocated for TRS resource set. Value of n is left to internal implementation |  |
|  | n+1 |  | SECOND_SET |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second TRS resource set configured in test, only applies to FR2 test |

#### –	NZP-CSI-RS-ResourceSetId for CSI

Table 7.3.1-11D: NZP-CSI-RS-ResourceSetId-CSI

| Derivation Path: Table 4.6.3-88 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | n | n is the NZP-CSI-RS-ResourceSetId allocated for resource set of CSI-RS for CSI reporting. Value of n is left to internal implementation |  |

#### –	NZP-CSI-RS-ResourceSetId for BM

Table 7.3.1-11E: NZP-CSI-RS-ResourceSetId-BM

| Derivation Path: Table 4.6.3-88 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | n | n is the NZP-CSI-RS-ResourceSetId allocated for resource set of CSI-RS for BM. Value of n is left to internal implementation |  |
|  | m | m is the NZP-CSI-RS-ResourceSetId allocated for resource set of aperiodic CSI-RS for BM. Value of m is left to internal implementation | APERIODIC |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-ResourceConfig for TRS

Table 7.3.1-12: CSI-ResourceConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-TRS |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-TRS |  |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 2 entries |  | SECOND_SET |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSetId[2] | NZP-CSI-RS-ResourceSetId-TRS with condition SECOND_SET | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second resource set for TRS configured in test, only applies to FR2 test |

#### –	CSI-ResourceConfig for CSI

Table 7.3.1-12A: CSI-ResourceConfig for CSI

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-CSI |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-CSI | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| } |  |  |  |

#### –	CSI-ResourceConfig for BM

Table 7.3.1-12B: CSI-ResourceConfig for BM

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-BM |  |  |
|  | CSI-ResourceConfigId-BM with condition APERIODIC |  | APERIODIC |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-BM |  |  |
|  | NZP-CSI-RS-ResourceSetId-BM with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| resourceType | aperiodic |  | APERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-ResourceConfigId for TRS

Table 7.3.1-12C: CSI-ResourceConfigId-TRS

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of TRS. Value of n is left to internal implementation |  |

#### –	CSI-ResourceConfigId for CSI

Table 7.3.1-12D: CSI-ResourceConfigId-CSI

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of CSI-RS for CSI reporting. Value of n is left to internal implementation |  |

#### –	CSI-ResourceConfigId for BM

Table 7.3.1-12E: CSI-ResourceConfigId-BM

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of CSI-RS for BM. Value of n is left to internal implementation |  |
|  | m | m is the CSI-ResourceConfigId allocated for resource config of aperiodic CSI-RS for BM. Value of m is left to internal implementation | APERIODIC |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-ReportConfig for CSI

Table 7.3.1-12F: CSI-ReportConfig for CSI

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-CSI |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-CSI |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic | Set according to parameters given in test cases |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSI-ReportConfig for BM

Table 7.3.1-12G: CSI-ReportConfig for BM

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-BM |  |  |
|  | CSI-ReportConfigId-BM with condition APERIODIC |  | APERIODIC |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-BM |  |  |
|  | CSI-ResourceConfigId-BM with condition APERIODIC |  | APERIODIC |
| reportConfigType CHOICE { |  |  |  |
| periodic | Set according to parameters given in test cases |  |  |
| aperiodic SEQUENCE { |  |  | APERIODIC |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF { | 1 entry |  |  |
| INTEGER[1] | 26 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RSRP | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For aperiodic CSI-RS resources |

#### –	CSI-ReportConfigId for CSI

Table 7.3.1-12H: CSI-ReportConfigId-CSI

| Derivation Path: Table 4.6.3-40 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfigId | n | n is the CSI-ReportConfigId allocated for report config of CSI-RS for CSI. Value of n is left to internal implementation |  |

#### –	CSI-ReportConfigId for BM

Table 7.3.1-12I: CSI-ReportConfigId-BM

| Derivation Path: Table 4.6.3-40 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfigId | n | n is the CSI-ReportConfigId allocated for report config of CSI-RS for BM. Value of n is left to internal implementation |  |
|  | m | m is the CSI-ReportConfigId allocated for report config of aperiodic CSI-RS for BM. Value of m is left to internal implementation | APERIODIC |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-AperiodicTriggerStateList

Table 7.3.1-12J: CSI-AperiodicTriggerStateList

| Derivation Path: Table 4.6.3-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF SEQUENCE { | 1 entry |  |  |
| associatedReportConfigInfoList[1] SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | CSI-ReportConfigId-BM with condition APERIODIC |  |  |
| resourcesForChannel[1] CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | NZP-CSI-RS-ResourceSetId-BM with condition APERIODIC |  |  |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF { | 2 entries |  |  |
| TCI-StateId[1] | TCI-StateId-RRM(0) | QCL Type C+D to SSB #0 |  |
| TCI-StateId[2] | TCI-StateId-RRM(1) | QCL Type C+D to SSB #1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesForInterference[1] | Not present |  |  |
| nzp-CSI-RS-ResourcesForInterference[1] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RACH-ConfigCommon

Table 7.3.1-13: RACH-ConfigCommon

| Derivation Path: TS 38.508-1 Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n48 |  |  |
| } |  |  |  |
| groupBconfigured | Not present |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| rsrp-ThresholdSSB | 51 |  |  |
| rsrp-ThresholdSSB-SUL | Not present |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 for the NR Cell. |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| restrictedSetConfig | unrestrictedSet |  |  |
| msg3-transformPrecoder | Not present | transform precoding is disabled for Msg3 PUSCH transmission and any PUSCH transmission scheduled with DCI format 0_0 |  |
| } |  |  |  |

#### –	RACH-ConfigGeneric

Table 7.3.1-14: RACH-ConfigGeneric

| Derivation Path: TS 38.508-1 Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 102 |  | FR1 |
|  | 190 |  | FR2 |
| msg1-FDM | one |  |  |
| msg1-FrequencyStart | 0 |  |  |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

#### –	ControlResourceSet

Table 7.3.1-15: ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| duration | 2 |  | FR1 |
|  | 1 |  | FR2 |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | Null |  |  |
| interleaved ::= SEQUENCE { |  |  | CCR |
| reg-BundleSize | n6 |  |  |
| interleaverSize | n2 |  |  |
| shiftIndex | 0 |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | TCI-StateId-RRM(0) | TCI State #0, QCLed to SSB index #0 |  |
|  | TCI-StateId-RRM(2) | TCI State #2, QCLed to TRS resource #4 in the first resource set | TRS |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CCR | When Control Channel RMC is configured |
| TRS | When at least one TRS resource set is configured. |

#### -	SchedulingRequestResourceConfig

Table 7.3.1-16: SchedulingRequestResourceConfig

| Derivation Path: Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 7 | With SCS = kHz15 results in repetition every 10 ms | SCS_15kHz |
| sl20 | 7 | With SCS = kHz30 results in repetition every 10 ms | SCS_30kHz |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for frequency of the cell according to clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| SCS_30kHz | SCS=30kHz for frequency of the cell according to clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |

#### -	SearchSpace

Table 7.3.1-17: SearchSpace

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl10 | 1 |  | SISS |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SISS | SearchSpace for SI |

#### –	PDSCH-Config

Table 7.3.1-18: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1+n1+n2+n3 entries | n1 = 1 if SSB configuration used in test case contains two SSBs in a burst, n1 = 0 otherwise n2 = 1 if TRS is configured in test case, n1 = 0 otherwise n3 = 1 if two resource sets of TRS are configured in test case, n3 = 0 otherwise |  |
| TCI-State[1] | TCI-State(0) | entry 1 QCLed to SSB index #0 |  |
| TCI-State[k, k=2..1+n1] | TCI-State(1) | entry ... QCLed to SSB index #1 | SECOND_SSB |
| TCI-State[k, k=2+n1..1+n1+n2] | TCI-State(2) | entry ... QCLed to TRS resource #4 in the first resource set | TRS |
| TCI-State[k, k=2+n1+n2..1+n1+n2+n3] | TCI-State(3) | entry ... QCLed to TRS resource #4 in the second resource set | SECOND_SET |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SSB | SSB configuration used in test case contain two SSBs in a burst |
| TRS | One resource set for TRS is configured in test case |
| SECOND_SET | Two resource sets for TRS are configured in test case, only applies to FR2. |

#### –	TCI-State

Table 7.3.1-19: TCI-State(Id)

| Derivation Path: Table 4.6.3-190 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-State ::= SEQUENCE { |  |  |  |
| tci-StateId | TCI-StateId-RRM(Id) |  |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| bwp-Id | BWP-Id of the active BWP |  | Id = 2 or 3 |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #0 |  | Id = 0 |
|  | SSB-Index of SSB #1 |  | Id = 1 |
| csi-rs | NZP-CSI-RS-ResourceId for TRS (4) |  | Id = 2 |
|  | NZP-CSI-RS-ResourceId for TRS (4) with condition SECOND SET |  | Id = 3 |
| } |  |  |  |
| qcl-Type | typeC |  | Id = 0 or 1 |
|  | typeA |  | Id = 2 or 3 |
| } |  |  |  |
| qcl-Type2 | Not present |  |  |
| qcl-Type2 SEQUENCE { |  |  | FR2 |
| cell | Not present |  |  |
| bwp-Id | Not present |  | Id = 0 or 1 |
|  | BWP-Id of the active BWP |  | Id = 2 or 3 |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index of SSB #0 |  | Id = 0 |
|  | SSB-Index of SSB #1 |  | Id = 1 |
| csi-rs | NZP-CSI-RS-ResourceId for TRS (4) |  | Id = 2 |
|  | NZP-CSI-RS-ResourceId for TRS (4) with condition SECOND SET |  | Id = 3 |
| } |  |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	TCI-StateId

Table 7.3.1-20: TCI-StateId(Id)

| Derivation Path: Table 4.6.3-191 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-StateId | n+Id | n is the first TCI-StateId allocated for TCI-State configured in RRM test. Value of n is left to internal implementation Id = 0,1,2,3 |  |

#### –	PUSCH-Config

Table 7.3.1-21: PUSCH-Config

| Derivation Path: Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  | APERIODIC |
| setup | PUSCH-TimeDomainResourceAllocationList-BM |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	PUSCH-TimeDomainResourceAllocationList

Table 7.3.1-22: PUSCH-TimeDomainResourceAllocationList-BM

| Derivation Path: Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF SEQUENCE { | 1 entriy | same number of entries as reportSlotOffsetList in Table 7.3.1-12G |  |
| k2[1] | 26 | Same with k2 configrued in reportSlotOffsetList in Table 7.3.1-12G |  |
| mappingType[1] | typeA |  |  |
| startSymbolAndLength[1] | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |

#### – 	ServingCellConfig

Table 7.3.1-23: ServingCellConfig

| Derivation Path: Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig CHOICE { |  |  |  |
| setup | csi-MeasConfig |  |  |
| } |  |  |  |

< End of changes >