3GPP TSG-RAN5 Meeting #90-e	R5-210540

# Electronic Meeting,  22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1130 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction 4CA Maximum Input Level test 7.4A.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | WE Certification, DISH Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 4CA test case for Maximum Input level has core requirements but the test case is missing from 38.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4CA test case for Maximum Input level is added to 38.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 4CA Maximum Input Level cannot be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.4A.2.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 with parameters specified in Table 7.4A.2.5-1 and Table 7.4A.2.5-2 as applicable.

Table 7.4A.2.5-1: Maximum input level for 3DL CA (Intra-band contiguous)

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232-TT | -232-TT | -252-TT |  |
|  |  | -253-TT | -253-TT | -273-TT |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: TT for each frequency is specified in Table 7.4A.2.5-3 for each CC. |  |  |  |  |  |

Table 7.4A.2.5-2: Maximum input level for 3DL CA (Intra-band non-contiguous, Inter-band), per CC

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -252-TT |  |  |  | -242-TT | -232-TT | -222-TT | -212-TT | -202-TT |  |  |  |  |
|  |  | -273-TT |  |  |  | -263-TT | -253-TT | -243-TT | -233-TT | -223-TT |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: TT for each frequency is specified in Table 7.4A.2.5-3 for each CC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.4A.2.5-3: Test Tolerance (Maximum input level), per CC

| f ≤ 3.0GHz | 3.0GHz < f ≤6.0GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

### 7.4A.3	Maximum input level for CA (4DL CA)

#### 7.4A.3.1	Test purpose

The same test purpose as defined in 7.4.1.

#### 7.4A.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 4DL CA.

#### 7.4A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

#### 7.4A.3.4	Test description

##### 7.4A.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.4A.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  | NS_01 by default |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for maximum input level testing. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The DL and UL Reference Measurement Channels are set according to Table 7.4A.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.4A.3.4.3.

##### 7.4A.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 7.4A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.4A.3.4.1-1 as appropriate for PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.4A.3.4.1-1. Since the UE has no payload data and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the value as defined in Table 7.4A.3.5-1 and Table 7.4A.3.5-2 according to the type of CA. Send uplink power control commands to the UE using 1dB step size to ensure that the PCC output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.4A.3.5-1 or Table 7.4A.3.5-2 as appropriate for at least the duration of the Throughput measurement ,where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

7.	Measure the average throughput for the component carrier(s) indicated in table 7.4A.3.4.2-1 for duration sufficient to achieve statistical significance according to Annex H.2A.

8.	Repeat steps 6 to 7 for all component carriers indicated in Table 7.4A.3.4.2-1.

Table 7.4A.3.4.2-1: Test repetition and measurement configuration

| CA configuration | Test ID (NOTE1) | CA configuration ID in REFSENS | Throughput measured on | Table with test parameters to select |
| --- | --- | --- | --- | --- |
| Intra-band contiguous | 1.2 | 15 | PCC,SCC1, SCC2, SCC3 | Table 7.4A.3.5-1 |
| Inter-band | 1,2 | 12, 22, 32 | SCC1, SCC2, SCC3 | Table 7.4A..5-2 |
| Intra-band contiguous + Inter-band | 1,2,3 | 13 | SCC2, SCC3 | Table 7.4A. .5-2 |
|  |  | 23 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-1 |
|  |  | 33 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-1 |
| Intra-band non-contiguous + Inter-band | 1,2,3 | 14 | SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 24 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 34 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-2 |
| NOTE 1: Refers to Test IDs in Table 7.4A.2.4.1-1 NOTE 2: CA configuration ID as defined in “Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band)” in table 7.3A.2.4.1-1. NOTE 3: CA configuration ID as defined in “Default Test Settings for a CA_XC-YA and CA_XB-YA Configurations (Intra-band contiguous + Inter-band)” in table 7.3A.2.4.1-1. NOTE 4: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band)” in table 7.3A.2.4.1-1. NOTE 5: CA configuration ID as defined in “Default Test Settings for a CA_nXD Configuration (Intra-band contiguous)” in table 7.3A.2.4.1-1. |  |  |  |  |

##### 7.4A.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

#### 7.4A.3.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 with parameters specified in Table 7.4A.3.5-1 and Table 7.4A.3.5-2 as applicable.

Table 7.4A.3.5-1: Maximum input level for 4DL CA (Intra-band contiguous)

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232-TT | -232-TT | -252-TT |  |
|  |  | -253-TT | -253-TT | -273-TT |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: TT for each frequency is specified in Table 7.4A.3.5-3 for each CC. |  |  |  |  |  |

Table 7.4A.3.5-2: Maximum input level for 4DL CA (Intra-band non-contiguous, Inter-band), per CC

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -252-TT |  |  |  | -242-TT | -232-TT | -222-TT | -212-TT | -202-TT |  |  |  |  |
|  |  | -273-TT |  |  |  | -263-TT | -253-TT | -243-TT | -233-TT | -223-TT |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: TT for each frequency is specified in Table 7.4A.3.5-3 for each CC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.4A.3.5-3: Test Tolerance (Maximum input level), per CC

| f ≤ 3.0GHz | 3.0GHz < f ≤6.0GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |