3GPP TSG-RAN5 Meeting #90-e	R5-210542

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1986 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MAC RACH Beam Failure test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specific message contents to be used in premble not needed as default value can also be used. Typo errors |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed preamble specific message contents Typo errors corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conforment UE may Fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.1.1.4	Random access procedure / Successful / Beam Failure / Preamble selected by MAC itself / Non Contention Free RACH procedure

7.1.1.1.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with no failureDetectionResources configured and RACH procedure due to beam failure is triggered }

ensure that {

when { contention free random access resources for beam failure recovery request associated with SS blocks are not provided by RRC }

then { UE selects initiates the non-contention free Random Access Procedure }

}

(2)

with { UE in RRC_CONNECTED state and RACH procedure due to beam failure is triggered }

ensure that {

when { contention free random access resources for beam failure recovery request associated with SS blocks are explicitly provided by RRC }

then { UE selects the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SS block and initiates the contention free Random Access Procedure }

}

(3)

with { UE in RRC_CONNECTED state and RACH procedure due to beam failure is triggered }

ensure that {

when { contention free random access resources for beam failure recovery request associated with CSI-RS are explicitly provided by RRC }

then { UE selects the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS and initiates the contention free Random Access Procedure }

}

(4)

with { UE in RRC_CONNECTED state with Preamble transmitted for contention free RACH procedure for beam failure }

ensure that {

when { ra-ResponseWindowBFR expires and the PDCCH addressed to the C-RNTI has not been received }

then { UE retransmits the PRACH Preamble }

}

(5)

with { UE in RRC_CONNECTED state with Preamble transmitted for contention free RACH procedure for beam failure }

ensure that {

when { before expiry of ra-ResponseWindowBFR the PDCCH addressed to the C-RNTI is received }

then { UE considers the RACH procedure to be successfully completed and stops retransmitting PRACH preambles }

}

7.1.1.1.4.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clause 5.1.2, 5.1.3, 5.1.4 and 5.17. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.1.2]

The MAC entity shall:

1>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.17); and

1>	if the beamFailureRecoveryTimer (in subclause 5.17) is either running or not configured; and

1>	if the contention-free Random Access Resources for beam failure recovery request associated with any of the SSBs and/or CSI-RSs have been explicitly provided by RRC; and

1>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB amongst the SSBs in candidateBeamRSList or the CSI-RSs with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the CSI-RSs in candidateBeamRSList is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the SSBs in candidateBeamRSList or a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the CSI-RSs in candidateBeamRSList;

2>	if CSI-RS is selected, and there is no ra-PreambleIndex associated with the selected CSI-RS:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the SSB in candidateBeamRSList which is quasi-collocated with the selected CSI-RS as specified in TS 38.214 [7].

2>	else:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB or CSI-RS from the set of Random Access Preambles for beam failure recovery request.

1>	else if the ra-PreambleIndex has been explicitly provided by PDCCH; and

1>	if the ra-PreambleIndex is not 0b000000:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex;

2>	select the SSB signalled by PDCCH.

1>	else if the contention-free Random Access Resources associated with SSBs have been explicitly provided by RRC and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB.

1>	else if the contention-free Random Access Resources associated with CSI-RSs have been explicitly provided by RRC and at least one CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

1>	else if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if the Random Access Resources for SI request have been explicitly provided by RRC:

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	select a Random Access Preamble corresponding to the selected SSB, from the Random Access Preamble(s) determined according to ra-PreambleStartIndex as specified in TS 38.331 [5];

2>	set the PREAMBLE_INDEX to selected Random Access Preamble.

1>	else (i.e. for the contention-based Random Access preamble selection):

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if Msg3 has not yet been transmitted:

3>	if Random Access Preambles group B is configured:

4>	if the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the first transmission of Msg3.

2>	if the association between Random Access Preambles and SSBs is configured:

3>	select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group.

2>	else:

3>	select a Random Access Preamble randomly with equal probability from the Random Access Preambles within the selected Random Access Preambles group.

2>	set the PREAMBLE_INDEX to the selected Random Access Preamble.

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if ra-AssociationPeriodIndex and si-RequestPeriod are configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to subclause 8.1 of TS 38.213 [6] corresponding to the selected SSB).

1>	else if an SSB is selected above:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to subclause 8.1 of TS 38.213 [6], corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB).

1>	else if a CSI-RS is selected above:

2>	if there is no contention-free Random Access Resource associated with the selected CSI-RS:

3>	determine the next available PRACH occasion from the PRACH occasions, permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, corresponding to the SSB in candidateBeamRSList which is quasi-collocated with the selected CSI-RS as specified in TS 38.214 [7] (the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the SSB which is quasi-collected with the selected CSI-RS).

2>	else:

3>	determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers, corresponding to the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS).

1>	perform the Random Access Preamble transmission procedure (see subclause 5.1.3).

NOTE:	When the UE determines if there is an SSB with SS-RSRP above rsrp-ThresholdSSB or a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS, the UE uses the latest unfiltered L1-RSRP measurement.

[TS 38.321, clause 5.1.4]

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for response to beam failure recovery request identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission is received from lower layers on the Serving Cell where the preamble was transmitted; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see subclause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes a MAC subPDU with RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	indicate the reception of an acknowledgement for SI request to upper layers.

3>	else:

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see subclause 5.2);

5>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP);

5>	if the Serving Cell for the Random Access procedure is SRS-only SCell:

6>	ignore the received UL grant.

5>	else:

6>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received; or

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if the PDCCH addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers;

4>	if this Random Access procedure was triggered for SI request:

5>	consider the Random Access procedure unsuccessfully completed.

3>	else if the Random Access Preamble is transmitted on a SCell:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	if the criteria (as defined in subclause 5.1.2) to select contention-free Random Access Resources is met during the backoff time:

4>	perform the Random Access Resource selection procedure (see subclause 5.1.2);

3>	else:

4>	perform the Random Access Resource selection procedure (see subclause 5.1.2) after the backoff time.

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response transmission.

7.1.1.1.4.3	Test description

7.1.1.1.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.1.4.3.2	Test procedure sequence

Table 7.1.1.1.4.3.2-1/1A illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1"and "T2"are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 7.1.1.1.4.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | E-UTRA Cell 1 | NR Cell 1 | NR Cell 1 Beam index #1 | NR Cell 1 Beam index #0 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | - | - | Beam#1 Switch ON and Beam#0 Switch OFF |
|  | Reference Power | dBm/SCS | - | -88 | - | - |  |
|  | CSI-RS EPRE SS/PBCH SSS EPRE, | dB | - | - | 0 | -57 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | - | - | Beam#1 Switch OFF and Beam#0 Switch ON |
|  | Reference Power | dBm/SCS | - | -88 | - | - |  |
|  | CSI-RS EPRE SS/PBCH SSS EPRE, | dB | - | - | -57 | 0 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | - | - | Beam#1 Switch ON and Beam#0 Switch OFF |
|  | Reference Power | dBm/SCS | - | -88 | - | - |  |
|  | CSI-RS EPRE SS/PBCH SSS EPRE, | dB | - | - | 0 | -57 |  |
| NOTE: "Beam index #1" refers to transmission of the SS/PBCH block with SSB index #1 (according to the ssb-PositionsInBurst) and CSI-RS with index #1 (according to the CSI-MeasConfig being signalled to the UE at step 1/8/17); "Beam index #0" refers to transmission of the SS/PBCH block with SSB index #0 (according to the ssb-PositionsInBurst) and CSI-RS with index #0 (according to the CSI-MeasConfig being signalled to the UE at step 1/8/17). |  |  |  |  |  |  |  |

Table 7.1.1.1.4.3.2-1A: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | E-UTRA Cell 1 | NR Cell 1 | NR Cell 1 Beam index #1 | NR Cell 1 Beam index #0 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -96 | - | - | - | Beam#1 Switch ON and Beam#0 Switch OFF |
|  | Reference Power | dBm/SCS | - | -82 | - | - |  |
|  | CSI-RS EPRE SS/PBCH SSS EPRE, | dB | - | - | 0 | -63 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -96 | - | - | - | Beam#1 Switch OFF and Beam#0 Switch ON |
|  | Reference Power | dBm/SCS | - | -82 | - | - |  |
|  | CSI-RS EPRE SS/PBCH SSS EPRE, | dBm/SCS | - | - | -63 | 0 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -96 | - | - | - | Beam#1 Switch ON and Beam#0 Switch OFF |
|  | Reference Power | dBm/SCS | - | -82 | - | - |  |
|  | CSI-RS EPRE SS/PBCH SSS EPRE, | dBm/SCS | - | - | 0 | -63 |  |
| NOTE: "Beam index #1" refers to transmission of the SS/PBCH block with SSB index #1 (according to the ssb-PositionsInBurst) and CSI-RS with index #1 (according to the CSI-MeasConfig being signalled to the UE at step 1/8/17); "Beam index #0" refers to transmission of the SS/PBCH block with SSB index #0 (according to the ssb-PositionsInBurst) and CSI-RS with index #0 (according to the CSI-MeasConfig being signalled to the UE at step 1/8/17). |  |  |  |  |  |  |  |

Table 7.1.1.1.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an NR RRCReconfiguration message to configure parameters for BFR. Note 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | UE responses NR RRCReconfigurationComplete message. Note 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS changes NR Cell 1 power level according to the row "T1" in table 7.1.1.1.4.3.2-1/1A. | - | - | - | - |
| 4 | Check: Does the UE transmit a preamble on PRACH for the non-contention free Random Access Procedure on NR Cell 1 Beam index #1? | --> | PRACH Preamble | 1 | P |
| 5 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing multiple RAR’s and one of the MAC sub headers contains a matching RAPID on NR Cell 1. | <-- | Random Access Response | - | - |
| 6 | UE sends a msg3 using the grant associated to the Random Access Response received in Step 5 on NR Cell 1. | --> | msg3 (C-RNTI MAC CONTROL ELEMENT) | - | - |
| 7 | SS schedules PDCCH transmission for UE C-RNTI. | <-- | Contention Resolution | - | - |
| 8 | The SS transmits an NR RRCReconfiguration to establish random access resources for BFR associated with SS blocks explicitly. Note 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 9 | UE responses NR RRCReconfigurationComplete message. Note 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 10 | The SS changes NR Cell 1 power level according to the row "T2" in table 7.1.1.1.4.3.2-1/1A. | - | - | - | - |
| 11 | Check: Does the UE transmit preamble on PRACH using a preamble with PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SS block provided by RRC on NR Cell 1 Beam index #0? | --> | PRACH Preamble | 2 | P |
| 12 | The SS waits for ra-ResponseWindowBFR expire. NOTE: The SS does not transmit Random Access Response to the UE. | - | - | - | - |
| 13 | Check: Does the UE retransmit a preamble on PRACH with ra-PreambleIndex same as the Step 11? | --> | PRACH Preamble | 4 | P |
| 14 | The SS transmits a MAC PDU addressed to UE C-RNTI, containing multiple RAR’s and one of the MAC sub headers contains a matching RAPID on NR Cell 1. | <-- | Random Access Response | - | - |
| 15 | The SS waits for ra-ResponseWindowBFR expire. | - | - | - | - |
| 16 | Check: Does the UE retransmit a preamble on PRACH? | - | - | 5 | F |
| - | EXCEPTION: Steps 17 to 25 describe behaviour that depends on the UE capability. | - | - | - | - |
| 17 | IF pc_csi_RS_CFRA_ForHO THEN the SS transmits an NR RRCReconfiguration message to establish random access resources for BFR associated with CSI-RS explicitly. Note 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 18 | UE responses NR RRCReconfigurationComplete message. Note 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 19 | The SS changes NR Cell 1 power level according to the row "T1" in table 7.1.1.1.4.3.2-1. | - | - | - | - |
| 20 | Check: Does the UE transmit preamble on PRACH using a preamble with PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS provided by RRC on NR Cell 1 Beam index #1? | --> | PRACH Preamble | 3 | P |
| 21 | The SS waits for ra-ResponseWindowBFR expire. NOTE: The SS does not transmit Random Access Response to the UE. | - | - | - | - |
| 22 | Check: Does the UE retransmit a preamble on PRACH with ra-PreambleIndex same as the Step 20? | --> | PRACH Preamble | 4 | P |
| 23 | The SS transmits a MAC PDU addressed to UE C-RNTI, containing multiple RAR’s and one of the MAC sub headers contains a matching RAPID on NR Cell 1. | <-- | Random Access Response | - | - |
| 24 | The SS waits for ra-ResponseWindowBFR expire. | - | - | - | - |
| 25 | Check: Does the UE retransmit a preamble on PRACH? | - | - | 5 | F |
| Note 1: for EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: for EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.1.4.3.3	Specific message contents

Table 7.1.1.1.4.3.3-1: Void

Table 7.1.1.1.4.3.3-2: Void

Table 7.1.1.1.4.3.3-3: RRCReconfiguration (Step 1, Step8, Step17 Table 7.1.1.1.4.3.2-2)

| Derivation path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration::=SEQUENCE{ |  |  |  |  |
| criticalExtensions CHOICE{ |  |  |  |  |
| rrcReconfiguration SEQUENCE{ |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig | OCTET STRING | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.1.4.3.3-4: CellGroupConfig (Table 7.1.1.1.4.3.3-3: RRCReconfiguration)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-5: ServingCellConfig (Table 7.1.1.1.4.3.3-4: CellGroupConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| csi-MeasConfig CHOICE { |  |  | Step 1 |
| setup | CSI-MeasConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-6: BWP-DownlinkDedicated (Table 7.1.1.1.4.3.3-5: ServingCellConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config | Not present |  | Step 17 |
| pdcch-Config CHOICE { |  |  | Step 1, Step 8 |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config |  |  |
| } |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| setup | RadioLinkMonitoringConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-7: RadioLinkMonitoringConfig (Table 7.1.1.1.4.3.3-6: BWP-DownlinkDedicated)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS { | 2 entries |  |  |
| RadioLinkMonitoringRS[1] SEQUENCE { |  | entry 1 |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | rlf |  | Step 1, Step 17 |
|  | both |  | Step 8 |
| detectionResource CHOICE { |  |  |  |
| csi-rs | 0 | NR Cell 1 Beam index #0 |  |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  | entry 2 |  |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  | Step 1, Step 8 |
|  | both |  | Step 17 |
| detectionResource CHOICE { |  |  |  |
| csi-rs | 1 | NR Cell 1 Beam index #1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| beamFailureInstanceMaxCount | n1 |  |  |
| beamFailureDetectionTimer | pbfd1 |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-8: PDSCH-Config (Table 7.1.1.1.4.3.3-6: BWP-DownlinkDedicated)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 3 entries |  |  |
| TCI-State[1] SEQUENCE { |  | entry 1 |  |
| tci-StateId | 0 |  |  |
| qcl-type1 SEQUENCE { |  |  |  |
| cell | ServCellIndex of NR SpCell | Cell ID |  |
| bwp-id | 0 | BWP ID |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | 1 | SSB index #1 |  |
| } |  |  |  |
| qcl-Type | type C |  |  |
| } |  |  |  |
| qcl-type2 | Not present |  |  |
| qcl-type2 SEQUENCE { |  |  | FR2 |
| cell | ServCellIndex of NR SpCell | Cell ID |  |
| bwp-id | 0 | BWP ID |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | 1 | SSB index #1 |  |
| } |  |  |  |
| qcl-Type | type D |  |  |
| } |  |  |  |
| } |  |  |  |
| TCI-State[2] SEQUENCE { |  | entry 2 |  |
| tci-StateId | 1 |  |  |
| qcl-type1 SEQUENCE { |  |  |  |
| cell | ServCellIndex of NR SpCell | Cell ID |  |
| bwp-id | 0 | BWP ID |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | 0 | SSB index #0 |  |
| } |  |  |  |
| qcl-Type | type C |  |  |
| } |  |  |  |
| qcl-type2 | Not present |  |  |
| qcl-type2 SEQUENCE { |  |  | FR2 |
| cell | ServCellIndex of NR SpCell | Cell ID |  |
| bwp-id | 0 | BWP ID |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | 0 | SSB index #0 |  |
| } |  |  |  |
| qcl-Type | type D |  |  |
| } |  |  |  |
| } |  |  |  |
| TCI-State[3] SEQUENCE { |  | entry 3 |  |
| tci-StateId | 2 |  |  |
| qcl-type1 SEQUENCE { |  |  |  |
| cell | ServCellIndex of NR SpCell | Cell ID |  |
| bwp-id | 0 | BWP ID |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 1 | Csi-Rs index #1 |  |
| } |  |  |  |
| qcl-Type | type A |  |  |
| } |  |  |  |
| qcl-type2 | Not present |  |  |
| qcl-type2 SEQUENCE { |  |  | FR2 |
| cell | ServCellIndex of NR SpCell | Cell ID |  |
| bwp-id | 0 | BWP ID |  |
| referenceSignal CHOICE { |  |  |  |
| Csi-rs | 1 | Csi-Rs index #1 |  |
| } |  |  |  |
| qcl-Type | type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-9: PDCCH-Config (Table 7.1.1.1.4.3.3-6: BWP-DownlinkDedicated)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[1] | ControlResourceSetid1 | entry 1 |  |
| ControlResourceSet[2] | ControlResourceSetid2 | entry 2 |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace with condition USS | entry 1 |  |
| SearchSpace[2] | SearchSpaceBFR | entry 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-10: ControlResourceSetId1 (Table 7.1.1.1.4.3.3-9: PDCCH-Config)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| tci-StatesPDCCH-ToAddList SEQUENCE (SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId { | 1 entry |  | Step 1 |
| TCI-StateId[1] | 2 | entry 1 TCI-State Id 2 |  |
| } |  |  |  |
| tci-StatesPDCCH-ToReleaseList SEQUENCE (SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId { | 1 entry |  | Step 8 |
| TCI-StateId[1] | 2 | entry 1 TCI-State Id 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-11: ControlResourceSetId2 (Table 7.1.1.1.4.3.3-9: PDCCH-Config)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-12: SearchSpaceBFR (Table 7.1.1.1.4.3.3-9: PDCCH-Config)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 4 |  |  |
| controlResourceSetId | 2 |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  |  |
| dci-Formats | formats0-0-And-1-0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-13: CSI-MeasConfig (Table 7.1.1.1.4.3.3-5: ServingCellConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE { | 2 entries |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-ResourceId0 |  |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-ResourceId1 |  |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSetid0 |  |  |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ReportConfigToAddModList | Not present |  |  |
| reportTriggerSize | Not present |  |  |
| aperiodicTriggerStateList | Not present |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-14: NZP-CSI-RS-ResourceId0 (Table 7.1.1.1.4.3.3-13: CSI-MeasConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 0 |  |  |
| resourceMapping | CSI-RS-ResourceMapping with condition TRS | TS 38.508-1 [4], Table 4.6.3-45 |  |
| qcl-InfoPeriodicCSI-RS | 0 | QCL to SSB #0 |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-15: NZP-CSI-RS-ResourceId1 (Table 7.1.1.1.4.3.3-13: CSI-MeasConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 1 |  |  |
| resourceMapping | CSI-RS-ResourceMapping with condition TRS | TS 38.508-1 [4], Table 4.6.3-45 |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset_Id1 |  |  |
| qcl-InfoPeriodicCSI-RS | 1 | QCL to SSB #1 |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-16: CSI-ResourcePeriodicityAndOffset_Id1 (Table 7.1.1.1.4.3.3-15: NZP-CSI-RS-ResourceId1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots80 | 11 |  | FR1 |
| slots320 | 41 |  | FR2 |
| } |  |  |  |

Table 7.1.1.1.4.3.3-17: NZP-CSI-RS-ResourceSetid0 (Table 7.1.1.1.4.3.3-13: CSI-MeasConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | 0 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2 |  |
| } |  |  |  |
| trs-Info | true |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-18: BWP-UplinkDedicated (Table 7.1.1.1.4.3.3-5: ServingCellConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| beamFailureRecoveryConfig | BeamFailureRecoveryConfig_SSB |  | Step8 |
|  | BeamFailureRecoveryConfig_CSIRS |  | Step17 |
|  | Not Present |  | Step1 |
| } |  |  |  |

Table 7.1.1.1.4.3.3-19: BeamFailureRecoveryConfig_SSB (Table 7.1.1.1.4.3.3-18: BWP-UplinkDedicated)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryConfig ::= SEQUENCE { |  |  |  |
| rootSequenceIndex-BFR | 0 | See TS 38.508-1[4] clause 4.4.2, Table 4.4.2-2 |  |
| rach-ConfigBFR | RACH-ConfigGeneric | 38.508-1[4] Table 4.6.3-130 |  |
| rsrp-ThresholdSSB | 57(-100dBm) |  |  |
| candidateBeamRSList SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR CHOICE{ |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb | 1 | NR Cell Beam#1 |  |
| ra-PreambleIndex | 56 | (0..63) |  |
| } |  |  |  |
| } |  |  |  |
| ssb-perRACH-Occasion | one |  |  |
| ra-ssb-OccasionMaskIndex | 0 |  |  |
| recoverySearchSpaceID | 4 |  |  |
| ra-Prioritization | Not Present |  |  |
| beamFailureRecoveryTimer | ms200 |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-20: BeamFailureRecoveryConfig_CSIRS (Table 7.1.1.1.4.3.3-18: BWP-UplinkDedicated)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryConfig ::= SEQUENCE { |  |  |  |
| rootSequenceIndex-BFR | 0 | See TS 38.508-1[4] clause 4.4.2, Table 4.4.2-2 |  |
| rach-ConfigBFR | RACH-ConfigGeneric | 38.508-1[4] Table 4.6.3-130 |  |
| rsrp-ThresholdSSB | 57(-100dBm) |  |  |
| candidateBeamRSList SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR CHOICE{ |  |  |  |
| csi-RS SEQUENCE { |  |  |  |
| csi-RS | 0 |  |  |
| ra-OccasionList SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1) { | 1 entry |  |  |
| INTEGER[1] | 0 | entry 1 NR Cell Beam#0 |  |
| } |  |  |  |
| ra-PreambleIndex | 59 |  |  |
| } |  |  |  |
| } |  |  |  |
| ssb-perRACH-Occasion | Not Present |  |  |
| ra-ssb-OccasionMaskIndex | Not Present |  |  |
| recoverySearchSpaceID | 4 |  |  |
| ra-Prioritization | Not Present |  |  |
| beamFailureRecoveryTimer | ms200 |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-21: PUCCH-Config (Table 7.1.1.1.4.3.3-18: BWP-UplinkDedicated)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| pucch-Config::= SEQUENCE { |  |  |  |
| pucch-PowerControl SEQUENCE { |  |  |  |
| pathlossReferenceRSs SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS { | 1 entry |  |  |
| PUCCH-PathlossReferenceRS[1] SEQUENCE { |  | entry 1 |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | 1 |  | Step1,Step17 |
|  | 0 |  | Step8 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.4.3.3-22: PUSCH-Config (Table 7.1.1.1.4.3.3-18: BWP-UplinkDedicated)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| pusch-Config::= SEQUENCE { |  |  |  |
| pusch-PowerControl SEQUENCE { |  |  |  |
| pathlossReferenceRSToAddModList SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS { | 1 entry |  |  |
| PUSCH-PathlossReferenceRS[1] SEQUENCE { |  | entry 1 |  |
| referenceSignal CHOICE{ |  |  |  |
| ssb-Index | 1 |  | Step1,Step17 |
|  | 0 |  | Step8 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |