3GPP TSG-RAN5 Meeting #90-e	R5-210632

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2006 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 6.2.3.4-N2L cell reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The value of Qqualmin for NR Cell 1 configured in 6.2.3.4.3.3-1 is different from that set in Table 6.2.3.4.3.2-1. - After step 6 and before step 7, UE may reselect back to E-UTRA Cell 1. The reasons are as follows: - The prority of serving cell (NR Cell 1) is 4, and the prority of neighbour cell (E-UTRA Cell 1) is 3. - For serving cell (NR Cell 1), Squal-NR Cell 1< threshServingLow-Q is satisfied, where Squal-NR Cell 1=13.04 and threshServingLow-Q=26. - iii)For neighbour cell (E-UTRA Cell 1), Squal-E-UTRA Cell 1 > threshX-Low-Q is satisfied, where Squal-E-UTRA Cell 1=8.56 and threshX-Low-Q=5. - After step 8 and before step 10, UE may reselect back to E-UTRA Cell 1. The reasons are as follows: - The prority of serving cell (NR Cell 1) is 4, and the prority of neighbour cell (E-UTRA Cell 1) is 5. - For neighbour cell (E-UTRA Cell 1), Squal-E-UTRA Cell 1 > threshX-High-Q is satisfied, where Squal-E-UTRA Cell 1=8.56 and threshX-High-Q=5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the value of q-QualMin IE in 6.2.3.4.3.3-1 from -20 to -25. - Modify the value of threshX-LowQ IE in Table 6.2.3.4.3.3-3 from 5 to10 to avoid the issue for Reason 2. - Modify the value of threshX-HighQ-r15 IE in Table 6.2.3.4.3.3-6 from 5 to 10 to avoid the issue for Reason 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.2.3.4	Inter-RAT cell reselection / From NR RRC_Idle to E-UTRA_IDLE (lower priority & higher priority, Squal based)

6.2.3.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority inter-RAT E-UTRA cell }

then { UE reselects the cell which belongs to the lower priority inter-RAT E-UTRA cell }

}

(2)

with { UE in NR RRC_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell }

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.304, clause 5.2.4.1, 5.2.4.2 and 5.2.4.5. Unless otherwise stated these are Rel-15 requirements.

[TS 38.304, clause 5.2.4.1]

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values).

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

In case UE receives RRCRelease with deprioritisationReq, UE shall consider current frequency and stored frequencies due to the previously received RRCRelease with deprioritisationReq or all the frequencies of NR to be the lowest priority frequency (i.e. lower than any of the network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection is performed on request by NAS (TS 23.122 [9]).

NOTE:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 38.133 [8] are still applicable.

The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters a different RRC state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS (TS 23.122 [9]).

NOTE 2:	Equal priorities between RATs are not supported.

The UE shall not consider any black listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e. T320 in NR and E-UTRA), if configured, at inter-RAT cell (re)selection.

NOTE 3:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 38.304, clause 5.2.4.1]

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority;

-	Otherwise, the UE shall perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 38.133 [8].

[TS 38.304, clause 5.2.4.5]

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority NR or EUTRAN RAT/frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT

Otherwise, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority NR frequency shall be based on ranking for intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority NR or E-UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/frequency shall take precedence over a lower priority RAT/frequency if multiple cells of different priorities fulfil the cell reselection criteria.

If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an NR frequency, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6;

-	If the highest-priority frequency is from another RAT, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

6.2.3.4.3	Test description

6.2.3.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and E-UTRA Cell 1

-	System information combination NR-6 as defined in TS 38.508-1 [7] clause 4.4.3.1, and message contents defined in clause 4.6.1 and clause 4.6.2 with QBASED condition is used in NR cell.

-	System information combination 31 as defined in TS 36.508 [7] clause 4.4.3.1, and message contents defined in clause 4.4.3.3 with QBASED condition is used in E-UTRA cell.

UE:

None.

Preamble:

With E-UTRA Cell 1 "Serving cell" and NR Cell 1 "Non-suitable "Off" cell" in accordance with TS 38.508-1 [4], Table 6.2.2.1-3, the UE is brought to state RRC_IDLE Connectivity (E) in accordance with the procedure described in TS 38.508-1 [4], Table 4.5.2.2-1. 4G-GUTI and eKSI are assigned and security context established

The UE is switched-off.

With E-UTRA Cell 1 "Non-suitable "Off" cell" and NR Cell 1 "Serving cell" in accordance with TS 38.508-1 [4], Table 6.2.2.1-3, the UE is brought to state 1N-A, RRC_IDLE Connectivity (NR), in accordance with the procedure described in TS 38.508-1 [4], Table 4.5.2.2-2. 5G-GUTI and ngKSI are assigned and security context established..

6.2.3.4.3.2	Test procedure sequence

Table 6.2.3.4.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1","T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.2.3.4.3.2-3.

Table 6.2.3.4.3.2-1: Time instances of cell power level and parameter changes for NR Cell (FR1)

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | - | -80 | The power level values are assigned to ensure UE reselected to E-UTRA Cell 1: both Squal ServingCell, NR Cell1 < ThreshServing, LowQ and SqualnonServingCell, E-UTRA Cell1 > ThreshX, LowQ |
|  | SS/PBCH SSS EPRE | dBm/SCS | -95 | - |  |
|  | RSRQ | dB | -21.20 | -3.76 |  |
|  | Qqualmin | dB | -25 | -20 |  |
|  | NocE-UTRA | dBm/15kHz | - | -95 |  |
|  | NocNR | dBm/SCS | -85 | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | - | -95 | The power level values are assigned to ensure UE reselected back to NR Cell 1: Squal nonServingCell, E-UTRA Cell1 > ThreshX, HighQ |
|  | SS/PBCH SSS EPRE | dBm/SCS | -80 | - |  |
|  | RSRQ | dB | -11.96 | -11.46 |  |
|  | Qqualmin | dB | -25 | -20 |  |
|  | NocE-UTRA | dBm/15kHz | - | -95 |  |
|  | NocNR | dBm/SCS | -85 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | - | -80 | The power level values are assigned to ensure UE reselected to E-UTRA Cell 1: Squal nonServingCell, NR Cell1 > ThreshX, HighQ |
|  | SS/PBCH SSS EPRE | dBm/SCS | -95 | - |  |
|  | RSRQ | dB | -21.20 | -3.76 |  |
|  | Qqualmin | dB | -25 | -20 |  |
|  | NocE-UTRA | dBm/15kHz | - | -95 |  |
|  | NocNR | dBm/SCS | -85 | - |  |

Table 6.2.3.4.3.2-2: Time instances of cell power level and parameter changes for NR Cell (FR2)

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | - | FFS | The power level values are assigned to ensure UE reselected to E-UTRA Cell 1: both Squal ServingCell, NR Cell1 < ThreshServing, LowQ and SqualnonServingCell, E-UTRA Cell1 > ThreshX, LowQ |
|  | SS/PBCH SSS EPRE | dBm/SCS | FFS | - |  |
|  | RSRQ | dB | FFS | FFS |  |
|  | Qqualmin | dB | -25 | -20 |  |
|  | NocE-UTRA | dBm/15kHz | - | FFS |  |
|  | NocNR | dBm/SCS | FFS | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | - | FFS | The power level values are assigned to ensure UE reselected back to NR Cell 1: Squal nonServingCell, E-UTRA Cell1 > ThreshX, HighQ |
|  | SS/PBCH SSS EPRE | dBm/SCS | FFS | - |  |
|  | RSRQ | dB | FFS | FFS |  |
|  | Qqualmin | dB | -25 | -20 |  |
|  | NocE-UTRA | dBm/15kHz | - | FFS |  |
|  | NocNR | dBm/SCS | FFS | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | - | FFS | The power level values are assignee to ensure UE reselected to E-UTRA Cell 1: Squal nonServingCell, NR Cell1 > ThreshX, HighQ |
|  | SS/PBCH SSS EPRE | dBm/SCS | FFS | - |  |
|  | RSRQ | dB | FFS | FFS |  |
|  | Qqualmin | dB | -25 | -20 |  |
|  | NocE-UTRA | dBm/15kHz | - | FFS |  |
|  | NocNR | dBm/SCS | FFS | - |  |

Table 6.2.3.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T1" in table 6.2.3.4.3.2-1/2/3. | - | - | - | - |
| 2 | Check: Does the test result of test steps 1 to 6 of generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2 indicate that the UE is camped on E-UTRA Cell 1? | - | - | 1 | - |
| 3 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE on E-UTRAN Cell 1. | <-- | E-UTRA RRC: RRCConnectionRelease | - | - |
| 4 | The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T2" in table 6.2.3.4.3.2-1/2/3. | - | - | - | - |
| 5 | The UE selects NR Cell 1 and performs the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 with condition 'connected without release'. | - | - | - | - |
| 6 | The SS transmits an RRCRelease message on NR Cell 1. | <-- | NR RRC: RRCRelease | - | - |
| 7 | The SS changes the E-UTRA cell priority broadcast in system information on NR Cell 1. | - | - | - | - |
| 8 | Notify UE change of System Information on NR Cell 1. | <-- | NR RRC: ShortMessage |  |  |
| 9 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 10 | The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T3" in table 6.2.3.4.3.2-1/2/3. | - | - | - | - |
| 11 | Check: Does the test result of test steps 1 to 6 of generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2 indicate that the UE is camped on E-UTRA Cell 1? | - | - | 2 | - |
| 12 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE on E-UTRAN Cell 1. | <-- | E-UTRA RRC: RRCConnectionRelease | - | - |

6.2.3.4.3.3	Specific message contents

Table 6.2.3.4.3.3-1:SIB1 for NR Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-QualMin | -25 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-2:SIB2 for NR Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLowQ | 26 |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-3:SIB5 NR Cell 1 (preamble, Table 6.2.3.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink E-UTRA ARFCN for E-UTRA Cell 1 |  |  |
| threshX-Q SEQUENCE { |  |  |  |
| threshX-HighQ | 20 |  |  |
| threshX-LowQ | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-4: SystemInformationBlockType1 for E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 36.508-1 [7], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-5:SystemInformationBlockType3 for E-UTRA Cell 1 preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| q-QualMin-r9 | -20 |  |  |
| threshServingLowQ-r9 | 26 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-6:SystemInformationBlockType24 for E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
| cellReselectionPriority-r15 | 5 |  |  |
| threshX-Q-r15 SEQUENCE { |  |  |  |
| threshX-HighQ-r15 | 10 |  |  |
| threshX-LowQ-r15 | 20 |  |  |
| } |  |  |  |
| q-QualMin-r15 | -25 |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-7: SIB5 NR Cell 1 (step 7, Table 6.2.3.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink E-UTRA ARFCN for E-UTRA Cell 1 |  |  |
| cellReselectionPriority | 5 |  |  |
| q-QualMin | -20 |  |  |
| threshX-Q SEQUENCE { |  |  |  |
| threshX-HighQ | 5 |  |  |
| threshX-LowQ | 20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-8: SIB1 of NR Cell 1 (step 9, Table 6.2.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| si-SchedulingInfo | SI-SchedulingInfo |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-9: SI-SchedulingInfo (si-SchedulingInfo in Table 6.2.3.4.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI-SchedulingInfo ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE(1..maxSI-Message)) OF SchedulingInfo { | 1 entry |  |  |
| SchedulingInfo[1] SEQUENCE { |  | entry 1 |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| type | sibType5 |  |  |
| valueTag | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-10: REGISTRATION REQUEST (Step 5, Table 6.2.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 6.2.3.4.3.3-11 |  |

Table 6.2.3.4.3.3-11: TRACKING AREA UPDATE REQUEST (Table 6.2.3.4.3.3-10)

Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-2 with condition Mapped EPS security context

<End of modified section 1>