3GPP TSG-RAN5 Meeting #90-e	R5-210684

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5009 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE TC 6.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, TC 6.1.2.3 is designed for those band which has only one test frequency. In TC 6.1.2.3, Cell 1 and 2 are used. However, according to 36.523-3 table 7.4.3.1-1, Both Cell 1 and 2 have even SFN offset for TDD. As a result, SIB1 of Cell 1 and 2 overlap in time and cause mutual interference to each other. Then for those UE which equips only one Rx antenna (i.e. Cat.1bis UE) Es/Iot of these cells may be too low for SIB1 reception. If UE fails to receive SIB1, UE will consider these cells are barred (36.331 cl. 5.2.2.5) and may exclude this cell during cell reselection for up to 300s (36.304 cl. 5.3.1). Then it will fail the test if UE is required to reselect to this cell in test procedure. To avoid this issue we suggest replace Cell 2 with Cell 11, Then there is no overlapping between SIBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Cell 2 is replaced with Cell 11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.2.3	Cell selection / Intra E-UTRAN / Serving cell becomes non-suitable (S<0 or barred)

6.1.2.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable (S<0)and there is a suitable neighbour cell (S>0) }

then { UE selects the suitable neighbour cell }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { the serving cell becomes barred and there is a suitable neighbour cell}

then { UE selects the suitable neighbour cell }

}

6.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.1 and TS 36.304, clause 4.1, 5.2.1, 5.2.3.2, 5.2.4.6 and 5.3.1.

[TS 36.300, clause 10.1.1.1]

Cell selection:

-	The UE NAS identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn ("initial cell selection") or make use of stored information to shorten the search ("stored information cell selection").

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of "forbidden tracking areas for roaming";

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred;

[TS 36.304, clause 4.1]

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE will, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed.

[TS 36.304, clause 5.2.1]

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Where:

Where:

the signalled value QrxlevminOffset is only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

| Srxlev | Cell Selection RX level value (dB) |
| --- | --- |
| Qrxlevmeas | Measured cell RX level value (RSRP). |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | max(PEMAX – PUMAX, 0) (dB) |
| PEMAX | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101] |
| PUMAX | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 36.304, clause 5.2.4.6]

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.304, clause 5.3.1]

Cell status and cell reservations are indicated in the SystemInformationBlockType1 [3] by means of two Information Elements:

-	cellBarred (IE type: "barred" or "not barred") 
In case of multiple PLMNs indicated in SIB1 sharing, this IE is common for all PLMNs

...

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell re-selection procedures.

...

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/re-select this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is a CSG cell:

-	the UE may select another cell on the same frequency if the selection/reselection criteria are fulfilled.

-	else

-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the IE intraFreqReselection in IE cellAccessRelatedInfo in SystemInformationBlockType1 is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell.

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

6.1.2.3.3	Test description

6.1.2.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 11 have different tracking areas according to table 6.0.1-2.

UE:

None.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1(serving cell) according to [18].

6.1.2.3.3.2	Test procedure sequence

Table 6.1.2.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.3.3.2-2.

Table 6.1.2.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE (FDD/TDD) | dBm/15kHz | "Off" | -85 | SrxlevCell 1 < 0 (NOTE 1) |
|  | Srxlev* | dB | - | 25 | Cell 11 becomes the strongest cell |
| T2 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -91 | -85 | SrxlevCell 11 > 0, SrxlevCell 1 > 0 , RCell 1 < RCell 11 |
|  | Srxlev* (FDD) | dB | 19 | 25 |  |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -89 | -85 |  |
|  | Srxlev* (TDD) | dB | 21 | 25 | Cell 1 becomes suitable |
|  | cellBarred | - | notBarred | barred | Serving cell becomes barred |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

NOTE: If the UE fails the test because of a failure to detect and reselect to a right cell, then the operator may re-run the test.

Table 6.1.2.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal level of Cell 1 and Cell 11 according to row "T1" in table 6.1.2.3.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 11? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 3 | SS changes Cell 1 signal level and SIB1 IE cellBarred according to row "T2" in table 6.1.2.3.3.2-1 and transmits a Paging message including systemInfoModification. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | - |

6.1.2.3.3.3	Specific message contents

Table 6.1.2.3.3.3-1: Void

Table 6.1.2.3.3.3-2: SystemInformationBlockType1 for cells 1 and 11 (preamble and all steps, Table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -55 (-110 dBm) |  |  |
| } |  |  |  |
| systemInfoValueTag | The value is increased in step 3 |  |  |
| } |  |  |  |

Table 6.1.2.3.3.3-2A: SystemInformationBlockType1-BR-r13 for cells 1 and 11 (preamble and all steps when UE under test is CAT M1, Table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -55 (-110 dBm) |  |  |
| } |  |  |  |
| systemInfoValueTag | The value is increased in step 3 |  |  |
| } |  |  |  |

Table 6.1.2.3.3.3-3: SystemInformationBlockType3 for cells 1 and 11 (Preamble and all steps, table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst | dB0 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -55 (-110 dBm) |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.3.3.3-4: SystemInformationBlockType1 for cell 11 (step 3, Table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellBarred | barred |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.1.2.3.3.3-4A: SystemInformationBlockType1-BR-r13 for cell 11 (step 3 when UE under test is CAT M1, Table 6.1.2.3.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellBarred | barred |  |  |
| intraFreqReselection | allowed |  |  |
| } |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.1.2.3.3.3-5: Paging (step 3, Table 6.1.2.3.3.2-2)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | True |  |  |
| etws-PrimaryNotificationIndication | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

<End of modified section 1>