3GPP TSG-RAN5 Meeting #90-e	R5-210685

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5010 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE TC 6.1.2.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, TC 6.1.2.12 is designed for those band which has only one test frequency. In TC 6.1.2.12, Cell 1, 2 and 4 are used. However, according to 36.523-3 table 7.4.3.1-1, Cell 1, 2 and 4 all have even SFN offset for TDD. As a result, SIB1 of Cell 1, 2 and 4 overlap in time and cause mutual interference to each other. Then for those UE which equips only one Rx antenna (i.e. Cat.1bis UE) Es/Iot of these cells may be too low for SIB1 reception. If UE fails to receive SIB1, UE will consider these cells are barred (36.331 cl. 5.2.2.5) and may exclude this cell during cell reselection for up to 300s (36.304 cl. 5.3.1). Then it will fail the test if UE is required to reselect to this cell in test procedure. To avoid this issue we suggest replace Cell 2 with Cell 11, Then there is only one cell with even SFN offset and one cell with odd SFN offset at the same time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Cell 2 is replaced with Cell 11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.2.12	Cell reselection / Cell-specific reselection parameters provided by the network in a neighbouring cell list

6.1.2.12.1	Test Purpose

(1)

with { the UE is in RRC_IDLE and SystemInformationBlockType4 contain a cell-specific Qoffset for a neighbour intra frequency cell }

ensure that {

when { the neighbour cell has lower power than the serving cell but it is higher ranked due to the cell-specific Qoffset }

then { the UE reselects the neighbour cell with cell-specific Qoffset }

}

(2)

with { the UE is in RRC_IDLE and SystemInformationBlockType4 contain a black listed cell }

ensure that {

when { a black listed cell becomes higher ranked than the serving cell }

then { the UE remains camped on the serving cell }

}

6.1.2.12.2	Conformance Requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.304, clauses 5.2.4.1 and 5.2.4.6.

[TS 36.304 clause 5.2.4.1]

The UE shall not consider any black listed cells as candidate for cell reselection.

[TS 36.304 clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

6.1.2.12.3	Test description

6.1.2.12.3.1	Pre-test conditions

System Simulator:

-	cells 1, 4, and 11 on the same E-UTRA frequency and in different tracking areas

-	only 2 cells are simultaneously active, Cell 4 can be implemented as minimum uplink cell

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	cell 1 is transmitting SystemInformationBlockType4 according to specific message contents (Qoffset1,11 is -24dB)

-	cell 11 is transmitting SystemInformationBlockType4 according to specific message contents (Qoffset11,1 is 24dB, cell 4 is on black list)

NOTE:	Qoffset11,1 does not serve any test purpose, but symmetrical Qoffset values may be more realistic.

UE:

None.

Preamble:

-	the UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

6.1.2.12.3.2	Test procedure sequence

Table 6.1.2.12.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Rows marked "T0" denote the conditions after the preamble, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.1.2.12.3.2-1: Time instances of cell power level and parameter change

| Ti | Parameter | Unit | Cell 1 | Cell 11 | Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | Off | Off | Only Cell 1 is on |
| T1 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -85 | -91 | Off | Cell 11 has lower power but is higher ranked due to cell-specific Qoffset1,11 |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -85 | -89 | Off |  |
| T2 | Cell-specific RS EPRE (FDD) | dBm/15kHz | Off | -91 | Off |  |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | Off | -89 | Off |  |
| T3 | Cell-specific RS EPRE (FDD) | dBm/15kHz | Off | -91 | -85 | Cell 4 has higher power than Cell 11 but is black listed |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | Off | -89 | -85 |  |

Table 6.1.2.12.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the cell-specific reference signal levels according to row "T1" in table 6.1.2.12.3.2-1. | - | - | - | - |
| 2 | Void |  |  |  |  |
| 2A-2E | Check: Does the test result of test steps 1 to 5 of generic procedure 6.4.2.7 indicate that the UE is camped on Cell 11? | - | - | 1 | - |
| 2F | The SS re-adjusts the cell-specific reference signal levels according to row "T2" in table 6.1.2.12.3.2-1 | - | - | - | - |
| 2G | The SS waits for 1s. Note: Delay is added to ensure cell power levels set at T2 has settled on correct level (36.523-3 cl. 7.4.2) before RRC connection is released. | - | - | - | - |
| 2H | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| 3 | The SS re-adjusts the cell-specific reference signal levels according to row "T3" in table 6.1.2.12.3.2-1. | - | - | - | - |
| 4 | Check: Does the UE initiate a random access procedure on Cell 4 within the next 120s? | - | - | 2 | F |

6.1.2.12.3.3	Specific message contents

Table 6.1.2.12.3.3-1: SystemInformationBlockType4 for cell 1 (all steps, Table 6.1.2.12.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellID of Cell 11 |  |  |
| q-OffsetCell[1] | dB-24 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.12.3.3-2: SystemInformationBlockType4 for cell 11 (all steps, Table 6.1.2.12.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList { | 1 entry |  |  |
| physCellId[1] | PhysicalCellID of Cell 1 |  |  |
| q-OffsetCell[1] | dB24 |  |  |
| } |  |  |  |
| intraFreqBlackCellList SEQUENCE { | 1 entry |  |  |
| start[1] | PhysicalCellID of Cell 4 |  |  |
| range[1] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>