3GPP TSG-RAN5 Meeting #90-e	R5-210699

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1724 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of V2X NAS IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, V2X NAS IEs is needed for V2X test but it is not in 38.508-1 yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add V2X NAS IEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | V2X test is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.7.5	V2X information elements

4.7.5.1	V2X information elements of MANAGE UE POLICY COMMAND message

–	UE policy section management list

Table 4.7.5.1-1: UE policy section management list

| Derivation Path: TS 24.501 Figure D.6.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy section management list IEI |  | FFS |  |  |
| Length of UE policy section management list contents |  | Set to the actual length of 'UE policy section management list contents' in bytes |  |  |
| UE policy section management list contents |  | See Table 4.7.5.1-2 |  |  |

–	UE policy section management list contents

Table 4.7.5.1-2: UE policy section management list contents

| Derivation Path: TS 24.501 Figure D.6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy section management sublist (PLMN-1) |  | See Table 4.7.5.1-3 |  |  |
| UE policy section management sublist (PLMN-2) |  | FFS |  |  |

–	UE policy section management sublist

Table 4.7.5.1-3: UE policy section management sublist

| Derivation Path: TS 24.501 Figure D.6.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of UE policy section management sublist |  | Set to the actual length of 'UE policy section management sublist' in bytes |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |
| UE policy section management sublist contents |  | See Table 4.7.5.1-4 |  |  |

–	UE policy section management sublist contents

Table 4.7.5.1-4: UE policy section management sublist contents

| Derivation Path: TS 24.501 Figure D.6.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Instruction 1 |  | See Table 4.7.5.1-5 |  |  |
| Instruction 2 |  | FFS |  |  |

–	Instruction

Table 4.7.5.1-5: Instruction

| Derivation Path: TS 24.501 Figure D.6.2.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Instruction contents length |  | Set to the actual length of 'Instruction contents' in bytes |  |  |
| UPSC |  | FFS |  |  |
| UE policy section contents |  | See Table 4.7.5.1-6 |  |  |

–	UE policy section contents

Table 4.7.5.1-6: UE policy section contents

| Derivation Path: TS 24.501 Figure D.6.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part 1 |  | See Table 4.7.5.1-7 |  |  |
| UE policy part 2 |  | FFS |  |  |

–	UE policy part

Table 4.7.5.1-7: UE policy part

| Derivation Path: TS 24.501 Figure D.6.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| Spare |  | ‘0000’B |  |  |
| UE policy part type |  | ‘0001’B |  | URSP |
|  |  | ‘0010’B |  | ANDSP |
|  |  | ‘0011’B |  | V2XP |
| UE policy part contents |  | See Table 4.7.5.4-1 |  | V2XP |

4.7.5.2	V2X information elements of MANAGE UE POLICY COMMAND REJECT message

–	UE policy section management result

Table 4.7.5.2-1: UE policy section management result

| Derivation Path: TS 24.501 Figure D.6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy section management result IEI |  | FFS |  |  |
| Length of UE policy section management result contents |  | Set to the actual length of 'UE policy section management result contents' in bytes |  |  |
| UE policy section management result contents |  | See Table 4.7.5.2-2 |  |  |

–	UE policy section management result contents

Table 4.7.5.2-2: UE policy section management result contents

| Derivation Path: TS 24.501 Figure D.6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy section management subresult (PLMN 1) |  | See Table 4.7.5.2-3 |  |  |
| UE policy section management subresult (PLMN 2) |  | FFS |  |  |

–	UE policy section management subresult

Table 4.7.5.2-3: UE policy section management subresult

| Derivation Path: TS 24.501 Figure D.6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Number of results |  | FFS |  |  |
| PLMN ID |  | Set according to parameter given in test case |  |  |
| UE policy section management subresult contents |  | See Table 4.7.5.2-4 |  |  |

–	UE policy section management subresult contents

Table 4.7.5.2-4: UE policy section management subresult contents

| Derivation Path: TS 24.501 Figure D.6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Result 1 |  | See Table 4.7.5.2-5 |  |  |
| Result 2 |  | FFS |  |  |

–	Result

Table 4.7.5.2-5: Result

| Derivation Path: TS 24.501 Figure D.6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UPSC |  | FFS |  |  |
| Failed instruction order |  | FFS |  |  |
| Cause |  | FFS |  |  |

4.7.5.3	V2X information elements for UE STATE INDICATION message

–	UPSI list

Table 4.7.5.3-1: UPSI list

| Derivation Path: TS 24.501 Figure D.6.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UPSI list IEI |  | FFS |  |  |
| Length of UPSI list contents |  | Set to the actual length of 'UPSI list contents' in bytes |  |  |
| UPSI sublist (PLMN 1) |  | See Table 4.7.5.3-2 |  |  |
| UPSI sublist (PLMN 2) |  | FFS |  |  |

–	UPSI sublist

Table 4.7.5.3-2: UPSI sublist

| Derivation Path: TS 24.501 Figure D.6.4.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of UPSI sublist |  | Set to the actual length of 'UPSI sublist' in bytes |  |  |
| PLMN ID |  | Set according to parameter given in test case |  |  |
| UPSC 1 |  | FFS |  |  |
| UPSC 2 |  | FFS |  |  |

–	UE policy classmark

Table 4.7.5.3-3: UE policy classmark

| Derivation Path: TS 24.501 Figure D.6.5.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Policy information IEI |  | FFS |  |  |
| Length of Policy information contents |  | Set to the actual length of 'Policy information contents' in bytes |  |  |
| Spare |  | ‘0000 000’B |  |  |
| SupportANDSP |  | ‘0’B |  |  |

–	UE OS Id

Table 4.7.5.3-4: UE OS Id

| Derivation Path: TS 24.501 Figure D.6.6.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| OS Id IEI |  | FFS |  |  |
| Length of OS Id information contents |  | Set to the actual length of 'OS Id information contents' in bytes |  |  |
| OS Id_1 |  | FFS |  |  |
| OS Id_2 |  | FFS |  |  |

4.7.5.4	V2X information elements for UE policy part

–	UE policy part when UE policy part type = {V2XP}

Table 4.7.5.4-1: UE policy part when UE policy part type = {V2XP}

| Derivation Path: TS 24.588 Figure 5.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| Spare |  | ‘0000’B |  |  |
| UE policy part type |  | ‘0001’B | UE policy part type={ V2XP } |  |
| UE policy part contents={V2XP contents} |  | See Table 4.7.5.4-2 |  |  |

–	V2XP contents

Table 4.7.5.4-2: V2XP contents

| Derivation Path: TS 24.588 Figure 5.2.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| V2XP info #1 |  | See Table 4.7.5.4-3 |  |  |
| V2XP info #2 |  | FFS |  |  |

–	V2XP info

Table 4.7.5.4-3: V2XP info

| Derivation Path: TS 24.588 Figure 5.3.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Spare |  | ‘0000’B |  |  |
| V2XP info type |  | ‘0001’B | UE policies for V2X communication over PC5 |  |
| Length of V2XP info contents |  | Set to the actual length of 'V2XP info contents' in bytes |  |  |
| V2XP info contents |  | See Table 4.7.5.5-1 |  |  |

4.7.5.5	V2X information elements of UE policies for V2X communication over PC5

–	V2XP info = {UE policies for V2X communication over PC5}

Table 4.7.5.5-1: V2XP info = {UE policies for V2X communication over PC5}

| Derivation Path: TS 24.588 Figure 5.3.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Spare |  | ‘0000’B |  |  |
| V2XP info type |  | ‘0001’B | UE policies for V2X communication over PC5 |  |
| Length of V2XP info contents |  | Set to the actual length of 'V2XP info contents' in bytes |  |  |
| Validity timer |  | FFS |  |  |
| VSITPMRI |  | ‘1’B | ‘V2X service identifier to PC5 RAT and Tx profiles mapping rules’ is present |  |
| Spare |  | ‘000 0000’B |  |  |
| Served by E-UTRA or served by NR |  | See Table 4.7.5.5-2 |  |  |
| Not served by E-UTRA and not served by NR |  | See Table 4.7.5.5-6 |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rules |  | See Table 4.7.5.5-12 |  |  |
| Privacy config |  | See Table 4.7.5.5-15 |  |  |
| V2X communication over PC5 in E-UTRA-PC5 |  | See Table 4.7.5.5-19 |  |  |
| V2X communication over PC5 in NR-PC5 |  | See Table 4.7.5.5-31 |  |  |

–	Served by E-UTRA or served by NR

Table 4.7.5.5-2: Served by E-UTRA or served by NR

| Derivation Path: TS 24.588 Figure 5.3.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of served by E-UTRA or served by NR contents |  | Set to the actual length of 'served by E-UTRA or served by NR contents' in bytes |  |  |
| Authorized PLMN and RATs combinations |  | See Table 4.7.5.5-3 |  |  |

–	Authorized PLMN and RATs combinations

Table 4.7.5.5-3: Authorized PLMN and RATs combinations

| Derivation Path: TS 24.588 Figure 5.3.1.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of authorized PLMN and RATs combinations contents |  | Set to the actual length of 'authorized PLMN and RATs combinations contents' in bytes |  |  |
| Authorized PLMN and RATs combination 1 |  | See Table 4.7.5.5-4 |  |  |
| Authorized PLMN and RATs combination 2 |  | FFS |  |  |

–	Authorized PLMN and RATs combination

Table 4.7.5.5-4: Authorized PLMN and RATs combination

| Derivation Path: TS 24.588 Figure 5.3.1.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PLMN ID |  | Set according to parameter given in test case |  |  |
| EPIEN |  | FFS |  |  |
| NPIEN |  | FFS |  |  |
| Spare |  | ’00 0000’B |  |  |

–	PLMN ID

Table 4.7.5.5-5: PLMN ID

| Derivation Path: TS 24.588 Figure 5.3.1.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MCC digit 2 |  | Set according to parameter given in test case |  |  |
| MCC digit 1 |  | Set according to parameter given in test case |  |  |
| MNC digit 3 |  | Set according to parameter given in test case |  |  |
| MCC digit 3 |  | Set according to parameter given in test case |  |  |
| MNC digit 2 |  | Set according to parameter given in test case |  |  |
| MNC digit 1 |  | Set according to parameter given in test case |  |  |

–	Not served by E-UTRA and not served by NR

Table 4.7.5.5-6: Not served by E-UTRA and not served by NR

| Derivation Path: TS 24.588 Figure 5.3.1.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of not served by E-UTRA and not served by NR contents |  | Set to the actual length of 'not served by E-UTRA and not served by NR contents' in bytes |  |  |
| EPIEN |  | FFS |  |  |
| NPIEN |  | FFS |  |  |
| Spare |  | ’00 000’B |  |  |
| VPNENNI |  | FFS |  |  |
| E-UTRA radio parameters per geographical area list |  | See Table 4.7.5.5-7 |  |  |
| NR radio parameters per geographical area list |  | See Table 4.7.5.5-7 |  |  |

–	Radio parameters per geographical area list

Table 4.7.5.5-7: Radio parameters per geographical area list

| Derivation Path: TS 24.588 Figure 5.3.1.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area list contents |  | Set to the actual length of 'radio parameters per geographical area list contents' in bytes |  |  |
| Radio parameters per geographical area info 1 |  | See Table 4.7.5.5-8 |  |  |
| Radio parameters per geographical area info 2 |  | FFS |  |  |

–	Radio parameters per geographical area info

Table 4.7.5.5-8: Radio parameters per geographical area info

| Derivation TS 24.588 Figure 5.3.1.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area contents |  | Set to the actual length of 'radio parameters per geographical area contents' in bytes |  |  |
| Geographical area |  | See Table 4.7.5.5-9 |  |  |
| Radio parameters |  | See Table 4.7.5.5-11 |  |  |
| MI |  | FFS |  |  |
| Spare |  | ‘000 0000’B |  |  |

–	Geographical area

Table 4.7.5.5-9: Geographical area

| Derivation Path: TS 24.588 Figure 5.3.1.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of geographical area contents |  | Set to the actual length of 'geographical area contents' in bytes |  |  |
| Coordinate 1 |  | See Table 4.7.5.5-10 |  |  |
| Coordinate 2 |  | FFS |  |  |

–	Coordinate area

Table 4.7.5.5-10: Coordinate area

| Derivation Path: TS 24.588 Figure 5.3.1.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Latitude |  | FFS |  |  |
| Longitude |  | FFS |  |  |

–	Radio parameters

Table 4.7.5.5-11: Radio parameters

| Derivation Path: TS 24.588 Figure 5.3.1.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters contents |  | Set to the actual length of 'radio parameters contents' in bytes |  |  |
| Radio parameters contents |  | See Table 4.10.1.1-1: SL-V2X-Preconfiguration in TS 36.508 [2] |  | E-UTRAN V2X |
|  |  | See Table 4.10.1-1: SL-PreconfigurationNR |  | NR V2X |

–	V2X service identifier to PC5 RAT and Tx profiles mapping rules

Table 4.7.5.5-12: V2X service identifier to PC5 RAT and Tx profiles mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 RAT and Tx profiles mapping rules contents |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rule 1 |  | See Table 4.7.5.5-13 |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to PC5 RAT and Tx profiles mapping rule

Table 4.7.5.5-13: V2X service identifier to PC5 RAT and Tx profiles mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 RAT and Tx profiles mapping rule contents |  | Set to the actual length of 'V2X service identifier to PC5 RAT and Tx profiles mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| Spare |  | ‘0000 00’B |  |  |
| PC5 RAT |  | ‘00’B | E-UTRA-PC5 |  |
|  |  | ‘01’B | NR-PC5 |  |
| Length of E-UTRA-PC5 Tx profiles |  | Set to the actual length of 'E-UTRA-PC5 Tx profiles' in bytes |  |  |
| E-UTRA-PC5 Tx profiles |  | FFS | Refer to IE v2x-TxProfileList in TS 36.331 |  |
| Length of NR-PC5 Tx profiles |  | Set to the actual length of 'NR-PC5 Tx profiles' in bytes |  |  |
| NR-PC5 Tx profiles |  | FFS |  |  |

–	V2X service identifiers

Table 4.7.5.5-14: V2X service identifiers

| Derivation Path: TS 24.588 Figure 5.3.1.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifiers contents |  | Set to the actual length of 'V2X service identifiers contents' in bytes |  |  |
| V2X service identifier 1 |  | FFS |  |  |
| V2X service identifier 2 |  | FFS |  |  |

–	Privacy config

Table 4.7.5.5-15: Privacy config

| Derivation Path: TS 24.588 Figure 5.3.1.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of privacy config contents |  | Set to the actual length of 'privacy config contents' in bytes |  |  |
| V2X services requiring privacy |  | See Table 4.7.5.5-16 |  |  |
| Privacy timer |  | FFS |  |  |

–	V2X services requiring privacy

Table 4.7.5.5-16: V2X services requiring privacy

| Derivation Path: TS 24.588 Figure 5.3.1.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X services requiring privacy contents |  | Set to the actual length of 'V2X services requiring privacy contents' in bytes |  |  |
| V2X service requiring privacy 1 |  | See Table 4.7.5.5-17 |  |  |
| V2X service requiring privacy 2 |  | FFS |  |  |

–	V2X service requiring privacy

Table 4.7.5.5-17: V2X service requiring privacy

| Derivation Path: TS 24.588 Figure 5.3.1.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service requiring privacy contents |  | Set to the actual length of 'V2X service requiring privacy contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Geographical areas |  | See Table 4.7.5.5-18 |  |  |

–	Geographical areas

Table 4.7.5.5-18: Geographical areas

| Derivation Path: TS 24.588 Figure 5.3.1.18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of geographical areas contents |  | Set to the actual length of 'geographical areas contents' in bytes |  |  |
| Geographical area 1 |  | FFS |  |  |
| Geographical area 2 |  | FFS |  |  |

–	V2X communication over PC5 in E-UTRA-PC5

Table 4.7.5.5-19: V2X communication over PC5 in E-UTRA-PC5

| Derivation Path: TS 24.588 Figure 5.3.1.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X communication over PC5 in E-UTRA-PC5 contents |  | Set to the actual length of 'V2X communication over PC5 in E-UTRA-PC5 contents' in bytes |  |  |
| DDL2II |  | FFS |  |  |
| VSIEFMRI |  | FFS |  |  |
| VSAPI |  | FFS |  |  |
| PPMR |  | FFS |  |  |
| Spare |  | ‘0000’B |  |  |
| V2X service identifier to destination layer-2 ID mapping rules |  | See Table 4.7.5.5-20 |  |  |
| PPPP to PDB mapping rules |  | See Table 4.7.5.5-22 |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rules |  | See Table 4.7.5.5-24 |  |  |
| V2X services authorized for PPPR |  | See Table 4.7.5.5-29 |  |  |
| Default destination layer-2 ID |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID mapping rules

Table 4.7.5.5-20: V2X service identifier to destination layer-2 ID mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID mapping rule 1 |  | See Table 4.7.5.5-21 |  |  |
| V2X service identifier to destination layer-2 ID mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID mapping rule

Table 4.7.5.5-21: V2X service identifier to destination layer-2 ID mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Destination layer-2 ID |  | FFS |  |  |

–	PPPP to PDB mapping rules

Table 4.7.5.5-22: PPPP to PDB mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of PPPP to PDB mapping rules contents |  | Set to the actual length of 'PPPP to PDB mapping rules contents' in bytes |  |  |
| PPPP to PDB mapping rule 1 |  | See Table 4.7.5.5-23 |  |  |
| PPPP to PDB mapping rule 2 |  | FFS |  |  |

–	PPPP to PDB mapping rule

Table 4.7.5.5-23: PPPP to PDB mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Spare |  | ‘0000 0’B |  |  |
| PPPP |  | FFS |  |  |
| PDB |  | FFS |  |  |

–	V2X service identifier to V2X E-UTRA frequency mapping rules

Table 4.7.5.5-24: V2X service identifier to V2X E-UTRA frequency mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.24 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rules contents |  | Set to the actual length of 'V2X service identifier to V2X E-UTRA frequency mapping rules contents' in bytes |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 1 |  | See Table 4.7.5.5-25 |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to V2X E-UTRA frequency mapping rule

Table 4.7.5.5-25: V2X service identifier to V2X E-UTRA frequency mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rule contents |  | Set to the actual length of 'V2X service identifier to V2X E-UTRA frequency mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| V2X E-UTRA frequencies with geographical areas list |  | See Table 4.7.5.5-26 |  |  |

–	V2X E-UTRA frequencies with geographical areas list

Table 4.7.5.5-26: V2X E-UTRA frequencies with geographical areas list

| Derivation Path: TS 24.588 Figure 5.3.1.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies with geographical areas list contents |  | Set to the actual length of 'V2X E-UTRA frequencies with geographical areas list contents' in bytes |  |  |
| V2X E-UTRA frequencies with geographical areas info 1 |  | See Table 4.7.5.5-27 |  |  |
| V2X E-UTRA frequencies with geographical areas info 2 |  | FFS |  |  |

–	V2X E-UTRA frequencies with geographical areas info

Table 4.7.5.5-27: V2X E-UTRA frequencies with geographical areas info

| Derivation Path: TS 24.588 Figure 5.3.1.27 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies with geographical areas info contents |  | Set to the actual length of 'V2X E-UTRA frequencies with geographical areas info contents' in bytes |  |  |
| V2X E-UTRA frequencies |  | See Table 4.7.5.5-28 |  |  |
| Geographical areas |  | FFS |  |  |

–	V2X E-UTRA frequencies

Table 4.7.5.5-28: V2X E-UTRA frequencies

| Derivation Path: TS 24.588 Figure 5.3.1.28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies contents |  | Set to the actual length of 'V2X E-UTRA frequencies contents' in bytes |  |  |
| V2X E-UTRA frequency 1 |  | FFS |  |  |
| V2X E-UTRA frequency 2 |  | FFS |  |  |

–	V2X services authorized for PPPR

Table 4.7.5.5-29: V2X services authorized for PPPR

| Derivation Path: TS 24.588 Figure 5.3.1.29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X services authorized for PPPR contents |  | Set to the actual length of 'V2X services authorized for PPPR contents' in bytes |  |  |
| V2X service authorized for PPPR 1 |  | See Table 4.7.5.5-30 |  |  |
| V2X service authorized for PPPR 2 |  | FFS |  |  |

–	V2X service authorized for PPPR

Table 4.7.5.5-30: V2X service authorized for PPPR

| Derivation Path: TS 24.588 Figure 5.3.1.30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service authorized for PPPR contents |  | Set to the actual length of 'V2X service authorized for PPPR contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Spare |  | ‘0000 0’B |  |  |
| PPPR |  | FFS |  |  |

–	V2X communication over PC5 in NR-PC5

Table 4.7.5.5-31: V2X communication over PC5 in NR-PC5

| Derivation Path: TS 24.588 Figure 5.3.1.31 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X communication over PC5 in NR-PC5 contents |  | Set to the actual length of 'V2X communication over PC5 in NR-PC5 contents' in bytes |  |  |
| DDL2IBI |  | FFS |  |  |
| VSINFMRI |  | FFS |  |  |
| Spare |  | ’00 0000’B |  |  |
| V2X service identifier to V2X NR frequency mapping rules |  | See Table 4.7.5.5-32 |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rules |  | See Table 4.7.5.5-37 |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rules |  | See Table 4.7.5.5-39 |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules |  | See Table 4.7.5.5-41 |  |  |
| V2X service identifier to PC5 QoS parameters mapping rules |  | See Table 4.7.5.5-43 |  |  |
| AS configuration |  | See Table 4.7.5.5-45 |  |  |
| Default destination layer-2 ID for broadcast |  | FFS |  |  |
| NR-PC5 unicast security policies |  | See Table 4.7.5.5-49 |  |  |
| V2X service identifier to default mode of communication mapping rules |  | See Table 4.7.5.5-52 |  |  |

–	V2X service identifier to V2X NR frequency mapping rules

Table 4.7.5.5-32: V2X service identifier to V2X NR frequency mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.32 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X NR frequency mapping rules contents |  | Set to the actual length of 'V2X service identifier to V2X NR frequency mapping rules contents' in bytes |  |  |
| V2X service identifier to V2X NR frequency mapping rule 1 |  | See Table 4.7.5.5-33 |  |  |
| V2X service identifier to V2X NR frequency mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to V2X NR frequency mapping rule

Table 4.7.5.5-33: V2X service identifier to V2X NR frequency mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X NR frequency mapping rule contents |  | Set to the actual length of 'V2X service identifier to V2X NR frequency mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| V2X NR frequencies with geographical areas list |  | See Table 4.7.5.5-34 |  |  |

–	V2X NR frequencies with geographical areas list

Table 4.7.5.5-34: V2X NR frequencies with geographical areas list

| Derivation Path: TS 24.588 Figure 5.3.1.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies with geographical areas list contents |  | Set to the actual length of 'V2X NR frequencies with geographical areas list contents' in bytes |  |  |
| V2X NR frequencies with geographical areas info 1 |  | See Table 4.7.5.5-35 |  |  |
| V2X NR frequencies with geographical areas info 2 |  | FFS |  |  |

–	V2X NR frequencies with geographical areas info

Table 4.7.5.5-35: V2X NR frequencies with geographical areas info

| Derivation Path: TS 24.588 Figure 5.3.1.35 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies with geographical areas info contents |  | Set to the actual length of 'V2X NR frequencies with geographical areas info contents' in bytes |  |  |
| V2X NR frequencies |  | See Table 4.7.5.5-36 |  |  |
| Geographical areas |  | FFS |  |  |

–	V2X NR frequencies

Table 4.7.5.5-36: V2X NR frequencies

| Derivation Path: TS 24.588 Figure 5.3.1.36 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies contents |  | Set to the actual length of 'V2X NR frequencies contents' in bytes |  |  |
| V2X NR frequency 1 |  | FFS |  |  |
| V2X NR frequency 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for broadcast mapping rules

Table 4.7.5.5-37: V2X service identifier to destination layer-2 ID for broadcast mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.37 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for broadcast mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 1 |  | See Table 4.7.5.5-38 |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for broadcast mapping rule

Table 4.7.5.5-38: V2X service identifier to destination layer-2 ID for broadcast mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.38 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for broadcast mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Destination layer-2 ID for broadcast |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for groupcast mapping rules

Table 4.7.5.5-39: V2X service identifier to destination layer-2 ID for groupcast mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.39 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for groupcast mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 1 |  | See Table 4.7.5.5-40 |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for groupcast mapping rule

Table 4.7.5.5-40: V2X service identifier to destination layer-2 ID for groupcast mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.40 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for groupcast mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Destination layer-2 ID for groupcast |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

Table 4.7.5.5-41: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.41 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 1 |  | See Table 4.7.5.5-42 |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

Table 4.7.5.5-42: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.42 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Destination layer-2 ID for unicast initial signalling |  | FFS |  |  |

–	V2X service identifier to PC5 QoS parameters mapping rules

Table 4.7.5.5-43: V2X service identifier to PC5 QoS parameters mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.43 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 QoS parameters mapping rules contents |  | Set to the actual length of 'V2X service identifier to PC5 QoS parameters mapping rules contents' in bytes |  |  |
| V2X service identifier to PC5 QoS parameters mapping rule 1 |  | See Table 4.7.5.5-44 |  |  |
| V2X service identifier to PC5 QoS parameters mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to PC5 QoS parameters mapping rule

Table 4.7.5.5-44: V2X service identifier to PC5 QoS parameters mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.46 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 QoS parameters mapping rule contents |  | Set to the actual length of 'V2X service identifier to PC5 QoS parameters mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| GFBRI |  | FFS |  |  |
| MFBRI |  | FFS |  |  |
| PLAMBRI |  | FFS |  |  |
| RI |  | FFS |  |  |
| Spare |  | ‘0000’B |  |  |
| PQI |  | FFS |  |  |
| Guaranteed flow bit rate |  | FFS |  |  |
| Maximum flow bit rate |  | FFS |  |  |
| Per-link aggregate maximum bit rate |  | FFS |  |  |
| Range |  | FFS |  |  |

–	AS configuration

Table 4.7.5.5-45: AS configuration

| Derivation Path: TS 24.588 Figure 5.3.1.46a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of AS configuration contents |  | Set to the actual length of 'AS configuration contents' in bytes |  |  |
| SLRB mapping rules |  | See Table 4.7.5.5-46 |  |  |

–	SLRB mapping rules

Table 4.7.5.5-46: SLRB mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.47 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of SLRB mapping rules contents |  | Set to the actual length of 'SLRB mapping rules contents' in bytes |  |  |
| SLRB mapping rule 1 |  | See Table 4.7.5.5-47 |  |  |
| SLRB mapping rule 2 |  | FFS |  |  |

–	SLRB mapping rule

Table 4.7.5.5-47: SLRB mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.48 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of SLRB mapping rule contents |  | Set to the actual length of 'SLRB mapping rule contents' in bytes |  |  |
| PC5 QoS profile |  | See Table 4.7.5.5-48 |  |  |
| Length of SLRB |  | Set to the actual length of 'SLRB' in bytes |  |  |
| SLRB |  | FFS |  |  |

–	PC5 QoS profile

Table 4.7.5.5-48: PC5 QoS profile

| Derivation Path: TS 24.588 Figure 5.3.1.49 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of PC5 QoS profile contents |  | Set to the actual length of 'PC5 QoS profile contents' in bytes |  |  |
| GFBRI |  | FFS |  |  |
| MFBRI |  | FFS |  |  |
| PLAMBRI |  | FFS |  |  |
| RI |  | FFS |  |  |
| PLOI |  | FFS |  |  |
| AWI |  | FFS |  |  |
| MDBVI |  | FFS |  |  |
| Spare |  | ‘0’B |  |  |
| PQI |  | FFS |  |  |
| Guaranteed flow bit rate |  | FFS |  |  |
| Maximum flow bit rate |  | FFS |  |  |
| Per-link aggregate maximum bit rate |  | FFS |  |  |
| Range |  | FFS |  |  |
| Spare |  | ‘0000 0’B |  |  |
| Priority level |  | FFS |  |  |
| Averaging window |  | FFS |  |  |
| Maximum data burst volume |  | FFS |  |  |

–	NR-PC5 unicast security policies

Table 4.7.5.5-49: NR-PC5 unicast security policies

| Derivation Path: TS 24.588 Figure 5.3.1.50 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policies contents |  | Set to the actual length of 'NR-PC5 unicast security policies contents' in bytes |  |  |
| NR-PC5 unicast security policy 1 |  | See Table 4.7.5.5-50 |  |  |
| NR-PC5 unicast security policy 2 |  | FFS |  |  |

–	NR-PC5 unicast security policy

Table 4.7.5.5-50: NR-PC5 unicast security policy

| Derivation Path: TS 24.588 Figure 5.3.1.51 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policy contents |  | Set to the actual length of 'NR-PC5 unicast security policy contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Security policy |  | See Table 4.7.5.5-51 |  |  |
| Geographical areas |  | FFS |  |  |

–	Security policy

Table 4.7.5.5-51: Security policy

| Derivation Path: TS 24.588 Figure 5.3.1.52 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Spare |  | ‘0’B |  |  |
| Signalling ciphering policy |  | FFS |  |  |
| Spare |  | ‘0’B |  |  |
| Signalling integrity protection policy |  | FFS |  |  |
| Spare |  | ‘0’B |  |  |
| User plane ciphering policy |  | FFS |  |  |
| Spare |  | ‘0’B |  |  |
| User plane integrity protection policy |  | FFS |  |  |

–	V2X service identifier to default mode of communication mapping rules

Table 4.7.5.5-52: V2X service identifier to default mode of communication mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.53 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to default mode of communication mapping rules contents |  | ‘Set to the actual length of 'V2X service identifier to default mode of communication mapping rules contents' in bytes |  |  |
| V2X service identifier to default mode of communication mapping rule 1 |  | See Table 4.7.5.5-53 |  |  |
| V2X service identifier to default mode of communication mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to default mode of communication mapping rule

Table 4.7.5.5-53: V2X service identifier to default mode of communication mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.54 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to default mode of communication mapping rule contents |  | ‘Set to the actual length of 'V2X service identifier to default mode of communication mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | FFS |  |  |
| Spare |  | ‘0000 00’B |  |  |
| DMC |  | FFS |  |  |

<End of modified section 1>