3GPP TSG-RAN5 Meeting #90-e	R5-210757

# Electronic, 22nd February - 5th March, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0318 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Annex C for TDD NB-IOT RSTD measurement test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The TDD NB-IOT RSTD measurement test cases 9.7.1, 9.7.2, 9.7.3, 9.8.1, 9.8.2 and 9.8.3 are introduced. MU and TT need also be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Annex C for 9.7.1, 9.7.2, 9.7.3, 9.8.1, 9.8.2 and 9.8.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case can’t be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.1.3, C.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

C.1.3	ECID and OTDOA Measurement requirements

Table C.1.3-1: Maximum Test System Uncertainty for ECID and OTDOA Measurement Requirements

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.1.1 E-UTRAN FDD UE Rx – Tx time difference case (Rel-9 to Rel-11) | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | Note: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [26] |
| 8.1.1A E-UTRAN FDD UE Rx – Tx time difference case (Rel-12 onwards) | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.1B E-UTRAN FDD UE Rx – Tx time difference case for UE Category 1bis | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.2 E-UTRAN TDD UE Rx – Tx time difference case (Rel-9 to Rel-11) | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.2A E-UTRAN TDD UE Rx – Tx time difference case (Rel-12 onwards) | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.2B E-UTRAN TDD UE Rx – Tx time difference case for UE Category 1bis | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.3 E-UTRAN FDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB dB averaged over BWConfig ±3TS Uplink signal transmit timing relative to downlink | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [26] |
| 8.1.4 E-UTRAN TDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Same as 8.1.3 | Same as 8.1.3 |
| 8.1.5 E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB dB averaged over BWConfig Ês3 / Noc ±0.3 dB dB averaged over BWConfig ±3Ts Uplink signal transmit timing relative to downlink | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Ês3 / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [26] |
| 8.1.6 E-UTRAN TDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 8.1.5 | Same as 8.1.5 |
| 8.1.7 E-UTRAN FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.8 E-UTRAN HD-FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.9 E-UTRAN TDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.1 | Same as 8.1.1 |
| 9.1.1 FDD RSTD Measurement Reporting Delay | Noc ±1.0 dB averaged over BWConfig PRS Ês1 / Noc ±0.6 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig PRS Ês2 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig PRS Ês3 / Noc ±0.6 dB averaged over BWConfig Ês3 / Noc ±0.6 dB averaged over BWConfig Response Time = ± 300 ms | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN PRS Ês / Noc and Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: PRS Ês / Noc and Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 9.1.1A FDD RSTD Measurement Reporting Delay for UE Category 1bis | Same as 9.1.1 |  |
| 9.1.2 TDD RSTD Measurement Reporting Delay | Same as 9.1.1 |  |
| 9.1.2A TDD RSTD Measurement Reporting Delay for UE Category 1bis | Same as 9.1.1 |  |
| 9.1.3 FDD RSTD Measurement Accuracy | Noc ±1.0 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 1 Ts | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |
| 9.1.3A FDD RSTD Measurement Accuracy for UE Category 1bis | Same as 9.1.3 |  |
| 9.1.4 TDD RSTD Measurement Accuracy | Same as 9.1.3 |  |
| 9.1.4A TDD RSTD Measurement Accuracy for UE Category 1bis | Same as 9.1.3 |  |
| 9.2.1 FDD-FDD inter-frequency RSTD measurement reporting delay | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig PRS Ês1 / Noc1 ±0.6 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.6 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig PRS Ês3 / Noc2 ±0.6 dB averaged over BWConfig Ês3 / Noc2 ±0.6 dB averaged over BWConfig Response Time = ± 300 ms | Note: PRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of cell 1 signal / AWGN for frequency 1 PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN for frequency 2 PRS Ês3 / Noc2 and Ês3 / Noc2 are the ratios of cell 3 signal / AWGN for frequency 2 PRS Ês / Noc and Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: PRS Ês / Noc and Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 9.2.1A FDD-FDD inter-frequency RSTD measurement reporting delay for UE Category 1bis | Same as 9.2.1 |  |
| 9.2.2 TDD-TDD inter-frequency RSTD measurement reporting delay | Same as 9.2.1 |  |
| 9.2.2A TDD-TDD inter-frequency RSTD measurement reporting delay for UE Category 1bis | Same as 9.2.1 |  |
| 9.2.4 FDD-FDD inter frequency RSTD Accuracy | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig PRS Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 2 Ts | Note: PRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of cell 1 signal / AWGN for frequency 1 PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN for frequency 2 |
| 9.2.4A FDD-FDD inter frequency RSTD Accuracy for UE Category 1bis | Same as 9.2.4 |  |
| 9.2.5 TDD-TDD inter frequency RSTD Accuracy | Same as 9.2.4 |  |
| 9.2.5A TDD-TDD inter frequency RSTD Accuracy for UE Category 1bis | Same as 9.2.4 |  |
| 9.3.1.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.1.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.2.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.2.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.3.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.3.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.4.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.4.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.5.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.5.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.6.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.6.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.7.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.7.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.8.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.8.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.9.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.9.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.10.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.10.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.11.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.11.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.12.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.12.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.4.1.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.1.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.2.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.2.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.3.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.3.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.4.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.4.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.5.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.5.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.6.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.6.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.7.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.7.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.8.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.8.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.9.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.9.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.10.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.10.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.11.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.11.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.12.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.12.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.5.1 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | Noc ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 1 Ts | Note: NPRS Ês1 / Noc and Ês1 / Noc are the ratios of nCell 1 signal / AWGN NPRS Ês2 / Noc and Ês2 / Noc are the ratios of nCell 2 signal / AWGN |
| 9.5.2 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.5.1 | Same as 9.5.1 |
| 9.5.3 HD-FDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Noc ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig NPRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms | Note: NPRS Ês1 / Noc and Ês1 / Noc are the ratios of nCell 1 signal / AWGN NPRS Ês2 / Noc and Ês2 / Noc are the ratios of nCell 2 signal / AWGN |
| 9.6.1 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 2 Ts | Note: NPRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of nCell 1 signal / AWGN for frequency 1 NPRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of nCell 2 signal / AWGN for frequency 2 |
| 9.6.2 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.6.1 | Same as 9.6.1 |
| 9.6.3 HD-FDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Same as 9.5.3 | Same as 9.5.3 |
| 9.7.1 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | Noc ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 1 Ts | Note: NPRS Ês1 / Noc and Ês1 / Noc are the ratios of nCell 1 signal / AWGN NPRS Ês2 / Noc and Ês2 / Noc are the ratios of nCell 2 signal / AWGN |
| 9.7.2 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.7.1 | Same as 9.7.1 |
| 9.7.3 TDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Noc ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig NPRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms | Note: NPRS Ês1 / Noc and Ês1 / Noc are the ratios of nCell 1 signal / AWGN NPRS Ês2 / Noc and Ês2 / Noc are the ratios of nCell 2 signal / AWGN |
| 9.8.1 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig NPRS Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig NPRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Cell Timing Difference = ± 2 Ts | Note: NPRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of nCell 1 signal / AWGN for frequency 1 NPRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of nCell 2 signal / AWGN for frequency 2 |
| 9.8.2 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.8.1 | Same as 9.8.1 |
| 9.8.3 TDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Same as 9.7.3 | Same as 9.7.3 |
| 10.1 FDD RSTD Measurement Reporting Delay for Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig PRS Ês1 / Noc1 ±0.6 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.6 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig PRS Ês3 / Noc2 ±0.6 dB averaged over BWConfig Ês3 / Noc2 ±0.6 dB averaged over BWConfig Response Time = ± 300 ms | Note: PRS Ês1 / Noc1 and Ês1 / Noc1 are the ratios of cell 1 signal / AWGN for frequency 1 PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN for frequency 2 PRS Ês3 / Noc2 and Ês3 / Noc2 are the ratios of cell 3 signal / AWGN for frequency 2 PRS Ês / Noc and Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: PRS Ês / Noc and Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 10.1A FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz | Same as 10.1 | Same as 10.1 |
| 10.1B FDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth | Same as 10.1 | Same as 10.1 |
| 10.1C FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Same as 10.1 | Same as 10.1 |
| 10.2 TDD RSTD Measurement Reporting Delay for Carrier Aggregation | Same as 10.1 | Same as 10.1 |
| 10.2A TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz | Same as 10.1 | Same as 10.1 |
| 10.2B TDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth | Same as 10.1 | Same as 10.1 |

<Unchanged Text Skipped>

C.2.2	ECID and OTDOA Measurement requirements

Table C.2.2: Test Parameter Relaxations for ECID and OTDOA Measurement requirements

| Clause | Test Parameter Relaxation |  |
| --- | --- | --- |
| 8.1.1 E-UTRAN FDD UE Rx – Tx time difference case (Rel-9 to Rel-11) |  |  |
| 8.1.1A E-UTRAN FDD UE Rx – Tx time difference case (Rel-12 onwards) |  |  |
| 8.1.1B E-UTRAN FDD UE Rx – Tx time difference case for UE Category 1bis |  |  |
| 8.1.2 E-UTRAN TDD UE Rx – Tx time difference case (Rel-9 to Rel-11) |  |  |
| 8.1.2A E-UTRAN TDD UE Rx – Tx time difference case (Rel-12 onwards) |  |  |
| 8.1.2B E-UTRAN TDD UE Rx – Tx time difference case for UE Category 1bis |  |  |
| 8.1.3 E-UTRAN FDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Parameters Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +1.00dB | Test Tolerance 0dB +0.3dB 0dB |
| 8.1.4 E-UTRAN TDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Same as 8.1.3 | Same as 8.1.3 |
| 8.1.5 E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Parameters Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB | Test Tolerance 0dB +0.4dB 0dB 0dB |
| 8.1.6 E-UTRAN TDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 8.1.5 | Same as 8.1.5 |
| 8.1.7 E-UTRAN FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.8 E-UTRAN HD-FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.9 E-UTRAN TDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.1 | Same as 8.1.1 |
| 9.1.1 FDD RSTD Measurement Reporting Delay | Response time | 300 ms |
| 9.1.1A FDD RSTD Measurement Reporting Delay for UE Category 1bis | Response time | 300 ms |
| 9.1.2 TDD RSTD Measurement Reporting Delay | Response time | 300 ms |
| 9.1.2A TDD RSTD Measurement Reporting Delay for UE Category 1bis | Response time | 300 ms |
| 9.1.3 FDD RSTD Measurement Accuracy | For Test 2 and Test 4: PRS Ês1 / Noc averaged over BWConfig PRS Ês2 / Noc averaged over BWConfig For all tests: Cell Timing Difference | +0.3 dB +0.3 dB ± 1 Ts |
| 9.1.3A FDD RSTD Measurement Accuracy for UE Category 1bis | Same as 9.1.3 | Same as 9.1.3 |
| 9.1.4 TDD RSTD Measurement Accuracy | Same as 9.1.3 | Same as 9.1.3 |
| 9.1.4A TDD RSTD Measurement Accuracy for UE Category 1bis | Same as 9.1.3 | Same as 9.1.3 |
| 9.2.1 FDD-FDD inter-frequency RSTD measurement reporting delay | Response time | 300 ms |
| 9.2.1A FDD-FDD inter-frequency RSTD measurement reporting delay for UE Category 1bis | Response time | 300 ms |
| 9.2.2 TDD-TDD inter-frequency RSTD measurement reporting delay | Response time | 300 ms |
| 9.2.2A TDD-TDD inter-frequency RSTD measurement reporting delay for UE Category 1bis | Response time | 300 ms |
| 9.2.4 FDD-FDD inter frequency RSTD Accuracy | PRS Ês1 / Noc1 averaged over BWConfig PRS Ês2 / Noc2 averaged over BWConfig Cell Timing Difference | +0.3 dB +0.3 dB ± 2 Ts |
| 9.2.4A FDD-FDD inter frequency RSTD Accuracy for UE Category 1bis | Same as 9.2.4 | Same as 9.2.4 |
| 9.2.5 TDD-TDD inter frequency RSTD Accuracy | Same as 9.2.4 | Same as 9.2.4 |
| 9.2.5A TDD-TDD inter frequency RSTD Accuracy for UE Category 1bis | Same as 9.2.4 | Same as 9.2.4 |
| 9.3.1.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.1.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.2.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.2.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.3.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.3.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.4.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.4.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.5.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.5.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.6.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.6.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.1.1 | Same as 9.1.1 |
| 9.3.7.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.7.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.8.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.8.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.9.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.9.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.10.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.10.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.11.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.11.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.12.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.12.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.1.3 | Same as 9.1.3 |
| 9.4.1.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.1.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.2.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.2.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.3.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.3.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.4.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.4.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.5.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.5.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.6.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.6.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Same as 9.2.1 | Same as 9.2.1 |
| 9.4.7.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.7.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.8.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.8.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.9.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.9.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.10.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.10.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.11.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.11.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.12.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.2.4 | Same as 9.2.4 |
| 9.4.12.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.2.4 | Same as 9.2.4 |
| 9.5.1 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | For Test 2 and Test 4: PRS Ês1 / Noc averaged over BWConfig PRS Ês2 / Noc averaged over BWConfig For all tests: Cell Timing Difference | +0.3 dB +0.3 dB ± 1 Ts |
| 9.5.2 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.5.1 | Same as 9.5.1 |
| 9.5.3 HD-FDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Response time | 300 ms |
| 9.6.1 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | PRS Ês1 / Noc1 averaged over BWConfig PRS Ês2 / Noc2 averaged over BWConfig Cell Timing Difference | +0.3 dB +0.3 dB ± 2 Ts |
| 9.6.2 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.6.1 | Same as 9.6.1 |
| 9.6.3 HD-FDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Response time | 300 ms |
| 9.7.1 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | PRS Ês1 / Noc averaged over BWConfig PRS Ês2 / Noc averaged over BWConfig For all tests: Cell Timing Difference | +0.3 dB +0.3 dB ± 1 Ts |
| 9.7.2 TDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.7.1 | Same as 9.7.1 |
| 9.7.3 TDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Response time | 300 ms |
| 9.8.1 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | PRS Ês1 / Noc1 averaged over BWConfig PRS Ês2 / Noc2 averaged over BWConfig Cell Timing Difference | +0.3 dB +0.3 dB ± 2 Ts |
| 9.8.2 TDD Inter-frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | Same as 9.8.1 | Same as 9.8.1 |
| 9.8.3 TDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Response time | 300 ms |
| 10.1 FDD RSTD Measurement Reporting Delay for Carrier Aggregation | Response time | 300 ms |
| 10.1A FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz | Response time | 300 ms |
| 10.1B FDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth | Response time | 300 ms |
| 10.1C FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Response time | 300 ms |
| 10.2 TDD RSTD Measurement Reporting Delay for Carrier Aggregation | Response time | 300 ms |

<End of Changes >