3GPP TSG-RAN5 Meeting #90-e	R5-210780

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5265 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.6.2.2_s A-SEM for sTTI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_sTTIandPT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - A-MPR test case for sTTI has been added but A-SEM test case for sTTI iwas still missing. - The requirements of NS_33 and NS_34 in 6.6.2.2 are not required since SEM for V2X has been tested in 6.6.2.2G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding A-SEM test case for sTTI. - Removing the requirements of NS_33 and NS_34 in 6.6.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP would not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.2, 6.6.2.2_s(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.6.2.2	Additional Spectrum Emission Mask

6.6.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.6.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

6.6.2.2.3	Minimum conformance requirements

6.6.2.2.3.1	Minimum requirement (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

When "NS_03", “NS_11”, “NS_20” or “NS_21” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.1-1.

Table 6.6.2.2.3.1-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.1.

6.6.2.2.3.2	Minimum requirement (network signalled value "NS_04")

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.2-1. or Table 6.6.2.2.3.2-2

Table 6.6.2.2.3.2-1: Additional requirements (network signalled value "NS_04") (Rel-11 and earlier)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -25 | -25 | -25 | -25 | 1 MHz |
| 6-10 |  |  | -25 | -25 | -25 | -25 | 1 MHz |
| 10-15 |  |  |  | -25 | -25 | -25 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

Table 6.6.2.2.3.2-2: Additional requirements (network signalled value "NS_04") (Rel-12 and onwards)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-9 | -25 | -13 | -13 | -13 | 1 MHz |
| 9-10 | -25 | -25 | -13 | -13 | 1 MHz |
| 10-13.5 |  | -25 | -13 | -13 | 1 MHz |
| 13.5-15 |  | -25 | -25 | -13 | 1 MHz |
| 15-18 |  |  | -25 | -13 | 1 MHz |
| 18-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.2.

6.6.2.2.3.3	Minimum requirement (network signalled value "NS_06" or NS_07)

When "NS_06" or "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.3-1.

Table 6.6.2.2.3.3-1: Additional requirements (network signalled value "NS_06" or "NS_07")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -13 | -13 | -15 | -18 | 30 kHz |
| 0.1-1 | -13 | -13 | -13 | -13 | 100 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.3.

6.6.2.2.3.4	Minimum requirement (network signalled value “NS_27”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When “NS_27” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.4-1.

Table 6.6.2.2.3.4-1: Additional requirements (network signalled value "NS_27")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -13 | -13 | -13 | -13 | Note 1 |
| 1-10 | -13 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  | -25 | -25 | -25 | 1 MHz |
| 15-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.5.

6.6.2.2.3.5	Minimum requirement (network signalled value “NS_35”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_35" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.5-1.

Table 6.6.2.2.3.5-1: Additional requirements (network signalled value "NS_35")

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.7.

|  | -13-12∆fOOBMHz |  |
| --- | --- | --- |
|  | -19-169∆fOOBMHz-0.5 |  |
|  | -27-2∆fOOBMHz-5.0 |  |

<Unchanged Text Skipped>

6.6.2.2.5	Test requirements

6.6.2.2.5.1	Test requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

When "NS_03" or “NS_11” or “NS_20” or “NS_21” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-1, 6.2.4.5-2, 6.2.4.5-11, 6.2.4.5-20 and 6.2.4.5-21 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2.5.1-1 or 6.6.2.2.5.1-2, as applicable.

Table 6.6.2.2.5.1-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.6.2.2_s	Additional Spectrum Emission Mask for subslot/slot TTI

6.6.2.2_s.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.6.2.2_s.2	Test applicability

This test case applies to all types of E-UTRA Power Class 3 UE release 15 and forward that support subslot/slot TTI.

6.6.2.2_s.3	Minimum conformance requirements

6.6.2.2_s.3.1	Minimum requirement (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

When "NS_03", “NS_11”, “NS_20” or “NS_21” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.1-1.

Table 6.6.2.2_s.3.1-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.1.

6.6.2.2_s.3.2	Minimum requirement (network signalled value "NS_04")

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.2-2.

Table 6.6.2.2_s.3.2-2: Additional requirements (network signalled value "NS_04")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-9 | -25 | -13 | -13 | -13 | 1 MHz |
| 9-10 | -25 | -25 | -13 | -13 | 1 MHz |
| 10-13.5 |  | -25 | -13 | -13 | 1 MHz |
| 13.5-15 |  | -25 | -25 | -13 | 1 MHz |
| 15-18 |  |  | -25 | -13 | 1 MHz |
| 18-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.2.

6.6.2.2_s.3.3	Minimum requirement (network signalled value "NS_06" or NS_07)

When "NS_06" or "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.3-1.

Table 6.6.2.2_s.3.3-1: Additional requirements (network signalled value "NS_06" or "NS_07")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -13 | -13 | -15 | -18 | 30 kHz |
| 0.1-1 | -13 | -13 | -13 | -13 | 100 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.3.

6.6.2.2_s.3.4	Minimum requirement (network signalled value “NS_27”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When “NS_27” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.4-1.

Table 6.6.2.2_s.3.4-1: Additional requirements (network signalled value "NS_27")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -13 | -13 | -13 | -13 | Note 1 |
| 1-10 | -13 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  | -25 | -25 | -25 | 1 MHz |
| 15-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.5.

6.6.2.2_s.3.5	Minimum requirement (network signalled value “NS_35”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_35" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.5-1.

Table 6.6.2.2_s.3.5-1: Additional requirements (network signalled value "NS_35")

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.7.

6.6.2.2_s.4	Test description

6.6.2.2_s.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2_s.4.1-1 through 6.6.2.2_s.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2_s.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM (Note 4) | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM (Note 4) | 15 | 15 |
| 3MHz |  |  |  | 16QAM (Note 4) | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | QPSK | 6 | 6 |
| 5MHz |  |  |  | 16QAM (Note 4) | 25 | 25 |
| 5MHz |  |  |  | 16QAM (Note 4) | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | QPSK | 6 | 6 |
| 10MHz |  |  |  | 16QAM (Note 4) | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM (Note 4) | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | QPSK | 8 | 8 |
| 15MHz |  |  |  | 16QAM (Note 4) | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM (Note 4) | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | QPSK | 10 | 10 |
| 20MHz |  |  |  | 16QAM (Note 4) | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM (Note 4) | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | NA |
| 1.4MHz |  |  |  | QPSK | 5 |  |
| 1.4MHz |  |  |  | 16QAM (Note 3) | 5 |  |
| 3MHz |  |  |  | QPSK | 15 |  |
| 3MHz |  |  |  | QPSK | 4 |  |
| 3MHz |  |  |  | 16QAM (Note 3) | 4 |  |
| 5MHz |  |  |  | QPSK | 25 |  |
| 5MHz |  |  |  | QPSK | 8 |  |
| 5MHz |  |  |  | 16QAM (Note 3) | 8 |  |
| 10MHz |  |  |  | QPSK | 50 |  |
| 10MHz |  |  |  | QPSK | 12 |  |
| 10MHz |  |  |  | 16QAM (Note 3) | 12 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-4: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Mid range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Test Number | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart |
| 1 | 10MHz | N/A for Additional Spectrum Emission Mask testing. |  | QPSK | 1 | 0 |
| 2 | 10MHz |  |  | QPSK | 8 | 0 |
| 3 | 10MHz |  |  | QPSK | 6 | 13 |
| 4 | 10MHz |  |  | QPSK | 20 | 13 |
| 5 | 10MHz |  |  | QPSK | 12 | 13 |
| 6 | 10MHz |  |  | 16QAM (Note 2) | 36 (Note 1) | 13 |
| 7 | 10MHz |  |  | QPSK | 16 | 19 |
| 8 | 10MHz |  |  | QPSK | 12 | 19 |
| 9 | 10MHz |  |  | 16QAM (Note 2) | 16 | 19 |
| 10 | 10MHz |  |  | QPSK | 30 | 19 |
| 11 | 10MHz |  |  | 16QAM (Note 2) | 30 (Note 1) | 19 |
| 12 | 10MHz |  |  | QPSK | 6 | 43 |
| 13 | 10MHz |  |  | QPSK | 2 | 48 |
| 14 | 10MHz |  |  | QPSK | 50 | 0 |
| 15 | 10MHz |  |  | QPSK | 12 | 0 |
| 16 | 10MHz |  |  | 16QAM (Note 2) | 50 (Note 1) | 0 |
| Note 1: Applies only for UE-Categories 2-5. Note 2: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-5: Test Configuration Table (network signalled value "NS_11")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range For 3 MHz Channel Bandwidth: a. 2001.5 MHz (NUL = 25515) b. 2004.5 MHz (NUL = 25545) For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2004.5 MHz (NUL = 25545) c. 2007.5 MHz (NUL = 25575) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2005.5 MHz (NUL = 25555) c. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2007.5 MHz (NUL = 25575) b. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_11 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 3MHz | N/A for A-MPR testing |  | QPSK | 6 |
| 2 | 3MHz |  |  | QPSK | 15 |
| 3 | 3MHz |  |  | 16QAM (Note 4) | 6 |
| 4 | 3MHz |  |  | 16QAM (Note 4) | 15 |
| 5 | 5MHz |  |  | QPSK | 1 |
| 6 | 5MHz |  |  | QPSK | 8 |
| 7 | 5MHz |  |  | QPSK | 25 |
| 8 | 5MHz |  |  | 16QAM (Note 4) | 8 |
| 9 | 5MHz |  |  | 16QAM (Note 4) | 25 |
| 10 | 10MHz |  |  | QPSK | 1 |
| 11 | 10MHz |  |  | QPSK | 12 |
| 12 | 10MHz |  |  | QPSK | 50 |
| 13 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| 14 | 10MHz |  |  | 16QAM (Note 4) | 50 |
|  |  |  |  |  | (Note 3) |
| 15 | 15MHz |  |  | QPSK | 1 |
| 16 | 15MHz |  |  | QPSK | 8 |
| 17 | 15MHz |  |  | QPSK | 25 |
| 18 | 15MHz |  |  | QPSK | 30 |
| 19 | 15MHz |  |  | QPSK | 75 |
| 20 | 15MHz |  |  | 16QAM (Note 4) | 8 |
| 21 | 15MHz |  |  | 16QAM (Note 4) | 25 |
| 22 | 15MHz |  |  | 16QAM (Note 4) | 30 |
| 23 | 15MHz |  |  | 16QAM (Note 4) | 75 |
| 24 | 20MHz |  |  | QPSK | 1 |
| 25 | 20MHz |  |  | QPSK | 10 |
| 26 | 20MHz |  |  | QPSK | 25 |
| 27 | 20MHz |  |  | QPSK | 100 |
| 28 | 20MHz |  |  | 16QAM (Note 4) | 10 |
| 29 | 20MHz |  |  | 16QAM (Note 4) | 25 |
| 30 | 20MHz |  |  | 16QAM (Note 4) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.6.2.2_s.4.1-6: Test Configuration Table (network signalled value "NS_20")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2007.5 MHz (NUL = 25575) c. 2012.5 MHz (NUL = 25625) d. 2017.5 MHz (NUL = 25675) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_20 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 8 |
| 2 | 5MHz |  |  | QPSK | 15 |
| 3 | 5MHz |  |  | QPSK | 25 |
| 4 | 5MHz |  |  | 16QAM (Note 4) | 15 |
| 5 | 5MHz |  |  | 16QAM (Note 4) | 25 |
| 6 | 10MHz |  |  | QPSK | 8 |
| 7 | 10MHz |  |  | QPSK | 12 |
| 8 | 10MHz |  |  | QPSK | 50 |
| 9 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| 10 | 10MHz |  |  | 16QAM (Note 4) | 50 |
|  |  |  |  |  | (Note 3) |
| 11 | 15MHz |  |  | QPSK | 6 |
| 12 | 15MHz |  |  | QPSK | 25 |
| 13 | 15MHz |  |  | QPSK | 36 |
| 14 | 15MHz |  |  | QPSK | 75 |
| 15 | 15MHz |  |  | 16QAM (Note 4) | 25 |
| 16 | 15MHz |  |  | 16QAM (Note 4) | 36 |
| 17 | 15MHz |  |  | 16QAM (Note 4) | 75 |
| 18 | 20MHz |  |  | QPSK | 8 |
| 19 | 20MHz |  |  | QPSK | 18 |
| 20 | 20MHz |  |  | QPSK | 25 |
| 21 | 20MHz |  |  | QPSK | 75 |
| 22 | 20MHz |  |  | QPSK | 100 |
| 23 | 20MHz |  |  | 16QAM (Note 4) | 18 |
| 24 | 20MHz |  |  | 16QAM (Note 4) | 25 |
| 25 | 20MHz |  |  | 16QAM (Note 4) | 75 |
| 26 | 20MHz |  |  | 16QAM (Note 4) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.6.2.2_s.4.1-7: Test Configuration Table (network signalled value "NS_21")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | Low range or High range |  |  |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | 5 MHz, 10 MHz |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | Mod'n |  | RB allocation | Mod'n |  | RB allocation FDD |  | RBstart FDD |  |
| 1 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 2 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 25 |  | 0 |  |
| 3 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 4 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 49 |  |
| 5 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 15 |  | 0 |  |
| 6 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 3 |  | 47 |  |
| 7 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 50 |  | 0 |  |
| 8 (note 1, 3) |  | 10 MHz |  |  |  |  | 16QAM (Note 4) |  | 50 |  | 0 |  |
| Note 1: Applies only for UE-Categories ≥2. Note 2: Applicable only to low range frequency testing. Note 3: Applicable only to high range frequency testing. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-8: Test Configuration Table (network signalled value "NS_27")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to Frequency column in test parameters |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_27 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low/Mid | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | High | 5MHz |  |  | QPSK | 1 | 24 |
| 3 | Low/Mid/High | 5MHz |  |  | QPSK | 25 | 0 |
| 4 | Low/Mid/High | 5MHz |  |  | 16QAM | 25 | 0 |
| 5 | Low/Mid | 10MHz |  |  | QPSK | 1 | 0 |
| 6 | High | 10MHz |  |  | QPSK | 1 | 49 |
| 7 | Low/Mid/High | 10MHz |  |  | QPSK | 50 | 0 |
| 8 (Note 2) | Low/Mid/High | 10MHz |  |  | 16QAM | 50 | 0 |
| 9 | Low/Mid | 15MHz |  |  | QPSK | 1 | 0 |
| 10 | Low | 15MHz |  |  | QPSK | 1 | 7 |
| 11 | Low/Mid | 15MHz |  |  | QPSK | 16 | 0 |
| 12 | High | 15MHz |  |  | QPSK | 1 | 74 |
| 13 | High | 15MHz |  |  | QPSK | 1 | 67 |
| 14 | High | 15MHz |  |  | QPSK | 16 | 74 |
| 15 | Low/Mid/High | 15MHz |  |  | QPSK | 75 | 0 |
| 16 (Note 2) | Low/Mid/High | 15MHz |  |  | 16QAM | 75 | 0 |
| 17 | Low/Mid | 20MHz |  |  | QPSK | 1 | 0 |
| 18 | Low | 20MHz |  |  | QPSK | 1 | 13 |
| 19 | Low | 20MHz |  |  | QPSK | 1 | 16 |
| 20 | Low/Mid | 20MHz |  |  | QPSK | 21 | 0 |
| 21 | High | 20MHz |  |  | QPSK | 1 | 99 |
| 22 | High | 20MHz |  |  | QPSK | 1 | 86 |
| 23 | High | 20MHz |  |  | QPSK | 1 | 83 |
| 24 | High | 20MHz |  |  | QPSK | 21 | 0 |
| 25 | Low/Mid/High | 20MHz |  |  | QPSK | 100 | 0 |
| 26 (Note 2) | Low/Mid/High | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-9: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 5MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 25 | NA |
| 5MHz |  |  |  | QPSK | 8 |  |
| 5MHz |  |  |  | 16QAM (Note 3) | 8 |  |
| 10MHz |  |  |  | QPSK | 50 |  |
| 10MHz |  |  |  | QPSK | 12 |  |
| 10MHz |  |  |  | 16QAM (Note 3) | 12 |  |
| 15MHz |  |  |  | QPSK | 75 |  |
| 15MHz |  |  |  | QPSK | 16 |  |
| 15MHz |  |  |  | 16QAM (Note 3) | 16 |  |
| 20MHz |  |  |  | QPSK | 100 |  |
| 20MHz |  |  |  | QPSK | 18 |  |
| 20MHz |  |  |  | 16QAM (Note 3) | 18 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Tables 6.6.2.2_s.4.1-1 to 6.6.2.2_s.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2_s.4.3.

6.6.2.2_s.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to applicable table from Table 6.6.2.2_s.4.1-1 to 6.6.2.2_s.4.1-9. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.4.5-1 to 6.2.4.5-28 as appropriate. The period of the measurement shall be at least the continuous duration 7 OS (0.5ms) for slot TTI and 2/3 OS for subslot TTI.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.2.2_s.5.1-1, 6.6.2.2_s.5.3-1 or Table 6.6.2.2_s.5.1-2, 6.6.2.2_s.5.3-2, or 6.6.2.2_s.5.2-1, 6.6.2.2_s.5.4-1, 6.6.2.2_s.5.5-1 as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2_s.4.3	Message contents

Same message contents as clause 6.6.2.2.4.3 with the following exceptions:

Table 6.6.2.1_s.4.3-1: PhysicalConfigDedicated-DEFAULT (preamble)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 with condition Short-TTI |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| PhysicalConfigDedicatedSTTI-r15 SEQUENCE { |  |  |  |  |
| shortTTI-r15 SEQUENCE { |  |  |  |  |
| dl-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 OR FDD_sTTI-27 |
| ul-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD_sTTI-22 | FDD cell environment and UEs supporting {subslot, subslot} combination |
| FDD_sTTI-27 | FDD cell environment and UEs supporting {subslot, slot} combination |

6.6.2.2_s.5	Test requirements

6.6.2.2_s.5.1	Test requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

When "NS_03" or “NS_11” or “NS_20” or “NS_21” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-1, 6.2.4.5-2 ,6.2.4.5-11, 6.2.4.5-20 and 6.2.4.5-21 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.1-1 or 6.6.2.2_s.5.1-2, as applicable.

Table 6.6.2.2_s.5.1-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2_s.5.1-2: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2_s.5.2	Test requirements (network signalled value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-3,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.2-1.

Table 6.6.2.2_s.5.2-1: Additional requirements (network signalled value "NS_04")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | --8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-9 | -23.5 |  |  |  | 1 MHz |
| 9-10 |  | -23.5 |  |  | 1 MHz |
| 10-13.5 |  |  |  |  | 1 MHz |
| 13.5-15 |  |  | -23.5 |  | 1 MHz |
| 15-18 |  |  |  |  | 1 MHz |
| 18-20 |  |  |  | -23.5 | 1 MHz |
| 20-25 |  |  |  |  | 1 MHz |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2_s.5.3	Test requirements (network signalled value "NS_06" or "NS_07")

When "NS_06" or "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-5, 6.2.4.5-6 and 6.2.4.5-7,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.3-1 or Table.6.6.2.2_s.5.3-2, as applicable.

Table 6.6.2.2_s.5.3-1: Additional requirements (network signalled value "NS_06" or "NS_07"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2_s.5.3-2: Additional requirements (network signalled value "NS_06" or "NS_07"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.2 | -11.2 | -13.2 | -16.2 | 30 kHz |
| 0.1-1 | -11.2 | -11.2 | -11.2 | -11.2 | 100 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  | 1 MHz |
| 10-15 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2_s.5.4	Test requirements (network signalled value "NS_27")

When "NS_27" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-27,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.4-1, as applicable.

Table 6.6.2.2_s.5.4-1: Additional requirements (network signalled value "NS_27")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -11.2 | -11.2 | -11.2 | -11.2 | Note 1 |
| 1-10 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 10-15 |  | -23.2 | -23.2 | -23.2 | 1 MHz |
| 15-20 |  |  | -23.2 | -23.2 | 1 MHz |
| 20-25 |  |  |  | -23.2 | 1 MHz |
| Note 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

6.6.2.2_s.5.5	Test requirements (network signalled value "NS_35")

When "NS_35" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-28,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.5-1, as applicable.

Table 6.6.2.2_s.5.5-1: Additional requirements (network signalled value "NS_35")

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 0.1-6 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 6-10 | -23.51 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 10-15 |  | -23.51 | -11.5 | -11.5 | 100 kHz |
| 15-20 |  |  | -23.51 | -11.5 | 100 kHz |
| 20-25 |  |  |  | -23.5 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

<End of Changes>