3GPP TSG-RAN5 Meeting #90-e	R5-210869

# Electronic Meeting, 22th Feb – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0285 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Test Purpose of PDCCH test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Test Purpose in clauses 5.3.2 and 5.3.3 are not clear and need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified Test Purpose in section 5.3.2 and 5.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be performed properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2, 5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

5.3.2	2RX requirements

5.3.2.1	FDD

The parameters specified in Table 5.3.2.1-1 are valid for all FDD tests unless otherwise stated.

Table 5.3.2.1-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  | nonInterleaved |  |
| REG bundle size |  | 6 |  |
| Shift index |  | 0 |  |

5.3.2.1.1	2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.2.1.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.1.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.1.1.3-1: Minimum performance for 1 Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.1 FDD | TDLA30-10 | 1x2 Low | 1 | 8.1 |
| 2 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.3 FDD | TDLC300-100 | 1x2 Low | 1 | 8.2 |
| 3 | 10 MHz | 48 | 2 | 4 | R.PDCCH.1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 5.5 |
| 4 | 10 MHz | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low | 1 | 4.4 |
| 5 | 10 MHz | 48 | 2 | 16 | R.PDCCH.1-2.6 FDD | TDLA30-10 | 1x2 Low | 1 | -2.1 |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.

5.3.2.1.1.4	Test description

5.3.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A in Figure A.3.1.7.2 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1 and Table 5.3.2.1.1-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.1.4.3.

5.3.2.1.1.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.2.1.1-1. The details of PDCCH are specified in Table 5.3.1, Table 5.3.2.1-1, Table 5.3.2.1.1.3-1 respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.1.1-1 as appropriate.

3. 	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.1.1-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.2.1.1-1 as appropriate.

5.3.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.2.1.1.4.3.1	Message exceptions for SA

Table 5.3.2.1.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3, 4, 5 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2, 3, 5 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 4 |  |
| tci-StatesPDCCH-ToAddList { |  |  |  |
|  | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 11000000000000 | Symbols 0 and 1 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1, Test 2 |
| aggregationLevel4 | n1 | AL4 | Test 3, Test 4 |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n1 | AL16 | Test 5 |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for tests 1, 4, 5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.1.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: this table is a duplication of the aboveand was renumbered to 3A. Conflict has to be resolved!

Table 5.3.2.1.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 4, Test 5 |
| } |  |  |  |

5.3.2.1.1.4.3.2	Message exceptions for NSA

Same as 5.3.2.1.1.4.3.1

5.3.2.1.1.4.4	Test requirement

Table 5.3.2.1.1.4.4-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.1.4.4-1.

Table 5.3.2.1.1.4.4-1: Test Requirement for 1Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.1 FDD | TDLA30-10 | 1x2 Low | 1 | 9.0 |
| 2 | 10 MHz | 24 | 2 | 2 | R.PDCCH.1-2.3 FDD | TDLC300-100 | 1x2 Low | 1 | 9.1 |
| 3 | 10 MHz | 48 | 2 | 4 | R.PDCCH.1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 6.4 |
| 4 | 10 MHz | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low | 1 | 5.3 |
| 5 | 10MHz | 48 | 2 | 16 | R.PDCCH.1-2.6 FDD | TDLA30-10 | 1x2 Low | 1 | -1.2 |

5.3.2.1.2	2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.2.1.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.1.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.1.2.3-1: Minimum performance for 2 Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x2 Low | 1 | 2.0 |
| 2 | 10 MHz | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x2 Low | 1 | -1.3 |
| 3 | 10 MHz | 48 | 1 | 8 | R.PDCCH.1-1.3 FDD | TDLA30-10 | 2x2 Low | 1 | -0.2 |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.

5.3.2.1.2.4	Test description

5.3.2.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.1-1 and Table 5.3.2.1.2.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.1.2.4.3.

5.3.2.1.2.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channelfor C_RNTI to transmit the DL RMC according to Table 5.3.2.1.2.3-1. The details of PDCCH are specified in Table  5.3.1, Table 5.3.2.1-1, Table 5.3.2.1.2.3-1 respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.1.2.3-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.1.2.4.4-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.2.1.2.3-1 as appropriate.

5.3.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.2.1.2.4.3.1	Message exceptions for SA

Table 5.3.2.1.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 2, 3 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 3 |  |
| tci-StatesPDCCH-ToAddList { |  |  |  |
|  | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n1 | AL4 | Test 1 |
| aggregationLevel8 | n1 | AL8 | Test 2, 3 |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for test 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.2.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1 |
| } |  |  |  |

5.3.2.1.2.4.3.2	Message exceptions for NSA

Same as 5.3.2.1.2.4.3.1

5.3.2.1.2.4.4	Test requirement

Table 5.3.2.1.2.4.4-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.2.4.4-1.

Table 5.3.2.1.2.4.4-1: Test Requirements for 2 Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x2 Low | 1 | 3.0 |
| 2 | 10 MHz | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x2 Low | 1 | -0.3 |
| 3 | 10 MHz | 48 | 1 | 8 | R.PDCCH.1-1.3 FDD | TDLA30-10 | 2x2 Low | 1 | 0.8 |

5.3.2.2	TDD

The parameters specified in Table 5.3.2.2-1 are valid for all TDD tests unless otherwise stated.

Table 5.3.2.2-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |  |
| CCE to REG mapping type |  | Test 3: non-interleaved Other tests: interleaved | interleaved |  |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | Test 3: 6 Other tests: 2 |  | 6 |  |
| Shift Index |  | 0 |  |  |  |

5.3.2.2.1	2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.2.2.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.2.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.2.1.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x2 Low | 1 | 7.0 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLC300- 100 | 1x2 Low | 1 | -3.8 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.2.2.

5.3.2.2.1.4	Test description

5.3.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2 for TE diagram and section A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.1.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.2.1.4.3.

5.3.2.2.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.2.2.1.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.1.3-1 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.2.1.3-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.2.1.5-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.2.2.1.3-1 as appropriate.

5.3.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.3.2.2.1.4.3.1	Message exceptions for SA

Table 5.3.2.2.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3 |  |
|  | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 3 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 1, 2 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  | Test 1, Test 2 |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| nonInterleaved | null |  | Test 3 |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1 |
| aggregationLevel4 | n1 | AL4 | Test 2 |
| aggregationLevel16 | n1 | AL16 | Test 3 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for test 2 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for tests 1 and 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.1.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: following table was renumbered to 3A. Conflict has to be resolved!

Table 5.3.2.2.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 3 |
| } |  |  |  |

5.3.2.2.1.4.3.2	Message exceptions for NSA

Same as 5.3.2.2.1.4.3.1.

5.3.2.2.1.5	Test requirement

Table 5.3.2.2.1.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.1.5-1.

Table 5.3.2.2.1.5-1: Test Requirement for 1Tx PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x2 Low | 1 | 7.9 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.9 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLC300- 100 | 1x2 Low | 1 | -2.9 |

5.3.2.2.2	2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.2.2.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.2.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.3.2.2.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.2.2.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 90 | 1 | 8 | R.PDCCH.2-1.3 TDD | TDLC300-100 | 2x2 Low | 1 | -1.2 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.2.2.

5.3.2.2.2.4	Test description

5.3.2.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause TBD.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and section A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.2.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.2.2.4.3.

5.3.2.2.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH with DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.3.2.2.2.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.2.3-1 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.2.2.3-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.2.2.5-1, pass the UE. Otherwise fail the UE.

5.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.2.2.2.4.3.1	Message exceptions for SA

Table 5.3.2.2.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 11111110 00000000 00000000 00000000 00000 | CORESET to use the least significant 90 RBs of the BWP Test 1 |  |
| Duration | 1 | SearchSpace duration of 1 symbols Test 1 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n6 |  | 2 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel8 | n1 | AL8 | Test 1 |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.2.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.3.2.2.2.4.3.2	Message exceptions for NSA

Same as 5.3.2.2.2.4.3.1.

5.3.2.2.2.5	Test requirement

Table 5.3.2.2.2.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.2.5-1.

Table 5.3.2.2.2.5-1: Test Requirement for 2Tx PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 90 | 1 | 8 | R.PDCCH.2-1.3 TDD | TDLC300-100 | 2x2 Low | 1 | -0.2 |

5.3.3	4RX requirements

5.3.3.1	FDD

The parameters specified in Table 5.3.3.1-1 are valid for all FDD tests unless otherwise stated.

Table 5.3.3.1-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  | nonInterleaved |  |
| REG bundle size |  | 6 |  |
| Shift index |  | 0 |  |

5.3.3.1.1	4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.3.1.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.1.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.1.1.3-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x4 Low | 1 | 2.2 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300- 100 | 1x4 Low | 1 | 2.7 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.2 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x4 Low | 1 | -0.4 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x4 Medium A | 1 | -3.2 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.1.

5.3.3.1.1.4	Test description

5.3.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.1-1 and Table 5.3.3.1.1.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.1.1.4.3.

5.3.3.1.1.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channelfor C_RNTI to transmit the DL RMC according to Table 5.3.3.1.1.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.1-1 and Table 5.3.3.1.1.3-1 respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.1.1.3-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.1.1.5-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.3.1.1.3-1 as appropriate.

5.3.3.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.3.1.1.4.3.1	Message exceptions for SA

Table 5.3.3.1.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3, 4, 5 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2, 3, 5 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 4 |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1, Test 2 |
| aggregationLevel4 | n1 | AL4 | Test 3, Test 4 |
| aggregationLevel16 | n1 | AL16 | Test 5 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for tests 1, 4, 5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.1.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: duplicated table below was renamed to 3A. Conflict has to be resolved!

Table 5.3.3.1.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 4, Test 5 |
| } |  |  |  |

5.3.3.1.1.4.3.2	Message exceptions for NSA

Same as 5.3.3.1.1.4.3.1.

5.3.3.1.1.5	Test requirement

Table 5.3.3.1.1.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.1.5-1.

Table 5.3.3.1.1.5-1: Test Requirement for 1Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x4 Low | 1 | 3.1 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300- 100 | 1x4 Low | 1 | 3.6 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 1.1 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x4 Low | 1 | 0.5 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x4 Medium A | 1 | -2.3 |

5.3.3.1.2	4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.3.1.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.1.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.1.2.3-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x4 Low | 1 | -1.9 |
| 2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x4 Low | 1 | -4.5 |
| 3 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.2 FDD | TDLA30-10 | 2x4 Low | 1 | -1.0 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.1.

5.3.3.1.2.4	Test description

5.3.3.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.4 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.1-1 and Table 5.3.3.1.2.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On) for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.1.2.4.3.

5.3.3.1.2.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.1.2.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.1-1 and Table 5.3.3.1.2.3-1 respectively. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.1.2.3-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.1.2.5-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.3.1.2.3-1 as appropriate.

5.3.3.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.3.1.2.4.3.1	Message exceptions for SA

Table 5.3.3.1.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 2, 3 |  |
|  | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP Test 1 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 1, 2 |  |
|  | 1 | SearchSpace duration of 1 symbol Test3 |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | AL4 | Test 1, Test 3 |
| aggregationLevel8 | n1 | AL8 | Test 2 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for tests 2 and 3 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for test 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.2.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: duplicated table below was renamed to 3A. Conflict has to be resolved!

Table 5.3.3.1.2.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1 |
| } |  |  |  |

5.3.3.1.2.4.3.2	Message exceptions for NSA

Same as 5.3.3.1.2.4.3.1.

5.3.3.1.2.5	Test requirement

Table 5.3.3.1.2.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.2.5-1.

Table 5.3.3.1.2.5-1: Test Requirement for 2Tx PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x4 Low | 1 | -0.9 |
| 2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x4 Low | 1 | -3.5 |
| 3 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.2 FDD | TDLA30-10 | 2x4 Low | 1 | 0 |

5.3.3.2	TDD

The parameters specified in Table 5.3.3.2-1 are valid for all TDD tests unless otherwise stated.

Table 5.3.3.2-1: Common Test Parameters

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |  |
| CCE to REG mapping type |  | Test 3: Non-interleaved Other tests: interleaved | interleaved |  |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | Test 3: 6 Other tests: 2 |  | 6 |  |
| Shift Index |  | 0 |  |  |  |

5.3.3.2.1	4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA

5.3.3.2.1.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.2.1.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.1.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.2.1.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x4 Low | 1 | 2.1 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300-100 | 1x4 Low | 1 | -0.9 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLA30-10 | 1x4 Medium A | 1 | -3.6 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.2.

5.3.3.2.1.4	Test description

5.3.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.1.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On) for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.2.1.4.3.

5.3.3.2.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.2.1.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.1.3-1 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.2.1.3-1 as appropriate.

3.   Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause [G.1.x]. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.2.1.5-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.3.2.1.3-1 as appropriate.

5.3.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.3.2.1.4.3.1	Message exceptions for SA

Table 5.3.3.2.1.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP Test 3 |  |
|  | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP Test 1, 2 |  |
| Duration | 2 | SearchSpace duration of 2 symbols Test 3 |  |
|  | 1 | SearchSpace duration of 1 symbol Test 1, 2 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  | Test 1, Test 2 |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| nonInterleaved | null |  | Test 3 |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.1.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel2 | n1 | AL2 | Test 1 |
| aggregationLevel4 | n1 | AL4 | Test 2 |
| aggregationLevel16 | n1 | AL16 | Test 3 |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 for test 2 | Long_DCI |
|  | formats0-0-And-1-0 | DCI Format 1_0 for test 1 and 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.1.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: duplicated table below was renamed to 3A. Conflict has to be resolved!

Table 5.3.3.2.1.4.3.1-3A: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  | Test 1, Test 3 |
| } |  |  |  |

5.3.3.2.1.4.3.2	Message exceptions for NSA

Same as 5.3.3.2.1.4.3.1.

5.3.3.2.1.5	Test requirement

Table 5.3.3.2.1.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.1.5-1.

Table 5.3.3.2.1.5-1: Test Requirement for 1Tx PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH.2-1.1 TDD | TDLA30-10 | 1x4 Low | 1 | 3 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH.2-1.2 TDD | TDLC300-100 | 1x4 Low | 1 | 0 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH.2-2.1 TDD | TDLA30-10 | 1x4 Medium A | 1 | -2.7 |

5.3.3.2.2	4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA

5.3.3.2.2.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4 Rx antenna ports.

5.3.3.2.2.3	Minimum conformance requirements

For the parameters specified in Table 5.3.3.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.2.3-1. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.2.2.3-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 90 | 1 | 8 | R.PDCCH.2-1.3 | TDLC300-100 | 2x4 Low | 1 | -4.3 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.2.

5.3.3.2.2.4	Test description

5.3.3.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.4 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.2.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.1.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.2.2.4.3.

5.3.3.2.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.2.2.3-1. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.2.3-1. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.2.2.3-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.2.2.5-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.3.2.2.3-1 as appropriate.

5.3.3.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.3.3.2.2.4.3.1	Message exceptions for SA

Table 5.3.3.2.2.4.3.1-1: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11111111 11111110 00000000 00000000 00000000 00000 | CORESET to use the least significant 90 RBs of the BWP Test 1 |  |
| Duration | 1 | SearchSpace duration of 1 symbols Test 1 |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n6 |  | 2 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList { | 0 | TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.2.4.3.1-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel8 | n1 | AL8 | Test 1 |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.2.4.3.1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.3.3.2.2.4.3.2	Message exceptions for NSA

FFS

5.3.3.2.2.5	Test requirement

Table 5.3.3.2.2.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.2.5-1.

Table 5.3.3.2.2.5-1: Test Requirement for 2Tx PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 90 | 1 | 8 | R.PDCCH.2-1.3 | TDLC300-100 | 2x4 Low | 1 | -3.3 |

<End of modified section>