3GPP TSG-RAN5 Meeting #90-e	R5-210882

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0114 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 7.1-Shorter Retry-after period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current TC, the SS set the Retry-after period of 60 seconds when replying the UE with 503 Service Unavailable. In real practise, it is not necessary to set such a long time, and it might exceed some UE’s internal threshold of waiting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the Retry-after period from 60 seconds to 20 seconds. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Legit UEs might fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

## 7.1	MTSI MO Voice Call / 503 Service Unavailable / 5GS

### 7.1.1	Test Purpose (TP)

(1)

with { UE having sent an initial INVITE request for MO Voice call }

ensure that {

when { UE receiving a 503 Service Unavailable response containing a Retry-After header indicating a period of 20 seconds }

then { UE does not reattempt the request until after the indicated period }

}

(2)

with { UE waiting for a period of 20 seconds to expire }

ensure that {

when { period expires }

then { UE sends initial INVITE request and completes setup of MO Voice call }

}

...

#### 7.1.3.2	Test procedure sequence

Table 7.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call | - | - | - | - |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel with Step 8, parallel behaviour defined in table 7.1.3.2-2 takes place | - | - | - | - |
| 8 | Step 1 of subclause A.4.2 happens | --> | INVITE | - | - |
| 9 | SS sends 503 (Service Unavailable) with Retry-After header indicating a period of 20 seconds. | <-- | 503 Service Unavailable | - | - |
| 10 | UE acknowledges the reception of 503 (Service Unavailable) message. | --> | ACK | - | - |
| 11 | The SS starts timer t_Waits=20s. | - | - | - | - |
| 12 | Check: Does the UE transmit INVITE request message. | --> | INVITE | 1 | F |
| 13 | The SS waits for expiry of t_Waits. | - | - | - | - |
| 14 | Step 1 of subclause A.4.1 happens | --> | INVITE | 2 | P |
| 15-25 | Steps 2 to 12 of subclause A.4.1 happens | - | - | - | - |

Table 7.1.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

#### 7.1.3.3	Specific message contents

Table 7.1.3.3-1: 503 Service Unavailable (step 9, table 7.1.3.2-1)

| Derivation path: TS 34.229-1 [2], Table in subclause A.4.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Retry-After |  |  |  |  |
| delta-seconds |  | 20 |  |  |

<End of modified section 1>