3GPP TSG-RAN5 Meeting #90-e	R5-211194
Electronic Meeting, 22 February – 5 March 2021

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On Vendor Declaration Related to Antenna Aperture D for DFF

Document for:	Discussion and Endorsement

# Introduction

This contribution is addressing the need for antenna aperture D declaration for DFF methodologies.

# Discussion

While the IFF methodology does not require any vendor declaration in terms of the antenna apertures, D, integrated inside of the DUT, e.g., clause B.2.4.1 of [1]

The applicability criteria of this test method are: - The total test volume, i.e., the quiet zone is defined as a sphere with radius R. - DUT must fit within the quiet zone for the entire duration of the test. - Either a single radiating aperture, multiple non-coherent apertures or multiple coherent apertures DUTs can be tested. - No manufacturer declaration of the antenna array size is needed.

On the other hand, since the range lengths of the DFF-based systems depend on the antenna apertures, D, i.e., Clause  B.2.2.4 of [1]

With this setup, the minimum range length, RDFF, can be determined as RDFF = RQZ – D/2 + RFF = RQZ – D/2 + 2D2/

it was well understood that such systems (DFF, hybrid DFF/IFF) could not support arbitrary antenna apertures, e.g., Table 1 outlines the required range lengths for DFF systems supporting a 30cm QZ. Clearly, systems with range lengths of  >> 1m are impractical and not feasible due to the low PSD testability issues.

Table 1: DFF Range Lengths for 30cm QZ DFF Systems

| f [GHz] D [cm] | 24.25 | 30 | 40 | 43.5 | 52.6 |
| --- | --- | --- | --- | --- | --- |
| 2 | 0.20 | 0.22 | 0.25 | 0.26 | 0.28 |
| 3 | 0.28 | 0.32 | 0.38 | 0.40 | 0.45 |
| 4 | 0.39 | 0.45 | 0.56 | 0.59 | 0.69 |
| 5 | 0.53 | 0.63 | 0.79 | 0.85 | 1.00 |
| 7.5 | 1.02 | 1.24 | 1.61 | 1.74 | 2.09 |
| 10 | 1.72 | 2.10 | 2.77 | 3.00 | 3.61 |
| 15 | 3.72 | 4.58 | 6.08 | 6.60 | 7.97 |
| 20 | 6.52 | 8.06 | 10.72 | 11.66 | 14.09 |

In [2], the applicability of DFF systems for PC3 was linked to whether D was limited to 5cm or not.

A permitted test method will have applicability to at least one test case. Applicability is a function of DUT Antenna Configuration as defined in Table 5.3-1 DUT Antenna Configuration can be chosen by an optional declaration from a manufacturer. Table 5.3-1: DUT Antenna Configuration DUT Antenna Configuration; Description. 1; Maximum one antenna panel with D ≤ 5 cm active at any one time. 2; More than one antenna panel D ≤ 5 cm without phase coherence between panels active at any one time. 3; Any phase coherent antenna panel of any size (e.g. sparse array). Table 5.3-2 indicates the high-level applicability of test methods by DUT Antenna Configuration. Table 5.3-2: Overview of test method applicability for permitted test methods DUT Antenna Configuration; Direct Far Field (DFF); Indirect Far Field (IFF); Near Field to far field transform (NFTF). 1; Yes; Yes; Yes. 2; Yes; Yes; Yes. 3; No; Yes; No. NOTE: A positive indication means that applicability exists for at least one RF test cases for the given DUT Antenna Configuration.

Subsequently, the applicability of DFF was specified in clause B.2.2.1 of [1]:

The applicability criteria of the DFF setup are: - The DUT radiating aperture is D ≤ 5 cm - Either a single radiating aperture, multiple non-coherent apertures, or multiple coherent apertures DUTs can be tested - If multiple antenna panels that are phase coherent are defined as a single array, the criterion on DUT radiating aperture applies to this single array - D is based on the MU assessment in Annex B.1.1.3 of TR 38.810 [24] - A measurement distance larger than the far-field criteria defined in B.2.2.4 is not precluded - If the uncertainties can be further optimized, the MU may be reduced or D may be increased - A manufacturer declaration on the following elements is needed unless the entire DUT size is contained in a sphere of diameter of ≤ 5 cm: - Manufacturer declares antenna array size

The applicability of DFF is captured in clause I.0 in [3] which is clearly linking the applicability to the DUT antenna configurations and whether D is ≤5cm or not.

The applicability of each permitted test method for the different RRM Angle of Arrival (AoA) setups as defined in Section A.9 is defined in Table I.0-1. Table I.0-1 AoA Test Setup applicability per permitted test method AoA Test Setup; No DUT antenna configuration declaration; DUT antenna configuration declaration. Configuration 1 (one antenna panel with D ≤ 5 cm active at any one time); Configuration 2 (More than one antenna panel D ≤ 5 cm without phase coherency between panels active at any one time); Configuration 3 (Any phase coherent antenna panel of any size). Setup 1; IFF; DFF, DFF simplification, IFF; DFF, DFF simplification, IFF; IFF. Setup 2a; IFF; DFF, DFF simplification, IFF; DFF, DFF simplification, IFF; IFF. Setup 2b; IFF; DFF, DFF simplification, IFF; DFF, DFF simplification, IFF; IFF. Setup 3; IFF; DFF, IFF, IFF+DFF; DFF, IFF, IFF+DFF; IFF. Setup 4; IFF; DFF, IFF, IFF+DFF; DFF, IFF, IFF+DFF; IFF. NOTE: D = DUT radiating aperture declared by UE vendor..

To date, radiating aperture declarations have not been captured in [4] mainly because the de-facto IFF methodology did not require any. Since some focus has shifted towards the DFF methodology, especially for 2 AoA RRM test cases, it is important to define the vendor declaration of the radiation aperture.

Observation 1: Radiating aperture declarations have not been captured in 38.508-2.

Observation 2: Since some focus has shifted towards the DFF methodology, especially for 2 AoA RRM test cases, it is important to define the vendor declaration of the radiation aperture in 38.508-2.

One approach could be to have UE vendors declare the actual antenna aperture of the largest antenna panel integrated in the UE, i.e., declaration of maximum radiation aperture (Option 1).

On the other hand, UE vendors have been reluctant to declare detailed antenna information which led to the adoption of the black-box instead of the white-box approach in RAN4. An alternate vendor declaration (Option 2) that would prevent UE vendors from declaring detailed antenna panel information could be to declare whether or not the antenna aperture D is ≤ 5cm or not. The benefit of this approach is that it would be aligned with the PC3 definitions of the DUT antenna configuration limits and the applicability definition of DFF in [1]. Additionally, it might not be entirely clear how the antenna aperture is defined by OEMs.

As a TE vendor, we do not have a strong view but wanted to outline various options and ask OEMs for feedback before implementing the agreed upon option in [4].

Observation 3: UE vendors have been reluctant to declare the detailed radiating aperture information

Proposal 1: OEMs to provide feedback on which antenna aperture declaration to incorporate in TS38.508-2 for DFF methodology: 
--- Option 1: UE vendors declares the exact maximum radiation aperture of any of the panels integrated in the UE
--- Option 2: UE vendor declares whether the maximum radiation aperture of any of the panels integrated in the UE is ≤5cm or not
--- Other options are not precluded

A CR to make the necessary changes is in [5].

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1: Radiating aperture declarations have not been captured in 38.508-2.

Observation 2: Since some focus has shifted towards the DFF methodology, especially for 2 AoA RRM test cases, it is important to define the vendor declaration of the radiation aperture in 38.508-2.

Observation 3: UE vendors have been reluctant to declare the detailed radiating aperture information

Proposal 1: OEMs to provide feedback on which antenna aperture declaration to incorporate in TS38.508-2 for DFF methodology: 
--- Option 1: UE vendors declares the exact maximum radiation aperture of any of the panels integrated in the UE
--- Option 2: UE vendor declares whether the maximum radiation aperture of any of the panels integrated in the UE is ≤5cm or not
--- Other options are not precluded

# References

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment, V16.6.0 (2020-12)

- TR38.810, Study on test methods, V16.6.1 (2020-09)

- TS 38.533, TS 38.533, User Equipment (UE) conformance specification; Radio Resource Management (RRM), V16.5.0 (2020-09)

- TS 38.508-2, User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma, V16.6.0

- R5-211195, CR to 38.508-2 on Antenna Aperture Declarations, Keysight Technologies, 3GPP TSG-RAN5 Meeting #90-e, March 2021