3GPP TSG-RAN5 Meeting #90-e                      	R5-211213

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1119 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.1 including TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case 5.6.2.1 is incomplete and there are some mistakes which RAN4 addressed in R4-2101701. |  |  |  |  |  |  |  |  |
| R4- |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed Noc, add Es and correct BB, Io values for inter-frequency measurement test cases. - Removed redundant Noc-related parameters, as no noise is applied - Updated Es/Iot to baseband value and correct Io in 5.6.2.1 and 5.6.2.3 test cases - Specified AWGN propagation per frequency - Test Tolerance applied - Editor’s notes removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

#### 5.6.2.1	EN-DC FR2-FR2 event-triggered reporting in non-DRX

5.6.2.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

5.6.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC.

5.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.5.6.2.1.

5.6.2.1.4	Test description

5.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.6.2.1.4.1-1.

Table 5.6.2.1.4.1-1: EN-DC FR2-FR2 event triggered reporting tests in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 5.6.2.1-1 | LTE FDD, 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 5.6.2.1-2 | LTE TDD, 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 5.6.2.1.4-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 | 13 | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 39 | 39 |  |
| SMTC-SSB parameters |  | Config 1,2 | SSB.3 FR2 |  | As specified in clause A.3.2 |
| A3-Offset | dB | Config 1,2 | -30 |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |
| CP length |  | Config 1,2 | Normal |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |
| T2 | s | Config 1,2 | 5.2 for PC1; 3.5 for other PC | 5.2 for PC1; 3.5 for other PC | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 5.6.2.1.4.1-3: Test Environment test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 5.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 5.6.2.1.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR1 cells in different frequencies. Cell 2 is the PSCell and Cell 3 is the target cell. The power levels and settings for Cell 2 are set according to Annex C.1.2 and Annex C.1.3. Cell 3 is switched off during the initial connection setup.

3.	If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

4. The AoA setup for this test is Setup 3 as defined in clause A.9

5.6.2.1.4.2	Test procedure

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR2 on NR RF channel 1 and NR cell 3 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 5.6.2.1.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table 5.6.2.1.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 5.6.2.1.4.1-2.

3. The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4. The UE shall transmit RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 5.6.2.1.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delay measured from the beginning of time period T2 is less than 5120 ms for UE supporting power class 1, or 3200 ms for UE supporting other power class for Test 1 and Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 3 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On  according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 5.6.2.1.4.1-2 as appropriate.

5.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 5.6.2.1.4.3-1: Common Exception messages for Additional EN-DC FR2-FR2 event triggered reporting tests in non-DRX test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with A3-offset = -30dB Table H.3.1-6 with Conditions gapFR2 and Pattern #13 for Test 2 Table H.3.4-4 with Condition gapUE Test 1 Table H.3.4-5 with Condition Pattern #0 for Test 1 Table H.3.1-7 with Condition INTER-FREQ Table H.3.4-1 Table H.3.4-2 Table H.3.4-3 |

5.6.2.1.5	Test requirement

Table 5.6.2.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 5.6.2.1.5-1: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter |  |  | Unit |  |  | Test configuration |  |  | Cell 2 |  |  |  |  | Cell 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  |  |  |  | Config 1,2 |  |  | Setup 3 as specified in clause A.9 |  |  |  |  |  |  |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Assumption for UE beamsNote 7 |  | Config 1,2 | Rough |  | Rough |  |
| NR RF Channel Number |  | Config 1,2 | 1 |  | 2 |  |
| Duplex mode |  | Config 1,2 | TDD |  | TDD |  |
| BWchannel | MHz | Config 1,2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW | MHz | Config 1,2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| TDD configuration |  | Config 1,2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Initial DL BWP |  | Config 1,2 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  | Config 1,2 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  | Config 1,2 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  | Config 1,2 | ULBWP.1.1 |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | Config 1,2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | Config 1,2 | TRS.2.1 TDD |  | NA |  |
| TCI configuration |  | Config 1,2 | CSI-RS.Config.0 |  | NA |  |
| PDSCH Reference measurement channel |  | Config 1,2 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  | Config 1,2 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 |  | Config 1,2 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2 | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  | Config 1,2 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Ês |  | dBm/SCS |  |  | Config 1,2 |  |  | -87 |  |  | -87 |  |  |  | -87 |  | -87 |  |  |
| SSB-RP Note 3 |  | dBm/SCS Note5 |  |  | Config 1,2 |  |  | -87 |  | -87 |  |  |  |  | -Infinity |  | -87 |  |  |
| BB Note 8 |  | dB |  |  | Config 1,2 |  |  | 1.89 |  | 1.89 |  |  |  |  | -Infinity |  | 1.89 |  |  |
| IoNote3 |  | dBm/95.04 MHz Note5 |  |  | Config 1,2 |  |  | -58.01 |  | -58.01 |  |  |  |  | -Infinity |  | -58.01 |  |  |
| Propagation Condition |  |  |  |  |  | Config 1,2 |  |  | AWGN |  |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 [6] Annex B.2.1.3, and does not limit UE implementation or test system implementation. Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap and in test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

///End of changes