3GPP TSG-RAN5 Meeting #90-e	R5-211345

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5011 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE test case 8.2.4.1 for CAT-M mode UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Handover from Cell 4 to Cell 1 at step 9 and handover from cell 1 to cell 11 at step 13 is triggered by sending RRC Connection Reconfiguration message but the message content is not including PhysicalConfigDedicated RRC Connection reconfiguration message carries , mpdcch_NarrowbandsToMonitor_r13 as part of prach_ParametersListCE_r13 which is enabling the UE to monitor MPDCCH for RAR on Target Cell UE is not provided with any information about monitoring of MPDCCH on Target cell as the field “ radioResourceConfigDedicated->PhysicalConfigDedicated->Extension_Addition_Group_40>epddch_config_r11 “ is missing This gap is creating Issue while running the test case in 10MHz bandwidth. The value mpdcch_NarrowbandsToMonitor_r13 is specified in 7.3.3.9 of 36.523-3 depending on the Physical cell id configuration. If RRCConnectionReconfiguration does not contain radioResourceConfigDedicated->PhysicalConfigDedicated->Extension_Addition_Group_40>epddch_config_r11 , then the mpdcch_config_r13 will not match between SS and UE as the UE will use the storted context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EPDCCH_Config_r11 to be included as part of PhysicalConfigDedicated while sending the RRC Connection Reconfiguration which is to trigger the HO [ step 9 and at step 13 of the test case ] |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase when executing the test case in 10MHz |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Outcome of R5-211023 with 1 revision |  |  |  |  |  |  |  |  |

### 8.2.4	Handover

#### 8.2.4.1	RRC connection reconfiguration / Handover / Success / Dedicated preamble

8.2.4.1.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement}

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a mobilityControlInfo with a rach-ConfigDedicated }

then { UE transmits an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a nextHopChainingCount which is different from the NCC associated with the currently active KeNB }

then { UE derives new KeNB key from the nextHopChainingCount }

}

(3)

with { UE having completed the radio bearer establishment and initial security activation procedure and performed the intra frequency measurement }

ensure that {
  when { UE receives an RRCConnectionReconfiguration message including a nextHopChainingCount which is same as the NCC associated with the currently active KeNB }

then { UE derives new KeNB key from the currently active KeNB }

}

8.2.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.5.4, 5.3.10.4 and 5.3.10.6.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	stop timer T310, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	if the carrierFreq is included:

2>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target cell to be one on the current frequency with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset MAC;

1>	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

1>	re-establish RLC for all RBs that are established;

1>	apply the value of the newUE-Identity as the C-RNTI;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2>	update the KeNB key based on the fresh KASME key taken into use with the previous successful NAS SMC procedure, as specified in TS 33.401 [32];

1>	else:

2>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	if the securityAlgorithmConfig is included in the securityConfigHO:

2>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	else:

2>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target cell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target cell by acquiring system information from that cell before performing RACH access in the target cell.

[TS 36.331, clause 5.3.10.4]

The UE shall:

1>	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

[TS 36.331, clause 5.3.10.6]

The UE shall:

1>	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1>	if the antennaInfo is included and set to 'explicitValue':

2>	if the configured transmissionMode is not 'tm3' or 'tm4' release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1>	else if the antennaInfo is included and set to 'defaultValue':

2>	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

8.2.4.1.3	Test description

8.2.4.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 4 and Cell 11.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.2.4.1.3.2	Test procedure sequence

Table 8.2.4.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 4 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | ”Off” | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy exit condition for event A3 (M4 < M1)(NOTE 1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | ”Off” | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy entry condition for event A3 (M4 > M1)(NOTE 1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | "Off" | The power level values are such that measurement results for Cell 1 (M1) and Cell 4 (M4) satisfy entry condition for event A3 (M1 > M4)(NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” | -79 | The power level value are such that measurement results for Cell 1 (M1) and Cell 11 (M11) satisfy entry condition for event A3 (M11 > M1). |
| NOTE 1: Power level “Off” is defined in TS36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.2.4.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T1" in table 8.2.4.1.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A3 on Cell 1 with the measured RSRP, RSRQ value for Cell 4. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 4. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 4 using dedicated preamble to confirm the successful completion of the intra frequency handover? Check2: Does the UE transmit an RRCConnectionReconfigurationComplete using the security key derived from the currently active KeNB? | --> | RRCConnectionReconfigurationComplete | 1,3 | P |
| - | EXCEPTION: Steps 6a1 to 6a2 describe behaviour that depends on the UE capability; the “lower case letter” identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 6a1 | IF pc_intraFreq-CE-NeedForGaps THEN the SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 4 | <- | RRCConnectionReconfiguration | - | - |
| 6a2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 4 | -> | RRCConnectionReconfigurationComplete | - | - |
| 7 | The SS changes Cell 1, Cell 4 and Cell 11 parameters according to the row "T2" in table 8.2.4.1.3.2-1. | - | - | - | - |
| 8 | The UE transmits a MeasurementReport message to report event A3 on Cell 4 with the measured RSRP, RSRQ value for Cell 1. | --> | MeasurementReport | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 using dedicated preamble to confirm the successful completion of the intra frequency handover? Check2: Does the UE transmit an RRCConnectionReconfigurationComplete using the security key derived from the nextHopChainingCount? | --> | RRCConnectionReconfigurationComplete | 1,2 | P |
| - | EXCEPTION: Steps 10a1 to 10a2 describe behaviour that depends on the UE capability; the “lower case letter” identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 10a1 | IF pc_intraFreq-CE-NeedForGaps THEN the SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 1 | <- | RRCConnectionReconfiguration | - | - |
| 10a2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 1 | -> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS changes Cell 1 and Cell 11 parameters and switches Cell 4 off according to the row "T3" in table 8.2.4.1.3.2-1. | - | - | - | - |
| 12 | The UE transmits a MeasurementReport message to report event A3 on Cell 1 with the measured RSRP,RSRQ value for Cell 11. | --> | MeasurementReport | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 11. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 11 using dedicated preamble to confirm the successful completion of the intra frequency handover? Check2: Does the UE transmit an RRCConnectionReconfigurationComplete using the security key derived from the nextHopChainingCount? | --> | RRCConnectionReconfigurationComplete | 1,2 | P |
| 14A | The UE transmit a TRACKING AREA UPDATE REQUEST message on Cell 11. | - | - | - | - |
| 14B | SS responds with a TRACKING AREA UPDATE ACCEPT message. | - | - | - | - |
| 14C | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| 15 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 11? | - | - | 1 | - |

8.2.4.1.3.3	Specific message contents

Table 8.2.4.1.3.3-0: Conditions for specific message contents
in Table 8.2.4.1.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.4.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.1.3.3-2: MeasConfig (Table 8.2.4.1.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-NeedForGaps to TRUE. |

Table 8.2.4.1.3.3-3: MeasurementReport (step 4, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 4 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  | CEmodeA CEmodeB |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 4 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-5: MobilityControlInfo (Table 8.2.4.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 4 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-6: MeasurementReport (step 8, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-7: RRCConnectionReconfiguration (step 9, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  | CEmodeA CEmodeB |
| securityConfigHO | SecurityConfigHO |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 1 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-8: MobilityControlInfo (Table 8.2.4.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  | - |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-9: SecurityConfigHO (step 9, Table 8.2.4.1.3.3-7)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| intraLTE SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-10: MeasurementReport (step 12, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measuredResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 11 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-11: RRCConnectionReconfiguration (step 13, Table 8.2.4.1.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  | CEmodeA CEmodeB |
| securityConfigHO | SecurityConfigHO |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  | CEmodeA CEmodeB |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 11 |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-12: MobilityControlInfo (Table 8.2.4.1.3.3-11)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 11 |  |  |
| carrierFreq | Not present |  |  |
| rach-ConfigDedicated SEQUENCE { |  |  |  |
| ra-PreambleIndex | 63 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 8.2.4.1.3.3-13: SecurityConfigHO (Table 8.2.4.1.3.3-11)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| intraLTE SEQUENCE { |  |  |  |
| nextHopChainingCount | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-14: RRCConnectionReconfiguration (step 6a1, step 10a1 Table 8.2.4.1.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.1.3.3-14: MeasConfig (Table 8.2.4.1.3.3-14)

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | Not present |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | Not present |  |  |
| measGapConfig | MeasGapConfig-CE |  |  |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-15: RadioResourceConfigDedicated-HO(Table 8.2.4.1.3.3-4, Table 8.2.4.1.3.3-7 , Table 8.2.4.1.3.3-11)

| Derivation Path: 36.508 Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-HO ::= SEQUENCE { |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated-DEFAULT |  |  |
| } |  |  |  |

Table 8.2.4.1.3.3-16: PhysicalConfigDedicated-DEFAULT(Table 8.2.4.1.3.3-15)

| Derivation Path: 36.508 Table 4.6.3-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| epdcch-Config-r11 |  | EPDCCH-Config-r11-DEFAULT with condition (CEmodeA OR CEmodeB) |  |  |
| } |  |  |  |  |