3GPP TSG-RAN5 Meeting #90-e	R5-211920

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5270 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to A-MPR test requirement of NS_22 in TC 6.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The values ​​of A-MPR test requirement of NS_22 are incorrect. The tolerance of NS_22 is +3/-4 dB for test requirement, however, the values of test requirement in Table 6.2.4.5-22 are calculated as +2.7/-2.7 dB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the values of A-MPR test requirement of NS_22 in Table 6.2.4.5-22 Correct the value of TT in 6.2.4 in Table 3.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR test cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is outcome of 1 revision of R5-211088. r1: Correct TT in 6.2.4 in Table 3.2-1 |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

6.2.4	Additional Maximum Power Reduction (A-MPR)

Editor’s note: The following aspects are either missing or not yet determined:

- TT and test points are TBD.

<<Unchaged sections skipped>>

6.2.4.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4.5-1 to Table 6.2.4.5-29. The allowed A-MPR values specified in Table 6.2.4.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

<<Unchaged tables skipped>>

Table 6.2.4.5-22: UE Power Class Test requirements (network signalled value "NS_22")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 23 | 2.0 | 26 | 19 |
| 2 | 1 | 0 | 0 | 22 | 2.0 | 26 | 18 |
| 3 | 0 | 0 | 0 | 23 | 2.0 | 26 | 19 |
| 4 | 1 | 0 | 0 | 22 | 2.0 | 26 | 18 |
| 5 | 1 | 0 | 0 | 22 | 2.0 | 26 | 18 |
| 6 | 1 | 4 | 0 | 18 | 4.0 | 26 | 13 |
| 7 | 1 | 3 | 0 | 19 | 3.5 | 26 | 14.5 |
| 8 | 0 | 3 | 0 | 20 | 2.5 | 26 | 16 |
| 9 | 0 | 3 | 0 | 20 | 2.5 | 26 | 16 |
| 10 | 1 | 3 | 0 | 19 | 3.5 | 26 | 14.5 |
| 11 | 1 | 3 | 0 | 19 | 3.5 | 26 | 14.5 |
| 12 | 1 | 5 | 0 | 17 | 5.0 | 26 | 11 |
| 13 | 1 | 4 | 0 | 18 | 4.0 | 26 | 13 |
| 14 | 0 | 3 | 0 | 20 | 2.5 | 26 | 16 |
| 15 | 0 | 3 | 0 | 20 | 2.5 | 26 | 16 |
| 16 | 1 | 3 | 0 | 19 | 3.5 | 26 | 14.5 |
| 17 | 1 | 3 | 0 | 19 | 3.5 | 26 | 14.5 |
| 18 | 1 | 5 | 0 | 17 | 5.0 | 26 | 11 |
| 19 | 1 | 4 | 0 | 18 | 4.0 | 26 | 13 |
| 20 | 0 | 3 | 0 | 20 | 2.5 | 26 | 16 |
| 21 | 0 | 3 | 0 | 20 | 2.5 | 26 | 16 |
| 22 | 1 | 3 | 0 | 19 | 3.5 | 26 | 14.5 |
| 23 | 1 | 3 | 0 | 19 | 3.5 | 26 | 14.5 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Test |  | Minimum Requirement in TS 36.101 |  | Test Tolerance (TT) |  | Test Requirement in TS 36.521-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.2.2 UE Maximum Output Power |  | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-3 dB 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.3 dB |  | Formula: Upper limit + TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-4.0 dB Power class 3: 23dBm ±3.3 dB |  |
| 6.2.2_1 UE Maximum Output Power for HPUE |  | f ≤ 3.0GHz Power class 1: 31dBm +2/-3dB Power class 2: 26dBm ±2dB 3.0GHz < f ≤ 4.2GHz Power class 2: 26 dBm +2/-3 dB |  | 0.7dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit - TT Power class 1: 31dBm +2.7/-3.7dB Power class 2: 26dBm ±2.7dB Power class 2: 26 dBm +3.0/-4.0 dB |  |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.2.2 |  | Same as 6.2.2 |  | Same as 6.2.2 |  |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) |  | Same as 6.2.2 |  | Same as 6.2.2 |  | Same as 6.2.2 |  |
| 6.2.2B UE Maximum Output Power for UL-MIMO |  | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2/-3 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-4 dB |  | Same as 6.2.2 |  | Formula: Upper limit +TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2.7/-3.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-5.0 dB Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.2E UE Maximum Output Power UE category 0 |  | Same as 6.2.2 |  | Same as 6.2.2 |  | Same as 6.2.2 |  |
| 6.2.2EA UE Maximum Output Power UE category M1 |  | Power class 3: 23dBm ±2 dB Power class 5: 20dBm ±2 dB Power class 6: 14dBm ±2.5 dB |  | 0.7 dB 0.7 dB 0.7 dB |  | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm ±2.7 dB Power class 5: 20dBm ±2.7 dB Power class 6: 14dBm ±3.2 dB |  |
| 6.2.2EB UE Maximum Output Power UE category 1bis |  | Same as 6.2.2 |  | Same as 6.2.2 |  | Same as 6.2.2 |  |
| 6.2.2EC UE Maximum Output Power for UE category M2 |  | Same as 6.2.2EA |  | Same as 6.2.2EA |  | Same as 6.2.2EA |  |
| 6.2.2F UE Maximum Output Power for UE category NB1 and NB2 |  | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] Power class 5: 20dBm ±2 dB |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB |  | Formula: Upper limit + TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2.7 dB Power class 4: [FFS] Power class 5: 20dBm ±2.7 dB |  |
| 6.2.2FA UE Maximum Output Power for category NB1 and NB2/Power Class 6 |  | f ≤ 3.0GHz Power class 6: 14dBm ±2.5dB |  | 0.7dB |  | Formula: Upper limit + TT, Lower limit - TT Power class 6: 14dBm ±3.2dB |  |
| 6.2.2G.1 UE maximum output power for V2X Commincation / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB |  | 1.3 dB |  | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm ±3.3 dB |  |
| 6.2.2G.2 UE maximum output power for V2X Commincation / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | 4.2GHz < f ≤ 6GHz Power class 3: 23dBm +2/-3 dB |  | 1.3 dB |  | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm +3.3/-4.3 dB |  |
| 6.2.2G.3 UE maximum output power for V2X Commincation / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB |  | 1.3 dB |  | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm ±3.3 dB |  |
| 6.2.2G.4.1 UE maximum output power for V2X Commincation / Power class 2 / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz Power class 2: 26dBm ±2 dB |  | 1.3 dB |  | Formula: Upper limit + TT, Lower limit - TT Power class 2: 26dBm ±3.3 dB |  |
| 6.2.2_s UE Maximum Output Power for subslot/slot TTI |  | Same as 6.2.2 |  | Same as 6.2.2 |  | Same as 6.2.2 |  |
| 6.2.3 Maximum Power Reduction |  | Power class 3 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: QPSK: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 4.7dB QPSK: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 6.0dB |  |
| 6.2.3_1 Maximum Power Reduction for HPUE |  | Power class 1 and 2 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB Power class 2: 3.0GHz < f ≤ 4.2GHz QPSK: MPR ≤ 1 dB 16QAM: MPR ≤ 1 dB 16QAM: MPR ≤ 2 dB |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 1: QPSK: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 5.7dB Power class 2: QPSK: 26dBm +2.7 / - 3.7dB 16QAM: 26dBm +2.7 / - 3.7dB 16QAM: 26dBm +2.7 / - 4.7dB QPSK: 26dBm +3.0 / -5.0 dB 16QAM: 26dBm +3.0 / -5.0 dB 16QAM: 26dBm +3.0 / -6.0 dB |  |
| 6.2.3_2 UE Maximum Output Power for Multi-Cluster PUSCH |  | Same as 6.2.2 |  | Same as 6.2.2 |  | Same as 6.2.2 |  |
| 6.2.3_3 Maximum Power Reduction (MPR) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz 64QAM: MPR ≤ 2dB 64QAM: MPR ≤ 3dB 3.0GHz < f ≤ 4.2GHz 64QAM: MPR ≤ 2dB 64QAM: MPR ≤ 3dB |  | 0.7 dB 0.7 dB 1.0 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: 64QAM: 23dBm +2.7 / - 4.7dB 64QAM: 23dBm +2.7 / - 6.2dB 64QAM: 23dBm +3.0 / - 5.0dB 64QAM: 23dBm +3.0 / - 6.5dB |  |
| 6.2.3_4 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 64QAM |  | Same as 6.2.3_3 |  | Same as 6.2.3_3 |  | Same as 6.2.3_3 |  |
| 6.2.3_5 Maximum Power Reduction (MPR) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz 256QAM: MPR ≤ 5dB 3.0GHz < f ≤ 4.2GHz 256QAM: MPR ≤ 5dB |  | 0.7 dB 1.0 dB |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: 256QAM: 23dBm +2.7 / - 7.7dB 256QAM: 23dBm +3.0 / - 8.0dB |  |
| 6.2.3_6 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 256QAM |  | Power class 3 f ≤ 3.0GHz 256 QAM: MPR = 5.5dB 256 QAM: MPR = 6 dB 256 QAM: MPR = 7 dB 256 QAM: MPR = 7.5dB 3.0GHz < f ≤ 4.2GHz 256 QAM: MPR = 5.5dB 256 QAM: MPR = 6 dB 256 QAM: MPR = 7 dB 256 QAM: MPR = 7.5dB |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit: +T(PCMAX_H_C) + TT, Lower limit: -T(PCMAX_L_C) – MPR – TT PCMAX_H_C = PPowerClass PCMAX_L_C = PPowerClass-MPR Power class 3: 256QAM: 23dBm +2.7 / - 11.2dB 256QAM: 23dBm +2.7 / - 11.7dB 256QAM: 23dBm +2.7 / - 12.7dB 256QAM: 23dBm +2.7 / - 13.2dB 256QAM: 23dBm +2.7 / - 11.5dB 256QAM: 23dBm +2.7 / - 12.0dB 256QAM: 23dBm +2.7 / - 13.0dB 256QAM: 23dBm +2.7 / - 13.5dB |  |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) |  | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB 3.0GHz < f ≤ 4.2GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 2.7dB 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 6.2dB 23dBm +2.7 / - 14.2dB 23dBm +3.0 / - 4.0dB (+2/-3 limit for B42) 23dBm +3.0 / - 4.0dB 23dBm +3.0 / - 5.0dB 23dBm +3.0 / - 6.5dB 23dBm +3.0 / - 14.5dB |  |
| 6.2.3A.1_1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz MPR = 2dB MPR = 3dB 3.0GHz < f ≤ 4.2GHz MPR = 2dB MPR = 3dB |  | 0.7 dB 0.7 dB 1.0 dB 1.0 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 6.2dB 23dBm +3.0 / - 5.0dB 23dBm +3.0 / - 6.5dB |  |
| 6.2.3A.1_2 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz MPR = 5dB 3.0GHz < f ≤ 4.2GHz MPR = 5dB |  | 0.7 dB 1.0 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 9.7dB 23dBm +3.0 / - 10.0dB |  |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) |  | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB 4.2GHz < f ≤ 6GHz MPR = 0dB MPR = 1dB MPR = 2dB |  | 0.7 dB 0.7 dB 0.7 dB 1.5 dB 1.5 dB 1.5 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +3.5 / - 4.5 dB 23dBm +3.5 / - 5.5 dB 23dBm +3.5 / - 6.5 dB |  |
| 6.2.3A.2_1 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz MPR = 2dB MPR = 3dB |  | 0.7 dB 0.7 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 5.7 dB 23dBm +2.7 / - 6.7 dB |  |
| 6.2.3A.2_2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz MPR = 5dB 3.0GHz < f ≤ 4.2GHz MPR = 5dB |  | 0.7 dB 1.0 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 7.7 dB 23dBm +2.7 / - 9.4 dB 23dBm +3.0 / - 8.0 dB 23dBm +3.0 / - 9.7 dB |  |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) |  | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB |  | 0.7 dB 0.7 dB 0.7 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·TC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |  |
| 6.2.3A.3_1 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB |  | 0.7 dB 0.7 dB 0.7 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·TC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |  |
| 6.2.3A.3_2 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB |  | 0.7 dB 0.7 dB 0.7 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·DTC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |  |
| 6.2.3A.4 Maximum Power Reduction (MPR) for CA (3UL CA) |  | Power class 3 f ≤ 3.0GHz for intra-band MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 3.5dB 3.0GHz < f ≤ 4.2GHz for intra-band MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 3.5dB f ≤ 3.0GHz for inter-band MPR = 0dB MPR = 1dB MPR = 1dB MPR = 2dB MPR = 2dB 3.0GHz < f ≤ 4.2GHz for intra-band MPR = 0dB MPR = 1dB MPR = 1dB MPR = 2dB MPR = 2dB |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1 dB 1 dB 1 dB 1 dB 1 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1 dB 1 dB 1 dB 1 dB 1 dB |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 2.7dB 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 5.2dB 23dBm +2.7 / - 6.2dB 23dBm +3 / -3 dB 23dBm +3 / -4 dB 23dBm +3 / -5 dB 23dBm +3 / -5.5 dB 23dBm +3 / -6.5 dB Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 5.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 5.7 dB 23dBm +3 / - 4 dB 23dBm +3 / - 4 dB 23dBm +3 / - 6 dB 23dBm +3 / - 4 dB 23dBm +3 / - 6 dB |  |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO |  | Same as 6.2.3 | Same as 6.2.3 |  |  | Same as 6.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.3E Maximum Power Reduction for UE category 0 |  | Same as 6.2.3E | Same as 6.2.3E |  |  | Same as 6.2.3E |  |
| 6.2.3EA Maximum Power Reduction for UE category M1 |  | Power class 3 MPR ≤ 1dB MPR ≤ 2dB Power class 5 MPR ≤ 1dB MPR ≤ 2dB Power class 6 MPR = 0dB MPR = 1dB MPR = 2dB | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – MPR – TT 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 20dBm +2.7 / - 3.7dB 20dBm +2.7 / - 4.7dB 14dBm +3.2 / - 3.2dB 14dBm +3.2 / - 4.2dB 14dBm +3.2 / - 6.2dB |  |
| 6.2.3EB Maximum Power Reduction for UE category 1bis |  | Same as 6.2.3 | Same as 6.2.3 |  |  | Same as 6.2.3 |  |
| 6.2.3F Maximum Power Reduction (MPR) for UE category NB1 and NB2 |  | Power class 3: f ≤ 3.0GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB Power class 5: f ≤ 3.0GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: BPSK: 23dBm +2.7 / - 2.7dB QPSK: 23dBm +2.7 / - 2.7dB QPSK: 23dBm +2.7 / - 3.2dB QPSK: 23dBm +2.7 / - 3.7dB QPSK: 23dBm +2.7 / - 4.7dB Power class 5: BPSK: 20dBm +2.7 / - 3.2dB QPSK: 20dBm +2.7 / - 3.2dB QPSK: 20dBm +2.7 / - 4.7dB QPSK: 20dBm +2.7 / - 5.2dB QPSK: 20dBm +2.7 / - 6.7dB |  |
| 6.2.3G.1.1 Maximum Power Reduction (MPR) for V2X Commincation / Non-concurrent with E-UTRA uplink transmissions |  | Power class 3: 4.2GHz < f ≤ 6GHz QPSK: MPR ≤ 1.5dB 16QAM: MPR ≤ 2dB | 1.3 dB 1.3 dB |  |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: QPSK: 23dBm +3.3 / - 4.8dB 16QAM: 23dBm +3.3 / - 5.3dB |  |
| 6.2.3G.1.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Power class 3: 5 RBs/10MHz: MPR ≤4.5dB 25 RBs/10MHz: MPR ≤3.0dB 40 RBs/10MHz: MPR ≤2.0dB 10 RBs/20MHz: MPR ≤4.5dB 50 RBs/20MHz: MPR ≤3.0dB 80 RBs/20MHz: MPR ≤2.0dB | 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm + 3.3 / - 9.8dB 23dBm + 3.3 / - 6.8dB 23dBm + 3.3 / - 5.3dB 23dBm + 3.3 / - 9.8dB 23dBm + 3.3 / - 6.8dB 23dBm + 3.3 / - 5.3dB |  |
| 6.2.3G.1.3 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | Power class 3 MPR ≤ 1.5dB MPR ≤ 2dB | 1.3 dB 1.3 dB |  |  | Formula: Upper limit + THIGH (PCMAX_H(p, q)) + TT, Lower limit – TLOW (PCMAX_L(p, q)) – MPR – ΔTIB,c – TT Power class 3: 23dBm +3.3 / -7.8dB 23dBm +3.3 / -8.3dB |  |
| 6.2.3G.1.4 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Intra-band contiguous multi-carrier operation |  | Power class 3 MPR ≤ 3dB | 1.3 dB |  |  | Formula: Upper limit + THIGH (PCMAX_H(p, q)) + TT, Lower limit – TLOW (PCMAX_L(p, q)) – MPR – ΔTIB,c – TT Power class 3: 23dBm +3.3 / -6.8dB |  |
| 6.2.3G.2.1 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Power class 2: QPSK : MPR ≤ 2dB 16QAM : MPR ≤ 2.5dB | 1.3 dB 1.3 dB |  |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 2: QPSK : 26dBm +3.3 / -5.3dB 16QAM : 26dBm +3.3 / -5.8dB |  |
| 6.2.3_s Maximum Power Reduction (MPR) for subslot/slot TTI |  | Same as 6.2.3 | Same as 6.2.3 |  |  | Same as 6.2.3 |  |
| 6.2.3G.2.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Power class 2 4.2GHz < f ≤ 6GHz For 10MHz BW: NRB_alloc 10: MPR = 4.5dB NRB_alloc 12: MPR = 4dB NRB_alloc 42: MPR = 2.5dB For 20MHz BW: NRB_alloc 10: MPR = 9dB NRB_alloc 22: MPR = 5.5dB NRB_alloc 82: MPR = 3dB | 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 2: For 10MHz BW: NRB_alloc 10: 26dBm +3.3 / -7.8dB NRB_alloc 12: 26dBm +3.3 / -7.3dB NRB_alloc 42: 26dBm +3.3 / -5.8dB For 20MHz BW: NRB_alloc 10: 26dBm +3.3 / -15.3dB NRB_alloc 22: 26dBm +3.3 / -9.3dB NRB_alloc 82: 26dBm +3.3 / -6.3dB |  |
| 6.2.4 UE Maximum Output Power with additional requirements |  | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 3: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB For network signalled value NS_03 , NS_04 (5MHz only), NS_05, to NS_06: A-MPR ≤ 1dB For network signalled value NS-04; Depending on the RB_start and RB allocation (10MHz, 15MHz and 20MHz): For 10MHz Region A with RB_start=0 – 12: A-MPR ≤ 3dB. Region B with RB_start=13 – 36 : A-MPR ≤ 2dB. Region C with RB_start=37 – 49 : A-MPR ≤ 3dB. For 15MHz Region A with RB_start=0 – 18: A-MPR ≤ 3dB. Region B with RB_start=19 – 55 : A-MPR ≤ 2dB. Region C with RB_start=56 – 74 : A-MPR ≤ 3dB. For 20MHz Region A with RB_start=0 – 24: A-MPR ≤ 3dB. Region B with RB_start=25 – 74 : A-MPR ≤ 2dB. Region C with RB_start=75 – 99 : A-MPR ≤ 3dB. For network signalled value NS-11; Depending on the RB_start and RB allocation (15MHz and 20MHz): For 15MHz Fc < 2012.5 Region A with RB_start=0 – 4: A-MPR ≤ 15dB. Region B with RB_start=5 – 21: A-MPR ≤ 7dB (LCRB=7-50) or ≤ 10dB (LCRB=0-6 & ≥50). Region C with RB_start=22 – 56: A-MPR ≤ 0dB (LCRB=≤25) or ≤ 6dB (LCRB>25). Region D with RB_start=57 – 74: A-MPR ≤ 15dB. For 15MHz Fc = 2012.5 Region A with RB_start=0 – 12: A-MPR ≤ 10dB. Region B with RB_start=13 – 39: A-MPR ≤ 6dB (LCRB=≥ 30) or ≤ 0dB (LCRB=< 30). Region C with RB_start=40 – 65: A-MPR ≤ 2dB. Region D with RB_start=66 – 74: A-MPR ≤ 6.5dB. For 20MHz Region A with RB_start=0 – 12: A-MPR ≤ 15dB. Region B with RB_start=13 – 29: A-MPR ≤ 7dB (LCRB=10-60) or ≤ 10dB (LCRB=1-9 & > 60). Region C with RB_start=30 – 68: A-MPR ≤ 0dB (LCRB=1-24) or ≤ 7dB (LCRB= ≥ 25). Region D with RB_start=69 – 99: A-MPR ≤ 15dB. For network signalled value NS-20; Depending on the RB_start and RB allocation (5MHZ, 10MHz, 15MHz and 20MHz): For 5MHz Region A with RB_start=≤24: A-MPR ≤ 17dB. Region B with RB_start=0 – 3: A-MPR ≤ 1dB (LCRB=15-19) or ≤ 4dB (LCRB= ≥ 20). Region C with RB_start=4 – 6: A-MPR ≤ 2dB. Region D with RB_start=≤24: A-MPR ≤ 0dB. For 10MHz Fc = 2005 Region A with RB_start=0 – 25: A-MPR ≤ 16dB. Region B with RB_start=26 – 34: A-MPR ≤ 2dB (LCRB=8-15) or ≤ 5dB (LCRB= > 15). Region C with RB_start=35 – 49: A-MPR ≤ 6dB. For 10MHz Fc = 2015 Region A with RB_start=0 – 5: A-MPR ≤ 4dB. Region B with RB_start=6 – 10: A-MPR ≤ 2dB. For 15MHz Region A with RB_start=0 – 14: A-MPR ≤ 11dB (LCRB=1-9 & 40-5) or ≤ 6dB (LCRB= 10-39). Region B with RB_start=15 – 24: A-MPR ≤ 1dB (LCRB=24-29) or ≤ 7dB (LCRB=≥30). Region C with RB_start=25 – 39: A-MPR ≤ 5dB. Region D with RB_start=61 – 74: A-MPR ≤ 6dB. For 20MHz Region A with RB_start=0 – 21: A-MPR ≤ 17dB. Region B with RB_start=22 – 31: A-MPR ≤ 12dB (LCRB=1-9 & 31-75) or ≤ 6dB (LCRB=10-30). Region C with RB_start=32 – 38: A-MPR ≤ 9dB. Region D with RB_start=39 – 49: A-MPR ≤ 7dB. Region E with RB_start=50 – 69: A-MPR ≤ 5dB. Region F with RB_start=70 – 99: A-MPR ≤ 16dB. For network signalled value NS-27; Depending on the RB_start and RB allocation (15MHz and 20MHz): For 15MHz With LCRB ≤15, LRB_start=0 – 6 or RB_end=68-74: A-MPR ≤ 4dB. With LCRB ≥ 60, LRB_start≥0: A-MPR ≤ 2dB. For 20MHz With LCRB ≤20, LRB_start=0 – 12 or RB_end=87-99: A-MPR ≤ 4dB. With LCRB ≤3, LRB_start=13 – 15 or RB_end=84-86: A-MPR ≤ 1dB. With LCRB ≥ 60, LRB_start≥0: A-MPR ≤ 3dB. | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB |  |  | Formula: Upper limit + TT, A: Lower limit – TT, B: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – MPR – TT, C: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – TT, D: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT Power class 3: Test Requirement Configuration ID versus Formula Above Network signalled value NS_03: [A]: 2, 5, 10, 15, 20, 25 [B]: 1, 3, 7 [C]: 9, 14, 19, 24 [D]: 4, 6, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 Network signalled value NS_04 (5, 10, 15, 20MHz): [A] 3 [B] 10, 11, 19, 20, 28, 29 [C] 2, 6, 7, 13, 14, 15, 16, 23, 24, 25, 32 [D] 1, 4, 5, 8, 9, 12, 17, 18, 21, 22, 26, 27, 30, 31 Network signalled value NS_05: [A] 1, 3, 4, 8, 9, 14, 15, [B] 2, 5, 10, 11, 16, 17 [C] None [D] 6, 7, 12, 13, 18, 19 Network signalled value NS_06: [A]: 2, 5, 8, 11, 14, 17 [B]: 1, 3, 4, 6, 7, 9, 10, 12, 13, 15, 16, 18 [C]: None [D]: None Network signalled value NS_07: [A]:3, 8, 12 [B]: 7, 9 [C]: 1, 2, 5, 13, 15 [D]: 4, 6, 10, 11, 14, 16 Network signalled value NS_08: [A]:1, 2, 4, 5, 12 [B]: 3, 6, 11, 13 [C]: None [D]: 7, 8, 9, 10, 14, 15, 16, 17 Network signalled value NS_11: [A]: 5c, 6b, 10c, 11c, 25 [B]: 8b, 12c, 13c, 14c, 17a, 17b, 21a, 21b, 28 [C]: 5a, 5b, 6a, 6c, 10a, 10b, 11, 11b, 15a, 15b, 16a, 16b, 24 [D]: 1a, 1b, 2a, 2b, 3a, 3b, 4a, 4b ,, 7a,7b,7c, 8a, 8c, 9a, 9b,9c, 12a, 12b, 13a, 13b, 14a, 14b, 18a, 18b, 19a, 19b, 20a, 20b, 22a, 22b, 23a, 23b, 26, 27, 29, 30 Network signalled value NS_12: [A]: 3, 8, 13 [B]: None [C]: 1, 4, 6, 11 [D]: 2, 5, 7, 9, 10, 12, 14, 15 Network signalled value NS_13: [A]: None [B]: 3, 4 [C]: 1 [D]: 2, 5 Network signalled value NS_14: [A]: None [B]: 2, 4, 7, 9 [C]: 1, 6 [D]: 3, 5, 8, 10 Network signalled value NS_15: [A]: 1, 7, 14 [B]: 3, 6, 10, 15, 19, 21, 22, 25 [C]: 12, 13 [D]: 2, 4, 5, 8, 9, 11, 16, 17, 18, 20, 23, 24, 26 Network signalled value NS_20: [A]: 1b, 1c, 1d, 6b, 7b [B]: 2d, 3d, 4d, 5d, 9b [C]: 1a, 6a, 7a, 11, 18, 19 [D]: 2a, 2b, 2c, 3a, 3b, 3c, 4a, 4b, 4c, 5a, 5b, 5c, 8a, 8b, 9a, 10a, 11b, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26 Network signalled value NS_27: TBD |  |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE |  | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 1: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB Power class 1: For network signalled value NS_06: A-MPR 0dB | 0.7dB |  |  | Formula: Upper limit + TT, (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT |  |
| 6.2.4_2 Additional Maximum Power Reduction (A-MPR) for UL 64QAM |  | Same as 6.2.4 | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.4_3 Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping |  | Same as 6.2.4 | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.4_4 Additional Maximum Power Reduction (A-MPR) for UL 256QAM |  | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 3: 256QAM: MPR ≤ 5dB A-MPR values as given by tables in section 6.2.4.3. | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB |  |  | Formula: Upper limit + TT, A: (UE Maximum Output Power from 6.2.2) - T(PCMAX) - MPR - TT, B: (UE Maximum Output Power from 6.2.2) - T(PCMAX) - A-MPR - MPR - TT Power class 3: Test Requirement Configuration ID versus Formula Above Network signalled value NS_03 [A]: 1, 3 [B]: 2, 4, 5, 6, 7, 8, 9, 10, 11 Network signalled value NS_04 [A]: 1, (2, 3, 4, 5) if not low range [B]: (2, 3, 4, 5) for low range Network signalled value NS_05 [A]: 1, 5, 6 [B]: 2, 3, 4, 7 Network signalled value NS_06 [A]: 1, 2, 3 [B]: None Network signalled value NS_07 [A]: 2 [B]: 1, 3, 4 Network signalled value NS_08 [A]: 1 [B]: 2, 3 Network signalled value NS_09 [A]: 1 [B]: 2, 3 Network signalled value NS_11 [A]: 3b, 5c, 6c, 8a with high range RB allocation [B]: 1a, 1b, 2a, 2b, 3a, 3c, 4a, 4b, 4c, 5a, 5b, 6a, 6b, 7a, 7b, 8a with low range RB allocation, 8b, 9a, 9b, 10a, 10b, 11, 12, 13 Network signalled value NS_12 [A]: None [B]: 1, 2, 3, 4 Network signalled value NS_13 [A]: None [B]: 1 Network signalled value NS_14 [A]: None [B]: 1, 2 Network signalled value NS_15 [A]: None [B]: 1, 2, 3, 4, 5 Network signalled value NS_16 [A]: 1, 2 for FC = 810.0 MHz, 3 for FC = 814.5 MHz [B]: 2 for FC < 810.0 MHz, 3 for FC < 814.5 MHz, 4 Network signalled value NS_17 [A]: 1, 2 [B]: None Network signalled value NS_18 [A]: None [B]: 1, 2, 3, 4 Network signalled value NS_19 [A]: 1, (2, 3, 4) with high range RB allocation [B]: (2, 3, 4) with low range RB allocation Network signalled value NS_20 [A]: 1b with high range RB allocation, 1c, 1d, 2c, 2d, 3a with high range RB allocation, 3b, 5a with high range RB allocation [B]: 1a, 1b with low range RB allocation, 2a, 2b, 3a with low range RB allocation, 4a, 4b, 5a with low range RB allocation, 6a, 7a, 8a, 9a, 10a, 11a Network signalled value NS_21 [A]: None [B]: 1 Network signalled value NS_22 [A]: 1, 2 [B]: 3, 4, 5, 6, 7, 8 Network signalled value NS_23 [A]: 1, 2 [B]: None Network signalled value NS_24 [A]: 1, 3, 5 [B]: 2, 4, 6 Network signalled value NS_25 [A]: 6 [B]: 1, 2, 3, 4, 5 Network signalled value NS_27 [A]: 1, 2 [B]: 3, 4 Network signalled value NS_35 [A]: 1, 2, 3, 4 [B]: None Network signalled value NS_36 [A]: 2, 4, 6 [B]: 1, 3, 5 |  |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) |  | A-MPR values as detailed in core requirement | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |  |
| 6.2.4A.1_1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.2.4A.1 | Same as 6.2.4A.1 |  |  | Same as 6.2.4A.1 |  |
| 6.2.4A.1_2 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.2.4A.1 | Same as 6.2.4A.1 |  |  | Same as 6.2.4A.1 |  |
| 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) |  | Same as 6.2.4 | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4A.2_1 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.2.4 | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4A.2_2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.2.4 | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) |  | A-MPR values as detailed in core requirement | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |  |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO |  | Same as 6.2.4 | Same as 6.2.4 |  |  | Same as 6.2.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.4E Additional Maximum Power Reduction (A-MPR) for UE category 0 |  | Same as 6.2.4 | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.4EA Additional Maximum Power Reduction (A-MPR) for UE category M1 |  | A-MPR values as detailed in core requirement | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |  |
| 6.2.4EB Additional Maximum Power Reduction (A-MPR) for UE category 1bis |  | Same as 6.2.4 | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.4EC Additional Maximum Power Reduction (A-MPR) for UE category M2 |  | Same as 6.2.4EA | Same as 6.2.4EA |  |  | Same as 6.2.4EA |  |
| 6.2.4G.1 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | A-MPR values as detailed in core requirement | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4G.2 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | A-MPR values as detailed in core requirement | 1.3dB |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4_s Additional Maximum Power Reduction (A-MPR) for subslot/slot TTI |  | Same as 6.2.4 | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.5 Configured UE transmitted Output Power |  | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |  |
| 6.2.5_s Configured UE Transmitted Output Power for subslot/slot TTI |  | Same as 6.2.5 | Same as 6.2.5 |  |  | Same as 6.2.5 |  |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE |  | f ≤ 3.0GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 23 ≤ PCMAX < 33 ± 2.7 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 |  |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) |  | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |  |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) |  | f ≤ 3.0GHz 16 ≤ PCMAX < 20 ± 5.0 11≤ PCMAX < 16 ± 6.0 -40 ≤ PCMAX < 11 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 16 ≤ PCMAX < 20 ± 5.7 11 ≤ PCMAX < 16 ± 6.7 -40 ≤ PCMAX < 11 ± 7.7 |  |
| 6.2.5A.4 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) |  | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 0.7 dB 0.7 dB 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 |  |
| 6.2.5B Configured UE transmitted output power for UL-MIMO |  | f ≤ 3.0GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] 3.0GHz < f ≤ 4.2GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] | Same as 6.2.5 |  |  | Formula: Upper limit + TT, Lower limit – TT [16] ≤ PCMAX < [20] ± [5.7] [11] ≤ PCMAX < [16] ± [6.7] [-40] ≤ PCMAX < [11] ± [7.7] [16] ≤ PCMAX < [20] ± [6.0] [11] ≤ PCMAX < [16] ± [7.0] [-40] ≤ PCMAX < [11] ± [8.0] Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.5E Configurated Transmitted Power for UE category 0 |  | Same as 6.2.5 | Same as 6.2.5 |  |  | Same as 6.2.5 |  |
| 6.2.5EA Configured UE transmitted Power for UE category M1 |  | Same as 6.2.5, with addition of power class 6: f ≤ 3.0GHz 5 ≤ PCMAX < 8 ± 6.0 -40 ≤ PCMAX < 5 ± 7.0 3.0GHz < f ≤ 4.2GHz 5 ≤ PCMAX < 8 ± 6.0 -40 ≤ PCMAX < 5 ± 7.0 | Same as 6.2.5, with addition of power class 6: 0.7 dB 0.7 dB 1.0 dB 1.0 dB |  |  | Same as 6.2.5, with addition of power class 6: Formula: Upper limit + TT, Lower limit – TT f ≤ 3.0GHz 5 ≤ PCMAX < 8 ± 6.7 -40 ≤ PCMAX < 5 ± 7.7 3.0GHz < f ≤ 4.2GHz 5 ≤ PCMAX < 8 ± 7.0 -40 ≤ PCMAX < 5 ± 8.0 |  |
| 6.2.5EB Configured UE transmitted Power for UE category 1bis |  | Same as 6.2.5 | Same as 6.2.5 |  |  | Same as 6.2.5 |  |
| 6.2.5F Configured UE transmitted Output Power for UE category NB1 and NB2 |  | Tolerances as described in core requirements | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT |  |
| 6.2.5G.1 Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  | 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 1.3 dB 1.3 dB 1.5 dB |  |  | Formula: Upper limit + TT, Lower limit - TT 13 ≤ PCMAX < 18 ± 6.3 8 ≤ PCMAX < 13 ± 7.3 -40 ≤ PCMAX < 8 ± 8.5 |  |
| 6.2.5G.2 Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | 16 ≤ PCMAX < 20 ± 5.0 11 ≤ PCMAX < 16 ± 6.0 -40 ≤ PCMAX < 11 ± 7.0 | 1.3 dB 1.3 dB 1.5 dB |  |  | Formula: Upper limit + TT, Lower limit - TT 16 ≤ PCMAX < 20 ± 6.3 11 ≤ PCMAX < 16 ± 7.3 -40 ≤ PCMAX < 11 ± 8.5 |  |
| 6.2.5G.3 Configured UE transmitted Output Power for V2X Communication / Intra-band contiguous multi-carrier operation |  | 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 | 1.3 dB 1.3 dB 1.5 dB |  |  | Formula: Upper limit + TT, Lower limit - TT 13 ≤ PCMAX < 18 ± 6.3 8 ≤ PCMAX < 13 ± 7.3 -40 ≤ PCMAX < 8 ± 8.5 |  |
| 6.3.2 Minimum Output Power |  | f ≤ 3.0GHz -40 dBm 3.0GHz < f ≤ 4.2GHz -40 dBm 4.2GHz < f ≤ 6GHz -40 dBm | 1.0 dB 1.3 dB 1.5 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–39 dBm UE min. output power =–38.7 dBm UE min. output power =–38.5 dBm |  |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.2 | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2A.4 Minimum Output Power for CA (3UL CA) |  | Same as 6.3.2 | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2B Minimum Output Power for UL-MIMO |  | Same as 6.3.2 | Same as 6.3.2 |  |  | Same as 6.3.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.2E Minimum Output Power for UE category 0 |  | Same as 6.3.2 | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2EA Minimum Output Power for UE category M1 |  | Same as 6.3.2 | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2EB Minimum Output Power for UE category 1bis |  | Same as 6.3.2 | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2F Minimum Output Power for UE category NB1 and NB2 |  | -40 dBm | 1.0 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–39 dBm |  |
| 6.3.2G.1 Minimum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz -30 dBm | 1.5 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–28.5 dBm |  |
| 6.3.2G.2 Minimum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz -40 dBm 4.2GHz < f ≤ 6GHz -30 dBm | 1.0 dB 1.5 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–39 dBm UE min. output power =–28.5 dBm |  |
| 6.3.2G.3 Minimum output power for V2X Communication / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz -3 0 dBm | 1.5 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–28.5 dBm |  |
| 6.3.3 Transmission ON/OFF Power |  | f ≤ 3.0GHz -50 dBm 3.0GHz < f ≤ 4.2GHz -50 dBm 4.2GHz < f ≤ 6GHz -50 dBm | 1.5 dB 1.8 dB 2.0 dB |  |  | Formula: Minimum Requirement + TT UE OFF Power -48.5 dBm UE OFF Power -48.2 dBm UE OFF Power -48.0 dBm |  |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3A.2 Transmit OFF Power for CA (inter-band DL CA and UL CA) |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3A.4 UE Transmit OFF power for CA (3UL CA) |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3B UE Transmit OFF power for UL-MIMO |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 Uplink power measurement applies to each Tx antenna connector |  |
| 6.3.3D UE Transmit OFF power for ProSe |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3E UE Transmit OFF Power for UE category 0 |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3EA UE Transmit OFF power for UE category M1 |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3EB UE Transmit OFF power for UE category 1bis |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3F Transmit OFF power for category NB1 and NB2 |  | Same as 6.3.3 | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3G.1 UE Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz -50 dBm | 2.0 dB |  |  | Formula: Minimum Requirement + TT UE OFF Power -48 dBm |  |
| 6.3.3G.2 UE Transmit OFF power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz -50 dBm 3.0GHz < f ≤ 4.2GHz -50 dBm 4.2GHz < f ≤ 6GHz -50 dBm | 1.5 dB 1.8 dB 2.0 dB |  |  | Formula: Minimum Requirement + TT UE OFF Power -48.5 dBm UE OFF Power -48.2 dBm UE OFF Power -48.0 dBm |  |
| 6.3.3G.3 UE Transmit OFF power for V2X Communication / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz -50 dBm | 2.0 dB |  |  | Formula: Minimum Requirement + TT UE OFF Power -48 dBm |  |
| 6.3.4.1 General ON/OFF time mask |  | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB | 1.5 dB 1.5 dB 1.8 dB 1.8 dB |  |  | Formula: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |  |
| 6.3.4.1_s General ON/OFF time mask for subslot/slot TTI |  | Same as 6.3.4.1 | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4.2 PRACH and SRS time mask |  | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB | 1.5 dB 1.5 dB 1.8 dB 1.8 dB |  |  | Formula: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |  |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.4.1 for each CC | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) |  | Same as 6.3.4.1 | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.4.1 for each CC | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.4 General ON/OFF time mask for CA (3UL CA) |  | Same as 6.3.4.1 for each CC | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.5 General ON/OFF time mask for CA (34UL CA) |  | Same as 6.3.4.1 for each CC | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO |  | Same as 6.3.4.1 | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 Uplink OFF power measurement applies to each Tx antenna connector Uplink ON power measurement applies to [FFS] |  |
| 6.3.4C.1 General ON/OFF time mask for Dual Connectivity |  | Same as 6.3.4.1 | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4E.1 General ON/OFF time mask for UE category 0 |  | Same as 6.3.4.1 | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4E.2 PRACH and SRS time mask for UE category 0 |  | Same as 6.3.4.2 | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4EA.1 General ON/OFF time mask for UE category M1 |  | Same as 6.3.4.1 | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4EA.2 PRACH and SRS ON/OFF time mask for UE category M1 |  | Same as 6.3.4.2 | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4EB.1 General ON/OFF time mask for UE category 1bis |  | Same as 6.3.4.1 | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4EB.2 PRACH and SRS ON/OFF time mask for UE category 1bis |  | Same as 6.3.4.2 | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4EC.2 PRACH and SRS ON/OFF time mask for UE category M2 |  | Same as 6.3.4.2 | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4F.1 General ON/OFF time mask for category NB1 and NB2 |  | Same as 6.3.4.1 | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4F.2 NPRACH time mask for category NB1 and NB2 |  | Same as 6.3.4.2 | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4G.1 General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB | 2.0dB 2.0dB |  |  | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48 dBm UE ON Power: Test value ± 8.0 dB |  |
| 6.3.4G.2 General ON/OFF time mask for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent 4.2GHz < f ≤ 6.0GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB | 1.5 dB 1.5 dB 1.8 dB 1.8 dB 2.0 dB 2.0 dB |  |  | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB UE OFF Power -48.0 dBm UE ON Power: Test value ± 8.0 dB |  |
| 6.3.4G.3 General ON/OFF time mask for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.3.4G.1 for each CC | Same as 6.3.4G.1 |  |  | Same as 6.3.4G.1 for each CC |  |
| 6.3.4G.4 PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.3.4G.1 | Same as 6.3.4G.1 |  |  | Same as 6.3.4G.1 |  |
| 6.3.5.1 Power Control Absolute power tolerance |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 4.2GHz < f ≤ 6GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB | 1.0 dB 1.0 dB 1.4 dB 1.4 dB 2.0 dB 2.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB Normal conditions ± 11.0 dB Extreme conditions ± 14.0 dB |  |
| 6.3.5.2 Power Control Relative power tolerance |  | TS 36.101 [2] clause 6.3.5.1 All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±2.5 dB 2 ≤ ΔP < 3; ±3.0 dB 3 ≤ ΔP < 4; ±3.5 dB 4 ≤ ΔP ≤ 10; ±4.0 dB 10 ≤ ΔP < 15; ±5.0 dB 15 ≤ ΔP; ±6.0 dB | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±3.2 dB 2 ≤ ΔP < 3; ±3.7 dB 3 ≤ ΔP < 4; ±4.2 dB 4 ≤ ΔP < 10; ±4.7 dB 10 ≤ ΔP < 15; ±5.7 dB 15 ≤ ΔP; ±6.7 dB |  |
| 6.3.5.3 Aggregate power control tolerance |  | Aggregate power control tolerance within 21 ms: PUCCH = ±2.5 dB PUSCH = ±3.5 dB | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit - TT PUCCH = ±3.2 dB PUSCH = ±4.2 dB |  |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB | 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB |  |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE |  | Same as 6.3.5.2 | Same as 6.3.5.2 |  |  | Same as 6.3.5.2 |  |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.5.1 | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.2 Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA) |  | Same as 6.3.5.1 | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.3 Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.5.1 | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.4 Power Control Absolute power tolerance for CA(3UL CA) |  | Same as 6.3.5.1 | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.5 Power Control Absolute power tolerance for CA(SRS carrier based switching) |  | Same as 6.3.5.1 | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.6 Power Control Absolute power tolerance for CA(4UL CA) |  | Same as 6.3.5.1 | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) |  | TS 36.101 [2] clause 6.3.5A.2 refers to Table 6.3.5.2.1-1. All combinations of PUSCH and PUCCH transitions: SCC power increase PCC 0dB (ΔP ≤ 1dB): Normal 0 ±1.0dB Exceptions: 0 ±6.0dB SCC +9dB (4 ≤ ΔP ≤ 10): Normal +9 ±4.0dB Exceptions: +9 ±6.0dB SCC power decrease PCC 0dB (ΔP ≤ 1dB): Normal 0 ±1.0dB Exceptions: 0 ±6.0dB SCC -9dB (4 ≤ ΔP ≤ 10): Normal -9 ±4.0dB Exceptions: -9 ±6.0dB PCC and SCC power increase PCC +9dB (4 ≤ ΔP ≤ 10): Normal +9 ±4.0dB Exceptions: +9 ±6.0dB SCC +9dB (4 ≤ ΔP ≤ 10): Normal +9 ±4.0dB Exceptions: +9 ±6.0dB PCC and SCC power decrease PCC -9dB (4 ≤ ΔP ≤ 10): Normal -9 ±4.0dB Exceptions: -9 ±6.0dB SCC -9dB (4 ≤ ΔP ≤ 10): Normal -9 ±4.0dB Exceptions: -9 ±6.0dB Each CC can have exceptions | -0.7, +0.8dB -0.7, +0.8dB -0.8, +0.7dB -0.8, +0.8dB -0.8, +0.7dB -0.8, +0.7dB -0.8, +0.8dB -0.8, +0.8dB -0.7, +0.8dB -0.7, +0.8dB -0.7, +0.8dB -0.7, +0.8dB -0.8, +0.7dB -0.8, +0.7dB -0.8, +0.7dB -0.8, +0.7dB |  |  | Relative power measurement uncertainty comprises two quantities: 1. Uplink Relative level measurement uncertainty of Test system 2. IQ image Leakage onto weaker CC at -25dBc is allowed for the UE, so is added arithmetically. It can only increase the measured power of the weaker CC. UL relative meas’t uncert ±0.7dB Leakage +0.17dB max for 11dB CC power difference, based on -25dBc. All limits rounded to nearest 0.1dB. SCC power increase PCC: Normal -1.7, +1.8dB Exceptions: -6.7, +6.8dB SCC: Normal +4.2, +13.7dB Exceptions: +2.2, +15.8dB SCC power decrease PCC: Normal -1.8, +1.7dB Exceptions: -6.8, +6.7dB SCC: Normal -13.8, -4.2dB Exceptions: -15.8, -2.2dB PCC and SCC power increase PCC: Normal +4.3, +13.8dB Exceptions: +2.3, +15.8dB SCC: Normal +4.3, +13.8dB Exceptions: +2.3, +15.8dB PCC and SCC power decrease PCC: Normal -13.8, -4.3dB Exceptions: -15.8, -2.3dB SCC: Normal -13.8, -4.3dB Exceptions: -15.8, -2.3dB The analysis is recorded in 3GPP TR 36.904 [17] When setting SCC power for the next step, three factors are relevant: 1. Uplink Relative level measurement uncertainty 2. IQ image Leakage onto weaker CC at -25dBc is allowed for the UE 3) TPC step 1dB ±1dB is allowed for the UE These are added arithmetically, giving an extra margin of (1+0.7+0.17) = 1.87dB (2dB is allowed) to avoid the same power step exception being counted twice. |  |
| 6.3.5A.2.2 Power Control Relative power tolerance for CA (inter-band DL CA and UL CA) |  | Same as 6.3.5.2 on both PCC and SCC separately. | Same as 6.3.5.2 on both PCC and SCC separately. |  |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |
| 6.3.5A.2.3 Power Control Relative power tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.5A.2.1 | Same as 6.3.5A.2.1 |  |  | Same as 6.3.5A.2.1 |  |
| 6.3.5A.2.4.1 Power Control Relative power tolerance for CA (intra-band contiguous 3DL CA and 3UL CA) |  | Same as 6.3.5A.2.1 | Same as 6.3.5A.2.1 |  |  | Same as 6.3.5A.2.1 |  |
| 6.3.5A.2.4.2 Power Control Relative power tolerance for CA (inter-band 3DL CA and 3UL CA) |  | Same as 6.3.5A.2.2 | Same as 6.3.5A.2.2 |  |  | Same as 6.3.5A.2.2 |  |
| 6.3.5A.2.4.3 Power Control Relative power tolerance for CA (intra-band non-contiguous 3DL CA and 3UL CA) |  | Same as 6.3.5A.2.3 | Same as 6.3.5A.2.3 |  |  | Same as 6.3.5A.2.3 |  |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.3 Aggregate power control tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.4 Aggregate Power Control Tolerance for CA (3UL CA) |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.5 Aggregate Power Control Tolerance for CA (4UL CA) |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO |  | Same as 6.3.5.1 | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO |  | Same as 6.3.5.2 | Same as 6.3.5.2 |  |  | Same as 6.3.5.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.5C.2 Power Control Relative power tolerance for Dual Connectivity |  | Same as 6.3.5.2 on both PCC and SCC separately. | Same as 6.3.5.2 on both PCC and SCC separately. |  |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |
| 6.3.5C.2_1 Power Control Relative power tolerance for asynchronous Dual Connectivity |  | Same as 6.3.5.2 on both PCC and SCC separately. | Same as 6.3.5.2 on both PCC and SCC separately. |  |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |
| 6.3.5E.1 Power Control Absolute power tolerance for UE category 0 |  | Same as 6.3.5.1 | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5E.2 Power Control Relative power tolerance for UE category 0 |  | Same as 6.3.5.2 | Same as 6.3.5.2 |  |  | Same as 6.3.5.2 |  |
| 6.3.5E.3 Aggregate power control tolerance for UE category 0 |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5EA.1 Power Control Absolute power tolerance for UE category M1 |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB | 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB |  |
| 6.3.5EA.2 Power Control Relative power tolerance for UE category M1 |  | TS 36.101 [2] clause 6.3.5.1 All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±2.5 dB 2 ≤ ΔP < 3; ±3.0 dB 3 ≤ ΔP < 4; ±3.5 dB 4 ≤ ΔP ≤ 10; ±4.0 dB 10 ≤ ΔP < 15; ±5.0 dB 15 ≤ ΔP; ±6.0 dB | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±3.2 dB 2 ≤ ΔP < 3; ±3.7 dB 3 ≤ ΔP < 4; ±4.2 dB 4 ≤ ΔP < 10; ±4.7 dB 10 ≤ ΔP < 15; ±5.7 dB 15 ≤ ΔP; ±6.7 dB |  |
| 6.3.5EA.3 Aggregate power control tolerance for UE category M1 |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5EA.3_1 Aggregate power control tolerance for UE category M1 (CE Mode B) |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5EB.1 Power Control Absolute power tolerance for UE category 1bis |  | Same as 6.3.5.1 | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5EB.2 Power Control Relative power tolerance for UE category 1bis |  | Same as 6.3.5.2 | Same as 6.3.5.2 |  |  | Same as 6.3.5.2 |  |
| 6.3.5EB.3 Aggregate power control tolerance for UE category 1bis |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5EC.3_1 Aggregate power control tolerance for UE category M2 (CE Mode B) |  | Same as 6.3.5.3 | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5F.1 Power Control Absolute power tolerance for category NB1 and NB2 |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB | 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB |  |
| 6.3.5F.2 Power Control Relative power tolerance for UE category NB1 and NB2 |  | TS 36.101 [2] clause 6.3.5F.2 NPRACH transitions: ΔP = 0; ±1.5 dB ΔP = 2; ±2.0 dB ΔP = 4; ±3.5 dB ΔP = 6; ±4.0 dB | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT NPRACH transitions: ΔP = 0; ±2.2 dB ΔP = 2; ±2.7 dB ΔP = 4; ±4.2 dB ΔP = 6; ±4.7 dB |  |
| 6.3.5F.3 Aggregate power control tolerance for category NB1 and NB2 |  | Aggregate power control tolerance within TBD ms: PUSCH = ±3.5 dB | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit - TT PUSCH = ±4.2 dB |  |
| 6.3.5G.1 Power Control Absolute power tolerance for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz Normal conditions ± 3.0 dB Extreme conditions ± 6.0 dB | 2.0 dB 2.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 5.0 dB Extreme conditions ± 8.0 dB |  |
| 6.3.5G.2 Power Control Absolute power tolerance for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 4.2GHz < f ≤ 6GHz Normal conditions ± 3.0 dB Extreme conditions ± 6.0 dB | 1.0 dB 1.0 dB 2.0 dB 2.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 5.0 dB Extreme conditions ± 8.0 dB |  |
| 6.3.5G.3 Power Control Absolute power tolerance for V2X Communication / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz Normal conditions ± 3.0 dB Extreme conditions ± 6.0 dB | 2.0 dB 2.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 5.0 dB Extreme conditions ± 8.0 dB |  |
| 6.5.1 Frequency Error |  | Modulated carrier, f ≤ 4.2GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens 3.0GHz < f ≤ 4.2GHz DL power: Refsens | 15 Hz 0.7 dB 1.0 dB |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB Refsens +1.0dB |  |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.1 | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1A.2 Frequency Error for CA (inter-band DL CA and UL CA) |  | Same as 6.5.1 per CC | Same as 6.5.1 per CC |  |  | Same as 6.5.1 per CC |  |
| 6.5.1A.3 Frequency Error for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.5.1 per CC | Same as 6.5.1 per CC |  |  | Same as 6.5.1 per CC |  |
| 6.5.1A.5 Frequency error for CA (4UL CA) |  | Same as 6.5.1 per CC | Same as 6.5.1 per CC |  |  | Same as 6.5.1 per CC |  |
| 6.5.1B Frequency Error for UL-MIMO |  | Same as 6.5.1 | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1D.1 Frequency Error for ProSe Direct Discovery |  | Modulated carrier, f ≤ 4.2GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz Sidelink power: Refsens | 15 Hz 0.7 dB |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT Sidelink power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB |  |
| 6.5.1D.2 Frequency Error for ProSe Direct Communication |  | Same as 6.5.1D.1 | Same as 6.5.1D.1 |  |  | Same as 6.5.1D.1 |  |
| 6.5.1E Frequency Error for UE category 0 |  | Same as 6.5.1 | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1EA Frequency Error for UE category M1 |  | Same as 6.5.1 | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1EB Frequency Error for UE category 1bis |  | Same as 6.5.1 | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1EC Frequency Error for UE category M2 |  | Same as 6.5.1 | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1EC_1 Frequency Error for UE category M2 (CE Mode B) |  | Same as 6.5.1 | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1F Frequency Error for category NB1 and NB2 |  | Modulated carrier, f ≤ 3.0GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens | 15 Hz 0.7 dB |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB |  |
| 6.5.1G.1 Frequency Error for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  | Modulated carrier, f ≤ 6.0GHz Within 0.1 ppm compared to the received carrier frequency | 36 Hz |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 36 Hz) |  |
| 6.5.1G.2 Frequency error for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Modulated carrier, f ≤ 3.0GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens Modulated carrier, 4.2GHz <f ≤ 6.0GHz Within 0.1 ppm compared to the received carrier frequency 4.2GHz <f ≤ 6.0GHz DL power: Refsens | 15 Hz 0.7 dB 36 Hz 1.5 dB |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7 dB Modulated carrier frequency error = (0.1 ppm + 36 Hz) Refsens +1.5 dB |  |
| 6.5.2.1 Error Vector Magnitude |  | EVM limit: BPSK :17.5 % QPSK: 17.5 % 16QAM: 12.5 % | 0% |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.1_1 Error Vector Magnitude (EVM) for UL 64QAM |  | EVM limit: 64QAM: 8.0 % | 0% |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.1_2 Error Vector Magnitude (EVM) for UL 256QAM |  | EVM limit: 256QAM: 3.5 % | 0.3%, 15dBm < PUL 0.8%, -25dBm < PUL ≤ 15dBm, 1.1%, -30dBm ≤ PUL ≤ -25dBm |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.1A PUSCH-EVM with exclusion period |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % | 0% |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_1 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 64QAM |  | Same as 6.5.2.1_1 | Same as 6.5.2.1_1 |  |  | Same as 6.5.2.1_1 |  |
| 6.5.2A.1.1_2 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 256QAM |  | EVM limit: 256QAM: 3.5 % | Same as 6.5.2.1_2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_1 EVM for CA (inter-band DL CA and UL CA) with UL 64QAM |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 256QAM |  | EVM limit: 256QAM: 3.5 % | Same as 6.5.2.1_2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_1 EVM for CA (intra-band non-contiguous DL CA and UL CA) with UL 64QAM |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_2 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  | EVM limit: 256QAM: 3.5 % | Same as 6.5.2.1_2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.4 Error Vector Magnitude (EVM) for CA (3UL CA) |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.5 Error Vector Magnitude (EVM) for CA (4UL CA) |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.5.2.1E.1 Error Vector Magnitude for UE category 0 |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1E.2 PUSCH-EVM with exclusion period for UE category 0 |  | Same as 6.5.2.1A | Same as 6.5.2.1A |  |  |  |  |
| 6.5.2.1EA.1 Error Vector Magnitude for UE category M1 |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1EA.2 PUSCH-EVM with exclusion period for UE category M1 |  | Same as 6.5.2.1A | Same as 6.5.2.1A |  |  |  |  |
| 6.5.2.1EB Error Vector Magnitude (EVM) for UE category 1bis |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1EC.1 Error Vector Magnitude for UE category M2 |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1EC.2 PUSCH-EVM with exclusion period for UE category M2 |  | Same as 6.5.2.1A | Same as 6.5.2.1A |  |  | Same as 6.5.2.1A |  |
| 6.5.2.1F.1 Error Vector Magnitude (EVM) for UE category NB1 and NB2 |  | Same as 6.5.2.1 | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1G.1 Error Vector Magnitude (EVM) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1G.2 Error Vector Magnitude (EVM) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1G.3 Error Vector Magnitude (EVM) for V2X Communication / Intra-band contiguous multi-carrier operation |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1_s Error Vector Magnitude (EVM) for subslot/slot TTI |  | Same as 6.5.2 | Same as 6.5.2 |  |  | Same as 6.5.2 |  |
| 6.5.2.2 Carrier leakage |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc | 0.8dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.2F Carrier leakage for UE category NB1 and NB2 |  | Same as 6.5.2.2 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2G.1 Carrier leakage for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2G.2 Carrier leakage for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | For E-UTRA carrier, same as 6.5.2.2 For V2X carrier, same as 6.5.2.2G.1 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2G.3 Carrier leakage for V2X Communication / Intra-band contiguous MCC operation |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.3 In-band emissions for non allocated RB |  | For general emissions: within the subPRB allocation -18-5∙¦∆SubG-1)/LSCG For general emissions for any non-allocated RB: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc -40 dBm ≤ Output power < -30 dBm -10dBc For each evaluated RB, the test requirement is calculated as the higher of PRB – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) | 0.8dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.4 EVM equalizer Spectrum flatness |  | Normal conditions. If (F-FUL_low ≥ [3MHz])&(FUL_high-F≥ [3MHz]) 4 dB else 8 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 5 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 7 dB Extreme conditions: If (F-FUL_low ≥ [5MHz])&(FUL_high-F≥ [5MHz]) 4 dB else 12 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 6 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 10 dB | 1.4dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.2.2 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2A.2.2 Carrier leakage for CA (inter-band DL CA and UL CA) |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc | 0.8dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.2.3 Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc | 0.8dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.2.4 Carrier leakage for CA (3UL CA) |  | Same as 6.5.2.2 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2A.2.5 Carrier leakage for CA (4UL CA) |  | Same as 6.5.2.2 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.2 In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.3 In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.4 In-band emissions for non allocated RB for CA (3UL CA) |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.5 In-band emissions for non allocated RB for CA (4UL CA) |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO |  | TBD | TBD |  |  | TBD |  |
| 6.5.2B.2 Carrier leakage for UL-MIMO |  | Same as 6.5.2.2 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO |  | Same as 6.5.2.4 | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.2E Carrier Leakage for UE category 0 |  | Same as 6.5.2.2 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2EA Carrier Leakage for UE category M1 |  | Same as 6.5.2.2 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2EB Carrier leakage for UE category 1bis |  | Same as 6.5.2.2 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2EC Carrier leakage for UE category M2 |  | Same as 6.5.2.2 | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.3E In-band emissions for non allocated RB for UE category 0 |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3EA In-band emissions for non allocated RB for UE category M1 |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3EB In-band emissions for non allocated RB for UE category 1bis |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3EC In-band emissions for non allocated RB for UE category M2 |  | Same as 6.5.2.3 | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3F In-band emissions for non allocated RB for UE category NB1 and NB2 |  | For general emissions: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc -40 dBm ≤ Output power < -30 dBm -10dBc For each evaluated tone, the test requirement is calculated as the higher of Ptone – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3G.1 In-band emissions for non-allocated RB for V2X Communication |  | For general emissions: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc For each evaluated RB, the test requirement is calculated as the higher of PRB – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) Sidelink Power Window >= 0dBm, (-30 dBm, 0 dBm) E-UTRA Uplink Power Window >= 0dBm, (-30 dBm, 0 dBm) | Emission: 0.8dB Sidelink Power: Absolute Transmitted Power tolerance: ±3dB referting to the TS 36.101 [2] clasue 6.3.5G.1 Wanted signal uncertainty: ±1.3 dB 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= -40dBm Uplink power Relative Transmitted Power tolerance: ±2.5dB referting to the TS 36.101 [2] clasue 6.3.5.2 Test system power measurement uncertainty: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |  | Formula for emission: Minimum Requirement + TT Sidelink power measurement window: Bottom of the window: the Configured sidelink power - TT of sidelink power Top of the window: the Configured sidelink power + TT of sidelink power TT of sidelink power comprises 2 quantities: 1. Absolute Transmitted Power tolerance :±3dB 2. Test system power measurement uncertainty: ±1.3 dB 4.2GHz < f ≤ 6.0GHz for UE powers>= 0dBm. ±1.5 dB 4.2GHz < f ≤ 6.0GHz, for UE powers >= -40dBm Items 1 and 2 are added arithmetically: TT of sidelink power: (3dB+1.3dB) =4.3dB, 4.2GHz < f ≤ 6.0GH for UE powers >= 0 dBm: TT of sidelink power: (3dB+1.5dB) =4.5dB, 4.2GHz < f ≤ 6.0GH for powers >= -40 dBm: Ensure the bottom of the sidelink power measurement window is aligned with the bottom of the sidelink power window of the corresponding IBE requirement, and the top of the sidelink power measurement window is below the top of sidelink power window. Uplink power measurement window: Bottom of the window: the Configured sidelink power - TT of sidelink power Top of the window: the Configured sidelink power + TT of sidelink power TT of uplink power comprises 2 quantities: 1. Relative Transmitted Power tolerance :±2.5dB 2. Test system power measurement uncertainty: ±0.7 dB for f ≤ 3.0GHz. ±1 dB for 3.0GHz < f ≤ 4.2GHz Items 1 and 2 are added arithmetically: For f ≤ 3.0GHz TT of uplink power: (2.5dB+0.7dB) =3.2dB For 3.0GHz < f ≤ 4.2GHz TT of uplink power: (2.5dB+1dB) =3.5dB, Ensure the bottom of the uplink power measurement window is aligned with the bottom of the uplink power window of the corresponding IBE requirement, and the top of the uplink power measurement window is below the top of uplink power window. |  |
| 6.5.2.3G.2 In-band emissions for non-allocated RB for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | same as 6.5.2.3 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink | same as 6.5.2.3 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  |  | same as 6.5.2.3 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  |
| 6.5.2.3G.3 In-band emissions for non-allocated RB for V2X Communication / Intra-band contiguous MCC operation |  | same as 6.5.2.3G.1 for each cc of E-UTRA V2X sidelink | same as 6.5.2.3G.1 for each cc of E-UTRA V2X sidelink |  |  | same as 6.5.2.3G.1 for each cc of E-UTRA V2X sidelink |  |
| 6.5.2.4E EVM equalizer spectrum flatness for UE category 0 |  | Same as 6.5.2.4 | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4EA EVM equalizer spectrum flatness for UE category M1 |  | Same as 6.5.2.4 | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4EB EVM equalizer spectrum flatness for UE category 1bis |  | Same as 6.5.2.4 | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4EC EVM equalizer spectrum flatness for UE category M2 |  | Same as 6.5.2.4 | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4G.1 EVM equalizer spectrum flatness for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.5.2.4 | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4G.3 EVM equalizer spectrum flatness for V2X Communication / Intra-band contiguous multi-carrier operation uplink transmissions |  | Same as 6.5.2.4 | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.6.1 Occupied bandwidth |  | For 1.4 MHz channel bandwidth: Occupied channel bandwidth = 1.4 MHz For 3.0 MHz channel bandwidth: Occupied channel bandwidth = 3.0 MHz For 5 MHz channel bandwidth: Occupied channel bandwidth = 5 MHz For 10 MHz channel bandwidth: Occupied channel bandwidth = 10 MHz For 15 MHz channel bandwidth: Occupied channel bandwidth = 15 MHz For 20 MHz channel bandwidth: Occupied channel bandwidth = 20 MHz | 0kHz |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) |  | For 25(20+5) MHz channel bandwidth: Occupied channel bandwidth = 24.95 MHz For 30(20+10) MHz channel bandwidth: Occupied channel bandwidth = 29.90 MHz For 30(15+15) MHz channel bandwidth: Occupied channel bandwidth = 30.00 MHz For 35(20+15)MHz channel bandwidth: Occupied channel bandwidth = 34.85 MHz For 40(20+20) MHz channel bandwidth: Occupied channel bandwidth = 39.80 MHz | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) |  | Same as 6.6.1 per CC | Same as 6.6.1 per CC |  |  | Same as 6.6.1 per CC |  |
| 6.6.1A.3 |  | Same as 6.6.1 for each CC | Same as 6.6.1 |  |  | Same as 6.6.1 for each CC |  |
| 6.6.1A.5 Occupied bandwidth for CA (4UL CA) |  | Same as 6.6.1 for each CC | Same as 6.6.1 |  |  | Same as 6.6.1 for each CC |  |
| 6.6.1B Occupied bandwidth for UL-MIMO |  | Same as 6.6.1 | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1E Occupied bandwidth for UE category 0 |  | Same as 6.6.1 | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1EA Occupied bandwidth for UE category M1 |  | Same as 6.6.1 | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1EB Occupied bandwidth for UE category 1bis |  | Same as 6.6.1 | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1EC Occupied bandwidth for UE category M2 |  | Same as 6.6.1 | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1F Occupied bandwidth for UE category NB1 |  | Occupied channel bandwidth = 200KHz | 0kHz |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.1G.1 Occupied bandwidth for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  | Same as 6.6.1 | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1G.2 Occupied bandwidth for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Same as 6.6.1 | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1G.3 Occupied bandwidth for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.6.1 | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.2.1 Spectrum Emission Mask |  | For 1.4 MHz BW: -10 dBm / 30kHz -25dBm to -10dBm / 1MHz For 3 MHz BW: -13 dBm / 30kHz -25dBm to -10dBm / 1MHz For 5 MHz BW: -15dBm / 30kHz -25dBm to -10dBm / 1MHz For 10 MHz BW: -18dBm / 30kHz -25dBm to -10dBm / 1MHz For 15 MHz BW: -20dBm / 30kHz -25dBm to -10dBm / 1MHz For 20 MHz BW: -21dBm / 30kHz -25dBm to -10dBm / 1MHz | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB |  |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |  |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH |  | Same as 6.6.2.1 | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) |  | For 24.95 MHz BW: -22 dBm / 30kHz -25 dBm to - 10 dBm / 1 MHz For 29.9 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 30 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 34.85 MHz BW: -23.5dBm / 30kHz -25dBm to -10dBm / 1MHz For 39.8 MHz BW: -24dBm / 30kHz -25dBm to -10dBm / 1MHz | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1A.2 Spectrum emission mask for CA (inter-band DL CA and UL CA) |  | Same as 6.6.2.1 per CC | Same as 6.6.2.1 per CC |  |  | Same as 6.6.2.1 per CC |  |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.2.1 for each CC | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 for each CC |  |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO |  | Same as 6.6.2.1 | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1E Spectrum Emission Mask for UE category 0 |  | Same as 6.6.2.1 | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1EA Spectrum Emission Mask for UE category M1 |  | Same as 6.6.2.1 | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1EB Spectrum Emission Mask for UE category 1bis |  | Same as 6.6.2.1 | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1EC Spectrum Emission Mask for UE category M2 |  | Same as 6.6.2.1 | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1F Spectrum Emission Mask for UE category NB1 |  | For all frequencies -35dBm to 26dBm / 30kHz For frequencies that are Foffset away from edge of NB1 channel edge as defined in Table 6.6.2.1F.3-2, same as 6.6.2.1 | All cases: f ≤ 3.0GHz 1.5dB |  |  | For all frequencies interpolated Minimum Requirement + TT |  |
| 6.6.2.1G.1 Spectrum Emission Mask for V2X Communication Non-concurrent with E-UTRA uplink transmissions |  | Bandwidth: 10MHz -20dBm /30kHz -25 ~ -10 / 1 MHz Bandwidth: 20MHz -21dBm /30kHz -25 ~ -10 / 1 MHz | ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |  |
| 6.6.2.1G.2 Spectrum Emission Mask for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions |  | Same as 6.6.2.1G.1 for sidelink Same as 6.6.2.1G.1 for E-UTRA uplink | Same as 6.6.2.1G.1 |  |  | Same as 6.6.2.1G.1 |  |
| 6.6.2.1G.3 Spectrum Emission Mask for V2X Communication / Intra-band contiguous MCC operation |  | Same as 6.6.2.1G.1 | Same as 6.6.2.1G.1 |  |  | Same as 6.6.2.1G.1 |  |
| 6.6.2.2 Additional Spectrum Emission Mask |  | For 1.4 MHz BW: NS_03, NS_04 -10 dBm / 30 kHz -25 dBm to -13 dBm / 1MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1MHz For 3 MHz BW: NS_03, NS_04 -13 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100kHz -25 dBm to -13 dBm / 1 MHz For 5 MHz BW: NS_03, NS_04 -15 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -15 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1 MHz For 10 MHz BW: NS_03, NS_04, -18 dBm / 30 kHz -25 dBm to - 13dBm / 1 MHz NS_06 or NS_07 -18 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to - 13dBm / 1 MHz For 15 MHz BW: NS_03, NS_04 -20 dBm / 30kHz -25 dBm to -13 dBm / 1 MHz For 20 MHz BW: NS_03, NS_04 -21 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB 4.2GHz < f ≤ 6.0GHz ±2.0 dB |  |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |  |
| 6.6.2.2_1 Additional Spectrum Emission Mask for UL 64QAM |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2_2 Additional Spectrum Emission Mask for UL 256QAM |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A-1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) |  | For network signalled value CA_NS_04: -21dBm/30kHz to -22.5dBm/kHz, BW-dependent, for 0-1MHz -13dBm/1MHz for 1MHz-5.5MHz -25dBm/1MHz beyond 5.5MHz | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.1_1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.2A.1 | Same as 6.6.2.2A.1 |  |  | Same as 6.6.2.2A.1 |  |
| 6.6.2.2A.1_2 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.2A.1 | Same as 6.6.2.2A.1 |  |  | Same as 6.6.2.2A.1 |  |
| 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_1 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.3 Additional Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2B Additional Spectrum Emission Mask for UL-MIMO |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2E Additional Spectrum Emission Mask for UE category 0 |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2EA Additional Spectrum Emission Mask for UE category M1 |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2EB Additional Spectrum Emission Mask for UE category 1bis |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2EC Additional Spectrum Emission Mask for UE category M2 |  | Same as 6.6.2.2 | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2G.1 Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | ΔfOOB(MHz) 0-0.5 [-13-12∆fOOBMHz] ΔfOOB(MHz) 0.5-5 [-19-169∆fOOBMHz-0.5] ΔfOOB(MHz) 5-10 [-27-2∆fOOBMHz-5.0-13-12∆fOOBMHz] | Same as 6.6.2.2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.2.2G.2 Additional Spectrum Emission Mask for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions |  | same as 6.6.2.2 for E-UTRA uplink same as 6.6.2.2G.1 for sidelink | same as 6.6.2.2 for E-UTRA uplink same as 6.6.2.2G.1 for sidelink |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.2.3 Adjacent Channel Leakage power Ratio |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 30 dB UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 | 0 dB 0.8 dB 0.8 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 29.2 dB UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 |  |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR for Power Class 1: 37 dB E-UTRA ACLR for Power Class 2: 31 dB | 0dB 0.8dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR for Power Class 1: 36.2 dB E-UTRA ACLR for Power Class 2: 30.2 dB |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3_5 Adjacent Channel Leakage power Ratio for UL 256QAM |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3_6 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 256QAM |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 E-UTRA ACLR: 30 dB | 0 dB 0.8 dB 0.8 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 E-UTRA ACLR: 29.2 dB |  |
| 6.6.2.3A.1_1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.3A.1 | Same as 6.6.2.3A.1 |  |  | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.1_2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.3A.1 | Same as 6.6.2.3A.1 |  |  | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.6.2.3 per component carrier E-UTRA ACLR2: 40dB | Same as 6.6.2.3 per component carrier 1.3dB |  |  | Same as 6.6.2.3 per component carrier E-UTRA ACLR: 38.7 dB |  |
| 6.6.2.3A.2_1 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.3 per component carrier | Same as 6.6.2.3 per component carrier |  |  | Same as 6.6.2.3 per component carrier |  |
| 6.6.2.3A.2_2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.3A.2 | Same as 6.6.2.3A.2 |  |  | Same as 6.6.2.3A.2 |  |
| 6.6.2.3A.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_1 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_2 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous 3DL CA and 3UL CA) |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.2 Adjacent Channel Leakage power Ratio for CA (inter-band 3DL CA and 3UL CA) |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous 3DL CA and 3UL CA) |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3E Adjacent Channel Leakage power Ratio for UE category 0 |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3EA Adjacent Channel Leakage power Ratio for UE category M1 |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3EB Adjacent Channel Leakage power Ratio for UE category 1bis |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3EC Adjacent Channel Leakage power Ratio for UE category M2 |  | Same as 6.6.2.3 | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3F Adjacent Channel Leakage power Ratio for UE category NB1 |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. GSM ACLR: 20 dB UTRA ACLR: 37 dB for UTRA ACLR | 0 dB 0.8 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT GSM ACLR: 19.2 dB UTRA ACLR: 36.2 dB for UTRA ACLR |  |
| 6.6.2.3G.1 Adjacent Channel Leakage power Ratio for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 30dB for power class 3 UE 31dB for power class 2 UE | 0.8 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 29.2dB for power class 3 UE 30.2dB for power class 2 UE |  |
| 6.6.2.3G.2 Adjacent Channel Leakage power Ratio for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Same as 6.6.2.3G.1 | Same as 6.6.2.3G.1 |  |  | Same as 6.6.2.3G.1 |  |
| 6.6.2.3G.3 Adjacent Channel Leakage power Ratio for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.6.2.3G.1 | Same as 6.6.2.3G.1 |  |  | Same as 6.6.2.3G.1 |  |
| 6.6.2.4 Additional ACLR requirements |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 43 dB for UTRA ACLR 2 | 0 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement – TT E-UTRA ACLR: 42.2 dB for UTRA ACLR 2 |  |
| 6.6.3.1 Transmitter Spurious emissions |  | 9 kHz f < 150 kHz: -36dBm / 1kHz 150 kHz f < 30 MHz: -36dBm / 10kHz 30 MHz f < 1 GHz: -36dBm / 100kHz 1 GHz f < 12.75 GHz: -30dBm / 1MHz 12.75 GHz ≤ f < 19 GHz: -30dBm / 1MHz | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.1_1Transmitter Spurious emissions for Multi-Cluster PUSCH |  | Same as 6.6.3.1 | Same as 6.6.3.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.6.3.1 | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) |  | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.3 Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.4.1 Transmitter Spurious emissions for CA (3UL CA) (intra-band contiguous 3DL CA and 3UL CA) |  | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.4.2 Transmitter Spurious emissions for CA (3UL CA) (inter-band 3DL CA and 3UL CA) |  | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.4.3 Transmitter Spurious emissions for CA (3UL CA) (intra-band non-contiguous 3DL CA and 3UL CA) |  | Same as 6.6.3.1 per CC | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO |  | Same as 6.6.3.1 | Same as 6.6.3.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3E.1 Transmitter Spurious emissions for UE category 0 |  | Same as 6.6.3.1 | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3E.2 Spurious emission band UE co-existence for UE category 0 |  | Same as 6.6.3.2 | Same as 6.6.3.2 |  |  |  |  |
| 6.6.3E.3 Additional spurious emissions for UE category 0 |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3EA.1 Transmitter Spurious emissions for UE category M1 |  | Same as 6.6.3.1 | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3EA.2 Spurious emission band UE co-existence for UE category M1 |  | Same as 6.6.3.2 | Same as 6.6.3.2 |  |  |  |  |
| 6.6.3EA.3 Additional spurious emissions for UE category M1 |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3EB.1 Transmitter Spurious emissions for UE category 1bis |  | Same as 6.6.3.1 | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3EB.2 Spurious emission band UE co-existence for UE category 1bis |  | Same as 6.6.3.2 | Same as 6.6.3.2 |  |  | Same as 6.6.3.2 |  |
| 6.6.3EB.3 Additional spurious emissions for UE category 1bis |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3EC.1 Transmitter Spurious emissions for UE category M2 |  | Same as 6.6.3.1 | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3EC.2 Spurious emission band UE co-existence for UE category M2 |  | Same as 6.6.3.2 | Same as 6.6.3.2 |  |  | Same as 6.6.3.2 |  |
| 6.6.3EC.3 Additional spurious emissions for UE category M2 |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3F.1 Transmitter Spurious emissions for UE category NB1 |  | Same as 6.6.3.1 | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3G.1 Transmitter Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 9 kHz f < 150 kHz: -36dBm / 1kHz 150 kHz f < 30 MHz: -36dBm / 10kHz 30 MHz f < 1 GHz: -36dBm / 100kHz 1 GHz f < 12.75 GHz: -30dBm / 1MHz 12.75 GHz ≤ f < 26 GHz: -30dBm / 1MHz | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.1_1 Spurious emission band UE co-existence for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.6.3G.1 | Same as 6.6.3G.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.2 Spurious emission for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions |  | Same as 6.6.3G.1 | Same as 6.6.3G.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.2_1 Spurious emission band UE co-existence for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions |  | Same as 6.6.3G.1 | Same as 6.6.3G.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.3 Spurious emission for V2X Communication / Intra-band contiguous MCC operation |  | Same as 6.6.3G.1 | Same as 6.6.3G.1 |  |  |  |  |
| 6.6.3G.3_1 Spurious emission for V2X Communication / Intra-band contiguous MCC operation |  | Same as 6.6.3G.1 | Same as 6.6.3G.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2 Spurious emission band UE co-existence |  | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz Frequencies as detailed in core requirement | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) |  | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz -15.5 dBm / 5MHz +1.6 dBm /5MHz Frequencies as detailed in core requirement | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.3 Spurious emission band UE co-existence for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.3.2 | Same as 6.6.3.2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.4 Spurious emission band UE co-existence for CA (3UL CA) |  | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz -15.5 dBm / 5MHz +1.6 dBm /5MHz Frequencies as detailed in core requirement | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.3 Additional spurious emissions |  | NS_05 1884.5MHz f 1915.7MHz: -41dBm / 300kHz NS_07 769MHz ≤ f ≤ 775MHz -57dBm / 6.25kHz NS_08 860MHz ≤ f ≤ 895MHz -40dBm / 1MHz NS_09 1475.9MHz ≤ f ≤ 1510.9MHz -35dBm / 1MHz | 0 dB 1.5dB 0 dB 0 dB |  |  | Formula: Minimum Requirement + TT -41dBm / 300kHz -55.5 dBm / 6.25kHz -40dBm / 1MHz -35dBm / 1MHz |  |
| 6.6.3.3_1 Additional spurious emissions for UL 64QAM |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3_2 Additional spurious emissions for UL 256QAM |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) |  | CA_NS_01 E-UTRA band 34: -50dBm / 1MHz 1884.5MHz f 1915.7MHz: -41dBm / 300kHz CA_NS_02 E-UTRA band 33: -50dBm / 1MHz E-UTRA band 34: -50dBm / 1MHz CA_NS_03 E-UTRA band 34: -50dBm / 1MHz E-UTRA band 39: -50dBm / 1MHz | 0 dB 0 dB 0 dB 0 dB 0 dB 0 dB |  |  | Formula: Minimum Requirement + TT -50 MHz / 1MHz -41 dBm / 300kHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz |  |
| 6.6.3.3A.1_1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.3.3A.1 | Same as 6.6.3.3A.1 |  |  | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.1_2 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.3.3A.1 |  |  | Same as 6.6.3.3A.1 | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.2 Additional spurious emissions for CA (inter-band DL CA and UL CA) |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_1 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_2 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.4 Additional spurious emissions for CA (3UL CA) |  | Same as 6.6.3.3A.1 | Same as 6.6.3.3A.1 |  |  | Same as 6.6.3.3A.1 |  |
| 6.6.3B.2 Spurious emission band UE co-existence for UL-MIMO |  | Same as 6.6.3.2 | Same as 6.6.3.2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3B.3 Additional spurious emissions for UL-MIMO |  | Same as 6.6.3.3 | Same as 6.6.3.3 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3F.2 Spurious emission band UE co-existence for category NB1 UE |  | Same as 6.6.3.2 | Same as 6.6.3.2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.7 Transmit intermodulation |  | Intermodulation Product 5MHz -29 dBc 10MHz -35 dBc CW Interferer level = -40 dBc | 0 dB |  |  | Formula: CW interferer Minimum Requirement– TT Intermod Products limits remain unchanged. CW interferer level = -40 dBc |  |
| 6.7A.1 Transmit intermodulation for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7A.2 Transmit intermodulation for CA (inter-band DL CA and UL CA) |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7A.5 Transmit intermodulation for CA (4UL CA) |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7B Transmit intermodulation for UL-MIMO |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7E Transmit intermodulation for UE category 0 |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7EA Transmit intermodulation for UE category M1 |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7EB Transmit intermodulation for UE category 1bis |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7EC Transmit intermodulation for UE category M2 |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7F Transmit intermodulation for UE category NB1 |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7G.1 Transmit intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7G.2 Transmit intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7G.3 Transmit intermodulation for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.7 | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.8B Time alignment error for UL-MIMO |  | The Time Alignment Error (TAE) shall not exceed 130 ns | 25 ns |  |  | Formula: Minimum Requirement+ TT |  |

<<End of change>>