3GPP TSG-RAN WG4 Meeting #98e						R4-2103897

E-meeting, 25 January – 5 February 2021

Title:	LS on Test Methodology for UE URLLC Ultra Low BLER CQI requirements

Release:	Rel-16

Work Item:	NR_L1enh_URLLC-Perf

Source:	RAN WG4

To:	RAN WG5

Cc:

Contact Person:

#### Name:		Dmitry Belov

E-mail Address:	dmitry.belov@intel.com

Attachments: N/A

- Overall Description

RAN4 discussed the test methodology for UE verification of support of CQI Table 3 as a part of discussion for Rel-16 WI Physical layer enhancements for NR URLLC. RAN4 agreed that CQI Table 3 will be verified in scenarios with static propagation conditions. RAN4 concluded that the early pass/fail test methodology can be used for CQI requirements with static propagation conditions to achieve feasible testing time. CQI requirements with static propagation conditions have the following minimum requirements:

  - The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

  - If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5

  - The reported CQI value according to the reference channel shall be greater than or equal to 1

Early pass/fail test methodology can be used for part b) in the following way:

- If DUT early pass the test using the PDSCH transport format indicated by median CQI, then DUT should early fail the test using the PDSCH transport format indicated by (median CQI+1). If DUT early fail the test using the PDSCH transport format indicated by median CQI, then DUT should early pass the test using the PDSCH transport format indicated by (median CQI-1).

Also, RAN4 concluded that confidence level 99% can be considered to achieve feasible testing time for CQI requirements with static propagation conditions.

2. Actions:

To RAN WG5

RAN4 respectfully asks RAN5 to take into account the respective agreements when defining the test methodology for URLLC CQI test cases.

3. Date of Next RAN WG4 Meetings:

TSG RAN WG4 Meeting #98-bis-e		12 – 20 April 2021			E-meeting

TSG RAN WG4 Meeting #99-e		19 – 27 May 2021			E-meeting