3GPP TSG-RAN WG5 Meeting #91-e	R5-212198

Online, , 17th May 2021 – 28th May 2021

Source:	Qualcomm Incorporated

Title:	Discussion paper on support of highest channel bandwidth capability and test impact

# Document for:	Agreement

Agenda Item:	4.2.1

## 1.	Introduction

Networks deploying 5G may not support highest channel bandwidths defined in [3] and this may prevent UE to support highest channel bandwidth which in turn will fail few conformance testcases. This issue was discussed in RAN5#90-e, R5-211362 [7] and impact was found to be on both Signalling, RF areas and to seek a common agreeable solution was advised.

## 2.	Discussion

UE can indicate the supported channel bandwidths for each subcarrier spacing in UE Capability Information message as defined in [2].

[[2], cl 4.2.7.2]

channelBWs-DL

Indicates for each subcarrier spacing the UE supported channel bandwidths.
Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry.

…

channelBWs-UL

Indicates for each subcarrier spacing the UE supported channel bandwidths.

Absence of the channelBWs-UL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry.

[[2], cl 4.2.7.6]

supportedBandwidthDL

Indicates maximum DL channel bandwidth supported for a given SCS that UE supports within a single CC (and in case of intra-frequency DAPS handover for the source and target cells), which is defined in Table 5.3.5-1 in TS 38.101-1 [2] for FR1 and Table 5.3.5-1 in TS 38.101-2 [3] for FR2.

For FR1, all the bandwidths listed in TS38.101-1 Table 5.3.5-1 for each band shall be mandatory with a single CC unless indicated optional. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. When this field is included in a band combination with a single band entry and a single CC entry (i.e. non-CA band combination), the UE shall indicate the maximum channel bandwidth for the band according to TS 38.101-1 [2] and TS 38.101-2 [3].

supportedBandwidthDL IE specifies UE shall indicate the maximum channel bandwidth for the band according to TS 38.101-1 [2] and TS 38.101-2 [3], but bitmap in channelBWs IE allows UE to indicate support of all the lower bandwidths as well.

## 3.   Summary

Based on the discussion in section 2, impacted testcases of [1], [4], [5] & [6] needs to be updated to ensure that UE’s supported highest channel bandwidth is configured instead of maximum value defined in [3].

## 4.   Proposal/Way Forward

For SIG impacted test cases, either one of below proposal can be adopted

Proposal #1 – For all impacted test cases of [1], configure NR Cell bandwidth to highest channel bandwidth supported by UE by defining a new PIXIT parameter in [8] with highest channel bandwidth for band under test.

Proposal #2 – Configure highest channel bandwidth for band under test based on channelBWs-DL IE of received UE Capability Information message from UE.

For RF\RMM impacted test cases

Proposal #3 - TBD, RF\RRM subgroup to further discuss and propose way forward or consider one of SIG proposals above

## 5.	References

[1]	TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	 TS 38.306: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio access capabilities

[3]	TS 38.101-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception

[4]	TS 38.521-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

[5]	TS 38.521-2: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

[6]	TS 38.521-3: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

[7]	R5-211362: Discussion paper on impact of UE’s highest channel bandwidth capability on Transport Block Selection test cases

[8]	         R5-21xxxx: Addition of new PIXIT parameters for maximum supported bandwidth