3GPP TSG-RAN5 Meeting #91-e	R5-212235

# Online,  17th May 2021 – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0324 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Derivation of Test Requirements for UE Rx-Tx tests for Category M1 and M2 UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | PCTEST Engineering Laboratory, LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the UE Rx-Tx time difference test cases for Category M1/M2 UEs 8.1.7 to 8.1.9, the values of the Test Tolerances used are incorrect as these are not the same values as used for test 8.1.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The values of Test Tolerances used are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect values will remain and compliant UEs may unfairly fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of Modified Section >

## C.4	Derivation of Test Requirements (This clause is informative)

The Test Requirements have been calculated by relaxing the Conformance requirement of the core specification using the Test Parameter Relaxations defined in clause C.2. When the Test Parameter Relaxation is zero, the Test Requirement will be the same as the Conformance requirement. When the Test Parameter Relaxation is non-zero, the Test Requirements will differ from the Conformance requirement, and the formula used for this relaxation is given in table C.4.1, C.4.2, C.4.3 and C.4.4.

Table C.4.1: Derivation of Test Requirements for A-GNSS Minimum Performance tests

| Test | Conformance requirement in TS 25.171 [31] or TS 25.172 [19] or TS 36.171 [3] or 38.171 [43] |  | Test Parameter Relaxation (TPR) | Test Requirement |
| --- | --- | --- | --- | --- |
| 5.2.1, 6.2.1, 7.1.1, 13.2.1 Sensitivity Coarse Time Assistance | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Absolute GPS L1 C/A signal level (test 5.2.1, 7.1.1, 13.2.1, sub-test 1) | -142, -147 dBm | 1 dB | Level + TPR: -141, -146 dBm |
|  | Absolute GNSS signal level (Galileo) | -142, -147 dBm | 1 dB | Level + TPR: -141, -146 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.2.1, 7.1.1, 13.2.1, sub-tests 4, 5, 8, and 10 to 13) | -142, -147 dBm | 1 dB | Level + TPR: -141, -146 dBm |
|  | Absolute GNSS signal level (GLONASS) | -142, -147 dBm | 1 dB | Level + TPR: -141, -146 dBm |
|  | Absolute GNSS signal level (BDS) | -136, -145 dBm | 1 dB | Level + TPR: -135, -144 dBm |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.2.2, 6.2.2, 7.1.2, 13.2.2 Sensitivity Fine Time Assistance | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Fine Time Assistance | 10 us | 1 us | UL-TPR, LL+TPR: 9 us |
|  | Absolute GPS L1 C/A signal level (test 5.2.2, 7.1.2, 13.2.2, sub-test 1) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Absolute GNSS signal level (Galileo) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.2.2, 7.1.2, 13.2.2, sub-tests 4, 5, 8 and 10 to 13) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Absolute GNSS signal level (GLONASS) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Absolute GNSS signal level (BDS) | -147 dBm | 1 dB | Level + TPR: -146 dBm |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.3, 6.3, 7.2, 13.3 Nominal Accuracy | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Absolute GPS L1 C/A signal level (test 5.3, 7.2, 13.3, sub-test 1) | -130 dBm | 0 dB | Formulas: Level + TPR: -130 dBm |
|  | Absolute GNSS signal level (Galileo) | -127 dBm | 0 dB | Level + TPR: -127 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.3, 7.2, 13.3, sub-tests 4, 5, 8 and 10 to 13) | -128.5 dBm | 0 dB | Level + TPR: -128.5 dBm |
|  | Absolute GNSS signal level (GLONASS) | -131 dBm | 0 dB | Level + TPR: -131 dBm |
|  | Absolute GNSS signal level (QZSS) | -128.5 dBm | 0 dB | Level + TPR: -128.5 dBm |
|  | Absolute GNSS signal level (SBAS) | -131 dBm | 0 dB | Level + TPR: -131 dBm |
|  | Absolute GNSS signal level (BDS) | -133 dBm | 0 dB | Level + TPR: -133 dBm |
|  | Position error | 30 m | 1.3 m | Error +TPR: 31.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.4, 6.4, 7.3, 13.4 Dynamic Range | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Absolute GPS L1 C/A signal level (test 5.4, 7.3, 13.4, sub-test 1) | -129 to -147 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Absolute GNSS signal level (Galileo) | -127.5 to -147 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.4, 7.3, 13.4, sub-tests 4, 5, 8 and 10 to 13) | -129 to -147 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Absolute GNSS signal level (GLONASS) | -131.5 to -147 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Absolute GNSS signal level (BDS) | -133.5 to -145 dBm | 1 dB | Level + TPR: each level +1 dBm |
|  | Relative GPS L1 C/A signal level (test 5.4, 7.3, 13.4, sub-test 1) | 18 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -128.2 dBm |
|  | Relative GNSS signal level (Galileo) | 19.5 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -126.7 dBm |
|  | Relative GNSS signal level (GPS) (test 6.4, 7.3, 13.4, sub-tests 4, 5, 8 and 10 to 13) | 18 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -128.2 dBm |
|  | Relative GNSS signal level (GLONASS) | 15.5 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -130.7 dBm |
|  | Relative GNSS signal level (BDS) | 11.5 dB | 0.2 dB | Level - TPR: highest level -0.2 dB: -132.7 dBm |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.5, 6.5, 7.4, 13.5 Multi-path scenario | Coarse Time Assistance | 2 s | 200 ms | Formulas: UL-TPR, LL+TPR: 1.8 s |
|  | Absolute GPS L1 C/A signal level (test 5.5, 7.4, 13.5, sub-test 1) | -130 dBm | 0 dB | Formulas: Level + TPR: -130 dBm |
|  | Absolute GNSS signal level (Galileo) | -127 dBm | 0 dB | Level + TPR: -127 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.5, 7.4, 13.5, sub-tests 4, 5, 8 and 10 to 13) | -128.5 dBm | 0 dB | Level + TPR: -128.5 dBm |
|  | Absolute GNSS signal level (GLONASS) | -131 dBm | 0 dB | Level + TPR: -131 dBm |
|  | Absolute GNSS signal level (BDS) | -133 dBm | 0 dB | Level + TPR: -133 dBm |
|  | Relative GPS L1 C/A signal level (test 5.5, 7.4, 13.5, sub-test 1) -142, -147 dBm | 6 dB | 0.2 dB | Relative level + TPR: relative level + 0.2dB: 6.2 dB |
|  | Relative GNSS signal level (all GNSSs) (test 6.5, 7.4, 13.5, sub-tests 4, 5, 8 and 10 to 13) | Y dB where “Y” is given in Table 4.2.2 | 0.2 dB | Relative level + TPR: relative level + 0.2dB: Y + 0.2 dB |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Response time | 20 s | 300 ms | Time + TPR: 20.3 s |
| 5.6, 6.6, 7.5, 7.5A., 13.6, 13.7 Moving scenario and periodic update | Absolute GPS L1 C/A Signal level (test 5.6, 7.5, 7.5A., 13.6, 13.7, sub-test 1) | -130 dBm | 0 dB | Formulas: Level + TPR: -130 dBm |
|  | Absolute GNSS signal level (Galileo) | -127 dBm | 0 dB | Level + TPR: -127 dBm |
|  | Absolute GNSS signal level (GPS) (test 6.6, 7.5, 7.5A., 13.6, 13.7, sub-tests 4, 5, 8 and 10 to 13) | -128.5 dBm | 0 dB | Level + TPR: -128.5 dBm |
|  | Absolute GNSS signal level (GLONASS) | -131 dBm | 0 dB | Level + TPR: -131 dBm |
|  | Absolute GNSS signal level (BDS) | -133 dBm | 0 dB | Level + TPR: -133 dBm |
|  | Position error | 100 m | 1.3 m | Error +TPR: 101.3 m |
|  | Differential response time | 2s +/- 20 % | 100 ms | Time +TPR: 1.5 s and 2.5 s |

Table C.4.2: Derivation of Test Requirements for ECID and OTDOA Measurement tests

| Test | Minimum Requirement in TS 36.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 36.571-1 |
| --- | --- | --- | --- |
| 8.1.1 E-UTRAN FDD UE Rx - Tx time difference case (Rel-9 to Rel-11) | Test 1: Noc: -98dBm/15kHz Ês / Noc: -3.0dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês / Noc: -3.0dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 | Test 1: 0dB 0.3dB Via mapping Test 2: 0dB 0.3dB Via mapping | Test 2: Noc: -98dBm/15kHz Ês / Noc: -2.7.0dB (Measured value from step 7 - 23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |
| 8.1.1A E-UTRAN FDD UE Rx – Tx time difference case (Rel-12 onwards) | Same as 8.1.1 | Same as 8.1.1 | Test 1: Noc: -98dBm/15kHz Ês / Noc: -2.7.0dB (Measured value from step 7 - 23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB Measured value from step 7 -10) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +10) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |
| 8.1.1B E-UTRAN FDD UE Rx – Tx time difference case for UE Category 1bis | Same as 8.1.1 | Same as 8.1.1 | Same as 8.1.1A |
| 8.1.2 E-UTRAN TDD UE Rx - Tx time difference case (Rel-9 to Rel-11) | Same as 8.1.1 except use Table 4.6.3-2 | Same as 8.1.1 | Same as 8.1.1 except use Table 4.6.3-2 |
| 8.1.2A E-UTRAN TDD UE Rx – Tx time difference case (Rel-12 onwards) | Same as 8.1.1 except use Table 4.6.3-2 | Same as 8.1.1 | Same as 8.1.1A except use Table 4.6.3-2 |
| 8.1.2B E-UTRAN TDD UE Rx – Tx time difference case for UE Category 1bis | Same as 8.1.1 except use Table 4.6.3-2 | Same as 8.1.1 | Same as 8.1.1A except use Table 4.6.3-2 |
| 8.1.3 E-UTRAN FDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Test 1: Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +1.00dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +1.00dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 | Test 1: 0dB 0.3dB 0dB Via mapping Test 2: 0dB 0.3dB 0dB Via mapping | Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -2.70dB Ês2 / Noc: +1.00dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -2.70dB Ês2 / Noc: +1.00dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |
| 8.1.4 E-UTRAN TDD UE Rx-Tx time difference under Time Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) | Same as 8.1.3 except use Table 4.6.3-2 | Same as 8.1.3 | Same as 8.1.3 except use Table 4.6.3-2 |
| 8.1.5 E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Test 1: Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -3.00dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 | Test 1: 0dB 0.4dB 0dB 0dB Via mapping Test 2: 0dB 0.4dB 0dB 0dB Via mapping | Test 1: Noc: -98dBm/15kHz Ês1 / Noc: -2.60dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Test 2: Noc: -98dBm/15kHz Ês1 / Noc: -2.60dB Ês2 / Noc: +3.00dB Ês3 / Noc: +1.00dB Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |
| 8.1.6 E-UTRAN TDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 8.1.5 except use Table 4.6.3-2 | Same as 8.1.5 | Same as 8.1.5 except use Table 4.6.3-2 |
| 8.1.7 E-UTRAN FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Noc: -98dBm/15kHz Ês / Noc: -3.0dB Reported RxTx time difference value: Measured value converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 | 0dB 0.3dB Via mapping | Noc: -98dBm/15kHz Ês / Noc: -2.7.0dB Category M1: (Measured value from step 7 - 23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +23) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 Category M2: (Measured value from step 7 -13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 To (Measured value from step 7 +13) Ts converted to RX-TX_TIME_DIFFERENCE according to Table 4.6.3-1 |
| 8.1.8 E-UTRAN HD-FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.7 | Same as 8.1.7 | Same as 8.1.7 |
| 8.1.9 E-UTRAN TDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA | Same as 8.1.7 except use Table 4.6.3-2 | Same as 8.1.7 | Same as 8.1.7 except use Table 4.6.3-2 |
| 9.1.1 FDD RSTD Measurement Reporting Delay | Response Time = 3 s | 300 ms | Time + TPR: 3.3 s |
| 9.1.1A FDD RSTD Measurement Reporting Delay for UE Category 1bis | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.1.2 TDD RSTD Measurement Reporting Delay | Response Time = 3 s | 300 ms | Time + TPR: 3.3 s |
| 9.1.2A TDD RSTD Measurement Reporting Delay for UE Category 1bis | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.1.3 FDD RSTD Measurement Accuracy | For Test 2 and Test 4: PRS Ês1 / Noc = -6dB PRS Ês2 / Noc = -13dB For All Tests: See Table 9.1.3.3-1 for measurement accuracy. | +0.3 dB +0.3 dB ± 1 Ts | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 9.1.3.5-2. |
| 9.1.3A FDD RSTD Measurement Accuracy for UE Category 1bis | Same as 9.1.3 |  |  |
| 9.1.4 TDD RSTD Measurement Accuracy | Same as 9.1.3 |  |  |
| 9.1.4A TDD RSTD Measurement Accuracy for UE Category 1bis | Same as 9.1.3 |  |  |
| 9.2.1 FDD-FDD inter-frequency RSTD measurement reporting delay | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.2.1A FDD-FDD inter-frequency RSTD measurement reporting delay for UE Category 1bis | Response Time = 11 s | 300 ms | Time + TPR: 11.3 s |
| 9.2.2 TDD-TDD inter-frequency RSTD measurement reporting delay | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.2.2A TDD-TDD inter-frequency RSTD measurement reporting delay for UE Category 1bis | Response Time = 11 s | 300 ms | Time + TPR: 11.3 s |
| 9.2.4 FDD-FDD inter frequency RSTD Accuracy | PRS Ês1 / Noc1 = -6dB PRS Ês2 / Noc2 = -13dB See TS 36.133 [23] Table 9.1.10.3-1 for measurement accuracy. | +0.3 dB +0.3 dB ± 2 Ts | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 9.2.4.5-2. |
| 9.2.4A FDD-FDD inter frequency RSTD Accuracy for UE Category 1bis | Same as 9.2.4 |  |  |
| 9.2.5 TDD-TDD inter frequency RSTD Accuracy | Same as 9.2.4 |  |  |
| 9.2.5A TDD-TDD inter frequency RSTD Accuracy for UE Category 1bis | Same as 9.2.4 |  |  |
| 9.3.1.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.3.1.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.3.2.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.3.2.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.3.3.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.3.3.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Response Time = 6 s | 300 ms | Time + TPR: 6.3 s |
| 9.3.4.1 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Response Time = 13 s | 300 ms | Time + TPR: 13.3 s |
| 9.3.4.2 FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Test 1: Response Time 13 s Test 2: Response Time 6 s | 300 ms | Test 1: Time + TPR: 13.3 s Test 2: Time + TPR: 6.3 s |
| 9.3.5.1 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Response Time = 13 s | 300 ms | Time + TPR: 13.3 s |
| 9.3.5.2 HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Test 1: Response Time 13 s Test 2: Response Time 6 s | 300 ms | Test 1: Time + TPR: 13.3 s Test 2: Time + TPR: 6.3 s |
| 9.3.6.1 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Response Time = 13 s | 300 ms | Time + TPR: 13.3 s |
| 9.3.6.2 TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Test 1: Response Time 13 s Test 2: Response Time 6 s | 300 ms | Test 1: Time + TPR: 13.3 s Test 2: Time + TPR: 6.3 s |
| 9.3.7.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.7.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.8.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.8.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.9.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.1.3 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.9.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.1.3 | Same as 9.1.3 | Same as 9.1.3 |
| 9.3.10.1 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | PRS Ês1 / Noc1 = -15dB PRS Ês2 / Noc2 = -15dB See TS 36.133 [23] Table 9.1.10.3-1 for measurement accuracy. | +0.3 dB +0.3 dB ± 2 Ts | Level + TPR, -14.7 dB Level + TPR, -14.7 dB See Table 9.3.10.1.5-2. |
| 9.3.10.2 FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.3.10.1 | Same as 9.3.10.1 | Same as 9.3.10.1 |
| 9.3.11.1 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.3.10.1 | Same as 9.3.10.1 | Same as 9.3.10.1 |
| 9.3.11.2 HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.3.10.1 | Same as 9.3.10.1 | Same as 9.3.10.1 |
| 9.3.12.1 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.3.10.1 | Same as 9.3.10.1 | Same as 9.3.10.1 |
| 9.3.12.2 TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.3.10.1 | Same as 9.3.10.1 | Same as 9.3.10.1 |
| 9.4.1.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Response Time = 16 s | 300 ms | Time + TPR: 16.3 s |
| 9.4.1.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Test 1: Response Time 16 s Test 2: Response Time 11 s | 300 ms | Test 1: Time + TPR: 16.3 s Test 2: Time + TPR: 11.3 s |
| 9.4.2.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Response Time = 16 s | 300 ms | Time + TPR: 16.3 s |
| 9.4.2.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Test 1: Response Time 16 s Test 2: Response Time 11 s | 300 ms | Test 1: Time + TPR: 16.3 s Test 2: Time + TPR: 11.3 s |
| 9.4.3.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1 | Response Time = 16 s | 300 ms | Time + TPR: 16.3 s |
| 9.4.3.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2 | Test 1: Response Time 16 s Test 2: Response Time 11 s | 300 ms | Test 1: Time + TPR: 16.3 s Test 2: Time + TPR: 11.3 s |
| 9.4.4.1 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Response Time = 42 s | 300 ms | Time + TPR: 42.3 s |
| 9.4.4.2 FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Test 1: Response Time 42 s Test 2: Response Time 11 s | 300 ms | Test 1: Time + TPR: 42.3 s Test 2: Time + TPR: 11.3 s |
| 9.4.5.1 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Response Time = 42 s | 300 ms | Time + TPR: 42.3 s |
| 9.4.5.2 HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Test 1: Response Time 42 s Test 2: Response Time 11 s | 300 ms | Test 1: Time + TPR: 42.3 s Test 2: Time + TPR: 11.3 s |
| 9.4.6.1 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1 | Response Time = 42 s | 300 ms | Time + TPR: 42.3 s |
| 9.4.6.2 TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2 | Test 1: Response Time 42 s Test 2: Response Time 11 s | 300 ms | Test 1: Time + TPR: 42.3 s Test 2: Time + TPR: 11.3 s |
| 9.4.7.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | See TS 36.133 [23] Table 9.1.10.3-1 for measurement accuracy. | ± 2 Ts | See Table 9.4.7.1.5-2. |
| 9.4.7.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.8.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.8.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.9.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.9.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.10.1 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.10.2 FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.11.1 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.11.2 HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.12.1 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.4.12.2 TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2 | Same as 9.4.7.1 | Same as 9.4.7.1 | Same as 9.4.7.1 |
| 9.5.1 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | NPRS Ês1 / Noc = -6dB NPRS Ês2 / Noc = -13dB For All Tests: See Table 9.5.1.3-2 for measurement accuracy. | +0.3 dB +0.3 dB ± 1 Ts | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 9.5.1.5-3. |
| 9.5.2 HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | NPRS Ês1 / Noc = -15dB NPRS Ês2 / Noc = -15dB For All Tests: See Table 9.5.2.3-2 for measurement accuracy. | +0.3 dB +0.3 dB ± 1 Ts | Level + TPR, -14.7 dB Level + TPR, -14.7 dB See Table 9.5.2.5-3. |
| 9.5.3 HD-FDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Response Time = 78 s | 300 ms | Time + TPR: 78.3 s |
| 9.6.1 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage | NPRS Ês1 / Noc1 = -6dB NPRS Ês2 / Noc2 = -13dB For All Tests: See Table 9.6.1.3-2 for measurement accuracy. | +0.3 dB +0.3 dB ± 2 Ts | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 9.6.1.5-3. |
| 9.6.2 HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage | NPRS Ês1 / Noc1 = -15dB NPRS Ês2 / Noc2 = -15dB For All Tests: See Table 9.6.2.3-2 for measurement accuracy. | +0.3 dB +0.3 dB ± 2 Ts | Level + TPR, -14.7 dB Level + TPR, -14.7 dB See Table 9.6.2.5-3. |
| 9.6.3 HD-FDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Inband Mode in enhanced coverage | Same as 9.5.3 | Same as 9.5.3 | Same as 9.5.3 |
| 10.1 FDD RSTD Measurement Reporting Delay for Carrier Aggregation | Test 1: Response Time = 3 s Test 2: Response Time = 6 s | 300 ms 300 ms | Time + TPR: 3.3 s Time + TPR: 6.3 s |
| 10.1A FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz | Same as 10.1 |  |  |
| 10.1B FDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth | Same as 10.1 |  |  |
| 10.1C FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Same as 10.1 |  |  |
| 10.2 TDD RSTD Measurement Reporting Delay for Carrier Aggregation | Same as 10.1 |  |  |
| 10.2A TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz | Same as 10.1 |  |  |
| 10.2B TDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz +5 MHz Bandwidth | Same as 10.1 |  |  |
| 10.2C TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Same as 10.1 |  |  |
| 10.2D TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz +10 MHz Bandwidth | Same as 10.1 |  |  |
| 10.3 FDD RSTD Measurement Accuracy for Carrier Aggregation | PRS Ês2 / Noc2 = -6dB PRS Ês3 / Noc2 = -13dB See TS 36.133 [23] Table 9.1.10.1-1 for measurement accuracy. | +0.3 dB +0.3 dB ± 1 Ts | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 10.3.5-2. |
| 10.3A FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-10 and Rel-11) | Same as 10.3 | Same as 10.3 | Same as 10.3 |
| 10.3A_1 FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-12 onwards) | PRS Ês2 / Noc2 = -6dB PRS Ês3 / Noc2 = -13dB See TS 36.133 [23] Table 9.1.10.1-1 for measurement accuracy. | +0.3 dB +0.3 dB ± 1 Ts | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 10.3A_1.5-1. |
| 10.3B FDD RSTD Measurement Accuracy for Carrier Aggregation for 5 MHz+5 MHz Bandwidth | PRS Ês2 / Noc2 = -6dB PRS Ês3 / Noc2 = -13dB See TS 36.133 [23] Table 9.1.10.1-1 for measurement accuracy. | +0.3 dB +0.3 dB ± 1 Ts | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 10.3B.5-2. |
| 10.3C FDD RSTD Measurement Accuracy for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Same as 10.3B | Same as 10.3B | Same as 10.3B |
| 10.4 TDD RSTD Measurement Accuracy for Carrier Aggregation | Same as 10.3 | Same as 10.3 | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 10.4.5-2. |
| 10.4A TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-10 and Rel-11) | Same as 10.3 | Same as 10.3 | Same as 10.4 |
| 10.4A_1 TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz (Rel-12 onwards) | PRS Ês2 / Noc2 = -6dB PRS Ês3 / Noc2 = -13dB See TS 36.133 [23] Table 9.1.10.1-1 for measurement accuracy. | +0.3 dB +0.3 dB ± 1 Ts | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 10.4A_1.5-1. |
| 10.4B TDD RSTD Measurement Accuracy for Carrier Aggregation for 5 MHz+5 MHz Bandwidth | Same as 10.3B | Same as 10.3B | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 10.4B.5-2. |
| 10.4C TDD RSTD Measurement Accuracy for Carrier Aggregation for 10 MHz+5 MHz Bandwidth | Same as 10.3B | Same as 10.3B | Same as 10.4B |
| 10.4D TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz+10 MHz Bandwidth | Same as 10.3 | Same as 10.3 | Same as 10.4 |
| 10.5 FDD 3 DL CA RSTD Measurement Reporting Delay | Same as 10.1 | Same as 10.1 | Same as 10.1 |
| 10.6 TDD 3 DL CA RSTD Measurement Reporting Delay | Same as 10.1 | Same as 10.1 | Same as 10.1 |
| 10.7 FDD RSTD Measurement Accuracy for 3DL Carrier Aggregation | PRS Ês3 / Noc3 = -6dB PRS Ês4 / Noc3 = -13dB See TS 36.133 [23] Table 9.1.10.1-1 for measurement accuracy or intra-band. See TS 36.133 [23] Table 9.1.10.2-1 for measurement accuracy or inter-band | +0.3 dB +0.3 dB ± 1 Ts ± 2 Ts | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 10.7.5-2 See Table 10.7.5-2 |
| 10.8 TDD RSTD Measurement Accuracy for 3DL Carrier Aggregation | Same as 10.7 | Same as 10.7 | Level + TPR, -5.7 dB Level + TPR, -12.7 dB See Table 10.8.5-2 See Table 10.8.5-2 |

Table C.4.3: Derivation of Test Requirements for MBS Minimum Performance tests

| Test | Conformance requirements in 11.1.3, 11.2.3, 11.3.3, 11.4.3 and 11.5.3 |  | Test Parameter Relaxation (TPR) | Test Requirement |
| --- | --- | --- | --- | --- |
| 11.1, 11.1A MBS Measurement Reporting Delay | Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | Response time | 12 seconds | 300 ms | Time+TPR: 12.3 s |
| 11.2 MBS Sensitivity Measurement Accuracy | Beacon power level | -130 dBm | 2 dB | Level+TPR: -128 dBm |
|  | Code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |
| 11.2A MBS Sensitivity Measurement Accuracy | Beacon power level | -130 dBm | 2 dB | Level+TPR: -128 dBm |
|  | Code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |
|  | Code phase difference | 9.3 10-5 ms | 5 ns | Error+TPR: 9.8 10-5 ms |
| 11.3 MBS Nominal Measurement Accuracy | Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | Code phase difference | 7.1 10-5 ms | 5 ns | Error+TPR: 7.6 10-5 ms |
| 11.3A MBS Nominal Measurement Accuracy | Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | TB1 Code phase difference | 7.1 10-5 ms | 5 ns | Error+TPR: 7.6 10-5 ms |
|  | TB2 Code phase difference | 2.8 10-5 ms | 5 ns | Error+TPR: 3.3 10-5 ms |
| 11.4 MBS Dynamic Range Measurement Accuracy | High Power Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | Code phase difference | 7.1 10-5 ms | 5 ns | Error+TPR: 7.6 10-5 ms |
|  | Low Power Beacon power level | -130 dBm | 2 dB | Level+TPR: -128 dBm |
|  | Code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |
| 11.4A MBS Dynamic Range Measurement Accuracy | High Power Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | TB1 High Power code phase difference | 7.1 10-5 ms | 5 ns | Error+TPR: 7.6 10-5 ms |
|  | TB2 High Power code phase difference | 2.8 10-5 ms | 5 ns | Error+TPR: 3.3 10-5 ms |
|  | Low Power Beacon power level | -130 dBm | 2 dB | Level+TPR: -128 dBm |
|  | TB1 Low Power code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |
|  | TB2 Low Power code phase difference | 9.3 10-5 ms | 5 ns | Error+TPR: 9.8 10-5 ms |
| 11.5, 11.5A MBS Measurement Accuracy in Multipath | Beacon power level | -30 dBm | 0 dB | Level+TPR: -30 dBm |
|  | Code phase difference | 2.35 10-4 ms | 5 ns | Error+TPR: 2.40 10-4 ms |

Table C.4.4: Derivation of Test Requirements for WLAN and BLE measurement tests

| Test | Conformance requirements in 12.1.1, 12.1.2 and 12.2.1 |  | Test Parameter Relaxation (TPR) | Test Requirement |
| --- | --- | --- | --- | --- |
| 12.1.1 WLAN AP Identification and reporting delay under nominal conditions | Response time | 20.85 seconds | 300 ms | Time+TPR: 21.15 s |
| 12.1.2 WLAN AP Identification and reporting delay under dynamic range conditions | Response time | 20.85 seconds | 300 ms | Time+TPR: 21.15 s |
|  | Low Power WLAN APs Received Power Level | WLAN 2.4 GHz band: -74 dBm WLAN 5 GHz band: -79 dBm | 1 dB | Power+TPR: WLAN 2.4 GHz band: -73 dBm WLAN 5 GHz band: -78 dBm |
| 12.2.1 Bluetooth identification | Response time | 10.85 seconds | 300 ms | Time+TPR: 11.15 s |

# < End of Modified Section >

# < END OF CHANGE >