3GPP TSG-RAN5 Meeting #91-e	R5-212285

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0138 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 37.571-4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2021-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 3GPP (RAN2) changed the number of TS 36.355 (LPP) to TS 37.355 during Release 15. It is now necessary to make this update in the various TS 37.571-X specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | References to TS 36.355 are updated to TS 37.355 and “Evolved Universal Terrestrial Radio Access (E-UTRA)” is deleted from the title. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | References will remain confusing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.1, 6.2.1, A.2.1, B.3, Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Similar CR for TS 37.571-1, TS 37.571-2, TS 37.571-3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<start of modified section>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 37.571-1: " User Equipment (UE) conformance specification for UE positioning; Part 1: Conformance test specification".

[3]	3GPP TS 37.571-2: " User Equipment (UE) conformance specification for UE positioning; Part 2: Protocol conformance".

[4]	3GPP TS 37.571-3: " User Equipment (UE) conformance specification for UE positioning; Part 3: Implementation Conformance Statement (ICS)".

[5]	3GPP TS 37.571-5: " User Equipment (UE) conformance specification for UE positioning; Part 5: Test scenarios and assistance data".

[6]	3GPP TS 36.523-3: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Test suites".

[7]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[8]	3GPP TS 34.123-3: "User Equipment (UE) conformance specification; Part 3: Abstract test suite (ATS)".

[9]	3GPP TS 34.108: "Common test environments for User Equipment (UE); Conformance testing".

[10]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[11]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[12]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification; Formats and coding".

[14]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[15]	ITU-T Recommendation X.880: "Information technology - Remote Operations: Concepts, model and notation".

[16]	ITU-T Recommendation Q.773: "Transaction capabilities formats and encoding".

[17]	ETSI ES 201 873-1: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language".

[18]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions".

[19]	3GPP TS 23.038: "Alphabets and language-specific information".

[20]	ETSI TR 101 666 (V1.0.0): "Information technology; Open Systems Interconnection Conformance testing methodology and framework; The Tree and Tabular Combined Notation (TTCN) (Ed. 2++)".

[21]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[22]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[23]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[24]	3GPP TS 38.523-3: "5GS; User Equipment (UE) conformance specification; Part 3: Protocol Test Suites".

[25]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[26]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[27]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[28]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

<end of modified section>

<start of modified section>

### 5.2.1	LTE GNSS test model

Figure 5.2.1-1: E-UTRA Test model for LTE positioning GNSS testing

The LTE positioning test model consists of an SS E-UTRAN protocol stack and a positioning simulator. The SS E-UTRA emulation part is the same as the model defined in 3GPP TS 36.523-3 [6].

The UE is configured in normal mode. On UE side Ciphering/Integrity (PDCP and NAS) is enabled. The tests are performed on C-Plane.

The ASPs at POS Port are used to:

-	Load a pre-defined test scenario into the positioning simulator.

-	Start or stop generating and broadcasting satellite signals from the positioning simulator.

-	Retrieve the GNSS assistance data from assistance data source.

Under the ASP commands, the upper tester loads or re-loads pre-defined GNSS test scenario to the positioning simulator. The generated satellite signals shall simulate the GNSS environment as specified in 3GPP TS 37.571-2 [3], clause 5.

The SS also sends the GNSS assistance data to the UE through LPP signalling (see 3GPP TS 37.355 [10]) to facilitate the UE acquiring and tracking satellites. Such assistance data shall be consistent to within 2 seconds with the satellite signals generated (see 3GPP TS 37.571-2 [3], clause 5.2.1).

The assistance data source shall provide the assistance data consistent to +1/-0 second with the GNSS test scenario currently running in the satellite simulator (i.e. the data shall be up to 1 second in advance of the scenario); this allows for a further 2 seconds of latency in the SS. In case multiple satellite systems are configured in a test, the assistance data are requested or retrieved in a single ASP command for all satellite systems.

The UE positioning services are encoded in a TCAP [16] ROSE component [15] of the facility information element [14] in the Supplement Service [13] conveyed in the NAS messages. The facility information element is encoded in BER in TTCN referred to 3GPP TS 24.080 [13], clause 3.6.1 and 3GPP TS 29.002 [14], clause 17.1.1.

<end of modified section>

<start of modified section>

### 6.2.1	LTE indoor positioning test model

Figure 6.2.1-1: E-UTRA Test model for LTE indoor positioning testing

The LTE Indoor Positioning test model consists of an SS E-UTRAN protocol stack and an Indoor Positioning simulator. Depending on the Indoor Positioning Technology to be tested, the Indoor Positioning simulator will generate:

-	MBS beacon signals, for MBS test cases;

-	WLAN signals, for WLAN test cases;

-	Bluetooth signals, for Bluetooth test cases.

-	The Indoor Positioning simulator will be switched off for Sensor test cases.

The SS E-UTRA emulation part is the same as the model defined in 3GPP TS 36.523-3 [6].

The UE is configured in normal mode. On UE side Ciphering/Integrity (PDCP and NAS) is enabled. The tests are performed on C-Plane.

The ASPs at POS Port are used to:

-	Load a pre-defined test scenario into the Indoor Positioning simulator.

-	Start or stop generating and broadcasting signals from the Indoor Positioning simulator.

-	Retrieve the appropriate (MBS, WLAN, Sensor) assistance data from the assistance data source.

Under the ASP commands, the upper tester loads or re-loads pre-defined test scenario to the Indoor Positioning simulator. The generated signals shall simulate the corresponding Indoor Positioning environment as specified in 3GPP TS 37.571-2 [3], clause 5.

The SS also sends the appropriate assistance data to the UE through LPP signalling (see 3GPP TS 37.355 [10]) to facilitate the UE acquisition of MBS Beacons or WLAN APs. For UE-based tests, the SS also sends the appropriate assistance data to enable the UE to calculate its position for MBS or WLAN tests, or calculate altitude based on barometric pressure sensor measurements.

The UE positioning services are encoded in a TCAP [16] ROSE component [15] of the facility information element [14] in the Supplement Service [13] conveyed in the NAS messages. The facility information element is encoded in BER in TTCN referred to 3GPP TS 24.080 [13], clause 3.6.1 and 3GPP TS 29.002 [14], clause 17.1.1.

<end of modified section>

<start of modified section>

## A.2.1	Baseline of specifications

Table A.2.1-1 shows the baseline of the relevant core specifications and the test specifications which the delivered TTCN test suites are referred to.

Table A.2.1-1: References of the test and Core specifications

| Core specifications | 3GPP TS 37.355 [10] |
| --- | --- |
| baseline | 3GPP TS 24.301 [11] |
|  | 3GPP TS 25.331 [12] |
|  | 3GPP TS 24.501 [25] |
|  | 3GPP TS 36.331 [26] |
|  | 3GPP TS 38.331 [27] |
| Test specifications | 3GPP TS 37.571-2 [3] |
|  | 3GPP TS 37.571-3 [4] |
|  | 3GPP TS 37.571-5 [5] |
|  | 3GPP TS 36.508 [7] |
|  | 3GPP TS 38.508-1 [28] |

The Test Suite in TTCN3 is contained in multiple ASCII files which accompany the present document.

<end of modified section>

<start of modified section>

# B.3	LPP ASN.1

The LPP asn.1 modules are extracted from 3GPP TS 37.355 [10] without any change.

<end of modified section>

<start of modified section>

Annex D (informative)
TTCN-3 Definitions

# D.1	PosSystem_ASP_TypeDefs

PosSystem_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_MaxPosSystems | integer | 12 |  |

PositioningSystemType

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | PositioningSystemType |
| Comment |  |
| gps |  |
| modernizedGPS |  |
| glonass |  |
| galileo |  |
| qzss |  |
| otdoa |  |
| ecid |  |
| bds |  |
| mbs |  |
| wlan |  |
| bluetooth |  |
| sensor |  |

PositioningSystemList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PositioningSystemList_Type |
| Comment |  |
| record length (1..tsc_MaxPosSystems) of PositioningSystemType |  |

Altitude_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Altitude_Type |  |  |
| Comment |  |  |  |
| Height | integer |  | Height above a sea level in meters |

## D.1.1	PosSystem_Load_Scenario

Load scenarios follow 3GPP TS 37.571-5 [5]

PosSystem_Load_Scenario: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| ScenarioNumber_Type | integer |  |

PositioningScenario_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PositioningScenario_Type |  |
| Comment |  |  |
| LTE_Positioning | ScenarioNumber_Type | Reference to 3GPP TS 37.571-5 [5] |
| AGNSS | ScenarioNumber_Type | Reference to 3GPP TS 37.571-5 [5] |
| V2X | ScenarioNumber_Type | V2X Reference to 3GPP TS 36.508 [7] |
| NR_Positioning | ScenarioNumber_Type | Reference to 3GPP TS 37.571-5 [5] |
| Aerial | ScenarioNumber_Type | Reference to 3GPP TS 36.508 [7] clause 4.12 |

## D.1.2	PosSystem_Retrieve_Data

### D.1.2.1	SatSystem_Retrieve_UTRAN_Data

UTRAN_GPS_AssistanceDataRequest_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_GPS_AssistanceDataRequest_Type |  |  |
| Comment |  |  |  |
| AssistanceDataReqGPS | bitstring |  | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GPS-AdditionalAssistanceDataRequest |

UTRAN_GANSS_AssistanceDataRequest_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_GANSS_AssistanceDataRequest_Type |  |  |
| Comment |  |  |  |
| AssistanceDataReqGANSS | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AdditionalAssistanceDataRequest |
| AssistanceDataReqGANSSv860ext | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AdditionalAssistanceDataRequest-v860ext |
| AssistanceDataReqGANSSvc50ext | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AdditionalAssistanceDataRequest-vc50ext |

UTRAN_AssistanceDataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UTRAN_AssistanceDataRequest_Type |  |
| Comment |  |  |
| GPS | UTRAN_GPS_AssistanceDataRequest_Type |  |
| GANSS | UTRAN_GANSS_AssistanceDataRequest_Type |  |

UTRAN_AssistanceDataRequestList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | UTRAN_AssistanceDataRequestList_Type |
| Comment |  |
| record length(1..tsc_MaxPosSystems) of UTRAN_AssistanceDataRequest_Type |  |

UTRAN_GPS_AssistanceData_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_GPS_AssistanceData_Type |  |  |
| Comment | The returned Almanac information is split into two fields: Almanac for satellites 1 to 16 in AssistanceDataGPSr7 together with other information; Almanac for satellites 17 to 31: in 'almanacSat17To31' |  |  |
| AssistanceDataGPSr7 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GPS-AssistanceData-r7 |
| AssistanceDataGPSr12 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GPS-AssistanceData-r12 |
| AlmanacSatInfoList17To31 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type AlmanacSatInfoList |

UTRAN_GANSS_AssistanceData_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_GANSS_AssistanceData_Type |  |  |
| Comment |  |  |  |
| AssistanceDataGANSS_r8 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AssistanceData-r8 |
| AssistanceDataGANSS_r12 | bitstring | opt | bitstring containing 3GPP TS 25.331 [12] type UE-Positioning-GANSS-AssistanceData-r12 |

UTRAN_AssistanceData_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UTRAN_AssistanceData_Type |  |
| Comment |  |  |
| GPS | UTRAN_GPS_AssistanceData_Type |  |
| GANSS | UTRAN_GANSS_AssistanceData_Type |  |

UTRAN_AssistanceDataList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | UTRAN_AssistanceDataList_Type |
| Comment |  |
| record length(1..tsc_MaxPosSystems) of UTRAN_AssistanceData_Type |  |

### D.1.2.2	PosSystem_Retrieve_LPP_Data

When data is to be requested for more than one positioning system, they will be included into one single template. This template will include a list with one element for each positioning system.
See template "cs_RequestAssistData_UEB_GpsAndGlonass" as an example.

LPP_AssistanceDataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | LPP_AssistanceDataRequest_Type |  |
| Comment |  |  |
| LPP_AssistanceData | bitstring | bitstring containing 3GPP TS 37.355 [10] type RequestAssistanceData-r9-IEs |

LPP_AssistanceDataProvide_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | LPP_AssistanceDataProvide_Type |  |
| Comment |  |  |
| LPP_AssistanceData | bitstring | bitstring containing 3GPP TS 37.355 [10] type ProvideAssistanceData-r9-IEs |

LPP_AssistanceDataRequestList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | LPP_AssistanceDataRequestList_Type |
| Comment |  |
| record length(1..tsc_MaxPosSystems) of LPP_AssistanceDataRequest_Type |  |

LPP_AssistanceDataList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | LPP_AssistanceDataList_Type |
| Comment |  |
| record length(1..tsc_MaxPosSystems) of LPP_AssistanceDataProvide_Type |  |

### D.1.2.3	PosSystem_Retrieve_Data_Common

AssistanceDataRetrieveRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | AssistanceDataRetrieveRequest_Type |  |
| Comment |  |  |
| LPP | LPP_AssistanceDataRequestList_Type | LTE/NR positioning |
| UTRA_GNSS | UTRAN_AssistanceDataRequestList_Type | Retrieve A-GNSS data list in the order of positioning system provided in PositioningSystemList |

AssistanceDataRetrieveResponse_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | AssistanceDataRetrieveResponse_Type |  |
| Comment |  |  |
| LPP | LPP_AssistanceDataList_Type | LTE/NR positioning |
| UTRA_AGNSS | UTRAN_AssistanceDataList_Type | Retrieve A-GNSS data list in the order of positioning system provided in PositioningSystemList |

## D.1.3	PosSystem_PowerControl

PowerControl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PowerControl_Type |  |
| Comment | V2X. The GNSS simulator is switched on/off without stopping the UTC time sequence |  |
| PowerOn | Null_Type | By default the power is on in the GNSS simulator |
| PowerOff | Null_Type |  |

## D.1.4	System_Interface

Common ASP definitions

PosSystemCrtlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PosSystemCrtlRequest_Type |  |
| Comment |  |  |
| Start | Null_Type |  |
| Stop | Null_Type |  |
| LoadScenario | PositioningScenario_Type |  |
| RetrieveData | AssistanceDataRetrieveRequest_Type |  |
| TriggerMove | Null_Type | V2X. Start moving to the next area/zone in the scenario |
| TriggerPowerOnOff | PowerControl_Type | V2X. Switch on/off the transmitter without stopping UTC time sequence |
| SetAltitude | Altitude_Type | Aerial Set the height above the sea level to the provided value |
| TriggerAerialMove | Altitude_Type | Aerial Start moving towards target height |

PosSystemCrtlConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PosSystemCrtlConfirm_Type |  |
| Comment | In general, to be sent after the configuration has been done |  |
| Start | Null_Type |  |
| Stop | Null_Type |  |
| LoadScenario | Null_Type |  |
| RetrieveData | AssistanceDataRetrieveResponse_Type |  |
| TriggerMove | Null_Type | V2X |
| TriggerPowerOnOff | Null_Type | V2X |
| SetAltitude | Null_Type | Aerial |
| TriggerAerialMove | Null_Type | Aerial |

POS_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | POS_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| PositioningSystemList | PositioningSystemList_Type |  | The list of positioning systems to be configured; there is only one system of one type in the list e.g. gps, galileo etc. |
| CnfFlag | boolean |  | true => SS shall send CNF |
| Request | PosSystemCrtlRequest_Type |  |  |

POS_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | POS_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| PositioningSystemList | PositioningSystemList_Type |  |  |
| Confirm | PosSystemCrtlConfirm_Type |  |  |

POS_MTC_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | POS_MTC_SYSTEM_PORT |  |
| Comment | Positioning: port at the MTC to be mapped to the system |  |
| out | POS_SYSTEM_CTRL_REQ |  |
| in | POS_SYSTEM_CTRL_CNF |  |

POS_MTC_PTC_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | POS_MTC_PTC_PORT |  |
| Comment | Positioning: port at the MTC to hand over information from the positioning system to the PTC(s) |  |
| out | POS_SYSTEM_CTRL_CNF |  |
| in | POS_SYSTEM_CTRL_REQ |  |

POS_PTC_MTC_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | POS_PTC_MTC_PORT |  |
| Comment | Positioning: port at the PTC(s) to send commands to the positioning system via the MTC |  |
| out | POS_SYSTEM_CTRL_REQ |  |
| in | POS_SYSTEM_CTRL_CNF |  |

# D.2	CommonDefs

CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| Null_Type | boolean (true) | dummy type for 'typeless' fields in unions |

# D.4	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| PosSystem_ASP_TypeDefs | POS/PosSystem_ASP_TypeDefs.ttcn | Rev 30522 |
| CommonDefs | Common/CommonDefs.ttcn | Rev 30485 |
| CommonAspDefs | Common/CommonAspDefs.ttcn | Rev 30512 |

<end of modified section>