3GPP TSG-RAN WG5 Meeting #91-e	R5-212303

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0143 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Default Message Content SIP 4xx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Warning header in SIP 403 and SIP 480 is not according to the BNF in RFC 3261 - It has not been clear whether SIP 403 is specified to be sent by the UE or the SS - Several headers have been missing in Table 5.5.2.19.1-1 which need to sent by the SS |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Warning header corrected in Table 5.5.2.19.1-1 and Table 5.5.2.19.4-1 - Clarification added to 5.5.2.19.1, that SIP 403 sent by the SS - Missing headers added to Table 5.5.2.19.1-1; editor's note removed from 5.5.2.19.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete message specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.19	SIP 4xx

##### 5.5.2.19.1	SIP 403 (Forbidden)

This message is sent by the SS.

Table 5.5.2.19.1-1: SIP 403 (Forbidden)

| Delivery Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "403" |  |  |  |
| Reason-Phrase | "Forbidden" |  |  |  |
| Via | same as received in the request |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as in the request |  |  |  |
| CSeq |  |  |  |  |
| value | same value as in the request |  |  |  |
| Warning |  |  | RFC 3261 [22] |  |
| warn-code[1] | "100" |  |  |  |
| warn-agent[1] |  | name or pseudonym of the server adding the Warning header |  |  |
| pseudonym | "MCX Server" |  |  |  |
| warn-text[1] | "function not allowed due to" <detailed reason> |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" |  |  |  |

##### 5.5.2.19.2	SIP 404 (Not Found)

Table 5.5.2.19.2-1: SIP 404 (Not Found)

| Delivery Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "404" |  |  |  |
| Reason-Phrase | "Not Found" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.19.2-1 needs to be reviewed

##### 5.5.2.19.3	SIP 423 (Interval Too Brief)

Table 5.5.2.19.3-1: SIP 423 (Interval Too Brief)

| Delivery Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "423" |  |  |  |
| Reason-Phrase | "Internal Too Brief" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.19.3-1 needs to be reviewed

##### 5.5.2.19.4	SIP 480 (Temporarily unavailable)

This message is sent by the UE.

Table 5.5.2.19.4-1: SIP 480 (Temporarily unavailable)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "480" |  |  |  |
| Reason-Phrase | "Temporarily Unavailable" |  |  |  |
| Via | same as received in request message |  | RFC 3261 [22] RFC 3581 [55] |  |
| From |  |  |  |  |
| addr-spec | same value as received in INVITE message |  |  |  |
| tag | same value as received in request message |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in request message |  |  |  |
| tag | same value as received in the INVITE or any value if missing in the INVITE. |  |  |  |
| Warning |  |  | RFC 3261 [22] |  |
| warn-code[1] | "110" |  |  |  |
| warn-agent[1] | any value |  |  |  |
| warn-text[1] | "user declined the call invitation" |  |  |  |
| Call-ID | same value as received in request message |  |  |  |
| CSeq | same value as received in request message |  |  |  |
| Content Length | if present |  |  |  |
| value | "0" | No message body included |  |  |

##### 5.5.2.19.5	SIP 486 (Busy Here)

Table 5.5.2.19.5-1: SIP 486 (Busy Here)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "486" |  |  |  |
| Reason-Phrase | "Busy Here" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.18.5-1 needs to be reviewed

##### 5.5.2.19.6	SIP 488 (Not Acceptable Here)

Table 5.5.2.19.6-1: SIP 488 (Not Acceptable Here)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "488" |  |  |  |
| Reason-Phrase | "Not Acceptable Here" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | "0" | No message body included - end of SIP message |  |  |

Editor’s note: Table 5.5.2.19.6-1 needs to be reviewed

##### 5.5.2.19.7	SIP 401 (Unauthorized)

Table 5.5.2.19.7-1: SIP 401 (Unauthorized)

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  | RFC 3261 [22] |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "401" |  |  |  |
| Reason-Phrase | "Unauthorized" |  |  |  |
| Via | Same value as received in the REGISTER message |  | RFC 3261 [22] |  |
| To |  |  | RFC 3261 [22] |  |
| addr-spec | Same value as received in the REGISTER message |  |  |  |
| tag | To-tag assigned by the SS |  |  |  |
| From | Same value as received in the REGISTER message |  | RFC 3261 [22] |  |
| Call-ID | Same value as received in the REGISTER message |  | RFC 3261 [22] |  |
| CSeq | Same value as received in the REGISTER message |  | RFC 3261 [22] |  |
| WWW-Authenticate |  |  | RFC 2617 [72] RFC 3310 [96] |  |
| Realm | px_MCX_DomainName_Organization_A |  |  |  |
| algorithm | "AKAv1-MD5" |  |  |  |
| qop-value | "auth" |  |  |  |
| nonce | Base 64 encoding of RAND and AUTN |  |  |  |
| opaque | arbitrary value (to be returned by the UE in subsequent REGISTER) |  |  |  |
| Security-Server |  |  | RFC 3329 [50] |  |
| mechanism-name | "ipsec-3gpp" |  |  |  |
| algorithm[1] | px_IpSecAlgorithm (hmac-md5-96 or hmac-sha-1-96) |  |  |  |
| spi-c[1] | SPI number of the inbound SA at the protected client port |  |  |  |
| spi-s[1] | SPI number of the inbound SA at the protected server port |  |  |  |
| port-c[1] | protected client port of SS |  |  |  |
| port-s[1] | protected server port of SS |  |  |  |
| Encrypt-algorithm[1] | des-ede3-cbc or aes-cbc |  |  |  |
| q[1] | "0.9" |  |  |  |
| mechanism-name[2] | "Ipsec-3gpp" |  |  |  |
| algorithm[2] | Algorithm not selected by px_IpSecAlgorithm (hmac-sha-1-96 or hmac-md5-96) |  |  |  |
| spi-c[2] | SPI number of the inbound SA at the protected client port |  |  |  |
| spi-s[2] | SPI number of the inbound SA at the protected server port |  |  |  |
| port-c[2] | protected client port of SS |  |  |  |
| port-s[2] | protected server port of SS |  |  |  |
| encrypt-algorithm[2] | des-ede3-cbc or aes-cbc |  |  |  |
| q[2] | "0.7" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | “0” |  |  |  |