3GPP TSG-RAN WG5 Meeting #91-e	R5-212331

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0172 | rev | - | Current version: | 14.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MCPTT Test Case 6.2.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Procedure 5.3.7 called with incorrect option |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct option in procedure 5.3.7 applied |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading procedures and message contents may result in incorrect MCPTT test case implementation, test case cannot be validated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.14	On-network / Private Call / Ambient listening call / Remotely initiated Ambient listening call / Remotely initiated ambient listening call release / Success / Client Originated (CO) / Server initiated ambient call release

6.2.14.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorized for MCPTT Service, including authorized to initiate Remotely initiated ambient listening }

ensure that {

when { UE (MCPTT User) requests the establishment of a remotely initiated ambient listening call }

then { UE (MCPTT Client) sends a SIP INVITE message (end-to-end security context provided) requesting the establishment of a remotely initiated ambient listening call, and, after indication from the MCPTT Server that the call was established notifies the MCPTT user, and does not do any floor request }

}

(2)

with { UE (MCPTT Client) having initiated a remotely initiated ambient listening call, and, having been notified by the server that the MCPTT server is capable of receiving a SIP BYE from an MCPTT client to release an ambient-listening call }

ensure that {

when { UE (MCPTT User) wants to release the MCPTT remotely initiated ambient listening call }

then { UE (MCPTT Client) sends a SIP BYE request and after receiving a SIP 200 (OK) terminates the call }

}

(3)

with { UE (MCPTT Client) having initiated a remotely initiated ambient listening call }

ensure that {

when { UE (MCPTT client) receives a SIP BYE message including a <release-reason> element set to "administrator-action" }

then { UE (MCPTT Client) sends a SIP 200 (OK) response and terminates the call }

}

6.2.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 11.1.6.2.1.1, 11.1.6.2.1.3, 11.1.6.2.1.4, 6.2.1, 6.4, F.1.3. Unless otherwise stated these are Rel-14 requirements.

[TS 24.379, clause 11.1.6.2.1.1]

Upon receiving a request from an MCPTT user to establish an MCPTT ambient listening call that has been authorised successfully by the requesting MCPTT client, the MCPTT client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	shall set the Request-URI of the SIP INVITE request to a public service identity of the participating MCPTT function serving the MCPTT user;

2)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

3)	shall include the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

4)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

5)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

6)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref contain with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

7)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <session-type> element set to a value of "ambient-listening";

8)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <ambient-listening-type> element set to a value of:

...

b)	"remote-init", if the MCPTT user has requested a remotely initiated ambient listening call;

9)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCPTT ID of the targeted MCPTT user, according to rules and procedures of IETF RFC 5366 [20];

NOTE 1:	the targeted MCPTT user is the listened-to MCPTT user in the case of a remotely initiated ambient listening call or the listening MCPTT user in the case of a locally initiated listening call.

10)	if an end-to-end security context needs to be established then:

a)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [78];

b)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [78];

c)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect private call communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [78];

d)	shall encrypt the PCK to a UID associated to the MCPTT client using the MCPTT ID and KMS URI of the invited user and a time related parameter as described in 3GPP TS 33.180 [78];

e)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [78];

f)	shall add the MCPTT ID of the originating MCPTT to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [78]; and

g)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCPTT user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [78];

11)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarification given in subclause 6.2.1 and with a media stream of the offered media-floor control entity;

...

13)	if this is a remotely initiated ambient listening call, shall comply with the conditions for an implicit request to grant the floor to the terminating MCPTT client as specified in subclause 6.4;

14) shall include in the SIP INVITE request a Priv-Answer-Mode header field with the value "Auto" according to the rules and procedures of IETF RFC 5373 [18]; and

15)	shall send the SIP INVITE request towards the participating MCPTT function according to 3GPP TS 24.229 [4].

...

Upon receiving a SIP 200 (OK) response to the SIP INVITE request the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	if this is a remotely initiated ambient listening call, shall notify the user that the call has been successfully established;

[TS 24.379, clause 11.1.6.2.1.3]

Upon receiving a request from an MCPTT user to release an MCPTT ambient listening call:

The MCPTT client:

...

2)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

3)	shall generate a SIP BYE request according to rules and procedures of 3GPP TS 24.229 [4] and IETF RFC 6086 [64]; and

4)	shall send the SIP BYE request within the dialog of the MCPTT ambient call session as specified in 3GPP TS 24.229 [4].

Upon receipt of the SIP 200 (OK) response to the SIP BYE request the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

[TS 24.379, clause 11.1.6.2.1.4]

Upon receipt of a SIP BYE request in the dialog of an ambient listening session, the MCPTT client:

1)	shall comply with the procedures of subclause 6.2.6;

[TS 24.379, clause 6.2.1]

The SDP offer shall contain only one SDP media-level section for MCPTT speech according to 3GPP TS 24.229 [4] and, if floor control shall be used during the session, shall contain one SDP media-level section for a media-floor control entity according to 3GPP TS 24.380 [5].

When composing an SDP offer according to 3GPP TS 24.229 [4] the MCPTT client:

...

2)	shall include an "m=audio" media-level section for the MCPTT media stream consisting of:

...

c)	if the SDP offer is for an ambient listening call:

i)	if this is a remotely initiated ambient listening call, include an "a=recvonly" attribute; or

[TS 24.379, clause 6.4]

An initial SIP INVITE request fulfilling the following criteria shall be regarded by the MCPTT server as an implicit request to grant the floor to the terminating MCPTT client when the originating MCPTT client:

1)	initiates a remotely initiated MCPTT ambient listening call; and

2)	includes the "mc_implicit_request" 'fmtp' attribute in the associated UDP stream for the floor control in the SDP offer/answer as specified in 3GPP TS 24.380 [5] clause 12.

[TS 24.379, clause F.1.3]

If the <mcpttinfo> contains the <mcptt-Params> element then:

...

2)	the <session-type> can be included with:

...

e)	a value of "ambient-listening" to indicate the MCPTT client wants to make an ambient listening call;

...

19)	the <anyExt> can be included with the following elements not declared in the XML schema:

a)	an <ambient-listening-type> of type "xs:string":

i)	set to a value of "remote-init" when the listening MCPTT user of an ambient listening call initiates the call; or

6.2.14.3	Test description

6.2.14.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlining "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The MCPTT User is authorized to initiate Remotely initiated ambient listening: the <allow-request-remote-initiated-ambient-listening> element of the <ruleset> element is present in the MCPTT user profile document and is set to "true"

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.2.14.3.2	Test procedure sequence

Table 6.2.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT User) request the establishment of an MCPTT remotely initiated ambient listening call. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) perform the Generic Test Procedure for MCPTT CO establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.7.3-1 for) the establishment of an MCPTT remotely initiated ambient listening call? Option a in TS 36.579-1 [2] Table 5.3.7.3-1 is used. | - | - | 1 | P |
| 3 | Void | - | - | - | - |
| 4 | The SS (MCPTT server) sends a Floor Taken message, the Permission to Request the Floor field set to '0'. | <-- | Floor Taken | - | - |
| 5 | Check: Does the UE (MCPTT client) notify the user that the remotely initiated ambient listening call has been successfully established? (NOTE 1) | - | - | 1 | P |
| 6 | Make the UE (MCPTT User) request to speak (e.g. pressing the PTT button) (NOTE 1) | - | - | - | - |
| 6A | Check: Does the UE (MCPTT client) send a Floor Request message in the next 5 sec? | --> | Floor Request | 1 | F |
| 7 | Make the UE (MCPTT User) request the release of the Remotely initiated ambient listening call. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT CO call release as described in TS 36.579-1 [2] Table 5.3.10.3-1 to end the call? | - | - | 2 | P |
| 9-10 | Void | - | - | - | - |
| 11 | Make the UE (MCPTT User) request the establishment of an MCPTT remotely initiated ambient listening call. (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCPTT client) perform the Generic Test Procedure for MCPTT CO establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.7.3-1 for the establishment of an MCPTT remotely initiated ambient listening call? Option a in TS 36.579-1 [2] Table 5.3.7.3-1 is used. |  | - | 1 | P |
| 13 | Void | - | - | - | - |
| 14 | The SS (MCPTT server) sends a Floor Taken message, the Permission to Request the Floor field set to '0'. | <-- | Floor Taken | - | - |
| 15 | Void | - | - | - | - |
| 16 | Check: Is the Generic Test Procedure for MCPTT CT call release as described in TS 36.579-1 [2] Table 5.3.12.3-1 to end the call correctly performed? | - | - | 3 | P |
| 17-18 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.2.14.3.3	Specific message contents

Table 6.2.14.3.3-1: SIP INVITE (Steps 2, 12, Table 6.2.14.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.7.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.5.1-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Answer-Mode | not present |  |  |  |
| Priv-Answer-Mode |  |  |  |  |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-param | “require” |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME-part-body | SDP Message as described in Table 6.2.14.3.3-3 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.2.14.3.3-2 |  |  |  |

Table 6.2.14.3.3-2: MCPTT-Info in SIP INVITE (Table 6.2.14.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.2.1-1, condition PRIVATE-CALL, INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| session-type | "ambient-listening" |  |  |  |
| anyExt |  |  |  |  |
| ambient-listening-type | "remote-init" | The anyExt field may contain other values - these need not be checked |  |  |

Table 6.2.14.3.3-3: SDP Message in SIP INVITE (Table 6.2.14.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.1.1-1, conditions PRIVATE-CALL, INITIAL_SDP_OFFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| media attribute |  | a=line attribute=recvonly |  |  |
| recvonly |  |  |  |  |

Table 6.2.14.3.3-4: SIP 200 (OK) (Steps 2, 12, Table 6.2.14.3.2-1; step 4, TS 36.579-1 [2] Table 5.3.7.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.17.1.2-1 condition INVITE_RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Feature-Caps |  |  |  |  |
| fcap-name | "g.3gpp.mcptt.ambient-listening-call-release" | Indicates that the MCPTT server is capable of receiving a SIP BYE from an MCPTT client to release an ambient-listening call |  |  |
| Message-body |  |  |  |  |
| SDP message | SDP message as described in Table 6.2.14.3.3-4A |  |  |  |

Table 6.2.14.3.3-4A: SDP Message in SIP 200 OK (Table 6.2.14.3.3-4)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.1.1-1, conditions PRIVATE-CALL, SDP_ANSWER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| media attribute |  | a=line attribute=sendonly |  |  |
| sendonly |  |  |  |  |

Table 6.2.14.3.3-5: Floor Taken (Steps 4, 14, Table 6.2.14.3.2-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.7-1 condition ON-NETWORK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Permission to Request the Floor |  |  |  |
| Permission to Request the Floor | "0" | The receiver is NOT permitted to request floor |  |

Table 6.2.14.3.3-6: Void

Table 6.2.14.3.3-7: SIP BYE (Step 16, Table 6.2.14.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.12.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.2.14.3.3-8 |  |  |  |

Table 6.2.14.3.3-8: MCPTT-Info in SIP BYE (Table 6.2.14.3.3-7)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| anyExt |  |  |  |  |
| release-reason | "administrator-action" |  |  |  |