3GPP TSG-RAN5 Meeting #91-e	R5-212367

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0015 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MCVideo Test Case 6.1.1.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Corrections needed to meet PRD13 requirements and changes in 36.579-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test Purpose Section 6.1.1.11.1 - #1 Edited with statement - #2, #3, #4 Edited with and then statements - Conformance Requirements Section 6.1.1.11.2 - Changed spec 9.2.1 in list to 9.2.1 2.1.2 in 1st paragraph - Deleted spec 9.2.1.2.1.3 from list in 1st paragraph - Deleted excerpt of spec 6.2.3.2.2 - Changed Rel 14 to Rel 15 in 1st paragraph - Deleted spec 6.2.3.2.2 snip - Test Sequence Main Behavior Table 6.1.1.11.3.2-1 - 1st Exception changed “which” to “that” - Step 1 deleted “manual commencement mode and” in Procedure column - 2nd Exception added “(MCVideo Client) twice after each “UE” in Procedure Column - Steps 3, 7, 8, 11, 14 deleted full text for Note 1 and added “(NOTE 1)” - Last Exception add “the” in Procedure column - Added last row with full text for Note 1 - Specific Message Contents 6.1.1.11.3.3 - Voided Tables 6.1.1.11.3.3-5 and 6.1.1.11.3.3-6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MCVideo client will not be properly tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <START OF CHANGES>

#### 6.1.1.11	On-network / On-demand Pre-arranged Group Call / Imminent Peril Group Call / Client Terminated (CT)

##### 6.1.1.11.1	Test Purpose (TP)

(1)

with { the UE (MCVideo Client) registered and authorized for MCVideo Service }

ensure that {

when { the MCVideo Client receives a SIP INVITE message of an MCVideo On-demand Pre-arranged Imminent Peril Group Call and the MCVideo User answers the call }

then { the MCVideo Client responds to the SS (MCVideo Server) with a SIP 200 (OK) message }

}

(2)

with { the UE (MCVideo Client) having an ongoing MCVideo Pre-arranged Imminent Peril Group Call }

ensure that {

when { the UE (MCVideo Client) receives a Media Transmission Notification message from the SS (MCVideo Server) }

then { the UE (MCVideo Client) provides media transmission notification to the MCVideo User and sends a Receive Media Request message to the SS (MCVideo Server) }

(3)

with { the UE (MCVideo Client) having an ongoing MCVideo On-network Pre-arranged Imminent Peril Group Call }

ensure that {

when { the MCVideo User requests to end the RTP media reception }

then { the UE (MCVideo Client) sends a Media Reception End Request message }

}

(4)

with { the UE (MCVideo Client) having an ongoing MCVideo On-demand Pre-arranged Imminent Peril Group Call }

ensure that {

when { the MCVideo User requests to terminate the On-demand Pre-arranged Imminent Peril Group Call }

then { the UE (MCVideo Client) sends a SIP BYE message and leaves the MCVideo Session }

}

##### 6.1.1.11.2	Conformance requirements

References: The conformance requirements covered in the current Test Case are specified in TS 24.281, clauses 9.2.1.2.1.2, TS 24.581 clauses 6.2.5.2.2, 6.2.5.3.2, 6.2.5.3.3, 6.2.5.4.5, 6.2.5.5.3, 6.2.5.6.4. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-15 requirements.

[TS 24.281, clause 9.2.1.2.1.2]

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcvideo-info+xml MIME body.

Upon receipt of an initial SIP INVITE request, the MCVideo client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [11] with the clarifications below.

The MCVideo client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCVideo client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCVideo function either with appropriate reject code as specified in 3GPP TS 24.229 [51] and warning texts as specified in subclause 4.4.2 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

NOTE:	If the SIP INVITE request contains an emergency indication or imminent peril indication, the MCVideo client can by means beyond the scope of this specification choose to accept the request.

3)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [11];

4)	if the SIP INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication that this is a SIP INVITE request for an MCVideo emergency group call and:

i)	should display the MCVideo ID of the originator of the MCVideo emergency group call contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body;

ii)	should display the MCVideo group identity of the group with the emergency condition contained in the <mcvideo-calling-group-id> element; and

iii)	if the <alert-ind> element is set to "true", should display to the MCVideo user an indication of the MCVideo emergency alert and associated information;

b)	shall set the MCVideo emergency group state to "MVEG 2: in-progress";

c)	shall set the MCVideo imminent peril group state to "MVIG 1: no-imminent-peril"; and

d)	shall set the MCVideo imminent peril group call state to "MVIGC 1: imminent-peril-gc-capable"; otherwise

5)	if the SIP INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication that this is a SIP INVITE request for an MCVideo imminent peril group call and:

i)	should display the MCVideo ID of the originator of the MCVideo imminent peril group call contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	should display the MCVideo group identity of the group with the imminent peril condition contained in the <mcvideo-calling-group-id> element; and

b)	shall set the MCVideo imminent peril group state to "MVIG 2: in-progress";

6)	may display to the MCVideo user the MCVideo ID of the inviting MCVideo user;

7)	shall perform the automatic commencement procedures specified in subclause 6.2.3.1.2 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCVideo service setting at the invited MCVideo client for answering the call is set to automatic commencement mode; or

b)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCVideo service setting at the invited MCVideo client for answering the call is set to manual commencement mode, yet the invited MCVideo client allows the call to be answered with automatic commencement mode;

8)	shall perform the manual commencement procedures specified in subclause 6.2.3.2.2 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCVideo service setting at the invited MCVideo client for answering the call is to use manual commencement mode; or

b)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCVideo service setting at the invited MCVideo client for answering the call is set to automatic commencement mode, yet the invited MCVideo client allows the call to be answered with manual commencement mode; and

9)	when the SIP 200 (OK) response to the SIP INVITE request is sent, may subscribe to the conference event package as specified in subclause 9.1.3.1.

[TS 24.581, clause 6.2.5.2.2]

When an MCVideo call is established, the terminating transmission participant:

1.	shall create an instance of a 'Transmission participant state transition diagram for general reception control operation'; and

2.	shall enter the 'U: reception controller' state.

NOTE:	From a transmission participant perspective the MCVideo call is established when the application and signalling plane sends the SIP 200 (OK) response.

[TS 24.581, clause 6.2.5.3.2]

Upon receiving the media transmission notification from the transmission control server, the transmission participant:

1.	if the first bit in the subtype of the media transmission notification message is set to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '6' (Media transmission notification); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	shall provide media transmission notification to the user;

3.	shall store the User ID and the SSRC of the user transmitting the media;

4.	may display the details of the incoming media to the user; and

5.	shall remain in the 'U: reception controller' state.

[TS 24.581, clause 6.2.5.3.3]

Upon receiving an indication from the user to request permission to receive media, the transmission participant:

1.	shall send the Receive Media Request message toward the transmission control server; The Receive Media Request message:

a.	if a different priority than the normal priority is required, shall include the Reception Priority field with the priority not higher than negotiated with the transmission control server as specified in subclause 14.3.3; and

b.	if the receive media request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Transmission Indicator field indicating the relevant call types;

2.	shall create an instance of the 'Transmission participant state transition diagram for basic reception control operation';

3.	shall map the stored User ID and the SSRC of the user transmitting the media with instance of 'Transmission participant state transition diagram for basic reception control operation' created in step 2; and

4.	shall remain in the 'U: reception controller' state.

[TS 24.581, clause 6.2.5.4.5]

Upon receiving a granted response for Receive media request message, the transmission participant:

1.	if the first bit in the subtype of the Receive media response message is set to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '7' (Receive media response); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	shall provide receive media success notification to the user;

3.	if the Receive Media Indicator field is included and the B-bit is set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	shall stop timer T103 (Receive Media Request); and

5.	shall enter the 'U: has permission to receive' state.

[TS 24.581, clause 6.2.5.5.3]

Upon receiving an indication from the user to end the RTP media reception, the transmission participant:

1.	shall send a Media reception end request message towards the transmission control server The Media reception end request message:

a.	if the session is a broadcast call and if the session was established as a normal call, shall include the Transmission Indicator with the A-bit set to '1' (Normal call); and

b.	shall include the SSRC of user transmitting the media in the Media reception end request message;

2.	shall remove the indication that the participant is overriding without revoke if this indication is stored;

3.	shall remove the indication that the participant is overridden without revoke if this indication is stored;

4.	shall start timer T104 (Receive Media Release) and initialize counter C104 (Receive Media Release) to 1; and

5.	shall enter the 'U: pending reception release' state.

[TS 24.581, clause 6.2.5.6.4]

Upon receiving a MRE response message, the transmission participant:

1.	if the first bit in the subtype of the MRE response message to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '3' (Media reception end response); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	may provide a Media reception end notification to the MCVideo user;

3.	if the Receive Media Indicator field is included and the B-bit set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	shall stop timer T104 (Receive Media Release); and

5.	shall enter the 'terminated' state.

##### 6.1.1.11.3	Test description

6.1.1.11.3.1	Pre-test conditions

System Simulator:

-	SS (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCVideo client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCPTT User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2A.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCVideo Client Application has been activated and User has registered-in as the MCVideo User with the Server as active user at the Client.

6.1.1.11.3.2	Test procedure sequence

Table 6.1.1.11.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The E-UTRA/EPC actions that are related to the MCVIDEO call establishment are described in TS 36.579-1 [2], subclause 5.4.4A 'Generic Test Procedure for MCVideo CT communication in E-UTRA'. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 1 | The SS (MCVideo Server) initiates an on-demand Imminent Peril group call with implicit Transmission Control. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Steps 2a1 through 2a4 describe behaviour that depends on the UE (MCVideo Client) implementation; the "lower case letter" identifies a step sequence that take place if the UE (MCVideo Client) responds to a SIP INVITE prior to the MCVIDEO user's acknowledgment. | - | - | - | - |
| 2a1 | (Optional) The UE (MCVideo Client) responds with a SIP 100 Trying provisional response. | --> | SIP 100 (Trying) | - | - |
| 2a2 | (Optional) The UE (MCVideo Client) respond with a SIP 183 (Session Progress) message? | --> | SIP 183 (Session Progress) | - | - |
| 2a3 | The SS (MCVideo Server) responds to the SIP 183 (Session Progress) message with a SIP PRACK message. | <-- | SIP PRACK | - | - |
| 2a4 | The UE (MCVideo Client) acknowledges the SIP PRACK message with SIP 200 (OK) message. | --> | SIP 200 (OK) | - | - |
| 3 | Make the MCVideo User answer the call. (NOTE 1) | - | - | - | - |
| 4 | Check: Does the UE (MCVideo Client) answer the call with a SIP 200 (OK)? | --> | SIP 200 (OK) | 1 | P |
| 5 | The SS (MCVideo Server) acknowledges the receipt of a SIP 200 (OK) for the SIP INVITE message. | <-- | SIP ACK | - | - |
| 6 | The SS (MCVideo Server) sends a Media Transmission Notification message. | <-- | Media Transmission Notification | - | - |
| 7 | Check: Does the UE (MCVideo Client) provide media transmission notification to the MCVideo User? | - | - | 2 | P |
| 8 | Make the MCVideo User request permission to receive media. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCVideo Client) send a Receive Media Request message? | --> | Receive Media Request | 2 | P |
| 10 | The SS (MCVideo Server) sends a Receive Media Response message. | <-- | Receive Media Response | - | - |
| 11 | Make the MCVideo User request to end the RTP media reception. (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCVideo Client) send a Media Reception End Request to indicate that the user wishes to end the RTP media reception? | --> | Media Reception End Request | 3 | P |
| 13 | The SS (MCVideo Server) responds with a Media Reception End Response message. | <-- | Media Reception End Response | - | - |
| 14 | Make the MCVideo User end the call. (NOTE 1) | - | - | - | - |
| 15 | Check: Does the UE (MCVideo Client) send a SIP BYE message to leave the MCVideo session? | --> | SIP BYE | 4 | P |
| 16 | The SS (MCVideo Server) sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| - | EXCEPTION: The SS (MCVideo Server) releases the E-UTRA connection. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.1.11.3.3	Specific message contents

Table 6.1.1.11.3.3-1: SIP INVITE (Step 1, Table 6.1.1.11.3.2-1)

| Derivation Path: TS 56.379-1 [2], Table 5.5.2.5.2-1, conditions GROUP-CALL, IMMPERIL-CALL, MANUAL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.1.1.11.3.3-2 |  |  |  |

Table 6.1.1.11.3.3-2: MCVideo-INFO in SIP INVITE (Table 6.1.1.11.3.3-1)

Derivation Path: TS 56.379-1 [2], Table 5.5.3.2.2-2, conditions GROUP-CALL, IMMPERIL-CALL

Table 6.1.1.11.3.3-3: SIP 183 (Session Progress) (Step 2a2, Table 6.1.1.11.3.2-1)

Derivation Path: TS 56.379-1 [2], Table 5.5.2.16.3.1-1, conditions MCVIDEO

Table 6.1.1.11.3.3-4: SIP 200 (OK) (Step 2a4, Table 6.1.1.11.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition MCVIDEO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | not present |  |  |  |
| Content-Length |  |  |  |  |
| value | '0' | No message body included |  |  |

Table 6.1.1.11.3.3-5: Void

Table 6.1.1.11.3.3-6: Void

Table 6.1.1.11.3.3-7: Receive Media Request (Step 9, Table 6.1.1.6.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.11.1.4-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Transmission Indicator |  |  |  |  |
| Transmission Indicator | '0000100000000000' | E = Imminent Peril |  |  |

Table 6.1.1.11.3.3-8: Receive Media Response (Step 10, Table 6.1.1.6.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.8-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Transmission Indicator |  |  |  |  |
| Transmission Indicator | '0000100000000000' | E = Imminent Peril |  |  |

Table 6.1.1.11.3.3-9: Media Reception End Request (Step 12, Table 6.1.1.6.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.11.3.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Transmission Indicator |  |  |  |  |
| Transmission Indicator | '0000100000000000' | E = Imminent Peril |  |  |

Table 6.1.1.11.3.3-10: SIP BYE (Step 13 Table 6.1.1.11.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.2.2-1, condition MO-CALL

Table 6.1.1.11.3.3-11: SIP 200 (OK) (Step 16, Table 6.1.1.11.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition MCVIDEO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | not present |  |  |  |
| Content-Length |  |  |  |  |
| value | '0' | No message body included |  |  |

### <END OF CHANGES>