3GPP TSG-RAN5 Meeting #91-e	R5-212421

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2156 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC TC 8.2.2.3.1-SRB3 and Split SRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, Step1 reconfig SRB1 to split SRB1, and SRB2 to change the PDCP type from EUTRA PDCP to NR PDCP.Therefore,SRB1 is expected to be split SRB1 with Logical channel in both NR and LTE, while SRB2 is expected to be MCG SRB with Logcial channel in LTE and PDCP in NR. It is not need to config Logcial channe in NR for SRB2 in Table 8.2.2.3.1.3.3-1B. [37.340] NOTE 1: Even if only SCG bearers are configured for a UE, for SRB1 and SRB2 the logical channels are always configured at least in the MCG, i.e. this is still an MR-DC configuration and a PCell always exists. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Delete RLC-BearerConfig for SRB2 in Table 8.2.2.3.1.3.3-1B, which confige RLC in NR side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.2.2.3.1	Simultaneous SRB3 and Split SRB / Sequential message flow on SRB3 and Split SRB with one UL path / EN-DC

8.2.2.3.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state in EN-DC mode with SRB3 and split SRB1 configured with no PDCP duplication}

ensure that {

when { UE receives RRCReconfiguration message on SRB3 to modify SCG DRB }

then { UE sends RRCReconfigurationComplete message on SRB3 over the SCG path }

}

(2)

with { UE in RRC_CONNECTED state in EN-DC mode with SRB3 and split SRB1 configured with no  PDCP  duplication }

ensure that {

when { UE receives an RRCConnectionReconfiguration message on split SRB1 on the SCG path to release SRB3 }

then { UE releases SRB3 and sends an RRCConnectionReconfigurationComplete message on split SRB1 over the MCG path }

}

(3)

with { UE in RRC_CONNECTED state in EN-DC mode with split SRB1 configured with no PDCP duplication}

ensure that {

when { UE receives an RRCConnectionReconfiguration message to release split SRB1 on SCG path }

then { UE releases split SRB1 and sends an RRCConnectionReconfigurationComplete message on SRB1 over the MCG path }

}

8.2.2.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 37.340, clause 7.6, TS 36.331, clauses 5.3.5.3 and TS 38.331:5.3.5.3, 5.3.5.5, 5.3.5.5.3, 5.3.5.6, 5.3.5.6.2 and 5.3.5.6.3. Unless and otherwise stated these are Rel-15 requirements.

[TS 37.340, clause 7.6]

Split SRB is supported for both SRB1 and SRB2 (split SRB is not supported for SRB0 and SRB3) in all MR-DC cases. RRC PDUs on split SRB are ciphered and integrity protected using NR PDCP.

Split SRB can be configured by the MN in Secondary Node Addition and/or Modification procedure, with SN configuration part provided by the SN. A UE can be configured with both split SRB and SRB3 simultaneously. SRB3 and the SCG leg of split SRB can be independently configured.

For the split SRB, the selection of transmission path in downlink depends on network implementation. For uplink, the UE is configured via MN RRC signalling whether to use MCG path or duplicate the transmission on both MCG and SCG.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the nr-SecondaryCellGroupConfig:

2>	perform NR RRC Reconfiguration as specified in TS 38.331 [82], clause 5.3.5.3;

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

…

2>	if the received RRCConnectionReconfiguration message included nr-SecondaryCellGroupConfig:

3>	include scg-ConfigResponseNR in accordance with TS 38.331 [82], clause 5.3.5.3;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

...

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration message contains the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

...

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (MCG is E-UTRA):

2>	if RRCReconfiguration was received via SRB1:

3>	submit the RRCReconfigurationComplete via the EUTRA MCG embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10].

[TS 38.331, clause 5.3.5.5.3]

The UE shall:

1>	for each logicalChannelIdentity value included in the rlc-BearerToReleaseList that is part of the current UE configuration (LCH release); or

1>	for each logicalChannelIdentity value that is to be released as the result of an SCG release according to 5.3.5.4:

2>	release the RLC entity or entities as specified in TS 38.322 [4] clause 5.1.3;

2>	release the corresponding logical channel.

[TS 38.331, clause 5.3.5.6.1

The UE shall perform the following actions based on a received RadioBearerConfig IE:

…

1>	if the RadioBearerConfig includes the srb-ToAddModList:

2>	perform the SRB addition or reconfiguration as specified in 5.3.5.6.3;

[TS 38.331, clause 5.3.5.6.3]

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP):

2>	establish a PDCP entity and configure it with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB/ KgNB) or secondary key (S-KgNB( as indicated in keyToUse, if applicable;

2>	if the current UE configuration as configured by E-UTRA in TS 36.331 includes an SRB identified with the same srb-Identity value:

3>	associate the E-UTRA RLC entity and DCCH of this SRB with the NR PDCP entity;

3>	release the E-UTRA PDCP entity of this SRB;

2>	if the pdcp-Config is included:

3>	configure the PDCP entity in accordance with the received pdcp-Config;

2>	else:

3>	configure the PDCP entity in accordance with the default configuration defined in 9.2.1 for the corresponding SRB;

8.2.2.3.1.3	Test description

8.2.2.3.1.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1 is the PCell and NR Cell 1 is the PSCell.

-	System Information combination as defined in TS 38.508-1 [4] clause 4.4.3.1.1 is used in E-UTRA Cell 1 and NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC),  bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4.

8.2.2.3.1.3.2	Test procedure sequence

Table 8.2.2.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits on SRB1 over MCG path/E-UTRA Cell 1 an RRCConnectionReconfiguration message to configure SRB3, to reconfigure SRB1 to split SRB1 without PDCP duplication and to reconfigure SRB2 to change the PDCP type from EUTRA PDCP to NR PDCP. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message on split SRB1 over the MCG path/EUTRA Cell 1. | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 3 | The SS transmits on SRB3/SCG an NR RRCReconfiguration message to reconfigure SCG DRB. | <-- | RRCReconfiguration | - | - |
| 4 | Check: Does the UE transmit an NR RRCReconfigurationComplete message on SRB3/NR Cell 1? | --> | RRCReconfigurationComplete | 1 | P |
| 5 | The SS transmits over split SRB1 SCG path/NR Cell 1 an RRCConnectionReconfiguration message to release SRB3. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |
| - | Void | - | - | - | - |
| 6 | Check: Does the UE transmits an RRCConnectionReconfigurationComplete message on split SRB1 over the MCG path/EUTRA Cell 1? | --> | RRCConnectionReconfigurationComplete(RRCReconfigurationComplete) | 2 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message to release SRB1 over SCG path/NR Cell 1. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |
| 8 | Check: Does the UE transmits an RRCConnectionReconfigurationComplete message on SRB1 over the MCG path/E-UTRA Cell 1? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | 3 | P |

Table 8.2.2.3.1.3.2-2: Void

8.2.2.3.1.3.3	Specific message contents

Table 8.2.2.3.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.3.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |  |  |  |
| mobilityControlInfo |  |  | MobilityControlInfo-HO-SameCell |  | As per Table 8.2.2.3.1.3.3-1E |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 |  |  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according table 8.2.2.3.1.3.3-1A |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| sk-Counter-r15 | Increment the value by 1 from the previous value |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 |  |  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [4], Table 4.6.3-132 with conditions EN-DC_DRB and Re-establish_PDCP and SRB3 |  |  |  |  |
| nr-RadioBearerConfig2-r15 |  |  | RadioBearerConfig according to Table 8.2.2.3.1.3.3-1C |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.2.2.3.1.3.3-1A: RRCReconfiguration (Table 8.2.2.3.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | OCTET STRING containing CellGroupConfig according to Table 8.2.2.3.1.3.3-1B. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-1B: CellGroupConfig (Table 8.2.2.3.1.3.3-1A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition EN-DC and PSCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | 1 |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig according to 38.508-1 [4], Table 4.6.3-148 with condition SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig according to 38.508-1 [4], Table 4.6.3-148 with condition SRB3 | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig according to 38.508-1 [4], Table 4.6.3-148 with conditions AM and DRB2 and Re-establish_RLC | entry 3 |  |
| } |  |  |  |
| mac-LogicalChannelConfig | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-1C: RadioBearerConfig (Table 8.2.2.3.1.3.3-1)

| Derivation Path: TS 38.508-1[4] table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { |  |  |  |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| srb-Identity | LogicalChannelIdentity with condition SRB1 |  |  |
| reestablishPDCP | true |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config according to table 8.2.2.3.1.3.3-1D |  |  |
| srb-Identity[2] | LogicalChannelIdentity with condition SRB2 |  |  |
| reestablishPDCP[2] | true |  |  |
| discardOnPDCP[2] | Not present |  |  |
| pdcp-Config[2] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-1D: PDCP-Config (Table 8.2.2.3.1.3.3-1C)

Derivation Path: TS 38.508-1[4] table 4.6.3-99 with condition Split_SRB

Table 8.2.2.3.1.3.3-1E: MobilityControlInfo-HO-SameCell (Table 8.2.2.3.1.3.3-1)

| Derivation Path: TS 36.508 [7], Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of E-UTRA Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-2: Void

Table 8.2.2.3.1.3.3-3: Void

Table 8.2.2.3.1.3.3-4: RRCReconfiguration (step 3, Table 8.2.2.3.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | OCTET STRING containing CellGroupConfigaccording to table 8.2.2.3.1.3.3-5. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-5: CellGroupConfig (Table 8.2.2.3.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | 1 |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig according to table 8.2.2.3.1.3.3-5a |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-5A: MAC-CellGroupConfig (Table 8.2.2.3.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| bsr-Config SEQUENCE { |  |  |  |
| periodicBSR-Timer | sf5 |  |  |
| retxBSR-Timer | sf40 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-6: RRCConnectionReconfiguration (step 5, Table 8.2.2.3.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING containing RRCReconfiguration according to Table 8.2.2.3.1.3.3-8. |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-RadioBearerConfig1-r15 | RadioBearerConfig according to Table 8.2.2.3.1.3.3-7. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-7: RadioBearerConfig (Table 8.2.2.3.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| srb3-ToRelease | true |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToReleaseList | Not present |  |  |
| securityConfig | Not present |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-8: RRCReconfiguration (Table 8.2.2.3.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | OCTET STRING containing CellGroupConfig according to table 8.2.2.1.1.3.3-9. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-9: CellGroupConfig (Table 8.2.2.3.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| cellGroupId | 1 |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1.. maxLC-ID)) OF LogicalChannelIdentity { | 1 entry |  |  |  |
| LogicalChannelIdentity[1] | 3 |  | entry 1 |  |
| } |  |  |  |  |
| mac-CellGroupConfig | Not present |  |  |  |
| physicalCellGroupConfig | Not present |  |  |  |
| sCellToAddModList | Not present |  |  |  |
| sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex { | 1 entry |  |  |  |
| SCellIndex[1] | SCellIndex | entry 1 NR Cell 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.2.3.1.3.3-10: RRCConnectionReconfiguration (step 7, Table 8.2.2.3.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 with condition HO-TO-EUTRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo |  | MobilityControlInfo-HO-SameCell | As per Table 8.2.2.3.1.3.3-1E |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { |  | 1 entry |  |  |
| SRB-ToAddMod[1] |  | SRB-ToAddMod-DEFAULT using condition SRB1 | entry 1 See TS 36.508 [7] subclause 4.8.2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING containing RRCReconfigurationaccording to Table 8.2.2.3.1.3.3-11 |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-RadioBearerConfig1-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-11: RRCReconfiguration (Table 8.2.2.3.1.3.2-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | OCTET STRING containing CellGroupConfig according to Table 8.2.2.3.1.3.3-12. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.3.1.3.3-12: CellGroupConfig (Table 8.2.2.3.1.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| cellGroupId | 1 |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1.. maxLC-ID)) OF LogicalChannelIdentity { | 1 entry |  |  |  |
| LogicalChannelIdentity[1] | 1 |  | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of modified section 1>