Title:	Discussion on Test Point analysis for FR2 EVM CA test cases

Source:		TTA

Introduction

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the Error Vector Magnitude CA test case is analyzed.

Discussion

In LTE specification [2], the test configuration for UL and DL intra-band contiguous CA is defined as table below.

Table 6.5.2A.1.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 2 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 3 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 4 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 5 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 6 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 7 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 8 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@25 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 11 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 14 | 50 | 25 |  | 16QAM | 25 | P_25@25 | S_0@0 | - | - |
| 15 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 16 | 50 | 25 |  | 16QAM | 8 | P_8@42 | S_0@0 | - | - |
| 17 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 18 | 25 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 25 | 50 |  | QPSK | 12 | P_12@13 | S_0@0 | - | - |
| 20 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 21 | 25 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 22 | 25 | 50 |  | 16QAM | 12 | P_12@13 | S_0@0 | - | - |
| 23 | 75 | 75 | N.A. | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 24 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 26 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 27 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 28 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 29 | 100 | 25 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 30 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 31 | 100 | 25 |  | QPSK | 25 | P_25@75 | S_0@0 | - | - |
| 320 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 33 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | - | - |
| 34 | 100 | 25 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 35 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 36 | 100 | 25 |  | 16QAM | 25 | P_25@75 | S_0@0 | - | - |
| 37 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 38 | 100 | 25 |  | 16QAM | 8 | P_8@92 | S_0@0 | - | - |
| 39 | 25 | 100 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 40 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 41 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 42 | 25 | 100 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 43 | 25 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 25 | 100 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 453 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 46 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 47 | 100 | 50 |  | QPSK | 50 | P_50@50 | S_0@0 | - | - |
| 48 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 49 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | - | - |
| 50 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | 16QAM | 50 | P_50@50 | S_0@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 12 | P_12@88 | S_0@0 | - | - |
| 55 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 56 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 57 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 58 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 59 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 60 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 61 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 75 | P_75@25 | S_0@0 | - | - |
| 64 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 65 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | - | - |
| 66 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 67 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 68 | 100 | 75 |  | 16QAM | 75 | P_75@25 | S_0@0 | - | - |
| 69 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 70 | 100 | 75 |  | 16QAM | 16 | P_16@84 | S_0@0 | - | - |
| 71 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 72 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 74 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 75 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 77 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 78 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 79 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 80 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 81 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 82 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

In [3] for NR single carrier EVM requirement, TP analysis provides with several modifications compared with LTE. In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline. It should be noted that in CA EVM test only PUSCH will be verified. Hence following discussion refer to PUSCH discussion in [3] mainly.

Test Environment

In FR2 single carrier, EVM is tested under normal condition as test environment. Since for FR2 no significant dependence of the EVM on temperature or voltage is expected, using the same test environment as for single carrier is a reasonable approach.

Proposal 1: Define test environment as Normal for CA EVM in FR2.

Test Subcarrier Spacing

In [3] lowest and highest SCS for EVM are selected for single carrier EVM testing. This could be applicable for EVM CA to ensure test coverage.

Proposal 2: Select lowest and highest SCS for CA EVM in FR2.

Test Frequencies

In FR1 CA test case, EVM is tested for Low range and High range for CA test cases. To balance the test coverage and test time, it is proposed to use the same low and high frequency range for NR FR2 EVM CA testing.

Proposal 3: Test EVM for Low range and High range frequencies for CA EVM in FR2.

Test Channel Bandwidth

In LTE and NR FR1, CA EVM is tested for Lowest and Highest channel bandwidths. NR FR2 CA EVM can follow this approach as well.

Proposal 4: Test EVM for Lowest and Highest channel bandwidth for CA EVM in FR2.

Test Modulations and waveforms for EVM CA

The minimum requirements are specified for Pi/2-BPSK, QPSK, 16 QAM and 64 QAM. In order to ensure that the UE meets all these requirements the test has to cover these. Due to the differences in the modulation scheme (with and without precoding) and due to the different CREST factors for DFT-s-OFDM and CP-OFDM both waveforms should be tested.

Proposal 5: Test EVM CA for the modulation waveform combinations stated in Table 2.5-1.

Table 2.5-1: Modulation and waveform combinations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

RB allocation for EVM CA

In LTE and NR FR1, EVM is measured with full allocation and the highest partial allocation where no MPR occurs. The modulations should be tested with outer_full and inner_full allocation to consider the different MPR since EVM depends on the power level.

Proposal 6: Test EVM CA with the modulation, waveform and RB allocation combinations comprised in Table 2.6-1.

Table 2.6-1: Modulation, waveform and RB combinations

| Test ID | Uplink Configuration |  |  |
| --- | --- | --- | --- |
|  | Modulation | PCC RB allocation | SCC RB allocation |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full | 0 |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full | 0 |
| 3 | DFT-s-OFDM QPSK | Inner Full | 0 |
| 4 | DFT-s-OFDM QPSK | Outer Full | 0 |
| 5 | DFT-s-OFDM 16 QAM | Inner Full | 0 |
| 6 | DFT-s-OFDM 16 QAM | Outer Full | 0 |
| 7 | DFT-s-OFDM 64 QAM | Inner Full | 0 |
| 8 | DFT-s-OFDM 64 QAM | Outer Full | 0 |
| 9 | CP-OFDM QPSK | Inner Full | 0 |
| 10 | CP-OFDM QPSK | Outer Full | 0 |
| 11 | CP-OFDM 16 QAM | Inner Full | 0 |
| 12 | CP-OFDM 16 QAM | Outer Full | 0 |
| 13 | CP-OFDM 64 QAM | Inner Full | 0 |
| 14 | CP-OFDM 64 QAM | Outer Full | 0 |

Test point

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR2.

EVM CA PUSCH:

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 14 | 112 |

Conclusion

To sum up, the following selection of test points is proposed for CA EVM in FR2:

Proposal 1: Define test environment as Normal for CA EVM in FR2.

Proposal 2: Select lowest and highest SCS for CA EVM in FR2.

Proposal 3: Test EVM for Low range and High range frequencies for CA EVM in FR2.

Proposal 4: Test EVM for Lowest and Highest channel bandwidth for CA EVM in FR2.

Proposal 5: Test EVM CA for the modulation waveform combinations stated in Table 2.5-1.

Table 2.5-1: Modulation and waveform combinations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

Proposal 6: Test EVM CA PUSCH with the modulation, waveform and RB allocation combinations comprised in Table 2.6-1.

Table 2.6-1: Modulation, waveform and RB combinations

| Test ID | Uplink Configuration |  |
| --- | --- | --- |
|  | Modulation | RB allocation (NOTE 1) |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 3 | DFT-s-OFDM QPSK | Inner Full |
| 4 | DFT-s-OFDM QPSK | Outer Full |
| 5 | DFT-s-OFDM 16 QAM | Inner Full |
| 6 | DFT-s-OFDM 16 QAM | Outer Full |
| 7 | DFT-s-OFDM 64 QAM | Inner Full |
| 8 | DFT-s-OFDM 64 QAM | Outer Full |
| 9 | CP-OFDM QPSK | Inner Full |
| 10 | CP-OFDM QPSK | Outer Full |
| 11 | CP-OFDM 16 QAM | Inner Full |
| 12 | CP-OFDM 16 QAM | Outer Full |
| 13 | CP-OFDM 64 QAM | Inner Full |
| 14 | CP-OFDM 64 QAM | Outer Full |

- References

- R5-19xxxx, Discussion of ……, 3GPP TSG RAN WG5 Meeting #84, Aug 2019

- 3GPP TS 36.521-1 v15.5.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

- R5-186244, “Discussion on test point selection for EVM in FR2”