3GPP TSG-RAN5 Meeting #91-e	R5-212688

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1865 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of nominal channel spacing in test frequencies for CA_n257x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Nominal channel spacing used in test frequency tables for some of the n257 NR intra-band contiguous configurations and CC CBW combinations are not in accordance to TS 38.101-2, clause 5.4A.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 4.3.1.2.3.1.3-1: Corrected nominal channel spacing for CA_n257D and CCBW combination 50+200. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test frequencies for CA_n257D remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.1.3	CA_n257D

Table 4.3.1.2.3.1.3-1: NR Intra-Band contiguous CA configuration CA_n257D, SCS=60 kHz, ΔFRaster 60 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+200 | CC1 | 50 | 60 | 66 | Downlink | Low | 26525.04 | 2054583 | 26501.28 | 2054187 | 0 | 120 |  | 22388 | 2054683 |  | 4 |  | 13 | 1 (8) | 21 |
|  |  |  |  |  | & | Mid | 27900 | 2077499 | 27802.8 | 2075879 | 102 |  |  | 22467 | 2077435 |  | 8 |  | 7 | 0 (0) | 109 |
|  |  |  |  |  | Uplink | High | 29278.08 | 2100467 | 28891.44 | 2094023 | 504 |  |  | 22547 | 2100475 |  | 8 |  | 5 | 1 (8) | 517 |
|  | Channel spacing CC1-CC2=121.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 60 | 264 | Downlink | Low | 26646.96 | 2056615 | 26551.92 | 2055031 | 0 | 120 |  | 22391 | 2055547 |  | 0 |  | 15 | 1 (8) | 23 |
|  |  |  |  |  | & | Mid | 28021.92 | 2079531 | 27853.44 | 2076723 | 102 |  |  | 22470 | 2078299 |  | 4 |  | 1 | 1 (8) | 111 |
|  |  |  |  |  | Uplink | High | 29400 | 2102499 | 28942.08 | 2094867 | 504 |  |  | 22550 | 2101339 |  | 4 |  | 7 | 1 (8) | 519 |
| 100+200 | CC1 | 100 | 60 | 132 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 8 |  | 11 | 1 (8) | 19 |
|  |  |  |  |  | & | Mid | 27900 | 2077499 | 27779.04 | 2075483 | 102 |  |  | 22466 | 2077147 |  | 8 |  | 8 | 1 (8) | 118 |
|  |  |  |  |  | Uplink | High | 29252.52 | 2100041 | 28842.12 | 2093201 | 504 |  |  | 22544 | 2099611 |  | 2 |  | 2 | 1 (8) | 514 |
|  | Channel spacing CC1-CC2=147.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 60 | 264 | Downlink | Low | 26697.48 | 2057457 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 10 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 28047.48 | 2079957 | 27879 | 2077149 | 102 |  |  | 22472 | 2078875 |  | 10 |  | 13 | 1 (8) | 123 |
|  |  |  |  |  | Uplink | High | 29400 | 2102499 | 28942.08 | 2094867 | 504 |  |  | 22550 | 2101339 |  | 4 |  | 7 | 1 (8) | 519 |
| 200+200 | CC1 | 200 | 60 | 264 | Downlink | Low | 26600.04 | 2055833 | 26505 | 2054249 | 0 | 120 |  | 22388 | 2054683 |  | 2 |  | 8 | 1 (8) | 16 |
|  |  |  |  |  | & | Mid | 27900 | 2077499 | 27731.52 | 2074691 | 102 |  |  | 22463 | 2076283 |  | 8 |  | 2 | 1 (8) | 112 |
|  |  |  |  |  | Uplink | High | 29200.08 | 2099167 | 28742.16 | 2091535 | 504 |  |  | 22538 | 2097883 |  | 0 |  | 5 | 0 (0) | 509 |
|  | Channel spacing CC1-CC2=199.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 60 | 264 | Downlink | Low | 26799.96 | 2059165 | 26704.92 | 2057581 | 0 | 120 |  | 22399 | 2057851 |  | 6 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 28099.92 | 2080831 | 27931.44 | 2078023 | 102 |  |  | 22475 | 2079739 |  | 0 |  | 13 | 1 (8) | 123 |
|  |  |  |  |  | Uplink | High | 29400 | 2102499 | 28942.08 | 2094867 | 504 |  |  | 22550 | 2101339 |  | 4 |  | 7 | 1 (8) | 519 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.1.3-2: NR Intra-Band contiguous CA configuration CA_n257D, SCS=120 kHz, ΔFRaster 120 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] |  | point A [MHz] |  | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+200 | CC1 | 50 | 120 | 32 | Downlink | Low | 26525.04 | 2054583 |  | 26502 |  | 2054199 |  | 0 | 120 |  | 22388 | 2054683 |  | 2 |  | 6 | 1 (4) | 20 |
|  |  |  |  |  | & | Mid | 27900 | 2077499 |  | 27730.08 |  | 2074667 |  | 102 |  |  | 22467 | 2077435 |  | 4 |  | 3 | 0 (0) | 210 |
|  |  |  |  |  | Uplink | High | 29278.08 | 2100467 |  | 28529.28 |  | 2087987 |  | 504 |  |  | 22547 | 2100475 |  | 4 |  | 2 | 1 (4) | 1020 |
|  | Channel spacing CC1-CC2=121.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 120 | 132 | Downlink | Low | 26646.96 |  | 2056615 |  | 26551.92 |  | 2055031 | 0 | 120 |  | 22391 | 2055547 |  | 6 |  | 7 | 1 (4) | 22 |
|  |  |  |  |  | & | Mid | 28021.92 |  | 2079531 |  | 27780 |  | 2075499 | 102 |  |  | 22470 | 2078299 |  | 8 |  | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 29400 |  | 2102499 |  | 28579.2 |  | 2088819 | 504 |  |  | 22550 | 2101339 |  | 8 |  | 3 | 1 (4) | 1022 |
| 100+200 | CC1 | 100 | 120 | 66 | Downlink | Low | 26550 | 2054999 |  | 26502.48 |  | 2054207 |  | 0 | 120 |  | 22388 | 2054683 |  | 10 |  | 5 | 1 (4) | 18 |
|  |  |  |  |  | & | Mid | 27900 | 2077499 |  | 27705.6 |  | 2074259 |  | 102 |  |  | 22466 | 2077147 |  | 4 |  | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 29252.52 | 2100041 |  | 28479.24 |  | 2087153 |  | 504 |  |  | 22544 | 2099611 |  | 1 |  | 1 | 1 (4) | 1018 |
|  | Channel spacing CC1-CC2=147.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 120 | 132 | Downlink | Low | 26697.48 |  | 2057457 |  | 26602.44 |  | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 5 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 28047.48 |  | 2079957 |  | 27805.56 |  | 2075925 | 102 |  |  | 22472 | 2078875 |  | 11 |  | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 29400 |  | 2102499 |  | 28579.2 |  | 2088819 | 504 |  |  | 22550 | 2101339 |  | 8 |  | 3 | 1 (4) | 1022 |
| 200+200 | CC1 | 200 | 120 | 132 | Downlink | Low | 26600.04 | 2055833 |  | 26505 |  | 2054249 |  | 0 | 120 |  | 22388 | 2054683 |  | 1 |  | 4 | 1 (4) | 16 |
|  |  |  |  |  | & | Mid | 27900 | 2077499 |  | 27658.08 |  | 2073467 |  | 102 |  |  | 22463 | 2076283 |  | 4 |  | 1 | 1 (4) | 214 |
|  |  |  |  |  | Uplink | High | 29200.08 | 2099167 |  | 28379.28 |  | 2085487 |  | 504 |  |  | 22538 | 2097883 |  | 6 |  | 2 | 0 (0) | 1012 |
|  | Channel spacing CC1-CC2=199.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 120 | 132 | Downlink | Low | 26799.96 |  | 2059165 |  | 26704.92 |  | 2057581 | 0 | 120 |  | 22399 | 2057851 |  | 3 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 28099.92 |  | 2080831 |  | 27858 |  | 2076799 | 102 |  |  | 22475 | 2079739 |  | 6 |  | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 29400 |  | 2102499 |  | 28579.2 |  | 2088819 | 504 |  |  | 22550 | 2101339 |  | 8 |  | 3 | 1 (4) | 1022 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |