3GPP TSG-RAN5 Meeting #91-e	R5-212689

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1866 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of nominal channel spacing in test frequencies for CA_n258x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Nominal channel spacing used in test frequency tables for some of the n258 NR intra-band contiguous configurations and CC CBW combinations are not in accordance to TS 38.101-2, clause 5.4A.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 4.3.1.2.3.2.3-1: Corrected nominal channel spacing for CA_n258D and CCBW combination 50+200. Table 4.3.1.2.3.2.6-1: Corrected nominal channel spacing for CA_n258G and CCBW combination 50+100. Table 4.3.1.2.3.2.7-1: Corrected nominal channel spacing for CA_n258H and CCBW combination 50+100+100. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test frequencies for CA_n258D, CA_n258G and CA_n258H remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2.3.2.3, 4.3.1.2.3.2.6 and 4.3.1.2.3.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.2.3	CA_n258D

Table 4.3.1.2.3.2.3-1: NR Intra-Band contiguous CA configuration CA_n258D (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] |  | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+200 | CC1 | 50 | 60 | 66 | Downlink | Low | 24275.04 | 2017083 | 24251.28 | 2016687 | 0 |  | 120 | 22257 |  | 2016955 | 4 | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 25774.92 | 2042081 | 25677.72 | 2040461 | 102 |  |  | 22344 |  | 2042011 | 2 | 7 | 0 (0) | 109 |
|  |  |  |  |  | Uplink | High | 27278.04 | 2067133 | 26891.4 | 2060689 | 504 |  |  | 22431 |  | 2067067 | 6 | 7 | 0 (0) | 511 |
|  | Channel spacing CC1-CC2=121.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 60 | 264 | Downlink | Low | 24396.96 | 2019115 | 24301.92 | 2017531 | 0 |  | 120 | 22260 |  | 2017819 | 0 | 4 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25896.84 | 2044113 | 25728.36 | 2041305 | 102 |  |  | 22347 |  | 2042875 | 10 | 0 | 1 (8) | 110 |
|  |  |  |  |  | Uplink | High | 27399.96 | 2069165 | 26942.04 | 2061533 | 504 |  |  | 22434 |  | 2067931 | 2 | 1 | 1 (8) | 513 |
| 100+200 | CC1 | 100 | 60 | 132 | Downlink | Low | 24300 | 2017499 | 24252.48 | 2016707 | 0 |  | 120 | 22257 |  | 2016955 | 8 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 25774.92 | 2042081 | 25653.96 | 2040065 | 102 |  |  | 22343 |  | 2041723 | 2 | 8 | 1 (8) | 118 |
|  |  |  |  |  | Uplink | High | 27252.48 | 2066707 | 26842.08 | 2059867 | 504 |  |  | 22428 |  | 2066203 | 0 | 4 | 0 (0) | 508 |
|  | Channel spacing CC1-CC2=147.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 60 | 264 | Downlink | Low | 24447.48 | 2019957 | 24352.44 | 2018373 | 0 |  | 120 | 22263 |  | 2018683 | 10 | 5 | 0 (0) | 5 |
|  |  |  |  |  | & | Mid | 25922.4 | 2044539 | 25753.92 | 2041731 | 102 |  |  | 22349 |  | 2043451 | 4 | 13 | 1 (8) | 123 |
|  |  |  |  |  | Uplink | High | 27399.96 | 2069165 | 26942.04 | 2061533 | 504 |  |  | 22434 |  | 2067931 | 2 | 1 | 1 (8) | 513 |
| 200+200 | CC1 | 200 | 60 | 264 | Downlink | Low | 24350.04 | 2018333 | 24255 | 2016749 | 0 |  | 120 | 22258 |  | 2017243 | 2 | 13 | 1 (8) | 21 |
|  |  |  |  |  | & | Mid | 25774.92 | 2042081 | 25606.44 | 2039273 | 102 |  |  | 22340 |  | 2040859 | 2 | 2 | 1 (8) | 112 |
|  |  |  |  |  | Uplink | High | 27200.04 | 2065833 | 26742.12 | 2058201 | 504 |  |  | 22423 |  | 2064763 | 10 | 14 | 1 (8) | 526 |
|  | Channel spacing CC1-CC2=199.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 60 | 264 | Downlink | Low | 24549.96 | 2021665 | 24454.92 | 2020081 | 0 |  | 120 | 22269 |  | 2020411 | 6 | 7 | 0 (0) | 7 |
|  |  |  |  |  | & | Mid | 25974.84 | 2045413 | 25806.36 | 2042605 | 102 |  |  | 22352 |  | 2044315 | 6 | 12 | 1 (8) | 122 |
|  |  |  |  |  | Uplink | High | 27399.96 | 2069165 | 26942.04 | 2061533 | 504 |  |  | 22434 |  | 2067931 | 2 | 1 | 1 (8) | 513 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.2.3-2: NR Intra-Band contiguous CA configuration CA_n258D (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+200 | CC1 | 50 | 120 | 32 | Downlink | Low | 24275.04 | 2017083 | 24252 | 2016699 | 0 | 120 | 22257 | 2016955 | 8 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 25775.04 | 2042083 | 25605.12 | 2039251 | 102 |  | 22344 | 2042011 | 0 | 3 | 0 (0) | 210 |
|  |  |  |  |  | Uplink | High | 27278.04 | 2067133 | 26529.24 | 2054653 | 504 |  | 22431 | 2067067 | 3 | 3 | 0 (0) | 1014 |
|  | Channel spacing CC1-CC2=121.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 120 | 132 | Downlink | Low | 24396.96 | 2019115 | 24301.92 | 2017531 | 0 | 120 | 22260 | 2017819 | 0 | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25896.96 | 2044115 | 25655.04 | 2040083 | 102 |  | 22347 | 2042875 | 4 | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 27399.96 | 2069165 | 26579.16 | 2055485 | 504 |  | 22434 | 2067931 | 7 | 0 | 1 (4) | 1016 |
| 100+200 | CC1 | 100 | 120 | 66 | Downlink | Low | 24300 | 2017499 | 24252.48 | 2016707 | 0 | 120 | 22257 | 2016955 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 25775.04 | 2042083 | 25580.64 | 2038843 | 102 |  | 22343 | 2041723 | 0 | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 27252.48 | 2066707 | 26479.2 | 2053819 | 504 |  | 22428 | 2066203 | 0 | 2 | 0 (0) | 1012 |
|  | Channel spacing CC1-CC2=147.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 120 | 132 | Downlink | Low | 24447.48 | 2019957 | 24352.44 | 2018373 | 0 | 120 | 22263 | 2018683 | 11 | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25922.52 | 2044541 | 25680.6 | 2040509 | 102 |  | 22349 | 2043451 | 7 | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 27399.96 | 2069165 | 26579.16 | 2055485 | 504 |  | 22434 | 2067931 | 7 | 0 | 1 (4) | 1016 |
| 200+200 | CC1 | 200 | 120 | 132 | Downlink | Low | 24350.04 | 2018333 | 24255 | 2016749 | 0 | 120 | 22258 | 2017243 | 7 | 6 | 1 (4) | 20 |
|  |  |  |  |  | & | Mid | 25775.04 | 2042083 | 25533.12 | 2038051 | 102 |  | 22340 | 2040859 | 0 | 1 | 1 (4) | 214 |
|  |  |  |  |  | Uplink | High | 27200.04 | 2065833 | 26379.24 | 2052153 | 504 |  | 22423 | 2064763 | 5 | 7 | 1 (4) | 1030 |
|  | Channel spacing CC1-CC2=199.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 200 | 120 | 132 | Downlink | Low | 24549.96 | 2021665 | 24454.92 | 2020081 | 0 | 120 | 22269 | 2020411 | 9 | 3 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 25974.96 | 2045415 | 25733.04 | 2041383 | 102 |  | 22352 | 2044315 | 2 | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 27399.96 | 2069165 | 26579.16 | 2055485 | 504 |  | 22434 | 2067931 | 7 | 0 | 1 (4) | 1016 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.2.4	CA_n258E

FFS

4.3.1.2.3.2.5	CA_n258F

FFS

4.3.1.2.3.2.6	CA_n258G

Table 4.3.1.2.3.2.6-1: NR Intra-Band contiguous CA configuration CA_n258G (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low | 24275.04 | 2017083 | 24251.28 | 2016687 | 0 | 120 | 22257 | 2016955 | 4 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 25824.96 | 2042915 | 25727.76 | 2041295 | 102 |  | 22347 | 2042875 | 8 |  | 1 | 1 (8) | 111 |
|  |  |  |  |  | Uplink | High | 27375.6 | 2068759 | 26988.96 | 2062315 | 504 |  | 22437 | 2068795 | 0 |  | 8 | 1 (8) | 520 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 24349.44 | 2018323 | 24301.92 | 2017531 | 0 | 120 | 22260 | 2017819 | 0 |  | 4 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25899.36 | 2044155 | 25778.4 | 2042139 | 102 |  | 22350 | 2043739 | 4 |  | 3 | 1 (8) | 113 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 27039.6 | 2063159 | 504 |  | 22440 | 2069659 | 8 |  | 9 | 1 (8) | 521 |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 24300 | 2017499 | 24252.48 | 2016707 | 0 | 120 | 22257 | 2016955 | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 25824.96 | 2042915 | 25704 | 2040899 | 102 |  | 22346 | 2042587 | 8 |  | 10 | 1 (8) | 120 |
|  |  |  |  |  | Uplink | High | 27350.04 | 2068333 | 26939.64 | 2061493 | 504 |  | 22434 | 2067931 | 6 |  | 4 | 1 (8) | 516 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 24399.96 | 2019165 | 24352.44 | 2018373 | 0 | 120 | 22263 | 2018683 | 10 |  | 5 | 0 (0) | 5 |
|  |  |  |  |  | & | Mid | 25924.92 | 2044581 | 25803.96 | 2042565 | 102 |  | 22352 | 2044315 | 10 |  | 15 | 1 (8) | 125 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 27039.6 | 2063159 | 504 |  | 22440 | 2069659 | 8 |  | 9 | 1 (8) | 521 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.2.6-2: NR Intra-Band contiguous CA configuration CA_n258G (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 120 | 32 | Downlink | Low | 24275.04 | 2017083 | 24252 | 2016699 | 0 | 120 | 22257 | 2016955 | 8 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 25824.96 | 2042915 | 25655.04 | 2040083 | 102 |  | 22347 | 2042875 | 4 | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 27375.6 | 2068759 | 26626.8 | 2056279 | 504 |  | 22437 | 2068795 | 6 | 3 | 1 (4) | 1022 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 24349.44 | 2018323 | 24301.92 | 2017531 | 0 | 120 | 22260 | 2017819 | 0 | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25899.36 | 2044155 | 25704.96 | 2040915 | 102 |  | 22350 | 2043739 | 8 | 1 | 1 (4) | 214 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 26676.72 | 2057111 | 504 |  | 22440 | 2069659 | 10 | 4 | 1 (4) | 1024 |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 24300 | 2017499 | 24252.48 | 2016707 | 0 | 120 | 22257 | 2016955 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 25824.96 | 2042915 | 25630.56 | 2039675 | 102 |  | 22346 | 2042587 | 4 | 5 | 1 (4) | 222 |
|  |  |  |  |  | Uplink | High | 27350.04 | 2068333 | 26576.76 | 2055445 | 504 |  | 22434 | 2067931 | 3 | 2 | 1 (4) | 1020 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 24399.96 | 2019165 | 24352.44 | 2018373 | 0 | 120 | 22263 | 2018683 | 11 | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25924.92 | 2044581 | 25730.52 | 2041341 | 102 |  | 22352 | 2044315 | 11 | 7 | 1 (4) | 226 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 26676.72 | 2057111 | 504 |  | 22440 | 2069659 | 10 | 4 | 1 (4) | 1024 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.2.7	CA_n258H

Table 4.3.1.2.3.2.7-1: NR Intra-Band contiguous CA configuration CA_n258H (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] |  | absoluteFrequencyPointA [ARFCN] |  | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] |  | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low | 24275.04 | 2017083 | 24251.28 | 2016687 |  | 0 |  | 120 |  | 22257 | 2016955 | 4 | 2 | 0 (0) | 2 |
| +100 |  |  |  |  | & | Mid | 25774.92 | 2042081 | 25677.72 | 2040461 |  | 102 |  |  |  | 22344 | 2042011 | 2 | 7 | 0 (0) | 109 |
|  |  |  |  |  | Uplink | High | 27275.64 | 2067093 | 26889 | 2060649 |  | 504 |  |  |  | 22431 | 2067067 | 10 | 2 | 1 (8) | 514 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 24349.44 | 2018323 | 24301.92 | 2017531 |  | 0 |  | 120 | 22260 |  | 2017819 | 0 | 4 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25849.32 | 2043321 | 25728.36 | 2041305 |  | 102 |  |  | 22347 |  | 2042875 | 10 | 0 | 1 (8) | 110 |
|  |  |  |  |  | Uplink | High | 27350.04 | 2068333 | 26939.64 | 2061493 |  | 504 |  |  | 22434 |  | 2067931 | 6 | 4 | 1 (8) | 516 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 24449.4 | 2019989 | 24401.88 | 2019197 |  | 0 |  | 120 | 22266 |  | 2019547 | 2 | 1 | 1 (8) | 9 |
|  |  |  |  |  | & | Mid | 25949.28 | 2044987 | 25828.32 | 2042971 |  | 102 |  |  | 22353 |  | 2044603 | 0 | 6 | 1 (8) | 116 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 27039.6 | 2063159 |  | 504 |  |  | 22440 |  | 2069659 | 8 | 9 | 1 (8) | 521 |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 24300 | 2017499 | 24252.48 | 2016707 |  | 0 |  | 120 |  | 22257 | 2016955 | 8 | 0 | 0 (0) | 0 |
| +100 |  |  |  |  | & | Mid | 25774.92 | 2042081 | 25653.96 | 2040065 |  | 102 |  |  |  | 22343 | 2041723 | 2 | 8 | 1 (8) | 118 |
|  |  |  |  |  | Uplink | High | 27250.08 | 2066667 | 26839.68 | 2059827 |  | 504 |  |  |  | 22428 | 2066203 | 4 | 7 | 0 (0) | 511 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 24399.96 | 2019165 | 24352.44 | 2018373 |  | 0 |  | 120 | 22263 |  | 2018683 | 10 | 5 | 0 (0) | 5 |
|  |  |  |  |  | & | Mid | 25874.88 | 2043747 | 25753.92 | 2041731 |  | 102 |  |  | 22349 |  | 2043451 | 4 | 13 | 1 (8) | 123 |
|  |  |  |  |  | Uplink | High | 27350.04 | 2068333 | 26939.64 | 2061493 |  | 504 |  |  | 22434 |  | 2067931 | 6 | 4 | 1 (8) | 516 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 24499.92 | 2020831 | 24452.4 | 2020039 |  | 0 |  | 120 | 22269 |  | 2020411 | 0 | 3 | 1 (8) | 11 |
|  |  |  |  |  | & | Mid | 25974.84 | 2045413 | 25853.88 | 2043397 |  | 102 |  |  | 22354 |  | 2044891 | 6 | 2 | 0 (0) | 104 |
|  |  |  |  |  | Uplink | High | 27450 | 2069999 | 27039.6 | 2063159 |  | 504 |  |  | 22440 |  | 2069659 | 8 | 9 | 1 (8) | 521 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.2.7-2: NR Intra-Band contiguous CA configuration CA_n258H (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] |  | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 120 | 32 | Downlink | Low | 24275.04 |  | 2017083 | 24252 | 2016699 | 0 | 120 |  | 22257 | 2016955 | 8 | 0 | 0 (0) | 0 |
| +100 |  |  |  |  | & | Mid | 25775.04 |  | 2042083 | 25605.12 | 2039251 | 102 |  |  | 22344 | 2042011 | 0 | 3 | 0 (0) | 210 |
|  |  |  |  |  | Uplink | High | 27275.64 |  | 2067093 | 26526.84 | 2054613 | 504 |  |  | 22431 | 2067067 | 11 | 0 | 1 (4) | 1016 |
|  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 24349.44 |  | 2018323 | 24301.92 | 2017531 | 0 | 120 | 22260 |  | 2017819 | 0 | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25849.44 |  | 2043323 | 25655.04 | 2040083 | 102 |  | 22347 |  | 2042875 | 4 | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 27350.04 |  | 2068333 | 26576.76 | 2055445 | 504 |  | 22434 |  | 2067931 | 3 | 2 | 1 (4) | 1020 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 24449.4 |  | 2019989 | 24401.88 | 2019197 | 0 | 120 | 22266 |  | 2019547 | 7 | 0 | 1 (4) | 8 |
|  |  |  |  |  | & | Mid | 25949.4 |  | 2044989 | 25755 | 2041749 | 102 |  | 22353 |  | 2044603 | 11 | 2 | 1 (4) | 216 |
|  |  |  |  |  | Uplink | High | 27450 |  | 2069999 | 26676.72 | 2057111 | 504 |  | 22440 |  | 2069659 | 10 | 4 | 1 (4) | 1024 |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 24300 |  | 2017499 | 24252.48 | 2016707 | 0 | 120 |  | 22257 | 2016955 | 4 | 0 | 0 (0) | 0 |
| +100 |  |  |  |  | & | Mid | 25775.04 |  | 2042083 | 25580.64 | 2038843 | 102 |  |  | 22343 | 2041723 | 0 | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 27250.08 |  | 2066667 | 26476.8 | 2053779 | 504 |  |  | 22428 | 2066203 | 8 | 3 | 0 (0) | 1014 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 24399.96 |  | 2019165 | 24352.44 | 2018373 | 0 | 120 | 22263 |  | 2018683 | 11 | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 25875 |  | 2043749 | 25680.6 | 2040509 | 102 |  | 22349 |  | 2043451 | 7 | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 27350.04 |  | 2068333 | 26576.76 | 2055445 | 504 |  | 22434 |  | 2067931 | 3 | 2 | 1 (4) | 1020 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 24499.92 |  | 2020831 | 24452.4 | 2020039 | 0 | 120 | 22269 |  | 2020411 | 6 | 1 | 1 (4) | 10 |
|  |  |  |  |  | & | Mid | 25974.96 |  | 2045415 | 25780.56 | 2042175 | 102 |  | 22354 |  | 2044891 | 2 | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 27450 |  | 2069999 | 26676.72 | 2057111 | 504 |  | 22440 |  | 2069659 | 10 | 4 | 1 (4) | 1024 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |