3GPP TSG-RAN WG5 Meeting #91-e	R5-212813

Electronic Meeting, May 17 – May 28, 2021

Title:		Discussion on FR2 UL CA Tx test config issues

Source:			Qualcomm Inc.

Agenda Item:		5.3.2.17

Document for:		Discussion & endorsement

# Introduction

The purpose of this paper is to bring to attention potential test configuration issues in FR2 UL CA Tx test cases.

# Discussion

Two issues are discussed in this paper:

- FR2 IBE test procedure:

The test procedure for FR2 IBE UL CA test case is shown below. The step # 1 in the test procedure is to retrieve LO position from parameter txDirectCurrentLocation.

TS 38.521-2 -> In-Band Emissions for CA -> 6.4A.2.3.1.4.2	Test procedure

1.	Retrieve the LO position from the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE.

2.	Configure SCC according to Annex C.0, C.1 and C.3.0 for all downlink physical channels.

3. 	…..

Observation 1: Currently there is no RRC framework defined to retrieve this LO position as indicated in the test procedure step # 1 above.

Proposal 1: Define a test framework to address LO position retrieval, without which the current FR2 IBE test procedure in TS38.521-2 cannot be implemented.

- FR2 DL CC configuration for all Tx UL CA test cases:

The highlighted portion in the snippet below from 38.101-2 -> 5.3A.4 indicates that all FR2 UL CA requirements apply for test condition DL CC >= UL CC.

### 5.3A.4	UE channel bandwidth per operating band for CA

For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class with associated bandwidth combination sets specified in clause 5.5A.1. For each carrier aggregation configuration, requirements are specified for all aggregated channel bandwidths contained in a bandwidth combination set, UE can indicate support of several bandwidth combination sets per carrier aggregation configuration. The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier.

Observation 2: Currently RAN 5 testing for all FR2 Tx UL CA test cases configures DL SCC as shown in example test procedure below but does not specify the number of DL SCC configured.

38.521-2 -> 6.2A.1.1.1.4.2	Test procedure

1. Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2. The SS shall configure SCC as per TS 38.508-1 [10] subclause 5.5.1. Message contents are defined in clause 6.2A.1.1.1.4.3.

3. 	…..

Proposal 2: For all multiple CC UL CA FR2 Tx test cases in TS 38.521-2 RAN 5 tests shall configure DL CC >= UL CC.

# Conclusion

Proposal 1: Define a test framework to address LO position retrieval, without which the current FR2 IBE test procedure in TS38.521-2 cannot be implemented.

Proposal 2: For all multiple CC UL CA FR2 Tx test cases RAN 5 tests shall configure DL CC >= UL CC.