3GPP TSG-RAN WG5 Meeting #91-e	R5-212830

# Electronic meeting, 17th - 28th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0188 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of A.4.1 for UE implementation types for SA CA UE radio technologies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The release info for Intra-Band Non-contiguous CA within FR1 in table A.4.1-4A is incorrect and should be corrected. In addition, the notation of Inter-band EN-DC including FR1 and FR2 in Table A.4.1-4 should be consistence with clause A.4.3.2B.2.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct release info for Intra-Band Non-contiguous CA within FR1 in table A.4.1-4A to Rel-16. - Editorial correction to Inter-band EN-DC including FR1 and FR2 in Table A.4.1-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE implementation types for SA CA UE radio technologies will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

A.4.1	UE Implementation Types

Table A.4.1-1: UE Radio Technologies

| Item | UE Radio Technologies | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR FDD | 38.101-1 | Rel-15 | pc_nrFDD |  |
| 2 | NR TDD | 38.101-1, 38.101-2 | Rel-15 | pc_nrTDD |  |

Table A.4.1-2: UE general functionality

| Item | UE Functionality | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Multiple NR FDD bands | 38.101-1, 5.2 | Rel-15 | pc_nrFDD_MultiBand |  |
| 2 | Multiple NR TDD bands | 38.101-1, 5.2, 38.101-2, 5.2 | Rel-15 | pc_nrTDD_MultiBand |  |
| 3 | NR SUL | 38.101-1 | Rel-15 | pc_nrSUL |  |
| 4 | NR SDL | 38.101-1 | Rel-15 | pc_nrSDL |  |
| 5 | Multiple NR SUL bands | 38.101-1, 5.2 | Rel-15 | pc_nrSUL_MultiBand |  |
| 6 | Multiple NR SDL bands | 38.101-1, 5.2 | Rel-15 | pc_nrSDL_MultiBand |  |
| 7 | Frequency range FR1 | 38.101-1, 5.1 | Rel-15 | pc_nrFR1 |  |
| 8 | Frequency range FR2 | 38.101-2, 5.1 | Rel-15 | pc_nrFR2 |  |

Table A.4.1-3: RAN-CN Interface Options

| Item | UE support of RAN-CN Interface Options | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NG-RAN NR | 38.300 | Rel-15 | pc_NG_RAN_NR | Option 2 |
| 2 | EN-DC | 37.340 | Rel-15 | pc_EN_DC | Option 3 |
| 3 | NE-DC | 37.340 | Rel-15 | pc_NE_DC | Option 4 |
| 4 | NG-RAN E-UTRA | 38.300 | Rel-15 | pc_NG_RAN_EUTRA | Option 5 |
| 5 | NGEN-DC | 37.340 | Rel-15 | pc_NGEN_DC | Option 7 |

Table A.4.1-4: NSA DC UE Radio Technologies

| Item | NSA UE Radio Technologies | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Intra-Band Contiguous EN-DC | 38.101-3, 5.5B.2 | Rel-15 | pc_IntraBand_Contiguous_ENDC |  |
| 2 | Intra-Band Non-Contiguous EN-DC | 38.101-3, 5.5B.3 | Rel-15 | pc_IntraBand_Non_Contiguous_ENDC |  |
| 3 | Inter-Band EN-DC within FR1 | 38.101-3, 5.5B.4 | Rel-15 | pc_InterBand_ENDC_WithinFR1 |  |
| 4 | Inter-Band EN-DC including FR2 | 38.101-3, 5.5B.5 | Rel-15 | pc_InterBand_ENDC_IncludingFR2 |  |
| 5 | Inter-band EN-DC including FR1 and FR2 | 38.101-3, 5.5B.6 | Rel-15 | pc_InterBand_ENDC_IncludingFR1_FR2 |  |
| 6 | Inter-band NR-DC between FR1 and FR2 | 38.101-3, 5.5B.7 | Rel-15 | pc_InterBand_NRDC_BetweenFR1_FR2 |  |

Table A.4.1-4A: SA CA UE Radio Technologies

| Item | SA UE Radio Technologies | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Intra-Band Contiguous CA within FR1 | 38.101-1, 5.5A.1 | Rel-15 | pc_IntraBand_Contiguous_CA_WithinFR1 |  |
| 2 | Intra-Band Non-contiguous CA within FR1 | 38.101-1, 5.5A.2 | Rel-16 | pc_IntraBand_NonContiguous_CA_WithinFR1 |  |
| 3 | Intra-Band Contiguous CA within FR2 | 38.101-2, 5.5A.1 | Rel-15 | pc_IntraBand_Contiguous_CA_WithinFR2 |  |
| 4 | Intra-Band Non-contiguous CA within FR2 | 38.101-2, 5.5A.2 | Rel-15 | pc_IntraBand_NonContiguous_CA_WithinFR2 |  |
| 5 | Inter-Band CA within FR1 | 38.101-1, 5.5A.3 | Rel-15 | pc_InterBand_CA_WithinFR1 |  |
| 6 | Inter-Band CA within FR2 | 38.101-2, 5.5A.3 | Rel-16 | pc_InterBand_CA_WithinFR2 |  |
| 7 | Inter-band CA between FR1 and FR2 | 38.101-3, 5.5A.1 | Rel-15 | pc_InterBand_CA_BetweenFR1_FR2 |  |

Table A.4.1-5: 5GS UE Core Technologies

| Item | 5GS UE Core Technologies | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | UE Supports 5G Core Network | 24.501 | Rel-15 | pc_5GCN |  |
| 2 | UE Supports 5G Core Network over non-3GPP Access Network | 24.501, 4.7 | Rel-15 | pc_5GCN_N3AN |  |

==============================================================

<< End of changes >>